3GPP TSG RAN WG2 Meeting #127												                    R2-2407499

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:		8.5.3

Title:						On-demand SIB1 for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The Release 19 WI for Network Energy Saving (NES) was approved in RAN#102 [1]. One of the objectives is the following study:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. Checkpoint for normative work in RAN#105

This contribution focuses on some of the relevant aspects concerning on-demand SIB1 transmissions for UEs in idle/inactive mode, according to the study mentioned above. In particular, the following questions stated in the “Agenda” of the RAN2 meeting #127 [2] are addressed.

Leftover issues from RAN2#126 and remaining essential issues for study (e.g. need of WUS configuration acquisition from NES cell (if needed, what’s use cases and how it works)?, need of SIB1 validity (if needed, what’s use case and how it works), whether/how to support case 3 defined in RAN1 (e.g. reuse/enhance on-demand SIB request procedure)?, etc.). Note study of OD-SIB1 needs to be concluded

# WUS Configuration Acquisition on NES Cell

So far, both RAN1 and RAN2 have focused on the possible design of on-demand SIB1 when the WUS configuration is carried by a different cell (named Cell A in ongoing discussions), corresponding to Scenario 1a in RAN2 discussions. For the completeness of the study, it is important to understand when it is beneficial to have the WUS configuration directly carried by the NES cell (standalone solution – Scenario 3 in RAN2) and which designs are possible.

## WUS Configuration on NES Cell: motivation and Use Cases

There are several advantages if the WUS configuration can also be acquired on the NES cell. In at least three scenarios WUS configuration in NES cell is important and cannot be replaced by WUS configuration in another cell without loss of functionality or performance:

- The UE is performing Initial Cell Search and the NES cell frequency is found first.

- A cell operating normally is preparing to switch to energy saving mode with on-demand SIB1. Any remaining UEs need to be informed.

- A UE is normally camped on NES cell and SIB1 becomes invalid after three hours (as stated in Section 5.2.2.2.1 of TS 38.331 [3]).

- Observation 1: There are several scenarios where UL WUS configuration in NES cell is important and cannot be replaced by UL WUS configuration in another cell without loss of functionality or performance.

These three cases have something in common. The UE is currently doing some operation on the NES cell, but to keep operation it needs the UL WUS configuration. If it is not available from NES Cell the UE needs to stop what is doing, search for a Cell A carrying the WUS configuration, synchronize to that cell, obtain system information (SI) from that cell, obtain the WUS configuration from Cell A and only then finally come back to the NES Cell to continue normal operation. Let us analyse each of these cases.

The Initial Cell Search is performed when the UE has no information about the RF environment, for example, if the UE is turned on for the first time or if the UE enters a different country. The order of search is UE implementation specific and there are thousands of potential frequencies (GSCNs) to be scanned. As soon as the UE finds one suitable cell, information about other cells can be obtained from the SI. Due to the implementation specific order, there is no guarantee that the UE will find a Cell A before finding an NES cell. The more popular on-demand SIB1 gets, the more often UEs will find a NES cells first. Without obtaining SIB1 the UE cannot even evaluate whether the cell is suitable.  As SIB1 from NES cell can only be obtained on-demand, the UE needs the UL WUS configuration. However, at this point the UE does not know that the UL WUS configuration can be obtained from another cell (Cell A). So, what is the UE supposed to do? Unless this procedure is changed, the UE will have to discard information about this frequency (GSCN) and search further until it finds a suitable cell currently broadcasting SIB1. Thus, the impossibility to evaluate a cell readily may lead to performance impact on Initial Cell Search whenever on-demand SIB1 is activated. The simpler alternative is that the UE can obtain WUS configuration directly from the NES cell. In this way the information is readily available no matter which cell is first identified by the UE. The initial cell search performance can be attained, and UE battery preserved.

TS 38.304 [5] (Figure 5.2.2-1) says that a UE should first check for stored cell information to optimize the Initial Cell Search. If the WUS configuration is not carried by the NES Cell, it may inadvertently increase how often the UEs need to do a full RF scan. Let us consider, for example, the possibility of users turning their mobiles off or putting them to flight mode at night; and the UE caches the cell data before shutting down the radio. Also, consider that meanwhile the gNB decides to put that cell into on-demand SIB1 mode for the night. As the user turns on the mobile again, it is worth considering what might happen to the cache. Since the cell is not broadcasting SIB1 anymore, it cannot be found as being a suitable cell by the UE. Unless there is further change of the Initial Cell Search procedure, the UE will declare the cell to be not suitable and start to scan other frequencies. Here again, the simpler operation and simpler spec is to support WUS configuration on NES cell.

NES adaptations typically are applicable during certain times of the day and on-demand SIB1 is no exception. A cell could be operating without any energy saving during the day broadcasting SIB1 normally while applying NES features like on-demand SIB1 during the night. This also means that the cell may perform a transition between these two states. When a gNB decides that a cell will start applying on-demand SIB1, the remaining UEs on that cell should be informed. The UEs on RRC_CONNECTED state could be informed e.g. by SuspendConfig and moved to RRC_INACTIVE. In this way, they will know the WUS configuration if they need to re-connect. The UEs camping on the cell could be informed that the cell is applying on-demand SIB1 via a change in  identification on PBCH (new k_ssb value), but the WUS configuration needs to be broadcast on the NES cell at least sparsely or during a time window after the change.

Even if a UE is doing no activity, it needs to re-obtain SI after three hours and this includes the SIB1. If the UE obtains WUS configuration from Cell A and then camps on the NES cell, after three hours the UE needs to send the WUS again to obtain SIB1. The WUS configuration cannot be considered valid forever, otherwise re-configuring it is not possible. Thus, the UE needs to obtain the WUS configuration again. It may not be acceptable  in terms of the resulting UE complexity and UE power consumption that the UE needs to find a Cell A again to re-obtain SI. Note that in this case the UE is not even doing any activity but only camping. The better alternative at that point in time is that the UE can obtain WUS configuration from the NES cell. Note also that if all camping UEs on all NES cells need to find a Cell A, this may also lead to excessive request rate on Cell A, leading also to reduced energy savings.

- Proposal 1: UL WUS configuration on NES Cell is supported.

## WUS Configuration on NES Cell – How it Works

In addition to the need of the WUS configuration on NES cell, also its feasibility is important. Some concerns about the WUS configuration on NES cell were: an eventual need for large changes on PBCH/MIB and whether energy can still be saved when WUS configuration is carried by the NES cell itself. We argue that these two issues can be avoided through proper design with little specification effort. As thoroughly analysed in the accompanying RAN1 contribution [7], other channels (instead of PBCH/MIB) can be used to carry the WUS configuration, namely, DCI (PDCCH) or PDSCH. Furthermore, the key to save energy while still broadcasting the WUS configuration is to efficiently multiplex it with the SSBs. In that context, the two most promising approaches are: carrying the WUS configuration on DCI close to SSB and a “SIB0” approach where the WUS configuration is sent on PDSCH that is FDM-ed to the SSBs. As shown in Figure 1 the energy saving gains of these two approaches are quite close to not transmitting SIB1 at all (only SSBs transmitted in the carrier). The DCI-based approach is more efficient and PDSCH based approach is slightly simpler in terms of specification effort[7]. It follows from these results that both approaches, a DCI-based and a SIB0 (PDSCH FDM-ed to SSB) are viable ways to accomplish very efficient transmission of WUS configuration on NES cell.

Observation 2: Both approaches, a DCI-based approach and a SIB0 (PDSCH FDM-ed to SSB) are viable to accomplish very efficient transmission of the WUS configuration in the NES cell.

Figure 1 – Energy saving gains for standalone on-demand SIB1, varying the amount of SSB beams per burst, load and carriage of UL WUS configuration.

# UL WUS Configuration Overhead

The energy savings of on-demand SIB1 depend not only on the reduced number of SIB1 transmissions, but also on the extra energy spent to transmit the UL WUS configuration. Therefore, it is essential to design the transport of the UL WUS configuration in an energy efficient way. The following applies to the three different cases:

- In Scenario 3 (WUS configuration NES Cell), the smaller the UL WUS configuration the better from an energy perspective.

- In Scenario 1a (WUS configuration on Cell A), a Cell A will carry UL WUS configuration for many NES cells. Therefore, even a moderate content per NES cell (e.g. 30 octets) can quickly grow large to an extent of spending too much energy for the broadcast on Cell A.

- In Scenario 1b (SIB1 obtained from Cell A), the existing mechanism for requesting Other SI can be reused. There is no need for a new UL WUS configuration. However, the transmission of SIB1 itself should be optimized.

Therefore, it is very important to reduce the UL WUS configuration overhead in Scenario 1a and 3, as well as SIB1 transmission overhead in scenario 1b.

Observation 2: It is very important to reduce UL WUS configuration overhead in Scenario 1a and Scenario 3.

As agreed in RAN1#117 [4] the following list of parameters is a starting point to discuss the contents of the UL WUS configuration. The estimated size of this list is about 30 octets and RAN1 should thoroughly discuss the design to minimize this overhead.

Agreement For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration. Table I. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResource; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. rsrp-ThresholdSSB. prach-RootSequenceIndex. msg1-SubcarrierSpacing. restrictedSetConfig. tdd-UL-DL-ConfigurationCommon. frequencyInfoUL; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz. SIB1 reception; pdcch-ConfigSIB1; ssb-SubcarrierOffset. controlResourceSetZero. searchSpaceZero. RAR Reception; pdcch-ConfigOD-SIB1-RAR; controlResourceSet. monitoringSlotPeriodicityAndOffset. Duration. monitoringSymbolsWithinSlot. aggregationLevels.

A first point of optimization, to reduce UL WUS configuration overhead, is the removal of parameters related to Random Access Response (RAR). The needed fields are relatively large (three bits per aggregation level, duration and monitoringSlotPeriodicityAndOffset may need up to 12 bits representation each, monitoringSymbolsWithinSlot is 14 bits long). A practical size may be around 60 bits just for this RAR configuration.  Furthermore, RAN2 already ruled out a combined UL WUS + RA procedure [6]. Therefore, the only use for RAR would be as an explicit acknowledgement of a successful UL WUS reception, which, however, is not needed at all. As already agreed, after the UE sends the UL WUS, since the UE will monitor for SIB1 during a time window,  the transmission of SIB1 itself can be an implicit acknowledgment for the UL WUS reception. This will not only save a lot of UL WUS configuration overhead but also make the procedure simpler.

Proposal 2: The transmission of on-demand SIB1 is used as implicit acknowledgement for UL WUS (SIB1 request) reception, i.e.  RAR is not required in the UL WUS Configuration.

# SIB1 Directly from Anchor Cell

Scenario 1b (Case 3 in RAN1) is a special case of the SIB1-less operation widely discussed in Rel-18 Study Item and captured in TR 38.864 [4]. An important difference being that the cost of broadcasting SIB1 for another cell in SI was considered to be always there (cross-carrier SIB as part of regularly transmitted SIs) while now it is on discussion to do that on-demand. For such on-demand operation, the simplest way would be to reuse the Other SI mechanism. The cross-carrier SIB can be put into some new SI container and that new SI is transmitted using the existing mechanism to request Other SI.

Proposal 3: Scenario 1b (case 3 in RAN1), if supported, should reuse the on-demand Other SI mechanism.

As discussed in the previous section, a single non-NES cell may have to carry information to many NES Cells. Because of that, it may be prohibitive to send fully sized SIB1s of other cells even if that is done only on request. Therefore, it is important to reduce the cross-carrier SIB1 overhead, for example by applying some kind of compression of the information.

Proposal 4: Scenario 1b (case 3 in RAN1), if supported, should compress the SIB1 information from NES cells.

# Conclusions

In this contribution, we discussed some aspects of on-demand SIB1 for NES. The following observations and proposals have been made:

- Observation 1: There are several scenarios where UL WUS configuration in NES cell is important and cannot be replaced by UL WUS configuration in another cell without loss of functionality or performance.

Observation 2: Both a DCI-based approach and a SIB0 (PDSCH FDM-ed to SSB) are viable ways to accomplish very efficient transmission of WUS configuration on NES cell.

- Proposal 1: UL WUS configuration on NES Cell is supported.

Proposal 2: The transmission of on-demand SIB1 is used as implicit acknowledgement for UL WUS (SIB1 request) reception, i.e.  RAR is not required in the UL WUS Configuration.

Proposal 3: Scenario 1b (case 3 in RAN1), if supported, should reuse the on-demand Other SI mechanism.

Proposal 4: Scenario 1b (case 3 in RAN1), if supported, should compress the SIB1 information from NES cells.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), 3GPP TSG RAN Meeting #102	 Edinburgh, Scotland, December 2023.

- “Agenda”, InterDigital (RAN2 Chair), 3GPP TSG-RAN WG2 Meeting #127, Maastricht, Netherlands, August 2024.

- TS 38.331, “	NR; Radio Resource Control (RRC); Protocol specification” ,  v18.2.0, July 2024.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117, Fukuoka, Japan, May 20th – 24th, 2024.

- TS 38.304, “	NR; User Equipment (UE) procedures in Idle mode and in RRC Inactive state” ,  v18.3.0, July 2024.

- R2-2405701, “Report from session on V2X/SL, R19 NES and MOB”, Samsung, 3GPP TSG RAN WG2 #126, Fukuoka, Japan, May 20th – 24th, 2024.

- R1-2407103, “On-demand SIB1 for NES”, Fraunhofer IIS, Fraunhofer HHI, Vodafone, Deutsche Telekom, CEWiT, 3GPP TSG RAN WG1 #118, Maastricht, Netherlands, August 2024