3GPP TSG-RAN WG2 Meeting #127                                    	R2-2407502

Maastricht, The Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.9.3

Source:	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot

Title:	Discussion on DSA and CRDSA Performance

Document for:	Discussion and Decision

- Introduction

RAN WID [1] has following objective for the EDT enhancements.

Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

- Msg3 transmission without msg1/ Random Access Response (RAR)

In this document, we provide some results on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted ALOHA (CRDSA) about the expected capacity gain for uplink Msg3-EDT transmissions.

- System Assumptions for Msg3-EDT Enhancements

In the previous ESA contribution [2], two solutions, namely Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted ALOHA (CRDSA) [3], have been introduced for boosting the uplink capacity enhancements using Msg3-EDT without Msg1 / RAR initial steps in satellite systems (both GEO and NGSO).

With respect to the previous document, hereafter the two solutions have been tailored to some possible NB-IoT NTN configurations to better quantify the capacity gain with respect to the legacy Slotted ALOHA (SA) approach.

As a reference scenario, we start considering the case of uplink single-tone transmission and 48 sub-carriers in the total bandwidth of 180 kHz (i.e., sub-carrier spacing equal to 3.75 kHz). Essentially, the IoT NTN UE is randomly selecting a single tone out of Nsc=48 available and then transmitting within a given EDT occasion. The eNB has allocated Nopp consecutive EDT occasions, and each NTN UE is transmitting K replica (K<=Nopp) within the whole EDT occasion period. It is important to remember that for each terminal only one replica can be transmitted in each EDT occasion randomly selected among the Nsc available sub-carriers (0≤R≤Nsc-1). This is to be compliant with the terminal transmit RF power constraint.

For sake of clarity, these general definitions and assumptions have been depicted in the following figure.

Figure 1: Pictorial view of Msg3 transmissions in N consecutive EDT occasions.

In order to derive the Msg3-EDT duration, among the allowed values in TS 36.213 (see Table 16.3.3-2), we have selected the TBS value equal to 328 bits, which represents the smallest packet size. In addition, to present different combinations of EDT duration and equivalently also the spectral efficiency of each Msg3-EDT transmission, we have selected two configurations:

- Config#1: by selecting IMCS=011 from the Table 16.3.3-3, the proper number of NRU is set to 3, that means a single EDT occasion duration is equal to 96 ms.

- Config#2: by selecting IMCS=100 from the Table 16.3.3-3, the proper number of NRU is set to 4, that means a single EDT occasion duration is equal to 128 ms.

The main system assumptions for the capacity evaluation with the new proposed schemes are summarized in the following table.

Table 1: Main system assumptions for the evaluation of SA, DSA, and CRDSA uplink capacity.

| NTN Scenario | GEO / 3GPP Set-1 |
| --- | --- |
| NTN Channel | AWGN |
| Sub-carrier spacing (SCS) | 3.75 kHz |
| Number of sub-carriers ( Nsc ) | 48 |
| Uplink SNR | 4.8 dB for all receive packets Note: From TR 36.763, the GEO UL SNR is 2.6 dB. We removed 2.2 dB margin for scintillations which appears unjustified for a typical (not worst-worst-case) link budget. |
| Msg3-EDT TBS ( Nbitspkt ) | 328 bits |
| Modulation | p/4-QPSK |
| Config#1 |  |
| Single EDT duration ( TEDT ) | 96 ms |
| FEC Coding rate ( r ) | 0.61 (see Appendix) |
| Number of EDT occasions ( Nopp ) | 2 or 3 or 4 |
| Number of replicas ( K ) | 2 or 3 |
| CRDSA SIC iterations ( Niter ) | 4 |
| Config#2 |  |
| Single EDT duration ( TEDT ) | 128 ms |
| FEC Coding rate ( r ) | 0.46 (see Appendix) |
| Number of EDT occasions ( Nopp ) | 2 or 3 |
| Number of replicas ( K ) | 2 or 3 |
| CRDSA SIC iterations ( Niter ) | 4 |

To simplify the uplink capacity evaluation, it is implemented a collision-based assessment of all proposed scheme:

- we assume a transmitted Msg3 is successful decoded if it does not collide with another transmitted message;

- if the Msg3 is successfully decode, all its replicas are removed (or cancelled) from the other EDT occasions;

- the decoding is unsuccessful if it collides with another message (unless all interferers have been cancelled).

It is important to underline that it is a worst-case scenario, because in certain conditions (e.g., large SNR, different FEC coding rates, and received packet power unbalance) the real physical layer turbo decoder might successfully decode a packet even in presence of collisions [4], [5]. In particular, the received packet power unbalance, which is a very realistic working condition due to different UE locations within the satellite coverage, will improve the overall system throughput. This is not the case for the assumptions made here in this example for Config#1 and Config#2.

Finally, for the capacity assessment, it is important to define the NTN UE traffic generation. The traffic is Poisson distributed and the normalized load in bits/symbol is computed as:

Gnorm=Npkr log2MNscNopp

where Npk is the average number of packets, Nsc is the number of NB-IoT subcarriers, Nopp is the number of EDT opportunities, r is the FEC coding rate, and M the modulation order (i.e., 4 for QPSK). The traffic load in terms of bit/s can be computed as

G=NpkNbitspktTEDTtot=GnormNsc NbitspktTEDT r log2M

where Nbitspkt corresponds to the number of bits in the Msg3-EDT packet, TEDTtot=NoppTEDT where TEDT is the single EDT opportunity duration in seconds. The throughput in bits/s is defined as:

T(G)=G[1-PLR(G)]

where the PLR function represents the traffic Packet Loss Ratio derived from the simulation.

## 2.1 Slotted Aloha (SA) Performance

The legacy Config#1 SA uplink performance is reported in Figure 2. In this case, each NTN IoT UE is transmitting only K=1 Msg3-EDT packet during the Nopp occasions.

The PLR is shown as function of traffic load in kbit/s and for two cases of total number of EDT occasions, Nopp=2 and Nopp =4, respectively. It can be observed that the results are not dependent on the number of EDT occasions.

Figure 2: Config#1 Slotted Aloha PLR vs. Traffic load in kbit/s as a function of Nopp. On the left side, a zoom is shown.

Using the Config#1 parameters, the obtained uplink capacity with 1% of packet loss ratio is about 1.8 kbit/s.

Figure 3: Config#2 Slotted Aloha PLR vs. Traffic load in kbit/s as a function of Nopp. On the left side, a zoom is shown.

The legacy Config#2 SA uplink performance is reported in Figure 3. Using the Config#2 parameters, the obtained uplink capacity with 1% of packet loss ratio is about 1.4 kbit/s. As expected, the throughput scales following the ratio between the FEC coding rates.

## 2.2 Diversity Slotted Aloha (DSA) Performance

The Config#1 DSA uplink performance is reported in Figure 4. In this case, each NTN IoT UE is transmitting K=2 replicas of the same Msg3-EDT packet during the Nopp occasions.

The PLR is shown as function of traffic load in kbit/s and for two cases of total number of EDT occasions, Nopp=2 and Nopp =4, respectively. It can be observed that the results are not dependent on the number of EDT occasions.

Figure 4: Config#1 Diversity Slotted Aloha PLR vs. Traffic load in kbit/s as a function of Nopp. On the left side, a zoom is shown.

Using the Config#1 parameters, the obtained uplink capacity with 1% of packet loss ratio is about 8 kbit/s. The capacity improvement is 450% with respect to Config#1 SA scenario.

Figure 5: Config#2 Diversity Slotted Aloha PLR vs. Traffic load in kbit/s as a function of Nopp. On the left side, a zoom is shown.

Using the Config#2 parameters (see Figure 5), the obtained uplink capacity with 1% of packet loss ratio is about 6 kbit/s. The capacity improvement is 450% with respect to Config#2 SA scenario.

Observation 1: The DSA solution is improving the Msg3-EDT throughput by about a factor of 4.5 without any impacts on the current receiver implementation at the network, and minimal specification impact overall.

## 2.3 Contention Resolution Diversity Slotted Aloha (CRDSA) Performance

The Config#1 CRDSA uplink performance is reported in Figure 6. For clarity, it is recalled that the duration of all consecutive EDT occasions, TEDTtot, is 192 ms, 288 ms, and 384 ms for Nopp=[2 ,3, 4], respectively.

Again, fixing the target PLR at 1%, the Config#1 CRDSA performance varies from 34 kbit/s (with the shortest TEDTtot) to 74 kbit/s (with the longest TEDTtot and K=3). The counterintuitive result that with 3 replicas the throughput is almost doubled is related to the reduction of the so-called loop phenomena [4], [5]. Interestingly, with the intermediate TEDTtot value, i.e., 288 ms, and keeping the number of replicas to K=3, the CRDSA performance is only slightly penalized, decreasing to 71 kbit/s.

Recalling that Config#1 SA results at PLR=1% is 1.8 kbit/s, it is straightforward to notify a huge capacity improvement compared to current SA-based standard approach, varying from 1888% to 4111%.

Figure 6: Config#1 CRDSA PLR vs. Traffic load in kbit/s as function of Nopp and number of replicas, K.

Figure 7: Config#2 CRDSA PLR vs. Traffic load in kbit/s as function of Nopp and number of replicas, K.

The Config#2 CRDSA uplink performance is reported in Figure 7. In this scenario, the TEDTtot is 256 ms, and 384 ms for Nopp=[2 ,3], respectively. The largest duration value is identical to the largest value in Config#1.

In this case, the single Msg#3-EDT transmission has a lower spectral efficiency (i.e., r log2M=0.86, instead of r log2M=1.14). As shown in the Appendix, this might be compensated with a higher power margin in case of additional fading and shadowing events, not visible in these simulations with constant received power.

Recalling the target PLR at 1%, the Config#2 CRDSA performance varies from 27 kbit/s (with the shortest TEDTtot) to 57 kbit/s (with the longest TEDTtot). Given the Config#2 SA results at PLR=1% is 1.4 kbit/s, it is straightforward to notify a huge capacity improvement compared to current SA baseline, varying from 1928% to 4071%.

Observation 2: The CRDSA solution is improving the Msg3-EDT throughput by about a factor from 20 to 40, depending on the selected configuration.

## 2.4 Final Performance Comparison

For completeness, Figure 8 and Figure 9 show the comparison among the current legacy SA method and the two proposed solutions for enhancing the NTN IoT uplink capacity. Again, the advantages of DSA and CRDSA are very well evident.

To support these technologies in NTN IoT Release 19 (both GEO and NGSO systems), the specifications impact might be summarized as:

- DSA: enable the support to have Msg#3 replicas (instead of supporting only Msg#3 repetitions);

- CRDSA: define the indexing strategy for retrieving the multiple replicas within the EDT occasions. For example, a simple scheme using a linear congruential generator (e.g., a hash function) to implement such a behavior is currently adopted by the DVB-RCS2 standard [6].

Proposal 1: RAN2 to specify the support of Msg#3-EDT replicas.

Proposal 2: RAN2 to specify the indexing strategy of the Msg#3-EDT replicas, when multiple copies are transmitted.

Figure 8: Config#1 final performance comparison.

Figure 9: Config#2 final performance comparison.

- Conclusion

Following observations and proposals are made:

Observation 1: The DSA solution is improving the Msg3-EDT throughput by about a factor of 4.5 without any impacts on the current receiver implementation at the network, and minimal specification impact overall.

Observation 2: Observation 2: The CRDSA solution is improving the Msg3-EDT throughput by about a factor from 20 to 40, depending on the selected configuration.

Proposal 1: RAN2 to specify the support of Msg#3-EDT replicas.

Proposal 2: RAN2 to specify the indexing strategy of the Msg#3-EDT replicas, when multiple copies are transmitted.

References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 2024.

[2] R2-2405202, “Discussion on EDT enhancements”, ESA, May 2024.

[3] E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random-Access Scheme for Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007.

[4] O. del Rio Herrero and R. De Gaudenzi, “Generalized Analytical Framework for the Performance Assessment of Slotted Random Access Protocols”, in IEEE Transactions on Wireless Communications, vol. 13, pp. 809-821, February 2014.

[5] A. Mengali, R. De Gaudenzi, and P. -D. Arapoglou, “Enhancing the Physical Layer of Contention Resolution Diversity Slotted ALOHA”, in IEEE Transactions on Communications, vol. 65, pp. 4295-4308, Oct. 2017.

[6] ETSI EN 301 545-2, “Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard”.

- Appendix

Based on the selection of SCS=3.75 kHz, a single resource unit (RU) is 32 ms, and it is composed of 112 QPSK symbols. Considering the presence of DM-RS, there are 96 data-symbols and 16 pilots for each RU. Therefore, the FEC coding rate for the proposed configurations is

- Config#1 (NRU=3): Total coded bits=96 × 3 × 2= 576. Then, the coding rate r: (328+24 CRC)/576= 0.61

- Config#2 (NRU=4): Total coded bits=96 × 4 × 2= 768. Then, the coding rate r: (328+24 CRC)/768= 0.46

Even though the turbo decoder performance is not relevant in the considered methodology for the capacity assessment (i.e., collision-based), we have reported below the physical layer BLER results. Both configurations achieve BLER<1% well before the selected UL SNR value of 4.8dB. As expected, the lower coding rate has an additional 2dB margin.

Figure 10: Physical layer turbo decoder performance for both configurations.

Please note that a single resource unit (RU) allocation is 32 ms in case of sub-carrier spacing equal to 3.75 kHz.