3GPP TSG-RAN WG2 Meeting #127                              R2-2407503

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.1.4

Source:	Samsung

Title:	Remaining FFSs on data collection options for UE-side model training

Document for:	Discussion and Decision

# Introduction

In RAN2#126 meeting, RAN2 discussed outcomes of two email discussions on the topic of data collection for UE-side model training:

- Report of [POST125bis][020][AI/ML PHY] UE side data collection [1] and

- Report of [AT126][020][AI/ML PHY] UE side data collections [2].

In conclusion, RAN2 agreed the following:

Agreements - Capture table below in TR (Rapporteur will clean up and refine and review over email) - Capture the privacy concerns from different stakeholders as informative annexes in the TR. All the options (1a, 1b, 2, 3) should respect those privacy concerns. Details are up to TR rapporteur.

| Aspects | 1a) OTT (3GPP Transparent) | 1b) The server for training data collection for UE-side models (3GPP non-transparent) | 2. Transfer via Core Network | 3. Transfer via OAM |
| --- | --- | --- | --- | --- |
| First termination entity | OTT server | The server for data collection for UE-side model training | Inside the CN (e.g., LMF) | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE-> CN ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE->gNB->OAM-> Server for data collection for UE-side model training/OTT server |
| UP/CP tunnel | UP tunnel | UP tunnel | CP tunnel (provided the data volume remains within the NAS signalling capacity) FFS: UP tunnel | CP tunnel (provided the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Has controllability FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Control granularity by NW | NA, the OTT server can directly request data from the UE. | Example: per PDU sessions | NAS procedure, FFS impact to other layers | RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility FFS: meaning of ‘partial/partially’ SAME for OPTION 3 | Full visibility for standardized data content. Partial visibility for partially standardized data content No standardized visibility FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | NA | SA2, SA3, RAN2 | SA2, SA3, RAN3 RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. (Subject to refinement and modification) - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. (Subject to refinement and modification, the scope does not exclude additional requisites, such as the ability to modify the collected data.) - Note 3: For Solution 1b, 2/3, the following options are identified to realize the different levels of data content visibility if different levels of data content visibility to MNO are considered. FFS on the data content visibility via SLA. - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No visibility for non-standardized data content. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. |  |  |  |  |

In this contribution, we discuss remaining FFSs and potential issues in Table 7.2.1.3.2-1 (Analysis of different data collection options for UE-side model training) in the report of [POST126][034][AIML PHY] TP for data collection [3].

# Discussion

## FFSs and issues in report of [POST126][034][AIML PHY] TP for data collection [3]:

The outcome of the email discussion “[POST126][034][AIML PHY] TP for data collection [3]” provided updates to TR 38.843 based on RAN2#126 agreements [4]. In this report, the Table 7.2.1.3.2-1 (Analysis of different data collection options for UE-side model training) includes several FFSs and issues that RAN2 needs to address. In the following we discuss some of those FFSs and issues:

Issue #1 (data transfer method based on data volume):

Table 7.2.1.3.2-1 (Analysis of different data collection options for UE-side model training)

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |

Regarding the two statements of “CP tunnel (provided case of the data volume remains within the NAS signalling capacity or RRC signalling capacity)”, it is not clear whether data transfer, in case of the data volume is larger than the NAS or RRC signalling capacity, is performed over:

- UP tunnel or LPP or

- CP tunnel using methods such as data prioritization or RRC message segmentation, etc.

In RAN2#121 meeting, RAN2 discussed pros and cons of transferring AI/ML models over CP (i.e. Solution 1a (via RRC) and Solution 2a (via NAS)) and concluded that transferring AI/ML models (or data) of size larger than 45kBytes may require using RRC segmentation to support data transfer. However, RRC segmentation may result in delay to data transfer over CP.

Observation 1: For Option 2/3, it is not clear whether AI/ML data is transferred over UP (or LPP) or CP, when the data volume does not remain within the NAS or RRC signalling capacity.

Observation 2: For Option 2/3, AI/ML data transfer over CP may require RRC segmentation when the data volume does not remain within the NAS or RRC signalling capacity.

Moreover, AI/ML data transfer over RRC signalling may cause delay to RRC messages carrying UE’s connection management and mobility data. One solution to address the delay problem, is to apply prioritization when sending data over CP (i.e. priority over RRC and/or NAS messages). For example, the priority level maybe based on configuration, data content, and/or data features (e.g. size, latency).

Observation 3: For Option 2/3, there is a need to apply priority to data sent over RRC or NAS signalling. The priority level for data content maybe based on configuration, data content and/or data features.

Proposal 1a: RAN2 agree that CP solution can support data prioritization when the data volume does not remain within the RRC or NAS signaling capacity.

Proposal 1b: RAN2 remove “FFS: UP tunnel” and support CP solution only.

Issue #2 (Level of controllability of data transfer):

Table 7.2.1.3.2-1 (Analysis of different data collection options for UE-side model training)

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - .... |  |  |  |  |

Option 1b):

For Option 1b) the UE may collect and transfer data directly (or via MNO’s network) to non-UE-vendor OTT server. The data is transferred via UP tunnel so MNO may have limited controllability on management of data transfer from the UE to the server for UE-side model data collection. For example, MNO may control the initiation or termination of data transfer process, without controlling the data itself. Nonetheless, in our understanding, the discussion on MNO’s controllability of data collection for Option 1b), is based on SLA which is out of RAN2 (or 3GPP) scope.

Observation 4: The discussion on MNO’s controllability of data collection and transfer may be achieved via SLA, which is out of 3GPP scope.

Proposal 2: For Option 1b), RAN2 to remove the “FFS: level of controllability”, as the discussion on level of controllability is out of 3GPP scope”.

Option 2/3:

In RAN2#126, RAN2 agreed that MNO should have “full controllability” on data transfer when using Option 2 or 3. The meaning of “full controllability” was explained in Note 1 in Table 7.2.1.3.2-1 [3]:

- Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed.

As indicated in the FFS, RAN2 may consider further refinement to Note 1 to clarify the above definition. For example, what is the meaning of fully managing data transfer? And whether it includes the following:

- Data modification or processing,

- Awareness of transferred data content, or

- A mix of standardized and/or non-standardized data format.

Proposal 3: RAN2 need to clarify in Note 1, the meaning of “fully managing data transfer”, and whether it includes:

- Data modification or processing,

- Awareness of data content, or

- A mix of standardized and/or non-standardized data format.

Issue #3 (MNO visibility to data content):

Table 7.2.1.3.2-1 (Analysis of different data collection options for UE-side model training)

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| - … - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - … |  |  |  |  |

MNO visibility to standardized data:

In RAN2#126, RAN2 discussed MNO’s data visibility for different UE-side data collection Options 1a), 1b), 2 and 3, and concluded that Option 1b) may offer MNO visibility to standardized data format, however, what is not clear is the level of data visibility to MNO. As for Option 2/3, RAN2 agreed that MNO may have three levels of data visibility to standardized data: full, partial or no visibility.

MNO visibility to non-standardized data:

On the other hand, for all Options 1a), 1b), 2 and 3, MNO should not have access to non-standardized data content, unless via SLA, which is out of 3GPP scope. The reason for restricting access to non-standardized data content is because this data could include information on UE proprietary implementation, i.e. vendor sensitive information used for model training (e.g., receive beam information, physical antenna information). So, if this data is delivered to a 3rd party (e.g., non-UE vendor), access to such data may expose this sensitive proprietary information. Moreover, the leakage of this data may result in serious threat to user privacy and security. Finally, there is also the data ownership issue that may arise when UE-side model data is disclosed to entities other than the UE-side vendor.

Proposal 4a: MNO has no visibility to non-standardized data content with Option 1a), 1b), 2, and 3 for UE-side data collection.

Proposal 4b: RAN2 to agree the following updates to Table 7.2.1.3.2-1 [3]:

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS: level of visibility for standardized data content. | Full visibility, Partial visibility , or No standardized visibility. | Full visibility, Partial visibility , or No standardized visibility. |
| - … - Note 2: Visibility of standardized data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of standardized data content visibility to the MNO: - Full visibility. - Partial visibility. - No visibility. - … - Note 5: No visibility to non-standardized data contents for all options in order to provide privacy and security protection. |  |  |  |  |

# Conclusion

In this contribution, we discussed remaining FFSs and issues in Table 7.2.1.3.2-1 (Analysis of different data collection options for UE-side model training) [3]. The following are observations and proposals:

Observation 1: For Option 2/3, it is not clear whether AI/ML data is transferred over UP (or LPP) or CP, when the data volume does not remain within the NAS or RRC signalling capacity.

Observation 2: For Option 2/3, AI/ML data transfer over CP may require RRC segmentation when the data volume does not remain within the NAS or RRC signalling capacity.

Observation 3: For Option 2/3, there is a need to apply priority to data sent over RRC or NAS signalling. The priority level for data content maybe based on configuration, data content and/or data features.

Observation 4: The discussion on MNO’s controllability of data collection and transfer may be achieved via SLA, which is out of 3GPP scope.

Proposal 1a: RAN2 agree that CP solution can support data prioritization when the data volume does not remain within the RRC or NAS signaling capacity.

Proposal 1b: RAN2 remove “FFS: UP tunnel” and support CP solution only.

Proposal 2: For Option 1b), RAN2 to remove the “FFS: level of controllability”, as the discussion on level of controllability is out of 3GPP scope”.

- Proposal 3: RAN2 need to clarify in Note 1, the meaning of “fully managing data transfer”, and whether it includes:

- Data modification or processing,

- Awareness of data content, or

- A mix of standardized and/or non-standardized data format.

Proposal 4a: MNO has no visibility to non-standardized data content with Option 1a), 1b), 2, and 3 for UE-side data collection.

Proposal 4b: RAN2 to agree the following updates to Table 7.2.1.3.2-1 [3]:

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS: level of visibility for standardized data content. | Full visibility, Partial visibility , or No standardized visibility. | Full visibility, Partial visibility , or No standardized visibility. |
| - … - Note 2: Visibility of standardized data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of standardized data content visibility to the MNO: - Full visibility. - Partial visibility. - No visibility. - … - Note 5: No visibility to non-standardized data contents for all options in order to provide privacy and security protection. |  |  |  |  |

# References

- R2-2405931 Report of [POST125bis][020][AI/ML PHY] UE side data collection (MediaTek)

- R2-2406000 Report of [AT126][020][AI/ML PHY] UE side data collections (MediaTek)

- R2-2407209 [POST126][034][AIML PHY] TP for data collection (Ericsson)

- RAN2#126 ChairNotes.

# Annex (RAN2 inputs to TR 38.843) [3]

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |