3GPP TSG-RAN WG2 Meeting #127	R2-2407508

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.2.3

Source:		Futurewei

Title:		Discussions on AIoT paging

Document for:		Discussion and Decision

# Introduction

In meeting #126, RAN2 agreed on when an Ambient IoT (AIoT) paging is sent, as highlighted below [1]:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

We can see that, for “inventory-only” and “inventory and command” use cases (FFS “command-only” use case), the AIoT paging message sent in Step A may trigger a random access (RA) procedure or a non-RA procedure in Step B.

RAN2 also agreed on the following with respect to the content in the AIoT paging message for various scenarios [1]:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

In this contribution, we provide our views on the paging content required for the various scenarios and procedures for the AIoT device to determine D2R access occasion.

# Discussions

## On the types of AIoT paging message

For the first scenario above, where the AIoT paging message contains an ID of a single AIoT device, the intended device doesn’t need to contend for the D2R access occasion as the designated D2R access occasion should be exclusively reserved for the intended device. Therefore, in this scenario, Step B should use a non-RA procedure to avoid unnecessary computation (which would further consume energy) at the devices.

Proposal 1. When the AIoT paging message sent in Step A contains an ID of a single AIoT device, it triggers a non-random-access procedure in Step B, where the intended device performs D2R transmission at the designated D2R access occasion without contention.

For the third scenarios above, where the AIoT paging message does not contain an ID (i.e., addressed for all devices that can receive the AIoT message), the total number of AIoT devices that may respond to the paging message and the identities of them are likely unknown to the reader. Therefore, the best strategy in this scenario is for the reader to announce a random access procedure in the AIoT paging message, where a specific number of D2R access occasions are scheduled by the reader and the AIoT devices determine whether and in which D2R access occasion they should respond in accordance with a contention-based algorithm.

Proposal 2. When the AIoT paging message sent in Step A does not contain an ID (i.e., addressed for all devices that can receive the AIoT message), it triggers a random access procedure in Step B, where the AIoT devices contend for a specific number of reader-scheduled D2R access occasions based on contention.

For the second scenario above, where the AIoT paging message contains a group ID that maps to multiple AIoT devices, and the fourth scenario above, where the AIoT paging message contains multiple IDs of AIoT devices, the reader may know the total number and the identities of all the intended AIoT devices. Therefore, in this situation, Step B may be carried out using a non-RA procedure, where the number of D2R access occasions scheduled by the reader (e.g., as announced in the AIoT paging message) is equal to the number of intended AIoT devices and each intended device determines in which D2R access occasion its should send its response based on explicit indications or specific mapping rules.

On the other hand, it is also possible that not all the intended devices in these two scenarios are still within the coverage of the reader or have sufficient energy to respond to the AIoT paging message. In this situation, some of the scheduled D2R access occasions may be wasted, reducing system capacity. In addition, for the second scenario above, where the AIoT paging message contains a group ID that maps to multiple AIoT devices, it is also possible that the number of AIoT devices in the group is unknown to the reader. For example, the device grouping may be managed at the NAS or application level and only the group ID is informed to the reader. Therefore, in all these situations, it may be more efficient to trigger a random access procedure in Step B, where a specific number of D2R access occasions are scheduled (and announced in the AIoT paging message) by the reader and the intended AIoT devices determine whether and in which D2R access occasion they should send a respective response in accordance with a contention-based algorithm, similar to the devices in the third scenario above.

Therefore, it is recommended that, for the second and the fourth scenarios above, RAN2 consider both random access and non-RA in Step B.

Proposal 3. For an AIoT paging message sent in Step A containing a group ID that maps to multiple AIoT devices or containing multiple IDs of AIoT devices, RAN2 consider both random access and non-random-access in Step B.

Therefore, there are two types of AIoT paging message: one that triggers a random access procedure and one that triggers a non-RA procedure. And this is true for both the “inventory-only” and “inventory and command” use cases because the “command” in the latter case is carried only in Step C and hence has no impact to Step A nor Step B.

## On the paging content

For an AIoT paging message that triggers a random access procedure, all intended AIoT devices contend for a scheduled D2R access occasion to send a response based on a respective random number (RN) that the devices generated individually. All contending devices need to know a common range of numbers from which each device randomly draws a number. A range too wide may result in too many empty D2R access occasions where no device makes an access attempt, while a range too narrow may result in too many collisions where two or more contending devices have picked a same RN and hence responded in a same D2R access occasion. Either case would result in poor channel utilization. Therefore, the reader should determine, e.g., based on the number (or estimated number) of the intended devices, a proper range for the RNs, balancing the need for minimizing the number of empty access occasions and the need for minimizing the number of collisions. In the AIoT paging message that triggers a random access procedure in Step B, the reader should provide information about the range of the RNs, either explicitly (e.g., by indicating the bit size of the RN to be drawn or the maximal RN value) or implicitly (e.g., implied from the total number of D2R access occasions announced in the AIoT paging message).

Proposal 4. The AIoT paging message that triggers a random access procedure in Step B should include information about the range of the random numbers from which the intended devices are to draw a number randomly.

For an AIoT paging message that triggers a non-RA procedure, the AIoT paging message should not include the information about the range of the RNs to avoid unnecessary signaling overhead and to avoid causing unnecessary computation (such as generating an RN) at the devices.

Proposal 5. The AIoT paging message that triggers a non-RA procedure in Step B should not include information about the range of the random numbers.

For the first and the third scenarios above, the devices are able to determine whether the AIoT paging message triggers a random access procedure or a non-RA procedure in Step B. However, for the second and the fourth scenarios above, if both random access and non-RA are possible, the AIoT paging message may need to include information indicating whether a random access procedure or a non-RA procedure is triggered in Step B. For example, this information may be the same information that indicates the presence of the information about the range of the RNs. If it is indicated that the information about the range of the RNs is included in the AIoT paging message, a random access procedure is triggered in Step B; otherwise, a non-RA procedure is triggered in Step B. Or vice versa, if it is indicated that a random access is triggered in Step B, the information about the range of the RNs is included in the AIoT paging message; otherwise, the information about the range of the RNs is not included in the AIoT paging message.

Proposal 6. RAN2 consider that the AIoT paging message include information indicating whether the AIoT paging message triggers a random access procedure or a non-RA procedure in Step B. This information can be the same information that indicates the presence of the information about the range of the random numbers from which the intended devices are to draw a number randomly.

## On the procedures for the device to determine D2R access occasion

For an AIoT paging message that triggers a non-RA procedure, if the AIoT paging message is intended for a single device, the intended device sends its response in the D2R access occasion immediately (subject to the TR2D_min and TR2D_max currently being defined by RAN1 and to be specified by RAN4) after the end of the R2D transmission that carries the AIoT paging message.

Proposal 7. For an AIoT paging message that contains an ID of a single AIoT device, the intended device sends its response in the D2R access occasion immediately (subject to the TR2D_min and TR2D_max requirements) after the end of the R2D transmission that carries the AIoT paging message.

For an AIoT paging message that triggers a non-RA procedure, if the AIoT paging message is intended for multiple devices, the reader should schedule multiple D2R access occasions, one unique D2R access occasion for each intended device. Information about the radio resources in time, and if FDM is used in D2R, in frequency for these D2R access occasions should be included in the AIoT paging message. The ranks among the scheduled D2R access occasions should be derivable by the devices (e.g., by the positions of these radio resources in time and frequency through standardized algorithm) or should be explicitly indicated in the AIoT paging message. The ranks among the intended devices should be derivable by the devices (e.g., by the positions of their corresponding IDs in the AIoT paging message or by a position of a device in the group, which is informed to the device during the group assignment) or should be explicitly indicated in the AIoT paging message. Then, the intended devices determine their respective D2R access occasions based on a one-to-one mapping between the ranks among the scheduled D2R access occasions and the ranks among the intended devices.

Proposal 8. For an AIoT paging message that triggers a non-RA procedure, if the AIoT paging message is intended for multiple devices, the reader should schedule multiple D2R access occasions, one unique D2R access occasion for each intended device. The intended devices determine their respective D2R access occasions based on a one-to-one mapping between the ranks among the scheduled D2R access occasions and the ranks among the intended devices.

For an AIoT paging message that triggers a random access procedure, the reader should also schedule multiple D2R access occasions. An intended device generates an RN within the range of the RNs as indicated in the AIoT paging message, and then uses the RN to determine whether and in which D2R access occasion it can send its response. In RFID, the tag (device) generates an RN to initialize a slot counter at the beginning of a random access procedure, then counts down its slot counter whenever receiving a Query command from the reader, and when its slot counter counts down to zero, the device responds in the next access occasion. We refer this approach as the slot-counting approach. In our paper [2] submitted to meeting #126, we have described a random-access mechanism that is based on instantaneous number-polling by the reader and number-comparison between the polled number(s) and the RN generated by the device for the device to determine whether it should send a response or not. We refer this approach as the number-polling approach. We recommend that RAN2 study both approaches for the random access procedure.

Proposal 9. For an AIoT paging message that triggers a random access procedure, the reader should schedule multiple D2R access occasions. An intended device generates an RN within the range of the RNs as indicated in the AIoT paging message, and then uses the RN to determine whether and in which D2R access occasion it can send its response.

Proposal 10. RAN2 can study both slot-counting and number-polling approaches for the random access procedure.

# Conclusions

We propose the following:

Proposal 1. When the AIoT paging message sent in Step A contains an ID of a single AIoT device, it triggers a non-random-access procedure in Step B, where the intended device performs D2R transmission at the designated D2R access occasion without contention.

Proposal 2. When the AIoT paging message sent in Step A does not contain an ID (i.e., addressed for all devices that can receive the AIoT message), it triggers a random access procedure in Step B, where the AIoT devices contend for a specific number of reader-scheduled D2R access occasions based on contention.

Proposal 3. For an AIoT paging message sent in Step A containing a group ID that maps to multiple AIoT devices or containing multiple IDs of AIoT devices, RAN2 consider both random access and non-random-access in Step B.

Proposal 4. The AIoT paging message that triggers a random access procedure in Step B should include information about the range of the random numbers from which the intended devices are to draw a number randomly.

Proposal 5. The AIoT paging message that triggers a non-RA procedure in Step B should not include information about the range of the random numbers.

Proposal 6. RAN2 consider that the AIoT paging message include information indicating whether the AIoT paging message triggers a random access procedure or a non-RA procedure in Step B. This information can be the same information that indicates the presence of the information about the range of the random numbers from which the intended devices are to draw a number randomly.

Proposal 7. For an AIoT paging message that contains an ID of a single AIoT device, the intended device sends its response in the D2R access occasion immediately (subject to the TR2D_min and TR2D_max requirements) after the end of the R2D transmission that carries the AIoT paging message.

Proposal 8. For an AIoT paging message that triggers a non-RA procedure, if the AIoT paging message is intended for multiple devices, the reader should schedule multiple D2R access occasions, one unique D2R access occasion for each intended device. The intended devices determine their respective D2R access occasions based on a one-to-one mapping between the ranks among the scheduled D2R access occasions and the ranks among the intended devices.

Proposal 9. For an AIoT paging message that triggers a random access procedure, the reader should schedule multiple D2R access occasions. An intended device generates an RN within the range of the RNs as indicated in the AIoT paging message, and then uses the RN to determine whether and in which D2R access occasion it can send its response.

Proposal 10. RAN2 can study both slot-counting and number-polling approaches for the random access procedure.

# References

R2_126_ChairNotes_24-05-24_13-10_eom.

R2-2404384, Discussion on AIoT random access, Futurewei.