3GPP TSG-RAN WG2 Meeting #127	R2-2407511

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.7.5

Source:		Futurewei

Title:		Discussions on RLC enhancements for Rel-19 XR

Document for:		Discussion and Decision

# Introduction

In meeting #126, RAN2 made the following progress with respect to avoiding unnecessary RLC AM retransmissions for Rel-19 XR [1]:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

- Rx initiated approach

- Tx initiated approach

RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

For Tx initiated approach:

    - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs

    - Tx side stops retransmit obsolete SDUs

    - Rx side updates state variables according to the information from Tx side

For Rx initiated approach:

    - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

    - Tx side just processes the status report as in legacy

    - FFS how Rx side determines that an SDU should be abandoned

In this contribution, we provide our views on how a much simpler solution can be used for avoiding unnecessary RLC AM retransmissions.

# Discussions

## On how to avoid unnecessary retransmissions

In 5G NR, the RLC layer no longer performs the function of reordering of RLC data PDUs, i.e., the RLC SDUs successfully received and reassembled will be delivered to the upper (i.e., PDCP) layer as soon as they are received and reassembled. Although discarding an obsolete RLC SDU, which is still pending a retransmission or pending a transmission of a remaining segment of, at the transmitting side of an AM RLC entity still creates an RLC SN gap at the receiving side of the peer RLC entity, that gap doesn’t prevent the peer RLC entity from delivering RLC SDUs that are subsequently received and successfully reassembled to the upper layer, as long as the receiving window is not stalled (i.e., no subsequently received RLC data PDUs are outside the receiving window). Therefore, the only issues for discarding an obsolete RLC SDU, for which an RLC transmission has begun, at the transmitting side are how to prevent the transmitting window from being stalled and how to prevent the RLC SN gap created at the receiving side of the peer RLC entity (due to the discarding at the transmitting side) from stalling the receiving window.

Observation 1. The RLC SN gap created at the receiving side of the peer RLC entity (due to the discarding at the transmitting side) doesn’t prevent the peer RLC entity from delivering RLC SDUs that are subsequently received and successfully reassembled to the upper layer, as long as the receiving window is not stalled.

Observation 2. The only issues for discarding an obsolete RLC SDU, for which an RLC transmission has begun, at the transmitting side is how to prevent the transmitting window from being stalled and how to prevent the RLC SN gap created at the receiving side of the peer RLC entity (due to the discarding at the transmitting side) from stalling the receiving window.

### How to prevent the transmitting window from being stalled

The transmitting window of an AM RLC entity is defined as the RLC SN range of [TX_Next_Ack, TX_Next_Ack + AM_Window_Size). The transmitting window will be stalled when an RLC SDU with an RLC SN equal to TX_Next_Ack has not been positively acknowledged in an RLC Status Report yet and the next to-be-transmitted RLC SDU has an RLC SDU equal to (TX_Next_Ack + AM_Window_Size), after considering the possibility of SN wrap-around.

Since the decision to discard an obsolete RLC SDU (and associated RLC data PDU(s)), for which an RLC transmission has begun, is made by the transmitting side, the transmitting side knows that it cannot rely on an ACK indication for this RLC SDU in the RLC Status Report to push its transmitting window forward. Instead, the transmitting side of the AM RLC entity should compare the RLC SN of the discarded RLC SDU, for which an RLC transmission has begun, with TX_Next_Ack, and if the RLC SN is greater than TX_Next_Ack after considering the possibility of SN wrap-around, set TX_Next_Ack to be equal to the RLC SN of the discarded RLC SDU. In this way, the transmitting window will not be stalled by the discarding of the obsolete RLC SDU, for which an RLC transmission has begun.

Proposal 1. When discarding an obsolete RLC SDU, for which an RLC transmission has begun, the transmitting side of the AM RLC entity should compare the RLC SN of the discarded RLC SDU with TX_Next_Ack, and if the RLC SN is greater than TX_Next_Ack after considering the possibility of SN wrap-around, set TX_Next_Ack to be equal to the RLC SN.

### How to prevent the RLC SN gap created at the receiving side of the peer RLC entity from stalling the receiving window

The receiving window of the peer AM RLC entity is defined as the RLC SN range of [RX_Next, RX_Next + AM_Window_Size). An RLC data PDU received with an RLC SN greater than or equal to (RX_Next + AM_Window_Size), after considering the possibility of SN wrap-around, will not be processed due to outside the receiving window and hence lost. We refer this as the receiving window stalling. The AM_Window_Size is the size of half of the RLC SN space. Therefore, the AM_Window_Size is 2048 for 12-bit RLC SN and 131072 for 18-bit RLC SN.

In 3GPP TR 26.928 [2], 150 Mbps has been cited as the highest video data rate (for AVC with 8k resolution), which can be translated into about 13107 packets (with a packet size of 1500 Bytes) per second. Therefore, 2048 SNs can last for at least 156 msec before the receiving window stalling may occur while the PDB is 10 msec for DL XR video (since high video resolutions, such as 8k, are typically for DL-only). As the UL XR video resolution is not as high as for the DL, 2048 SNs can last much longer. Therefore, the receiving side of the peer AM RLC entity should be able to determine that a missing RLC SDU has passed its PDB long before the missing RLC SDU can stall its receiving window. There may be many ways to do so by implementation. For example, when the receiving side of the peer AM RLC entity receives an RLC data PDU with an RLC SN of X after an RLC SN gap, it may start a timer with an initial value larger than or at least equal to the PDB. When the timer expires, the receiving side determines that any RLC SDUs with an RLC SN before X (considering the possibility of SN wrap-round) are obsolete and then updates RX_Next to the value of the SN following the last in-sequence completely received RLC SDU, considering those obsolete RLC SNs as if they are completely received. In this way, the receiving window will be pushed forward long before a missing RLC SDU can cause the loss of a subsequently received RLC data PDU.

Proposal 2. The receiving side of the peer AM RLC entity should be able to determine that a missing RLC SDU has passed its PDB long before the missing RLC SDU can stall its receiving window. Implementation should ensure that the receiving window is not stalled by the missing RLC SDUs that have become obsolete (e.g., having passed the PDB).

## Drawbacks in the Tx initiated approach and the Rx initiated approach

The Tx initiated approach requires the transmitting side of the AM RLC entity to send over-the-air signaling to “notify the receiving RLC side about the obsolete SDUs”, while our analysis in section 2.1.2 above shows that the receiving side of the peer AM RLC entity can determine that on its own, and based thereon, taking necessary actions (such as updating RX_Next) long before the missing RLC SDUs can stall its receiving window.

The Rx initiated approach requires the receiving side of the peer AM RLC entity to “determine that an SDU should be abandoned” and to “indicate abandoned SDUs to the transmitter”, while our analysis in section 2.1.1 above shows that the transmitting side of the AM RLC entity is in a much better position to determine that, and based thereon, taking necessary actions (such as updating TX_Next_Ack) to prevent its transmitting window from being stalled by the discarded and obsolete RLC SDU, for which a transmission has begun.

# Conclusions

Observation 1. The RLC SN gap created at the receiving side of the peer RLC entity (due to the discarding at the transmitting side) doesn’t prevent the peer RLC entity from delivering RLC SDUs that are subsequently received and successfully reassembled to the upper layer, as long as the receiving window is not stalled.

Observation 2. The only issues for discarding an obsolete RLC SDU, for which an RLC transmission has begun, at the transmitting side is how to prevent the transmitting window from being stalled and how to prevent the RLC SN gap created at the receiving side of the peer RLC entity (due to the discarding at the transmitting side) from stalling the receiving window.

Based on these observations, we propose the following:

Proposal 1. When discarding an obsolete RLC SDU, for which an RLC transmission has begun, the transmitting side of the AM RLC entity should compare the RLC SN of the discarded RLC SDU with TX_Next_Ack, and if the RLC SN is greater than TX_Next_Ack after considering the possibility of SN wrap-around, set TX_Next_Ack to be equal to the RLC SN.

Proposal 2. The receiving side of the peer AM RLC entity should be able to determine that a missing RLC SDU has passed its PDB long before the missing RLC SDU can stall its receiving window. Implementation should ensure that the receiving window is not stalled by the missing RLC SDUs that have become obsolete (e.g., having passed the PDB).

# References

R2-2405706, Report from session on R18 MBS, R18 QoE and R19 XR, Session Chair (Huawei).

3GPP TR 26.928 V17.0.0.