3GPP TSG-RAN WG2 Meeting #127	R2-2407515

Masstricht, Netherlands, Aug. 19th – 23rd, 2024

Agenda item:		7.17.3

Source:		Samsung

Title:			Remaing issues on MUSIM

Document for:	Discussion & Decision

# 1 Introduction

This contribution discusses leftover issues on MUSIM.

# 2 Discussion

Upon T348 expiry the UE can apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation including musim-CapRestriction. But it is observed that companies' views are split on how the UE handles dedicated configuration associated with musim-CapRestriction. Some companies think that the UE autonomously releases dedicated configuration associated with musim-CapRestrpction. Other think that the UE still keeps its dedicated configuration but does not perform any required behavior/activity accordinly. Thus RAN2 could not reach consensus on any understanding in RAN2#126 meeting as shown below.

R2-2405642 Discussion on UE behavior upon T348 stop and T348 expiry Samsung Electronics Czech discussion Rel-18 NR_DualTxRx_MUSIM-Core Proposal #1: RAN2 to confirm which understanding is correct upon T348 expriy: - Understanding #1: UE autonomously releases dedicated configuration associated with musim-CapRestriction. - Understanding #2: UE still considers the latest configuration as the current configuration but does not perform any required behaviors about dedicated configuration associated with musim-CapRestriction. Discussions: - QC, Intel, Huawei think #2 is correct. - Samsung want to check the intended UE behaviour for the case of PScell release, and think this in this case UE can just release the SCG configuration. ZTE think this create issue if later UE receive more configurations for SCG. - Intel think SCG release is a corner case and do not require a normative behaviour. Nokia, CATT agree. - Samsung think we can confirm #2 and we need to remove ‘releases SCG’ from the informative table. ?? Understanding #2 is confirmed. ?? ‘UE may apply the temporary capability restriction that SCG is not supported if ServCellIndex of PSCell was included in indicated MUSIM-CellToRelease-r18, UE releases SCG.’ Is removed from the informative table. ?? No other changes required to RRC spec. Chair: can discuss in later stage, with the understanding that this is mainly about improving the informative table and it does not impact the RRC procedure text.

One issue was how the UE reacts if ServCellIndex of PSCell was included in MUSIM-CellToRelease-r18 upon T348 expiry. But it was concluded that the UE considers that SCG is not supported, which implies that the UE does not autonomously release dedicated configuration associated with musim-CapRestriction even for this case. To avoid any further confusion in the implementation, we proposed to capture the following understanding in the minutes.

Proposal 1: Capture the following UE behavior in the minutes:

-	Upon T348 expiry, the UE still considers the latest configuration as the current configuration but does not perform any required behaviors about dedicated configuration associated with musim-CapRestriction.

In the latest TS 38.331 [1] the following procedure text are specified.

1> if transmission of the UEAssistanceInformation message is initiated to provide musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musimGap-KeepPreference, or provide MUSIM assistance information for leaving RRC_CONNECTED according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

4>	if UE has a preference for MUSIM gap priority;

5>	include the musim-GapPriorityPreferenceList the UE prefers to be configured;

5>	if the UE has preference to keep all colliding MUSIM gaps for periodic MUSIM gap(s):

6>	include the musim-GapKeepPreference;

It is quite misleading whether UE can include the musim-GapKeepPreference without musim-GapPriorityPreferenceList. The first bullet 1> mentions that the UE can trigger the UAI procedure to provide musim-GapKeepPreference only due to 'and/or'. However, the following bullets 4> and 5> state that the UE includes the musim-GapKeepPreference only if the  musim-GapPriorityPreferenceList is included. In our understanding, RAN2 didn't make any concrete conclusion, which needs to be discussed.

Proposal 2: RAN2 to discuss whether the UE can include the musim-GapKeepPreference without the musim-GapPriorityPreferenceList in the UEAssistanceInformation message.

During handover, source gNB may send the measurement gap requirements i.e. the IE NeedForGapsInfoNR-r16 to the target gNB. If the last reported measurement gap requirements is done via the musim-needForGapsInfoNR in UAI, the same is included in the AS-Context. In [1] source gNB also can include the fields needForGapNCSG-InfoNR and needForInterruptionInfoNR in the AS-Context. So if the source gNB includes the musim-needForGapsInfoNR received in the UAI along with the NeedForInterruptionsInfoNR received in RRCReconfigurationComplete/RRCResumeComplete, the target gNB may wrongly understand the interruption requirements. For e.g. the UE would have reported the band X as first entry in NeedForGapsInfoNR in the RRCReconfigurationComplete. Now if the UE includes the interruption requirements in the same message, the first entry in the NeedForInterruptionsInfoNR maps to band X. Let us consider that UE reports musim-needForGapsInfoNR with the first entry as band Y. If the source gNB includes musim-needForGapsInfoNR and NeedForInterruptionsInfoNR in the HandoverPreparationInformation, target gNB will interpret, the first entry in the interruption requirements belong to band Y.

We also note that the interruption requirements would have changed due to MUSIM operations and the UE will not report them since NeedForInterruptionsInfoNR is not included in the UAI. Thus the NeedForInterruptionsInfoNR received in a previous RRCReconfigurationComplete/RRCResumeComplete doesn’t report the actual interruption requirements in the UE at the time of sending HandoverPreparationInformation. So it shouldn’t be included in the AS-Context.

Similarly, for NeedForNCSG, the needForGapNCSG-InfoNR  received in RRCReconfigurationComplete/RRCResumeComplete may not represent the actual gap and NCSG requirements when the last received gap requirements are in musim-NeedForGapsInfoNR.

Proposal 3: If the measurement gap requirements received in the musim-needForGapsInfoNR is included in the AS-Context, source gNB does not include the needForInterruptionInfoNR and needForGapNCSG-InfoNR (as in TP).

# 3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: Capture the following UE behavior in the minutes:

-	Upon T348 expiry, the UE still considers the latest configuration as the current configuration but does not perform any required behaviors about dedicated configuration associated with musim-CapRestriction.

Proposal 2: RAN2 to discuss whether the UE can include the musim-GapKeepPreference without the musim-GapPriorityPreferenceList in the UEAssistanceInformation message.

Proposal 3: If the measurement gap requirements received in the musim-needForGapsInfoNR is included in the AS-Context, source gNB does not include the needForInterruptionInfoNR and needForGapNCSG-InfoNR (as in TP).

# 4	Reference

[1] 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification

# 5	Annexure

## TP for Proposal 3

### 11.2.2	Message definitions

<non relevant section is not provided>

AS-Context field descriptions

configRestrictInfoDAPS Includes fields for which source cell explicitly indicates the restriction to be observed by target cell during DAPS handover.

mbsInterestIndication Includes the information last reported by the UE in the NR MBSInterestIndication message, where the plmn-Index (if included by the UE in tmgi) is replaced by the PLMN ID, if needed. A TMGI for which the plmn-Index points to a non-serving SNPN is removed from the NR MBSInterestIndication message.

needForGapsInfoNR Includes measurement gap requirement information of the UE for NR target bands. The field includes needForGapsInfoNR in RRCReconfigurationComplete message, needForGapsInfoNR in RRCResumeComplete message or musim-needForGapsInfoNR in UEAssistanceInformation message that is last reported by the UE, if any. The source gNB does not include needForGapNCSG-InfoNR and needForInterruptionInfoNR if this field includes musim-needForGapsInfoNR.