3GPP TSG-RAN WG2 Meeting #127	R2-2407516

Maastricht, NL, Aug 19-23, 2024

Source:	III

Title:	Primary use cases for multi-modality support in RAN

Agenda item:	8.7.2

Document for:	Discussion

1. Introduction

During the RAN2#126 meeting [1], the following LS was noted:

In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN).

To address issues related to multi-modal service synchronization requirements, this contribution discusses the cases of synchronization requirements and analyzed the corresponding existing solutions depicted in TR 22.847. Furthermore, based on our analysis, primary use cases and general solutions are proposed to focus RAN2 efforts in the right place.

2. Discussion

2.1 Multi-modal service synchronization requirements

According to [PR 5.1.6-2] of TR 22.847 [2], the synchronization requirements are depicted below:

[PR 5.1.6-2] Due to the separate handling of the multiple media components, synchronization between different media components is critical in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization). Applying synchronization thresholds in the 5G system may be helpful in support of immersive multi-modal VR applications when the synchronization threshold between two or more modalities is less than the latency KPI for the application. Table 5.1.6-2: Typical synchronization thresholds for immersive multi-modal VR applications synchronization threshold (note 1). audio-tactile; audio delay: 50 ms; tactile delay: 25 ms. visual-tactile; visual delay: 15 ms; tactile delay: 50 ms. NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other..

The relationship between latency KPIs and synchronization threshold for DL use cases of the multi-modal service in TR 22.847 [2] was summarized in Table 2.1.

Table 2.1 relationship between latency KPIs and synchronization threshold for DL use case of the multi-modal service

| Notation: “D”: Max allowed end-to-end latency of the QoS flow. “Y”: The Max allowed end-to-end latency of the QoS flow may exceed synchronization threshold. “N”: The Max allowed end-to-end latency of the QoS flow never exceed synchronization threshold. “N/A”: In TR 22.847 were not mentioned the need of synchronization threshold/requirement of this use case, or latency KPIs of this use case were not provided. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | audio-video NOTE 1, NOTE 2 |  | audio-tactile NOTE 1, NOTE 3 |  | visual-tactile NOTE 1, NOTE 3 |  |
| Multi-modal service | Audio delay | Video delay | Audio delay | Tactile delay | Visual delay | Tactile delay |
| 5.1 | N | N | N | N | N | N |
| 5.2 NOTE 4 | N | N | N | Y if D > 25ms for haptic feedback | N | Y if D > 50ms for haptic feedback |
| 5.3 | N | N | N | N | Y set D > 15 for video | N |
| 5.4 NOTE 5 | N/A | N/A | N/A | N/A | N/A | N/A |
| 5.5 | Y if D > 125ms for audio | Y if D > 45ms for video | Y if D > 50ms for audio | Y D = 50ms > 25ms for haptic feedback | Y if D > 15ms for video | N |
| 5.6 NOTE 5 | N/A | N/A | N/A | N/A | N/A | N/A |
| 5.7 NOTE 6 | Y if D > 125ms for audio | Y if D > 45ms for video | Y if D > 50ms for audio | Y if D > 25ms for haptic feedback | Y if D > 15ms for video | N |
| 5.8 NOTE 7 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. NOTE 2: Audio-video synchronization threshold is specified in clause 7.6 of TS 22.261 NOTE 3: Audio-tactile/visual-tactile synchronization threshold is from Table 6.43.1-1 of TS 22.261. NOTE 4: Assume that synchronization thresholds are applicable to this use case, because clause 5.2.2 and clause 5.2.3 of TR 22.847 mention the term “synchronous”. NOTE 5: Synchronization requirement were not proposed in this use case. See TR 22.847. NOTE 6: Synchronization requirement were proposed in [PR 5.7.6-3] of TR 22.847, but synchronization thresholds are not provided. NOTE 7: Latency KPIs were not mentioned. See clause 5.8 of TR 22.847. NOTE 8: For max allowed end-to-end latency of a certain modality, it can be set within an acceptable range. For example, max allowed end-to-end latency of DL haptic feedback of multi-modal service 5.3 is 10~20ms. |  |  |  |  |  |  |

Proposal 1: At least use cases 5.5 and 5.7 in TS 22.847 shall be the primary use cases for RAN2 to study the relationship between synchronization threshold and latency KPI of the multi-modal service.

The relationship between synchronization thresholds and latency KPI of the multi-modal service can be classified into following cases with the notation below:

| Notation | Description |
| --- | --- |
| Dmax(i) | Max allowed end-to-end delay of QoS flow i, where Dmax(i) > 0. |
| ΔDmax(i) | Decrement of max allowed end-to-end delay of QoS flow i, where ΔDmax(i) >= 0. See definition (2.1), (2.2), and (2.3) |
| D(i) | End-to-end delay of QoS flow i, where D(i) >= 0. |
| ΔD(i) | Increment/decrement of end-to-end delay of QoS flow i, where ΔD(i) >= 0. See definition (4.1), (4.2), and (4.3) |
| D(i, j) | Synchronization threshold for QoS flow-i is delayed compared to QoS flow-j, where D(i, j) >= 0 |
| The detailed definition of above notation can be found in Annex 5.1. In this contribution, only end-to-end QoS flow level synchronization was discussed. |  |

Given D(i, j), D(j, i), Dmax(i), Dmax(j), all the cases illustrating the relationship between the synchronization threshold and the latency KPI of the multi-modal service are listed in Table 2.1-1.

Table 2.1-1 Multi-modal Service Synchronization Requirement Case

| Case 1: D(i, j) | Case 1.1: D(i, j) >= Dmax(i) |
| --- | --- |
|  | Case 1.2: Dmax(i) - Dmax(j) <= D(i, j) < Dmax(i) |
|  | Case 1.3: D(i, j) < Dmax(i) - Dmax(j) |
| Case 2: D(j, i) | Case 2.1: D(j, i) >= Dmax(j) |
|  | Case 2.2: D(j, i) < Dmax(j) |
| NOTE1: When we mention the term 'Case 1', it includes Case 1.1, Case 1.2, and Case 1.3. Similarly, the term 'Case 2' includes Case 2.1 and Case 2.2. NOTE2: For a multi-modal service with both D(i, j) and D(j, i), it belongs to a sub-case of Case 1 ( e.g. Case 1.1 ) and belongs to a sub-case of Case 2 ( e.g. Case 2.1 ) NOTE3: Without loss of generality, we assume that Dmax(i) >= Dmax(j). |  |

Observation 1: The use cases of synchronization requirements can be classified into two cases based on the relationship between the synchronization threshold and the end-to-end latency.

2.2 Analysis on general solutions to the cases of synchronization requirements

This section analyses general solutions for the cases in Table 2.1-1 when QoS flow-i and QoS flow-j are not synchronized. i.e.

D(i) - D(j) > D(i, j) for Case 1

or

D(j) - D(i) > D(j, i) for Case 2

Four solutions are concluded and listed in Table 2.2-1 below.

Table 2.2-1 General solutions to the cases in Table 2.1-1

|  | Description |  |
| --- | --- | --- |
| Solution 1 | Increase either end-to-end delay of QoS flow-i or end-to-end delay of QoS flow-j |  |
|  | For Case 1 | Increase D(j) by ΔD(j) to meet synchronization threshold D(i, j) |
|  | For Case 2 | Increase D(i) by ΔD(i) to meet synchronization threshold D(j, i) |
| Solution 2 | Decrease either end-to-end delay of QoS flow-i or end-to-end delay of QoS flow-j |  |
|  | For Case 1 | Decrease D(i) by ΔD(i) to meet synchronization threshold D(i, j) |
|  | For Case 2 | Decrease D(j) by ΔD(j) to meet synchronization threshold D(j, i) |
| Solution 3 | Increase end-to-end delay of one QoS flow and decrease end-to-end delay of the other QoS flow |  |
|  | For Case 1 | Increase D(j) by ΔD(j) and decrease D(i) by ΔD(i) to meet synchronization threshold D(i, j) |
|  | For Case 2 | Increase D(i) by ΔD(i) and decrease D(j) by ΔD(j) to meet synchronization threshold D(j, i) |
| Solution 4 | Provide a more stringent max allowed end-to-end delay to either QoS flow-i or QoS flow-j |  |
|  | For Case 1 | Decrease Dmax(i) by ΔDmax(i) such that a case in Table 2.1-1 (e.g., Case 1.3) transforms into another case (e.g., Case 1.1 or Case 1.2) |
|  | For Case 2 | Decrease Dmax(j) by ΔDmax(j) such that a sub-case in Table 2.1-1 (e.g., Case 2.2) transforms into another case (e.g., Case 2.1) |
| NOTE1: Only consider adjustments D(i) and/or D(j) and do not consider adjustments to Dmax(i) and/or Dmax(j) in solution 1/2/3. NOTE2: Solution 4 can transform a case into any other cases in Table 2.1-1. Only the transformation from Case 1.3 to Case 1.2 was consider in this proposal. The reason is listed in section 2.4. |  |  |

Solution 1: increasing end-to-end delay of a QoS flow can be achieved by application layer-based method, such as The Real-time Transport Protocol and the Real-time Transport Control Protocol (RTP and RTCP, IETF RFC 3550), which can be used to synchronize multiple media flows associated to the multi-modality sessions. See solution #40 in TR 23.700-60 [3] for a detailed description.

Solution 2: the end-to-end delay of a QoS flow can be decreased by scheduling enhancement at RAN e.g. provide the multi-modal service information, including Multi-modal Service ID and synchronization threshold between QoS flows. See solution #66 in TR 23.700-60 [3] for a detailed description.

Solution 3: it can be achieved by applying Option 1 and Option 2 simultaneously.

Solution 4: PCF can provide a set of appropriate Packet Delay Budget, e.g. a more stringent PDB, to meet synchronization thresholds.

Table 2.2-2 summarizes the applicability for Solution 1, Solution 2, and Solution 3 to the cases in Table 2.1-1. The applicability for Solution 4 is discussed in Section 2.4.

Table 2.2-2 Summary of Applicability for General Solutions to Cases in Table 2.1-1

| Notation: ‘Yes’: Applicable – This option can be used in this case. ‘No’: Not Applicable – This option cannot be used in this case. ‘--’: QoS flow-i and QoS flow-j are always synchronized in this case, so we don’t have to consider any solutions. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Case 1 | Case 1.1 | -- |  |  |
|  | Case 1.2 | solution 1 | Applicability | Yes |
|  |  | solution 2 | Applicability | Yes |
|  |  | solution 3 | Applicability | Yes |
|  | Case 1.3 | solution 1 | Applicability | No |
|  |  | solution 2 | Applicability | Yes |
|  |  | solution 3 | Applicability | Yes |
| Case 2 | Case 2.1 | -- |  |  |
|  | Case 2.2 | solution 1 | Applicability | Yes |
|  |  | solution 2 | Applicability | Yes |
|  |  | solution 3 | Applicability | Yes |
| NOTE1: QoS flow-i and QoS flow-j are always synchronized in this case, NOTE2: Detailed description for this table can be found in Annex 5.2. |  |  |  |  |

From Table 2.2-1, it was observed that synchronization threshold is not applicable to Case 1.1 and Case 2.1. Therefore, only the solutions Case 1.2, Case 1.3, and Case 2.2 shall be considered for further study.

2.3 Solutions to the Case 1.2 and Case 2.2 in Table 2.2-1

From Table 2.2-1, it was observed that both Case 1.2 and Case 2.2 have an existing solution, that is Solution 1, which can be achieved through an application layer-based method. Additionally, Solution 1 can be implemented by enhancing scheduling at the RAN, specifically by increasing the delay of either QoS flow-i or QoS flow-j. This approach reduces radio resource requirements and introduces capacity gains.

Observation 2: Solution 1 is applicable to Case 1.2 and Case 2.2 in Table 2.2-1. Additionally, Solution 1 can be implemented by enhancing scheduling at the RAN,

Observation 3: Increasing the delay of either QoS flow-i or QoS flow-j at the RAN, which can reduce radio resource requirements and introduce capacity gains.

2.4 Solutions to the Case 1.3 in Table 2.2-1

Solution 4 provides a more stringent max allowed end-to-end delay of QoS flow-i and transforms Case 1.3 to Case 1.2.

Additionally, because Solution 1 is applicable to Case 1.2, we application layer-based method can be used to synchronize QoS flow-i and QoS flow-j.

However, applying Solution 4 may lead to capacity loss. In Solution 4, the decrement of max allowed end-to-end delay of QoS flow-i, ΔDmax(i), is within the range:

Dmax(i) - Dmax(j) - D(i, j) <= ΔDmax(i) < Dmax(i) - D(i, j)

To minimize negative impacts (e.g., potential capacity loss) at the RAN, Solution 4 should use the minimum decrement of the maximum allowed end-to-end delay for QoS flow-i, Dmax(i) - Dmax(j) - D(i, j). If this decrement causes significant negative impacts, an alternative solution, such as Solution 2, should be considered. Solution 2 can be achieved through scheduling enhancements at the RAN.

Proposal 2: Case 1.3 in Table 2.1-1 can apply Solution 4 and Solution 1 simultaneously to meet the synchronization requirements. However, the capacity gain/loss comparing to Solution 2 shall be the focus of further study in RAN2, which can be achieved by scheduling enhancement at the RAN.

3. Conclusion

Proposal 1: At least use cases 5.5 and 5.7 in TS 22.847 shall be the primary use cases for RAN2 to study the relationship between synchronization threshold and latency KPI of the multi-modal service.

Observation 1: The use cases of synchronization requirements can be classified into two cases based on the relationship between the synchronization threshold and the end-to-end latency.

Observation 2: Solution 1 is applicable to Case 1.2 and Case 2.2 in Table 2.2-1. Additionally, Solution 1 can be implemented by enhancing scheduling at the RAN,

Observation 3: Increasing the delay of either QoS flow-i or QoS flow-j at the RAN, which can reduce radio resource requirements and introduce capacity gains.

Proposal 2: Case 1.3 in Table 2.1-1 can apply Solution 4 and Solution 1 simultaneously to meet the synchronization requirements. However, the capacity gain/loss comparing to Solution 2 shall be the focus of further study in RAN2, which can be achieved by scheduling enhancement at the RAN.

4. Reference

- R2-2405782 LS on multi-modality awareness at RAN

- 3GPP TR 22.847, Study on supporting tactile and multi-modality communication services, Rel-18

- 3GPP TR 23.700-60, Study on XR (Extended Reality) and media services, Rel-18

- 3GPP TS 22.261, Service requirements for the 5G system, Rel-19

5. Annex

  - Detailed definition of notations in relationship between synchronization threshold and latency KPI:

(1) To ensure Quality of Service, we must ensure end-to-end delay D(i) for QoS flow-i remains within its max allowed end-to-end delay Dmax(i), and similarly, D(j) for QoS flow-j must not exceed its max allowed end-to-end delay Dmax(j) i.e.

D(i) <= Dmax(i) and D(j) <= Dmax(j)

(2.1) To ensure synchronization requirement e.g D(i, j), and D(j, i) for QoS flow-i and QoS flow-j, we may adjust Dmax(i) and Dmax(j)

(2.2) Dmax(i) - ΔDmax(i) represents that we provide a decreased Dmax(i), a more stringent max allowed end-to-end delay.

(2.3) Dmax(i) + ΔDmax(i) is invalid. We cannot provide a increased Dmax(i).

(3.1) If D(i) > D(j), we say that QoS flow-i is delayed compared to QoS flow-j.

(3.2) If D(j) > D(i), we say that QoS flow-j is delayed compared to QoS flow-i.

(4.1) To ensure Quality of Service, including max allowed end-to-end delay and synchronization threshold, we may adjust D(i) and D(j).

(4.2) D(i) + ΔD(i) represents increasing D(i) to ensure Quality of Service

(4.3) D(i) - ΔD(i) represents decreasing D(i) to ensure Quality of Service

(5.1) If D(i) - D(j) > D(i, j) , we say that QoS flow-i and QoS flow-j are not synchronized because QoS flow-i is delayed compared to QoS flow-j.

(5.2) If D(j) - D(i) > D(j, i), we say that QoS flow-i and QoS flow-j are not synchronized because QoS flow-j is delayed compared to QoS flow-i.

(6.1) We say that QoS flow-i and QoS flow-j are synchronized if

D(i) - D(j) <= D(i, j),  when D(i) >= D(j)

D(j) - D(i) <= D(j, i),  when D(j) >= D(i)

  - Detailed description of Table 2.2-2

| Notation: ‘Yes’: Applicable – This option can be used in this case. ‘No’: Not Applicable – This option cannot be used in this case. ‘--’: QoS flow-i and QoS flow-j are always synchronized in this case, so we don’t have to consider any options. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Case 1 | Case 1.1 NOTE1 | -- |  |  |
|  | Case 1.2 | Option 1 | Applicability | Yes |
|  |  |  | Range of increment of D(j), ΔD(j) | ΔD(j) ∈ [ D(i) - D(j) - D(i, j), min( D(i) – D(j), Dmax(j) – D(j) ) ] |
|  |  | Option 2 | Applicability | Yes |
|  |  |  | Range of decrement of D(i), ΔD(i) | ΔD(i) ∈ [ D(i) - D(j) - D(i, j), D(i) - D(j) ] |
|  |  | Option 3 | Applicability | Yes |
|  |  |  | Range of increment of D(j), ΔD(j) and range of decrement of D(i), ΔD(i) | ΔD(i) + ΔD(j) ∈ [ D(i) - D(j) - D(i, j), D(i) – D(j) ] |
|  | Case 1.3 | Option 1 | Applicability | No |
|  |  |  | Range of increment of D(j), ΔD(j) |  |
|  |  | Option 2 | Applicability | Yes |
|  |  |  | Range of decrement of D(i), ΔD(i) | ΔD(i) ∈ [ D(i) - D(j) - D(i, j), D(i) - D(j) ] |
|  |  | Option 3 | Applicability | Yes |
|  |  |  | Range of increment of D(j), ΔD(j) and range of decrement of D(i), ΔD(i) | ΔD(i) + ΔD(j) ∈ [ D(i) - D(j) - D(i, j), D(i) - D(j) ] with constraint ΔD(i) >= D(i) - ( Dmax(j) + D(i, j) ), if D(i) > Dmax(j) + D(i, j) |
| Case 2 | Case 2.1 NOTE1 | -- |  |  |
|  | Case 2.2 | Option 1 | Applicability | Yes |
|  |  |  | Range of increment of D(i), ΔD(i) | ΔD(i) ∈ [ D(j) - D(i) - D(j, i), D(j) – D(i) ] |
|  |  | Option 2 | Applicability | Yes |
|  |  |  | Range of decrement of D(j), ΔD(j) | ΔD(j) ∈ [ D(j) - D(i) - D(j, i), D(j) - D(i) ] |
|  |  | Option 3 | Applicability | Yes |
|  |  |  | Range of increment of D(i), ΔD(j) and range of decrement of D(j), ΔD(j) | ΔD(i) + ΔD(j) ∈ [ D(j) - D(i) - D(j, i), D(j) - D(i) ] |