3GPP TSG-RAN WG2 Meeting #127			     R2-2407520

Maastricht, Netherlands, 19– 23 Aug, 2024

Agenda Item:	8.5.4 (Netw_Energy_NR-enh-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on common signal and channel adaptation

Document for: 	Discussion and Decision

1 Introduction

The following were agreed in the previous meeting RAN2#125-bis.

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, we share our view on common signal/channel adaptation.

2 Discussion

RAN2 decided to study the following two mechanisms for paging occasion adaptation across the time domain:

- Option a. Bundle paging frames

- Option b. Extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

Paging frames continues to stay allocated in the time domain for option a. gNB sleep state is extended as the time interval of the radio frame without a paging frame is increased. Option b is the mechanism that extends the period of the paging frame, increasing the time the gNB stays in sleep state. As the number of PFs decreases due to the extension of the paging frame cycle, the number of POs within the PFs need to be increased.

Uniform distribution of POs in the time domain balance the RACH resource per PO, but prevents the gNB from staying in the sleep state for a long time within the paging cycle. In both options, the RACH resource per PO is reduced compared to legacy paging, since the space between PFs and the space between POs reduce in option a and option b respectively. Therefore, the paging adaptations can increase RACH congestion. When comparing the above options, it is necessary to analyze not only NES gain but also RACH congestion for each paging adaptation technique. Longer sleep state delays the paging frame transmission, which also delays the monitoring of PO in both options. It is also necessary to analyze paging latency probability when comparing the options.

2.1 Performance comparison of options without legacy UE

Figure 1 shows an example of legacy PRACH, paging, option a and option b. T(DRX cycle) is set to 32rf, i.e. 32 radio frame. The first image of Figure 1 is an example of legacy PRACH, which index is 111 from table 6.3.3.2-3 of 38.211 [1]. The second image is an example of legacy paging. It is assumed that N, number of total paging frames in T, is set to T/8 and Ns, number of paging occasions for a PF, is set to 1. The two paging frames in the allocation of NES PFs for option a are positioned side-by-side in the time domain, and each NES PF has one PO as shown in third image. In the allocation of NES PFs for option b, as shown in the fourth image, N is set to T/8, and each NES PF has one PO.

< Figure 1. Example of legacy PRACH, paging, option a, option b >

First, we will compare NES gain of options in terms of total sleep duration. NES gain is the sleep duration ratio of option/legacy paging. We will not focus on the sleep duration between POs in a PF.

The total sleep duration can be calculated as follows:

-	Total sleep duration = DRX cycle – total transition time – PF period

- Total transition time: total transition time entering and leaving the sleep state.

  - Transition time for Deep sleep: 50ms.

- PF period: Time duration of PF

-	NES gain: Total sleep duration for option / Total sleep duration for legacy

For option a, the time duration excluding PF period is 280ms so that the NES cell can stay in deep sleep state for 230ms within 32 radio frames. Therefore, total sleep duration is 230ms, and NES gain is 3.5 since total sleep duration of legacy paging is 80ms. Total sleep duration of option b is 200ms, NES gain is 2.5. In terms of NES gain, there is not much difference between option a and option b.

Second, we will compare the options in terms of paging latency probability. RO latency is not considered in the paging latency probability for simplicity. We calculate paging latency probability as follows:

- Paging latency probability= Total paging delay duration/ DRX cycle

For legacy paging, a paging frame is transmitted by the radio access network in the 8th frame, when the core network requests paging from 0th to 7th frame. However, a paging frame is transmitted by the radio access network in the 32nd frame for option a, when the core network requests paging from 4th to 23th frame. Paging delay duration for option a is 20ms within 32 frames, so the paging latency probability is 0.625 for option a. For option b, a paging frame is transmitted by the radio access network in the 16th frame when the core network requests paging from 0th to 7th frame. In this case, paging delays since a paging frame is transmitted by the radio access network in the 8th frame for legacy paging. Paging delay duration for option b is 14ms within 32 frames. So the Paging latency probability for option b is 0.4375. In terms of paging latency probability, there is not much difference between option a and option b.

Third, we will compare options in terms of RACH resource congestion in NES cells in case the legacy UE is not allowed to camp on the NES cell. Peak ratio of PO to RO (Rach Occasions) is used to compare RACH congestion, and it can be calculated as follows:

-	RACH congestion = max (number of PO / number of RO that UE used for PO)

There are two PRACH slots within a radio frame for every eight frames, and there are two ROs within each PRACH slot. The more RACH resources per PO, the RACH congestion is minimized. To prevent delay in RACH of the UE receiving the paging, it is assumed that the UE performs RACH only using the ROs in the frame closest to the PO.

For legacy paging, the value of Ns is set to 1. The UE that receives one PO in the 0th frame will use the resources of four ROs in the first frame. Every ratio of PO to RO is the same in DRX cycle, so peak ratio of PO to RO for legacy paging is 0.25. For option a, the value of Ns is also set to 1. The UE that receives one PO in the 1st, 2nd, 3rd frame will use the resources of four ROs in the 9th frame. Peak ratio of PO to RO for option a is 0.75 since four RO resources are used for three PO resources. For option b, the value of Ns is set to 2 to compensate total number of paging occasions. Peak ratio of PO to RO for option b is 0.5 since four RO resources are used for two PO resources. For option a, peak ratio of PO to RO is three times larger than that in legacy paging. RACH congestion for option b is doubled compared to that in legacy paging. In terms of the RACH congestion, both options have worse RACH congestion that that in legacy UE.

Table 1. Performance comparison of legacy paging, option a and option b

|  | Total sleep duration | NES gain | RACH congestion | Paging latency probability |
| --- | --- | --- | --- | --- |
| Legacy paging | 80ms | 1 | 0.25 | - |
| option a | 280ms | 3.5 | 0.75 | 0.625 |
| option b | 200ms | 2.5 | 0.5 | 0.4375 |

Table 1 summarizes the performance comparison between option a and option b. We note that, compared to legacy paging, RACH congestion is significantly increased in both options. Considering that increased RACH congestion may result in connection failure, it is worth developing other paging adaptation methods to not increase RACH congestion by non-trivial amount.

2.2 New paging bundling option (variant of option B)

The larger RACH congestion in Option a is mainly due to longer bundling interval. To reduce RACH congestion, the new paging method should have similar bundling interval as Option b. Given the similar bundling interval as Option b, we can consider the new paging method to distribute the POs over multiple PFs, rather than concentrating them within a single PF as in Option b. Figure 2 shows the new paging bundling method, which we call option C.

< Figure 2. Example of legacy PRACH, paging, option c >

The distribution of POs over multiple PFs is effective in further reducing RACH congestion while the resulting lowered NES gain due to shortened idle period between two bundles is marginal. NES gain, RACH congestion and paging latency probability are calculated, and their values are in table 2. For NES gain, the time duration excluding PF period is 140ms within 16 frame, so the NES cell can stay in deep sleep state for 90ms within 16 radio frame. Therefore, total sleep duration is 180ms within DRX cycle, NES gain is 2.25 since total sleep duration of legacy paging is 80ms. For RACH congestion, the UE that receives one PO in the 1st frame will use the resources of four ROs in the 9th frame. Peak ratio of PO to RO for option a is 0.25 since four RO resources are used for one PO resources. For paging latency probability, a paging frame is transmitted by the radio access network in the 16th frame when the core network requests paging from 1th to 7th frame. The paging frame is transmitted by the radio access network in the 8th frame for legacy paging. Paging delay duration for option c is 7ms within 16 frames. So the paging latency probability for option c is 0.4375.

Table 2. Performance comparison of legacy paging, option a, option b and option c

|  | Total sleep duration | NES gain | RACH congestion | Paging latency probability |
| --- | --- | --- | --- | --- |
| Legacy paging | 80ms | 1 | 0.25 | - |
| option a | 280ms | 3.5 | 0.75 | 0.625 |
| option b | 200ms | 2.5 | 0.5 | 0.4375 |
| option c | 180ms | 2.25 | 0.25 | 0.4375 |

In table 2, we summarize performance comparison between option a, option b and option c. As shown in the table, option c has comparable NES gain as option B while RACH congestion is 100% lower than option B. That is, Option C provides much better performance trade-off than Option B. From this comparison, we think Option c (bundling of POs over multiple consecutive PFs) should be supported for NES.

Proposal 1.	PO bundling over multiple consecutive PFs should be supported for NES cells without legacy UE.

2.3 Whether to allow camping legacy UE in NES cell applying paging adaptation

RAN2 needs to discuss if legacy UE can camp on NES cell applying paging adaptation. RAN2 agreed the following.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

If legacy UE is allowed to camp on NES cell applying paging adaptation, the NES cell should provide paging in legacy manner. There are various manners to implement the option a, option b and option c with legacy PF/POs. We suggest examples of options for NES cell with legacy UEs. Figure 3 is an example of legacy PRACH, paging, option a, option b and option c. T(DRX cycle) is set to 64rf, i.e. 64 radio frame.

< Figure 3. Example of legacy PRACH, paging, option a, option b and option c >

First image of Figure 3 is the example of legacy PRACH, which index is 110 from table 6.3.3.2-3 of 38.211 [1]. The second image is given as a reference of PF configuration in legacy cell, and each PF has two PO, and the third and fourth image correspond to the examples of option a and option b with legacy UEs. The green PF is the PF that a legacy UE and R19-UE can monitor, and the blue PF is the PF that R19-UE can only monitor. In the images for options, the total number of PO within green PFs (PF monitored by legacy UE and R-19 UE) in one paging cycle is reduced compared to the reference case. Then, the reduced paging opportunities are compensated by adding blue PFs (PF which can only be monitored by R-19 UE) and/or adding POs within green PF. The total number of POs is the same across legacy paging as reference, option a, option b and option c for a fair comparison.

Table 3. Comparison of option a, option b and option c with legacy UE

|  | Total sleep duration | NES gain | RACH congestion | Paging latency probability |
| --- | --- | --- | --- | --- |
| Legacy paging | 400ms | 1 | 0.5 | - |
| option a | 380ms | 0.95 | 1 | - |
| option b | 400ms | 1 | 1.3 | - |
| Option c | 380ms | 0.95 | 1.3 | - |

Table 3 compares option a, option b and option c with legacy UE. NES gain in all options are lower than 1 since the number of paging frame is increased. So, there is no paging latency. RACH congestion in all options are higher than that in legacy paging. All options have not only less NES gain than legacy paging, but also more RACH congestion. Therefore, NES cells with legacy UE should not be considered.

Proposal 2. 	Network cannot allow legacy UEs to camp on NES cell applying paging adaptation.

3 Conclusion

Proposal 1.	PO bundling over multiple consecutive PFs should be supported for NES cells without legacy UE.

Proposal 2. 	Network cannot allow legacy UEs to camp on NES cell applying paging adaptation.

4 References

- 3GPP TS 38.211, “NR; Physical channels and modulation (Release 18)”, V18.1.0 (2024-01).