3GPP TSG-RAN WG2 Meeting #127	R2-2407530

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	7.25.1

Source:	Samsung

Title:	Clarifications for ATG timing advance reporting procedures

WID/SID:	NR_ATG

Document for:	Discussion and Decision

# Introduction

In RAN#95-e the WID for Air-to-Ground was agreed. The work item is a RAN4-exclusive work item and the latest WID can be seen below [1]. Furthermore, the work is captured in 38.876 [2].

In RAN2#126 the following was discussed:

R2-2405529	Clarification on ATG Timing advance Reporting procedures	Samsung	discussion	Rel-18	NR_ATG-Core

Proposal 1: RAN2 to discuss how to handle switching of SCSes via BWP-switching for timing advance reporting:

•	offsetThresholdTA-r18

•	Timing Advance field in TAR MAC CE

Proposal 2: RAN2 to choose between

1) Clarifying that SCS is based on SCS of active BWP

2) Clarifying that SCS is based on SCS of initial BWP

3) Any other solution

=>	we will address the issue next meeting (if needed)

-	Samsung would prefer option 2.  ZTE would prefer a simple solution. Huawei thinks it should be the active BWP.  CATT and Qualcomm think that the current spec already specifies option 1, so there is no issue in current spec.

# Discussion

## Bandwidth part operation

- In NR release 15, the concept of Bandwidth Part operation was introduced to allow for allocating a UE with chunk of the bandwidth to allow for more flexible bandwidth management.

- A Bandwidth Part configuration consist of a location in frequency, a bandwidth and subcarrier spacing. A downlink BWP may be configured with a PDCCH, PDSCH, SPS and an RLM BWP-specific configuration. An uplink BWP may be configured with a RACH, PUSCH, PUCCH, CG, SRS and BFR BWP-specific configuration.

- There are a maximum of 4 DL/UL BWPs, with one of the BWPs being the initial BWP, which is used when the UE accesses the network. The remaining BWPs can be dedicatedly configured.

- Bandwidth part switching is either done by the network or the UE. The network can use PDCCH to dynamically switch between the configured BWPs. The UE can only switch from non-initial BWP to initial BWP for some specific cases, such as to perform random access, certain failures, etc.

## ATG Timing advance reporting

- In NTN, the timing advance reporting was introduced in order for the UE to report its currently applied Timing Advance that the UE has estimated. This can be used to adjust the Koffset or to adjust the Timing Advance with Timing Advance commands. The granularity of the reported Timing Advance is in number of slots in 15 kHz subcarrier spacing. This makes the field absolute in time. The threshold trigger the Timing Advance report is also in absolute time, defined in steps of milliseconds, 1 to 15 milliseconds, including 0.5 ms.

Observation 1: In NTN, the timing advance reporting mechanism is based on absolute time.

- In ATG, the range of the propagation delays are a lot smaller compared to an NTN network. This means that the absolute range can be a lot smaller. As a result, the reported value in the Timing Advance Report was agreed to be in number of symbols. Similarly, the threshold to trigger the Timing Advance Report is also in number of slots, defined in steps 1 to 56 slots. This means that the threshold and the reported value of the Timing Advance report is not in absolute time, but dependent on the SCS.

Observation 2: In ATG, the timing advance reporting mechanism is not based on absolute time.

## BWP switching with ATG timing advance reporting

- As explained above, a UE may be configured with BWPs that have different SCSes in the same cell. Thus, when the UE or the network switches between different BWPs, the SCS may change dynamically. For this purpose, many of the configurations that depend on the SCS are configured per BWP.

Observation 3: The SCS may change dynamically with BWP switching and UE may be configured with multiple SCSes in a cell.

- One issue with the current specifications, for both the reported value in the Timing Advance Report is that it is still not 100% which SCS that it is being referred:

-	Timing Advance: In FR1 except for ATG, the Timing Advance field indicates the least integer number of slots, using subcarrier spacing of 15 kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1).

For ATG in FR1, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarriers spacing the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used.

The length of the field is 14 bits.

- Similarly, for the offsetThresholdTA-18, it is not made fully clear how to interpret the symbol duration according to different SCS:

TAR-Config-r18 ::=                      SEQUENCE {

offsetThresholdTA-r18               INTEGER (1..56)              OPTIONAL,    -- Need R

timingAdvanceSR-r18                 ENUMERATED {enabled}         OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For ATG, network only configures offsetThresholdTA-r18, which is in unit of symbols.

timingAdvanceSR Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

- One way to solve this would be to make it clear to which SCS that the reported Timing Advance value and the threshold refers. For instance, it can be clarified that the SCS refers to the current active BWP.

- One issue in Timing Advance Reporting MAC CE, is that if the SCS changes, then if a TAR MAC CE is triggered in BWP#1 with SCS#1, and then the BWP changes to BWP#2 with SCS#2, then the receiving gNB will interpret the TAR MAC CE using SCS#2.

- One issue with the offsetThresholdTA and clarifying that it is up to the active BWP, is that this may mean that the absolute threshold changes if the SCS changes. Thus for instance if the UE is configured with offsetThresholdTA=50 symbols in 15 kHz SCS, then when the SCS changes to 30 kHz SCS, then the absolute timing value that the threshold corresponds to will be half of what it was in 15 kHz SCS. This is unlikely to be what a network would want to occur. This is likely to trigger unnecessary reporting of the TAR MAC CE when the BWP changes. This can be seen in Figure 1.

Figure 1.

- We thus think that there needs to be a fixed value that does not change with the when the SCS changes dynamically for the offsetThresholdTA and the Timing Advance field in TAR MAC CE.

- After offline discussions we have identified the following fixes:

- SCS is assumed to be based on initial BWP SCS for offsetThresoldTA and Timing Advance field in TAR MAC CE

- SCS is assumed to be based on 15 kHz SCS for offsetThresholdTA and Timing Advance field in TAR MAC CE

- SCS is assumed to be based on the SCS when the UE is configured with Timing Advance reporting is configured for offsetThresholdTA and Timing Advance field in TAR MAC CE

- SCS is based on active BWP SCS for offsetThresholdTA and Timing Advance field in TAR MAC CE

  - This may have some drawbacks as stated above

- Thus we propose that RAN2 decides from the following:

- Proposal 1: For clarifying the applied SCS for ATG offsetThresholdTA and Timing Advance field in TAR MAC CE, RAN2 chooses from:

- 1) SCS is assumed to be based on 15 kHz

- 2) SCS is assumed to be based on initial BWP

- 3) SCS is assumed to be based on the SCS when the UE is configured with Timing Advance reporting is configured

- 4) SCS is based on active BWP SCS

- 5) Other solution

# Conclusion

In this contribution we discussed issues related to ATG Timing advance reporting.

Observation 1: In NTN, the timing advance reporting mechanism is based on absolute time.

Observation 2: In ATG, the timing advance reporting mechanism is not based on absolute time.

Observation 3: The SCS may change dynamically with BWP switching and UE may be configured with multiple SCSes in a cell.

- Proposal 1: For clarifying the applied SCS for ATG offsetThresholdTA and Timing Advance field in TAR MAC CE, RAN2 chooses from:

- 1) SCS is assumed to be based on 15 kHz

- 2) SCS is assumed to be based on initial BWP

- 3) SCS is assumed to be based on the SCS when the UE is configured with Timing Advance reporting is configured

- 4) SCS is based on active BWP SCS

- 5) Other solution

# Reference

RP-230279, Revised WID on Air-to-Ground network for NR, CMCC, RAN#99, March, 2023.

3GPP TR 38.876, “Study on Air-to-Ground network for NR”, Release 18, V0.4.0, May 2023.