3GPP TSG-RAN WG2 Meeting #127	R2-2407531

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	    8.5.4

Source:	III

Title:	Adaptation of common signal/channel transmissions

Document for:	Discussion

Introduction

In RAN2#125bis [1] meeting, the agreements of adaption of common signal/channel for paging are as follows:

Agreements on paging adaptation: 1. From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. 2. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. 3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, adaptation of the Paging occasions in time domain and compatibility of legacy UEs and NES UEs in IDLE/INACTIVE are discussed.

Discussion

Adaptation of paging occasions in time domain

Adaptation of paging frame by compact/bundle

In TR 38.864[2], it is shown that adaptation of PF/PO can achieve BS energy savings by 0.2%~42.3%. In RAN1#116 email discussion [3], compact/bundle PF periodicity were mentioned [4] [5]. The mechanism for compact/bundle PF adaptation was descripted in Figure 1 and Figure 2.

In TS 38.304[6], the text and formula for PF and PO are as follows:

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO. The PF and PO for paging are determined by the following formulae: The PF and PO for paging are determined by the following formulae: SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns N: number of total paging frames in T Ns: number of paging occasions for a PF PF_offset: offset used for PF determination UE_ID: If an eDRX cycle is configured by RRC or upper layers and eDRX-Allowed is signalled in SIB1: - 5G-S-TMSI mod 4096 else: - 5G-S-TMSI mod 1024

For energy saving consideration, the gNB should transmit paging frames in a limited time duration and keep rested as long as possible. Two methods are discussed – compact paging frames and bundle paging frames.

For compact PF, all paging frames will be concentrated in T/M  time duration, where T is the paging cycle and M is the compact factor.

Paging cycle can be divided into M segments, the related modification is as below.

(SFN + PF_offset/M) mod T = [((T/M) div N)*(UE_ID mod N)]

Figure 1: Compact PF in paging cycle

All PFs can be compacted into one segment and the NES cell can sleep in other M-1 segments, as figure 1 (N=4, M=2).

For bundle PF, all paging frames will be consecutive and occupy N frames in one paging cycle, where N is the number of PFs in one paging cycle.

Bundle PFs can be achieved by setting M=T/N in compact PF method as Figure 2:

Figure 2: Bundle PF in paging cycle

Since the number of paging frames on NES cell is the same as number of legacy paging frames, the number of paging occasions doesn’t need to be changed. The legacy PO calculation/allocation method can be reused for compact/bundle PF.

Observation 1: Compact PF is feasible for cell energy saving.

Observation 2: Bundle PF can be achieved by setting M=T/N in compact PF.

Observation 3: The legacy PO calculation/allocation method can be reused for compact/bundle PF.

Extend the values of N to have increased interval between PFs

The number of PFs in each paging cycle should be decreased as small as possible in NES cell for NES-capable UE. NES cell could have more sleep time for energy saving.

In TR 38.331, PF was determined by nAndPagingFrameOffset in PCCH-Config. The minimum number of PFs for legacy UE is shown as the last row in table 1.  Nmin = 2,4,8,16 for T = 32rf, 64rf, 128rf, 256rf separately.

Table 1: number of PFs determined by nAndPagingFrameOffset for legacy UE

| T(cycle) N(PF) | 32rf | 64rf | 128rf | 256rf |
| --- | --- | --- | --- | --- |
| oneT | 32 | 64 | 128 | 256 |
| halfT | 16 | 32 | 64 | 128 |
| quarterT | 8 | 16 | 32 | 64 |
| oneEighthT | 4 | 8 | 16 | 32 |
| oneSixteenthT | 2 | 4 | 8 | 16 |

Extend the values of N to have increased interval between PFs for Rel-19 NES UE in table 2.

Table 2: number of PFs determined by nAndPagingFrameOffset for Rel-19 NES UE

| T(cycle) N(PF) | 32rf | 64rf | 128rf | 256rf |
| --- | --- | --- | --- | --- |
| oneT | 32 | 64 | 128 | 256 |
| halfT | 16 | 32 | 64 | 128 |
| quarterT | 8 | 16 | 32 | 64 |
| oneEighthT | 4 | 8 | 16 | 32 |
| oneSixteenthT | 2 | 4 | 8 | 16 |
| one32thT | 1 | 2 | 4 | 8 |
| one64thT |  | 1 | 2 | 4 |
| one128thT |  |  | 1 | 2 |
| one256thT |  |  |  | 1 |

Proposal 1: RAN2 should further discuss the cases of one32thT, one64thT, one128thT, one256thT.

When only one PF was configured in the paging cycle for Rel-19 NES UEs, all UE_IDs will be allocated to corresponding POs (at most four in legacy PO allocation). This will cause congestion. One possible solution is to increase the number of POs in this PF. This is easily to add new value of ns in TS 38.331 DownlinkConfigCommonSIB for Rel-19 NES UE.

For example, a new value “eight” is added in ns.

PCCH-Config ::= SEQUENCE { defaultPagingCycle PagingCycle, nAndPagingFrameOffset CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15) }, ns ENUMERATED { ,eight ,four, two, one},

More POs may occupy more time domain resource and extend the PF to be larger than one frame as Figure 3.

Figure 3: Frame extension by new value of ns

The number of extended frames depends on the number of POs configured in NES cell.

Observation 4: One PF solution will occupy time resource more than one frame when the number of POs is increased.

Compatibility of legacy RRC_IDLE/RRC_INACTIVE UE

Solutions for option 1 and option 2 are discussed below:

Option 1: Prevent the access of legacy UE via barring.

When legacy UEs will not be allowed to camp on the NES cell, legacy idle/inactive mode UEs can be barred by NES cell based on Rel-18 NES barring procedures.

In TS 38.331, Rel-18 NES cell barred procedures:

1> if the UE supports NES cell DTX/DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting NES cell DTX/DRX is in RRC_CONNECTED while T311 is running: 2> if cellBarred in the acquired MIB is set to barred: 3> if cellBarredNES is absent in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

Proposal 2: Legacy idle/inactive mode UEs can be barred by NES cell. Barring mechanism for NES Rel-19 UE can follow Rel-18 NES barring procedures, e.g. introduce IE - cellBarredNES-Paging-r19.

Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs

In RAN2#125bis agreement, the "Paging resources for legacy UEs and Rel-19 NES UEs" considers the compatibility of legacy UEs. Legacy UE design should not affect BS energy saving, especially when there is excessive PFs in paging cycle. The implementation of this agreement ensures that paging resources are balanced for legacy UEs and Rel-19 NES UEs. The following is considered for paging resource of NES cell:

One solution for separating paging resources is to align the PFs of Rel-19 NES UEs with PFs of legacy UEs and minimize the number of PFs of NES UEs as Figure 4. (Assume paging cycle is 32rf, number of PF is 2 for legacy UE and number of PF is 1 for Rel-19 NES UE).

Figure 4: PF of Rel-19 NES UEs in paging cycle

The PFs for NES UE can be one PF solution or bundle/compact solution mentioned in section 2.1.

The other solution for separating paging resources is to overlap the PFs of Rel-19 NES UEs with PFs of legacy UEs as Figure 5.

Figure 5: overlap the PFs of Rel-19 NES UEs with PFs of legacy UEs

The PDCCH monitoring occasions (PMO) of Legacy UE and Rel-19 NES UE can be separated in time domain. One example is as in figure 6.

Figure 6: PDCCH monitoring occasions (PMO) of Legacy UE and Rel-19 NES UE

Proposal 3: Separated paging resources can be achieved in time domain for overlapped paging frame.

Consideration for adaption of paging frame during Cell DTX/DRX non-active period

In RAN2#121bis-e agreement [7], the UE does not transmit SPS/SR/CG during Cell DTX/DRX non-active period.

There is no agreement about common channel behavior during Cell DTX/DRX non-active period, e.g., paging channel for idle mode UE. The compact PF is one feasible method for REL-19 NES UE receiving paging message during cell DTX on-duration.  Further clarifications are needed for this issue:

- In R18 Cell DTX/DRX discussions, only connected mode UEs were addressed, How to extend the cell DTX/DRX mechanism for idle/inactive mode UE.

- How to align the compact/bundle PF with cell DTX on-duration for idle/inactive mode UE.

Proposal 4: Compact/bundle PF should be further studied at least in the scenario of cell DTX/DRX mechanism for idle/inactive mode UE.

Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: Compact PF is feasible for cell energy saving.

Observation 2: Bundle PF can be achieved by setting M=T/N in compact PF.

Observation 3: The legacy PO calculation/allocation method can be reused for compact/bundle PF.

Proposal 1: RAN2 should further discuss the cases of one32thT, one64thT, one128thT, one256thT.

Observation 4: One PF solution will occupy time resource more than one frame when the number of POs is increased.

Proposal 2: Legacy idle/inactive mode UEs can be barred by NES cell. Barring mechanism for NES Rel-19 UE can follow Rel-18 NES barring procedures, e.g. introduce IE - cellBarredNES-Paging-r19.

Proposal 3: Separated paging resources can be achieved in time domain for overlapped paging frame.

Proposal 4: Compact/bundle PF should be further studied at least in the scenario of cell DTX/DRX mechanism for idle/inactive mode UE.

- References

- R2-2403731,” Report from session on V2X/SL, R19 NES and MOB”, RAN WG2 #125bis, April, 2024

- TR 38.864,” Study on network energy savings for NR”, Mar 2023.

- R1-2401870,” Final summary of AI 9.5.3 for R19 NES”, Ericsson, Feb 2024.

- R1-2400371,” Adaptation of common signal/channel transmissions for Network Energy Saving”, Intel, Feb 2024.

- R1-2401145,” Adaptation of common signal/channel transmissions for NES”, Ericsson, Feb 2024.

- TS 38.304 V17.2.0,” User Equipment (UE) procedures in Idle mode and RRC”, Sep 2022.

- R2-2304203,” Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”, InterDigital , April 2023.