3GPP TSG-RAN WG2 Meeting #127	R2-2407532

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.8.2

Source: 	Sequans Communications

Title: 	Downlink coverage enhancement SMTC impacts

Document for:	Discussion and Decision

# Introduction

During previous meetings, RAN2 started discussing Downlink coverage enhancement aspects related to beam hopping, and made the following agreements:

Agreements:

- With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

- We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

Ask RAN1 to keep us informed on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values.

More in general, ask RAN1 whether/how the solution they are investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive

Also remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status will have to relate to beams visible to the UE (e.g. SSB beams)

Also ask RAN1 whether they are also working on UL beam hopping (and whether this is separate from DL beam hopping)

Agreements;

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

- RAN2 assumes that both EFC and EMC are supported

In parallel, RAN1 has made some progress and observations related to SSB periodicity. In this contribution, we address corresponding aspects related to SMTCs.

# Discussion

## Background

From the WID ‎[1] justification:

DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the « nominal » EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS. DL coverage enhancements can be considered at both - Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design. - System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN

RAN1 has progressed and made following observations:

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint: - 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%. - 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively. - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams. - 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%. - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth. Note: RAN1 will further investigate the impact of SSB periodicity extension Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...)

The SSB periodicity extension was discussed at last RAN plenary, and the WID updated to clarify it was not excluded.

## Impact on SMTCs

The rationale behind SSB periodicity extension is the following: to comply with the limited number of simultaneous active beams, it is required to multiplex in time the transmissions of SSBs across the satellite footprint.

At cell level, the SSBs can be time multiplexed as part of the SSB burst pattern using different NR beams within the cell. However, at the satellite footprint level, this requires using different SSB offsets (typically with a half-frame granularity) for each cell. This is somehow similar to what was discussed for the soft satellite switch with resync (see ‎[2]), except that now the SSB offset would change in the space domain rather than in the time domain.

According to initial RAN1 observations, we anticipate that the SSBs would need to be spread out across the satellite footprint over up to 80ms or 320ms time range depending of the scenarios.

Observation 1: SSBs needs to be time multiplexed across the satellite footprint over up to 80ms or 320ms range

Typically, the serving cell would have up to 6 neighbors, as illustrated below:

Figure 1 – Intra-frequency cells SSB offsets

In legacy NR, the SSBs for a given frequency layer are assumed to be confined within a 5ms max duration window, i.e. there is one SMTC per frequency layer (modulo additional SMTCs for short/long periodicity monitoring). In NTN, it was decided to extend the number of SMTCs to 4 per frequency layer (each associated with a list of PCIs), to account for up to 4 satellites.

Observation 2: Existing SMTCs framework is limited to up to 4 SMTCs per frequency layer

In our understanding, this would not be enough given typically a cell has 6 neighbors.

Proposal 1: RAN2 to study SMTC impact of SSB time multiplexing across the satellite footprint

As a baseline, it seems necessary to extend the number of SMTCs that can be configured by frequency layer. This could be done by extending the existing SMTC4 list, or by defining a new list.

Proposal 2: Consider extending the NTN SMTC list / defining a new list to signal at least all neighbor SMTCs

However, it seems not reasonable to expect the UE to monitor up to 6 SMTCs. This might dramatically increase the cell search complexity.

In our view, some additional changes would be useful to help the UE to select the SMTCs on which it has to perform measurements (differently said, to choose which SMTC to consider for measurements). Typically, the UE should only consider SMTCs corresponding to the N closest neighbor cells, depending on its capabilities (N=1 in general, exceptionally 2).

Observation 3: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells

Idle/Inactive mode

So far, the reference location (or moving reference location in case of EMC) is broadcasted. This allows for instance location-based measurement triggering.

In addition, the reference location (or moving reference location in case of EMC) of neighbor cells could be broadcasted. This would enable the UE to know which SMTC(s) to select/measure. The SMTCs/ Reference location would be associated by e.g. the PCI of the neigbor cell).

Proposal 3: Consider broadcast of neighbor cells reference location to allow location-based UE SMTC selection

Connected mode

In connected, it is up to the NW to configure the SMTCs for measurements by the UE.

For QEF cells, without considering UE mobility, no specific impact is expected (this is not a scenario with mobility to neighbor cells).

For EMC, or QEF/EF with UE mobility to neighbor cells, ideally the NW would need the UE location to configure only the needed SMTCs. In general, the NW may not have a precise UE location available. Coarse location can be requested but at the expense of signaling overhead. Moreover, a moving UE would also need to update its location, which would then require an update of the SMTCs configured by the NW.

Observation 4: In connected, NW may not have UE location information needed to configure SMTCs to UE

One solution would be to use R18 location-based CHO, without associated measurement. In such case, no SMTCs need to be configured for measurements before HO execution. The SMTC associated to each HO candidate can be configured as usual as part of each target cell candidate handover command.

Using location-based CHO without associated measurement may still have some drawbacks. It would increase the handover interruption time. Moreover, it is an optional feature, and it comes with additional complexity.

Observation 5: R18 location-based CHO without associated measurement avoids SMTCs configuration, but adds complexity and may increase handover interruption time

In our view, it could also be beneficial to allow the location-based UE SMTC selection in connected. With this scheme, the NW would configure SMTCs without a priori knowledge of UE position (or with limited knowledge), while the UE would only select/measure the needed SMTCs based on its position and neighbor cells (moving) reference.

At least for intra-satellite intra-frequency mobility (such as the mobility inside the satellite footprint), there should be no drawbacks with such solution.

Proposal 4: Consider location-based UE SMTC selection also in connected

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: SSBs needs to be time multiplexed across the satellite footprint over up to 80ms or 320ms range

Observation 2: Existing SMTCs framework is limited to up to 4 SMTCs per frequency layer

Proposal 1: RAN2 to study SMTC impact of SSB time multiplexing across the satellite footprint

Proposal 2: Consider extending the NTN SMTC list / define a new list to signal at least all neighbor SMTCs

Observation 3: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells

Proposal 3: Consider broadcast of neighbor cells reference location to allow location-based UE SMTC selection

Observation 4: In connected, NW may not have UE location information needed to configure SMTCs to UE

Observation 5: R18 location-based CHO without associated measurement avoids SMTCs configuration, but adds complexity and may increase handover interruption time

Proposal 4: Consider location-based UE SMTC selection also in connected

# References

- RP-241667,	 “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, THALES

- R2-2405680, “Soft satellite switch SSB time offset and SMTC impact”, Sequans Communications