3GPP TSG-RAN WG2 Meeting # 127                	R2-2407533

Maastricht, The Netherlands, August 19th - 23rd, 2024

Source:	Philips International B.V.

Title:	Discussion on Paging procedure for Ambient IoT

Agenda Item:	8.2.3

Document for:	Discussion and Decision

# Introduction

The latest RAN2 TP for TR 38.769 [1] captures the following paging procedure for Ambient IoT. This contribution discusses our views on additional information which may be included in the Ambient IoT paging message.

### 6.2.3 A-IoT paging functionality In AS layer, the A-IoT paging functionality is to indicate device(s) that need to respond. As to the A-IoT paging message, the identifier may be required to identify the device/group of devices in this trigger message (e.g. for the case of reaching a single or a group of devices). Following cases are studied: - The A-IoT paging message containing an identifier of a single A-IoT device - The A-IoT paging message containing a group ID that maps to multiple A-IoT devices - The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond - The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in [SA2 TR 23.700-13]. NOTE 1: The details of the above identifier and group ID and also the use case/scenario are studied in [SA2 TR 23.700-13]. As to the A-IoT paging message, it can additionally indicate the information from which the device(s) can determine the resource(s) to be used for D2R response message(s). For A-IoT device paging functionality, it is understood that the legacy paging message, legacy paging occasion and legacy DRX from NR are not supported (See TS 38.300 for references for any legacy NR functionality). From RAN2 perspective, it is assumed that the A-IoT device can receive as long as there is enough energy.

# Discussion

## Total number of access occasions and resource

It has been agreed by RAN1 and RAN2 to use slotted-ALOHA as the baseline random access procedure. The total number of access occasions need to be announced by the reader to all ambient IoT devices to be paged, so that each ambient IoT device can determine and select the access occasion it wants to use.

Proposal 1: Ambient IoT paging message may include the total number of access occasions for slotted-ALOHA based random access procedure.

RAN1 #116bis had the following agreement on D2R multiple access.

Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

Frequency-domain multiple access from the ambient IoT device to the reader will be studied in RAN1. If RAN1 finally agrees FDMA is feasible for ambient IoT D2R communication, frequency domain related multiple access resource configuration may also be provided in the paging message.

Proposal 2: Whether frequency-domain related multiple access resource configuration may be included in the paging message is up to RAN1 further study.

## Access occasion announcement

According to our companion contribution in [2], the reader may transmit an access occasion announcement message in the network for ambient IoT devices to identify whether its access occasion arrives or not.

Proposal 3: Reader may transmit access occasion announcement message.

A tag that selects an access occasion number needs to determine when the occasion arrives. This requires keeping track of the access occasion announcement messages, which can be prone to errors when the tags are in a tight energy budget.

Observation 1: Keeping track of the access occasion announcement messages can be prone to errors when the tags are in a tight energy budget.

## Inventory Session ID for the inventory procedure

At RAN3 #124 [3], RAN3 discussed about the inventory procedure, and had the following summary.

Agree the TP R3-243872 (revised from R3-243234) Capture the following open issues in the chairman notes: In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included: - Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS. - Estimated Number of expected responses from devices, or Minimum number of required responses from devices - FFS the usage of these information - Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection. It is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

RAN3 discussed about running parallel inventory procedures, then an Inventory Session ID may become necessary, and the Inventory Session ID may be included in paging message.

Proposal 4: Whether Inventory Session ID may be included in paging message for the inventory procedure is up to RAN3 and SA2 discussion.

# Conclusion

This contribution elaborates on the ambient IoT paging procedure and discusses about the additional information which may be included in the paging message.

A summary of the proposals can be found below:

Proposal 1: Ambient IoT paging message may include the total number of access occasions for slotted-ALOHA based random access procedure.

Proposal 2: Whether frequency-domain related multiple access resource configuration may be included in the paging message is up to RAN1 further study.

Proposal 3: Reader may transmit access occasion announcement message.

Proposal 4: Whether Inventory Session ID may be included in paging message for the inventory procedure is up to RAN3 and SA2 discussion.

Observation 1: Keeping track of the access occasion announcement messages can be prone to errors when the tags are in a tight energy budget.

# References

- R2-2406271, TP for TR 38.769 update, Huawei	, 3GPP TSG-RAN WG2 Meeting #127, Maastricht, Netherlands, August 19-23, 2024

- R2-2407536, Discussion on Random Access procedure for Ambient IoT, Philips International B.V., 3GPP TSG-RAN WG2 Meeting #127, Maastricht, Netherlands, August 19-23, 2024

- R3-243808, Summary of Offline Discussion – CB: # AIoT2_CNRANinterface, Huawei (moderator), 3GPP TSG-RAN WG3 Meeting #124, Fukuoka, Japan, May 20-24, 2024