3GPP TSG-RAN WG2 Meeting # 127                	R2-2407536

Maastricht, The Netherlands, August 19th - 23rd, 2024

Source:	Philips International B.V.

Title:	Discussion on Random Access procedure for Ambient IoT

Agenda Item:	8.2.4

Document for:	Discussion and Decision

# Introduction

The latest RAN2 TP for TR 38.769 [1] captures the following random access procedure for Ambient IoT. This contribution discusses our views on some FFS(s) of this procedure.

### 6.2.4 A-IoT random access procedure A-IoT random access procedure is captured in this sub-clause, which is used for the Ambient IoT device(s) to access the network for data transmission. The A-IoT random access is triggered by the reader, including triggering the access for a single A-IoT device, group of A-IoT devices, or all A-IoT devices under the coverage of the reader. The slotted-ALOHA is the baseline for A-IoT random access procedure. When the A-IoT device is selected to respond in accordance to the sub-clause 6.2.3, the A-IoT device performs the following procedure: - Step 1: Random access type and access occasion/resource determination: - If the random access is contention-free access: - selects the indicated D2R occasion/resource; - skips the contention resolution in Step 2 and performs the Step 3 for data transmission. - If the random access is contention-based random access: - performs access occasion/resource determination/selection: [FFS]; - performs the Step 2 for contention resolution. - Step 2: Contention resolution of contention-based random access: - There are two candidate solutions being studied for the contention resolution, as below: Solution 1: A-IoT Msg1 without data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one random ID generated by the A-IoT device to the reader. NOTE 1: How the random ID is generated by the A-IoT device, e.g. randomly generated or generated based on the device ID, can be further discussed. Editor’s Note: FFS on size of the random ID. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. NOTE 2: The A-IoT Msg2 is used for contention resolution, since it is assumed that the size of random ID in A-IoT Msg1 should be sufficient for contention resolution purpose. The A-IoT devices, which select the same access occasion/resource, sending the same value of the random ID in A-IoT Msg1 will be sufficiently low probability case, with the sufficient value range of random ID. Solution 2: A-IoT Msg1 with data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data. Editor’s Note: FFS whether the random ID is additionally included in A-IoT Msg1 of solution 2. - A-IoT Msg2: The reader may respond with the successfully received [FFS information]. If the A-IoT device receives the A-IoT Msg2 including a [FFS information], which is the echo to the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. - Step 3: Data transmission: - After the A-IoT device considers the contention resolution as successful if the contention-based random access is used, or if the contention-free access is used, it may perform the upper layer data transmission with the reader, which can be the device ID and/or any other upper layer data, if any. Editor’s Note: In Step 3, it is understood that the subsequent R2D transmission after the D2R transmission does not need to be always sent. The usage/presence of this subsequent R2D transmission is to be further studied, e.g. it can be considered later in this study to handle the D2R transmission failure (due to various reasons). This is to be captured after RAN2 makes clear conclusions.

# Discussion

## Access occasion and resource determination and selection

Ambient IoT device has limited energy storage, and the communication is always triggered by the reader, i.e. initial triggering message, also known as paging message. Then based on the information included in the paging message, the ambient IoT device may determine the random access type and access occasion/resource:

- If the random access type is contention-free random access (CFRA), the ambient IoT device will select the access occasion/resource indicated in the paging message, skip the contention resolution step, and perform D2R data transmission;

- If the random access type is contention-based random access (CBRA), the ambient IoT device needs to determine and select the access occasion/resource first, which is FFS for RAN2 to discuss, and this is the first aspect we want to discuss in this contribution.

Depending on the information included in the paging message, there may be two approaches to determine and select the access occasion/resource for the ambient IoT device.

- Fully autonomous selection by the ambient IoT device: as agreed by RAN1 and RAN2, slotted-ALOHA is the baseline for ambient IoT random access procedure, therefore paging message may contain a parameter related to total number of access occasions of the slotted-ALOHA process, the ambient IoT device may use some mechanism such as randomly selecting a number from one to the total number of access occasions as its selected access occasion;

- Reader-assisted access occasion selection: in this approach, reader may include some additional information to assist the ambient IoT device to select its access occasion.

Observation 1: There may be two approaches for ambient IoT device to determine and select the access occasion/resource: fully autonomous selection by the ambient IoT device, and reader-assisted access occasion selection by the ambient IoT device.

Reader-assisted access occasion selection can be considered as an optimization of fully autonomous selection by the ambient IoT device. RAN2 may start with fully autonomous selection by the ambient IoT device as the baseline.

Proposal 1: RAN2 may consider fully autonomous selection by the ambient IoT device as the baseline for access occasion and resource selection.

## Access occasion announcement

According to [2], ambient IoT device has a large Sampling Frequency Offset (SFO) of ~10^5 PPM representing ~10% timing error due to its extreme low complexity and low power design. As a result, it is not quite feasible for the ambient IoT devices to align the time domain clock alone with the reader accurately.

Observation 2: Ambient IoT device has a large SFO, and Ambient IoT system is an asynchronous system.

Slotted-ALOHA has been agreed to be the baseline random access procedure, where it is required for every ambient IoT device in the network to select and identity an access occasion out of the total number of access occasions indicated in the paging message. Since ambient IoT device cannot maintain an accurate clock to keep track of the current access occasion, it needs a mechanism for all ambient IoT devices in the system to obtain the current access occasion number accurately. One approach may be using reader to transmit the access occasion announcement message, and the access occasion announcement message can help ambient IoT device to identify whether its access occasion arrives or not.

Proposal 2: Reader may transmit access occasion announcement message.

Proposal 3: The ambient IoT device may use the access occasion announcement message to identify whether its access occasion arrives or not.

The access occasion announcement message can be a standalone message. However, we also know ambient IoT device needs to be triggered by the paging message for communication. Then the access occasion announcement message may be embedded into the paging message as an information element to reduce the total number of signalling messages transmitted in the system.

Proposal 4: The access occasion announcement message may be included in the paging message.

# Conclusion

This contribution elaborates on the ambient IoT random access procedure for access occasion/resource determination/selection.

A summary of the observations and proposals can be found below:

Observation 1: There may be two approaches for ambient IoT device to determine and select the access occasion/resource: fully autonomous selection by the ambient IoT device, and reader-assisted access occasion selection by the ambient IoT device.

Observation 2: Ambient IoT device has a large SFO, and Ambient IoT system is an asynchronous system.

Proposal 1: RAN2 may consider fully autonomous selection by the ambient IoT device as the baseline for access occasion and resource selection.

Proposal 2: Reader may transmit access occasion announcement message.

Proposal 3: The ambient IoT device may use the access occasion announcement message to identify whether its access occasion arrives or not.

Proposal 4: The access occasion announcement message may be included in the paging message.

# References

- R2-2406271, TP for TR 38.769 update, Huawei, 3GPP TSG-RAN WG2 Meeting #127, Maastricht, Netherlands, August 19-23, 2024

- R1-2403954, On general aspects of physical layer design for Ambient IoT, Huawei and HiSilicon, 3GPP TSG-RAN WG1 Meeting #117, Fukuoka, Japan, May 20-24, 2024