3GPP TSG-RAN WG2 Meeting #127	R2-2407537

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.9.2

Source: 	Sequans Communications

Title: 	Support of Store & Forward

Document for:	Discussion and Decision

# Introduction

During previous meetings, RAN2 made the following assumptions and agreements:

RAN2 assumptions:

- S&F implies that at least the full eNB will be onboard

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

3.	The S&F satellite operation is common for NB-IoT and eMTC.

4.	The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In parallel, SA2 concluded its study with conclusions captured in ‎[1]. Two options are supported: full CN onboard the satellite and Split MME architecture, while “eNB onboard only” is not pursued.

In this contribution, we address some aspects related to support of Store & Forward from AS point of view.

# Discussion

## SA2 architecture background

SA2 has concluded on 2 different types of architecture, that we reproduce below for convenience.

### Split MME architecture

This solution works according to the following principles:

1)	In the Split MME architecture, HSS is on the ground.

2)	MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP.

3)	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

4)	For MO SMS, upon reception of the MO SMS the MME-onboard stores the MO-SMS and may immediately send the delivery report(i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC).

The impacts are as follows:

1)	When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information).

NOTE 1:	The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal.

2)	When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes:

a)	A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. […]

b)	Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN.

c)	Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

3)	How the UE process this information is up to UE implementation and during the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service.

4)	[…]

5)	When the wait timer has expired given to the UE in step 2, if the UE has not successfully attached to another PLMN and the UE finds the cell which broadcast the Satellite ID valid to re-attempt the attach procedure, the UE re-sends the Attach or TAU Request message.

6)	During the Attach or TAU procedure with the UE, the MME may also provide a list of Satellite IDs over which the UE may exchange the signalling and data, and a wait-timer that indicates to the UE the time it should wait before attempting signalling and data exchanges in those satellites.

7)	The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

NOTE 2:	The estimated delivery time is the estimated time to send data from the UE to Gateway.

8)	The core network can indicate to external SCS/AS whether UE is registered in S&F Mode and the estimated delivery time.

NOTE 3:	Whether any existing monitoring events or procedures can be used or enhanced to achieve above will be determined during normative phase.

NOTE 4:	SA WG2 will align further based on agreements in SA WG3 e.g. on security procedures and Attach/TAU procedures.

### Full CN onboard the satellite

This solution works according to the following principles:

-	The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc.

-	The implementation of the proxies and the interface between them is out of 3GPP scope.

-	The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the monitoring list.

NOTE 5:	MT traffic is delivered to the UE after it performs an ATTACH.

NOTE 6:	After mobility from S&F operation, MT traffic could be stuck in the ground proxy and will only be retrieved once the UE goes back to S&F operation.

For MT traffic, the UE attaches to a satellite and to allow delivery of MO traffic from the user or applications on the UE. A UE, based on implementation, could first wait for an indication from the user or from an application on the UE of pending MT traffic or could wait based on knowledge of when MT traffic may arrive, before performing the attach.

-	Depending on the deployment and implementation (i.e. outside the scope of 3GPP in this release), the HSSs on the satellites may be populated with subscription data either for only the UEs that may access satellite or all UEs that may access the satellite.

-	Depending on the deployment, the UE may have a USIM enhanced for IOPS, or a USIM dedicated to the satellite network.

NOTE 7:	The solution does not support the roaming architectures defined in TS 23.501 [2] or TS 23.401 [5].

With the following normative impacts:

-	Store and forward is only supported by EPS.

-	Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification.

NOTE 8:	The S&F monitoring list may assist the UE in retrieving MT data.

-	The UE needs to be aware that a satellite supports S&F mode.

NOTE 9:	How the UE is aware that a satellite supports S&F mode of operation depends on RAN.

-	A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

## RAN2 impact

We can derive some RAN2 impact from above SA2 proposed architectures.

### S&F mode

As per 1), the NW should inform about whether S&F Satellite operation is applied. In our understanding, this corresponds to the fact that the satellite would operate in “S&F mode” rather than in “normal mode”. We assume this is supposed to match with the availability of a feeder link, i.e. the satellite operates in “S&F mode” when no feeder link is present while it operates in normal mode when the feeder link is available. However, there is no need to make such linkage; a satellite could e.g. always operate in S&F mode independently of feeder link presence (possibly because it can’t transmit all the data stored). That is to say, we believe it makes more sense to consider an indication that the satellite operates in S&F mode, rather than an indication that the feeder link is available.

Observation 1: It is more generic to use a “S&F mode” indication, rather than a “feeder link available” indication

It is also important to understand the rationale of this indication. This is not detailed in SA2 conclusions. In our understanding, this indication could be used e.g.:

- To prevent not supported operations

  - For instance, a R19 UE that would not support S&F mode NAS capability should likely not try to perform Attach/TAU as it would be rejected, since this clearly needs some NAS support. We expect this to be specified in NAS specifications.

- For NW selection

  - As S&F mode is a degraded form or service, this might be considered while performing PLMN selection

- For application transport profile selection

  - E.g. TCP should be avoided in S&F mode, instead UDP/SMS etc should be used

RAN2 agreed to have an indication in SIB, while assuming no NAS indication is needed. In our understanding a SIB indication can be as well NAS or AS, or even both. Typically NAS only broadcasted parameters are just forwarded to upper layers (e.g. tracking area information).

Observation 2: The “S&F mode” indication is expected to be used by NAS/application, and should be forwarded to upper layers

It is not clear there would be any AS impact related to this indication. One question is whether UE not supporting S&F mode should be barred when the eNB operates in S&F mode, but we don’t think it should necessarily be the case (as explained later). In any case, even if forwarded to upper layers, the “S&F mode” indication might still be used by AS.

Proposal 1: The eNB broadcast a “S&F mode” indication (FFS details), that is forwarded to upper layers

The NAS behavior will be different if using split MME architecture or Full CN. Hence, the S&F mode could further indicate the supported architecture. This would be useful if e.g. separate UE NAS capabilities are defined for both architectures (otherwise this could be directly indicated by NAS signaling). In our view, this is more up to SA2 discussions, and they will notify RAN2 if this is required.

Similarly, if e.g. multiple PLMNs/GW would need to be supported, the indication would need to be per PLMN. But this would be up to SA2 to decide.

### S&F mode switching

A straightforward approach for the “S&F mode” indication would be to broadcast one bit. However, this has some drawbacks.

In general, whenever the feeder link is available, it would make sense for the eNB to indicate “normal mode” (i.e. to not indicate “S&F mode”), given the service provided would then be “Real Time” and much more efficient. In normal mode, the eNB could offer services to UEs not supporting S&F.

We assume this is also SA2 understanding and request, as they indicate: “When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied”. However this will introduce some additional complexity, hence we propose to confirm this understanding.

Proposal 2: Support switching between “S&F mode” and “normal mode”

Depending of feeder link availability, that would lead to switch continuously between “S&F mode” and “normal mode”. Some UEs might see always “S&F mode”, some “normal mode”, and some might always see the switch as the satellite crosses the sky.

As a result, if S&F operation mode switching is supported, we believe the switching should not trigger SI change notification. This would be both costly for NW resources, and for UE power consumption.

Observation 3: Toggling a “S&F mode” bit would yield signaling overhead/UE power consumption

Instead of broadcasting a “S&F mode” bit, an alternative would be to indicate the S&F mode time range (e.g. start/stop times of as UTC time). This avoids SI change overhead. This can also be used for neighbor cells.

Proposal 3: Consider broadcasting “S&F mode” time range instead of a “S&F mode” bit, to avoid SI change notification

### Satellite ID

As per 2) c) and NAS will use optional satellite lists. RAN2 already broadcasts a Satellite ID, which is used for indexing the satellite assistance information for serving/neighbour cells.

#### –	SatelliteId

The IE SatelliteId is used to identify the satellite assistance information of the serving or neighbour satellites.

SatelliteId information element

-- ASN1START

SatelliteId-r18 ::= INTEGER (0..255)

-- ASN1STOP

It seems straightforward that NAS reuses the existing Satellite ID, i.e. there is no need to introduce new Satellite IDs for S&F. It also enables to reuse AS features such as discontinuous coverage which indicates when a given satellite will provide coverage.

Proposal 4: NAS Satellite IDs corresponds to existing broadcasted SatelliteIds (no need for new parameter)

### Handling of UEs not supporting S&F

An eNB operating in S&F mode provide a degraded service compared to normal mode. In general, it seems difficult for a UE not supporting S&F to access.

However, there are some scenarios/use cases where a S&F system might accommodate even UEs not supporting S&F. For instance, it seems SMS service could be supported by Full CN architecture. Similarly, if an operator deploys a specific delay tolerant service based on UDP, the full CN architecture could support that.

Observation 4: In some scenarios, UEs not supporting S&F could still be accommodated by S&F eNB

Hence, we believe it should be up to NW deployment to allow or disallow access of UEs not supporting S&F.

Proposal 5: It should be up to NW deployment to allow/disallow access of UEs not supporting S&F

UEs not supporting S&F are twofold:

- Legacy UEs

- R19 UEs not supporting S&F

Regarding legacy UEs, as reminded during last meeting, they can be barred (if desired) by legacy cellBarred and cellBarredNTN. A new barring indication would then be required for R19 UEs.

Regarding R19 UEs not supporting S&F, they could be considered barred by using the “S&F mode” indication. I.e., whenever the eNB operates in “S&F mode”, such UEs would be barred. To enable above flexibility, this should be under NW control, e.g. there should be an indication on whether UEs not supporting S&F are barred or not during “S&F mode” operation.

Proposal 6: Add an indication to control access of R19 UEs not supporting S&F operation

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: It is more generic to use a “S&F mode” indication, rather than a “feeder link available” indication

Observation 2: The “S&F mode” indication is expected to be used by NAS/application, and should be forwarded to upper layers

Proposal 1: The eNB broadcast a “S&F mode” indication (FFS details), that is forwarded to upper layers

Proposal 2: Support switching between “S&F mode” and “normal mode”

Observation 3: Toggling a “S&F mode” bit would yield signaling overhead/UE power consumption

Proposal 3: Consider broadcasting “S&F mode” time range instead of a “S&F mode” bit, to avoid SI change notification

Proposal 4: NAS Satellite IDs corresponds to existing broadcasted SatelliteIds (no need for new parameter)

Observation 4: In some scenarios, UEs not supporting S&F could still be accommodated by S&F eNB

Proposal 5: It should be up to NW deployment to allow/disallow access of UEs not supporting S&F

Proposal 6: Add an indication to control access of R19 UEs not supporting S&F operation

# References

- 3GPP TR 23.700-29 V19.0.0,	 “Study on integration of satellite components in the 5G architecture Phase 3”