3GPP TSG-RAN WG2 Meeting #127	R2-2407539

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	 8.7.4

Source: Rakuten Mobile Inc

Title:  Discussion on UL related Scheduling Enhancements for XR

Document for: Discussion

# Introduction

In this contribution, we discuss enhancements related to uplink scheduling for XR traffic, focusing on the use of delay-sensitive information and improved DSR mechanisms. The enhancements aim to address issues of data with low remaining time being delayed due to data from other LCHs with no delay-critical data, while also considering the impact on UE complexity.

Objective

# Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. LCP implementation complexity need to be taken into account when evaluating solutions

RAN2#125Bis Agreements

Agreement : RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: Alternative 1: Enhance LCP restrictions/LCH selection. Alternative 2: Enhance LCH prioritization. RAN2 should consider potential impact on traffic from SRBs

### 2.1 Scheduling Feedback Mechanism.

Observation 1: The current DSR mechanism doesn't include a feedback component that tells the UE how delay-critical data is handled after it's reported. Without this feedback, managing high-priority and delay-sensitive data becomes uncertain, potentially leading to inefficient transmission strategies, increased packet loss, or delays.

If the gNB could provide feedback on which SDUs are scheduled for transmission and which ones might expire, the UE could prioritize its transmissions more effectively and avoid trying to resend SDUs that likely won't be delivered in time.

Proposal 1: Introduce a feedback mechanism after DSR and scheduling that allows the network to send back information to the UE regarding the scheduling and expiration status of data reported in DSRs.

### 2.2 Delay Status Report (DSR) related enhancements

RAN2#125Bis Agreements

Agreement : RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data Agreements from RAN2#126: - Enhance DSR to report multiple pairs of remaining time and buffer size for the LCG. - FFS: - Whether DSR triggering is impacted. - Whether PDU set importance needs to be included.

Observation 2: The current DSR mechanism does not adequately differentiate between the urgency and priority of various data types within the buffer.

Multiple pairs of remaining time and buffer size reporting for each LCG can provide more granularity.

Including importance levels for different types of XR data allows the gNB to make more informed scheduling decisions.

Example DSR MAC Control Element

| Field name | Existing MAC DSR (TS 38.321 rel-18) | Proposed Enhancements |
| --- | --- | --- |
| LCGi | Indicates the presence of delay information for each LCG. | Expanded to include flags specifying whether the data is high-priority, low-priority, or both. |
| Remaining Time | Reports the shortest remaining discard time for an LCG. | Split into two fields: Remaining Time F (for high-priority data) and Remaining Time S (for standard-priority data). |
| Buffer Size | Reports the total buffer size of delay-critical UL data for an LCG. | Split into two fields: Buffer Size F (for high-priority data) and Buffer Size S (for standard-priority data). |
| Priority Indicator | None | Introduces an I-bit to indicate the importance level of the reported data (F or S). |
| PDU Set Information | None | Includes PDU Set ID, remaining time, buffer size, and priority level. |

Proposal 2: Revise the DSR MAC Control Element to introduce multi-tiered reporting for each LCG, detailing the urgency (Delay criticality) and buffer sizes for different data priority levels.

Proposal 3: Introduce a Priority Indicator Field in the DSR MAC CE to indicate the priority of the data.

Proposal 4: Modify the 'Remaining Time' field to include multiple entries per LCG, each corresponding to different priority levels or individual PDU sets.

Proposal 5:Modify LCGi field to reflect the complexity of the data reported, indicating whether the DSR includes high-priority, low-priority, or both types of data.

### 2.3 Impact on DSR Triggering

The introduction of multiple pairs of remaining time and buffer size in DSRs will likely impact how and when DSRs are triggered. Enhanced granularity in reporting will necessitate more frequent updates, which can increase signaling overhead and affect network performance.

Observation 3: Current DSR trigger conditions may not be sufficient to handle the enhanced reporting format. New thresholds for buffer occupancy and remaining time might need to be established to ensure that DSRs are sent at the appropriate times.

Observation 4: Dynamic adjustment of DSR intervals based on buffer occupancy and priority can help manage uplink resources efficiently.

Proposal 7: Define new MAC Control Elements or RRC signaling procedures for configuring buffer occupancy thresholds with corrosponding Trigger intervals for DSR triggering.

Proposal 8: RAN2 to discuss event driven DSR Trigger, for example arrival of high priority Delay critical data or buffer status change.

# Conclusion

In this contribution, we discuss two enhancements for XR uplink and proposed following observations and Proposals.

Observation 1: The current DSR mechanism doesn't include a feedback component that tells the UE how delay-critical data is handled after it's reported. Without this feedback, managing high-priority and delay-sensitive data becomes uncertain, potentially leading to inefficient transmission strategies, increased packet loss, or delays.

If the gNB could provide feedback on which SDUs are scheduled for transmission and which ones might expire, the UE could prioritize its transmissions more effectively and avoid trying to resend SDUs that likely won't be delivered in time.

Proposal 1: Introduce a feedback mechanism after DSR and scheduling that allows the network to send back information to the UE regarding the scheduling and expiration status of data reported in DSRs.

Observation 2: The current DSR mechanism does not adequately differentiate between the urgency and priority of various data types within the buffer. Multiple pairs of remaining time and buffer size reporting for each LCG can provide more granularity.Including importance levels for different types of XR data allows the gNB to make more informed scheduling decisions.

Proposal 2: Revise the DSR MAC Control Element to introduce multi-tiered reporting for each LCG, detailing the urgency (Delay criticality) and buffer sizes for different data priority levels.

Proposal 3: Introduce a Priority Indicator Field in the DSR MAC CE to indicate the priority of the data.

Proposal 4: Modify the 'Remaining Time' field to include multiple entries per LCG, each corresponding to different priority levels or individual PDU sets.

Proposal 5:Modify LCGi field to reflect the complexity of the data reported, indicating whether the DSR includes high-priority, low-priority, or both types of data.

Observation 3: Current DSR trigger conditions may not be sufficient to handle the enhanced reporting format. New thresholds for buffer occupancy and remaining time might need to be established to ensure that DSRs are sent at the appropriate times.

Observation 4: Dynamic adjustment of DSR intervals based on buffer occupancy and priority can help manage uplink resources efficiently.

Proposal 7: Define new MAC Control Elements or RRC signaling procedures for configuring buffer occupancy thresholds with corrosponding Trigger intervals for DSR triggering.

Proposal 8: RAN2 to discuss event driven DSR Trigger, for example arrival of high priority Delay critical data or buffer status change.

# References

TS 38.321 v18.0.0

### [2] R2_125bis R18 MBS, R18 QoE and R19 XR session notes (Dawid) 2024-04-18 1300.docx

### [3] R2-2403690 UL related Scheduling Enhancements for XR Rakuten Mobile, Inc discussion Rel-18

[4] R2-2402325 Discussion on scheduling enhancements for XR OPPO discussion

[5] R2-2403591 Discussion on UL scheduling enhancements MediaTek Inc. discussion Rel-19

[6] 38.321 Rel-18

[7] R2-24xxxxx_[POST126][032][AIML PHY] LCM (Intel_Samsung)_Phase 2_v30_Rapp.docx

[8] RAN2# 126 meeting notes.