3GPP TSG RAN WG2 Meeting #127                                                             R2-2407540

# Maastricht, Netherlands, August 19-23, 2024

Agenda item:		8.5.3

Source:			III

Title:		On-demand SIB1 for Idle/Inactive mode UEs

Document for:	 	Discussion

- Introduction

A Rel-19 work item on network energy saving enhancements was approved in RAN#102 and revised in RAN#103 [1], whose objectives are listed as follows.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

For the second issue, i.e., on-demand SIB1 for UEs in idle/inactive mode, a number of agreements [2] were made in previous RAN2#126, including:

Agreements: Further clarification 1. Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a (Case 2 in RAN1) with multiple Cells A and/or NES Cells: - More than one Cell A may provide configuration for the same NES cell. - The same Cell A may assist more than one NES Cells. 2. RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3. Information of WUS configuration 3. Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell. 4. For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline. Transmission of WUS configuration 5. Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB. Triggering condition of WUS transmission 6. If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. 7. After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell. UE request SIB1 to perform initial access in NES cell 8. RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell. Legacy UE impact 9. NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism) 10. How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that). Barring for R19 NES UE 11. If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell. Paging and SIB1 update in NES cell 12. After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc. RAR monitoring for MSG1 based OD-SIB1 REQ 13. RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline. UE behaviour on RACH failure of OD-SIB1 REQ 14. As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. SIB1 acquisition upon SIB change notification in NES cell 15. Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell. Scenario 3 (case 1 in RAN1) 16. RAN2 to wait for RAN1’s progress whether to support scenario 3.

This contribution provides some input regarding identification of an NES cell (part of information of WUS configuration) and transmission of WUS configuration.

- Discussion

Identification of an NES cell

In section 6.2 of [3], the structure of MIB (aka PBCH payload) is illustrated, where only 1 bit (spare) is reserved. Therefore, it is very challenging to utilize the bit as the identification of an NES cell without any impact on legacy systems.

MIB ::= SEQUENCE {

systemFrameNumber BIT STRING (SIZE (6)), 6bit

subCarrierSpacingCommon ENUMERATED {scs15or60, scs30or120}, 1bit

ssb-SubcarrierOffset INTEGER (0..15), 4bit

dmrs-TypeA-Position ENUMERATED {pos2, pos3}, 1bit

pdcch-ConfigSIB1 INTEGER (0..255), 8bit

cellBarred ENUMERATED {barred, notBarred}, 1bit

intraFreqReselection ENUMERATED {allowed, notAllowed}, 1bit

spare BIT STRING (SIZE (1)), 1bit

}

In [4,5,6], it is observed that utilizing cell identification of NES cells in the UL WUS configuration might bring the benefit of effective use of dedicated PRACH resource for SIB1 request.

Observation 1. Identification of NES cell with on-demand SIB1 by WUS configuration introduces less impact on legacy systems than by PBCH payload of NES cell.

Proposal 1. Identification of NES cell with on-demand SIB1 by WUS configuration needs to be further studied.

Transmission of WUS configuration

In the section 5.2 of [3], it is stated that the physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2,976 bits. Therefore, if Cell A’s existing SIB is used to configure on-demand SIB1 related configuration for neighbor NES cells, a size problem might arise, especially when the same Cell A assists more than one NES Cells.

Observation 2. If Cell A’s existing SIB is used to configure on-demand SIB1 related configuration for neighbor NES cells, a size problem might arise, especially when the same Cell A assists more than one NES Cells.

Proposal 2. When Cell A is used to configure on-demand SIB1 related configuration for neighbor NES cells, new SIB is preferable and needs to be further studied.

- Conclusion

In this contribution, we have the following observations and proposals:

Observation 1. Identification of NES cell with on-demand SIB1 by WUS configuration introduces less impact on legacy systems than by PBCH payload of NES cell.

Proposal 1. Identification of NES cell with on-demand SIB1 by WUS configuration needs to be further studied.

Observation 2. If Cell A’s existing SIB is used to configure on-demand SIB1 related configuration for neighbor NES cells, a size problem might arise, especially when the same Cell A assists more than one NES Cells.

Proposal 2. When Cell A is used to configure on-demand SIB1 related configuration for neighbor NES cells, new SIB is preferable and needs to be further studied.

References

[1] RP-240170 Revised WID for Enhancements of network energy savings for NR.

[2] Draft_R2_126_Meeting_Report_v2.

[3] 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[4] R2-2403600 On-demand SIB1 for Idle/Inactive mode UEs, Nokia, Nokia Shanghai Bell.

[5] R2-2402661 On-demand SIB1 for Idle/Inactive mode UEs, III.

[6] R2-2405640 On-demand SIB1 for Idle/Inactive mode UEs, III.