3GPP TSG-RAN WG2 Meeting #127											        R2-2407548

Maastricht, 19-23 Aug. 2024

Agenda item:			8.8.5

Title:						Considerations on RACH-less handover for regenerative payload

Source:					Continental Automotive

Document for:		Discussion and Decision

# Introduction

During RAN2#126 [1] the following agreements were made:

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In this paper, we share our views on RACH-less handover for regenerative payload in NTN.

# 2. Discussion

In NTN, UEs may support mobility between gNBs operating with NTN payloads in different orbits (e.g., GSO, NGSO at different altitudes). However, even satellites belonging to the same constellation are not permanently operating on identical altitudes and may also need to slightly adjust their orbital position from time to time. In scenarios with regenerative payloads, such altitude differences between neighbouring gNBs can impact RACH-less handover performance, since there may be significant differences between source and target NTA values, respectively.

Proposal 1: RAN2 should further enhance RACH-less handover for scenarios with regenerative payloads.

There are two options for configuring smooth RACH-less handovers: pre-allocated or dynamic UL grants. However, in both cases knowledge of timing-related information is crucial for the network. Otherwise, resources may be wasted or UEs scheduled unsuccessfully.

Proposal 2: In scenarios with earth-moving cells, timing-related information, e.g., UE timing advance, should be exchanged between gNBs for configuring UL grant resources.

# Conclusions

The following proposals have been made in this contribution:

Proposal 1: RAN2 should further enhance RACH-less handover for scenarios with regenerative payloads.

Proposal 2: In scenarios with earth-moving cells, timing-related information, e.g., UE timing advance, should be exchanged between gNBs for configuring UL grant resources.

# References

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#126, Fukuoka, Japan, May 20th-24th, 2024.