3GPP TSG-RAN WG2 #127	R2-2407550

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda Item:	8.8.5

Source:	Ericsson

Title:	Regenerative payload

Document for:	Discussion, Decision

# 1	Introduction

The work item [1] has the objective:

- Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

- Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

And last meeting RAN2 made the following agreements:

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In this contribution, we discuss regenerative payload and possible stage 2 updates.

# 2	Discussion

RAN3 have endorsed a baseline CR [2] for stage 2, for your convenience we have included the full tdoc [2] in the appendix.

RAN3 has endorsed a baseline CR for 38.300 that includes almost all changes needed.

Summary of RAN3 changes to stage 2:

In 3.2 Definitions we have only changes to “NTN payload” definition.

RAN3 adds a regenerative figure in 16.14.1 and clarifies the existing figure is for transparent payload.

RAN3 do not touch 16.14.2.1, it may be beneficial if RAN2 clarifies that the figure is for transparent architecture.

The timing and synchronization in 38.300 section 16.14.2.1 is out of RAN3s scope.

RAN3 made modifications to feeder link switch over in 16.14.4, and they indicate that they will update 16.14.6 but have not concluded on the updates there yet.

Then for Appendix B.4, RAN3 clarified that the figure is an example of transparent payload.

Clarify that the figure in 16.14.2.1 is for transparent payload.

Consider the text proposal in section 4.

There are many possible configurations of common TA and Kmac in a regenerative payload, there is no need to capture this in the specification.

Specific configurations to regenerative architecture of common TA and Kmac are not captured in Stage 2.

# 4	Stage 2 Text proposal

#### 16.14.2.1	Scheduling and Timing

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset (see clause 4.2 of TS 38.213 [38]).

To accommodate the propagation delay in NTNs, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two offsets Koffset and kmac:

-	Common TA is a configured timing offset that is equal to the RTT between the RP and the NTN payload.

-	Koffset is a configured scheduling offset that needs to be larger or equal to the sum of the service link RTT and the Common TA.

-	kmac is a configured offset that is approximately equal to the RTT between the RP and the gNB.

The scheduling offset Koffset is used to allow the UE sufficient processing time between a downlink reception and an uplink transmission, see TS 38.213 [38].

The offset kmac is used to delay the application of a downlink configuration indicated by a MAC CE command on PDSCH, see TS 38.213 [38], and in estimation of UE-gNB RTT, see TS 38.321 [6]. It may be provided by the network when downlink and uplink frame timing are not aligned at gNB. The kmac is also used in the random access procedure, to determine the start time of RAR window/MsgB window after a Msg1/MsgA transmission (see TS 38.213 [38]).

The Service link RTT, Feeder link RTT, RP, Common TA, kmac and TTA (see clause 16.14.2.2) are illustrated in Figure 16.14.2.1-1.

Figure 16.14.2.1-1: Illustration of timing relationship (for collocated gNB and NTN Gateway with transparent payload)

The network may configure the HARQ operation as follows:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process (as specified in clause 5.3.2.2 and clause 5.7 of TS 38.321 [6]). Disabling HARQ feedback allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

-	For uplink, HARQ mode (i.e. HARQ mode A or HARQ mode B) can be configured per HARQ process (as specified in clause 5.4.3.1 and clause 5.7 of TS 38.321 [6]). HARQ mode B allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

NOTE:	For the HARQ processes configured with HARQ feedback enabled/disabled, it is up to network implementation to ensure a proper configuration of HARQ feedback (e.g., either all enabled or all disabled) for HARQ processes used by an SPS configuration. For the HARQ processes configured with HARQ mode, it is up to network implementation to ensure a proper configuration of HARQ mode (e.g., either all HARQ mode A or all HARQ mode B) for HARQ processes used by a CG configuration.

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN3 has endorsed a baseline CR for 38.300 that includes almost all changes needed.

Observation 2	The timing and synchronization in 38.300 section 16.14.2.1 is out of RAN3s scope.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify that the figure in 16.14.2.1 is for transparent payload.

Proposal 2	Consider the text proposal in section 4.

Proposal 3	Specific configurations to regenerative architecture of common TA and Kmac are not captured in Stage 2.

# 6	References

RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#104, Jun. 2024.

R3-243949, “Support for Regenerative Payload in NR NTN”, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, RAN3#124, May 2024.