3GPP TSG-RAN WG2 #127	R2-2407551

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda Item:	8.8.2

Source:	Ericsson

Title:	DL coverage enhancements

Document for:	Discussion/Decision

# 1	Introduction

The WID was updated in [1]:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

  - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any)

  - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements.

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

  - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Last meeting RAN2 made these agreements:

Agreements;

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

- RAN2 assumes that both EFC and EMC are supported

RAN2 has started to study on RAN2 aspects of DL coverage enhancement. To progress the study, RAN2 has identified questions to RAN1 for aspects where the input is required.

Related to this, RAN2 would like to request RAN1 to provide feedback on the following questions.

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?

Question2 : Can RAN1 provide the information on whether/how the solution RAN1 is investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive?

Question3 :  Can RAN1 provide the feedback on whether UL beam hopping is also being studied in RAN1 (and whether this is separate from DL beam hopping)?

Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.

Question5 : Can RAN1 provide the feedback on whether the beam status in different beams (visible to the UE) of one cell are the same or can be different in any given time, i.e.. the beam status is cell specific or beam specific?

And sent an LS to RAN1 [2] with these questions:

Further the Agenda has the following instructions for this agenda item:

Contributions should focus on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

Here we discuss further details of this WI objective.

# 2	Discussion

This objective shall be studied and specified if beneficial.

DL coverage enhancements shall only be specified if studies show it to be beneficial.

The area to evaluate is Satellite beam power sharing and feeder link rate limits, including link and system level enhancements, and the R18 network energy saving techniques are the baseline.

This will require RAN1 to define KPIs and potential effects on the coverage.

We believe there are no RAN2 aspects of DL coverage enhancements that are not dependent on the evaluation in RAN1.

For example, cell/beam level on/off mechanism for solving the EIRP reduction issue:

  - On/off is just endpoints of a varying power per cell, what about power sharing (as stated in the WID) to allow each cell to send SIBs and serve some traffic (using part of the cell resources).

  - There needs to be an evaluation of what the benefits are and whether coverage can be provided by some hierarchical scheme when a beam is off or some time sharing to provide accessibility

  - Comparison to Network Energy Savings is needed

RAN1 has already started extensive evaluations on these.

Therefore, RAN2 needs to wait the results in RAN1.

RAN2 wait for RAN1 conclusions on downlink coverage enhancements.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	DL coverage enhancements shall only be specified if studies show it to be beneficial.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 wait for RAN1 conclusions on downlink coverage enhancements.

# References

RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#104, Jun. 2024.

R2-2405767, “LS on DL coverage enhancements”, Contact: CMCC, To:RAN1, May. 2024.