3GPP TSG-RAN WG2 #127 	R2-2407552

# Maastricht, Netherlands, 19th - 23rd Aug 2024

Agenda Item:	8.9.3

Source:	Ericsson

Title:	UL capacity enhancements objectives for IoT NTN

Document for:	Discussion

# 1	Introduction

In RAN#104, the WID for “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things” was updated [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2#126, the following agreements

1 RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

In this document, we discuss how the preamble-less EDT transmission may work.

# 2	Discussion

## 2.1	Preamble-less EDT

### 2.1.1	Terminology

During RAN2#126, a significant portion of the discussion was centred around the decision of whether the new feature should adopt EDT or PUR as its baseline. Several companies argued that this is merely a matter of nomenclature. However, we think that EDT and PUR are intrinsically distinct, given their respective original purposes. Specifically, PUR is designed for periodic traffic and requires dedicated configuration. Moreover, PUR is associated with a suite of predefined procedures (e.g., Layer 1 Acknowledgement) and UE behaviours (e.g., PUR-responseWindowTimer) that may not be applicable in the NTN context.

As expressed in the WID, RAN2 should take EDT as the baseline for further enhancements, considering the contention of Msg3 due to the absence Msg1 and Msg2, primarily addressing MO traffic. To avoid further confusion and debates over terminology, we suggest that the feature be designated as Preamble-less Early Data Transmission (pl-EDT).

The Rel-19 enhancement of EDT without Msg1 and Msg2 transmission is referred to as preamble-less EDT.

### 2.1.2	Benefits of preamble-less EDT

The justification for the objective is UL capacity enhancements.

The first (RAN1) part of the objective aims at enabling multiplexing multiple UEs on NPUSCH format 1 and NPRACH. This hints at 1) improved NPUSCH multiplexing and 2) improved NPRACH collisions.

The second part of the objective require RAN2 to “Study and specify, if beneficial” the preamble-less EDT.

Here we do a quick study of the capacity aspects of Contention Resolution Diversity Slotted Aloha (CRDSA), one of the proposed methods, and compare it to the existing 4-step RA from a capacity point of view.

CRDSA

For CRDSA we use similar models as described in R2-2405202 [3].

We assume Poisson traffic (for CDRSA and 4-step RA) and a frame N of 100 slots, and each UE sends 2 or 3 replicas within the frame.

We assume NPUSCH collisions can always be resolved if one of the packets is correctly decoded on a different slot.

We ignore overhead from the CRDSA scheme (that is the header info on which other slots in the window where the UE transmitted the same packet, this overhead would be 7 bits with 2 replicas and 14 bits with 3 replicas) and assume 4-step msg3 and CRDSA use the same packet size and resource size (same frequency and time resources and same number of repetitions, but of course the 4-step has the overhead of the preamble resources that we discuss later).

Further we assume none of the messages can be received when there is a NPUSCH collision (no power imbalances nor power capture [4]), and that the message is always correctly received if there is no collision.

4-step RA

For 4-step RA, we assume NB-IoT with 48 subcarriers (that is sometimes termed preambles, but we also look at what happens when there is only one preamble (pure slotted Aloha)) and if preambles collide, then the msg3 will not be correctly received. With these simple assumptions the number of users attempting access each RA opportunity is Poisson distributed {P[k users access] = (lambda*T)^k*exp(-lambda*T)/k!} and then probability of collisions is easily calculated. NOTE that a Poisson distributed variable that is randomly divided in several other variables are also Poisson distributed with a lower average.

Evaluation

With these models, 4-step RA collisions and resource usage can be easily calculated, while for CRDSA it is a bit more complex though it is possible to calculate results it is way easier to just simulate. In the Appendix we have described how the simulation is done.

We present results for two cases of CRDSA with frame size N=100:

- CRDSA 2 replicas, and

- CRDSA 3 replicas.

The CRDSA results corresponds almost exactly to the results shown in R2-2405202 [3].

The CRDSA scheme resource usage is always 100 message occasions regardless of the load in the system.

For 4-step, we show three cases:

- 1 subcarrier and 100 RA opportunities

  - This corresponds approximately to the Slotted Aloha scheme in R2-2405202 [3], however resource usage for the preamble opportunities is always there.

- 48 subcarrier and 10 RA opportunities

  - This uses only 10 RA opportunities, which means the load on each opportunity will be ten times higher, but adding the 48 subcarriers the load on each subcarrier will be much lower than for A.

- 48 subcarrier and 100 RA opportunities

  - This is just to show the effect when adding subcarriers.

100 RA opportunities means that there are 100 RA opportunities within the frame of N=100 slots to transmit preambles. This means the resource usage will always be 100 preamble transmission occasions plus one message occasion per detected preamble.

10 RA opportunities means that there are 10 RA opportunities within the frame of N=100 slots to transmit preambles, thus for this case the load per opportunity will be ten times higher (and thus collision rate is higher unless we add a higher number of subcarriers). This means the resource usage will always be 10 preamble transmission occasions plus one message occasion per detected preamble.

For resource usage (figure 3), we assume one preamble transmission occasion uses the same amount of resources as one message transmission. The CRDSA always uses 100 message transmission occasions in one frame N=100, while for 4-step it will depend on how many preambles that are detected.

In Figure 1, we can see that the packet loss ratio (PLR) versus the load – as we only model collisions as lost packets, PLR corresponds to the collisions. A pure slotted aloha has much worse performance compared to the CRDSA schemes.

In the 4-step RA scheme, we can see that adding more subcarriers (preambles) decreases the collisions substantially and that decreasing the RA opportunities increases the collisions.

A well configured 4-step RA scheme has substantially lower packet loss rate than CRDSA schemes at high load.

Figure 1: Packet loss ratio versus load

Figure 2: Throughput versus load

In Figure 2 we have the throughput versus load. As can be seen the CRDSA 3 replicas scheme has an sweet spot somewhere between 0.35 and 0.65 packets per slot compared to the reasonable configuration for 4-step RA with 10 RAO and 48 subcarriers. However, the CRDSA schemes breaks down when load increases, which the 4-step RA will do too but at much higher load.

A well configured 4-step RA scheme has substantially higher throughput than CRDSA schemes at high load.

Figure 3: 4-step RA resource usage versus load versus load

In Figure 3 we have plotted the resource usage for the different 4-step RA schemes when assuming that one RAO uses the same number of resources as one message transmission, and then each detected preamble (collision or not) requires one more packet transmission. The CRDSA scheme always uses 100 packet transmissions for a frame of N=100.

As can be seen the reasonable 4-step RA configuration (10 RAO, 48 subcarrier) uses much lower number of packet transmissions at low loads, and at really high load it gets close to 100.

A well configured 4-step RA scheme has lower resource usage that CRDSA schemes for all load levels, but especially at low load.

A correctly configured 4-step RA scheme uses substantially lower uplink resources that can be used to support other users.

The objective of the preamble-less EDT is to improve capacity but non of the proposed schemes are doing that.

As there are no capacity benefits for the preamble-less EDT enhancements compared to 4-step RA, there is no reason to make such enhancements.

### 2.1.3	Uplink resource allocation

Preamble-less EDT transmission without Msg1/Msg2 is somewhat similar to NR’s 2-step random access. In this procedure, the cell specific PRACH and PUSCH resource parameters for the transmission of MsgA are configured by MsgA-ConfigCommon included in SIB1, and the network can provide separate PRACH and PUSCH configurations for a specific feature such as small data transmission (SDT, which is similar to EDT in LTE) in FeatureCombinationPreambles. For preamble-less EDT, cell specific common PUSCH resources for Msg3 transmission can be configured via system information. Signalling details such as which SIB to use can be further studied.

In case preamble-less EDT is agreed beneficial, System Information is used to provide preamble-less EDT cell specific PUSCH resources for Msg3 transmission. FFS on signalling details.

### 2.2.4	Configuration of uplink resources

LTE-M and NB-IoT devices support coverage enhancement (CE) to compensate the simplifications in their radio interfaces and improve connectivity in challenging coverage conditions. The most significant CE technique is the support of repetitions.

In EDT, PRACH parameters are configured per CE level in edt-PRACH-ParametersListCE. The UE selects the CE level for the preamble transmission (Msg1) based on network configuration. In addition, transport block size (TBS) and number of repetitions for Msg3 transmission on PUSCH depend on the PRACH CE level.

In legacy EDT, the number of repetitions and the largest TBS for Msg3 PUSCH depend on the CE level.

In legacy EDT, RSRP thresholds are configured for the UE to select the CE level.

For preamble-less EDT, the support of CE is also necessary. Similar to legacy, we think the network should provide cell specific common PUSCH resources for Msg3 EDT transmission per CE level. For each CE level, PUSCH resources can be associated with a certain number of repetitions, largest allowed TBS and a RSRP selection threshold.

In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold and largest TBS for Msg3 transmission.

### 2.2.5	UL time alignment for Msg3 transmission

For uplink timing handling, RAN2 already agreed that the UE needs to verify/update the uplink synchronization (e.g., get GNSS fix, precompensate TA) just before sending Msg3. In our view, this is sufficient to ensure the required uplink synchronization for Msg3 transmission and there is no need to use TAT. Note that the UE may not receive any Timing Advance Command (in the current serving cell) before sending Msg3 thus it has no trigger to start TAT and therefore it is meaningless to introduce TAT for Msg3 transmission without Msg1 and Msg2.

The UE will verify uplink synchronization before sending Msg3.

The UE may not receive any Timing Advance Command before sending Msg3.

In case preamble-less EDT is agreed beneficial, Preamble-less EDT does not require a running TAT for Msg3 transmission.

### 2.2.6	Contention mechanism: collision detection and backoff

Msg3 transmission may fail due to collision or bad reception quality at the network side. In NR 2-step random access, when the transmission of MsgA fails, the UE may have another MsgA transmission attempt with a random backoff and an increased MsgA transmission power (i.e., power ramping). The UE would switch to 4-step random access if the number of MsgA transmissions has reached msgA-TransMax, provided that preambleTransMax is greater than msgA-TransMax. A similar mechanism can be introduced for contention based Msg3 EDT transmission to handle Msg3 transmission failures.

In case preamble-less EDT is agreed beneficial, After a failure in Msg3 transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power.

In case preamble-less EDT is agreed beneficial, After a number of failed Msg 3 transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

### 2.2.7	UE identity for Msg4 transmission

Another open issue for preamble-less EDT is which identity (e.g., RNTI) should be used to schedule Msg4 transmission. In NR’s 2-step random access, MsgB-RNTI is derived based on the time/frequency position of the MsgA PRACH occasion. This principle can naturally be applied to preamble-less EDT, i.e., the RNTI used to schedule Msg4 transmission is derived based on the used Msg3 PUSCH time-frequency resource. The RNTI may be associated with either one or multiple Msg3 PUSCH time-frequency resources, which is up to network implementation.

In case preamble-less EDT is agreed beneficial, The RNTI used to schedule Msg4 transmission is derived based on the PUSCH time-frequency resource(s) used for contention based Msg3 EDT transmission.

### 2.2.8	Efficient delivery of Msg4

The second WI objective of this topic is the efficient delivery (reduced overhead) of Msg4. In last RAN2 meeting, [2] showed how the overhead reduction cannot come from optimizations in the RRCEarlyDataComplete ASN.1 structure. Thus, RAN2 should focus on different alternatives such as the transmission of a shared Msg4.

In 2-step random access, MsgB can consist of one or multiple successRAR and/or fallbackRAR sub-PDUs. Each successRAR consists of UE Contention Resolution Identity, the PDSCH-to-HARQ feedback timing indicator for MSGB HARQ feedback and Timing Advance Command, etc. Hence, a MsgB can be used to respond to multiple UEs which have transmitted MsgA, i.e., a shared MsgB.

NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

In our view, a similar design can be applied to Msg4, i.e., Msg4 can consist of multiple contention resolution identities for different UEs associated with the PDSCH-to-HARQ feedback timing indicator for Msg4 HARQ feedback, where the contention resolution Identity could be e.g., the TMSI included in RRCEarlyDataRequest in Msg3, or just the first 48 bits of Msg3 (similar to a regular 4-step random access or a successRAR sub-PDU in MsgB). This avoids the transmission of separate Msg4 for each individual UE whose Msg3 transmission is decoded successfully.

In case preamble-less EDT is agreed beneficial, Preamble-less EDT supports multiplexing several UE Contention Resolution Identities in Msg4.

## 2.2	CR Diversity Slotted ALOHA

RAN2 agreed to continue discussing Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for 2-step Msg3-EDT transmissions. As described above, repetition is a crucial feature for LTE-M and NB-IoT. If introducing DSA and CRDSA for 2-step Msg3-EDT transmissions, we shall make sure they can work together with repetition. In our view, RAN2 shall first discuss the feasibility and complexity of integrating DSA and CRDSA with repetition for 2 step Msg3-EDT transmission. DSA and CRDSA shall only be introduced if they can work together with repetition.

In case preamble-less EDT is agreed beneficial, For DSA and CRDSA, RAN2 shall first discuss whether it is feasible to integrate them with repetition.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A well configured 4-step RA scheme has substantially lower packet loss rate than CRDSA schemes at high load.

Observation 2	A well configured 4-step RA scheme has substantially higher throughput than CRDSA schemes at high load.

Observation 3	A well configured 4-step RA scheme has lower resource usage that CRDSA schemes for all load levels, but especially at low load.

Observation 4	A correctly configured 4-step RA scheme uses substantially lower uplink resources that can be used to support other users.

Observation 5	The objective of the preamble-less EDT is to improve capacity but non of the proposed schemes are doing that.

Observation 6	In legacy EDT, the number of repetitions and the largest TBS for Msg3 PUSCH depend on the CE level.

Observation 7	In legacy EDT, RSRP thresholds are configured for the UE to select the CE level.

Observation 8	The UE will verify uplink synchronization before sending Msg3.

Observation 9	The UE may not receive any Timing Advance Command before sending Msg3.

Observation 10	NR’s 2-step random access already supports multiplexing multiple UE Contention Resolution Identities in MsgB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The Rel-19 enhancement of EDT without Msg1 and Msg2 transmission is referred to as preamble-less EDT.

Proposal 2	As there are no capacity benefits for the preamble-less EDT enhancements compared to 4-step RA, there is no reason to make such enhancements.

Proposal 3	In case preamble-less EDT is agreed beneficial, System Information is used to provide preamble-less EDT cell specific PUSCH resources for Msg3 transmission. FFS on signalling details.

Proposal 4	In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

Proposal 5	In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold and largest TBS for Msg3 transmission.

Proposal 6	In case preamble-less EDT is agreed beneficial, Preamble-less EDT does not require a running TAT for Msg3 transmission.

Proposal 7	In case preamble-less EDT is agreed beneficial, After a failure in Msg3 transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power.

Proposal 8	In case preamble-less EDT is agreed beneficial, After a number of failed Msg 3 transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

Proposal 9	In case preamble-less EDT is agreed beneficial, The RNTI used to schedule Msg4 transmission is derived based on the PUSCH time-frequency resource(s) used for contention based Msg3 EDT transmission.

Proposal 10	In case preamble-less EDT is agreed beneficial, Preamble-less EDT supports multiplexing several UE Contention Resolution Identities in Msg4.

Proposal 11	In case preamble-less EDT is agreed beneficial, For DSA and CRDSA, RAN2 shall first discuss whether it is feasible to integrate them with repetition.

# 4	References

RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things”, MediaTek Inc., RAN#104, June 2024

R2-2405201, “Efficient delivery of RRCEarlyDataComplete message”, TOYOTA Info Technology Center, RAN2#126, May, 2024.

R2-2405202, “Discussion on EDT enhancements”, ESA, RAN2#126 Fukuoka, May 2024.

J. Moberg, M. Löfgren, and R S Karlsson, “Throughput of the WCDMA random access channel”, in IST Mobile Communication Summit, vol. 10. Oct. 2000.

# Appendix	Models for evaluating UL capacity enhancements

## A	CRDSA evaluation

- Given a fixed number of users 1, 2, 3,… , N, …, 2*N users that do pl-EDT transmission in the frame N, find the packet loss ratio (PLR) and save the number for each fixed number of users

  - simplest is to do a short Monte Carlo simulation to get the spread in selection of slots to transmit and collisions etc.

  - PLR is the number of packets that cannot be correctly decoded within the frame N (all replicas of the packet is colliding with at least one other packet that cannot be correctly decoded)

- Calculate for a Poisson load lambda, what is probability of getting 1, 2, …, 2*N users in the frame N

- Multiply probability of 1, 2, 3, 4, … users with the PLR results from A. giving the average of packet loss ratio for that lambda

- Repeat from B. with a different load

- Possibly repeat from A. for a different window size or different time between pl-EDT occasions