3GPP TSG-RAN WG2 #127	R2-2407553

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda Item:	7.6.2

Source:	Ericsson

Title:	R18 IoT NTN GNSS extension

Document for:	Discussion/Decision

# 1	Introduction

Here we discuss two remaining issues related to GNSS acquisition.

# 2	Discussion

## 2.1	GNSS validity duration reporting in MAC

The GNSS validity duration reporting is triggered after an GNSS fix operation.

Last meeting, we agreed

- When the MAC entity receives a trigger to send the GNSS validity duration report, the UE shall stop the timeAlignmentTimer of the pTAG

- In MAC 5.4.10, add a NOTE that during inactive time of C-DRX, it is up to UE implementation whether to stop timeAlignmentTimer and initiate CBRA

And this was captured in the MAC spec as

### 5.4.10	GNSS validity duration reporting

For an NB-IoT UE, a BL UE or a UE in enhanced coverage in a non-terrestrial network, an indication may be sent by upper layers to report the remaining GNSS measurement validity duration.

If the MAC entity receives an indication from upper layers to report the remaining GNSS measurement validity duration:

-	stop the timeAlignmentTimer associated with the pTAG, if running.

-	initiate a Random Access procedure (see clause 5.1).

NOTE:	When in RRC_CONNECTED, if the UE autonomously starts and completes GNSS acquisition using available idle periods, it is up to UE implementation whether to stop timeAlignmentTimer and initiate a Random Access procedure.

If the GNSS validity duration reporting procedure has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI, and;

-	if the allocated UL resources can accommodate the GNSS Validity Duration Report MAC control element plus its subheader, as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate the GNSS Validity Duration Report MAC control element as defined in clause 6.1.3.23.

All triggered GNSS validity duration reports shall be cancelled when a GNSS Validity Duration Report MAC control element is included in a MAC PDU for transmission.

:

This is fine besides the one issue that was overlooked.

The TA formula is (36.211 clause 8):

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs

where NTA is the term that the network can adjust with TACs, it is this term the network controls by sending TA Command MAC CEs.

If the UE is using an outdated GNSS position the NW can compensate the TA of the UE by sending TA Command MAC CEs and these TA commands are accumulated in the NTA but NW can compensate other errors than errors in GNSS position.

When the UE makes a new GNSS position measurement and the new location differs from the old position, the UE needs to either stop the TAT of pTAG and initiate RA procedure (that was decided for non-autonomous GNSS acquisition to ensure the UEs get an accurate NTA), or the UE itself needs to compensate a step in TA by adjusting NTA to remove the step in TA that was related to the change in GNSS position (for example as we described at last meeting [1]).

UEs that autonomously starts and completes GNSS acquisition using available idle periods runs the risk of getting step changes in the TA when a new GNSS acquisition is completed.

This can be avoided by enhancing the NOTE:

Add “In this case, the UE may compensate TA (see TS 36.211 [7] clause 8) to avoid time misalignment due to changes in the GNSS position.” to the NOTE in MAC spec section 5.4.10.

## 2.2	GNSS measurement triggering and reporting in RRC

The RRC GNSS measurement and reporting is defined in RRC:

### 5.5.9	GNSS measurement triggering and reporting

For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods.

The UE shall:

1>	if an indication to perform GNSS measurement is received from lower layers:

2>	perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23];

2>	stop timer T390, if running;

1>	if gnss-AutonomousEnabled is configured:

2>	perform GNSS measurement using an autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry, with a gap length indicated by lower layers or equal to the latest reported time duration required for the UE to acquire a GNSS position if not indicated by lower layers;

NOTE:	UE can also autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid, and the exact time of starting GNSS measurements during available idle periods is left to UE implementation.

1>	upon starting GNSS measurement:

2>	stop timer T318, if running;

1>	upon indication that a new GNSS position becomes valid:

2>	instruct lower layers to report the remaining GNSS measurement validity duration (see TS 36.321 [6]);

1>	upon indication that GNSS measurement has failed:

2>	if GNSS position is out-of-date; and

2>	if ul-TransmissionExtensionEnabled is not configured or T390 is not running:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

However, the point when gnss-AutonomousEnabled is configured contains four different possibilities that are not clear to the reader and it repeats some words several times. That is:

- The gap starts from T390 expiry and either:

  - The gap length is indicated from lower layers, or

  - The gap length is equal to the latest reported time duration required for the UE to acquire a GNSS position

- The gap starts from GNSS validity duration expiry and either:

  - The gap length is indicated from lower layers, or

  - The gap length is equal to the latest reported time duration required for the UE to acquire a GNSS position

The autonomous gap behaviour is not straight forward described in RRC.

Consider the text proposal below.

### 5.5.9	GNSS measurement triggering and reporting

For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods.

The UE shall:

1>	if an indication to perform GNSS measurement is received from lower layers:

2>	perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23];

2>	stop timer T390, if running;

1>	if gnss-AutonomousEnabled is configured:

2>	if gap length is indicated by lower layers:

3>	set the autonomous gap length to the gap length indicated by lower layers;

2>	else:

3>	set the autonomous gap length to the latest reported time duration required for the UE to acquire a GNSS position;

2>	perform GNSS measurement using the autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry;

NOTE:	UE can autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid. The exact time of starting GNSS measurements during available idle periods is left to UE implementation.

1>	upon starting GNSS measurement:

2>	stop timer T318, if running;

1>	upon indication that a new GNSS position becomes valid:

2>	instruct lower layers to report the remaining GNSS measurement validity duration (see TS 36.321 [6]);

1>	upon indication that GNSS measurement has failed:

2>	if GNSS position is out-of-date; and

2>	if ul-TransmissionExtensionEnabled is not configured or T390 is not running:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	UEs that autonomously starts and completes GNSS acquisition using available idle periods runs the risk of getting step changes in the TA when a new GNSS acquisition is completed.

Observation 2	The autonomous gap behaviour is not straight forward described in RRC.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add “In this case, the UE may compensate TA (see TS 36.211 [7] clause 8) to avoid time misalignment due to changes in the GNSS position.” to the NOTE in MAC spec section 5.4.10.

Proposal 2	Consider the text proposal below.

# References

R2-2405451, “R18 IoT NTN GNSS extension” Ericsson, RAN2#127, Fukuoka, May. 2024.