3GPP TSG-RAN WG2 Meeting #127                    	                 R2-2407558

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.3.2.1

Source: 	MediaTek Inc.

Title:  	Evaluation and Simulation Results for AIML RRM Prediction

Document for:	Discussion and decision

# Introduction

For AI/ML for RRM measurement prediction, in the RAN2 #126 meeting the agreements was made to support evaluation of use cases.

Agreements 3 sub cases are introduced for cell level RRM measurement prediction: Sub case 1: L1 beam level measurement result(s) is predicted based on actual L1 beam level measurement result(s) and then cell level measurement result is generated Sub case 2: Cell level measurement result(s) is predicted based on actual cell level measurement result(s) Sub case 3: Cell level measurement result(s) is predicted based on L1 beam level measurement result(s) Editor Note 1: The RRM measurement refers to either L3 cell/beam level measurement and/or L1 beam level measurement

The prioritized evaluation scenarios for RRM measurement prediction were agreed as follow:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this paper, we present our initial evaluation and simulation results on RRM measurement prediction in temporal (Case 2), frequency (Case 3) and spatial domain (Case 6) with high and middle priority, which target to the 1st goal to reduce the measurement effort.

We understand that case 4, which involves FR2 to FR2 intra-frequency temporal domain case A aimed at enhancing handover performance (the 2nd goal), is applicable for the indirect prediction of measurement events. This case necessitates system-level simulation to assess handover performance. It is deemed relevant for the application scenario of measurement event prediction. Therefore, the simulation outcomes for this specific case are not included in this document.

# Motivation

The RRM measurement procedure is critical for maintaining the performance and reliability of mobile network. With the evolution of cellular technology and the introduction of higher frequency bands such as FR2, the number of measurement reports can increase even further due to dynamic channel conditions, beamforming, dense deployments, and smaller cell sizes. To facilitate UE RRM measurements, the network transmits various types of reference signals, representing a significant system overhead that consumes both radio resources and network energy.

At R18, 3GPP has studied AI/ML for Beam management, the evaluation results show AI/ML can provide good beam prediction accuracy with fewer measurements/RS overhead at temporal domain and spatial domain. For mobility, with certain spatial consistency AI/ML can be extended to work on cross-cell RRM measurement prediction.

With certain consistency in temporal/spatial domain, the prediction can lead to overhead reduction and energy saving. However, prediction is not always perfect, there is always a difference between prediction and actual value. For mobility, the prediction measurement difference will influence the system performance. The correlation between the discrepancy in predicted versus actual values and the overall system performance does not inherently follow a linear or proportional pattern. For instance, minor prediction errors may not substantially affect the overall system performance, but when the error surpass a certain threshold, the system performance may precipitously decline.

Our objective is to minimize the reference signal overhead and UE measurement effort by leveraging AI/ML techniques, while maintaining the system's performance. Thus, a comprehensive assessment of AI model cannot just be judged solely by intermediated KPI (e.g., average RSRP difference), but necessitates a comprehensive evaluation on system performance KPIs.

Observation 1: The correlation between the discrepancy in predicted versus actual values and the overall system performance does not inherently follow a linear or proportional pattern. Minor prediction errors may not substantially affect the overall system performance, but when the error surpass a certain threshold, the system performance may precipitously decline.

Proposal 1: To evaluate the performance of RRM measurement prediction, both intermediate KPIs and system performance KPIs should be studied.

Driven by this, we provide evaluation and preliminary results including intermediate KPIs and system performance KPIs, demonstrating that with decrease signaling overhead and reduction of the UE measurement effort and power consumption, crucially, these improvements are achieved without compromising the overall system performance.

# Evaluation methodology and preliminary results

## Preliminary Results for Temporal domain prediction on serving cell and neighbouring cells (Case 2)

For the temporal domain prediction AI approach, using the simulation assumption of FR1 in Appendix Table 1. we use historical measurements instances (M*40ms), to predict the future measurement instance (N*40). From 3 sub-use cases for cell level RRM measurement, considering different speed (30km/h, 90km/h), different model applicable cell range and its input-output, we provide the beam/cell level RSRP difference, computation and storage complexity. Comparing AI approach with non-AI baseline (sample and hold), which using the measured temporal instance to replace the instance that need to be predicted.

The simulation assumption and information of the AI/ML are listed in Table 1.

Table 1: Simulation table for case 2 - case B

| Report parameters |  | MediaTek |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 30/90 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | Yes |
|  | inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | full |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | Spatial consistency A + coarse update |
|  | Number of TX beams | 1 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50%/80% |
|  | Observation window(Note3) | 200ms, 80ms |
|  | Prediction window (Note3) | 200ms |
|  | Any other parameters (Note 4) | NA |
| Data Size (Number of Samples) | Training/validity | 200,000/10,000 |
|  | Testing | 100,000 |
| AI/ML model input/output | Model input (Note 5) | RSRP according to sub-use case |
|  | Model output(Note 6) | RSRP according to sub-use case |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | CNN |
|  | Model complexity in a number of parameters(M) | See Table 2/3/4 |
|  | Model complexity in model size (e.g. Mbyte) | See Table 2/3/4 |
|  | Computational complexity [FLOPs] | See Table 2/3/4 |
| Metrics | Average L3 cell level RSRP difference (dB) | See Table 2/3/4 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | See Table 2/3/4 |

In table 2, we summarize the evaluation results of measurement reduction with 50% for 30km/h, we use historical measurements instances (5*40ms=200ms), to predict the future measurement instance (5*40=200ms). The cell level RSRP differ of AI approach is lower than 1 dB for both cluster-specific and cell-specific models. The specifics of the cluster-specific and cell-specific models are detailed in our accompanying paper [1]. The L3 to L3 prediction has smallest RSRP differ compare with L1 to L1 and L1 to L3 prediction. The cluster-specific model has lower RSRP differ than cell-specific model, while with fewer computation complexity and storage complexity.

Table 2 Intermediate simulation results for Case 2 with 50% measurement reduction(30km/h)

|  | Measurement level | Beam RSRP differ | Cell RSRP differ | Computational complexity [FLOPs] | Model complexity in a number of parameters | Model complexity in model size |
| --- | --- | --- | --- | --- | --- | --- |
| Non-AI | L1L1 | 1.85 | 0.88 | NA | NA | NA |
| Cell-specific (1 model applicable for 1 cell, 21cells) | L1L1 | 1.59 | 0.85 | 4155200*21 =87 259 200 | 30261*21 =635,481 | 170.1KB*21 =3.48MB |
|  | L1L3 | NA | 0.67 |  |  |  |
|  | L3L3 | NA | 0.25 |  |  |  |
| Cluster specific (1 model applicable for 21 cells) | L1L1 | 1.48 | 0.78 | 64 663 200 | 47961 | 311.7KB |
|  | L1L3 | NA | 0.63 |  |  |  |
|  | L3L3 | NA | 0.21 |  |  |  |

In Table 3, we summarize the evaluation results of measurement reduction from with 50% for 90km/h, we use historical measurements instances (5*40ms=200ms), to predict the future measurement instance (5*40=200ms). The cell level RSRP difference of AI approach is lower than 1.2 dB for both cluster-specific and cell-specific models. Compare with Table 1, the RSRP difference increases as the speed increases. As velocity increases, the channel may show more variability and fluctuations further cause the prediction accuracy decreases.

Table 3 Intermediate simulation results for Case 2 with 50% measurement reduction(90km/h)

|  | Measurement level | Beam RSRP differ | Cell RSRP differ | Computational complexity [FLOPs] | Model complexity in a number of parameters | Model complexity in model size |
| --- | --- | --- | --- | --- | --- | --- |
| Non-AI | L1L1 | 2.30 | 1.20 | NA | NA | NA |
| Cell-specific (1 model applicable for 1 cell, 21cells) | L1L1 | 2.04 | 1.16 | 4155200*21 =87,259,200 | 30261*21 =635,481 | 170.1KB*21 =3.48MB |
|  | L1L3 | NA | 1.03 |  |  |  |
|  | L3L3 | NA | 0.35 |  |  |  |
| Cluster specific (1 model applicable for 21 cells) | L1L1 | 1.89 | 1.11 | 64,663,200 | 47,961 | 311.7KB |
|  | L1L3 | NA | 0.95 |  |  |  |
|  | L3L3 | NA | 0.31 |  |  |  |

Observation 2: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in speed leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

In Table 4, we summarize the evaluation results of measurement reduction with 80% for 30km/h, we use historical measurements instances (2*40ms=80ms), to predict the future measurement instance (8*40=320ms). The cell level RSRP difference of AI approach is lower than 1.3 dB for both cluster-specific and cell-specific models. The L3 to L3 prediction still has smallest RSRP difference compare with L1 to L1 and L1 to L3 prediction. The cluster-specific model has lower RSRP difference than the cell-specific model, while with fewer computation complexity and storage complexity. Compare with Table 1, with more measurement reduction, the RSRP difference also increases.

Table 4 Intermediate simulation results for Case 2 with 80% measurement reduction(30km/h)

|  | Measurement level | Beam RSRP differ | Cell RSRP differ | Computational complexity [FLOPs] | Model complexity in a number of parameters(M) | Model complexity in model size (e.g. Mbyte) |
| --- | --- | --- | --- | --- | --- | --- |
| Non-AI | NA | 2.16 | 1.23 | NA | NA | NA |
| Cell-specific (1 model applicable 1 cell, 21cells) | L1L1 | 1.84 | 1.03 | 4155200*21 =87 259 200 | 23,376*21 =490,896 | 96.3KB*21 =1.97MB |
|  | L1L3 | NA | 1.09 |  |  |  |
|  | L3L3 | NA | 0.46 |  |  |  |
| Cluster specific (1 model applicable for 21 cells) | L1L1 | 1.75 | 1.01 | 90 619 200 | 51,696 | 209.5KB |
|  | L1L3 | NA | 1.01 |  |  |  |
|  | L3L3 | NA | 0.44 |  |  |  |

Observation 3: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in the measurement reduction ration leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

Observation 4:  For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), among all the sub-cases, the L3 to L3 prediction exhibits the smallest cell-level RSRP difference compared to both the L1 to L1 and L1 to L3 prediction.

We further provide the system level performance results as following:

Figure 1 provides system simulation results for cluster specific AI/ML model, we can observe that with 50% measurement reduction, the HOF, Ping-pong, data interruption time cause by RLF/HOF/handover is not increased for L3 to L3 prediction.

(a) Data interruption time rate                 (b)  HO success/HOF times

(c) Ping-pong times                           (d) Average time of stay(ms)

Figure 1 System simulation results for Case 2 with 50% measurement reduction, cluster-specific model

Observation 5: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort can be achieved, without any degradation in mobility performance compared with legacy L3 HO in the metrics such as HOF, Ping-pong, data interruption time, and average TOS.

Figure 2 provides system simulation results for cell-specific model, we can observe that with 50% measurement reduction, the HOF, Ping-pong, data interruption time cause by RLF/HOF/handover is not increased for L1 to L3 prediction, and slightly increased for L1 to L1 prediction, and even decreased for L3 to L3 prediction.

(a) Data interruption time rate                 (b)  HO success/HOF times

(c) Ping-pong times                        (d) Average time of stay(ms)

Figure 2 System simulation results for Case 2 with 50% measurement reduction, cell-specific model

Observation 6: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort, cluster-specific model has better system level results than cell-specific model.

Proposal 2: RAN2 investigates the comparison between different speed, measurement reduction rate for RRM measurement prediction to observe possible trends on both intermediate KPI and system KPIs performance and further capture the performance results into TR with evaluation spreadsheet.

## Preliminary Results for Frequency domain prediction on serving cell and neighbouring cell (Case 3)

The simulation assumption and information of the AI/ML are listed in Table 5.

Table 5: Simulation table for Case 3

| Report parameters |  | MediaTek |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 30/90 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | Yes |
|  | inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | full |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | Spatial consistency A + coarse update |
|  | Number of TX beams | 1 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | NA |
|  | Observation window(Note3) | NA |
|  | Prediction window (Note3) | NA |
|  | Any other parameters (Note 4) | NA |
| Data Size (Number of Samples) | Training/validity | 160,000 |
|  | Testing | 40,000 |
| AI/ML model input/output | Model input (Note 5) | RSRP according to sub-use case |
|  | Model output(Note 6) | RSRP according to sub-use case |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | CNN |
|  | Model complexity in a number of parameters(M) | See Table 6 |
|  | Model complexity in model size (e.g. Mbyte) | See Table 6 |
|  | Computational complexity [FLOPs] | See Table 6 |
| Metrics | Average L3 cell level RSRP difference (dB) | See Table 6 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | NA |

For Case 3 (FR1 to FR1 inter-frequency, frequency-domain prediction), the simulation assumption of FR1 in Appendix Table 1 is used. The AI model is used to learn data characteristics based on the beam measurement information with one frequency information to predict the beam information of another frequency. Preliminary simulation results for frequency domain beam prediction in Table 6 indicates that using an AI model to predict beam information in a 4GHz cell based on data from a 2GHz cell, the minimum difference between the predicted and actual RSRP values is relative lower than non-AI baseline when the same beam pattern (32 beams) is used. However, when different beam settings (with different beam direction) are applied, the AI model's performance deteriorated.

The non-AI approach has 2 solutions, (a) directly considering prediction (4GHz) equals to observation (2GHz), the RSRP differ equals to prediction subtract ground truth; (b) Taking path loss into account, considering prediction (4GHz) equals to observation (2GHz) subtract path loss difference between 4GHz and 2GHz.

Table 6(a): Intermediate simulation results for Case 3 inter-frequency prediction (30km/hr)

| Inter-freq: FR1, 30km | AI Input/ output | Abs L3 Cell RSRP diff (dB) | Computational complexity [FLOPs] | Model complexity in a number of parameters(M) | Model complexity in model size |
| --- | --- | --- | --- | --- | --- |
| Non-AI | NA | (a) 6.17 (b) 2.34 | NA | NA | NA |
| Option 1: Cluster | L3 to L3 | 1.64 | 3,029,460 | 51,594 | 201KB |
|  | L1 to L3 | 1.93 |  |  |  |
|  | L1 to L1 | 1.97 |  |  |  |
| Option 2: Cell-specific | L3 to L3 | 2.29 | 142,260 * 21 = 2,987,460 | 7,134*21 = 147,924 | ~585KB |
|  | L1 to L3 | 3.26 |  |  |  |
|  | L1 to L1 | 2.53 |  |  |  |

Table 6(b): Intermediate simulation results for Case 3 inter-frequency prediction (90km/hr)

| Inter-freq FR1, 90km | AI Input/ output | Abs L3 Cell RSRP diff (dB) | Computational complexity [FLOPs] | Model complexity in a number of parameters(M) | Model complexity in model size |
| --- | --- | --- | --- | --- | --- |
| Non-AI | NA | (a) 6.08 (b) 2.31 | NA | NA | NA |
| Option 1: Cluster | L3 to L3 | 1.49 | 3,029,460 | 51,594 | 201KB |
|  | L1 to L3 | 2.00 |  |  |  |
|  | L1 to L1 | 2.12 |  |  |  |
| Option 2: Cell-specific | L3 to L3 | 2.20 | 142,260 * 21 = 2 987,460 | 7,134*21 = 147,924 | ~585KB |
|  | L1 to L3 | 2.26 |  |  |  |
|  | L1 to L1 | 2.99 |  |  |  |

Observation 7: In Case 3, it demonstrates that reducing the frequency domain measurement effort and the need for measurement gaps results in better AI/ML prediction performance under the same beam pattern settings compared to different ones. This suggests that the consistency of beam patterns is essential for AI to achieve accurate predictions.

## Preliminary Results for Spatial domain prediction on serving cell and neighbouring cell (Case 6)

The simulation assumption and information of the AI/ML are listed in Table 7.

Table 7: Simulation table for Case 6

| Report parameters |  | MediaTek |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 30/90 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | Yes |
|  | inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | full |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | Spatial consistency A + coarse update |
|  | Number of TX beams | 1 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50% |
|  | Observation window(Note3) | NA |
|  | Prediction window (Note3) | NA |
|  | Any other parameters (Note 4) | NA |
| Data Size (Number of Samples) | Training/validity | 200,000/10,000 |
|  | Testing | 100,000 |
| AI/ML model input/output | Model input (Note 5) | RSRP according to sub-use case |
|  | Model output(Note 6) | RSRP according to sub-use case |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | CNN |
|  | Model complexity in a number of parameters(M) | See Table 8 |
|  | Model complexity in model size (e.g. Mbyte) | See Table 8 |
|  | Computational complexity [FLOPs] | See Table 8 |
| Metrics | Average L3 cell level RSRP difference (dB) | See Table 8 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | See Table 8 |

Considering spatial RRM measurement prediction, using the simulation assumption of FR2 in Appendix Table 1. The AI model is deployed to learn data characteristics, taking quality of partial beam-pairs as input while output quality of full beam-pairs information. As the measurement beam numbers decreased, the average difference between the predicted L3 filtering RSRP value and the actual L3 filtering RSRP gradually expanded. Specifically, when the beam measurement number is reduced by 50%, the minimum RSRP difference is observed to be below 1dB. The L1 to L3 prediction is not good as L1 to L1 prediction.

Table 8 Intermediate simulation results for Case 6 with 50% measurement reduction(30km/h)

|  | Measurement level | Beam RSRP differ | Cell RSRP differ | Computational complexity [FLOPs] | Model complexity in a number of parameters(M) | Model complexity in model size |
| --- | --- | --- | --- | --- | --- | --- |
| cell-specific | L1L1 | 0.97 | 0.43 | 132,915,200*21= 2731,219,200 | 86,016*21 =1,806,336 | 95.1KB*21 =1.95MB |
|  | L1L3 | NA | 2.20 | 33,382,400*21= 701,030,400 | 23,040*21 =483,840 | 87.3KB*21 =1.79MB |
| Cluster-specific (1 model applicable for 21 cells) | L1L1 | 0.54 | 0.29 | 566,415,360 | 987,392 | 3.7MB |
|  | L1L3 | NA | 2.08 | 354,009,600 | 77,312 | 1.3MB |

Observation 8: In Case 6 with 50% measurement reduction, beam-level and L3 cell-level average RSRP difference are relatively low than 1 dB for L1 to L1 prediction.

Proposal 3: Capture the simulation results and following performance observations for each case in the TR.

- Observation 2: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in speed leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

- Observation 3: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in the measurement reduction ration leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

- Observation 4:  For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), among all the sub-cases, the L3 to L3 prediction exhibits the smallest cell-level RSRP difference compared to both the L1 to L1 and L1 to L3 prediction.

- Observation 5: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort can be achieved, without any degradation in mobility performance compared with legacy L3 HO in the metrics such as HOF, Ping-pong, data interruption time, and average TOS.

- Observation 6: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort, cluster-specific model has better system level results than cell-specific model.

- Observation 7: In Case 3, it demonstrates that reducing the frequency domain measurement effort and the need for measurement gaps results in better AI/ML prediction performance under the same beam pattern settings compared to different ones.

- Observation 8: In Case 6 with 50% measurement reduction, beam-level and L3 cell-level average RSRP difference are relatively low than 1 dB for L1 to L1 prediction.

# Spatial consistency

In TR 38.901, spatial consistency is modeled for large-scale parameters (LSP), small-scale parameters (SSP), and multiple frequencies, using procedure A or procedure B. Procedure A dynamically updates the channel parameters, e.g., LSP, SSP based on UE movement from t0 to tk. Procedure B provides location-dependent channel parameters update based on correlation distance, ensuring spatial consistency across different locations. In mobility simulations, moving UEs may have different starting points, when two UEs arrive nearby locations, they should see a similar channel.

For procedure A only spatial consistency procedure, different starting points can lead to varying initial SSPs, which may result in significant discrepancies when UEs move to nearby locations, but still have different SSPs, which is unreasonable. As shown in Figure 3 ProcA only, the UE1 and UE2 with different starting points at t0 have different starting environments and thus different initial SSPs (angle/delay/power), updating SSPs using Procedure A for tk’s, even if these UEs move to nearby locations UE1 at (80.72,73.88) and UE 2 at (83.56,73.11), they have different SSPs and exhibit varying beam patterns and beam-level RSRP distribution.

For implementation of procedure B, LSPs and SSPs may be interpolated within the coherence length or time of the respective parameter. The interpolation process can be computationally demanding; otherwise, it will fail to accurately capture channel changes.

Observation 9: With Procedure A only, when the channels evolve from very different starting points to nearby locations, the observed channel environment, exhibit beam patterns and beam-level RSRP distribution are totally different.

Spatial consistency of channel is crucial, the channel characteristics reflect the environment, leading to reliable and robust algorithm performance. Without proper spatial consistency, the generated channel data is essentially irrelevant (garbage in), leading to unreliable and ineffective AI/ML model outputs (garbage out). Thus, to address the spatial consistency issue, we propose integrating a coarse update mechanism with Procedure A. This mechanism allows SSP updates based on the UE’s location to ensure that UEs arriving at nearby locations have more similar SSPs. Coarse update refers to a method of adjusting small-scale parameters (SSPs) based on the UE’s current location within a predefined grid or spatial resolution. When a UE moves from one grid to another, SSPs are updated to reflect the new grid’s predefined parameters (coarse update), while procedure A ensures that SSPs are updated based on the UE's current location within the grid.

Observation 10: With the combination of coarse update and procedure A, the observed channel environment, exhibit beam patterns and beam-level RSRP distribution are similar when UEs arrive nearby locations.

Figure 3 Beam pattern of different spatial consistency procedure for mobility UE

Proposal 4: Recommend that RAN2 take the spatial consistency issue into account during simulation evaluation, e.g., considering the implementation of Procedure A with a coarse update mechanism.

# Conclusion

Preliminary Results for Temporal domain prediction on serving cell and neighbouring cells (Case 2)

Observation 1: The correlation between the discrepancy in predicted versus actual values and the overall system performance does not inherently follow a linear or proportional pattern. Minor prediction errors may not substantially affect the overall system performance, but when the error surpass a certain threshold, the system performance may precipitously decline.

Observation 2: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in speed leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

Observation 3: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in the measurement reduction ration leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

Observation 4:  For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), among all the sub-cases, the L3 to L3 prediction exhibits the smallest cell-level RSRP difference compared to both the L1 to L1 and L1 to L3 prediction.

Observation 5: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort can be achieved, without any degradation in mobility performance compared with legacy L3 HO in the metrics such as HOF, Ping-pong, data interruption time, and average TOS.

Observation 6: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort, cluster-specific model has better system level results than cell-specific model.

Proposal 1: To evaluate the performance of RRM measurement prediction, both intermediate KPIs and system performance KPIs should be studied.

Proposal 2: RAN2 investigates the comparison between different speed, measurement reduction rate for RRM measurement prediction to observe possible trends on both intermediate KPI and system KPIs performance and further capture the performance results into TR with evaluation spreadsheet.

Preliminary Results for Frequency domain prediction on serving cell and neighbouring cell (Case 3)

Observation 7: In Case 3, it demonstrates that reducing the frequency domain measurement effort and the need for measurement gaps results in better AI/ML prediction performance under the same beam pattern settings compared to different ones. This suggests that the consistency of beam patterns is essential for AI to achieve accurate predictions.

Preliminary Results for Spatial domain prediction on serving cell and neighbouring cell (Case 6)

Observation 8: In Case 6 with 50% measurement reduction, beam-level and L3 cell-level average RSRP difference are relatively low than 1 dB for L1 to L1 prediction.

Performance observations for different cases

Proposal 3: Capture the simulation results and following performance observations for each case in the TR.

- Observation 2: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in speed leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

- Observation 3: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in the measurement reduction ration leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

- Observation 4:  For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), among all the sub-cases, the L3 to L3 prediction exhibits the smallest cell-level RSRP difference compared to both the L1 to L1 and L1 to L3 prediction.

- Observation 5: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort can be achieved, without any degradation in mobility performance compared with legacy L3 HO in the metrics such as HOF, Ping-pong, data interruption time, and average TOS.

- Observation 6: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort, cluster-specific model has better system level results than cell-specific model.

- Observation 7: In Case 3, it demonstrates that reducing the frequency domain measurement effort and the need for measurement gaps results in better AI/ML prediction performance under the same beam pattern settings compared to different ones.

- Observation 8: In Case 6 with 50% measurement reduction, beam-level and L3 cell-level average RSRP difference are relatively low than 1 dB for L1 to L1 prediction.

Spatial consistency

Observation 9: With Procedure A only, when the channels evolve from very different starting points to nearby locations, the observed channel environment, exhibit beam patterns and beam-level RSRP distribution are totally different.

Observation 10: With the combination of coarse update and procedure A, the observed channel environment, exhibit beam patterns and beam-level RSRP distribution are similar when UEs arrive nearby locations.

Proposal 4: Recommend that RAN2 take the spatial consistency issue into account during simulation evaluation, e.g., considering the implementation of Procedure A with a coarse update mechanism.

# Reference

[1] R2-2406580, Cluster-based Approach for RRM Prediction and Other Aspects.

# Appendix

Potential Evaluation scenarios

Table 1 Assumptions for SLS

| Parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model (7 sites, 3 sectors/cells per site) | 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; Inter-frequency correlation model is optional. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; |
| System BW | 20MHz | 80MHz |
| UE speed | 30,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement | 30,,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 1, 1, 1) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 with coarse update | Procedure A in TR38.901 with coarse update |
| UE trajectory model | 3 options in 38.843 section 6.3.1 | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | circle-bouncing model, | circle-bouncing model, |
| Sampling period | 40ms | 20ms |