3GPP TSG-RAN WG2 Meeting #127	R2-2407562

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	ZTE Corporation

Title:            Discussion on RAN3 LS on intra-SN SCPAC in MN format

Agenda item:	7.4.1

Document for:	Discussion and decision

Introduction

At last meeting, a RAN3 reply LS [1] on inter-node RRC message for intra-SN SCPAC in MN format was sent to RAN2 as follows:

1. Overall Description: RAN3 thanks RAN2 for the LS on inter-node RRC message for intra-SN SCPAC in MN format. Regarding the following agreement made by RAN2, RAN3 has discussed and would like to further clarify. - RAN2 understands both CG-Config message associated with the source SCG and CG-CandidateList message associated with candidate PSCell(s) can be included in one SN Modification Required message for intra-SN subsequent CPAC in MN format. From RAN3’s point of view, currently, one SN Modification Required message can only include either CG-Config message or CG-CandidateList message. In addition, RAN3 would like to understand the following two questions: Q1: Are both CG-Config and CG-CandidateList related to same configuration of intra-SN subsequent CPAC in MN format, and must be sent together in the same SN Modification Required XnAP message? Q2: If both RRC messages must be sent together, is it feasible to include both CG-Config message and CG-CandidateList messages in a single RRC message (new or existing), which could then be transferred in a single SN Modification Required XnAP message? 2. Actions: To RAN2 group. ACTION: RAN3 kindly asks RAN2 to provide answers to the questions above.

In the LS, RAN3 asked two questions about inter-node RRC message transfer for intra-SN SCPAC in MN format. In this contribution, we discussed the corresponding responses and provided the draft reply LS.

Discussions

Discussion on Q1

In RAN3 LS, the following question was raised:

Q1: Are both CG-Config and CG-CandidateList related to same configuration of intra-SN subsequent CPAC in MN format, and must be sent together in the same SN Modification Required XnAP message?

According to the previous RAN2 discussion in [2], the CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format (i.e. via candidateCellInfoListCPC), which is associated with the current serving SCG configuration. The CG-CandidateList message is used to transfer the information related to candidate PSCell/SCG(s), including the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format (i.e. via candidateCellInfoListSubsequentCPC). Thus, these two messages are related to different configuration for intra-SN SCPAC in MN format.

- Currently, the CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format, while CG-CandidateList message is used to transfer the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format. These two messages are related to different configuration for intra-SN SCPAC in MN format.

The detailed description for these two messages in the current RRC spec [3] is shown as below:

TS 38.331 v18.2.0 – CG-CandidateList This message is used to transfer the SCG radio configuration for one or more candidate cells for Conditional PSCell Addition (CPA), Conditional PSCell Change (CPC), inter-SN subsequent CPAC, or CHO with candidate SCG(s) as generated by the candidate target SgNB, or the serving SgNB in case of intra-SN subsequent CPAC in MN format. Direction: Secondary gNB to master gNB or eNB. CG-CandidateList message -- ASN1START -- TAG-CG-CANDIDATELIST-START CG-CandidateList ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ cg-CandidateList-r17 CG-CandidateList-r17-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } CG-CandidateList-r17-IEs ::= SEQUENCE { cg-CandidateToAddModList-r17 SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF CG-CandidateInfo-r17 OPTIONAL, cg-CandidateToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF CG-CandidateInfoId-r17 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } CG-CandidateInfo-r17 ::= SEQUENCE { cg-CandidateInfoId-r17 CG-CandidateInfoId-r17, candidateCG-Config-r17 OCTET STRING (CONTAINING CG-Config) } CG-CandidateInfoId-r17::= SEQUENCE { ssbFrequency-r17 ARFCN-ValueNR, physCellId-r17 PhysCellId } -- TAG-CG-CANDIDATELIST-STOP -- ASN1STOP – CG-Config This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB. It can also be used by a CU to request a DU to perform certain actions, e.g. to request the DU to perform a new lower layer configuration. Direction: Secondary gNB or eNB to master gNB or eNB, alternatively CU to DU. ... candidateCellInfoListCPC Contains information regarding candidate target cells for Conditional PSCell Change (CPC) or inter-SN subsequent CPAC that the source secondary gNB suggests the target secondary gNB to consider configuring for CPC or subsequent CPAC, and/or that the source secondary gNB prepares for intra-SN subsequent CPAC in MN format. This field is only used in SN initiated CPC and SN initiated subsequent CPAC.. candidateCellInfoListSubsequentCPC Contains information regarding candidate target cells for subsequent CPAC that candidate secondary gNB (or the serving secondary gNB in case of intra-SN subsequent CPAC in MN format) suggests the master gNB to consider configuring for subsequent CPAC. This field is only used in MN initiated and SN initiated subsequent CPAC. This field is only included in a CG-Config message which is contained within a CG-CandidateList message..

These two messages are related to different parts of configuration for intra-SN SCPAC in MN format, and the MN needs to collect all related configurations from the serving SN to generate the final RRC message for inra-SN SCPAC in MN format and send to the UE via one RRCReconfiguration message. Thus, from RAN2 perspective, it’s more efficient to include both CG-Config and CG-CandidateList in the same SN Modification Required XnAP message.

Besides, according to the stage-2 flow chart for intra-SN SCPAC in MN format in the current 37.340 spec [4], there is only one SN Modification Required XnAP message sent from the SN to the MN to trigger the preparation procedure.

TS 37.340 v18.2.0 SN initiated intra-SN subsequent CPAC with MN involvement This procedure is initiated by the SN for intra-SN subsequent CPAC with MN involvement. Figure 10.20-3: Intra-SN subsequent CPAC - SN initiated with MN involvement

If we include the CG-Config and CG-CandidateList into two SN Modification Required XnAP messages, there must be two SN Modification Required messages in step 1and two SN Modification Confirm messages in step 6, which is not the usual practice for SN modification procedure.

- According to the stage-2 flow chart for intra-SN SCPAC in MN format in the current 37.340 spec, there is only one SN Modification Required XnAP message sent from the SN to the MN to trigger the preparation procedure.

Based on observations above, we proposed that:

- From RAN2 perspective, CG-Config and CG-CandidateList are related to different configuration of intra-SN subsequent CPAC in MN format, and should be sent together in the same SN Modification Required XnAP message.

Discussion on Q2

In RAN3 LS, the second question was mentioned as below:

Q2: If both RRC messages must be sent together, is it feasible to include both CG-Config message and CG-CandidateList messages in a single RRC message (new or existing), which could then be transferred in a single SN Modification Required XnAP message?

Currently, CG-Config message and CG-CandidateList message are two different inter-node RRC messages. There is no additional inter-node RRC message to include them together in the current RRC spec.

- There is no existing inter-node RRC message can include both CG-Config message and CG-CandidateList messages together.

Considering that ASN.1 was frozen at last meeting, RAN2 does not expect to introduce a new inter-node RRC message to contain these two messages together. Thus, it would be better to request RAN3 to extend the SN Modification Required XnAP message to include both CG-Config message and CG-CandidateList message together.

- From RAN2 perspective, CG-Config message and CG-CandidateList message can not be included in a single existing inter-node RRC message. RAN2 sends the reply LS to RAN3 to kindly ask RAN3 to include them in a single SN Modification Required XnAP message.

A draft reply LS is provided in the Annex.

- RAN2 to approve the reply LS in the Annex.

Conclusion and Proposals

In this contribution, we discussed the RAN3 LS on intra-SN SCPAC in MN format with the following observations and proposals:

- Currently, the CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format, while CG-CandidateList message is used to transfer the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format. These two messages are related to different configuration for intra-SN SCPAC in MN format.

- According to the stage-2 flow chart for intra-SN SCPAC in MN format in the current 37.340 spec, there is only one SN Modification Required XnAP message sent from the SN to the MN to trigger the preparation procedure.

- There is no existing inter-node RRC message can include both CG-Config message and CG-CandidateList messages together.

- From RAN2 perspective, CG-Config and CG-CandidateList are related to different configuration of intra-SN subsequent CPAC in MN format, and should be sent together in the same SN Modification Required XnAP message.

- From RAN2 perspective, CG-Config message and CG-CandidateList message can not be included in a single existing inter-node RRC message. RAN2 sends the reply LS to RAN3 to kindly ask RAN3 to include them in a single SN Modification Required XnAP message.

- RAN2 to approve the reply LS in the Annex.

References

R2-2406217	  Reply LS on intra-SN SCPAC in MN format (R3-243775; contact: ZTE)	  RAN3

R2-2402745	  Discussion on inter-node RRC message for intra-SN SCPAC in MN format	 ZTE Corporation

TS 38.331 v18.2.0

TS 37.340 v18.2.0

- Annex

Title:	Reply LS on intra-SN SCPAC in MN format

Response to:	R2-2406217/R3-243775

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN2

To:	RAN3

Cc:	-

Contact Person:

Name:	Xiaohui Song

E-mail Address:	song.xiaohui@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks RAN3 for the reply LS on inter-node RRC message for intra-SN SCPAC in MN format. Regarding the following questions, RAN2 has discussed and would like to provide the responses as below:

Q1: Are both CG-Config and CG-CandidateList related to same configuration of intra-SN subsequent CPAC in MN format, and must be sent together in the same SN Modification Required XnAP message?

RAN2 response: The CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format, while CG-CandidateList message is used to transfer the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format. These two messages are related to different configuration for intra-SN SCPAC in MN format. From RAN2 perspective, these two messages should be sent together in the same SN Modification Required XnAP message.

Q2: If both RRC messages must be sent together, is it feasible to include both CG-Config message and CG-CandidateList messages in a single RRC message (new or existing), which could then be transferred in a single SN Modification Required XnAP message?

RAN2 response: From RAN2 perspective, CG-Config message and CG-CandidateList message can not be included in a single existing inter-node RRC message. And RAN2 does not expect to introduce a new inter-node RRC message to contain these two messages together, considering that ASN.1 was frozen at last meeting.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 kindly asks RAN3 to take the above feedback into consideration and requests RAN3 to include both CG-Config and CG-CandidateList in a single SN Modification Required XnAP message.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #127-bis	14 - 18 October 2024	Hefei, CN

TSG-RAN2 Meeting #128	18 - 22 November 2024	Orlando, US