3GPP TSG-RAN2#127 	R2-2407564

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda item:		6.1.2 (NR_feMIMO-Core)

Source:	LG Electronics Inc., Ericsson

Title: 	Discussion on PHR for mTRP PUSCH repetition

Document for:	Discussion and Decision

1.	Introduction

In RAN2#125bis, RAN2 sent LS [1] to RAN1 to ask whether/when Type 3 PH value is reported for a serving cell configured with mTRP PUSCH repetition, but in reply LS [2], RAN1 informed RAN2 of that RAN1 has no consensus for the case where the serving cell is configured with mTRP PUSCH repetition and configured with two UL carriers.

In this contribution, we present our view on this issue.

2.	Discussion

In the current MAC specification, one Type 3 PH value can be obtained instead of two Type 1 PH values for a serving cell configured with mTRP PUSCH repetition, and PHR MAC CE format for mTRP PUSCH repetition also supports to report both one Type 3 PH value and two Type 1 PH values.

[TS 38.321 S 5.4.6] 4> if this MAC entity is configured with twoPHRMode: 5> if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN and the MAC entity this Serving Cell belongs to is configured with twoPHRMode: 6> obtain two values of the Type 1 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell. 5> else if this Serving Cell is configured with multiple TRP PUSCH repetition (i.e., not configured with multipanelSchemeSDM or multipanelSchemeSFN) and the MAC entity this Serving Cell belongs to is configured with twoPHRMode: 6> obtain two values of the Type 1 or the value of Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell. [TS 38.321 S 6.1.3.51]

However, in TS 38.213, it is not clear whether/when one Type 3 PH value is provided for a serving cell configured with mTRP PUSCH repetition instead of two Type 1 PH values, while it is clearly specified that two Type 1 PH value is provided for the serving cell configured with mTRP PUSCH repetition.

If a UE is provided twoPHRMode, and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' on active UL BWP b of carrier f of serving cell c, the UE provides two Type 1 power headroom reports in a slot n, where - if the UE provides a first Type 1 power headroom report for an actual PUSCH repetition of a PUSCH transmission starting earliest in slot n that is associated with one SRS resource set, - if the UE transmits PUSCH repetitions associated with the other SRS resource set in slot n, the UE provides a second Type 1 power headroom report for a first actual PUSCH repetition associated with the other SRS resource set that overlaps with slot n - else, the UE provides a second Type 1 power headroom report for a reference PUSCH transmission associated with the other SRS resource set, where - if the other SRS resource set is the first SRS resource set, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS or is obtained from PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=0. If the UE is provided dl-OrJointTCI-StateList or TCI-UL-State that indicate a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, the UE provides the second Type 1 power headroom report using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State. - else, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 1, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 1 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS or is obtained from PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList2 if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=1 if the UE is provided twoPUSCH-PC-AdjustmentStates, or l=0 if the UE is not provided twoPUSCH-PC-AdjustmentStates. If the UE is provided dl-OrJointTCI-StateList or TCI-UL-State that indicate a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, the UE provides the second Type 1 power headroom report using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the second TCI-State or TCI-UL-State. - else, if the UE provides a Type 1 power headroom report for a reference PUSCH transmission associated with the first SRS resource set, the UE provides a Type 1 power headroom report for a reference PUSCH transmission associated with the second SRS resource set, where - for the first Type 1 power headroom report, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS, or is obtained from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=0. - for the second Type 1 power headroom report, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 1, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 1 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS, or is obtained from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList2 if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=1 if the UE is provided twoPUSCH-PC-AdjustmentStates or l=0 if the UE is not provided twoPUSCH-PC-AdjustmentStates - if a UE is provided dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, the UE provides the first or the second Type 1 power headroom reports using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State or with the second TCI-State or TCI-UL-State, respectively, if the reference PUSCH transmission is associated with the first TCI-State or TCI-UL-State or with the second TCI-State or TCI-UL-State, respectively

In RAN2#125bis, RAN2 discussed this issue and sent LS [1] to RAN1 to ask whether/when Type 3 PH value can be reported for the serving cell, and RAN2 received a reply LS [2] as follows.

Response to RAN2 about Enhanced Multiple Entry PHR for multiple TRP MAC CE for Rel-17 mTRP PUSCH repetition: Question a: Whether UE can provide one type 3 PH value instead of two type 1 PH values for a serving cell that is configured with mTRP PUSCH repetition? Answer on Question a: According to current RAN1 specification, for a serving cell configured with mTRP PUSCH repetition and configured with a single UL carrier, the UE cannot provide one type 3 PH value instead of two type 1 PH values for the serving cell. For a serving cell configured with mTRP PUSCH repetition and configured with two UL carriers, RAN1 has no consensus on whether/how current RAN1 specification can support the UE to report one type 3 PH value instead of two type 1 PH values. Question b: If answer to Question a is yes, in which case will the UE report the type 3 PH value for this serving cell, and in which case will the UE report two Type 1 PH values for this serving cell.

According to RAN1 reply, for single UL carrier case, i.e., a serving cell configured with mTRP PUSCH repetition and configured with a single UL carrier, RAN1 confirms that the UE cannot provide one Type 3 PH value instead of two Type 1 PH values.

However, for two UL carrier case, i.e., a serving cell configured with mTRP PUSCH repetition and configured with two UL carriers, RAN1 did not reach a consensus on whether/how the UE can report one Type 3 PH value instead of two Type 1 PH values. Thus, RAN2 needs to decide whether/how to report one Type 3 PH value for two UL carrier case.

Proposal 1. RAN2 decides whether/how to report one Type 3 PH value for a serving cell configured with mTRP PUSCH repetition and configured with two UL carriers.

In our view, it would be better not to obtain one Type 3 PH value and to obtain only two Type 1 PH value for two UL carrier case, i.e., two Type 1 PH values are obtained for both single carrier case and two UL carrier case.

First, from RAN2 perspective, the ambiguity of RAN1 specification can be avoided.

In our understanding, Type 1 PH value is provided for a UL carrier configured for PUSCH transmission and Type 3 PH value is provided for a UL carrier configured for SRS transmission only. If a serving cell is configured with two UL carriers and is configured with mTRP PUSCH repetition, at least one UL carrier may be configured for PUSCH transmission and two Type 1 PH values are always provided for the UL carrier configured for PUSCH transmission of the serving cell.

Therefore, regardless of whether Type 3 PH value is provided or not, the MAC entity can always obtain two Type 1 PH values for the serving cell configured with mTRP PUSCH repetition and configured with two UL carriers.

Second, the specification impact can be minimized. As RAN1 confirms that one Type 3 PH value is not provided for single UL carrier case, if RAN2 decides to support to obtain one Type 3 PH value for two UL carrier case, the MAC specification may be complicated to specify separately the UE behaviour for single UL carrier case and two UL carrier case. However, we are not sure whether there is a big gain compared to the complication from Type 3 reporting.

If RAN2 decides to obtain two Type 1 PH values for two UL carrier case, the issue can be resolved by removing the UE behaviour to obtain Type 3 and slightly modifying the format and the description of PHR MAC CE for mTRP, e.g., Type X to Type 1 for PH2.

Proposal 2. Two Type 1 PH values are always obtained for a serving cell configured with mTRP PUSCH repetition, if the MAC entity the serving cell belongs to is configured with twoPHRMode.

In addition to Type 3 issue, one change is needed further for the format of PHR MAC CE for mTRP.

Currently, in the format of PHR MAC CE for mTRP, there are Type 2 PH1 and PH2 fields for SpCell.

However, Type 2 PH2 field for SpCell is not needed because Type 2 PH value for SpCell is included for E-UTRA MAC entity and the E-UTRA MAC entity is not configured twoPHRMode. Thus, Type 2 PH2 value for SpCell is not provided.

Therefore, Type 2 PH2 field for SpCell should be removed in the current PHR MAC CE for mTRP.

Proposal 3. Remove Type 2 PH2 field for SpCell in PHR for mTRP MAC CE.

Based on Proposal 1-3, we provide CR R2-2407565 and R2-2407566 with some editorial [3][4].

3.	Conclusion

In this document, we present our views on PHR for mTRP PUSCH repetition, and made proposals as follows.

Proposal 1. RAN2 decides whether/how to report one Type 3 PH value for a serving cell configured with mTRP PUSCH repetition and configured with two UL carriers.

Proposal 2. Two Type 1 PH values are always obtained for a serving cell configured with mTRP PUSCH repetition, if the MAC entity the serving cell belongs to is configured with twoPHRMode.

Proposal 3. Remove Type 2 PH2 field for SpCell in PHR for mTRP MAC CE.

4.	Reference

- R2-2403968	LS on type 3 PH value for the serving cell configured with mTRP	ZTE

- R2-2406214	Reply to RAN2 LS on type 3 PH value for the serving cell configured with mTRP	RAN1

- R2-2407565	CR for Correction on PHR for mTRP PUSCH repetition (R17)	LG Electronics Inc., Ericsson

- R2-2407566	CR for Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc., Ericsson