3GPP TSG-RAN WG2 Meeting #127	R2-2407567

Maastricht, Netherlands, 19th August – 23th August, 2024 	  Revision of R2-2406942

Agenda Item:	8.2.2

Source:	Huawei,

HiSilicon, Wiliot Ltd., Quanray, Telit Communications S.p.A., Nordic Semiconductor ASA,

China Unicom, China Telecom, China Southern Power Grid, NTT DOCOMO INC., CMCC, LG Uplus,

LG Electronics Inc., Spreadtrum Communications, Xiaomi, ASUSTeK, Kyocera, ETRI, CAICT, NEC, Apple, TCL, Google Inc.

Title:	Views on the segmentation of A-IoT

Document for: Discussion and Decision

1	Introduction

Following agreements are achieved related to the segmentation of A-IoT:

RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

2	Discussion

In the RAN TR 38.848, the RAN design targets are defined, and for A-IoT devices, the maximum message size to be received or to be transmitted is approximately 1000 bits, as shown below:

## 5.5 Maximum message size The design target of maximum message size is approximately 1000 bits to be received by the Ambient IoT device, and approximately 1000 bits to be transmitted from the Ambient IoT device, based on the maximum application layer packet size.

Observation 1:	The message size is less than 1000bits in SA1 requirement and RAN design target.

Considering that the D2R data sent by A-IoT device is typically small (e.g. less than 1000bits), the reader should be able to schedule enough resources for an A-IoT device to perform its D2R transmission in its occupied transmission opportunity/slot. Even if there is some limitation on the maximum TBS from physical layer, it should be considered as the AS capacity to deliver the upper layer message. Then, the upper layer message design should not exceed this physical layer maximum TBS, i.e., regardless the RAN1 conclusion on the maximum TBS, the maximum message size should be able to be accommodated by the maximum TBS.

Observation 2:	RAN2 assumes that the physical layer designed maximum TBS can accommodate the maximum message size.

The possible D2R message with large size will be probably the data response to the read command. Assuming that the upper layer of A-IoT device can be aware of the maximum TBS, the upper layer of A-IoT can perform segmentation of the D2R message with large size considering the maximum TBS. In that case, it can be upper layer implementation (via the read command) to fetch a small size of data (i.e. a fraction of the data) several times. For instance, the server can send four read sub-command one-by-one with each to fetch 250bits data fraction, if there is the need to read 1000bits data.

Observation 3:	The upper layer (e.g. NAS-like or application layer) can do the “segmentation”, e.g. the read command can fetch a small size of data (i.e. a fraction of the data) several times. Upper layer “segmentation” can be done without AS involvement (e.g. security can also be per upper layer segment).

Before the BS reader allocates different TBS for the D2R transmission, the reader can have some assisting information on the message size from core network (or the message size reporting from device). Based on the rough understanding on the expected message size, the reader can and should allocate sufficient TBS for the device, so that the device can transmit its D2R data within one transmission opportunity.

In the case that A-IoT device can determine the transmission size, the A-IoT device determines the D2R transmission size according to the message size. If the data size is less than the maximum TBS size, the remaining resource may not be used and the end of data can be indicated, for example, by a postamble.

Observation 4:	RAN2 can assume that the A-IoT device is able to transmit its D2R data within one transmission opportunity:

the reader implementation can and should ensure it, in case of reader allocating TBS;

the device can manage it, in case of device deciding the actual TBS.

If we really need to consider the rare case, when the A-IoT device cannot sent the whole message by one D2R transmission opportunity, e.g. due to some allocated TBS limitation or due to energy limitation, the straight forward A-IoT device behaviour should be just giving up/skipping this D2R transmission. Then, the reader can always handle this A-IoT device by another scheduling or re-access. Note that the small energy storage not even being able to transmit one single TB is not proper implementation design.

Another observation on the RAN1 design is their agreed assumptions for evaluation: “For the link level simulation in coverage evaluation, {20 bits, 96 bits, 400 bits} are considered for message size.” For reference data rate, only “1 kbps (M), 5 ~ 7 kbps (M)” are currently agreed by RAN1. With the assumption of such typical TBS size and data rate, it is reasonable to assume below observation.

Observation 5:	The A-IoT device can just skip the D2R transmission opportunity, if it considers the message cannot be transmitted by one shot in some rare case (e.g. due to TBS limitation etc.), and waits for the next access opportunity provided by the reader.

It is critical to understand that the segmentation functionality requires some additional AS buffer/register:

- The AS buffer to store the segmentation position of the previous D2R transmission (or even the whole un-transmitted segments, e.g. about 500-bits for half split segmentation and about 1000-bits for the worst case, if an approximate 1000bits message needs to be segmented and buffered.);

- The AS buffer to store the read command itself, so that the A-IoT device can construct the next segment according to the same read command.

In addition, segmentation also complicates the security handling on the device and requires energy harvesting AIoT device to perform memory retention of this buffer and segmentation info for the entire segmentation process, which can take multiple Tx opportunities.

Therefore, it is the common concerns and the view from the co-sourcing companies that the segmentation requires additional AS buffer in the A-IoT device, which, together with its operations, requires unnecessary and not negligible complexity in the A-IoT device.

Observation 6:	Segmentation requires additional AS buffer in the A-IoT device (e.g. about 1000bits), which, together with its operations, requires unnecessary and not negligible complexity in the A-IoT device.

With the above analysis, proposal is given based on the following logics:

- The message size and solution design for A-IoT does not require segmentation;

- The segmentation functionality, if introduced, will cause the A-IoT device additional not negligible complexity.

Proposal 1:	For A-IoT device, RAN2 assumes that there is no need of AS segmentation functionality. (This assumption can be revisited, if needed, based on other WGs further progress.)

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	The message size is less than 1000bits in SA1 requirement and RAN design target.

Observation 2:	RAN2 assumes that the physical layer designed maximum TBS can accommodate the maximum message size.

Observation 3:	The upper layer (e.g. NAS-like or application layer) can do the “segmentation”, e.g. the read command can fetch a small size of data (i.e. a fraction of the data) several times. Upper layer “segmentation” can be done without AS involvement (e.g. security can also be per upper layer segment).

Observation 4:	RAN2 can assume that the A-IoT device is able to transmit its D2R data within one transmission opportunity:

the reader implementation can and should ensure it, in case of reader allocating TBS;

the device can manage it, in case of device deciding the actual TBS.

Observation 5:	The A-IoT device can just skip the D2R transmission opportunity, if it considers the message cannot be transmitted by one shot in some rare case (e.g. due to TBS limitation etc.), and waits for the next access opportunity provided by the reader.

Observation 6:	Segmentation requires additional AS buffer in the A-IoT device (e.g. about 1000bits), which, together with its operations, requires unnecessary and not negligible complexity in the A-IoT device.

Proposal 1:	For A-IoT device, RAN2 assumes that there is no need of AS segmentation functionality. (This assumption can be revisited, if needed, based on other WGs further progress.)