3GPP TSG-RAN WG2 Meeting #127	R2-2407571
Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	9.1

Source: 		Vice Chairman (Samsung)

Title: 			Report from session on V2X/SL, R18/19 MOB, and R19 NES

Document for:	Approval

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

## R18 NBC Change

1. ltm-FastProcessingConfig-r18 is corrected to per UE capability (see R2-2407584).

2. fr1-r18, fr2-r18, fr1-AndFR2-r18 in ltm-FastUE-Processing-r18 are corrected to OPTIONAL (see R2-2407584)

3. pdcch-RACH-AffectedBands-r18, pdcch-RACH-SwitchingTimeList-r18, pdcch-RACH-PrepTime-r18 are corrected to all independent (see R2-2407584)

## R19 NES OD-SIB1 study conclusion

RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

## Approved outgoing LSs

R2-2407591	LS on IUC Scheme-2 and Random Selection	RAN2	LSout	To: RAN1

R2-2407606	LS on L3 measurement based LTM mobility support over F1	RAN2	LSout	To: RAN3

## List and Status of Offline/Email Discussions

SUPER-SHORT EMAIL DISC:

[POST127][110][MOB] (Intel)

Scope: Update the draft CRs (if needed), discuss and finalize them. Prepare a LS to RAN1/4 to inform capability agreement and ask the question (see the discussion on R2-2406476).

Intended outcome: Endorsed 38.331 CR in R2-2407599 and 38.306 CR in R2-2407600. Approved LS in R2-2407601.

Deadline: Super short email discussion.

SHORT EMAIL DISC:

[POST127][101][V2X/SL] (LG)

Scope: Discuss changes in R2-2407381 (only correction 1) and capture all agreements made in RAN2#127. => It is extended to short email discussion. Capture all RAN2#127 agreements.

Intended outcome: MAC CR in R2-2407581. => Agreed MAC CR in R2-2407603.

Deadline: Short offline discussion.

[POST127][105][MOB] (Ericsson)

Scope: Discuss P1, P2, and P3 in R2-2406332, P3 in R2-2406337, P1, P2, P3, P6, P7 in R2-2407050 and R2-2407175. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements.

Intended outcome: Agreed RRC CR in R2-2407585. Discussion summary in R2-2407595 (if needed). If proposals are straight-forward, it is ok to directly discuss based on the CR (up to offline discussion rapporteur). => In short email discussion, no need of discussion summary.

Deadline: Comeback in Thursday CB session => Short email discussion

[POST127][106][MOB] (ZTE)

Scope: Discuss R2-2406417 and R2-2407091. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements

Intended outcome: Endorsed 37.340 CR/TP in R2-2407586. Discussion summary in R2-2407596 (if needed). If proposals are straight-forward, it is ok to directly discuss based on draft TP/CR (up to offline discussion rapporteur). => Agreed 37.340 CR in R2-2407604. In short email discussion, no need of discussion summary.

Deadline: Comeback in Thursday CB session. => Short email discussion

[POST127][111][MOB] (CATT)

Scope: Prepare a LS to SA3 to inform RAN2 agreements to ask them to take it into account for their works.

Intended outcome: Approved LS in R2-2407602.

Deadline: Short email discussion.

LONG EMAIL DISC:

[POST127][109][NES] (OPPO)

Scope: For each direction (N extension vs PF bundling): 1) List proposed options, 2) Discuss to understand each option better, 3) Down select options, 4) Discuss pros and cons for each direction or down-selected options (with the consideration of UE impacts, system impacts and specification job). F2F offline discussion in Brk3 room. => It is extended to long email discussion to continue detailed analysis of pros and cons for each direction.

Intended outcome: Discussion summary in R2-2407598. => Updated discussion summary

Deadline: Comeback in Thursday CB session. => Long email discussion.

COMPLETED AT OFFLINE DISC:

[AT127][102][V2X/SL] (OPPO)

Scope: Discuss changes in R2-2406265 and R2-2407372, capture all agreements made in RAN2#127 and update RRC CR with merging all corrections (if agreeable).

Intended outcome: RRC CR in R2-2407582.

Deadline: Comeback in Thursday CB session. => Completed.

[AT127][103][V2X/SL] (IDC)

Scope: Discuss changes in R2-2407388, R2-2406554 and update CR with merging all corrections (if agreeable)

Intended outcome: 38.300 CR in R2-2407583.

Deadline: Comeback in Thursday CB session. => Completed.

[AT127][104][MOB] (MediaTek, ZTE)

Scope: Discuss proposals in R2-2406355 and R2-2406418. Make agreeable proposals based on companies’ understanding and inputs. F2F offline discussion in Brk3.

Intended outcome: Discussion summary (including TP if possible) in R2-2407584.

Deadline: Comeback in Thursday CB session. => Completed.

[AT127][107][V2X/SL] (OPPO)

Scope: Ask RAN1 if configuration of both IUC scheme 2 and random resource selection is supported or not.

Intended outcome: LS in R2-2407591

Deadline: Email approval (until end of 8/21 Wednesday) => Completed.

[AT127][108][MOB] (Huawei)

Scope: Discuss R2-2406331, R2-2406349, P5, P6 in R2-2406439, R2-2407197, P1 in R2-2407433, and R2-2406853. Update the CR with merging all corrections (if agreeable).

Intended outcome: Endorsed MAC CR/TP in R2-2407587. Discussion summary in R2-2407597 (if needed). If proposals are straight-forward, it is ok to directly discuss based on draft CR/TP (up to offline discussion rapporteur).

Deadline: Comeback in Thursday CB session. => Completed.

## 4.2	V2X and Sidelink corrections Rel-15 and earlier

REL-15 and Earlier WIs related to V2x and Sidelink are in scope but not listed explicitly (long list).

This Agenda Item is treated in the V2X and Sidelink Breakout session

Tdoc Limitation: 1 tdocs

## 5.2	NR V2X

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Aug 20; WID: RP-200129).

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please contact / coordinate with CR rapporteur company first for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

Tdoc Limitation: 1 tdocs

R2-2407464	MAC correction on resource selection	LG Electronics Inc.	CR	Rel-16	38.321	16.16.0	1915	-	D	5G_V2X_NRSL-Core

R2-2407472	MAC correction on resource selection	LG Electronics Inc.	CR	Rel-17	38.321	17.9.0	1916	-	A	5G_V2X_NRSL-Core

R2-2407475	MAC correction on resource selection	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1917	-	A	5G_V2X_NRSL-Core

- In cover page, typos of “reources pool” should be changed to typos of “resources pool”

- Other specs affected should be clicked for “N”

- Agreed in R2-2407588, R2-2407589, R2-2407590 with the changes above.

R2-2406699	Correction to MAC on cast type	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.16.0	1888	-	F	5G_V2X_NRSL-Core

- Agreed.

[Huawei]: We may consider to reflect it into NOTE2. Note we have similar note for HARQ enabled/disabled in Rel-16, which indicates is disabled for MAC CE. [Apple]: Prefer Huawei’s suggestion. [Session chair]: Note this change was agreed last meeting for Rel-17. It may be simpler to mimic same change for Rel-16.

## 6.6	NR Sidelink enhancements

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

Tdoc Limitation: 1 tdoc

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please contact / coordinate with CR rapporteur company first for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

R2-2406262	Correction on resource (re)selection for IUC 	OPPO	CR	Rel-17	38.321	17.9.0	1873	-	F	NR_SL_enh-Core

R2-2406263	Correction on resource (re)selection for IUC 	OPPO	CR	Rel-18	38.321	18.2.0	1874	-	A	NR_SL_enh-Core

For the first change:

[LG]: Consider the current text has no problem. [OPPO]: The removed condition can be also applicable to the three conditions. It is not exclusive condition. That’s why “the latency requirement of the Sidelink Inter-UE Coordination Information transmission (if applicable)” is added for each three cases with the removal of the exclusive condition. [LG]: Current MAC specifies two parts of resource selections. One is the resource selection for the data and another one is explicitly for IUC Information. If we remove the condition, it becomes less clear for the resource selection for IUC. [Apple]: Share the same view as LG. [OPPO]: Want to have more time to discuss with other companies. Would like to revisit it next meeting.

- Revisit it next meeting.

For the second change:

[LG]: Based on RAN1 agreement, random resource selection with sl-interUECoordinationScheme2 is not supported, so if we really want to align with RAN1, we can simply remove “random selection” in the first paragraph. [OPPO]: Have somewhat different view on RAN1 status, however it can be one of possible solutions. [OPPO, LG]: It is better to ask RAN1 if configuration of both IUC scheme 2 and random resource selection is supported or not.

- LS is sent to RAN1 to ask if configuration of both IUC scheme 2 and random resource selection is supported or not.

[AT127][107][V2X/SL] (OPPO)

Scope: Ask RAN1 if configuration of both IUC scheme 2 and random resource selection is supported or not.

Intended outcome: LS in R2-2407591

Deadline: Email approval (until end of 8/21 Wednesday) => Completed.

R2-2407591	LS on IUC Scheme-2 and Random Selection	RAN2	LSout	To: RAN1

- Approved.

R2-2406514	Correction on prioritization between SL transmission and MAC CE-triggered SR	ASUSTeK	CR	Rel-17	38.321	17.9.0	1886	-	F	NR_SL_enh-Core

R2-2406515	Correction on prioritization between SL transmission and MAC CE-triggered SR	ASUSTeK	CR	Rel-18	38.321	18.2.0	1887	-	A	NR_SL_enh-Core, NR_SL_enh2

- Noted.

[LG]: Based on RAN2 previous agreement on prioritization rule between UL MAC CE and SL, RAN2 agreed UL MAC CE is always considered as higher priority than UL threshold and it was agreed that SL is only compared to SL threshold, e.g. if SL has higher priority than SL threshold, SL is prioritized. Otherwise UL MAC CE is prioritized. The current MAC is ok. [Samsung]: Is it realistic scenario that NR-U (or IAB) and SL/V2X co-exist? If not, we may not need to consider LBT failure MAC CE and Pre-emptive BSR MAC CE. [Ericsson]: Share the view with Samsung. [CATT]: Prioritization rule was introduced in Rel-16 and we haven’t considered new MAC CEs introduced from Rel-17. It is better to have more time to think of. [OPPO]: Agree with LG, however it was for PUSCH with UL MAC CE. Cannot recall whether RAN2 discussed and made a specific decision for SR. [OPPO]: Remember SR prioritization actually inherited from URLLC. Need to double check. [CATT]: In Rel-16, we only considered BSR MAC CE then the priority of the corresponding LCH data was considered in prioritization rule. [Huawei]: Remember SR triggered by LCH data was only considered in Rel-16. We haven’t discussed other cases, e.g. SR triggered by UL MAC CE. It is good to discuss it in common session. [OPPO]: Based on companies inputs, it is not for correction. It is for new feature, which should be discussed for TEI. [Session chair]: Let’s have more time to think of and come back next meeting. It would be good to discuss in common session. Target agenda item should be discussed with RAN2 chair-lady.

R2-2407011	Correction on SL IUC and SL DRX configurations for SL enhancements	Huawei, HiSilicon	CR	Rel-17	38.331	17.9.0	4913	-	F	NR_SL_enh-Core

R2-2407012	Correction on SL IUC and SL DRX configurations for SL enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4914	-	A	NR_SL_enh-Core

- Agreed.

[Ericsson]: Support the proposal. [ZTE]: Many co-existence features are captured in stage 2. Prefer to capturing it into stage 2. [Huawei]: It is configuration aspect, so prefer RRC. [Vivo]: UE behaviour is clear in MAC so we may not need this CR. [Toyota]: It should be specified somewhere, in either stage 2 or stage 3.

R2-2407019	Correction to MAC on HARQ feedback indicator	Ericsson	CR	Rel-17	38.321	17.9.0	1895	-	F	NR_SL_enh-Core

R2-2407246	Correction to MAC on HARQ feedback indicator	Ericsson	CR	Rel-18	38.321	18.2.0	1906	-	A	NR_SL_enh-Core

- NOTE 2 will be updated to “NOTE 2:	HARQ feedback enabled/disabled indicator is set to disabled for the transmission of a MAC PDU only carrying Sidelink MAC CE(s), e.g. Sidelink CSI reporting MAC CE, Sidelink DRX Command MAC CE, Sidelink Inter-UE Coordination Request MAC CE or Sidelink Inter-UE Coordination Information MAC CE.”

- With the NOTE 2 above, agreed in R2-2407592 and R2-2407593.

R2-2407020	Correction to MAC on cast type	Ericsson	CR	Rel-18	38.321	18.2.0	1896	-	A	NR_SL_enh-Core	Withdrawn

R2-2407405	Correction on resource selection for IUC 	LG Electronics Inc.	CR	Rel-17	38.321	17.9.0	1909	-	F	NR_SL_enh-Core	Withdrawn

R2-2407412	Correction on resource selection for IUC 	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1910	-	A	NR_SL_enh-Core	Withdrawn

## 7.4	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

Time budget: 0 TU)

Tdoc Limitation: 2 tdocs.

### 7.4.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2406217	Reply LS on intra-SN SCPAC in MN format (R3-243775; contact: ZTE)	RAN3	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

- Noted.

R2-2406227	Reply LS on LTM L1 intra and inter-frequency measurements (R4-2410303; contact: Ericsson)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2	Cc:RAN1

- Noted.

### 7.4.2	Control plane corrections

Including stage 2 and control plane (e.g. RRC) corrections (including [Post126][514][R18MobE]). A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note RRC CR rapporteur’s summary and suggestion (based on the submitted contributions) may be provided.

UE measured TA:

P4, P5, P6 and P7 in R2-2406337 (MediaTek)

Proposal 4: In clause 5.3.5.18.6, correct the procedural text so that the UE stops the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

- Noted. RRC CR rapporteur will see if anything is missed in CR preparation.

[LG]: Whether to start/stop UE-based TA measurement is up to UE implementation. [Session chair]: Based on the proposed TP, “else” includes “if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID”, then shouldn’t the UE continue UE-based TA (or the UE should be configured with UE-based TA)? [Qualcomm]: Nothing is broken even though the UE performs UE-based TA for SpCell. [Vivo]: Understand the motivation, however the TP seems not so accurate.

Proposal 5: In clause 5.3.5.18.6, correct the procedural text so that the UE does not start the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

- Intention is agreed and we will add a note. Detailed wordings can be discussed as part of RRC CR preparation (taking Qualcomm’s suggestion into account).

[Qualcomm]: We can simply add a note, e.g. UE is not expected to perform UE-based TA measurement for SpCell (instead of updating many changes in the procedure).

Proposal 6: In clause 5.3.5.18.6, remove the unnecessary procedural text to stop the UE-based TA measurements when the LTM candidate which becomes the current SpCell does not have field ltm-UE-MeasuredTA-ID.

- Noted. RRC CR rapporteur will see if anything is missed in CR preparation.

Proposal 7: RAN2 to discuss if and how to capture handling of the UE-based TA measurements for the scenario where the SpCell changes by L3 mobility procedure (handover/PSCell change) while LTM is configured.

- Not pursued.

[Qualcomm]: Upon L3 HO execution, does the UE still keep LTM configuration? [MediaTek]: Yes. We can have similar texts at L3 HO. [CATT, OPPO, LG, Apple]: Understand the intention, however thinks nothing is needed. We can count on NW implementation. We have similar situation for SCPAC and we count on NW implementation. NW configuration should be reasonable/updated accordingly.

Proposal 1: RAN2 to clarify that if there is only one TCI state in ltm-CandidateConfig for the candidate cell indicated by LTM Cell Switch Command MAC CE, the network ensures this TCI state is identical with the TCI state indicated by LTM Cell Switch Command MAC CE.

- Intention is correct. No spec change is needed.

P1a, P2, P3 in R2-2407200 (Huawei)

Proposal 1a: Confirm the understanding that the information to lower layers that the UE is configured with UE-based TA measurements for a candidate remains valid after LTM cell switch, if there is no new explicit information.

- Agreed.

Proposal 2: Capture in the LTM candidate configuration addition/modification procedures that, when ltm-UE-MeasuredTA-ID is not present for a LTM candidate configuration, the UE informs lower layers that UE-based TA measurement is not configured. (See Annex TP)

- Agreed with intention. Detailed wordings and MediaTek’s concerned case will be further discussed as part of RRC CR preparation.

[MediaTek]: It is defined as optional, M, so if not removed earlier, the information should be continued even though it is absent. [Huawei]: We have a case it is removed and then added again. In the case, it should inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

Proposal 3: Capture that when the network configures LTM candidates with ltm-UE-MeasuredTA-ID, the network ensures that the UE has a value stored in ltm-ServingCellUE-MeasuredTA-ID.

- Agreed. It can be included in the corresponding field description.

LTM Configuration:

P1 in R2-2406726 (Apple)

Proposal 1: RAN2 clarify whether the nested ltm-Config is supported or not.

- Nested ltm-Config (in both reference configuration and candidate configuration) is not supported.

- Add the clarification above into the corresponding field description.

[MediaTek]: Understand the nested LTM-config is not the intention in R18 LTM design, and it can not work in subsequent LTM. [Vivo]: Ok with the conclusion. Wonders what’s spec impact. Adding the corresponding clarification would be sufficient.

Proposal 2: If the nested ltm-Config is supported, RAN2 confirm the understanding in Observation 3 and Observation 4 on how to handle the nested ltm-Config.

Observation 3: According to current spec description, the nested ltm-Config in ltm-ReferenceConfiguration is useless.

Observation 4: According to current spec description, UE will update the ltm-Config by the nested ltm-Config in ltm-CandidateConfig at LTM cell switch execution.

Proposal 3: Confirm that that network may provide the ltm-Config without ltm-CSI-ResourceConfig.

- Agreed.

SpCell reporting:

P2 in R2-2406337 (MediaTek)

Proposal 2: In the field description of spcellInclusion, clarify that it cannot be set to True if SpCell resources are not included in LTM-CSI-ResourceConfig.

- Agreed.

[OPPO, Samsung]: Nothing is broken. The existing condition for spcellInclusion is when SpCell is included in candidate LTM configuration, which sounds already cover proposal 2. [MediaTek]: “SpCell is included in candidate LTM configuration” doesn’t mean SpCell resources are included in LTM-CSI-ResourceConfiguration. [Huawei, Vivo]: Support the proposal.

Unified TCI state type:

P3 in R2-2406337 (MediaTek)

Proposal 3: In the field description of unifiedTCI-StateType, specify that the UE applies value separate if unifiedTCI-StateType is absent in LTM-TCI-Info. In addition, change the need code of unifiedTCI-StateType to Need S.

- No change of need code.

- Add a clarification NW always signals this IE in the field description. Detailed wordings are up to RRC CR rapporteur.

[CATT]: Even with change of need code to S, consider it is NBC change. [LG]: Huawei proposed different solution, i.e. just network makes sure it is signalled. Prefer Huawei proposal. [Huawei]: In Rel17 MIMO, it was mandatory IE. NW always signalling is quite aligned with Rel17. [Qualcomm]: Share the view with Huawei. [Samsung]: LTM should be always operated based on unified TCI. In that point of view, Huawei’s option sounds better.

Early decoding:

P1 in R2-2406847 (Nokia)

Proposal 1: NW indicates which cells (e.g. by providing a list of cells) are subject to early ASN.1 decoding. This indication is included in the RRCReconfiguration message used to configure LTM.

- RAN2 understands NW indicating which cells (e.g. by providing a list of cells) for early ASN.1 decoding is not required in Rel-18.

[LG]: What was motivation to introduce this capability? In early phase, we assumed it is up to UE implementation. [Nokia]: It was RAN4 decision. [Huawei]: There is no definition regarding how fast. [Nokia]: There are numbers defined in RAN4 (either 10ms or 0ms). [Huawei]: Think NW knowledge of which candidate cell is configured for early decoding is not really useful. [Qualcomm, Samsung]: Share the view with Huawei. Sounds optimization that is not essential. [OPPO]: With Nokia proposal, which node will configure cells for early decoding, source cell or target cell? [Nokia]: Consider it should be the source cell. [LG]: UE implementation to decide which cell is enough. [Apple]: Think even NW configures for a candidate cell, cell switch may happen to the other candidate cell. Then how it works? [Nokia]: Think with RAN2 conclusion, it is good to inform RAN4. [Ericsson]: Don’t see the need to send LS to RAN4.

P5 in R2-2407200 (Huawei)

Proposal 5: The network does not indicate which LTM candidate configurations can be fast processed by the UE. Fast RRC processing for LTM candidate configurations is up to UE implementation.

Configuration for UL early sync:

R2-2407370	Discussion on PRACH occasion validation for LTM	Qualcomm Incorporated	discussion

Proposal 1: RAN2 to agree adding a new IE ltm-tdd-UL-DL-ConfigurationCommon-r18 in LTM-Candidate-r18.

- Noted. If we receive RAN1 LS this meeting, we can reflect new parameters in RRC CR (in short email discussion on RRC CR).

[Ericsson]: Technically agree with the proposal. However, it is under RAN1 discussion this meeting. And it seems more parameters are missed. It is better to wait for RAN1 inputs.

L2 reset only in inter-DU only or allowed for intra-DU also?

P5 in R2-2407050 (Samsung)

Proposal 5: RAN2 is kindly asked to confirm that L2 reset is also applicable for the intra-DU LTM case, and an LS can be sent to RAN3.

- L2 reset (RLC reestablishment and PDCP data recovery) is also applicable for the intra-DU LTM case.

[Ericsson, Huawei]: Proposal 5 is correct. Do we really need to send LS to RAN3? [Samsung]: RAN3 has already discussed it in the past. It would be good to inform our agreement officially. [Session chair]: Showing RAN2 agreement captured in the meeting reporting should be sufficient.

Measurement gap for L1 and L3 measurements:

P8 and P9 in R2-2407050 (Samsung)

Proposal 8: In NR-DC, MN considers the LTM configuration in SN (and MN) and the capabilities 39-2 and 39-3 while configuring measurement gaps.

Proposal 9: SN can inform the NR-ARFCN, physical cell-id (PCI) and SSB configuration of its LTM candidate cells to MN in CG-Config INM.

P1 in R2-2407176 (Ericsson)

Proposal 1: The existing measurement gaps framework is re-used for LTM without the need to introduce new signalling.

[Ericsson]: Understand typically if MR gap is required for L3 measurement for a given frequency, it would be also required for L1 Measurement. [Qualcomm]: It is somewhat risky assumption since they defined separate capabilities for L1 and L3. We need to carefully check. [CATT, ZTE]: Share the view with Ericsson. [Qualcomm]: Would like to have some time for check until Thursday CB session. [Apple]: Want to clarify gap is same but NW will consider LTM capability also for the gap configuration. [Ericsson]: Confirms it.

- Comeback in Thursday CB session.

- After offline discussion, P1 in R2-2407176 is agreed.

SRB5 handling during SCPAC:

P11 in R2-2407050 (Samsung)

Proposal 11: Capture the handling of SRB5 during Subsequent CPAC execution (as in TP4).

- Agreed.

[Ericsson]: Ok with the proposal 11.

discardOnPDCP and reestablishRLC in SCPAC:

P1 in R2-2406531 (OPPO)

Proposal 1: Add the restriction that the NW does not include discardOnPDCP and reestablishRLC for SRB3 in case of SCPAC in MN format.

[Ericsson, Nokia]: NW implementation can handle it. [OPPO]: We already have similar restrictions for others. [OPPO]: After real-time offline discussion with companies, they now understand the proposal better. Would like to comeback in Thursday CB session.

- Comeback in Thursday CB session.

- After offline discussion, intention is agreed. Detailed wording (up to RRC rapporteur) can be further discussed as part of RRC CR preparation.

UL Skipping and LTM cell switch:

P2 in R2-2407176 (Ericsson)

Proposal 2: If UL skipping is configured, the UE ignore the UL skipping indication for the first transmission in the target cell during an LTM cell switch procedure, i.e. the UE always performs the first transmission.

- Not pursued.

[Apple]: Even w/o new UE behaviour, we can survive with RRC reconfiguration. [Huawei]: Remember it was already discussed and no UE behaviour was introduced. [Qualcomm, CATT]: It only happens for SN change w/o SRB3. Assume it is not often happened and we can count on NW implementation.

PDCCH monitoring upon LTM Cell Switch Command:

P3 in R2-2407176 (Ericsson)

Proposal 3: RAN2 to clarify at which point, after the reception of the LTM cell switch MAC CE, the UE starts to monitor the PDCCH.

- Not pursued.

[Xiaomi]: From UE point of view, the UE will monitor PDCCH immediately after reception of LTM Cell Switch Command. [MediaTek]: Understand UE can only monitor PDCCH after RRC processing time (upon end of region 3). [Huawei]: Understand the situation is quite similar to normal RRC reconfiguration w/o consideration of LTM switch. [Apple]: Agree with Huawei. [Nokia]: Agree with Huawei and don’t see a real need to capture anything.

UE Capabilities:

R2-2406476	Report of [Post126][514][R18MobE] UE capabilities Open Issues (Intel)	Intel Corporation	report	Rel-18	NR_Mob_enh2-Core

Proposal #1 (modified): No dependency between LTM and L1 measurements is captured in RAN2 specs (306, 331, 300) for both FR1 and FR2.  Inform RAN1/4 of RAN2 decisions.

Proposal #2: BC of the Intra and Inter-frequency measurements is “Option 1: The BC granularity is BC of serving cells”.

Proposal #4:  Chose option 2+4 for LTM MCG and SCG capabilities.  That is, define capabilities per band consistent across all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands. And introduce another capability bit (per UE) to indicate inter-frequency LTM (UE indicating this capability shall also support intra-freq, (MCG or SCG) LTM i.e, the default indicated by the per band bits).

Proposal #5: Send LS to RAN1 to get clarification on whether RAN1 capabilities 45-3a/4a, 45-5/5a/6 can also be different intra and inter-frequency.

- Proposal 1, 2, 4, 5 are agreed.

[ZTE]: For FR2, it would be good to have clear RAN2 agreement.

Proposal #3: Discuss whether to introduce another capability bit which indicates that UE supports inter-frequency L1 measurements only in bands of that BC

[Huawei]: One additional bit per UE would be ok and it distinguishes two cases, i) UE supports inter-F measurements only for candidate cells in the BC that inter-F measurement capability is reported and ii) UE supports inter-F measurements for any UE’s supported bands. [ZTE]: With one bit per UE capability, we need to discuss what is default meaning. Understand default should be ii) UE supports inter-F measurements for any supported bands. [Nokia]: W/o additional bit, what will happen?

- More time for discussion seems needed. We can revisit it either CB session or next meeting.

In CB session:

[Huawei]: Additional bit is per UE. It will be applicable on the BC that is indicated where L1 inter-F measurement is supported. [MediaTek]: It is not clear how per UE bit works. Also not clear where to add the bit. [Session chair]: Suggest to revisit it next meeting (with TP).

- Will be revisited next meeting (with TP).

R2-2406477	Draft 331 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.331	18.2.0	NR_Mob_enh2-Core

R2-2406478	Draft 306 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.306	18.2.0	NR_Mob_enh2-Core

[Intel]: Propose short email discussion on the draft CRs.

[POST127][110][MOB] (Intel)

Scope: Update the draft CRs (if needed), discuss and finalize them. Prepare a LS to RAN1/4 to inform capability agreement and ask the question (see the discussion on R2-2406476).

Intended outcome: Endorsed 38.331 CR in R2-2407599 and 38.306 CR in R2-2407600. Approved LS in R2-2407601.

Deadline: Super short email discussion.

R2-2406852	Discussion on LTM fast processing capabilities	Google Ireland Limited	discussion	Rel-18	38.306

Proposal 1: Clarify in the field description that maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check as specified clause 6.3 in TS 38.133.

- Agreed.

Proposal 2: Clarify the meaning of maxNumberConfigs-r18 in TS 38.306 using one of the following options:

Option 1: maxNumberConfigs-r18 represents the total number of LTMCandidateConfigs and ltm-ReferenceConfiguration for which the UE can perform early ASN.1 decoding and validity check.

Option 2 (modified): maxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133.

- Option 2 is agreed.

[Xiaomi]: Reference configuration can be included in candidate configuration. So reference configuration and candidate configuration may not be considered in separate. [MediaTek]: ASN.1 decoding and validity check for a candidate configuration already means option 2. [Apple]: What is validity check? [Google, MediaTek]: It is clear what ASN.1 decoding and validity check are according to RAN4 definition. [Intel]: Within RAN2 spec, normally when we talk about ASN.1 decoding, it already contains validity check. Question is if we need to specify validity check in RAN2 spec.

Proposal 3: maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s), and Scell(s) in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s) for MCG and SCG, that UE can store the configurations.

- Not pursued.

[MediaTek]: With the similar reason discussed for option2, we do not need to consider option3.

Proposal 4: RAN2 should discuss and clarify whether UEs not supporting LTM fast processing should indicate maxNumberStoredConfigCells-r18.

- RAN2 understands only UE supporting LTM fast processing indicates maxNumberStoredConfigCells-r18.

[MediaTek]: This UE capability information is for fast processing of LTM candidate cell. So if LTM fast processing is not supported, there is no point to report it. [Xiaomi]: Is it related to 39-6 or 39-7? [Session chair]: Think it is 39-6 based on RAN4 capability excel.

R2-2406355	Leftover LTM UE capability issues	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406418	Inter-node coordination on L1 measurement for LTM	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

[AT127][104][MOB] (MediaTek, ZTE)

Scope: Discuss proposals in R2-2406355 and R2-2406418. Make agreeable proposals based on companies’ understanding and inputs. F2F offline discussion in Brk3.

Intended outcome: Discussion summary (including TP if possible) in R2-2407584.

Deadline: Comeback in Thursday CB session. => Completed.

R2-2407584	Discussion summary on [AT127][104][MOB]	MediaTek, ZTE	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Clarify that the capability value of ltm-FastProcessingConfig-r18 indicates the maximum number across all the supported bands, rather than the current band. Adopt the TP 1 in this paper.

[Qualcomm, MediaTek]: RAN4 indicates it is per UE capability with FR1 and FR2 differentiation. [Intel]: Then is it ok to change it to per UE capability (with FR1/2 differentiation)?

- Assuming it is correct that RAN4 indicates it is per UE capability with FR1/FR2 differentiation, change this capability to per UE capability with FR1/FR2 (NBC change). TDD/FDD differentiation will be checked. Detailed ASN.1 change and wordings are up to UE capability CR rapporteur. It will be discussed as part of super short email discussion.

Proposal 2: Add separate optional markers for the three components of the feature R4 39-7 (ltm-FastUE-Processing) (NBC change). Adopt the TP 2 in this paper.

- Agreed.

Proposal 3: The signalling structure in option2 is not pursued. Keep the current signalling structure for the capabilities with per band pair per band combination granularity.

- Agreed.

Proposal 4: RAN2 people ask internally to RAN4 to see whether the capabilities R4 39-4, 39-4a and 39-5 can be indicated independently, and come back on Thursday:

•	If these capabilities can be indicated independently, adopt the TP 3 in this paper.

•	If these capabilities are always coupled, no correction is needed.

[MediaTek]: After checking, 39-4a and 39-5 are coupled but 39-4 is decoupled. Suggest to follow RAN4 guideline, i.e. to make all independent for safe.

- Make all independent (NBC change). Change will be captured in UE capability CR discussion.

Proposal 5: In order to ensure that UE capabilities on L1 measurement are not exceeded, the MN indicates the maximum number of L1 measurement resources/configurations the SN is allowed to configure for SCG LTM, including:

-	The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement;

-	The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps;

-	The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement;

-	The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement;

-	Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement;

-	Maximum number of LTM CSI report configs, including aperiodic configs, periodic configs, and semi-persistent configs, respectively;

-	Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement

- Agreed.

Proposal 6: The SN can also request the MN for new maximum values of the number of L1 measurement resources/configurations (as listed in proposal 1) that the SN can configure for SCG LTM. And it’s up to the MN whether to accommodate the SN request.

- Agreed.

Proposal 7: Regarding the capabilities across MCG and SCG per frequency layer, the MN indicates the maximum number of resources per frequency that the SN is allowed to configure. And the SN can request the MN for a new maximum value of the number that the SN can configure. I.e. similar to P1 and P2 above. The capabilities include:

-	The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps;

-	The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps;

-	The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps;

-	The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps.

- Agreed.

Proposal 8: Inter-node coordination is not supported for UE capability that defined as maximum number of resources configured in one slot, i.e. the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement.

- Agreed.

Proposal 9: For each coordinated capability above, the MN indicates a single value to the SN and vice versa. And the value is applicable to all BCs in the allowed BC list.

- Agreed.

Proposal 10:  RAN2 to adopt the TP in the Annex as baseline.

- Agreed. Further details can be discussed as part of CR preparations.

Others to be discussed as part of CR preparation:

P1, P2, and P3 in R2-2406332 (CATT)

P3 in R2-2406337 (MediaTek)

P1, P2, P3, P6, and P7 in R2-2407050 (Samsung)

CR in R2-2407175 (Ericsson)

[POST127][105][MOB] (Ericsson)

Scope: Discuss P1, P2, and P3 in R2-2406332, P3 in R2-2406337, P1, P2, P3, P6, P7 in R2-2407050 and R2-2407175. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements.

Intended outcome: Agreed RRC CR in R2-2407585. Discussion summary in R2-2407595 (if needed). If proposals are straight-forward, it is ok to directly discuss based on the CR (up to offline discussion rapporteur). => In short email discussion, no need of discussion summary.

Deadline: Comeback in Thursday CB session => Short email discussion

37.340 Correction:

R2-2406417	Miscellaneous corrections for mobility enhancements	ZTE Corporation	CR	Rel-18	37.340	18.2.0	0399	-	F	NR_Mob_enh2-Core

R2-2407091	Draft CR for subsequent CPAC corrections	Ericsson	draftCR	Rel-18	37.340	18.2.0	F	NR_Mob_enh2-Core

[POST127][106][MOB] (ZTE)

Scope: Discuss R2-2406417 and R2-2407091. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements

Intended outcome: Endorsed 37.340 CR/TP in R2-2407586. Discussion summary in R2-2407596 (if needed). If proposals are straight-forward, it is ok to directly discuss based on draft TP/CR (up to offline discussion rapporteur). => Agreed 37.340 CR in R2-2407604. In short email discussion, no need of discussion summary.

Deadline: Comeback in Thursday CB session. => Short email discussion

R2-2407586	Miscellaneous corrections for mobility enhancements	ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.2.0	0399	1	F	NR_Mob_enh2-Core

38.300 Correction:

R2-2407449	TP for Stage 2 in coexistence case	Lenovo	discussion	Rel-18	NR_Mob_enh2-Core

- Noted.

[Qualcomm]: Ok with “stop” case. However not sure of “resume” case. [Lenovo]: Nothing new is proposed. All principles were already agreed. [MediaTek]: No need to specify it. We can count on UE implementation. [Lenovo]: We already captured many coexistence cases. [Nokia]: Good to capture it if it was already agreed. [Apple]: Assume it is not a realistic configuration. [CATT]: It is similar to legacy PSCell change and CHO evaluation is not stopped in the case.

Response to RAN3 LS:

R2-2407562	Discussion on RAN3 LS on intra-SN SCPAC in MN format	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: From RAN2 perspective, CG-Config and CG-CandidateList are related to different configuration of intra-SN subsequent CPAC in MN format, and should be sent together in the same SN Modification Required XnAP message.

[CATT]: Think there is no problem if two are sent in separate. Source node can combine them.

Proposal 2: From RAN2 perspective, CG-Config message and CG-CandidateList message can not be included in a single existing inter-node RRC message. RAN2 sends the reply LS to RAN3 to kindly ask RAN3 to include them in a single SN Modification Required XnAP message.

Proposal 3: RAN2 to approve the reply LS in the Annex.

[Session chair]: Revisit it next meeting since there is no consensus.

- Revisit it next meeting.

R2-2407770: L3 measurement based LTM mobility support over F1 (Disc)

Proposal 1: RAN2 should ask RAN3 to introduce F1 support (F1 signalling from CU to DU) for the case where LTM is based on UE L3 measurements.

- Agreed.

R2-2407771: L3 measurement based LTM mobility support over F1 (LSout)

- Action should be updated to “RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.”

- Next RAN2 Meeting should be “TSG-RAN2”.

- Work item should be “NR_Mob_enh2-Core”.

- Source should be “RAN2”.

- With the changes above, approved in R2-2407606.

[Huawei]: Why not source CU sends L3 measurement to source DU? [Qualcomm]: However, we still need new message exchange from CU to DU.

Others:

S-Measure and L3 measurements on L1 LTM candidate cells (P1 in R2-2406438, Vivo)

Proposal 1: UE should always perform L3 measurement for LTM candidate cells even if s-Measure criterion is satisfied.

- Not pursued.

[Samsung]: To remove this restriction, we may need to consult RAN4. [Vivo]: With S-measure, the UE may need to perform measurements on unknown cells, which is not allowed by RAN4. [Apple]: Better to discuss it in RAN4. [MediaTek]: If the UE is located in the cell centre, L3 measurements on neighbouring cells is not triggered, then L1 measurement is not required. Don’t see any issue with s-Measure. [LG]: In LTM cell switch w/o L1 measurement report, there is no issue with s-Measure. [Nokia]: W/o s-Measure, more UE power consumption is concerned. Think the proposal is not required. [Vivo]: Without proposal, the consequence would be that the UE doesn’t perform L3 and L1 measurements if the current serving cell is better than s-Measure.

Security configuration for SCPAC and EMR reselection MR (R2-2407072, Nokia)

R2-2406332	Miscellaneous Corrections for SCPAC	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406337	Rel-18 LTM CP remaining issues	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406355	Leftover LTM UE capability issues	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406418	Inter-node coordination on L1 measurement for LTM	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406438	Discussion on the impact of s-Measure on L1 measurement	vivo	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406476	Report of [Post126][514][R18MobE] UE capabilities Open Issues (Intel)	Intel Corporation	report	Rel-18	NR_Mob_enh2-Core

R2-2406477	Draft 331 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.331	18.2.0	NR_Mob_enh2-Core

R2-2406478	Draft 306 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.306	18.2.0	NR_Mob_enh2-Core

R2-2406531	Discussion on remaining issues for SCPAC	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406552	Triggering LTM Cell Switch without L1 Measurement and Reports	Google Ireland Limited	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406726	Clarification on LTM configuration	Apple	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406847	Miscellaneous Rel-18 LTM Corrections (Early decoding, TA acquisition and estimation)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406852	Discussion on LTM fast processing capabilities	Google Ireland Limited	discussion	Rel-18	38.306

R2-2407050	RRC corrections for Mobility Enhancements	Samsung	discussion

R2-2407072	correction related to security configuration for SCPAC and corrections on EMR reselection measurement reporting	Nokia	discussion

R2-2407091	Draft CR for subsequent CPAC corrections	Ericsson	draftCR	Rel-18	37.340	18.2.0	F	NR_Mob_enh2-Core

R2-2407175	Misc RRC corrections for feMob	Ericsson (Rapportuer)	CR	Rel-18	38.331	18.2.0	4930	-	F	NR_Mob_enh2-Core

R2-2407176	Remaining issues related to LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407177	Summary of RRC proposals for feMob	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core	Late

R2-2407200	RRC issues for LTM	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407370	Discussion on PRACH occasion validation for LTM	Qualcomm Incorporated	discussion

R2-2407410	Correction on LTM related procedure for UE-based TA measurement	Sharp	CR	Rel-18	38.331	18.2.0	4950	-	F	NR_Mob_enh2-Core

R2-2407449	TP for Stage 2 in coexistence case	Lenovo	discussion	Rel-18	NR_Mob_enh2-Core

### 7.4.3	User plane corrections

Including user plane (e.g. MAC) corrections. A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note MAC CR rapporteur’s summary and suggestion (based on the submitted contributions) may be provided.

Carrier selection for RACH-less LTM:

P1 in R2-2406530 (OPPO)

Proposal 1	For RACH-LESS LTM, RAN2 to discuss the way to determine UL carrier with the following three alternatives:

a.	introduce a RSRP_threshold for NUL/SUL selection;

b.	indicate NUL/SUL in LTM cell switch MAC CE;

c.	CG resource for RACH-LESS LTM execution is configured on NUL.

- Not pursued.

[ZTE]: In RACH-less LTM, CG is only configured either NUL or SUL. Not for both. Do not think we need any new. [Huawei]: Understand CG resource can be configured in both NUL and SUL. And the UE will use the first incoming CG resource. We can follow same principle. [CATT]: Share the view with ZTE. [Xiaomi]: Understands it can be configured for both, but not in the same occasion/slot.

CG retransmission timer:

P1 in R2-2406853 (NEC)

Proposal 1: The UE stops the cg-RRC-RetransmissionTimer when the uplink grant for a new transmission on the same HARQ process used for the first PUSCH transmission is received.

- Noted.

[Huawei]: Understand if LTM cell switch is considered as completed in this case, then the CG resources are no more valid so this timer is anyway stopped. [LG]: Understand it is only true when RRC reconfiguration complete is sent over RACH-less LTM switch.

MR Gap and first PUSCH:

P3 in R2-2406853 (NEC)

Proposal 3: During activated measurement gaps, the UE can transmit the first PUSCH transmission during RACH-less LTM cell switch or first PUSCH transmission during RACH-less handover.

- Noted.

[LG]: Consider it is too fancy after R18 was frozen. It is not needed. [Samsung]: Agree with LG. [LG]: Clarified it was discussed last meeting and it was agreed DRX and measurement gap are applied in LTM switch.

RACH based LTM and 2 TAs:

P1, P2, P3, and P4 in R2-2406439 (Vivo)

Proposal 1: Support the co-existence between RACH-based LTM and R18 MIMO two TA, i.e., the LTM candidate cell could be configured with R18 MIMO two TA.

Proposal 2: RAN2 to confirm the mismatch issue occurs when the TCI state ID indicated in the LTM Cell Switch MAC CE is associated to one TAG while the TA value in the RACH RAR is associated to another TAG, assuming the co-existence between RACH-based LTM and R18 MIMO two TA is supported.

Proposal 3: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH based LTM procedure are associated to different TAGs:

−	Option 2: Target DU sends a new TCI state in RACH Msg 4.

−	Option 3: If the mismatch issue occurs, UE follows the TCI state associate with the RACH based LTM procedure. Otherwise, UE follows the indicated TCI-state in the LTM cell switch command.

−	Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with indicated TCI state in LTM Cell Switch Command MAC CE during the RACH based LTM procedure.

Proposal 4: If Option 3 in Proposal 2 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

[Huawei]: The rapporteur's understanding is that it is already clear that this co-existence is supported, the mismatch can occur but may not be so likely, and as explained in option 1, the network can solve the problem by sending a PDCCH order to trigger another RACH. [Apple]: What’s the use case? [Samsung]: Target cell can have two TAs in LTM. [LG]: NW can know this situation and it can be handled by NW implementation, e.g. by PDCCH order. [Ericsson]: Agree with LG. [Vivo]: If we count on NW implementation, the consequence would be no data transmission before TA is corrected by PDCCH order.

- RAN2 understand NW can handle this situation, e.g. by PDCCH order. No need of new option.

Fall-back RACH from RACH-less LTM:

P1, P2 in R2-2406517 (ASUSTek)

Proposal 1:	For RACH-less LTM, the UE falls back to RACH-based LTM in the target Cell when the SS-RSRP of the SSB corresponding to the configured uplink grant that has the same SSB index as the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE is lower than a threshold.

Proposal 2:	Adopt text proposal below as baseline for fallback RACH for LTM.

- Not pursued.

[Huawei]: Fallback was discussed and it was not agreed. [Ericsson]: Agreed with Huawei. [ZTE]: Recall that we discussed fallback for CFRA, not for fallback for LACH-less.

Other corrections:

P2, P3, P4 in R2-2407199 (Huawei)

Proposal 2: Remove the action to stop cg-RRC-RetransmissionTimer in 5.3.1 upon reception of a DL assignment address to the UE C-RNTI or G-RNTI during RACH-less LTM cell switch.

- Not pursued.

[Xiaomi]: Understand nothing is wrong if stop it. CG resource is not used, then stopping it is natural operation. [Nokia]: Support the proposal. [LG, CATT]: Agree with Xiaomi.

Proposal 3: The UE considers RACH-less LTM cell switch as complete upon receiving PDCCH indicating a new transmission on a HARQ process for which there was a previous transmission to the target cell.

- Not pursued.

[Samsung]: It was already discussed during [108] offline and companies didn’t agree with it. [Huawei]: Confirmed P3 and P4 were treated in [108] offline.

Proposal 4: During RACH-less LTM cell switch, the UE can use all HARQ processes of the CG configured for RACH-less LTM cell switch.

[Huawei]: Based on offline discussion, it was not agreed and companies thought that there is only one HARQ process for the CG for RACH-less LTM cell switch. We can capture that understanding.

- Only one HARQ process for the CG for RACH-less LTM cell switch. Will be captured in RRC.

P2, P3, P4 in R2-2407433 (ZTE)

Others to be discussed as part of CR preparation:

R2-2406331 (CATT)

R2-2406349 (Samsung)

P5, P6 in R2-2406439 (Vivo)

R2-2407197 (Huawei)

P1 in R2-2407433 (ZTE)

P2 in R2-2406853 (NEC)

[AT127][108][MOB] (Huawei)

Scope: Discuss R2-2406331, R2-2406349, P5, P6 in R2-2406439, R2-2407197, P1 in R2-2407433, and R2-2406853. Update the CR with merging all corrections (if agreeable).

Intended outcome: Endorsed MAC CR/TP in R2-2407587. Discussion summary in R2-2407597 (if needed). If proposals are straight-forward, it is ok to directly discuss based on draft CR/TP (up to offline discussion rapporteur).

Deadline: Comeback in Thursday CB session. => Completed.

R2-2407597	[AT127][108][MOB] MAC CR discussion summary (Huawei)	Huawei	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406331 (CATT)

- 1st change is useful (add reference to the MAC CE section).

- 2nd change is not needed.

- modified 3rd change is useful: " The configured candidate cell TCI states are initially deactivated upon (re-)configuration by upper layer and after reconfiguration with sync that is not triggered by LTM."

R2-2406349 (Samsung)

Both changes are needed.

P5, P6 in R2-2406439 (Vivo)

1st proposal:

RAN2 understands that the UE performs L1 measurement report for LTM and does RACH if ordered by PDCCH for any candidate cell as in the LTM configuration, whether the candidate happens to be a deactivated SCell or not has not impact on this. No clarification in specifications is needed. This can be captured in meeting minutes.

2nd proposal: change is useful (but add "minus 1").

R2-2407197 (Huawei)

Changes are useful (editorial).

R2-2407433 (ZTE)

P1 is not needed.

No change needed for P2.

P2 in R2-2406853 (NEC)

No need (and this is not different from CG-SDT).

R2-2407199 (Huawei)

- In DL assignment reception, remove the condition on "new transmission".

- Completion is only upon UL grant for the HARQ process of the first PUSCH transmission (not of any additional transmission). No change is needed.

- Only one HARQ process can be used in the CG for RACH-less LTM cell switch. No change needed (could be captured in RRC).

- All offline discussion outcomes above are agreed.

R2-2407587	User plane corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1904	2	F	NR_Mob_enh2-Core

- Agreed.

R2-2406331	Corrections to TS 38.321 for LTM	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406349	MAC corrections for LTM	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406439	Discussion on MAC open issue for LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406517	Discussion on fallback RACH for LTM	ASUSTeK	discussion	Rel-18	38.321	NR_Mob_enh2-Core

R2-2406530	Discussion on carrier selection for RACH-LESS LTM	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406853	Corrections of MAC issues for RACH-less LTM	NEC	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407197	User plane corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1904	-	F	NR_Mob_enh2-Core

=> Revised in R2-2407563

R2-2407563	User plane corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1904	1	F	NR_Mob_enh2-Core

R2-2407198	MAC CR rapporteur summary	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core	Late

R2-2407199	Issues for RACH-less LTM cell switch	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407433	Consideration on Remaining Issues on LTM UP Aspect	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

## 7.15	NR Sidelink evolution

(NR_SL_enh2; leading WG: RAN1; REL-18; WID: RP-230077)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.15.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2406209	Reply LS on Sidelink Feature Co-configuration (R1-2405530; contact: OPPO)	RAN1	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2

- Noted.

- MAC CR rapporteur will update the corresponding MAC changes (if needed).

### 7.15.2	Corrections

Including corrections to all specifications. A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note RRC and MAC CR rapporteurs’ summary and suggestion (based on the submitted contributions) may be provided.

TX carrier selection for SL MAC CE:

P1 in R2-2406746 (LG)

Proposal 1. Carrier selection of SL IUC Request MAC CE, Condition based SL IUC Information MAC CE and SL DRX command MAC CE uses the same procedure as the carrier selection procedure of logical channel data.

P1 in R2-2407131 (Nokia)

Proposal 1: Enhancements to Tx carrier (re-)selection for SL IUC Request MAC CE, Condition based SL IUC Information MAC CE should not be considered.

[Session chair]: TX carrier is part of resource selection. How the UE performs TX carrier for MAC CE according to the current MAC? [Huawei]: MAC will wait until grant is created due to other data transmission. Should be ok since those MAC CEs are not delay sensitive. [LG]: In Rel-16 and Rel-17, MAC generates a new grant for MAC CE. It doesn’t make a sense that MAC should wait until a grant is made for LCH data transmission in Rel-18. [Ericsson, Apple, NEC]: Share the view with LG.

- P1 in R2-2406746 is agreed. Detailed wordings will be discussed in MAC CR preparation.

Legacy carrier selection in TX carrier selection:

R2-2406700	Correction on legacy carrier selection	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.2.0	1889	-	F	NR_SL_enh2-Core

[CATT]: if the upper layer indicates single carrier, is it natural to assume no need to perform TX carrier selection among multiple carriers? [Session chair]: Suggest offline discussion among interested companies, e.g. whether the current MAC works w/o change? Or with adding the new sentence (to make it clearer), any real issue or further spec impacts?

- Comeback in Thursday CB session.

In CB session:

R2-2407800	Discussion on legacy carrier selection	ZTE Corporation, Sanechips	Discussion	NR_SL_enh2-Core

Proposal1: If PDCP duplicated is enabled, regarding the LCH carring QoS flow of Tx profile = BC, RAN2 is suggested to discuss following options on how to use the legacy carrier:

| Not ignore CBR threshold | Network set the CBR threshold of legacy carrier to 100%, specifically for the dedicated prio of the LCH carrying QoS flows of TxProfile = BC | (Option1a)Current spec works well | (Option1)Spec change is not needed |
| --- | --- | --- | --- |
| Ignore CBR threshold | single carrier branch& multiple carrier branch is performed per LCH channel, RLC with legacy carrier will perform single carrier branch | (Option1b)Current spec works well |  |
|  |  | (Option2)determination of single/multiple carrier operation is now per UE, not per LCH, spec change is needed to capture this understanding | Spec change is needed |
|  | Clairfy in 5.22.1.11 that UE will select legacy carrier directly, if indicated by upper layer to use | (Option3)by a Note or Normative Text |  |

[OPPO]: Think option1a should be sufficient. Assume backward compatible data and non-backward compatible data would be set as different QoS flows. [Xiaomi]: Don’t think option2 is correct. Resource selection that will trigger TX carrier selection occurs once LCH data is arrived. It is not per UE operation. [Qualcomm]: Agree with Xiaomi. Don’t think MAC is broken. [ZTE]: Triggering is per LCH, however when to compare whether it’s single carrier or multiple carriers, it is defined per UE. [IDC, LG, Apple, Vivo]: Prefer Note based option. [OPPO]: It is ok with adding a note. However, in the example TP, it is too general. It needs to be more precise according to RAN2 previous agreement. [ZTE, IDC]: Example TP was made to be aligned with what is specified in RRC. [Nokia]: Prefer original proposal (normative text in TX carrier selection).

- Add a note to clarify the intention according to what RAN2 agreed into MAC. Detailed wording will be discussed as part of MAC CR preparation.

TP on IUC/DRX in Co-Ex:

R2-2406584	Discussion on MAC corrections when supporting IUC and DRX in Co-Ex	LG Electronics France	discussion	Rel-18	38.321	NR_SL_enh2

[Apple, Ericsson]: Looks quite complicated. Can we consider another structure to capture it? [OPPO, Vivo, Huawei]: Thinks this change is clear and it is the best we can do. Support the change. [Vivo]: Would like to have more time to check more carefully (e.g. in short email discussion). [Huawei]: Ok to agree with it now. [Apple]: Will prepare simpler MAC change until Thursday CB session.

- Apple’s MAC correction is provided in R2-2407594

- Comeback in Thursday CB session. Will compare two versions and select one of two.

R2-2407594	TP for reosuece selection when supporting both IUC and Co-Ex	Apple, Ericsson	discussion	Rel-18	38.321	NR_SL_enh2

[LG]: Don’t think with this TP, all UE behaviours in various cases are really readable. [OPPO]: Think both works and prefer simpler version (Apple). [Qualcomm]: IUC is performed in UC only while Co-Ex supports all cast types. Not sure if this TP covers all cases. [Session chair]: For scheme 2, it seems IUC only case is now removed.

- R2-2407594 is baseline for IUC + Co-Ex correction.

- For scheme 2, IUC only case needs to be included.

- Further details will be discussed as part of MAC CR preparation.

Correction on MCSt:

R2-2406316	Correction on selection of resources for MCSt	CATT, CICTCI	draftCR	Rel-18	38.321	18.2.0	F	NR_SL_enh2

- Intention (to specify selection of multi-shots resources for MCSt approach 2) is agreed.

- Detailed wordings can be further discussed in MAC CR preparation.

[LG]: Understand “one transmission opportunity” already covers multiple slots if a number of consecutive slots is provided by L1. [CATT]: One transmission opportunity has been an old term, which indicates single slot from Rel-16. [Qualcomm]: Understand the intention. With this change, do we need additional specification efforts to define a relationship between a grant and a number of consecutive slots, e.g. a number of consecutive slots will be considered as single grant or multiple grants? How does it impact HARQ processes and retransmissions? etc. [Vivo]: Support the proposal. On Qualcomm’s question, think no more specification effort is needed. Think a number of consecutive slots will be considered as single grant. [CATT]: Suggest to agree with intention. We can further discuss detailed wordings or other aspects during MAC CR preparation.

Correction on prioritization:

R2-2406518	Discussion on SR prioritization regarding SL transmission	ASUSTeK	discussion	Rel-18	38.321	NR_SL_enh2

- Noted.

[Session chair]: For the first proposal that is related to SL-PRS, it would be better to discuss it in positioning session. [OPPO]: Current MAC specifies a collision with two channels and it is clear a collision with three channels would bring the problem as described. Then NW should avoid this collision by implementation. [LG]: Although understand the intention, also agree with OPPO. We can rely on NW implementation to avoid this issue. [ASUSTek]: Not clear NW can always avoid this issue. If cannot avoid the issue, we need to define UE behaviour.

Correction on LCP restriction:

R2-2406806	Correction to per-LCH carrier set restriction in LCP	Ericsson	draftCR	Rel-18	38.321	18.2.0	NR_SL_enh2

- Agreed.

[Huawei]: RAN2 agreed this information is for LCP. We should keep it. [OPPO]: sl-AllowedCarriers is not the only one to be considered. It will be further filtered out based on other factors, e.g. UE capabilities, etc. [OPPO]: Allowed carriers is the term that the upper layer indicates to MAC after all filtering out. Note the allowed carriers are already considered in LCP.

Others on MAC:

R2-2406596	Miscellaneous corrections for SL evolution	Huawei, HiSilicon	discussion	Rel-18	NR_SL_enh2-Core

R2-2407381	MAC corrections on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1907	-	F	NR_SL_enh2

- Correction 1 was agreed.

[POST127][101][V2X/SL] (LG)

Scope: Discuss changes in R2-2407381 (only correction 1) and capture all agreements made in RAN2#127. => It is extended to short email discussion. Capture all RAN2#127 agreements.

Intended outcome: MAC CR in R2-2407581. => Agreed MAC CR in R2-2407603.

Deadline: Short offline discussion.

R2-2407581	MAC corrections on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1922	-	F	NR_SL_enh2 => Will revised to R2-2407603

R2-2406265	Miscellaneous correction on R18 SL Evolution	OPPO	CR	Rel-18	38.331	18.2.0	4863	-	F	NR_SL_enh2

- Agreed.

R2-2407372	Correction on setuprelease type sidelink fields handling	Google	CR	Rel-18	38.331	18.2.0	4821	1	F	NR_SL_enh2	R2-2405322

- Not pursued.

[AT127][102][V2X/SL] (OPPO)

Scope: Discuss changes in R2-2406265 and R2-2407372, capture all agreements made in RAN2#127 and update RRC CR with merging all corrections (if agreeable).

Intended outcome: RRC CR in R2-2407582.

Deadline: Comeback in Thursday CB session. => Completed.

In CB session:

[OPPO]: During offline discussion, most companies didn’t agree with R2-2407372. We can only agree with R2-2406265 (no merged CR in R2-2407582). [Session chair]: Ask any need to extend it to short email discussion to capture any further RAN2 agreement. [OPPO]: No.

R2-2407388	Rapporteur Stage 2 Corrections for NR Sidelink Evolution	InterDigital France R&D, SAS, NEC	CR	Rel-18	38.300	18.2.0	0893	-	F	NR_SL_enh2

R2-2406554	TP for SL CA in TS38300	NEC	discussion	NR_SL_enh2

[AT127][103][V2X/SL] (IDC)

Scope: Discuss changes in R2-2407388, R2-2406554 and update CR with merging all corrections (if agreeable)

Intended outcome: 38.300 CR in R2-2407583.

Deadline: Comeback in Thursday CB session. => Completed.

R2-2407583	Rapporteur Stage 2 Corrections for NR Sidelink Evolution	InterDigital France R&D, SAS, NEC	CR	Rel-18	38.300	18.2.0	0893	-	F	NR_SL_enh2

- Agreed.

R2-2406264	Discussion on LS R1-2405530	OPPO	discussion	Rel-18	NR_SL_enh2

R2-2406746	Discussion on carrier selection for SL MAC CE(s)	LG Electronics Inc.	discussion	Rel-18	38.321	NR_SL_enh2

R2-2407131	Carrier reselection for IUC	Nokia	discussion

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID:  RP-241650 )

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Including incoming LSs and rapporteur inputs.

- R19 NES CR rapporteurs:

RRC: Ericsson

MAC: IDC

38.300: Huawei

38.304: Apple

38.306: Intel

### 8.5.2	On-demand SSB SCell operation

RAN2 spec impacts and high-level solutions.

Scenarios on OD-SSB:

P1 in R2-2406425 (Xiaomi)

Proposal 1: RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

- Agreed.

OD-SSB transmisson MAC CE:

P4, P5 in R2-2406721 (Vivo)

Proposal 4: RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- Agreed.

[Xiaomi]: Understand RRC can be used for activation/deactivation even after initial activation. [Huawei]: Agree with Xiaomi. [Apple]: Agree with P4. It is similar to what we already have for cell DTX activation. [OPPO]: Agree with Apple. If RRC is used for dynamic activation/deactivation purpose, RRC and MAC are introduced for the same purpose. In general, MAC CE is used for dynamic activation/deactivation. [Nokia]: In initial configuration, 1bit would be included. However, that can be changed/reconfigured later. We don’t need any restriction.

Proposal 5: MAC CE based OD-SSB transmission indication is used to activate the OD-SSB when the SCell is in deactivated (i.e., Scenarios 2) or indicate the activation state of OD-SSB when activating the SCell via MAC CE (i.e., Scenarios 2A).

P2, P3 in R2-2406669 (Apple)

Proposal 2: Following RAN1 agreement, configuration on OD-SSB transmission pattern (e.g. frequency, SSB position and transmission periodicity) doesn’t need to be included in MAC-CE, i.e., it is sufficient to only include 1bit indication of activation / deactivation of OD-SSB transmission on the corresponding SCell in MAC-CE.

- Noted. We need to see more RAN1 progress.

[IDC]: RAN1 agreed transmission periodicity is included in MAC CE. [Xiaomi]: Multiple SSB transmission patterns is under RAN1 discussion. We may need an indication which transmission pattern is used in MAC CE. [Session chair]: Let’s wait for more RAN1 progress.

Proposal 3: RAN2 down select between the following two alternatives of new OD-SSB MAC-CE design:

•	Alt-1: The new MAC-CE is fixed size only to indicate OD-SSB transmission, and NW send both new MAC-CE and legacy SCell activation MAC-CE for Scenario 2A.

o	The Bi field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Bi field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

•	Alt-2: The new MAC-CE is variable size to indicate either only OD-SSB transmission (scenario 2), or joint OD-SSB transmission and SCell activation / deactivation (scenario 2A).

o	The first Oct (i.e. Oct1) is mandatory present and the meaning of Bi field is same as Alt-1.

o	The second Oct (i.e. Oct2) is optional present. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated (i.e. same as legacy SCell activation / deactivation MAC-CE).

[Nokia, NEC]: Minimum need is to define a new MAC CE for OD-SSB transmission indication. [Xiaomi]: For scenario 2, we need the new MAC CE. For scenario 2A, we may consider a merged MAC CE for both the new MAC CE and SCell activation/deactivation MAC CE. [Huawei]: Agree with Nokia and NEC. For scenario 2A, prefer not to introducing additional MAC CE. Prefer having common solution for both 2 and 2A. [IDC, Samsung]: Agree with Huawei. [Ericsson]: Scell index will be included. If we merge OD-SSB transmission indication and SCell activation/deactivation, it can save overhead. [Huawei]: Scell index is not needed for OD-SSB transmission indication. Scell is implicitly associated based on the 1bit location, so no additional overhead as Ericsson mentioned.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

P1 in R2-2406347 (Samsung)

Proposal 1: RAN2 to discuss whether on-demand SSB measurements for L3 measurement (i.e. RRM) are used for

•	Option 1: Case #1 only

•	Option 2: both Case #1 and Case #2

[Apple]: Support case 2. In case 2, would like to include/consider both OD-SSB and periodic SSB. For case 1, extra spec impact will be introduced. [Huawei]: We first decide case 1 since observe that companies have different understanding on option 2. Understanding A) With OD-SSB, periodic SSB’s periodicity is changed. Understanding B) Additional OD-SSB is signalled in addition to periodic SSB. We need to understand which understanding is correct. [OPPO]: Share the view with Huawei. We need to first secure case 1. [Nokia]: Case 2 is more similar to what we already have. Should consider case 2.

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

P2 in R2-2407414 (Sharp)

Proposal 2 (modified): For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- Agreed.

[Xiaomi]: For case 2, the UE anyway needs to perform measurements. How does the UE know whether it is case 1 or case 2. If not known, the UE needs to perform measurements anyway. [Fujitsu]: Agree with intention. [Huawei, Vodafone]: Agree with proposal 2. Otherwise the MR is useless. [Apple]: Agree with P2. The UE can know whether case 1 or case 2 based on the configuration.

Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

[Huawei]: NW configures TTT and NW knows when to start OD-SSB. The problem may not happen. [Apple]: Similar issue happens for measurement on CSI-RS, e.g. when CSI-RS is deconfigured in the middle of TTT running. [Xiaomi]: Similar issue already exists in legacy measurement.

P8 in R2-2406669 (Apple)

Proposal 8: RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

- Agreed.

[OPPO]: Instead of SMTC adaptation, we can consider the UE just measure according to OD-SSB pattern that is configured by NW. [Apple]: SMTC is per frequency (not per cell). OD-SSB transmission pattern is only for the SCell. [Xiaomi]: We may not need any new one, e.g. NW may always send OD-SSB only during SMTC measurement duration.

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

R2-2406266	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406347	On-demand SSB SCell Operation	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406425	Discussion on on-demand SSB	Xiaomi	discussion

R2-2406444	On-demand SSB SCell operation in connected mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406469	RAN2 impacts to enable on-demand SSB SCell	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406620	On-demand SSB Scell operation discussion	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406669	Discussion on RAN2 work of on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406721	Discussion on on-demand SSB SCell operation	vivo	discussion	Rel-19

R2-2406749	Discussion on on-demand SSB Scell operation	Spreadtrum Communications	discussion	Rel-19

R2-2406889	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-19

R2-2406895	Discussion on on-demand SSB	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406954	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406979	Discussion on on-demand SSB	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407002	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407039	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407123	Discussion on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407158	On-demand SSB SCell operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407161	Discussion on On-demand SSB SCell Operation	Qualcomm 	discussion

R2-2407185	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407271	Discussion on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407304	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407414	Discussion on on-demand SSB SCell operation	Sharp	discussion

### 8.5.3	On-demand SIB1

Leftover issues from RAN2#126 and remaining essential issues for study (e.g. need of WUS configuration acquisition from NES cell (if needed, what’s use cases and how it works)?, need of SIB1 validity (if needed, what’s use case and how it works), whether/how to support case 3 defined in RAN1 (e.g. reuse/enhance on-demand SIB request procedure)?, etc.). Note study of OD-SIB1 needs to be concluded

Support of RAN1 case 3 (option 2 + B + Y):

P2 in R2-2406955 (Nokia)

Proposal 2: RAN2 to consider Case 2, i.e. scenario 1a, only for normative work.

P8 in R2-2406980 (CMCC)

Proposal 8: For the scenario of acquiring on-demand SIB1 with only Cell A involved, the following agreements made for Scenario 1a can be reused:

1.	Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

2.	For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

3.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

[Huawei]: Understand the support of case 3 is not complicated. We can reuse the existing mechanism. Understand case 3 is better in energy saving point of view. Support case 3. [Vodafone]: Both case 2 and case 3 supports anchor scenario. It is not good to have multiple options to handle same scenario. [ZTE]: Share the same view as Nokia. Case2 was already decided in earlier time, so we don’t need to introduce another option for the same scenario. [LG, ITRI]: Think case 2 is the baseline, however case 3 can give maximum energy saving gain. Support to have case 3. We can reuse many of case 2 for case 3. Do not expect much specification efforts. [Rakuten]: In case 2, SIB1 is always acquired from a NES cell while in case 3, SIB1 is acquired from a NES cell and cell A. Prefer simple option (case 2). [Xiaomi]: Cell A may need to provide multiple SIB1s, which make case 3 more complicated. [Fujitsu]: Support Nokia proposal. [Session chair]: Let’s check companies views.

- Case 2 only: Vodafone, ZTE, Vivo, Rakuten, Fujitsu, CATT, Google, Ericsson, Xiaomi, IDC, Nokia, DT (12)

- Case 2 and case 3: Sony, Samsung, Huawei, ITRI, LG, Qualcomm, Lenovo, KDDI, DCM, CMCC (10)

[Apple]: We can make a conclusion that case 2 is baseline and case 3 is feasible, but final conclusion can be left to RAN1. [Nokia]: What’s RAN1 impact on RAN1. Case 3 needs more RAN2/3 works. [ZTE]: Based on our RAN1 colleagues, RAN1 waits for RAN2 decision for case3. Suggest to follow slight majority companies’ views. [Ericsson]: Concluding just feasible for case 3 is not sufficient for RAN discussion. Please note we have spent most of time for OD-SIB1. We still have many works for other scopes. [Vodafone]: Concern the information overhead of inter-node information exchange in case3. [Lenovo]: Expect SIB1 does not change often.

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

P5, P6 in R2-2406470 (Intel)

Proposal 5: To re-use validity time concept (which is currently defined for other SIBs) for OD-SIB1. I.e., when Rel-19 NES UE acquires OD-SIB1, this UE uses it as a valid stored version.

Proposal 6: Legacy notification of SI change can be reused for OD-SIB1. NOTE: network is excluded to trigger a notification of SI change due to changes of the H-SFN value.

P3, P4 in R2-2406955 (Nokia)

Proposal 3:  We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

Proposal 4: Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

- P3 and P4 in R2-2406955 are agreed.

P2 in R2-2406670 (Apple)

Proposal 2: SIB1 validity mechanism is supported, i.e., the UE can know when the acquired OD-SIB1 is invalid. Its details can be discussed in normative phase.

[Samsung]: A new timer is not needed for optimization. Current 3 hours existing timer works with value tag and it is not applied to SIB1. [Session chair]: Ask if validity timer means 3 hours existing one or newly introduced timer for SIB1 validity? [LG]: Do we talk about 1) while the UE stays in cell A or 2) while the UE camps on NES cell. [Nokia]: Confirms current 3 hours timer does not work for SIB1. [Nokia]: Understand most companies think that existing mechanism is applied to SIB1.

WUS configuration of NES cell from NES cell:

P2 in R2-2407041 (Ericsson)

Proposal 2: Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

- P2 in R2-2407041 is agreed.

P2, P3 in R2-2406780 (OPPO)

Proposal 2: RAN2 confirms that acquiring the UL WUS configuration from NES Cell is unnecessary.

Proposal 3: The outdated UL WUS configuration stored by the NES UE causes the failure of SIB1 acquisition and eventually requires the UE to go back to Cell A for the latest UL WUS configuration.

[OPPO]: Question is if NES cell only provides its own WUS configuration or if NES cell can provide neighbouring cells’ WUS configuration. [Rakuten]: If NES cell does not do that, the UE needs to back to cell A, which is too much for the UE. [Qualcomm]: W/o WUS configuration of neighbouring NES cells, the UE always need to back cell A whenever cell reselection happens to another NES cell. [Huawei]: Support Ericsson proposal and share the view with Qualcomm.

MSG3 based on OD-SIB1 REQ:

P5 in R2-2406659 (Fujitsu)

Proposal 5: Msg3 based SIB1 request is triggered if dedicated RA resources for SIB1 request is not configured. Details will be discussed in a normative phase.

P3 in R2-2406605 (Huawei)

Proposal 3: Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

- P3 in R2-2406605 is agreed.

[Apple]: Support Huawei’s proposal. We have only single SIB1, so there is no motivation to use MSG3 based OD-SIB1 procedure. If we allow MSG3 based OD-SIB1, it will increase WUS configuration overheads as well. [ZTE]: With MSG3 based OD-SIB1, NES cell needs to monitor MSG3 and send MSG4. MSG3 based OD-SIB1 is not needed. [LG]: Since case 3 seems ruled out, no motivation for MSG3 based OD-SIB1 REQ. [NEC]: WUS contains dedicated RA, so no need for MSG3-based OD-SIB1 REQ.

Access restriction for legacy UEs:

P11 in R2-2406346 (Samsung)

Proposal 11: RAN2 to discuss how to handle Legacy UEs (i.e. UEs not supporting on-demand SIB1).

•	Option 1: Legacy UEs bar the on-demand SIB1 cell based on cellBarred bit set to barred in MIB.

•	Option 2: Legacy UEs bar the on-demand SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

•	Option 3: Network includes cells supporting on demand SIB1 to list of excluded cells

- Above example options can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

[Nokia]: With option1, we need an additional barring bit for NES UE. [Google]: For Rel18 legacy UEs (e.g. RedCap UE), with option1, the UE still needs to check SIB1 to determine whether the cell is barred or not. [Lenovo]: When Kssb has a value indicating there is no SIB1, combined with SIB1-PDCCH CORESET and search space, the UE may try to other frequency. Operator needs to carefully set the configuration. [BT]: With option1, legacy UE may not get an emergency call. Want barring but allowing emergency call for legacy UE in NES cell. [KDDI]: Concern if the legacy UE in the market actually bars the cell according to option2 and option3. [Vodafone]: Legacy UE cannot have even emergency call if the UE cannot get SIB1 in NES cell.

Impacts to RRC connected UEs:

P4 in R2-2406670 (Apple)

Proposal 4: OD-SIB1 doesn’t impact both legacy and Rel-19 RRC_CONNECTED UEs (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

P2 in R2-2406359 (Xiaomi)

Proposal 2 (modified): RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

- P2 in R2-2406359 is agreed.

[Nokia]: With the original proposal 2, it sounds like always no impact to RRC connected UE. There may be some cases. Think this conclusion somewhat risky.

SI conclusion:

P6 in R2-2406670 (Apple)

Proposal 6 (modified): RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

- Agreed.

Agreements on OD-SIB1

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

R2-2406346	On-demand SIB1	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406359	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

R2-2406445	Remaining issues of on-demand SIB1 in idle and inactive mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406470	Remaining details to enable on-demand SIB1	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406569	The procedure for the on-demand SIB1 transmission	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406605	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406621	On-demand SIB1 for IDLE/INACTIVE UEs	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406622	Further study on Case 3 in on-demand SIB1	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406653	Discussion on On-demand SIB1 for RAR	KDDI Corporation	discussion	Rel-19

R2-2406659	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu Limited	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406670	Finalize study of on-demand SIB1	Apple Inc, BT Plc	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406722	Discussion on  on-demand SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19

R2-2406780	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406804	Discussion on on-demand SIB1	Sharp	discussion

R2-2406896	Discussion on left issues of on-demand SIB1	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406955	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406980	Discussion on on-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407003	Consideration on on-demand SIB1 issues	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407041	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407042	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407043	Discussion on On-demand SIB1 procedure and UL WUS configuration	NEC	discussion

R2-2407051	Detection and access of NES cells with OD-SIB1	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407159	 On-demand transmission of SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407183	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407351	Further discussion on on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407438	Remaining essential issues for study	Lenovo	discussion	Netw_Energy_NR-Core

R2-2407455	Discussion on on-demand SIB1	NTT DOCOMO INC..	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407499	On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2407540	On-demand SIB1 for Idle/Inactive mode UEs	III	discussion

R2-2407162	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

### 8.5.4	Adaptation of common signal/channel transmissions

Further consideration of adapation of paging occasions in time domain, legacy UE impact (including barring aspect for paging adaptation), configuration aspect for paging adaptation, RAN2 spec impact and solutions for RACH adaptation and SSB (with consideration of RAN1 progress), etc.

Paging adaptation:

[POST127][109][NES] (OPPO)

Scope: For each direction (N extension vs PF bundling): 1) List proposed options, 2) Discuss to understand each option better, 3) Down select options, 4) Discuss pros and cons for each direction or down-selected options (with the consideration of UE impacts, system impacts and specification job). F2F offline discussion in Brk3 room. => It is extended to long email discussion to continue detailed analysis of pros and cons for each direction.

Intended outcome: Discussion summary in R2-2407598. => Updated discussion summary

Deadline: Comeback in Thursday CB session. => Long email discussion.

R2-2407598	Summary of [109]	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1	Option-a) is about the bundling of PF for R19 NES UEs.

Proposal 3	RAN2 observe that the option-a) and option-b) can be designed to configure the POs at same time position.

- P1 and P3 are agreed.

Proposal 2	For option-a), R2 further discuss whether to allow gaps between bundled PFs.

- Can be discussed as part of long email discussion.

Agreements on paging adaptation

- Option-a) is about the bundling of PF for R19 NES UEs.

- RAN2 observe that the option-a) and option-b) can be designed to configure the POs at same time position.

R2-2406270	Discussion on PO confinement options	OPPO, Samsung, ZTE, Huawei, HiSilicon, Qualcomm	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406348	Adaptation of common signal channel transmissions	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406360	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19

R2-2406446	Further consideration on paging occasion adaptation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406471	RAN2 impacts to enable adaptation of paging and RACH in time	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406523	Discussion on paging adaptation	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2405428

R2-2406544	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406671	Further discussion on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406723	Discussion on adaptation on common signal transmissions	vivo	discussion	Rel-19

R2-2406750	Discussion on adaptation of common signal channel transmissions	Spreadtrum Communications	discussion	Rel-19

R2-2406866	Discussion on the paging occasion adaptation for NES cell	ITRI	discussion	Netw_Energy_NR_enh-Core

R2-2406890	Paging statistics from field and PRACH adaptation	Lenovo	discussion	Rel-19

R2-2406897	Discussion of adaption of paging occasions	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406956	Common signal aspects of NES WI	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406981	Discussion on adaptation of common signal channel transmissions	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407004	Consideration on adaptation of common signalchannel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407048	PRACH and paging adaptation	NEC	discussion

R2-2407163	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated	discussion

R2-2407184	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407245	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2405290

R2-2407305	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407352	Discussion on adaptation of common channel transmissions	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407440	Discussion on RACH adaptation 	SHARP	discussion	Rel-19

R2-2407454	Discussion on Adaptation of paging occasions	NTT DOCOMO INC..	discussion	Rel-19	Netw_Energy_NR_enh

R2-2407486	Adaptation of Common Signals and Channels for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2407520	Discussion on common signal and channel adaptation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh

R2-2407531	Adaptation of common signal/channel transmissions	III	discussion

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-241515)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Including incoming LSs, WI rapporteur inputs, etc.

R2-2406244	Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases (S3-242400; contact: Apple)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN3

- Noted.

R2-2406693	Important topics for resolution for Rel-19 Mob Enh WI (Rapporteur)	Apple Inc, China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

- R19 MOB CR rapporteurs:

RRC on all of inter-CU and conditional LTM: Ericsson

RRC on L1 event-triggered MR: Huawei

MAC: Vivo

37.340: China Telecom

38.300: Apple

38.323 (if needed): ZTE

### 8.6.2	Inter-CU LTM

Remaining open issues and further details of LTM preparation phase (e.g. single or multiple reference configuration(s)?, details of RRC signaling structure, etc.), early sync phase, execution phase (e.g. support of mixture of intra-CU and inter-CU, etc.), and LTM cell switch completion phase. Initial discussion on inter-CU LTM with DC (high-level spec impacts and solutions).

Single or multiple reference configuration(s)?

P1 in R2-2406694 (Apple)

Proposal 1: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations are supported.

- P1 in R2-2406694 is agreed.

P1 in R2-2407269 (LG)

Proposal 1. Multiple reference configurations are provided for inter-CU LTM, where each reference configuration is CU specific.

[Samsung]: Multiple reference configuration gives more flexibility. NW still can achieve single reference configuration, which is up to NW. [Huawei]: Assume it does not happen often and saved bits are not significant. It is considered as optimization for some cases, which is not essential. [ZTE]: With multiple reference configurations, it simplifies inter-CU signalling. [Xiaomi]: With multiple reference configurations, it increases UE complexity and it requires more UE memory capability. [Ericsson]: Single reference configuration works as baseline. [Nokia]: With single reference configuration, the reference configuration setup procedure needs to be repeated whenever any CU reconfigures some configuration. [Session chair]: Check companies’ positions. If there is no clear majority companies, we can stick to what we already have (single reference configuration).

- Multiple reference configurations: LG, Samsung, CMCC, Nokia, Fujitsu, Rakuten, ITRI, KDDI, ZTE, BT, IDC, KT, ETRI (13)

- Single reference configuration: Google, Honor, CATT, Lenovo, Vivo, Apple, OPPO, Spredtrum, Xiaomi, NEC, Intel, MediaTek, Qualcomm, China Telecom, Ericsson, Sharp (16)

CSI resource and report configuration:

P3, P4 in R2-2406419 (ZTE)

Proposal 3: The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

Proposal 4: The source CU is responsible to generate the common CSI resource configuration.

- P3 and P4 in R2-2406419 are agreed.

P5 in R2-2407269 (LG)

Proposal 5. As in Rel-18 LTM configuration, CSI resources for LTM L1 measurement for all LTM candidate cells are provided in the form of a single list.

Inter-node RRC message for inter-CU LTM:

P5 in R2-2407421 (Samsung)

Proposal 5: RAN2 is kindly asked to discuss the LTM candidate configuration (ltm-candidate IE) transfer from candidate gNB-CU to the source gNB-CU during the LTM cell preparation procedure by considering the following two options:

	Option 1: Define a new inter-node RRC message for LTM candidate configuration

	Option 2: Define new XnAP IEs to include other information in the LTM candidate configuration

- Noted.

[NEC, Ericsson]: Think we can discuss it later. [Samsung]: Handover request/response procedure is performed per each candidate cell. [Vodafone]: Understand it is under RAN3 discussion. [Session chair]: Let’s wait for more time to think of and check RAN3 discussion status.

Early DL sync:

P3 in R2-2406305 (CATT)

Proposal 3: For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

- Agreed.

Who decides inter-CU LTM switch?

P3 in R2-2406694 (Apple)

Proposal 3 (modified): For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

- Agreed.

[NEC]: Understand source CU initiates inter-CU LTM preparation, however source DU triggers inter-CU LTM cell switch. [LG]: With the agreement that LTM and L1 measurement are decoupled, without L1 measurement, can it be the source CU the node to trigger inter-CU LTM cell switch? [Nokia]: Understands source CU decides it however source DU sends the MAC CE. [Vodafone]: Decoupling between inter-CU LTM and L1 measurement will be continued in Rel-19? [Samsung, OPPO]: Whether source CU triggers inter-CU LTM to source DU is under RAN3 discussion. We may need to wait for further RAN3 progress. [Vivo]: Understand it is RAN2 issue. [Rakuten]: The proposal is which node triggers the MAC CE. No one disagree with source DU. Regarding who decides inter-CU LTM, we may wait for RAN3 progress. [Nokia]: Propose to send a LS to RAN3 to inform RAN2 agreements. [Ericsson]: LTM is based on L1 measurement as baseline. LTM based on L3 measurement is done by NW implementation. [Ericsson, Rakuten]: No need to send a LS to RAN3. [BT]: Think for inter-CU case, if we want to have L3 measurement based inter-CU LTM, we need to standardize the corresponding CU-DU interface to support L3 measurement based LTM. [Vodafone]: We can agree with proposal 3 and we can make further agreement such as “if L3 measurement is used for inter-CU LTM, source CU decides inter-CU LTM cell switch and source DU still triggers the MAC CE.” Also propose to send a LS to RAN3. [Session chair]: Seems for additional agreement Vodafone proposed, there is no consensus right now. Ask Vodafone to have offline discussion with interested companies and if consensus can be made, we can handle the proposal and LS in CB session.

In CB session:

Corresponding proposal and LS are handled in Rel-18 Mobility session (R2-2407770, R2-2407771)

Indication of inter-CU LTM (RLC and PDCP re-establishment)

P4 in R2-2407201 (Huawei)

Proposal 4 (modified): Introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update.

P7, P8 in R2-2406819 (Xiaomi)

Proposal 7: If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. For the issue how to indicate that the security key update is required, we can wait for SA3’s progress on the security of inter-CU LTM.

Proposal 8: If the security key update is not required, reuse the Rel-18 L2 handling mechanism and the Rel-18 NoResetID is reused to determine whether to perform RLC re-establishment and PDCP data recovery when the UE performs the LTM cell switch procedure.

[CATT]: Prefer separate Rel-19 ID. It is also compatible to conditional LTM. [MediaTek]: Based on the current spec, re-establishment and security key change are always coupled. [LG]: Share the view with MediaTek. [OPPO, Vivo]: Support Huawei’s proposal. [Xiaomi]: Conditional LTM could be restricted to intra-CU LTM. [Nokia]: For recovery case, Rel-19 ID is helpful. [Session chair]: Let’s assume recovery is done to different CU, doesn’t the UE need to change a security key anyway? Wonders if security key update can be commonly applicable to all cases. [IDC]: Although we need to wait for SA3 response for security aspect (NCC change), it sounds reasonable to configure re-establishment control by RRC.

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch

P4 in R2-2406919 (Ericsson)

Proposal 4 (modified): As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

- Agreed.

[ZTE]: For DC case, proposal 4 may not work. The UE may need to release LTM candidate configuration. It is better to clarify it is for non-DC inter-CU LTM now.

Handling of DRBs in subsequent inter-CU LTM:

P6 in R2-2407421 (Samsung)

Proposal 6: RAN2 is kindly asked to discuss whether different LTM candidate cells can have different admission results (i.e., different LTM candidate cells accept different PDU sessions).

	Option 1: After accessing to a LTM candidate cell, UE automatically releases the PDU session, which has been released before. This introduces specification impact (RAN2).

	Option 2: The inter-CU LTM candidate cell is prepared only if all LTM candidate cells have the same admission result. This seems to be the principle applied in intra-CU LTM.

[Ericsson]: In Rel-18, it is not a real issue since all candidate cells are under the same CU. [Samsung]: Seems first we need to have clear understanding on the concerned scenario in Rel-18 (including whether it exists in Rel-18 and how it was resolved). Then we can further discuss possible way-forward for Rel-19. [Session chair]: Suggest to have offline discussion among interested companies. Will comeback in Thursday CB session. [ZTE]: Why not directly discuss it in RAN3? [Samsung]: The issue is in the UE side, it is reasonable to discuss it in RAN2.

- Comeback at Thursday CB session.

R2-2407793	Summary on offline discussion on DRB handling in subsequent LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 assumes that different LTM candidate cells with different admission results towards PDU sessions/DRBs is a valid case for both intra-CU and inter-CU LTM.

Proposal 2: For the solutions, RAN2 is kindly asked to discuss whether this issue can be firstly discussed in RAN2. If not, a post-email can be requested for the LS to RAN3.

[Huawei]: It is general issue, not specific to LTM. [Ericsson]: Agree with Huawei. And NW can control to avoid this situation, e.g. NW may reconfigure after the first LTM cell switch or NW may include only cells that have same PDU sessions support into LTM candidates. [Intel]: Agree with Huawei and Ericsson. [Samsung]: Some companies understand all candidate have same admission control and others understands each candidate has different admission control. [Lenovo]: Agree with Huawei. [Rakuten]: For inter-CU LTM, yes NW can control it, but source CU and candidate CUs need to exchange information for LTM candidate reconfiguration/release, which would be new to RAN3. [Ericsson]: It is supported by the existing inter-node procedures. [Apple]: Does NW control means if a candidate does not support all DRBs, the source CU will not perform cell switch to that candidate? [Ericsson]: Yes.

- RAN2 understand NW implementation can handle it.

LTM and L3 HO:

P9 in R2-2406819 (Xiaomi)

Proposal 9 (modified): L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

	When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

	The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

	For the execution order between CHO and LTM, Rel-18 principle is applied.

- Agreed.

[Apple]: Hope NW provides correct LTM configuration. [Nokia]: It would be good to change the last bullet to clarify Rel-18 principle.

Inter-CU SCG LTM:

P8, P9, P10, P11 in R2-2406305 (CATT)

Proposal 8 (modified): Inter-CU SCG LTM preparation can be initiated by source SN.

- Agreed.

Proposal 9 (modified): The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- Agreed.

Proposal 10: For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- Agreed.

Proposal 11 (modified): RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Agreed.

[Xiaomi]: It may be discussed in SA3. Prefer waiting for SA3 response. [Nokia]: It would be good to inform SA3 by LS. Previous LS didn’t include it. [LG]: New SA3 WI already include DC case. Prefer waiting for SA3 response. [Apple]: If the complete message is sent to target SN, SCPAC option may not work. We may need new mechanism. We first need to discuss signaling flows (including whether complete message is sent to MN or target SN). [OPPO]: In Rel-18, it is sent to MN with sk-counter, then MN forwards it to SN. It should be the baseline for Rel-19. [ZTE]: With P9, it means the complete message is sent to MN. It is ok with P11.

For P8:

[Vivo]: If L3 measurement is used for LTM, think MN can also initiated preparation. [Xiaomi]: Agree with Vivo. MN initiated preparation is already applicable to SCPAC. [CATT]: It is not reasonable directly inherit everything from SCPAC. The intended scenario is different, e.g. for LTM, it is mainly for mobility. [Ericsson, Huawei]: Agree with CATT.

For P9:

[Nokia]: It depends on whether there is SRB3 or not. It would be ok when SRB3 is not there, however it is not clear when SRB3 is there.

[POST127][111][MOB] (CATT)

Scope: Prepare a LS to SA3 to inform RAN2 agreements to ask them to take it into account for their works.

Intended outcome: Approved LS in R2-2407602.

Deadline: Short email discussion.

P8 in R2-2406919 (Ericsson)

Proposal 8: Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

- Agreed.

Inter-CU MCG LTM:

P9, P10, P11, P12 in R2-2406419 (ZTE)

Proposal 9: RAN2 to clarify that inter-CU MCG LTM with SCG unchanged just means that PSCell is not changed during inter-CU LTM execution, but the other change related to SCG is allowed, e.g. radio bearer type and termination change.

[Apple]: The simplest option would be to agree that SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). [Huawei]: Think SCG release is the simplest option.

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

Proposal 10: Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- Agreed.

Proposal 11 (modified): For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

- Agreed.

[CATT]: Think there is no need to add a new sk-Counter since MN-key will be anyway changed due to inter-CU MN LTM, then it will make change of SN-key. There is no security key issue even with the same sk-Counter. [Nokia]: Agree with CATT. [Huawei]: To avoid any confusion, we can simply clarify legacy Rel-15 RRC reconfiguration with sync procedure is applied.

Proposal 12: 	RAN2 to discuss how to handle the co-existence case of inter-CU MCG LTM with SCG unchanged and inter-CU MCG LTM with SCG release with the following options:

•	Option 1: The NW ensures that the UE does not switch to an LTM candidate cell with SCG configuration if the UE has released DC prior to the LTM cell switch. Additional Xn interaction is required.

•	Option 2: The UE does not apply the SCG configuration (if any) in the target LTM candidate configuration if the NW triggers the LTM cell switch to an LTM candidate cell with SCG configuration but the UE has released DC prior to the LTM cell switch.

•	Option 3: The NW can trigger LTM cell switch to any configured LTM candidate cells, regardless of whether the target candidate cell has the SCG configuration and whether the UE is in NR-DC currently. I.e. inter-CU MCG LTM with SCG addition is supported, this may be considered as out of the current WI scope.

- Noted.

[Ericsson]: We agreed that “SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration)”. With this agreement, we do not need any new option more. [Samsung]: This issue is similar to the issue raised in R2-2407421. Suggest we should first clarify the scenario in R2-2407421 and then comeback if needed. [ZTE]: Difference is that for PDU session case, it impacts to CN also while SCG release/reconfiguration does not. Solution can be different. [MediaTek]: Option2 means the UE does not follow NW configuration. Would like to avoid option 2.

Coexistence of MCG LTM and SCG LTM:

P15 in R2-2406419 (ZTE)

Proposal 15: 	RAN2 to discuss the following options for the coexistence on different types of LTM candidates:

•	Option 1: The coexistence of Intra-MN MCG LTM and Inter-SN SCG LTM, and the coexistence of Inter-MN MCG LTM and Inter-SN SCG LTM are not supported, i.e. not configured simultaneously.

•	Option 2: The coexistence of Intra-MN MCG LTM and Inter-SN SCG LTM, and the coexistence of Inter-MN MCG LTM and Inter-SN SCG LTM are supported, i.e. can be configured simultaneously. But the execution of one type of LTM shall trigger the UE to release the other type of LTM candidate configurations autonomously.

P8 in R2-2406694 (Apple)

Proposal 8: If MN has inter-CU LTM candidates configured, then the UE can be configured with intra-CU LTM in SN, but if the LTM switch in MN is inter-CU, then the UE should not have SN configured after the LTM switch.

R2-2407201	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: RAN2 works on a solution to resolve the data rate drop issue after cell switch.

- Noted.

[Qualcomm]: Understand it is also applicable to intra-LTM (not Rel-19 LTM specific issue). [Nokia]: Recall it was discussed in TEI-18 in earlier time. [Apple]: It is clear that this enhancement is not within this WID scope. [ZTE]: Observed this issue in the legacy L3 HO. Support the proposal 3. [Sony]: UE can perform aperiodic CSI report after HO/LTM. With it, can it address the concern? [Xiaomi]: In RACH based HO/LTM, the UE can report CSI in MSG3. With it, can it address this concern. [CMCC]: Support the proposal. Understand it is under RAN1 discussion. We may wait for RAN1 progress. [Futurewei]: Think it is severer issue in inter-CU LTM. [China Unicom]: Support the proposal. [LG]: Agree with Apple, it is not within this WID scopes.

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

R2-2406305	Discussion on inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406356	Further discussion on Inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406386	Discussion on inter-CU LTM	ETRI	discussion	Rel-19

R2-2406419	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406430	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406532	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406623	LTM for Inter-CU	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2406658	Discussion on Inter-CU LTM	InterDigital, Inc.	discussion	Rel-19

R2-2406694	View on open issues in inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406743	Discussion on inter-CU LTM	KT Corp.	discussion

R2-2406775	Discussion on Inter CU LTM 	Lekha Wireless Solutions	discussion	Rel-19

R2-2406819	Discussion on Inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406820	Initial considerations for inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2406854	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406863	Cell switch command for subsequent inter-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2406867	Discussion on the reference configuration for inter-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2406919	Important aspects regarding inter-CU LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406982	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407023	Further detailed discussion on supporting inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2407033	Security impacts of Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407073	On Inter-CU LTM Open Issues	Nokia	discussion

R2-2407107	Radio Resource aspects for intra-CU and inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407108	Discussion on Inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407133	Fast LTM recovery in DC scenarios	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407155	RACH-less LTM completion in inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407201	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407269	Discussion on inter-CU LTM	LG Electronics	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407320	Discussion on subsequent inter-CU or inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407348	Further discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407374	Inter-gNB LTM with no change of RRC/PDCP anchor	Qualcomm Incorporated, NTT DOCOMO, Vodafone, Bharti Airtel (India), Sony	discussion

R2-2407407	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407421	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407439	Discussion on inter-CU LTM 	Kyocera 	discussion	Rel-19

R2-2407441	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407448	Discussion on Inter-CU LTM	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2407465	Discussion on inter-CU LTM	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407478	RRC Modelling for Inter-CU LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2407483	LTM enhancements for Inter-CU mobility	CEWiT	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	Measurement event evaluation

Remaining open issues and further details of measurement event evaluation (e.g. need of event LTM1 (if needed, what’s use case)? , what beam(s) of the serving cell and neighboring cell is used for event evaluation?, Need of cell level measurement result for event evaluation (if needed, what’s use case and how it works)? Further details on event configuration signaling design, e.g. how to associate with resource configuration,, etc.)

Need of event LTM1:

P1 in R2-2406286 (Huawei)

Proposal1: Support Event LTM1 (Beam of serving cell becomes better than absolute threshold).

P1 in R2-2406733 (Qualcomm)

Proposal 1: Event LTM1 is not defined.

- P1 in R2-2406733 is agreed.

[IDC]: With the intention proposed in R2-2406286, LTM1 is not needed since we can just reuse A1. [ZTE]: LTM1 can be still used for CSI measurement reconfiguration purpose. [Ericsson, LG]: Agree with Huawei and ZTE. [Nokia, Samsung]: Agree with Qualcomm. Do not think NW will use LTM1 (instead of A1) for the intention. [Samsung]: Original intention of event triggered L1 LTM MR is to support UE mobility. For other purposes, we use A events. LTM1 is not essential. [Xiaomi]: Agree with Samsung. Note we already agreed the main intentions of LTM events.

Serving cell’s beam for L1 MR event evaluation:

P2 in R2-2406431 (Vivo)

Proposal 2: Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Proposal 2 in R2-2406431 is agreed.

P2 in R2-2406968 (CMCC)

Proposal 2: The best beam of the serving cell and neighboring cell measured by the UE should be used for event evaluation.

[Fujitsu]: Similar discussion happened in RAN1 (RAN1 had offline discussion and will come back to RAN1 online session). Assume it is RAN1 issue. Note RAN1 clarified all possible options. [Apple]: Think we need to continue each WG job. If controversial, it can be further discussed in RAN. [Apple]: In MIMO, the serving beam is compared to the neighbouring beam. And in most of cases, current beam would be the best beam. It will be good to follow same principle for mobility. [MediaTek]: Prefer CMCC proposal, however we need a restriction, i.e. should be configured by LTM configuration. [Nokia]: What matters is NW indicated beam (regardless of best or worst).

- Option1: Current beam (indicated beam by TCI state): Samsung, Apple, Vivo, Xiaomi, Qualcomm, Spreadtrum, Ericsson, Fujitsu, Rakuten, NEC, Huawei, Honor, Nokia, Lenovo, Panasonic, Sharp, IDC (17)

- Option2: Best beam: 9 companies

Candidate cell’s beam for L1 MR event evaluation:

P3 in R2-2406851 (Nokia)

Proposal 3: For the beam level event evaluation for LTM 3/4/5 the measurement resources listed in the R18 resource configuration are used for evaluating beam level events.

P4, P5 in R2-2406286 (Huawei)

Proposal4: For LTM events involving beam of the neighboring cell as the triggering quantity (e.g., Event LTM A3/5), only LTM candidate cells are applicable.

Proposal5: For LTM events with beam-level measurement of candidate cell as triggering quantity, the applicable beam of the candidate cell is the best beam of that cell.

[MediaTek]: Best beam of the candidate beams is not clear since the UE cannot measure all candidate beams. [Apple]: Agree with MediaTek. For Nokia proposal, if any beam among the beams configured by LTM RS configuration meets the event, it triggers MR. [CATT]: Agree with Nokia. Note we agreed beam-level measurement, which is more aligned with Nokia proposal. [Ericsson]: Understand best beam among all candidate RSs, not all possible RSs.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

Need of applying cell level measurement for L1 MR event evaluation:

P5 in R2-2406728 (Apple)

Proposal 5 (modified): Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- P5 in R2-2406728 is agreed.

P4 in R2-2407506 (LG)

Proposal 4: Evaluation of Event LTM based on consolidated result of multiple RSs of a serving cell and/or multiple RSs of a candidate cell is supported.

[Spreadtrum]: Think for cell level measurements, L3 measurement can be reused. [Samsung]: Share the view with Apple. If need to address LG’s concern, we can allow MR includes multiple beams (like N best beams). [Xiaomi]: Agree with Spreadtrum. [MediaTek, ZTE]: Let’s assume a beam meets event and TTT starts, during TTT if another beam that belongs to the same candidate cell meets the event, what will happen? [Vivo]: Understand each beam that meet the event has its own TTT. Support P5 in R2-2406728. [Lenovo]: For cell level L1 measured result, we can just reuse the legacy way. [Huawei]: Support cell level measurement is applied to LTM event. [CATT]: Cell level measurement is not essential for LTM L1 event evaluation. We can rely on e.g. L3 measurement if needed. However, prefer putting FFS for conditional LTM case.

LTM L1 event-triggered MR configuration:

P10, P11 in R2-2406728 (Apple)

Proposal 10: R19 LTM event triggered measurement configuration can be taken as baseline:

-	LTM measurement resource configuration is provided in LTM-config.

-	Event triggered report config is provided in serving cell config.

- P10 in R2-2406728 is agreed.

[Xiaomi, Huawei]: If MR is reported via MAC CE, there would be some issue. [Nokia]: Directly agreeing with Figure 4 is too detailed now. Prefer removing figure 4 from the conclusion.

Proposal 11: FFS on whether the event configuration can be shared across all candidate cells.

P12 in R2-2407506 (LG)

Proposal 12: Common RS pool introduced for Rel-18 LTM is extended to support configuring CSI RS.

Layer to perform event evaluation:

P7 in R2-2407446 (Kyocera)

Proposal 7: RAN2 should agree that MAC layer handles the event evaluation and measurement report triggering.

- Agreed.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

R2-2406287	Discussion on event triggered report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406306	Measurement Event Evaluation	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406357	Discussion on measurement and signalling design	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406420	Discussion on measurement event evaluation	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406431	Discussion on LTM measurement event evaluation	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406524	Discussion on beam for evaluation of LTM event-triggered reporting	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406533	Open issues for event triggered  L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406545	Open issues of measurement event evaluation for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406707	Discussion on the measurement event evaluation for LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406728	LTM measurement event evaluation and configuration	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406733	Measurement enhancements for LTM	Qualcomm Incorporated	discussion

R2-2406756	Discussion on measurement event evaluation for L1 measurement event	Spreadtrum Communications	discussion	Rel-19

R2-2406851	Further View on Measurement-related Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4	R2-2405149

R2-2406886	L1 Measurement enhancements	Lenovo	discussion	Rel-19

R2-2406908	Discussion on measurement event evaluation for LTM 	Lekha Wireless Solutions	discussion	Rel-19

R2-2406920	Important aspects regarding event triggered L1 measurements	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406968	Discussion on measurement event evaluation	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407024	Discussion on event triggered L1 measurement configuration for LTM	Transsion Holdings	discussion	Rel-19

R2-2407109	Discussion on measurement event evaluation for LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407124	Event evaluation of L1 measurement reporting	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407141	Event triggered L1 measurement evaluation for LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407160	Event based L1 measurements triggered LTM candidate cell addition/release	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407195	Reference resource configuration for L1 measurement event	Panasonic	discussion

R2-2407349	Discussion on measurement event evaluation	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407393	Discussion on measurement event evaluation	KDDI Corporation	discussion	Rel-19

R2-2407408	Discussion issues on related to measurement event evaluation	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407422	Remaining Issues for Measurement Event Evaluation	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407446	Measurement event evaluation for LTM enhancement	Kyocera	discussion	Rel-19

R2-2407470	Discussion on event triggered L1 measurement	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407506	Event LTM - Event variant, new event, RS determination, configuration	LG Electronics France	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.4	Measurement reporting

Remaining open issues and further details of measurement reporting procedure (e.g. what information in the measurement report? MAC CE or UCI for measurement report?, need of measurement reporting once leaving condition is met, etc.)

MAC or UCI for MR?

P1 in R2-2406921 (Ericsson)

Proposal 1: Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

- P1 in R2-2406921 is agreed.

P1 in R2-2407423 (Samsung)

Proposal 1: RAN2 confirm the RAN1 agreements using UCI L1 measurement reporting to align the event based L1 measurement reporting is used for LTM.

[Samsung]: UCI-based L1 measurement reporting was agreed for MIMO and it would be good to have common framework also for L1 LTM. [Session chair]: Check companies’ views.

- UCI: 3 companies (Apple, Samsung, Xiaomi)

- MAC CE: 24 companies

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Need of MR when leaving condition is met:

P3 in R2-2406432 (Vivo)

Proposal 3: It is configurable by gNB whether a UE initiates L1 measurement reporting when leaving condition is satisfied (e.g. reportOnLeave-like parameter in event configuration).

Need of event-triggered periodic MR:

P4 in R2-2406432 (Vivo)

Proposal 4: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report (e.g. reportAmount-like parameter in event configuration).

P5 in R2-2406358 (MediaTek)

Proposal 5: The event triggered L1 MR is a one-shot report. Multiple reporting for one trigger event is not introduced. (This does not preclude multiple report by multiple trigger events configuration).

Information in MR:

P1 in R2-2407285 (NTTDCM)

Proposal 1: Event-triggered L1 measurement includes below contents;

・Beam information (i.e., SSBRI or CRI)

・Beam quantity (up to RAN1)

・Triggered event information (e.g., ReportConfigID)

P2 in R2-2407423 (Samsung)

Proposal 2: Event triggered L1 measurement reporting for LTM includes following information:

-	L1 RSRP of the current beam

-	L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition)

-	DL RS resource index (SSB or CSI-RS)

R2-2406286	Discussion on measurement event evaluation	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406307	Measurement Reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406358	Discussion on measurement reporting of event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406421	Discussion on measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406432	Discussion on event-triggered L1 measurement reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406534	Discussion on the UL signalling for L1 reproting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406546	Open issues of measurement reporting procedure for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406708	Event-based measurement reporting procedure for L1 measurement	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406729	LTM event triggered measurement reporting	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406757	Discussion on measurement reporting for L1 measurement event	Spreadtrum Communications	discussion	Rel-19

R2-2406921	Discussion on which layer reports the event triggered L1 measurements	Ericsson, Honor, Huawei, InterDigital Inc., MediaTek Inc., NEC, Nokia, NTT DOCOMO, Oppo, T-Mobile USA, Vivo, ZTE corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406969	Discussion on measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407025	Discussion on L1 measurement reporting for LTM	Transsion Holdings	discussion	Rel-19

R2-2407110	Discussion on measurement reporting for LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407125	Details of event triggered L1 measurement report	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407142	Event triggered L1 measurement reporting for LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407208	Event triggered reporting enhancements for LTM	Panasonic	discussion	Rel-19

R2-2407285	Discussion on Event-triggered L1 measurement reporting	NTT DOCOMO, INC.	discussion	Rel-19

R2-2407350	Discussion on measurement reporting	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407394	Discussion on event triggered L1 measurement reporting	KDDI Corporation	discussion

R2-2407409	Discussion issues on related to measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407423	Support of Event Triggered L1 Measurement Reporting	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407447	Measurement reporting procedures for LTM enhancements	Kyocera	discussion	Rel-19

R2-2407507	Event LTM - Report triggering, report contents and transmission procedure	LG Electronics France	discussion	Rel-19	NR_Mob_Ph4-Core