3GPP TSG-RAN WG2 Meeting #127	R2-2407587

# Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1904 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User plane corrections for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editorial issues a)There are different wordings for ongoing RACH-less LTM cell switch b) Text for UE-based TA measurement is unclear and not aligned with 38.331 c) The MAC CE description refers to ltm-UL-TCI-StatesToAddModList, which is not the exact name d) The wording in 5.2 on "the PTAG indicated by the LTM cell switch MAC CE command" gives the impression that the PTAG is a field of this MAC CE, while it is not. e) It is not clear what "invalid" Target Configuration ID exactly means. Functional issues 1) From R2-2406331 It is not captured that the configured candidate cell TCI states are initially deactivated upon (re-)configuration by upper layer and after reconfiguration with sync that is not triggered by LTM. 2) From R2-2406349 According to 6.1.3.75, when the LTM target cell is configured with MIMO 2TA and separate TCI states, the UE considers the TA command to always apply to the main TAG because the UE considers DL TCI state and not the UL TCI state. 3) From R2-2406349 According to 5.1.1, the power ramping counter is to be reset at early RACH if the cell is different from the last cell where the UE has performed the last preamble transmission, but in NR-DC this may be done by the other MAC entity, in which case the power ramping counter should not be reset 4) From R2-2407199 During RACH-less LTM cell switch, upon reception of a DL assignment addressed to the MAC entity's C-RNTI, to declare RACH-less LTM cell switch to be complete, there is a condition on a "new transmission". This condition is not useful because there was no previous transmission in any DL HARQ process so any such DL assignment is for a new transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial issues a) in 5.3.1, 5.4.1: use the same wording at every place for "ongoing RACH-less LTM cell switch" b) in 5.18.35: for UE-based TA measurement, align the wording with TS 38.331, refer to the SpCell and add "configuration" after "LTM target" (this aligns with the field name in the MAC CE) c) in 6.1.3.75, correct the field name d) Refer to 6.1.3.75 that captures information on the PTAG. f) change "invalid" to "not matching any configured ltm-CandidateId" Functional issues 1) in 5.18.36, capture that the configured candidate cell TCI states are initially deactivated upon (re-)configuration by upper layer and after reconfiguration with sync that is not triggered by LTM. 2) In 6.1.3.75, for the TA command, the UE considers tag-id-ptr in the UL TCI state when indicated in the LTM cell switch MAC CE command. 3) In 5.1.1, change "the UE" to "the MAC entity" 5) In 5.3.1, remove the condition on "new transmission" Impact analysis Impacted 5G architecture options: NR standalone (except for 3), NR-DC Impacted functionality: LTM Inter-operability: If the UE is implemented according to this CR while the network is not: 1) The UE may perform unnecessary tracking of neighbour cell L1 RS, possibly resulting in poorer performance. 2) to 4) No interoperability issue. If the network is impleted according to this CR while the UE is not: 1) and 4) No interoperability issue. 2) If the target cell is configured with MIMO 2TA and separate TCI states, the UE may apply the TA command to the wrong TAG, resulting in RACH-less LTM cell switch failure. 3) In NR-DC, early RACH may fail due to no power ramping |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing wordings and incorrect field names for LTM operation remain in TS 38.321. 1) The UE may perform unnecessary tracking of neighbour cell L1 RS, possibly resulting in poorer performance. 2) RACH-less LTM cell switch to a candidate cell with MIMO 2TA and separate TCI states may fail 3) In NR-DC, early RACH may fail due to no power ramping 4) Unnecessary UE check remains for RACH-less LTM cell switch completion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3.1, 5.4.1, 5.18.35, 5.18.36, 6.1.3.75 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 5	MAC procedures

## 5.2	Maintenance of Uplink Time Alignment

RRC configures the following parameters for the maintenance of UL time alignment:

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG;

-	inactivePosSRS-TimeAlignmentTimer which controls how long the MAC entity considers the Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned;

-	cg-SDT-TimeAlignmentTimer which controls how long the MAC entity considers the uplink transmission for CG-SDT to be uplink time aligned;

-	inactivePosSRS-ValidityAreaTAT which controls how long the MAC entity considers Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned when SRS positioning validity area is configured.

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26:

3>	if SRS positioning validity area is configured:

4>	start or restart the inactivePosSRS-ValidityAreaTAT associated with the indicated TAG.

3>	else:

4>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with the indicated TAG.

2>	else:

3>	start or restart the timeAlignmentTimer associated with the indicated TAG.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell configured with two TAGs or in a MSGB for an SpCell configured with two TAGs:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for the TAG indicated in the received Random Access Response message or MSGB;

3>	start or restart the timeAlignmentTimer associated with TAG indicated in the received Random Access Response message or MSGB.

2>	else if the timeAlignmentTimer associated with the TAG indicated in the received Random Access Response message or MSGB is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5:

4>	stop the timeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell not configured with two TAGs or in a MSGB for an SpCell not configured with two TAGs:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for this TAG;

3>	start or restart the timeAlignmentTimer associated with this TAG.

2>	else if the timeAlignmentTimer associated with this TAG is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5; or

3>	when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE:

4>	stop timeAlignmentTimer associated with this TAG.

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5:

4>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

5>	set the NTA value to the value before applying the received Timing Advance Command as in TS 38.211 [8].

3>	when the Contention Resolution is considered successful for Random Access procedure while the CG-SDT procedure is ongoing:

4>	stop timeAlignmentTimer associated with this TAG;

4>	start or restart the cg-SDT-TimeAlignmentTimer associated with this TAG.

3>	when the Contention Resolution is considered successful for Random Access procedure while SRS transmission in RRC_INACTIVE is ongoing:

4>	if SRS positioning validity area is configured:

5>	start or restart the inactivePosSRS-ValidityAreaTAT associated with the indicated TAG.

4>	else:

5>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE, as specified in clause 5.1.4a, for an SpCell configured with two TAGs:

2>	apply the Timing Advance Command for the PTAG indicated in the Absolute Timing Advance Command MAC CE;

2>	start or restart the timeAlignmentTimer associated with this PTAG.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE, as specified in clause 5.1.4a, for an SpCell not configured with two TAGs:

2>	apply the Timing Advance Command for PTAG;

2>	if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26:

3>	if SRS positioning validity area is configured:

4>	start or restart the inactivePosSRS-ValidityAreaTAT associated with the indicated TAG.

3>	else:

4>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with PTAG.

2>	else:

3>	start or restart the timeAlignmentTimer associated with PTAG.

1>	when the MAC entity is configured with rach-LessHO:

2>	set the NTA value (as defined in TS 38.211 [8]) to the value indicated by targetNTA in rach-LessHO for PTAG;

2>	start the timeAlignmentTimer associated with PTAG.

1>	when the indication is received from upper layer for stopping the inactivePosSRS-TimeAlignmentTimer:

2>	stop the inactivePosSRS-TimeAlignmentTimer.

1>	when the indication is received from upper layer for starting the inactivePosSRS-TimeAlignmentTimer:

2>	start or restart the inactivePosSRS-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for starting the cg-SDT-TimeAlignmentTimer:

2>	start the cg-SDT-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for stopping the cg-SDT-TimeAlignmentTimer:

2>	consider the cg-SDT-TimeAlignmentTimer as expired.

1>	when the indication is received from upper layer for starting the inactivePosSRS-ValidityAreaTAT:

2>	start or restart the inactivePosSRS-ValidityAreaTAT.

1>	when the indication is received from upper layer for stopping the inactivePosSRS-ValidityAreaTAT:

2>	stop the inactivePosSRS-ValidityAreaTAT.

1>	when instruction from the upper layer has been received for starting the TimeAlignmentTimer associated with PTAG:

2>	start the TimeAlignmentTimer associated with the indicated PTAG.

1>	when an LTM Cell Switch Command MAC CE is received and the Timing Advance Command is not set as FFF:

2>	apply the Timing Advance Command for the PTAG as specified in clause 6.1.3.75;

2>	start or restart the timeAlignmentTimer associated with the PTAG as specified in clause 6.1.3.75.

1>	when an LTM Cell Switch Command MAC CE is received, and the Timing Advance Command is set as FFF, and the UE has successfully measured the Timing Advance as in clause 5.18.35:

2>	apply the measured Timing Advance for the PTAG;

2>	start or restart the timeAlignmentTimer associated with the PTAG.

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or

2>	if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

2>	else:

3>	if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or

3>	if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running:

4>	flush all HARQ buffers for all such SCells;

4>	notify RRC to release PUCCH, if configured for all such SCells;

4>	notify RRC to release SRS, if configured for all such SCells;

4>	clear any configured downlink assignments and configured uplink grants for all such SCells;

4>	clear any PUSCH resource for semi-persistent CSI reporting for all such SCells;

4>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

3>	else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells:

4>	clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer;

4>	clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer;

4>	clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer;

4>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

1>	when the inactivePosSRS-TimeAlignmentTimer expires:

2>	notify RRC to release Positioning SRS for RRC_INACTIVE configuration(s).

1>	when the cg-SDT-TimeAlignmentTimer expires:

2>	clear any configured uplink grants;

2>	if a PDCCH addressed to the MAC entity's C-RNTI after initial transmission for the CG-SDT with CCCH message has not been received:

3>	consider ongoing CG-SDT procedure as terminated;

3>	indicate the expiry of cg-SDT-TimeAlignmentTimer to the upper layer.

2>	flush all HARQ buffers;

2>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell not configured with two TAGs due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

When the MAC entity stops uplink transmissions associated to a STAG for an SCell configured with two TAGs due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the STAG as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer(s) associated with all TAG(s) to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing. Furthermore, when the timeAlignmentTimer(s) associated with all PTAG(s) is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer or inactivePosSRS-ValidityAreaTAT is not running. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.

## 5.3	DL-SCH data transfer

### 5.3.1	DL Assignment reception

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, CS-RNTI, G-RNTI or G-CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI, or G-RNTI configured for multicast MTCH:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or G-CS-RNTI, or a configured downlink assignment for unicast or MBS multicast; or

2>	if the downlink assignment is for the MAC entity's G-RNTI configured for multicast MTCH, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or G-CS-RNTI, or other G-RNTI, or C-RNTI, or a configured downlink assignment for unicast or MBS multicast:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	stop the cg-SDT-RetransmissionTimer, if it is running, for the corresponding HARQ process for initial transmission with CCCH message;

2>	stop the cg-RRC-RetransmissionTimer, if it is running, for the corresponding HARQ process for initial transmission of RACH-less handover or for the first PUSCH transmission at RACH-less LTM cell switch;

2>	stop the configuredGrantTimer, if it is running, for the corresponding HARQ process for initial transmission with CCCH message;

2>	if the downlink assignment has been received on the PDCCH for the MAC entity's C-RNTI after the first PUSCH transmission to the Serving Cell

:

3>	if there is an ongoing RACH-less handover procedure:

4>	consider the RACH-less handover to be successfully completed and indicate it to upper layers.

3>	else if RACH-less LTM cell switch is ongoing:

4>	consider the LTM cell switch to be successfully completed and indicate it to upper layers.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI or G-CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is running, and the Serving Cell is not configured with two TAGs; or

4>	if the Serving Cell on which the HARQ feedback is to be transmitted is configured with two TAGs and if the timeAlignmentTimer of the TAG, associated with the TCI state(s) used for transmitting the HARQ feedback, is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in clause 5.8.1 or in clause 5.8.1a;

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments without harq-ProcID-Offset, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))]
	modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

For configured downlink assignments with harq-ProcID-Offset, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))]
	modulo nrofHARQ-Processes + harq-ProcID-Offset

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

NOTE 1:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured downlink assignments.

NOTE 2:	CURRENT_slot refers to the slot index of the first transmission occasion of a bundle of configured downlink assignment.

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

When the MAC entity needs to read MCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the MCCH-RNTI or Multicast MCCH-RNTI:

2>	indicate a downlink assignment and redundancy version for the selected HARQ process to the HARQ entity.

When the MAC entity needs to read broadcast MTCH, the MAC entity may, based on the scheduling information from RRC and DCI:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the G-RNTI configured for broadcast MTCH:

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information for the selected HARQ process to the HARQ entity.

## 5.4	UL-SCH data transfer

### 5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.

For a BWP configured with sTx-2Panel, the MAC entity considers the PUSCH duration of one uplink grant overlaps with the PUSCH duration of another uplink grant if they are overlapping in time and associated with an srs-ResourceSetId corresponding to the same coresetPoolIndex.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

2>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running.

2>	stop the cg-RRC-RetransmissionTimer for the corresponding HARQ process, if running.

2>	if the uplink grant has been received on the PDCCH for the MAC entity's C-RNTI after the first PUSCH transmission to the Serving Cell; and

2>	if the uplink grant is for a new transmission on the same HARQ process used for the first PUSCH transmission to the Serving Cell:

3>	if there is an ongoing RACH-less handover procedure:

4>	consider the RACH-less handover to be successfully completed and indicate to upper layers.

3>	else if there is an ongoing RACH-less LTM cell switch:

4>	consider the LTM cell switch to be successfully completed and indicate it to upper layers.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running;

3>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running;

3>	stop the cg-RRC-RetransmissionTimer for the corresponding HARQ process, if running;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity;

3>	if a logical channel associated with a DRB configured with survivalTimeStateSupport is multiplexed in the MAC PDU stored in the HARQ buffer for the corresponding HARQ process:

4>	trigger activation of PDCP duplication for all configured RLC entities of the DRB.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in clause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

For each Serving Cell and each configured uplink grant, if configured and activated and available for use as specified in clause 5.8.2, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

1>	if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured and cg-SDT-RetransmissionTimer is not configured, and cg-RRC-RetransmissionTimer is not configured (i.e. new transmission):

3>	if the configured uplink grant is for the initial transmission for the CG-SDT with CCCH message; or

3>	if there is an ongoing CG-SDT procedure and PDCCH addressed to the MAC entity's C-RNTI has been received; or

3>	if the configured uplink grant is for the first PUSCH transmission during an ongoing RACH-less LTM cell switch; or

3>	if the configured uplink grant is for the first PUSCH transmission during an ongoing RACH-less handover procedure; or

3>	if there is no ongoing CG-SDT nor ongoing RACH-less LTM cell switch nor ongoing RACH-less handover procedure:

4>	consider the NDI bit for the corresponding HARQ process to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process:

3>	if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant):

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-SDT-RetransmissionTimer is configured and not running for the corresponding HARQ process; or

2>	if the cg-RRC-RetransmissionTimer is configured and not running for the corresponding HARQ process:

3>	if the configured uplink grant is for the first PUSCH transmission at RACH-less LTM cell switch (i.e., initial new transmission); or

3>	if the configured uplink grant is for the initial transmission of RACH-less handover (i.e., initial new transmission); or

3>	if the configured uplink grant is for the initial transmission for the CG-SDT with CCCH message (i.e., initial new transmission); or

3>	if the configuredGrantTimer is not running or not configured, and PDCCH addressed to the MAC entity's C-RNTI has been received after the initial transmission of the CG-SDT with CCCH message (i.e., subsequent new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	if PDCCH addressed to the MAC entity's C-RNTI has not been received and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for initial transmission of CG-SDT with CCCH message or for its retransmission (i.e., retransmission for initial CG-SDT transmission); or

3>	if RACH-less handover is not successfully completed and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for initial transmission of RACH-less handover or for its retransmission (i.e., retransmission for initial RACH-less handover transmission); or

3>	if RACH-less LTM cell switch is ongoing and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for first PUSCH transmission at RACH-less LTM cell switch or for its retransmission (i.e., retransmission for initial transmission at RACH-less LTM cell switch):

4>	consider the NDI bit to have not been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants that are not part of a multi-PUSCH configured grant and neither configured with harq-ProcID-Offset2 nor with cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

For configured uplink grants that are not part of a multi-PUSCH configured grant and configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

For a multi-PUSCH configured grant (as specified in clause 5.8.2) configured with neither harq-ProcID-Offset2 nor cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes

For a multi-PUSCH configured grant configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period × floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]; alternatively, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is configured, CURRENT_symbol = ((H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSFNperH-SFN, numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive frames per H-SFN, the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. For a multi-PUSCH configured grant, ID_OFFSET equals 0 for the first configured uplink grant within a periodicity of the configuration and K for the Kth (1 ≤ K < nrofSlotsInCG-Period) valid configured uplink grant after the first configured uplink grant within the same periodicity. A configured uplink grant in a multi-PUSCH configured grant is considered valid if it satisfies the conditions specified in clause 6.1 in TS 38.214 [7].

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

NOTE 1:	If a configured uplink grant is associated with a multi-PUSCH configured grant, CURRENT_symbol refers to the symbol index of the first transmission occasion in the first configured uplink grant within the same periodicity. Otherwise, CURRENT_symbol refers to the symbol index of the first transmission occasion of a bundle of configured uplink grant.

NOTE 2:	A HARQ process is configured for a configured uplink grant where neither harq-ProcID-Offset nor harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes. A HARQ process is configured for a configured uplink grant where harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is greater than or equal to harq-ProcID-Offset2 and less than sum of harq-ProcID-Offset2 and nrofHARQ-Processes for the configured grant configuration.

NOTE 3:	If the MAC entity receives a grant in a Random Access Response (i.e. MAC RAR or fallbackRAR), or addressed to Temporary C-RNTI or determines a grant as specified in clause 5.1.2a for MSGA payload and if the MAC entity also receives an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI/Temporary C-RNTI/MSGB-RNTI/the MSGA payload transmission or the grant for its C-RNTI or CS-RNTI.

NOTE 4:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured uplink grants.

NOTE 5:	If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this de-prioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. If this de-prioritized uplink grant is configured with autonomousTx, the cg-RetransmissionTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running.

When the MAC entity is configured with lch-basedPrioritization, for each uplink grant delivered to the HARQ entity and whose associated PUSCH can be transmitted by lower layers, the MAC entity shall:

1>	if this uplink grant is received in a Random Access Response (i.e. in a MAC RAR or fallback RAR), or addressed to Temporary C-RNTI, or is determined as specified in clause 5.1.2a for the transmission of the MSGA payload:

2>	consider this uplink grant as a prioritized uplink grant.

1>	else if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI:

2>	if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup;

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

1>	else if this uplink grant is a configured uplink grant:

2>	if there is no overlapping PUSCH duration of another configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUSCH duration of an uplink grant addressed to CS-RNTI with NDI = 1 or C-RNTI which was not already de-prioritized, in the same BWP, whose priority is higher than or equal to the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup.

NOTE 6:	If the MAC entity is configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants whose priorities are equal, the prioritized uplink grant is determined by UE implementation.

NOTE 7:	If the MAC entity is not configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants, it is up to UE implementation to choose one of the configured uplink grants.

NOTE 8:	If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the PUSCH duration of an uplink grant overlaps with the PUCCH resource for an SR transmission.

## 5.18	Handling of MAC CEs

### 5.18.35	LTM Cell Switch Command

The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75.

The MAC entity shall:

1>	if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell:

2>	indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE;

2>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

3>	if Timing Advance Command value (hexa-decimal) is not set as FFF:

4>	process the received Timing Advance Command (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing;

4>	if the MAC entity is associated with SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration:

4>	process the measured Timing Advance (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing.

4>	if the MAC entity is associated with SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2);

3>	indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

### 5.18.36	Candidate Cell TCI States Activation/Deactivation

The network may activate and deactivate the TCI states of LTM candidate cell(s) configured in CandidateTCI-State and CandidateTCI-UL-State by sending the Candidate Cell TCI States Activation/Deactivation MAC CE described in clause 6.1.3.76. The network deactivates the TCI state(s) for one LTM candidate cell by not including the corresponding TCI state ID field(s) in the Candidate Cell TCI States Activation/Deactivation MAC CE. The configured candidate cell TCI states are initially deactivated upon (re-)configuration by upper layer and after reconfiguration with sync that is not triggered by LTM.

The MAC entity shall:

1>	if the MAC entity receives a Candidate Cell TCI States Activation/Deactivation MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the Candidate Cell TCI States Activation/Deactivation MAC CE.

# 6	Protocol Data Units, formats and parameters

## 6.1	Protocol Data Units

### 6.1.3	MAC Control Elements (CEs)

#### 6.1.3.75	LTM Cell Switch Command MAC CE

The LTM Cell Switch Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields (Figure 6.1.3.75-1):

-	R: Reserved bit, set to 0;

-	Target Configuration ID: This field indicates the index of candidate target configuration to apply for LTM cell switch, corresponding to ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits;

-	Timing Advance Command: This field indicates whether the TA is valid for the LTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID field). If the value of this field is set to FFF, this field indicates that no valid timing adjustment is available for the PTAG of the LTM target cell; otherwise, this field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6], and that the UE can skip the Random Access procedure for this LTM cell switch. If tag-Id-ptr is configured for the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM target cell and tag-Id-ptr is set to value n1, this field indicates the TA for the TAG indicated by the tag2-Id of the LTM target cell; otherwise, this field indicates the TA for the TAG indicated by the tag-id of the LTM target cell. The length of the field is 12 bits;

-	TCI state ID: This field indicates and activates the TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The TCI state is identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList as specified in TS 38.331 [5]. If the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is joint, this field is for joint TCI state, otherwise, this field is for downlink TCI state. The length of the field is 7 bits;

-	UL TCI state ID: This field indicates and activates the uplink TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The UL TCI state is identified by TCI-UL-StateId in ltm-UL-TCI-StateToAddModList as specified in TS 38.331 [5]. The octet containing this field (i.e. this field and the two reserved bits in the same octet) is included if the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is separate. The length of the field is 6 bits;

-	C: This field indicates the presence of the contention-free Random Access Resources fields. If the value of this field is set to 1, the following fields are present: Random Access Preamble index field, S/U field, SS/PBCH index field, PRACH Mask index field, Repetition number field and the reserved bits in the same octet. If the value of this field is set to 0, these fields are absent.

-	S/U: This field indicates which UL carrier to transmit the PRACH of the contention-free Random Access Resources. If the value of this field is set to 1, SUL is used; otherwise, NUL is used. The length of the field is 1 bit;

-	Random Access Preamble index: This field indicates the Random Access Preamble index of the contention-free Random Access Resources. This field should not be set to 0b000000. The length of the field is 6 bits;

-	SS/PBCH index: This field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission of the contention-free Random Access Resources. The length of the field is 6 bits;

-	PRACH Mask index: This field indicates the RACH occasion(s) associated with the SS/PBCH indicated by 'SS/PBCH index' for the PRACH transmission of the contention-free Random Access Resources. It indicates a subset of RACH occasion(s) from the rach-ConfigDedicated for the UL carrier (indicated by S/U field), (if provided, otherwise it indicates a subset of RACH occasion(s) from the rach-ConfigCommon for the UL carrier (indicated by S/U field) in the UL BWP configuration of firstActiveUplinkBWP-Id as specified in TS 38.331 [5]. When the repetition number field is not set to 0, the UE ignores this field. The length of the field is 4 bits;

-	Repetition number: This field indicates the Msg1 repetition number to be applied to the contention-free Random Access. If this field is set to 0, Msg1 repetition number does not apply. If this field is set to 1, the Msg1 repetition number is 2. If this field is set to 2, the Msg1 repetition number is 4. If this field is set to 3, the Msg1 repetition number is 8. The length of the field is 2 bits.

NOTE 1:	A non-zero Msg1 repetition number value may only be included in the LTM Cell Switch Command MAC CE when the LTM target cell configuration has contention-based Random Access Resources with a FeatureCombinationPreambles with the same Msg1 repetition number value and featureCombination indicating only msg1-Repetitions.

Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

NOTE 2:	If UE receives the LTM Cell Switch Command MAC CE with a Target Configuration ID value not matching any configured ltm-CandidateId minus 1, as specified in TS 38.331 [5], the procedue of handling LTM Cell Switch Command MAC CE in clause 5.18.35 does not apply.

#### 6.1.3.76	Candidate Cell TCI States Activation/Deactivation MAC CE

The Candidate Cell TCI States Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size consisting of following fields:

-	Candidate Cell ID: This field indicates the identity of an LTM candidate cell for which the MAC CE applies, corresponding to the ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits;

-	Pi: This field indicates whether each TCI codepoint has multiple TCI states or a single TCI state. If the Pi field is set to 1, the ith TCI codepoint includes the DL TCI state and the UL TCI state. If the Pi field is set to 0, the ith TCI codepoint includes only the DL/joint TCI state or the UL TCI state. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields;

-	D/U: This field indicates whether the TCI state ID in the same octet is for a joint/downlink or an uplink TCI state. If this field is set to 1, the TCI state ID in the same octet is for joint/downlink TCI state. If this field is set to 0, the TCI state ID in the same octet is for uplink TCI state;

-	TCI state ID: This field indicates the TCI state identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList or TCI-UL-StateId in ltm-UL-TCI-StatesToAddModList as specified in TS 38.331 [5]. If D/U is set to 1, 7-bits length TCI state ID i.e. TCI-StateId as specified in TS 38.331 [5] is used. If D/U is set to 0, the most significant bit of TCI state ID is considered as the reserved bit and remaining 6 bits indicate the TCI-UL-StateId as specified in TS 38.331 [5]. The maximum number of activated TCI states is 16;

-	R: Reserved bit, set to 0.

Figure 6.1.3.76-1: Candidate Cell TCI state activation/deactivation MAC CE