3GPP TSG-RAN2 Meeting # 127	R2-2407634

# Maastricht, The Netherlands, August 19th - 23rd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0890 | rev | 1 | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Transport Channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In clause 5.5 Transport Channels, it is not correct to characterise Paging Channel (PCH) to be broadcast by beamforming different BCH instances. Instead, it should beamform different PCH instances. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.5 Transport Channels, characterise Paging Channel (PCH) to be broadcast by beamforming different PCH instances, i.e. change “BCH” to “PCH”. Impact analysis Impacted 5G architecture options: NR SA, NE-DC and NR-DC Impacted functionality: Stage 2 Paging Channel (PCH) Inter-operability: If the network implements this CR but not the UE, there is no interoperability issue. If the UE implements this CR but not the network, there is no interoperabiilty issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is wrong to broadcast Paging Channel (PCH) by beamforming BCH instance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - R2-2407282: initial version; - R2-2407634: rev1, implement the following agreement at RAN2 #127: - All agreed except the R16 CR which should be cat A. Use R2-2407634 for the update of the R16 CR. |  |  |  |  |  |  |  |  |

Start of Change

## 5.5	Transport Channels

The physical layer offers information transfer services to MAC and higher layers. The physical layer transport services are described by how and with what characteristics data are transferred over the radio interface. An adequate term for this is "Transport Channel". This should be clearly separated from the classification of what is transported, which relates to the concept of logical channels at MAC sublayer.

Downlink transport channel types are:

1.	Broadcast Channel (BCH) characterised by:

-	fixed, pre-defined transport format;

-	requirement to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different BCH instances.

2.	Downlink Shared Channel (DL-SCH) characterised by:

-	support for HARQ;

-	support for dynamic link adaptation by varying the modulation, coding and transmit power;

-	possibility to be broadcast in the entire cell;

-	possibility to use beamforming;

-	support for both dynamic and semi-static resource allocation;

-	support for UE discontinuous reception (DRX) to enable UE power saving.

3.	Paging Channel (PCH) characterised by:

-	support for UE discontinuous reception (DRX) to enable UE power saving (DRX cycle is indicated by the network to the UE);

-	requirement to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different PCH instances;

-	mapped to physical resources which can be used dynamically also for traffic/other control channels.

Uplink transport channel types are:

1.	Uplink Shared Channel (UL-SCH) characterised by:

-	possibility to use beamforming;

-	support for dynamic link adaptation by varying the transmit power and potentially modulation and coding;

-	support for HARQ;

-	support for both dynamic and semi-static resource allocation.

2.	Random Access Channel(s) (RACH) characterised by:

-	limited control information;

-	collision risk.

Sidelink transport channel types are:

1.	Sidelink broadcast channel (SL-BCH) characterised by:

-	pre-defined transport format.

2.	Sidelink shared channel (SL-SCH) characterised by:

-	support for unicast transmission, groupcast transmission and broadcast transmission;

-	support for both UE autonomous resource selection and scheduled resource allocation by NG-RAN;

-	support for both dynamic and semi-static resource allocation when UE is allocated resources by the NG-RAN;

-	support for HARQ;

-	support for dynamic link adaptation by varying the transmit power, modulation and coding.

Association of transport channels to physical channels is described in TS 38.202 [20].

End of Change