3GPP TSG-RAN WG2 Meeting #127                                                               R2-2407770

MAASTRICHT 19.08.2024-23.08.2024

Source: 	Vodafone, BT, Qualcomm, Samsung, Apple, Vivo

Title: 		Support of L3 measurement based LTM over F1 Interface

Document for: Decision

Agenda Item: 7.4.2

## Introduction

In the late phase of Rel-18 LTM definition, it was decided, that LTM can also be triggered based on L3 measurements. To that end, RAN2 has also agreed that the UE capability for LTM is decoupled from the capability to support L1 measurements. Consequently, L3 measurements by the UE are sent via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution.

## Discussion

During Release 19 LTM discussion, RAN WG 2 agreed on proposal 3 in R2-2406694: “For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target CU of the target LTM cell”.

During this discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case where LTM is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards.

## Conclusions

Proposal 1: RAN2 should ask RAN3 to introduce F1 support (F1 signalling from CU to DU) for the case where LTM is based on  UE L3 measurements