3GPP TSG-RAN WG2 Meeting #127	R2-2407793

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:	8.6.2

Source:	Samsung

Title:	Summary on offline discussion on DRB handling in subsequent LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Issue and discussion

When preparing one inter-CU LTM candidate cell, the candidate gNB-CU should perform the admission control for the PDU sessions. If different inter-CU LTM candidate cells have different admission results (i.e., different LTM candidate cells have different accepted PDU sessions), the UE may need perform the PDU session addition/release during the subsequent LTM cell switch. Fig. 1 shows one example. The source cell admits PDU session 1&2&3. However, when preparing the LTM candidate cells, the LTM candidate cell 1 can only accept PDU session 1&2, while the LTM candidate cell 2 can accept PDU session 1&2&3. During the subsequent LTM cell switch in the order of “source cell  LTM candidate cell 1  LTM candidate cell 2”, the UE encounters a situation that a released PDU session appear in a new cell without establishment, i.e., :

- After UE accesses cell 2 (LTM candidate cell 1), it will release PDU session 3, which may result in the service release at the application layer.

- After UE accesses cell 3(LTM candidate cell 2), the RRC reconfiguration of cell 3 contains the DRB(s) for the PDU session 3. However, the UE already releases PDU session 3 in cell 2. Normally, a PDU session served by the DRB should be established first. However, this case results in that the released PDU session (i.e., PDU session 3) in cell 2 appears in cell 3 without PDU session establishment procedure.

Fig. 1 Example on the appearance of a released PDU session without PDU session setup

Scenario: due to different admission results of different LTM candidate cells, a PDU session (and the corresponding DRB(s)) released in one LTM candidate cell (e.g., cell1) may appear again in another LTM candidate cell (e.g., cell 2) when performing the LTM cell switch from cell 1 to cell 2.

Question 1:  Is the above scenario a valid scenario for both Rel18 and Rel19 from RAN2 perspective?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes but see comments | From LTM configuration point of view, we agree above “LTM candidate cell1+LTM candidate cell 2” is a possible configuration. But as we commented online, after LTM cell switch is triggered from source cell to candidate cell 1, the target CU knows that PDU session 3 should be deleted, and it will inform CN about the release of PDU session. After that, the target CU(becomes new serving CU) is responsible to reconfigure other LTM candidates to avoid the “invalid configuration (e.g. PDU session3)”. So, we think the reasonable NW implementation won’t trigger LTM cell switch directly to LTM candidate cell 2 without reconfigure the candidate configuration. Note that after PDU session has been removed, it can’t be added back even if NW keeps the PDU session configuration in LTM candidate. So, the proposal on “UE automatically release” seems to allow the NW to not update the candidate configuration after knowing candidate configurations come invalid? |
| Intel | No (with comments) | It was discussed briefly in R18 and some claimed it can be supported but there was no discussion or clear understanding of how it will be supported in my understanding. Considering the complexity, we do not see sufficient motivation to support this. |
| Fujitsu | See comment | 1st LTM cell switch in the picture may be possible. At LTM cell switch, if there are no DRBs associated to the PDU session 3, UE can release SDAP entity associated to the PDU session 3, and the AMF may be able to initiate PDU session release to release the PDU session 3 by receiving path switch command from the target CU. 2nd LTM cell switch in the picture is not possible because PDU session establishment is need to be initiated by the core network or the UE and this is not possible at the same time with LTM cell switch. But adding PDU session 3 may be possible after LTM cell switch to the candidate cell 2. |
| Rakuten | See comment | We think a 3rd option is also feasible. During the inter-CU LTM candidate cell preparation, the outcome of the admission control is expected to be notified to the source gNB-CU (based on preparation in other HO schemes). Further during LTM execution, when the source gNB-DU notifies the source gNB-CU about the inter-CU LTM cell switch, the source gNB-CU may inform the new serving configuration of the UE to the other candidate gNB-CUs. Therefore, the candidate gNB-CUs can also release the corresponding PDU session, which is already released by the new source gNB-CU. However, we think that a source gNB-DU should be made aware of such admission control limitations during the LTM candidate cell preparation itself, so that LTM cell switch to such a cell can be avoided, if possible. |
| Nokia | See comments | General comment on the scenario and need to support In our view it is possible that across potential LTM candidate cells the list of admitted bearer may vary (DRB list). This should be allowed as it is valid scenario. If there are some issues related to subsequent LTM as indicated in the figure RAN2/RAN3 can discuss whether we can have solution that avoid ‘another RRC Reconfiguration’ across switching or if it requires another RRC Reconfiguration how to allow /enable can be discussed. Restricting or releasing the bearers at candidate-cells at the pre-configuration itself will have impacts on overall performance. Handling of DRB configuration for LTM LTM configurations needs to be either complete configuration OR delta over reference configuration. So every candidate config should include its admitted DRB list in the candidate configuration. And the UE checks the DRB-list of candidate config and decide on what to release. So presence of DRB in list of C2 does not lead to UE automatically assume that this PDU session is already active. Instead this information can be used to trigger the initiation of PDU session. However, addition cannot be handled automatically during LTM switch (i.e cell switch from C1 and C2). In this case there should new procedure to setup this PDU session after switching either from UE /NW side that will involve CN signalling. And it will also delay the subsequent switching until this is completed. The feasibility of NW implementation for this to be checked as first step. Observation: Scenario is valid. The UE should be able to handle the mismatch after cell switch via new signalling (RRC /NAS). |
| LGE | See comment | We share the view with ZTE. LTM configuration described in Figure 1 is possible. We think the network can handle PDU session change during subsequent LTM cell switches. |
| CATT | See comments | We agree that the admission control for the PDU sessions on different LTM candidate cells could be different. But we see no issue here, similar view as ZTE. If a PDU session is released on a target cell,the target CU will communicate with CN to trigger PDU session release to UE and the target CU will send the update LTM configurations(DRB associated to the released PD session is removed) to UE. This is legacy procedure. If a PDU session is added on a target cell, the target CU will communicate with CN to trigger PDU session seup to UE and the target CU will send the update LTM configurations(DRB associated to the new PD session is added) to UE. This is legacy procedure as well. |
| Huawei, HiSilicon | No | Same view like Intel |
| OPPO | See comments | We understand this scenario is valid, the admission control may be different for different candidate cells. And we also share similar view with ZTE above that this issue can be handled with legacy procedure. Form UE perspective, UE always apply the target configuration upon LTM execution, regardless the PDU session and the corresponding DRB are established or released. |
| IDC | Yes for the 1st hop | We share ZTE and Fujitsu view. The LTM to LTM candidate cell 1 should be supported while LTM to candidate 2 with PDU session 3 recovery looks infeasible as PDU session 3 can’t be “resumed”. To avoid the reconfiguration for PDU session deletion, it might be good idea to apply UE autonomous release. |
| Ericsson | No | Agree with Intel |
| Google | See comments | There are different scenarios where the network may add or release DRBs as shown below. - The UE makes a voice call after receiving a LTM candidate configuration. Then the network configures a DRB for communicating voice packets. In this case, the network should update the LTM candidate configuration to include configurations related to the DRB. Otherwise, the voice call cannot continue after a LTM cell switch with the LTM candidate configuration. - The network configures a DRB for a voice call in a LTM candidate configuration transmitted to the UE. Then the UE ends the voice call and the network releases the DRB in response. After a LTM cell switch with the LTM candidate configuration, the UE establishes the DRB according to the LTM candidate configuration, although there is no voice call ongoing. The network should update the LTM candidate configuration by removing these configurations related to the DRB. Otherwise, the UE considers the DRB is an erroneous configuration after the LTM cell switch. The UE should not have any autonomous actions in the LTM cell switch execution. If DRBs are added or released for a UE after LTM candidate configurations are transmitted to the UE, the network should update the LTM candidate configurations to avoid misconfiguration. We are fine to clarify this in stage-2. |
| Vodafone | No | I think that all candidate cell shall provide a configuration to all PDU sessions and admission control should happen in the target as at the target of preparation, the target can not now when the device will come to this target |
| Qualcomm | See comment | Agree with others that adding PDU session 3 at the second switch is up to the NW. This is not a specific problem for LTM and no new solution is needed. |

Summary (14 companies response)

- 10 companies think the scenario is possible.

- 4 companies think the scenario is not valid.

Proposal 1: RAN2 assumes that different LTM candidate cells with different admission results towards PDU sessions/DRBs is a valid case for both intra-CU and inter-CU LTM.

If the above scenario is confirmed, we need figure out how to resolve this issue. In [R2-2407421 (Samsung)], two options are given to solve this issue:

  - Option 1: After accessing to a LTM candidate cell, UE automatically releases the PDU session, which has been released before. This introduces specification impact (RAN2).

  - Option 2: The inter-CU LTM candidate cell is prepared only if all LTM candidate cells have the same admission result. This seems to be the principle applied in intra-CU LTM.

However, during online discussion, some companies think that this issue should be addressed by the network side, which means that RAN3 would be a suitable WG group to figure out the solutions on this issue. So, it is better to have a discussion on which WG should take the responsibility on this issue.

Question 2:  if the above scenario is confirmed, which WG (RAN2 or RAN3) should take the main responsibility to figure out the solution of this issue?

| Company | RAN2 or RAN3 | Comments |
| --- | --- | --- |
| ZTE | See comments | We prefer to leave it to NW implementation, so none of Option 1 or Option 2 is needed. If companies have different understandings, we can consider to send LS to RAN3, ask them to discuss this issue, without providing the potential solutions. |
| Intel | RAN2 | We think the solutions need more discussion. As it will involve UE, we think it should first be discussed by RAN2. |
| MediaTek | No view | Our understanding is that the UE releases also SDAP entity for PDU session 3 upon the 1st LTM cell switch, refer to 38.331 clause 5.3.5.18.7: "1> release the radio bearer(s) and the logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete)." The above means that the addition of the DRB in the 2nd LTM cell switch will also trigger the UE to setup a new SDAP entity for PDU session 3 and let the upper layers know about the available user plane resources for the PDU session, refer to 38.331 clause 5.3.5.6.5: "3> if an SDAP entity with the received pdu-Session does not exist: 4> establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1; 4> if an SDAP entity with the received pdu-Session did not exist prior to receiving this reconfiguration: 5> indicate the establishment of the user plane resources for the pdu-Session to upper layers;" In this procedure, the UE has no requirement to check whether the PDU session actually exists in the upper layers, i.e., the UE only indicates upper layers about setup of SDAP. (Consider also the normal PDU session establishment procedure where NAS message PDU SESSION RESOURCE SETUP REQUEST is embedded in the same RRCReconfiguration which add a DRB – there is no requirement for the UE to check whether the RRC signalling and the embedded NAS signalling actually match.) We think this same principle should be kept, which means the Option 1 is not agreeable to us. If option 2 is not agreeable either, we should find a 3rd option (for example, the approach explained by ZTE). |
| Fujitsu | No strong view See comment | In our view, RAN3 needs to be involved regardless of which working group take the responsibility. |
| Rakuten | See comment | Since the major impact is to RAN3, we can send an LS indicating the issue to be addressed. |
| Nokia | See comments | RAN2 can take the responsibility to check the feasibility (even not optimised for subsequent LTM) that enable variation in admitted DRB at different candidates for LTM switching from the radio-aspects. If there are no issues to support this scenario ( NW implementation can handle it via new signalling after switching). RAN3 need to check the impact on the internal contexts and data forwarding related impacts of this mismatch. In that case LS to RAN3 to check the RAN3 impacts for scenario is preferred to have the study in parallel at RAN3. |
| LGE | See comment | MTK’s observation is valid, so Option 1 is not agreeable. Option 2 is also not acceptable because that kind of restriction for network to select target cell may not be desirable. We can consider another option, e.g., the ZTE’s suggestion. |
| CATT | See comment | We think it can be handled via legacy procedure. but OK to send LS to RAN3 to check whether it is a issue to address. |
| Huawei, HiSilicon | RAN2 | We think the solutions need more discussion. As it will involve UE, we think it should first be discussed by RAN2. |
| OPPO | See comments | Agree send LS to RAN3 to check their understanding. |
| IDC | See comment | If companies are agreeable on UE automatic release, then RAN2 should take a lead and inform RAN3. Or if companies prefer leaving up to NW impelemntation, then we will do nothing. |
| Ericsson | See comment | We think that this can be left to network implementation. However, if companies want to solve this, more discussion on this issue is needed in RAN2. |
| Google | See comments | RAN2 should take the lead. No need to send a LS to RAN3. Having said, we think at most we can clarify that the network takes responsibilities to avoid misconfiguration in the LTM cell switch execution. |
| Vodafone | See my comments | I do not see the case |
| vivo | RAN3 | From RAN2 point of view, we agree with ZTE that this issue can be solved by NW implementation. For example, the NW can ensure that all LTM candidate cells have the same admission result as in option 2, or the NW can ensure that LTM cell switch from “LTM candidate cell 1 to LTM candidate cell 2” will not happen before the LTM candidate cells configuration is reconfigured. Anyway, this issue is related to the NW configuration and can be discussed in RAN3. |
| Qualcomm | None | RAN3 can discuss this on their now if needed. We don’t need to send an LS. |

Summary (16 companies response)

- 8 companies indicates that RAN3 can discuss this issue.

- 5 companies indicates that this issue can be discussed in RAN2 first.

Proposal 2: For the solutions, RAN2 is kindly asked to discuss whether this issue can be firstly discussed in RAN2. If not, a post-email can be requested for the LS to RAN3.

If this issue is handled in RAN3, it may be beneficial to send LS to RAN3 to indicate RAN2’s confirmation on the above scenario. Hence, a draft LS is provided as below.

3GPP TSG-RAN WG2 Meeting #127 R2-24xxxxxx Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024 Title: [draft] LS on DRB handling in subsequent LTM Release: Rel-19 Work Item: NR_Mob_Ph4-Core Source: RAN2 To: RAN3 Contact Person: Name: Weiwei Wang E-mail Address: ww1016.wang@samsung.com Attachments: None - Overall Description: When preparing one inter-CU LTM candidate cell, the candidate gNB-CU should perform the admission control for the PDU sessions. If different inter-CU LTM candidate cells have different admission results (i.e., different LTM candidate cells have different accepted PDU sessions), the UE may need perform the PDU session addition/release during the subsequent LTM cell switch. Fig. 1 shows one example. The source cell admits PDU session 1&2&3. However, when preparing the LTM candidate cells, the LTM candidate cell 1 can only accept PDU session 1&2, while the LTM candidate cell 2 can accept PDU session 1&2&3. During the subsequent LTM cell switch in the order of “source cell LTM candidate cell 1 LTM candidate cell 2”, the UE encounters a situation that a released PDU session appear in a new cell without establishment, i.e., : - After UE accesses cell 2 (LTM candidate cell 1), it will release PDU session 3, which may result in the service release at the application layer. - After UE accesses cell 3(LTM candidate cell 2), the RRC reconfiguration of cell 3 contains the DRB(s) for the PDU session 3. However, the UE already releases PDU session 3 in cell 2. Normally, a PDU session served by the DRB should be established first. However, this case results in that the released PDU session (i.e., PDU session 3) in cell 2 appears in cell 3 without PDU session establishment procedure. Fig. 1 Example on the appearance of a released PDU session without PDU session setup - Actions: The above scenario can be summarized as: - due to different admission results of different LTM candidate cells, a PDU session (and the corresponding DRB(s)) released in one LTM candidate cell (e.g., cell1) may appear again in another LTM candidate cell (e.g., cell 2) when performing the LTM cell switch from cell 1 to cell 2. RAN2 confirms that the above scenario is a valid one for both Rel18 and Rel19, and think that it is better to address this issue in RAN3. To RAN4: ACTION: RAN2 kindly requests RAN3 to consider the above scenario and provide feedback if needed. 3. Date of Next TSG-RAN WG4 Meetings: TSG-RAN WG2 Meeting #127-bis China, CN October 14 – October 18, 2024 TSG-RAN WG2 Meeting #128 Orlando, USA November 18 – November 22, 2024

Question 3:  if RAN3 is the suitable WG to address this issue, do we need to send LS to RAN3? If it is needed, please provide the comment to the above draft LS.

| Company | Send LS to RAN3 (Yes or No) | Comments |
| --- | --- | --- |
| Rakuten | Yes | RAN3 impacts are more than RAN2 in this issue. Hence we think an LS could be sent describing the problem. |
| vivo | Yes | As we mentioned above, we think an LS could be considered. |

- 2 Conclusion

Proposal 1: RAN2 assumes that different LTM candidate cells with different admission results towards PDU sessions/DRBs is a valid case for both intra-CU and inter-CU LTM.

Proposal 2: For the solutions, RAN2 is kindly asked to discuss whether this issue can be firstly discussed in RAN2. If not, a post-email can be requested for the LS to RAN3.