3GPP TSG-RAN WG2 #127	R2-2407817

Maastricht, Netherlands, 19th – 23th August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4959 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EnteringLeavingReport condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Ericsson, Qualcomm Inc., ZTE, Apple, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For a measurement reporting configuration for which enteringLeavingReport is configured, the UE sets a fitstEntering flag to a cell in measurement report only if the cell fulfills the entering condition for the first time since measId has been configured. The condition “since measId has been configured” unnecessarily prevents UE from indicating firstEntering for a cell more than once since measId has been configured. To illustrate this issue, let us consider the following sequence of measurement reporting triggered by measurement configuration for which enteringLeavingReport is enabled: - At t1, measId is configured - At t2, MR1 is triggered by cellsTriggeredList = {C1,C2} for the measId - At t3, MR2 is triggered by cellsTriggeredList = {C2} for the measId - At t4, MR3 is triggered by cellsTriggeredList = {C1,C2} for the measId - At t5, MR4 is triggered by cellsTriggeredList = {} for the measId - At t6, MR5 is triggered by cellsTriggeredList = {C1} for the measId For this MR sequence, the resulting firstEntering is as follows: - At t2, C1 and C2 are reported as firstEntering cells in MR1. - At t4, C1 is not reported as firstEntering cell in MR3. - At t6, C1 is not reported as firstEntering cell in MR5. - From now on, C1 and C2 are never reported as firstEntering cell until the measId is reconfigured. The resuling firstEntering would result in restrictive or incorrect network knowledge on ‘firstEntering’ cells. Furthermore, UE is enforced to remember all cells that has ever fulfilled the entry condition since the measId has been configured, which increases UE implementation complexity. The desired behavior on ‘firstEntering’ for the above example is as follows. - At t2, C1 and C2 are reported as firstEntering cells in MR1. - At t4, C1 is not reported as firstEntering cell in MR3. - At t6, C1 is reported as firstEntering cell in MR5. The desired UE behavior is enabled if evaluating a ‘firstEntering’ cell is done based on the scope of a given cellsTriggeredList, and if existing cellsTriggeredList is discarded due to cellsTriggeredList being empty, UE should forget the previous ‘firstEntering’ events for the cellsTriggeredList and then evaluate ‘firstEntering’ from the scratch (ignoring the previous history of firstEntering for the measId). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The condition to set firstEntering flag for a cell is changed - From: the cell is added to cellsTriggeredList for the first time since the measId has been configured. - To: the cell is added to cellsTriggeredList for the first time since the cellsTriggeredList for the measId is generated or regenerated. - The incomplete condition described in parenthesis is removed. Impact analysis Impacted functionality: measurement report triggered by event for which enteringLeavingReport is configured. Inter-operability: If the UE is implemented according to the CR and the network is not, network may have incorrect knowede on firstEntering cells If the network is implemented according to the CR and the UE is not, network may have incorrect knowede on firstEntering cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the changes are not approved, the UE behaviours on when to disable feedback are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.5.5	Measurement reporting

5.5.5.1	General

<unaffected part is omitted>

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and if enteringLeavingReport is configured:

2>	for each cell that is included in measResultNeighCells:

3>	if the measObject associated with this measId concerns NR:

4>	if event entry condition for the event that triggered this measurement report has been fulfilled for the first time since the cellsTriggeredList for the measId is generated or regenerated:

5>	set firstEntering to true for the concerned NR cell;

2>	if the field cellsMetLeavingCond within the VarMeasReportList for this measId is not empty:

3>	set cellsMetReportOnLeaveList to include the cell(s) in cellsMetLeavingCond;

<unaffected part is omitted>

End of changes