3GPP TSG-RAN WG2 Meeting #127	R2-2407872

Maastricht, The Netherlands, 19-23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.355 | CR | 0005 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to SLPP specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To address following Typos and issues in TS 38.355 v18.2.0. From R2-2406375 1 resourde ID should be resource ID 2 abbreviations SL-MO-LR, SL-MT-LR, LCS are missing 3 Deleted empty line, add blank line; 4 Deleted unused “sl-ElevationAoA-LCS-GCS-Translation” from the field description of SL-AoA-ProvideLocationInformation From R2-2406809 1. In clause 2 the TS 38.305 has been defined as reference [3] but is not used yet elsewhere in the specification. 2. Clause 6.5, CommonIEsProvideLocationInformation field descriptions: in the descriptions of radialVelocityComponent and transverseVelocityComponent the term “device” should be replaced by “endpoint” for consistency reasons. 3. Clause 6.7, 6.8, 6.9, 6.10: In the field description of tenMsUnitResponseTime the referenced enumerated value “ten-milli-seconds” is not aligned with ASN.1 and should be replaced by “tenMilliSeconds”. 4. Clause 6.7, SL-AoA-ProvideLocationInformation field descriptions: The fields sl-AzimuthAoA-LCS-GCS-Translation and sl-ElevationAoA-LCS-GCS-Translation do not exist in ASN.1 but sl-LCS-GCS-Translation. Therefore, field name sl-AzimuthAoA-LCS-GCS-Translation should be replaced by sl-LCS-GCS-Translation. 5. Clause 6.8, SL-RTT-ProvideLocationInformation field descriptions: Field description of sl-RTT-Error is missing. 6. Other minor editorial issues should be fixed (typos, missing “IE” to referenced IE names, missing settings in italics etc.). From R2-2407369 The Tx time information in the measurement report is the associated SL-PRS transmission timestamp. But the description of Tx time information is missed in SLPP spec. From R2-2407146 1. In the CommonIEsRequestLocationInformation/ CommonIEsProvideLocationInformation there are different definitions for 'accuracy' without description. At some places, 'accuracy' is defined as INTEGER(0..255), at other places as INTEGER(0..127) without a definition. Similar for the semi-major/semi-minor axes of the uncertainty ellipse. 2. SLPP supports various location coordinates and velocity shapes. However, there are no associated capabilities defined. 3. Several field descriptions are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Address following editorial changes: From R2-2406375 1 Changed resourde ID to resource ID 2 Added abbreviations SL-MO-LR, SL-MT-LR, LCS 3 Deleted empty line, add blank line; 4 Deleted unused “sl-ElevationAoA-LCS-GCS-Translation” from the field description of SL-AoA-ProvideLocationInformation From R2-2406809 1. References to TS 38.305 have been added in clause 5.1.1, 5.2.1, 5.3.1. 2. Clause 6.5, CommonIEsProvideLocationInformation field descriptions: in the descriptions of radialVelocityComponent and transverseVelocityComponent the term “device” has been replaced by “endpoint”. 3. Clause 6.7, 6.8, 6.9, 6.10: In the field description of tenMsUnitResponseTime the referenced enumerated value “ten-milli-seconds” has been replaced by “tenMilliSeconds”. 4. Clause 6.7, SL-AoA-ProvideLocationInformation field descriptions: The field name sl-AzimuthAoA-LCS-GCS-Translation has been replaced by sl-LCS-GCS-Translation. 5. Clause 6.8, SL-RTT-ProvideLocationInformation field descriptions: Field description of sl-RTT-Error has been added. 6. Other minor editorial issues have been fixed. From R2-2407369 Capture the description of Tx time information for tx-TimeInfo in the spec. From R2-2407146 - TS 23.032 defines 3 encodings for 'accuracy'/'uncertainty': - 'Uncertainty' value range 0..127, mapping to 0m – 1800 km - 'High Accuracy Uncertainty' range 0..255, mapping to 0m - ~46m - 'High Accuracy Extended Uncertainty' value range 0..255, mapping to 0m – 200m. The latter seems suitable for sidelink and is proposed for the various 'accuracy' fields. The azimuth/elevation accuracy in the CommonIEsRequestLocationInformation is aligned with the corresponding elements in the CommonIEsProvideLocationInformation. - The supported GAD/velocity shapes are a capability in LPP (e.g., A-GNSS-Provide-Capabilities, NR-DL-TDOA-ProvideCapabilities). The CommonIEsRequestLocationInformation in LPP indicates the LocationCoordinateTypes/VelocityTypes that the target device may return when a location estimate/velocity is obtained by the target. The same solution is proposed for SLPP. Missing field descriptions have been added. Impact analysis: Impacted 5G architecture options: NR SA Impacted functionality: - Support of SLPP positioning Inter-operability issues: R2-2406375, R2-2406809, R2-2407369 There are no interoperability issues. The changes 1 and 2 from R2-2407146 are mandatory for the concerned functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Issues exist in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.1.2, 4.3.3.1, 4.3.3.2, 5.1.1, 5.1.4, 5.2.1, 5.2.4, 5.3.1, 5.3.4, 5.3.6, 6.2.2, 6.3.1, 6.5, 6.6, 6.7, 6.8, 6.9, 6.10, 6.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Coversheet update by MCC: Replaced “non-backwards compatible” with “mandatory for the concerned functionality" in Impact analysis. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the Sidelink Positioning Protocol (SLPP) for the interface between UEs and between UE and LMF.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[4]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[5]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[6]	3GPP TS 38.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation".

[7]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[8]	3GPP TR 38.901: "Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz".

[9]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[10]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[11]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[12]	3GPP TS 23.586: "Technical Specification Group Services and System Aspects; Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 23.304: "Technical Specification Group Services and System Aspects; Proximity based Services (ProSe) in the 5G System (5GS)".

[15]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[16]	3GPP TS 38.215: "NR; Physical layer measurements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Field: The individual contents of an information element are referred to as fields.

Ranging: Refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE via PC5 interface.

Ranging/Sidelink Positioning: AS functionality enabling ranging-based services and sidelink positioning as specified in TS 23.586 [12].

SL Anchor UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. over the Sidelink interface.

SL Server UE: A UE offering position method determination, assistance data distribution and/or location calculation functionalities for sidelink positioning and ranging based services. It interacts with other UEs over PC5 as necessary in order to determine a ranging/SL position method, distribute assistance data and calculate the location of the target UE. A Target UE or SL Anchor UE can act as SL Server UE if any of the functionalities is supported.

SL Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple SL Anchor UEs using Sidelink in the Ranging based service and Sidelink positioning.

UE-only Operation: Operation of Ranging/Sidelink Positioning in which the service request handling and result calculation are performed by UE.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DFN	Direct Frame Number

GCS	Global Coordinate System (as defined in TR 38.901 [17])

LCS	LoCation Services

Local Coordinate System (as defined in TR 38.901 [17])

LMF	Location Management Function

LOS	Line-of-Sight

NLOS	Non-Line-of-Sight

RTD	Relative Time Difference

SL	Sidelink

SL-AoA	Sidelink Angle-of-Arrival

SL-MO-LR	Sidelink Mobile Originating Location Request

SL-MT-LR	Sidelink Mobile Terminating Location Request

SLPP	Sidelink Positioning Protocol

SL-PRS	Sidelink Positioning Reference Signals

SL-PRS-RSRP	Sidelink Positioning Reference Signals based Reference Signal Received Power

SL-PRS-RSRPP	Sidelink Positioning Reference Signals based Reference Signal Received Path Power

SL-PRS-RSTD	Sidelink Positioning Reference Signals based Reference Signal Time Difference

SL-PRS-RTOA	Sidelink Positioning Reference Signals based Relative Time of Arrival

SL-RTT	Sidelink Round Trip Time

SL-TDOA	Sidelink Time Difference Of Arrival

SL-TOA	Sidelink Time Of Arrival

UE	User Equipment

# 4	Functionality of Protocol

## 4.1	General

### 4.1.1	SLPP Configuration

SLPP is used point-to-point between Endpoints, e.g. Location Server (SL Server UE or LMF) and target in order to obtain absolute position, relative position, or ranging information of target UE using sidelink measurements obtained by one or more reference sources.

Figure 4.1.1-1: Void

### 4.1.2	SLPP Sessions and Transactions

An SLPP session is used between UEs or a Location Server and a UE in order to obtain location related measurements based on NR PC5 radio signals, a location estimate or to transfer assistance data. A single SLPP session is used to support a single location request (e.g., for a single SL-MT-LR, or SL-MO-LR). Multiple SLPP sessions can be used between the same endpoints to support multiple location requests (as required by TS 23.273 [5]). For UE-only Operation, the instigator of an SLPP session which is the Endpoint who receives the LCS request, initiates an SLPP session by sending an SLPP message containing an assigned session ID (session identifier) to the other endpoint(s). All constituent messages within a session shall contain the same session ID. For LMF involved Operation, the session ID is assigned by target UE and contained in the SLPP messages used for communication between UEs. The session ID may be included in the SLPP message for the communication between a UE and the LMF.

Each SLPP session comprises one or more SLPP transactions, with each SLPP transaction performing a single operation (capability exchange, assistance data transfer, or location information transfer). The SLPP transactions are realized as SLPP procedures. The instigator of an SLPP session will always instigate the first SLPP transaction, but subsequent transactions may be instigated by either end. SLPP transactions within a session may occur serially or in parallel. SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response).

Messages within a transaction are linked by a common transaction identifier.

### 4.1.3	SLPP Positioning Methods

This version of the specification defines SL-TDOA, SL-TOA, SL-AoA and SL-RTT positioning methods based on NR PC5 radio signals.

### 4.1.4	SLPP Messages

Each SLPP transaction involves the exchange of one or more SLPP messages between Endpoint A and Endpoint B. The general format of an SLPP message consists of a set of common fields followed by a body. The body (which may be empty) contains information specific to a particular message type. Each message type contains information specific to one or more positioning methods and/or information common to all positioning methods.

The common fields are as follows:

| Field | Role |
| --- | --- |
| sessionID | Identify messages belonging to the same session |
| transactionID | Identify messages belonging to the same transaction |
| endTransaction | Indicate when a transaction (e.g. one with periodic responses) has ended |
| sequenceNumber | Enable detection of a duplicate SLPP message at a receiver |
| acknowledgement | Enable an acknowledgement to be requested and/or returned for any SLPP message |

The following message types are defined:

-	Request Capabilities;

-	Provide Capabilities;

-	Request Assistance Data;

-	Provide Assistance Data;

-	Request Location Information;

-	Provide Location Information;

-	Abort;

-	Error.

## 4.2	Common SLPP Session Procedure

The purpose of this procedure is to support an SLPP session comprising a sequence of SLPP transactions. The procedure is described in Figure 4.2-1.

Figure 4.2-1 SLPP Session Procedure

1.	Endpoint A, which is the Endpoint who receives the LCS request, initiates an SLPP session by sending an SLPP message containing an assigned session ID for an initial SLPP transaction j to the other endpoint B.

2.	Endpoints A and B may exchange further messages to continue the transaction started in step 1.

3.	Either endpoint may instigate further transactions by sending additional SLPP messages.

4.	A session is terminated by a final transaction N in which SLPP messages will be exchanged between the two endpoints.

Within the same session, all constituent messages shall contain the same session ID and within each transaction, all constituent messages shall contain the same transaction ID. The last message sent in each transaction shall have the field endTransaction set to TRUE. Transactions that occur in parallel shall use different transaction IDs; transaction IDs for completed transactions may be reused at any time after the final message of the previous transaction with the same ID is known to have been received.

## 4.3	SLPP Transport

### 4.3.1	Transport Layer Requirements

SLPP requires reliable, in-sequence delivery of SLPP messages from the underlying transport layers. This clause describes the transport capabilities that are available within SLPP. A UE implementing SLPP shall support SLPP reliable transport (including all three of duplicate detection, acknowledgement, and retransmission).

### 4.3.2	SLPP Duplicate Detection

A sender shall include a sequence number in all SLPP messages sent for a particular location session. The sequence number shall be distinct for different SLPP messages sent by the same endpoint for the same endpoint in the same location session (e.g., may start at zero in the first SLPP message and increase monotonically in each succeeding SLPP message). Sequence numbers used in the messages transmitted from different endpoints or for different endpoints are independent (e.g., can be the same).

A receiver shall record the most recent received sequence number for each pair of endpoints of each location session. If a message is received carrying the same sequence number as that last received for the same pair of endpoints and the associated location session, it shall be discarded. Otherwise (i.e., if the sequence number is different or the sequence number is same but for different pair of endpoints), the message shall be processed.

Sending and receiving sequence numbers shall be deleted in a server when the associated location session is terminated and shall be deleted in the UE(s) when there has been no activity for a particular location session for 10 minutes.

### 4.3.3	SLPP Acknowledgement

#### 4.3.3.1	General

Each SLPP message may carry an acknowledgement request and/or an acknowledgement indicator. An SLPP message including an acknowledgement request (i.e., that include the field ackRequested set to TRUE) shall also include a sequence number. Upon reception of an SLPP message which includes the field ackRequested set to TRUE, a receiver returns an SLPP message with an acknowledgement response (i.e., that includes the field ackIndicator set to the same sequence number of the message being acknowledged). An acknowledgement response may contain no SLPP message body (in which case only the sequence number being acknowledged is significant); alternatively, the acknowledgement may be sent in an SLPP message along with an SLPP message body. An acknowledgement is returned for each received SLPP message that requested an acknowledgement including any duplicate(s). Once a sender receives an acknowledgement for an SLPP message, and provided any included sequence number is matching, it is permitted to send the next SLPP message. No message reordering is needed at the receiver since this stop-and-wait method of sending ensures that messages normally arrive in the correct order.

When an SLPP message is transported via a NAS SL-MO-LR request, the message does not request an acknowledgement.

#### 4.3.3.2	Procedure related to Acknowledgement

Figure 4.3.3.2-1 shows the procedure related to acknowledgement.

Figure 4.3.3.2-1: SLPP Acknowledgement procedure

1.	Endpoint A sends an SLPP message N to Endpoint B which includes the field ackRequested set to TRUE and a sequence number.

2.	If SLPP message N is received and Endpoint B is able to decode the ackRequested value and sequence number, Endpoint B shall return an acknowledgement for message N. The acknowledgement shall contain the field ackIndicator set to the same sequence number as that in message N.

3.	When the acknowledgement for SLPP message N is received and provided the included field ackIndicator matches the sequence number sent in message N, Endpoint A sends the next SLPP message N+1 to Endpoint B when this message is available.

### 4.3.4	SLPP Retransmission

#### 4.3.4.1	General

This capability builds on the acknowledgement and duplicate detection capabilities. When an SLPP message which requires acknowledgement is sent and not acknowledged, it is resent by the sender following a timeout period up to three times. If still unacknowledged after that, the sender aborts all SLPP activity for this Endpoint. The timeout period is determined by the sender implementation but shall not be less than a minimum value of 250 ms.

#### 4.3.4.2	Procedure related to Retransmission

Figure 4.3.4.2-1 shows the procedure related to retransmission when combined with acknowledgement and duplicate detection.

Figure 4.3.4.2-1: SLPP Retransmission procedure

1.	Endpoint A sends an SLPP message N to Endpoint B for a particular location session and includes a request for acknowledgement along with a sequence number.

2.	If SLPP message N is received and Endpoint B is able to decode the ackRequested value and sequence number (regardless of whether the message body can be correctly decoded), Endpoint B shall return an acknowledgement for message N. If the acknowledgement is received by Endpoint A (such that the acknowledged message can be identified and sequence numbers are matching), Endpoint A skips steps 3 and 4.

3.	If the acknowledgement in step 2 is not received after a timeout period, Endpoint A shall retransmit SLPP message N and shall include the same sequence number as in step 1.

4.	If SLPP message N in step 3 is received and Endpoint B is able to decode the ackRequested value and sequence number (regardless of whether the message body can be correctly decoded and whether or not the message is considered a duplicate), Endpoint B shall return an acknowledgement. Steps 3 may be repeated one or more times if the acknowledgement in step 4 is not received after a timeout period by Endpoint A. If the acknowledgement in step 4 is still not received after sending three retransmissions, Endpoint A shall abort all procedures and activity associated with SLPP support for this Endpoint B.

5.	Once an acknowledgement in step 2 or step 4 is received, Endpoint A sends the next SLPP message N+1 for the location session to Endpoint B when this message is available.

# 5	SLPP Procedures

## 5.1	Procedures related to capability transfer

### 5.1.1	General

The purpose of the procedures that are grouped together in this clause is to enable the transfer of capabilities from Endpoint A to Endpoint B. Capabilities in this context refer to positioning and protocol capabilities related to SLPP and the positioning methods supported by SLPP. These procedures instantiate the Capability Transfer procedure from TS 38.305 [3].

### 5.1.2	Capability Transfer procedure

The Capability Transfer procedure is shown in Figure 5.1.2-1.

Figure 5.1.2-1: SLPP Capability Transfer procedure

1.	Endpoint B sends a RequestCapabilities message to Endpoint A. Endpoint B may indicate the types of capability requested.

2.	Endpoint A responds with a ProvideCapabilities message to Endpoint B. The capabilities shall correspond to the capability types specified in step 1. This message shall include the field endTransaction set to TRUE.

### 5.1.3	Capability Indication procedure

The Capability Indication procedure allows the Endpoint A to provide unsolicited capabilities to the Endpoint B and is shown in Figure 5.1.3-1.

Figure 5.1.3-1: SLPP Capability Indication procedure

1.	Endpoint A sends a ProvideCapabilities message to Endpoint B. This message shall include the field endTransaction set to TRUE.

### 5.1.4	Transmission of SLPP Request Capabilities

When triggered to transmit a RequestCapabilities message, Endpoint B shall:

1>	set the positioning method specific RequestCapabilities PDUs in accordance with the information received from upper layers.

1>	deliver the message to lower layers for transmission.

### 5.1.5	Reception of SLPP Request Capabilities

Upon receiving a RequestCapabilities message, Endpoint A shall generate a ProvideCapabilities message as a response.

Endpoint A shall:

1>	for each positioning method for which a request for capabilities is included in the message:

2>	if Endpoint A supports this positioning method:

3>	include the capabilities of Endpoint A for that supported positioning method in the response message;

1>	set the field sessionID in the response message to the same value as the field sessionID in the received message if received;

1>	set the field transactionID in the response message to the same value as the field transactionID in the received message;

1>	deliver the response message to lower layers for transmission.

### 5.1.6	Transmission of SLPP Provide Capabilities

When triggered to transmit a ProvideCapabilities message, Endpoint A shall:

1>	for each positioning method whose capabilities are to be indicated:

2>	set the corresponding fields to include Endpoint A's capabilities;

1>	deliver the response to lower layers for transmission.

## 5.2	Procedures related to Assistance Data Transfer

### 5.2.1	General

The purpose of the procedures that are grouped together in this clause is to enable Endpoint A to request assistance data from Endpoint B to assist in positioning, and to enable Endpoint B to transfer assistance data to Endpoint A without a request. These procedures instantiate the Assistance Data Transfer procedure from TS 38.305 [3].

### 5.2.2	Assistance Data Transfer procedure

The Assistance Data Transfer procedure is shown in Figure 5.2.2-1.

Figure 5.2.2-1: SLPP Assistance data transfer procedure

1.	Endpoint A sends a RequestAssistanceData message to Endpoint B.

2.	Endpoint B responds with a ProvideAssistanceData message to Endpoint A containing assistance data. The transferred assistance data should match or be a subset of the assistance data requested in step 1. Endpoint B may also provide any not requested information that it considers useful to Endpoint A. If step 3 is not expected, this message shall set the field endTransaction to TRUE.

3.	Endpoint B may transmit one or more additional ProvideAssistanceData messages to Endpoint A containing further assistance data. The transferred assistance data should match or be a subset of the assistance data requested in step 1. Endpoint B may also provide any not requested information that it considers useful to Endpoint A. The last message shall include the field endTransaction set to TRUE.

### 5.2.3	Assistance Data Delivery procedure

The Assistance Data Delivery procedure allows Endpoint B to provide unsolicited assistance data to Endpoint A and is shown in Figure 5.2.3-1.

Figure 5.2.3-1: SLPP Assistance data transfer procedure

1.	Endpoint B sends a ProvideAssistanceData message to Endpoint A containing assistance data. If step 2 is not expected, this message shall set the field endTransaction to TRUE.

2.	Endpoint B may transmit one or more additional ProvideAssistanceData messages to Endpoint A containing additional assistance data. The last message shall include the field endTransaction set to TRUE.

### 5.2.4	Transmission of SLPP Request Assistance Data

When triggered to transmit a RequestAssistanceData message, Endpoint A shall:

1>	set the positioning method specific RequestAssistanceData PDUs in accordance with the information received from upper layers.

1>	deliver the message to lower layers for transmission.

### 5.2.5	Reception of SLPP Request Assistance Data

Upon receiving a RequestAssistanceData message, Endpoint B shall generate a ProvideAssistanceData message as a response.

Endpoint B shall:

1>	for each positioning method for which a request for assistance data is included in the message:

2>	if Endpoint B supports this positioning method:

3>	include the assistance data for that supported positioning method in the response message;

1>	set the field sessionID in the response message to the same value as the field sessionID in the received message if received;

1>	set the field transactionID in the response message to the same value as the field transactionID in the received message;

1>	deliver the response message to lower layers for transmission.

### 5.2.6	Reception of SLPP Provide Assistance Data

Upon receiving a ProvideAssistanceData message, Endpoint A shall:

1>	for each positioning method contained in the message:

2>	deliver the related assistance data to upper layers.

## 5.3	Procedures related to Location Information Transfer

### 5.3.1	General

The purpose of the procedures that are grouped together in this clause is to enable Endpoint B to request location measurement data and/or a location estimate from Endpoint A, and to enable Endpoint A to transfer location measurement data and/or a location estimate to Endpoint B without a request. These procedures instantiate the Location Information Transfer procedure from TS 38.305 [3].

### 5.3.2	Location Information Transfer procedure

The Location Information Transfer procedure is shown in Figure 5.3.2-1.

Figure 5.3.2-1: SLPP Location Information transfer procedure

1.	Endpoint B sends a RequestLocationInformation message to Endpoint A to request location information, indicating the type of location information requested and optionally the associated QoS.

2.	Endpoint A sends a ProvideLocationInformation message to Endpoint B to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless Endpoint B explicitly allows additional location information. If step 3 is not expected, this message shall set the field endTransaction to TRUE.

3.	If requested in step 1, Endpoint A sends additional ProvideLocationInformation messages to Endpoint B to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless Endpoint B explicitly allows additional location information. The last message shall include the field endTransaction set to TRUE.

### 5.3.3	Location Information Delivery procedure

The Location Information Delivery procedure allows Endpoint A to provide unsolicited location information to Endpoint B. The procedure is shown in Figure 5.3.3-1.

Figure 5.3.3-1: SLPP Location Information Delivery procedure

1.	Endpoint A sends a ProvideLocationInformation message to Endpoint B to transfer location information. If step 2 is not expected, this message shall set the field endTransaction to TRUE.

2.	Endpoint A may send one or more additional ProvideLocationInformation messages to Endpoint B containing additional location information data. The last message shall include the field endTransaction set to TRUE.

### 5.3.4	Transmission of Request Location Information

When triggered to transmit a RequestLocationInformation message, Endpoint B shall:

1>	set the positioning method specific RequestLocationInformation PDUs in accordance with the information received from upper layers.

1>	deliver the message to lower layers for transmission.

### 5.3.5	Reception of Request Location Information

Upon receiving a RequestLocationInformation message, Endpoint A shall:

1>	if the requested information is compatible with Endpoint A's capabilities and configuration:

2>	include the requested information in a ProvideLocationInformation message;

2>	set the field sessionID in the response message to the same value as the field sessionID in the received message if received;

2>	set the field transactionID in the response to the same value as the field transactionID in the received message;

2>	deliver the ProvideLocationInformation message to lower layers for transmission.

1>	else if one or more positioning methods are included that Endpoint A does not support:

2>	continue to process the message as if it contained only information for the supported positioning methods;

2>	handle the signaling content of the unsupported positioning methods by SLPP error detection as in 5.4.3.

### 5.3.6	Transmission of Provide Location Information

When triggered to transmit a ProvideLocationInformation message, Endpoint A shall:

1>	for each positioning method contained in the message:

2>	set the corresponding fields to include the available location information;

1>	deliver the response to lower layers for transmission.

## 5.4	Error Handling Procedures

### 5.4.1	General

This clause describes how a receiving endpoint behaves in cases when it receives erroneous or unexpected data or detects that certain data are missing.

### 5.4.2	Procedures related to Error Indication

Figure 5.4.2-1 shows the Error indication procedure.

Figure 5.4.2-1: SLPP Error Indication procedure

1.	Endpoint A sends an SLPP message to Endpoint B.

2.	Endpoint B determines that the SLPP message in step 1 contains an error. Endpoint B returns an Error message to Endpoint A indicating the error or errors and discards the message in step 1. If Endpoint B is able to determine that the erroneous SLPP message in step 1 is an SLPP Error or Abort Message, Endpoint B discards the message in step 1 without returning an Error message to Endpoint A.

### 5.4.3	SLPP Error Detection

Upon receiving any SLPP message, the receiving endpoint shall attempt to decode the message and verify the presence of any errors and:

1>	if decoding errors are encountered:

2>	if the receiver cannot determine that the received message is an SLPP Error or Abort message:

3>	return an SLPP Error message to the sender and include the field sessionID (if PC5-U is used as transport layer) and the received transactionID, if they were decoded, and type of error;

3>	discard the received message and stop the error detection procedure;

1>	if the message is a duplicate of a previously received message:

2>	discard the message and stop the error detection procedure;

1>	if the field transactionID matches the field transactionID for a procedure that is still ongoing for the same session and the message type is invalid for the current state of the procedure:

2>	abort the ongoing procedure;

2>	return an SLPP Error message to the sender and include the field sessionID (if PC5-U is used as transport layer), the received field transactionID and type of error;

2>	discard the message and stop the error detection procedure;

1> if the message type is an SLPP RequestCapabilities and some of the requested information is not supported:

2>	return any information that can be provided in a normal response.

1>	if the message type is an SLPP RequestAssistanceData or RequestLocationInformation and some or all of the requested information is not supported:

2>	return any information that can be provided in a normal response, which includes indications on other information that is not supported.

### 5.4.4	Reception of an SLPP Error Message

Upon receiving an Error message, Endpoint A shall:

1>	abort any ongoing procedure associated with the field sessionID and the field transactionID if included in the received message.

Endpoint A may:

1>	restart the aborted procedure taking into consideration the returned error information.

## 5.5	Abort Procedure

### 5.5.1	General

The purpose of the abort procedure is to allow Endpoints to abort an ongoing procedure due to some unexpected event (e.g., cancellation of a location request by an LCS client). It can also be used to stop an ongoing procedure (e.g., periodic location reporting from an Endpoint).

### 5.5.2	Procedures related to Abort

Figure 5.5.2-1 shows the Abort procedure.

Figure 5.5.2-1: SLPP Abort procedure

1.	A procedure P is ongoing between endpoints A and B.

2.	Endpoint A determines that the procedure must be aborted and sends an Abort message to Endpoint B carrying the field sessionID (if PC5-U is used as transport layer) and the field transactionID for procedure P. Endpoint B aborts procedure P.

### 5.5.3	Reception of an SLPP Abort Message

Upon receiving an Abort message, Endpoint shall:

1>	abort any ongoing procedure associated with the field sessionID and the field transactionID indicated in the message.

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.1	General

The contents of each SLPP message is specified in clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax. The syntax of the information elements that are defined as stand-alone abstract types is further specified in a similar manner in clause 6.3.

The ASN.1 in this clause uses the same format and coding conventions as described in Annex A of TS 38.331 [2].

Transfer syntax for SLPP messages is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [4]. The encoded SLPP message always contains a multiple of 8 bits.

Transfer syntax for SLPP IEs is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [4]. The encoded SLPP IE always contains a multiple of 8 bits. This applies when a single SLPP IE is encoded as the basic production, i.e. for other purposes than encoding the SLPP IE within an SLPP message.

When specifying information elements which are to be represented by BIT STRINGs, if not otherwise specifically stated in the field description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	the last bit (rightmost bit) contains the least significant bit (LSB).

## 6.2	SLPP messages

### 6.2.1	General message structure

#### –	SLPP-PDU-Definitions

This ASN.1 segment is the start of the SLPP PDU definitions.

-- ASN1START

-- TAG-SLPP-PDU-DEFINITIONS-START

SLPP-PDU-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CommonIEsRequestCapabilities,

CommonIEsProvideCapabilities,

CommonIEsRequestAssistanceData,

CommonIEsProvideAssistanceData,

CommonIEsRequestLocationInformation,

CommonIEsProvideLocationInformation

FROM

SLPP-PDU-CommonContents

CommonSL-PRS-MethodsIEsRequestCapabilities,

CommonSL-PRS-MethodsIEsProvideCapabilities,

CommonSL-PRS-MethodsIEsRequestAssistanceData,

CommonSL-PRS-MethodsIEsProvideAssistanceData,

CommonSL-PRS-MethodsIEsRequestLocationInformation,

CommonSL-PRS-MethodsIEsProvideLocationInformation

FROM

SLPP-PDU-CommonSL-PRS-MethodsContents

SL-AoA-RequestCapabilities,

SL-AoA-ProvideCapabilities,

SL-AoA-RequestAssistanceData,

SL-AoA-ProvideAssistanceData,

SL-AoA-RequestLocationInformation,

SL-AoA-ProvideLocationInformation

FROM

SLPP-PDU-SL-AoA-Contents

SL-RTT-RequestCapabilities,

SL-RTT-ProvideCapabilities,

SL-RTT-RequestAssistanceData,

SL-RTT-ProvideAssistanceData,

SL-RTT-RequestLocationInformation,

SL-RTT-ProvideLocationInformation

FROM

SLPP-PDU-SL-RTT-Contents

SL-TDOA-RequestCapabilities,

SL-TDOA-ProvideCapabilities,

SL-TDOA-RequestAssistanceData,

SL-TDOA-ProvideAssistanceData,

SL-TDOA-RequestLocationInformation,

SL-TDOA-ProvideLocationInformation

FROM

SLPP-PDU-SL-TDOA-Contents

SL-TOA-RequestCapabilities,

SL-TOA-ProvideCapabilities,

SL-TOA-RequestAssistanceData,

SL-TOA-ProvideAssistanceData,

SL-TOA-RequestLocationInformation,

SL-TOA-ProvideLocationInformation

FROM

SLPP-PDU-SL-TOA-Contents;

-- TAG-SLPP-PDU-DEFINITIONS-STOP

-- ASN1STOP

NOTE 1:	An implementation needs to include only the supported "Method" PDUs. Not supported methods do not need to be included. For example, if SL-RTT is not supported by an implementation, the SLPP-PDU-SL-RTT-Contents PDU does not need to be included in the protocol.

NOTE 2:	An implementation supporting SL-RTT, SL-AoA, SL-TDOA, or SL-TOA must also support the SLPP-PDU-CommonSL-PRS-MethodsContents PDU.

#### –	SLPP-Message

The SLPP-Message provides the complete set of information for an invocation or response pertaining to an SLPP transaction.

-- ASN1START

-- TAG-SLPP-MESSAGE-START

SLPP-Message ::=            SEQUENCE {

applicationLayerID          OCTET STRING        OPTIONAL,

transactionID               INTEGER (0..255)    OPTIONAL,

endTransaction              BOOLEAN,

sequenceNumber              SequenceNumber      OPTIONAL,

sessionID                   SessionID           OPTIONAL,

acknowledgement             Acknowledgement     OPTIONAL,

slpp-MessageBody            SLPP-MessageBody    OPTIONAL,

nonCriticalExtension        SEQUENCE {}         OPTIONAL

}

SequenceNumber ::= INTEGER (0..255)

SessionID ::= OCTET STRING (SIZE (6))

Acknowledgement ::= SEQUENCE {

ackRequested        BOOLEAN,

ackIndicator        SequenceNumber        OPTIONAL

}

-- TAG-SLPP-MESSAGE-STOP

-- ASN1STOP

SLPP-Message field descriptions

acknowledgement This field is included in an SLPP acknowledgement and in any SLPP message requesting an acknowledgement and is omitted otherwise. - ackRequested: This field indicates whether an SLPP acknowledgement is requested (TRUE) or not (FALSE). A value of TRUE may only be included when an slpp-MessageBody is included. - ackIndicator: This field indicates the sequence number of the message being acknowledged.

applicationLayerID This field indicates the application layer ID of the UE which is sending the message.

endTransaction This field indicates whether an SLPP message is the last message carrying an slpp-MessageBody in a transaction (TRUE) or not last (FALSE).

sequenceNumber This field may be included when an slpp-MessageBody is included but shall be omitted otherwise.

sessionID This field indicates the session ID which is used to identify messages belonging to the same session.

slpp-MessageBody This field may be omitted in the case the message is sent only to acknowledge a previously received message.

transactionID This field is omitted if an slpp-MessageBody is not present (i.e. in an SLPP message sent only to acknowledge a previously received message) or if it is not available to the transmitting endpoint (e.g., in an SLPP-Error message triggered by a message that could not be parsed). If present, this field shall be ignored at a receiver in an SLPP message for which the slpp-MessageBody is not present.

#### –	SLPP-MessageBody

The SLPP-MessageBody identifies the type of an SLPP message and contains all SLPP information specifically associated with that type.

-- ASN1START

-- TAG-SLPP-MESSAGEBODY-START

SLPP-MessageBody ::= CHOICE {

c1                          CHOICE {

requestCapabilities         RequestCapabilities,

provideCapabilities         ProvideCapabilities,

requestAssistanceData       RequestAssistanceData,

provideAssistanceData       ProvideAssistanceData,

requestLocationInformation  RequestLocationInformation,

provideLocationInformation  ProvideLocationInformation,

abort                       Abort,

error                       Error,

spare8 NULL, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension    SEQUENCE {}

}

-- TAG-SLPP-MESSAGEBODY-STOP

-- ASN1STOP

### 6.2.2	Message body information elements definitions

#### –	RequestCapabilities

The RequestCapabilities message body in an SLPP message is used by Endpoint B to request Endpoint A capability information for SLPP and the supported individual positioning methods.

-- ASN1START

-- TAG-REQUESTCAPABILITIES-START

RequestCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

requestCapabilities         RequestCapabilities-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestCapabilities-IEs ::= SEQUENCE {

commonIEsRequestCapabilities                  OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestCapabilities

commonSL-PRS-MethodsIEsRequestCapabilities    OCTET STRING    OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsRequestCapabilities

sl-AoA-RequestCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-AoA-RequestCapabilities

sl-RTT-RequestCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-RTT-RequestCapabilities

sl-TDOA-RequestCapabilities                   OCTET STRING    OPTIONAL, -- Containing SL-TDOA-RequestCapabilities

sl-TOA-RequestCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-TOA-RequestCapabilities

lateNonCriticalExtension                      OCTET STRING    OPTIONAL,

nonCriticalExtension                          SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	ProvideCapabilities

The ProvideCapabilities message body in an SLPP message indicates the SLPP capabilities of Endpoint A to Endpoint B.

-- ASN1START

-- TAG-PROVIDECAPABILITIES-START

ProvideCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

provideCapabilities         ProvideCapabilities-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

ProvideCapabilities-IEs ::= SEQUENCE {

commonIEsProvideCapabilities                  OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideCapabilities

commonSL-PRS-MethodsIEsProvideCapabilities    OCTET STRING    OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsProvideCapabilities

sl-AoA-ProvideCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-AoA-ProvideCapabilities

sl-RTT-ProvideCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-RTT-ProvideCapabilities

sl-TDOA-ProvideCapabilities                   OCTET STRING    OPTIONAL, -- Containing SL-TDOA-ProvideCapabilities

sl-TOA-ProvideCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-TOA-ProvideCapabilities

lateNonCriticalExtension                      OCTET STRING    OPTIONAL,

nonCriticalExtension                          SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDECAPABILITIES-STOP

-- ASN1STOP

#### –	RequestAssistanceData

The RequestAssistanceData message body in an SLPP message is used by the Endpoint A to request assistance data from Endpoint B.

-- ASN1START

-- TAG-REQUESTASSISTANCEDATA-START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions        CHOICE {

requestAssistanceData     RequestAssistanceData-IEs,

criticalExtensionsFuture  SEQUENCE {}

}

}

RequestAssistanceData-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData                  OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestAssistanceData

commonSL-PRS-MethodsIEsRequestAssistanceData    OCTET STRING    OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsRequestAssistanceData

sl-AoA-RequestAssistanceData                    OCTET STRING    OPTIONAL, -- Containing SL-AoA-RequestAssistanceData

sl-RTT-RequestAssistanceData                    OCTET STRING    OPTIONAL, -- Containing SL-RTT-RequestAssistanceData

sl-TDOA-RequestAssistanceData                   OCTET STRING    OPTIONAL, -- Containing SL-TDOA-RequestAssistanceData

sl-TOA-RequestAssistanceData                    OCTET STRING    OPTIONAL, -- Containing SL-TOA-RequestAssistanceData

lateNonCriticalExtension                        OCTET STRING    OPTIONAL,

nonCriticalExtension                            SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

#### –	ProvideAssistanceData

The ProvideAssistanceData message body in an SLPP message is used by Endpoint B to provide assistance data to Endpoint A either in response to a request from Endpoint A or in an unsolicited manner. Upon receiving an SLPP message ProvideAsssistanceData, the UE releases the value received in previous SLPP message ProvideAsssistanceData if any.

-- ASN1START

-- TAG-PROVIDEASSISTANCEDATA-START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions        CHOICE {

provideAssistanceData     ProvideAssistanceData-IEs,

criticalExtensionsFuture  SEQUENCE {}

}

}

ProvideAssistanceData-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData                  OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideAssistanceData

commonSL-PRS-MethodsIEsProvideAssistanceData    OCTET STRING    OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsProvideAssistanceData

sl-AoA-ProvideAssistanceData                    OCTET STRING    OPTIONAL, -- Containing SL-AoA-ProvideAssistanceData

sl-RTT-ProvideAssistanceData                    OCTET STRING    OPTIONAL, -- Containing SL-RTT-ProvideAssistanceData

sl-TDOA-ProvideAssistanceData                   OCTET STRING    OPTIONAL, -- Containing SL-TDOA-ProvideAssistanceData

sl-TOA-ProvideAssistanceData                    OCTET STRING    OPTIONAL, -- Containing SL-TOA-ProvideAssistanceData

lateNonCriticalExtension                        OCTET STRING    OPTIONAL,

nonCriticalExtension                            SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

#### –	RequestLocationInformation

The RequestLocationInformation message body in an SLPP message is used by Endpoint B to request positioning measurements or a position estimate from Endpoint A.

-- ASN1START

-- TAG-REQUESTLOCATIONINFORMATION-START

RequestLocationInformation ::= SEQUENCE {

criticalExtensions             CHOICE {

requestLocationInformation     RequestLocationInformation-IEs,

criticalExtensionsFuture       SEQUENCE {}

}

}

RequestLocationInformation-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation                  OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestLocationInformation

commonSL-PRS-MethodsIEsRequestLocationInformation    OCTET STRING    OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsRequestLocationInformation

sl-AoA-RequestLocationInformation                    OCTET STRING    OPTIONAL, -- Containing SL-AoA-RequestLocationInformation

sl-RTT-RequestLocationInformation                    OCTET STRING    OPTIONAL, -- Containing SL-RTT-RequestLocationInformation

sl-TDOA-RequestLocationInformation                   OCTET STRING    OPTIONAL, -- Containing SL-TDOA-RequestLocationInformation

sl-TOA-RequestLocationInformation                    OCTET STRING    OPTIONAL, -- Containing SL-TOA-RequestLocationInformation

lateNonCriticalExtension                             OCTET STRING    OPTIONAL,

nonCriticalExtension                                 SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	ProvideLocationInformation

The ProvideLocationInformation message body in an SLPP message is used by Endpoint A to provide positioning measurements or position estimates to Endpoint B.

-- ASN1START

-- TAG-PROVIDELOCATIONINFORMATION-START

ProvideLocationInformation ::= SEQUENCE {

criticalExtensions             CHOICE {

provideLocationInformation    ProvideLocationInformation-IEs,

criticalExtensionsFuture      SEQUENCE {}

}

}

ProvideLocationInformation-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation                  OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideLocationInformation

commonSL-PRS-MethodsIEsProvideLocationInformation    OCTET STRING    OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsProvideLocationInformation

sl-AoA-ProvideLocationInformation                    OCTET STRING    OPTIONAL, -- Containing SL-AoA-ProvideLocationInformation

sl-RTT-ProvideLocationInformation                    OCTET STRING    OPTIONAL, -- Containing SL-RTT-ProvideLocationInformation

sl-TDOA-ProvideLocationInformation                   OCTET STRING    OPTIONAL, -- Containing SL-TDOA-ProvideLocationInformation

sl-TOA-ProvideLocationInformation                    OCTET STRING    OPTIONAL, -- Containing SL-TOA-ProvideLocationInformation

lateNonCriticalExtension                             OCTET STRING    OPTIONAL,

nonCriticalExtension                                 SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	Abort

The Abort message body in an SLPP message carries a request to abort an ongoing SLPP procedure.

-- ASN1START

-- TAG-ABORT-START

Abort ::= SEQUENCE {

criticalExtensions    CHOICE {

abort                       Abort-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

Abort-IEs ::= SEQUENCE {

commonIEsAbort             CommonIEsAbort  OPTIONAL,

lateNonCriticalExtension   OCTET STRING    OPTIONAL,

nonCriticalExtension       SEQUENCE {}     OPTIONAL

}

-- TAG-ABORT-STOP

-- ASN1STOP

#### –	Error

The Error message body in an SLPP message carries information concerning an SLPP message that was received with errors.

-- ASN1START

-- TAG-ERROR-START

Error ::= CHOICE {

criticalExtensions    CHOICE {

error                       Error-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

Error-IEs ::= SEQUENCE {

commonIEsError              CommonIEsError  OPTIONAL,

lateNonCriticalExtension    OCTET STRING    OPTIONAL,

nonCriticalExtension        SEQUENCE {}     OPTIONAL

}

-- TAG-ERROR-STOP

-- ASN1STOP

## 6.3	SLPP information elements

### 6.3.1	Common information elements

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-2 [10] and TS 38.101-1 [11].

-- ASN1START

-- TAG-ARFCN-VALUENR-START

ARFCN-ValueNR ::= INTEGER (0..3279165)

-- TAG-ARFCN-VALUENR-STOP

-- ASN1STOP

#### –	CommonIEsAbort

The IE CommonIEsAbort carries common IEs for an Abort SLPP message Type.

-- ASN1START

-- TAG-COMMONIESABORT-START

CommonIEsAbort ::= SEQUENCE {

abortCause        ENUMERATED { undefined, stopPeriodicReporting }

}

-- TAG-COMMONIESABORT-STOP

-- ASN1STOP

CommonIEsAbort field descriptions

abortCause This field defines the request to abort an ongoing procedure. The abort cause 'stopPeriodicReporting' is used by an endpoint to stop any ongoing location reporting configured as periodicalReporting in the IE CommonIEsRequestLocationInformation.

#### –	CommonIEsError

The IE CommonIEsError carries common IEs for an Error SLPP message Type.

-- ASN1START

-- TAG-COMMONIESERROR-START

CommonIEsError ::= SEQUENCE {

errorCause         ENUMERATED { undefined, slppMessageHeaderError, slppMessageBodyError, incorrectDataValue }

}

-- TAG-COMMONIESERROR-STOP

-- ASN1STOP

CommonIEsError field descriptions

errorCause This field defines the cause for an error. 'slppMessageHeaderError' and 'slppMessageBodyError' are used if a receiver is able to detect a coding error in the SLPP header (i.e., in the common fields) or SLPP message body respectively. 'incorrectDataValue' is used if a receiver receives an incorrect data value.

#### –	GNSS-ID-Bitmap

The IE GNSS-ID-Bitmap is used to indicate several GNSSs using a bit map.

-- ASN1START

-- TAG-GNSS-ID-BITMAP-START

GNSS-ID-Bitmap ::= BIT STRING { gps (0), sbas (1), qzss (2), galileo (3), glonass (4), bds (5), navic (6) } (SIZE (1..16))

-- TAG-GNSS-ID-BITMAP-STOP

-- ASN1STOP

#### –	LCS-GCS-Translation

The IE LCS-GCS-Translation provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8].

-- ASN1START

-- TAG-LCS-GCS-TRANSLATION-START

LCS-GCS-Translation ::= SEQUENCE {

alpha                    INTEGER (0..3599),

beta                     INTEGER (0..3599),

gamma                    INTEGER (0..3599)

}

-- TAG-LCS-GCS-TRANSLATION-STOP

-- ASN1STOP

LCS-GCS-Translation field descriptions

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [8]. Scale factor 0.1 degree; range 0 to 359.9 degrees.

beta This field specifies the downtilt angle β for the translation of the LCS to a GCS as defined in TR 38.901 [8]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [8]. Scale factor 0.1 degree; range 0 to 359.9 degrees.

#### –	LOS-NLOS-Indicator

The IE LOS-NLOS-Indicator provides information on the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver.

-- ASN1START

-- TAG-LOS-NLOS-INDICATOR-START

LOS-NLOS-Indicator ::= SEQUENCE {

indicator              CHOICE {

soft                   INTEGER (0..10),

hard                   BOOLEAN

}

}

-- TAG-LOS-NLOS-INDICATOR-STOP

-- ASN1STOP

LOS-NLOS-Indicator field descriptions

indicator This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS. - soft: Integer value '0' indicates likelihood 0, integer value '10' indicates likelihood 1. Scale factor 0.1; range 0 to 1. - hard: FALSE indicates likelihood '0', TRUE indicates likelihood '1'.

#### –	NCGI

The IE NCGI specifies the NR Cell Global Identifier (NCGI) which is used to identify NR cells globally (TS 38.331 [2]).

-- ASN1START

-- TAG-NCGI-START

NCGI ::= SEQUENCE {

mcc                SEQUENCE (SIZE (3))     OF INTEGER (0..9),

mnc                SEQUENCE (SIZE (2..3))  OF INTEGER (0..9),

nr-CellIdentity    BIT STRING (SIZE (36))

}

-- TAG-NCGI-STOP

-- ASN1STOP

#### –	NR-PhysCellID

The IE NR-PhysCellID specifies the NR physical cell identifier (TS 38.331 [2]).

-- ASN1START

-- TAG-NR-PHYSCELLID-START

NR-PhysCellID ::= INTEGER (0..1007)

-- TAG-NR-PHYSCELLID-STOP

-- ASN1STOP

#### –	PositioningModes

The IE PositioningModes is used to indicate several positioning modes using a bit map. This is represented by a bit string, with a one value at the bit position means the particular positioning mode is addressed; a zero value means not addressed.

-- ASN1START

-- TAG-POSITIONINGMODES-START

PositioningModes ::= BIT STRING { sl-TargetUE-Based (0), sl-TargetUE-Assisted (1) } (SIZE (1..8))

-- TAG-POSITIONINGMODES-STOP

-- ASN1STOP

#### –	SL-RTD-Info

The IE SL-RTD-Info provides time synchronization information of SL Anchor UEs.

-- ASN1START

-- TAG-SL-RTD-INFO-START

SL-RTD-Info ::= SEQUENCE ( SIZE (1.. maxNrOfUEs)) OF RTD-InfoListPerAnchorUE

RTD-InfoListPerAnchorUE ::= SEQUENCE {

applicationLayerID          OCTET STRING,

referenceRTD-Info           ReferenceRTD-Info                  OPTIONAL,

rtd-BetweenAnchorUEs        CHOICE {

subframeOffset              INTEGER (0..1966079),

sl-OffsetDFN                INTEGER (0..1000)

}                                                              OPTIONAL,

rtd-Quality                 SL-TimingQuality                   OPTIONAL,

syncSourceType              ENUMERATED { gnss, gNB-eNB, ue}    OPTIONAL

}

ReferenceRTD-Info ::= SEQUENCE {

syncSourceType        ENUMERATED { gnss, gNB-eNB, ue},

applicationLayerID    OCTET STRING                       OPTIONAL,

nrCell-Identify       SEQUENCE {

nr-PhysCellID             NR-PhysCellID              OPTIONAL,

nr-ARFCN                  ARFCN-ValueNR              OPTIONAL,

nr-CellGlobalID           NCGI                       OPTIONAL

}                                                        OPTIONAL

}

-- TAG-SL-RTD-INFO-STOP

-- ASN1STOP

SL-RTD-Info field descriptions

nrCell-Identify This field provides NR cell identity information. The field is present only if the synchronization source of an SL Anchor UE is gNB/eNB.

referenceRTD-Info This field defines the reference RTD and comprises the following sub-fields: - syncSourceType: This field specifies the synchronization source type. - applicationLayerID: This field provides the application layer ID of the reference UE if the syncSourceType is set to UE.

rtdBetweenAnchorUEs This field specifies the RTD between SL Anchor UEs: - subframeOffset: This field specifies the subframe boundary offset at the TRP antenna location between the reference UE and this neighbour UE in time units where Δfmax=480∙103 Hz and (TS 38.211 [6]). The offset is counted from the beginning of a subframe #0 of the reference UE to the beginning of the closest subsequent subframe of this neighbour UE. Scale factor 1 Tc. - sl-OffsetDFN: This field indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

rtd-Quality This field specifies the quality of the RTD.

#### –	SL-TimeStamp

The IE SL-TimeStamp defines the UE measurement associated time stamp.

-- ASN1START

-- TAG-SL-TIMESTAMP-START

SL-TimeStamp ::= CHOICE {

dfn-Time                    SEQUENCE {

syncSourceType              ENUMERATED { gnss, ue}    OPTIONAL,

applicationLayerID          OCTET STRING              OPTIONAL,

dfn                         INTEGER (0.. 1023),

nr-Slot                     CHOICE {

scs15                       INTEGER (0..9),

scs30                       INTEGER (0..19),

scs60                       INTEGER (0..39),

scs120                      INTEGER (0..79)

}

},

sfn-Time                    SEQUENCE {

nr-PhysCellID               NR-PhysCellID             OPTIONAL,

nr-ARFCN                    ARFCN-ValueNR             OPTIONAL,

nr-CellGlobalID             NCGI                      OPTIONAL,

nr-SFN                      INTEGER (0..1023),

nr-Slot                     CHOICE {

scs15                       INTEGER (0..9),

scs30                       INTEGER (0..19),

scs60                       INTEGER (0..39),

scs120                      INTEGER (0..79)

}

}

}

-- TAG-SL-TIMESTAMP-STOP

-- ASN1STOP

SL-TimeStamp field descriptions

dfn-Time This field provides the DFN based time stamp.

sfn-Time This field provides the SFN based time stamp. If this field is present, at least one of nr-PhysCellID, nr-ARFCN, or nr-CellGlobalID shall be present.

#### –	SL-TimingQuality

The IE SL-TimingQuality defines the quality of a timing value (e.g., of a TOA measurement).

-- ASN1START

-- TAG-SL-TIMINGQUALITY-START

SL-TimingQuality ::= SEQUENCE {

timingQualityValue        INTEGER (0..31),

timingQualityResolution   ENUMERATED {mdot1, m1, m10, m30}

}

-- TAG-SL-TIMINGQUALITY-STOP

-- ASN1STOP

SL-TimingQuality field descriptions

timingQualityValue This field provides an estimate of uncertainty of the timing value for which the IE SL-TimingQuality is provided in units of metres.

timingQualityResolution This field provides the resolution used in the timingQualityValue field. Enumerated values mdot1, m1, m10, m30 correspond to 0.1, 1, 10, 30 metres, respectively.

### 6.3.2	UE capability information elements

#### –	ScheduledLocationTimeSupportPerMode

The IE ScheduledLocationTimeSupportPerMode is used by the endpoint to indicate the time bases supported for scheduled location requests for each positioning mode indicated by PositioningModes.

-- ASN1START

-- TAG-SCHEDULEDLOCATIONTIMESUPPORTPERMODE-START

ScheduledLocationTimeSupportPerMode ::= SEQUENCE {

utcTime                                 PositioningModes                              OPTIONAL,

gnssTime                                SEQUENCE {

posModes                                PositioningModes,

gnss-TimeIDs                            GNSS-ID-Bitmap

}                                                                                     OPTIONAL,

nrTime                                  PositioningModes                              OPTIONAL,

relativeTime                            PositioningModes                              OPTIONAL

}

-- TAG-SCHEDULEDLOCATIONTIMESUPPORTPERMODE-STOP

-- ASN1STOP

### 6.3.3	Void

## 6.4	Multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxNrOfUEs                                  INTEGER ::= 256        -- Max number of Tx UEs or Rx UEs

nrMaxBands                                  INTEGER ::= 1024       -- Maximum number of supported bands in UE capability

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

#### –	End of SLPP-PDU-Definitions

-- ASN1START

END

-- ASN1STOP

## 6.5	SLPP PDU Common Contents

#### –	SLPP-PDU-CommonContents

This ASN.1 segment is the start of the SLPP PDU Common Contents definitions.

-- ASN1START

-- TAG-SLPP-PDU-COMMONCONTENTS-START

SLPP-PDU-CommonContents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ARFCN-ValueNR,

NCGI,

NR-PhysCellID

FROM

SLPP-PDU-Definitions;

-- TAG-SLPP-PDU-COMMON-CONTENTS-STOP

-- ASN1STOP

#### –	CommonIEsRequestCapabilities

-- ASN1START

-- TAG-COMMONIESREQUESTCAPABILITIES-START

CommonIEsRequestCapabilities ::= SEQUENCE {

...

}

-- TAG-COMMONIESREQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	CommonIEsProvideCapabilities

-- ASN1START

-- TAG-COMMONIESPROVIDECAPABILITIES-START

CommonIEsProvideCapabilities ::= SEQUENCE {

...

}

-- TAG-COMMONIESPROVIDECAPABILITIES-STOP

-- ASN1STOP

#### –	CommonIEsRequestAssistanceData

-- ASN1START

-- TAG-COMMONIESREQUESTASSISTANCEDATA-START

CommonIEsRequestAssistanceData ::= SEQUENCE {

...

}

-- TAG-COMMONIESREQUESTASSISTANCEDATA-STOP

-- ASN1STOP

#### –	CommonIEsProvideAssistanceData

-- ASN1START

-- TAG-COMMONIESPROVIDEASSISTANCEDATA-START

CommonIEsProvideAssistanceData ::= SEQUENCE {

...

}

-- TAG-COMMONIESPROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

#### –	CommonIEsRequestLocationInformation

The IE CommonIEsRequestLocationInformation carries common IEs for a Request Location Information SLPP message Type.

-- ASN1START

-- TAG-COMMONIESREQUESTLOCATIONINFORMATION-START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType                 LocationInformationType,

periodicalReporting                     PeriodicalReportingCriteria OPTIONAL,

additionalInformation                   AdditionalInformation       OPTIONAL,

qos                                     QoS                         OPTIONAL,

environment                             Environment                 OPTIONAL,

scheduledLocationTime                   ScheduledLocationTime       OPTIONAL,

locationCoordinateTypes                 LocationCoordinateTypes     OPTIONAL,

velocityTypes                           VelocityTypes               OPTIONAL,

...

}

LocationInformationType ::= ENUMERATED { locationEstimateRequired, locationMeasurementsRequired, locationEstimatePreferred,

locationMeasurementsPreferred, rangeEstimateRequired, rangeMeasurementsRequired,

rangeEstimatePreferred, rangeMeasurementsPreferred, directionEstimateRequired,

directionMeasurementsRequired, directionEstimatePreferred, directionMeasurementsPreferred,

rangeDirectionEstimateRequired, rangeDirectionMeasurementsRequired,

rangeDirectionEstimatePreferred, rangeDirectionMeasurementsPreferred,

relativeLocationEstimateRequired, relativeLocationMeasurementsRequired,

relativeLocationEstimatePreferred, relativeLocationMeasurementsPreferred, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

PeriodicalReportingCriteria ::= SEQUENCE {

reportingAmount                 ENUMERATED { ra2, ra4, ra8, ra16, ra32, ra64, ra-Infinity },

reportingInterval               ENUMERATED { ri1, ri2, ri4, ri8, ri10, ri16, ri20, ri32, ri64}

}

AdditionalInformation ::= ENUMERATED { onlyReturnInformationRequested, mayReturnAdditionalInformation}

QoS ::= SEQUENCE {

horizontalAccuracy              HorizontalAccuracy    OPTIONAL,

verticalCoordinateRequest       BOOLEAN,

verticalAccuracy                VerticalAccuracy      OPTIONAL,

rangeAccuracy                   RangeAccuracy         OPTIONAL,

azimuthAccuracy                 AzimuthAccuracy       OPTIONAL,

elevationAccuracy               ElevationAccuracy     OPTIONAL,

responseTime                    ResponseTime          OPTIONAL,

velocityRequest                 BOOLEAN

}

HorizontalAccuracy ::= SEQUENCE {

accuracy               INTEGER(0..255),

confidence             INTEGER(0..100)

}

VerticalAccuracy ::= SEQUENCE {

accuracy             INTEGER(0..255),

confidence           INTEGER(0..100)

}

RangeAccuracy ::= SEQUENCE {

accuracy          INTEGER(0..255),

confidence        INTEGER(0..100)

}

AzimuthAccuracy ::= SEQUENCE {

accuracy            INTEGER(0..127),

confidence          INTEGER(0..100)

}

ElevationAccuracy ::= SEQUENCE {

accuracy              INTEGER(0..63),

confidence            INTEGER(0..100)

}

ResponseTime ::= SEQUENCE {

time             INTEGER (1..128),

tenMilliSeconds  ENUMERATED { true}    OPTIONAL

}

Environment ::= ENUMERATED { badArea, notBadArea, mixedArea}

ScheduledLocationTime ::= SEQUENCE {

utc-Time                  UTCTime                                       OPTIONAL,

gnss-Time                 SEQUENCE {

gnss-TOD-Msec              INTEGER (0..3599999),

gnss-TimeID                GNSS-ID

}                                                                       OPTIONAL,

nr-Time                   SEQUENCE {

nr-PhysCellID             NR-PhysCellID,

nr-ARFCN                  ARFCN-ValueNR,

nr-CellGlobalID           NCGI                                      OPTIONAL,

nr-SFN                    INTEGER (0..1023),

nr-Slot                   CHOICE {

scs15                     INTEGER (0..9),

scs30                     INTEGER (0..19),

scs60                     INTEGER (0..39),

scs120                    INTEGER (0..79)

}                                                                   OPTIONAL

}                                                                       OPTIONAL,

relativeTime              INTEGER (1..1024)                             OPTIONAL

}

GNSS-ID ::= ENUMERATED{ gps, sbas, qzss, galileo, glonass, bds, navic, spare1}

LocationCoordinateTypes ::= SEQUENCE {

ellipsoidPoint                                          BOOLEAN,

ellipsoidPointWithUncertaintyCircle                     BOOLEAN,

ellipsoidPointWithUncertaintyEllipse                    BOOLEAN,

polygon                                                 BOOLEAN,

ellipsoidPointWithAltitude                              BOOLEAN,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid       BOOLEAN,

ellipsoidArc                                            BOOLEAN,

relative2D-LocationWithUncertaintyEllipse               BOOLEAN,

relative3D-LocationWithUncertaintyEllipsoid             BOOLEAN,

rangeAndDirection-range                                  BOOLEAN,

rangeAndDirection-azimuth                                BOOLEAN,

rangeAndDirection-elevation                              BOOLEAN,

...

}

VelocityTypes ::= SEQUENCE {

horizontalVelocity                                      BOOLEAN,

horizontalWithVerticalVelocity                          BOOLEAN,

horizontalVelocityWithUncertainty                       BOOLEAN,

horizontalWithVerticalVelocityAndUncertainty            BOOLEAN,

relativeVelocityWithUncertainty                         BOOLEAN,

...

}

-- TAG-COMMONIESREQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

CommonIEsRequestLocationInformation field descriptions

additionalInformation This field indicates whether the UE is allowed to return additional information to that requested. If this field indicates 'onlyReturnInformationRequested' then the UE shall not return any additional information. If this field indicates 'mayReturnAdditionalInformation' then the UE may return additional information. If a location estimate is returned, any additional information is restricted to that associated with a location estimate (e.g. might include velocity if velocity was not requested but cannot include measurements). If measurements are returned, any additional information is restricted to additional measurements (e.g. might include SL-AoA measurements if SL-TDOA measurements were requested but not SL-AoA measurements).

environment This field provides the UE with information about expected multipath and non line of sight (NLOS) in the current area. The following values are defined: - badArea: possibly heavy multipath and NLOS conditions (e.g. bad urban or urban). - notBadArea: no or light multipath and usually LOS conditions (e.g. suburban or rural). - mixedArea: environment that is mixed or not defined. If this field is absent, a default value of 'mixedArea' applies.

locationCoordinateTypes This field provides a list of the types of location estimate that an endpoint may return when a location estimate is obtained by the endpoint.

locationInformationType This field indicates whether an endpoint requires a location estimate or measurements. For 'locationEstimateRequired', 'rangeEstimateRequired' or 'relativeLocationEstimateRequired', the UE shall return a location or range estimate if possible, or indicate a location error if not possible. For 'locationMeasurementsRequired', 'rangeMeasurementsRequired' or 'relativeLocationMeasurementsRequired', the UE shall return measurements if possible, or indicate a location error if not possible. For 'locationEstimatePreferred', 'rangeEstimatePreferred' or 'relativeLocationEstimatePreferred', the UE shall return a location or range estimate if possible, but may also or instead return measurements for any requested position methods for which a location or range estimate is not possible. For 'locationMeasurementsPreferred, 'rangeMeasurementsPreferred' or 'relativeLocationMeasurementsPreferred', the UE shall return location or range measurements if possible, but may also or instead return a location estimate for any requested position methods for which return of location measurements is not possible.

periodicalReporting This field indicates that periodic reporting is requested and comprises the following subfields: - reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is set to 'ra-Infinity', the UE should continue periodic reporting until an SLPP Abort message is received. - reportingInterval indicates the interval between location information reports and the response time requirement for the first location information report. Enumerated values correspond to reporting intervals of 1, 2, 4, 8, 10, 16, 20, 32, and 64 seconds, respectively. Measurement reports containing no measurements or no location estimate are required when a reportingInterval expires before a UE is able to obtain new measurements or obtain a new location estimate.

qos This field indicates the quality of service and comprises a number of sub-fields. In the case of measurements, some of the sub-fields apply to the location estimate that could be obtained by the endpoint from the measurements provided by the UE assuming that the measurements are the only sources of error. This field comprises the following subfields: - azimuthAccuracy indicates the maximum (single-sided) horizontal direction error at an indicated confidence level. Scale factor of accuracy is 1 degree; range 0 to 127 degrees. 'confidence' corresponds to confidence as defined in TS 23.032 [7]. - elevationAccuracy indicates the maximum (single-sided) vertical direction error at an indicated confidence level. Scale factor of accuracy is 1 degree; range 0 to 63 degrees. 'confidence' corresponds to confidence as defined in TS 23.032 [7]. - horizontalAccuracy indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracy' corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7] and 'confidence' corresponds to confidence as defined in TS 23.032 [7]. - verticalCoordinateRequest indicates whether a vertical coordinate is required (TRUE) or not (FALSE) - verticalAccuracy indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracy' corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7] and 'confidence' corresponds to confidence as defined in TS 23.032 [7]. - rangeAccuracy indicates the maximum range error at an indicated confidence level. The 'accuracy' corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7] and 'confidence' corresponds to confidence as defined in TS 23.032 [7]. - responseTime - time indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. - tenMilliSeconds indicates the unit of the time field. If this field is present, the unit/resolution is 0.01 seconds. If this field is absent, the unit/resolution is 1 second. - velocityRequest indicates whether velocity (or measurements related to velocity) is requested (TRUE) or not (FALSE). All QoS requirements shall be obtained by the UE to the degree possible but it is permitted to return a response that does not fulfill all QoS requirements if some were not attainable.

scheduledLocationTime This field indicates that the UE is requested to obtain location measurements or location estimate valid at the scheduledLocationTime T and comprises the following subfields: - utc-Time provides T in UTC in the form of YYMMDDhhmmssZ. - gnss-Time provides T in GNSS system time of the GNSS indicated by gnss-TimeID. - gnss-TOD-Msec specifies the GNSS TOD in 1-milli-second resolution rounded down to the nearest millisecond unit. - nr-Time provides T in NR network time. - nr-PhysCellID, nr-ARFCN , nr-CellGlobalID identifies the reference cell (NR) that is used for the network time. - nr-SFN specifies the system frame number in NR. - nr-Slot specifies the slot number in NR for the indicated subcarrier spacing (SCS). The total NR network time is given by nr-SFN + nr-Slot. - relativeTime provides T in seconds from current time, where current time is defined as the time the IE CommonIEsRequestLocationInformation was received. NOTE 1: A location estimate returned to an LCS Client, AF or UE for a scheduled location time can be treated by the LCS Client, AF or UE as an estimate of the location of the UE at the scheduled location time (see TS 23.273 [5]). NOTE 2: If this field is present, at least one of utc-Time, gnss-Time, nr-Time, or relativeTime shall be present.

velocityTypes This fields provides a list of the types of velocity estimate that an endpoint may return when a velocity estimate is obtained by the endpoint.

#### –	CommonIEsProvideLocationInformation

The IE CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information SLPP message Type.

-- ASN1START

-- TAG-COMMONIESPROVIDELOCATIONINFORMATION-START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate                        LocationCoordinates            OPTIONAL, -- locationTargetUe-sl-pos

rangeAndOrDirection                     RangeAndOrDirection            OPTIONAL,

velocityEstimate                        Velocity                       OPTIONAL,

relativeLocationEstimate                RelativeLocationCoordinates    OPTIONAL,

locationError                           LocationError                  OPTIONAL,

...

}

LocationCoordinates ::= CHOICE {

ellipsoidPoint                                      EllipsoidPoint,

ellipsoidPointWithUncertaintyCircle                 EllipsoidPointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse                EllipsoidPointWithUncertaintyEllipse,

polygon                                             Polygon,

ellipsoidPointWithAltitude                          EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid   EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc                                        EllipsoidArc

}

RelativeLocationCoordinates ::= CHOICE {

relative2D-LocationWithUncertaintyEllipse                                      Relative2D-LocationWithUncertaintyEllipse,

relative3D-LocationWithUncertaintyEllipsoid                                    Relative3D-LocationWithUncertaintyEllipsoid,

...

}

Relative2D-LocationWithUncertaintyEllipse ::=   SEQUENCE {

x                                               INTEGER (-134217728..134217727),   -- 28 bit field

y                                               INTEGER (-134217728..134217727),   -- 28 bit field

uncertaintySemiMajor                            INTEGER (0..255),

uncertaintySemiMinor                            INTEGER (0..255),

orientationMajorAxis                            INTEGER (0..179),

confidence                                      INTEGER (0..100)

}

Relative3D-LocationWithUncertaintyEllipsoid ::= SEQUENCE {

x                                               INTEGER (-134217728..134217727),     -- 28 bit field

y                                               INTEGER (-134217728..134217727),     -- 28 bit field

z                                               INTEGER (-16777216..16777215),       -- 25 bit field

uncertaintySemiMajor                            INTEGER (0..255),

uncertaintySemiMinor                            INTEGER (0..255),

orientationMajorAxis                            INTEGER (0..179),

uncertaintyAltitude                             INTEGER (0..255),

confidence                                      INTEGER (0..100)

}

Velocity ::= CHOICE {

horizontalVelocity                              HorizontalVelocity,

horizontalWithVerticalVelocity                  HorizontalWithVerticalVelocity,

horizontalVelocityWithUncertainty               HorizontalVelocityWithUncertainty,

horizontalWithVerticalVelocityAndUncertainty    HorizontalWithVerticalVelocityAndUncertainty,

relativeVelocityWithUncertainty                 RelativeVelocityWithUncertainty

}

LocationError ::= SEQUENCE {

locationfailurecause        LocationFailureCause

}

LocationFailureCause ::= ENUMERATED { undefined, requestedMethodNotSupported, positionMethodFailure, periodicLocationMeasurementsNotAvailable}

EllipsoidPoint ::= SEQUENCE {

latitudeSign        ENUMERATED {north, south},

degreesLatitude     INTEGER (0..8388607),        -- 23 bit field

degreesLongitude    INTEGER (-8388608..8388607)  -- 24 bit field

}

EllipsoidPointWithUncertaintyCircle ::= SEQUENCE {

latitudeSign                             ENUMERATED {north, south},

degreesLatitude                          INTEGER (0..8388607),        -- 23 bit field

degreesLongitude                         INTEGER (-8388608..8388607), -- 24 bit field

uncertainty                              INTEGER (0..127)

}

EllipsoidPointWithUncertaintyEllipse ::= SEQUENCE {

latitudeSign                             ENUMERATED {north, south},

degreesLatitude                          INTEGER (0..8388607),        -- 23 bit field

degreesLongitude                         INTEGER (-8388608..8388607), -- 24 bit field

uncertaintySemiMajor                     INTEGER (0..127),

uncertaintySemiMinor                     INTEGER (0..127),

orientationMajorAxis                     INTEGER (0..179),

confidence                               INTEGER (0..100)

}

EllipsoidPointWithAltitude ::= SEQUENCE {

latitudeSign                   ENUMERATED {north, south},

degreesLatitude                INTEGER (0..8388607),        -- 23 bit field

degreesLongitude               INTEGER (-8388608..8388607), -- 24 bit field

altitudeDirection              ENUMERATED {height, depth},

altitude                       INTEGER (0..32767)           -- 15 bit field

}

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid ::= SEQUENCE {

latitudeSign                                          ENUMERATED {north, south},

degreesLatitude                                       INTEGER (0..8388607),        -- 23 bit field

degreesLongitude                                      INTEGER (-8388608..8388607), -- 24 bit field

altitudeDirection                                     ENUMERATED {height, depth},

altitude                                              INTEGER (0..32767),          -- 15 bit field

uncertaintySemiMajor                                  INTEGER (0..127),

uncertaintySemiMinor                                  INTEGER (0..127),

orientationMajorAxis                                  INTEGER (0..179),

uncertaintyAltitude                                   INTEGER (0..127),

confidence                                            INTEGER (0..100)

}

EllipsoidArc ::= SEQUENCE {

latitudeSign                ENUMERATED {north, south},

degreesLatitude             INTEGER (0..8388607),        -- 23 bit field

degreesLongitude            INTEGER (-8388608..8388607), -- 24 bit field

innerRadius                 INTEGER (0..65535),          -- 16 bit field,

uncertaintyRadius           INTEGER (0..127),

offsetAngle                 INTEGER (0..179),

includedAngle               INTEGER (0..179),

confidence                  INTEGER (0..100)

}

RangeAndOrDirection ::= SEQUENCE {

range                   Range       OPTIONAL,

azimuth                 Azimuth     OPTIONAL,

elevation               Elevation   OPTIONAL

}

Range ::= SEQUENCE {

rangeResult                  INTEGER (0..134217727),

uncertainty                  INTEGER (0..255),

confidence                   INTEGER (0..100)             OPTIONAL

}

Azimuth ::= SEQUENCE {

azimuthResult                INTEGER (0..3599),

uncertainty                  INTEGER (0..127),

confidence                   INTEGER (0..100)             OPTIONAL

}

Elevation ::= SEQUENCE {

elevationResult              INTEGER (0..1800),

uncertainty                  INTEGER (0..63),

confidence                   INTEGER (0..100)             OPTIONAL

}

HorizontalVelocity ::= SEQUENCE {

bearing               INTEGER(0..359),

horizontalSpeed       INTEGER(0..2047)

}

HorizontalWithVerticalVelocity ::= SEQUENCE {

bearing                            INTEGER(0..359),

horizontalSpeed                    INTEGER(0..2047),

verticalDirection                  ENUMERATED{upward, downward},

verticalSpeed                      INTEGER(0..255)

}

HorizontalVelocityWithUncertainty ::= SEQUENCE {

bearing                               INTEGER(0..359),

horizontalSpeed                       INTEGER(0..2047),

uncertaintySpeed                      INTEGER(0..255)

}

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing                                          INTEGER(0..359),

horizontalSpeed                                  INTEGER(0..2047),

verticalDirection                                ENUMERATED{upward, downward},

verticalSpeed                                    INTEGER(0..255),

horizontalUncertaintySpeed                       INTEGER(0..255),

verticalUncertaintySpeed                         INTEGER(0..255)

}

Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

PolygonPoints ::= SEQUENCE {

latitudeSign      ENUMERATED {north, south},

degreesLatitude   INTEGER (0..8388607),        -- 23 bit field

degreesLongitude  INTEGER (-8388608..8388607)  -- 24 bit field

}

RelativeVelocityWithUncertainty ::=        SEQUENCE {

radialVelocityComponent                    SEQUENCE {

unitsRadialVelocity                        ENUMERATED { mPerS, cmPerS, ... },

radialVelocity                             INTEGER (-2048..2047),

uncertaintyRadialVelocity                  INTEGER (0..255),

confidenceUncertaintyRadialVelocity        INTEGER (0..100)

}                                                                                             OPTIONAL,

transverseVelocityComponent                SEQUENCE {

unitsTransverseVelocity                    ENUMERATED { degPerSec1, degPerSec0-1, ... },

azimuth                                    SEQUENCE {

azimuthRateOfChange                        INTEGER (0..1023),

uncertaintyAzimuthRateOfChange             INTEGER (0..255),

confidenceUncertaintyAzimuthRateOfChange   INTEGER (0..100)

}                                                                                         OPTIONAL,

elevation                       SEQUENCE {

elevationRateOfChange                      INTEGER (0..1023),

uncertaintyElevationRateOfChange           INTEGER (0..255),

confidenceUncertaintyElevationRateOfChange INTEGER (0..100)

}                                                                                         OPTIONAL

}                                                                                             OPTIONAL,

...

}

-- TAG-COMMONIESPROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

CommonIEsProvideLocationInformation field descriptions

locationError This field shall be included if and only if a location estimate and measurements are not included in the SLPP PDU. The field includes information concerning the reason for the lack of location information. The enumerated value 'periodicLocationMeasurementsNotAvailable' shall be used by the UE if periodic location reporting was requested, but no measurements or location estimate are available when the reportingInterval expired.

locationEstimate This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [7]. Coding of the values of the various fields internal to each geographic shape follows the rules in TS 23.032 [7]. The conditions for including this field are defined for the locationInformationType field in a Request Location Information message.

rangeAndOrDirection This field provides a range and/or direction estimate as defined in TS 23.032 [7] for the "Range and Direction". - range provides the range component and comprises the following subfields: - rangeResult provides the range estimate in units of milli-metres, as defined in TS 23.032 [7] for the "Range and Direction". - uncertainty provides the uncertainty of the rangeResult and corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - confidence provides the confidence level for the uncertainty and corresponds to confidence as defined in TS 23.032 [7]. - azimuth provides the horizontal direction component and comprises the following subfields: - azimuthResult provides the horizontal direction (azimuth) as defined in TS 23.032 [7] for the "Range and Direction". Scale factor 0.1 degree; range 0 to 359.9 degrees. - uncertainty provides the single-sided uncertainty of the azimuthResult. Scale factor 1 degree; range 0 to 127 degrees. - confidence provides the confidence level for the uncertainty and corresponds to confidence as defined in TS 23.032 [7]. - elevation provides the vertical direction component and comprises the following subfields: - elevationResult provides the vertical direction (elevation) as defined in TS 23.032 [7] for the "Range and Direction". Scale factor 0.1 degree; range 0 to 180.0 degrees. - uncertainty provides the single-sided uncertainty of the elevationResult. Scale factor 1 degree; range 0 to 63 degrees. - confidence provides the confidence level for the uncertainty and corresponds to confidence as defined in TS 23.032 [7].

relativeLocationEstimate This field provides a relative location estimate. - relative2D-LocationWithUncertaintyEllipse provides the "relative 2D location with uncertainty ellipse" as defined in TS 23.032 [7] and comprises the following subfields: - x, y provides the value (in the unit of milli-metres) on x-axis and y-axis, respectively, of the relative location in the local cartesian system, as defined in TS 23.032 [7]. The origin of the cartesian system is the reference location of the relative positioning. - uncertaintySemiMajor provides the semi-major axis of the uncertainty ellipse. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - uncertaintySemiMinor provides the semi-minor axis of the uncertainty ellipse. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - orientationMajorAxis provides the orientation angle of the major axis as defined in TS 23.032 [7]. - confidence provides the confidence level for the uncertainty ellipse and corresponds to confidence as defined in TS 23.032 [7]. - relative3D-LocationWithUncertaintyEllipsoid provides the "relative 3D location with uncertainty ellipsoid" as defined in TS 23.032 [7] and comprises the following subfields: - x, y, z provides the value (in the unit of milli-meters) on x-axis, y-axis and z-axis, respectively, of the relative location in the local cartesian system, as defined in TS 23.032 [7]. The origin of the cartesian system is the reference location of the relative positioning. - uncertaintySemiMajor provides the semi-major axis of the uncertainty ellipsoid. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - uncertaintySemiMinor provides the semi-minor axis of the uncertainty ellipsoid. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - orientationMajorAxis provides the orientation angle of the major axis as defined in TS 23.032 [7]. - uncertaintyAltitude provides uncertainty altitude. The value corresponds to the encoded high accuracy extended uncertainty as defined in TS 23.032 [7]. - confidence provides the confidence level for the uncertainty ellipsoid and corresponds to confidence as defined in TS 23.032 [7].

relativeVelocityWithUncertainty This field provides the relative velocity with uncertainty as defined in TS 23.032 [7] and comprises the following subfields: - radialVelocityComponent provides the radial velocity component characterised by a rate of change of range between the endpoint A and endpoint B: - unitsRadialVelocity provides the unit for the radialVelocity. Enumerated values 'mPerS' and 'cmPerS' indicate units m/s and cm/s, respectively. - radialVelocity provides the radial velocity as defined in TS 23.032 [7] in units given in the unitsRadialVelocity field. Positive values indicate increasing range between endpoint A and B; negative values indicate decreasing range between endpoint A and B. - uncertaintyRadialVelocity provides the (single-sided) uncertainty of the radialVelocity in increments of 1 the unit given in the unitsRadialVelocity field. - confidenceUncertaintyRadialVelocity provides the confidence of the uncertaintyRadialVelocity, as defined in TS 23.032 [7] for the "Confidence". - transverseVelocityComponent provides the transverse velocity component characterised by a rate of change of direction to the endpoint B from the endpoint A: - unitsTransverseVelocity provides the unit for the azimuth and elevation components. Enumerated values 'degPerSec1' and 'degPerSec0-1' indicate units 1-degree per second and 0.1 degree per second, respectively. - azimuthRateOfChange provides the rate of change of azimuth measured clockwise from North in a horizontal plane through the endpoint A as defined in TS 23.032 [7] in units given in the unitsTransverseVelocity field. - uncertaintyAzimuthRateOfChange provides the (single-sided) uncertainty of the azimuthRateOfChange in increments of 1 units given in the unitsTransverseVelocity field. - confidenceUncertaintyAzimuthRateOfChange provides the confidence of the uncertaintyAzimuthRateOfChange, as defined in TS 23.032 [7] for the "Confidence". - elevationRateOfChange provides the rate of change of elevation measured from Zenith in a vertical plane through the endpoint A and B, as defined in TS 23.032 [7] in units given in the unitsTransverseVelocity field. - uncertaintyElevationRateOfChange provides the (single-sided) uncertainty of the elevationRateOfChange in increments of 1 in units given in the unitsTransverseVelocity field. - confidenceUncertaintyElevationRateOfChange provides the confidence of the uncertaintyElevationRateOfChange, as defined in TS 23.032 [7] for the "Confidence".

velocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [7]. Coding of the values of the various fields internal to each velocity shape (execpt for relativeVelocityWithUncertainty) follows the rules in TS 23.032 [7].

#### –	End of SLPP-PDU-CommonContents

-- ASN1START

END

-- ASN1STOP

## 6.6	SLPP PDU Common SL-PRS Methods Contents

#### –	SLPP-PDU-CommonSL-PRS-MethodsContents

This ASN.1 segment is the start of the SLPP PDU Common SL-PRS Methods Contents definitions.

-- ASN1START

-- TAG-SLPP-PDU-COMMONSL-PRS-METHODSCONTENTS-START

SLPP-PDU-CommonSL-PRS-MethodsContents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

EllipsoidPoint,

EllipsoidPointWithUncertaintyEllipse,

EllipsoidPointWithAltitude,

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid

FROM

SLPP-PDU-CommonContents

SL-TimeStamp,

maxNrOfUEs,

nrMaxBands

FROM

SLPP-PDU-Definitions;

-- TAG-SLPP-PDU-COMMONSL-PRS-METHODSCONTENTS-STOP

-- ASN1STOP

#### –	CommonSL-PRS-MethodsIEsRequestCapabilities

-- ASN1START

-- TAG-COMMONSL-PRS-METHODSIESREQUESTCAPABILITIES-START

CommonSL-PRS-MethodsIEsRequestCapabilities ::= SEQUENCE {

...

}

-- TAG-COMMONSL-PRS-METHODSIESREQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	CommonSL-PRS-MethodsIEsProvideCapabilities

-- ASN1START

-- TAG-COMMONSL-PRS-METHODSIESPROVIDECAPABILITIES-START

CommonSL-PRS-MethodsIEsProvideCapabilities ::= SEQUENCE {

--R1 41-1-1a Common SL-PRS processing capability per UE

sl-PRS-CommonProcCapabilityPerUE               SL-PRS-CommonProcCapabilityPerUE            OPTIONAL,

sl-PRS-CapabilityBandList                      SEQUENCE (SIZE (1..nrMaxBands)) OF SL-PRS-CapabilityPerBand,

...

}

SL-PRS-CapabilityPerBand ::=                SEQUENCE {

freqBandIndicatorNR                         INTEGER (1..1024),

--R1 41-1-1 Common SL-PRS processing capability in an SL BWP

sl-PRS-CommonProcCapabilityPerBand          SL-PRS-CommonProcCapabilityPerBand             OPTIONAL,

--R1 41-1-19 ARP location provision for sidelink as assistance data

sl-PositioningARP-LocationProvision         ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-19a Report of Rx ARP-ID with SL positioning measurements

sl-PositioningMeasReportWithRxARP-ID        ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-19b Report of Tx ARP-ID to LMF or another UE for the transmitted SL PRS

sl-PRS-ReportTxARP-ID                       ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-2 Receiving SL-PRS in a shared resource pool

sl-PRS-RxInSharedResourcePool               ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-3 Receiving SL-PRS in a dedicated resource pool

sl-PRS-RxInDedicatedResourcePool            SL-PRS-RxInDedicatedResourcePool               OPTIONAL,

--R1 41-1-4a Transmitting SL-PRS in a shared resource pool

sl-PRS-TxInSharedResourcePool               ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-4b Transmitting SL-PRS mode 1 in a dedicated resource pool

sl-PRS-TxScheme1InDedicatedResourcePool     ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-4c Transmitting SL-PRS mode 2 in a dedicated resource pool

sl-PRS-TxScheme2InDedicatedResourcePool     ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-7e SL PRS measurement for SL PRS-RSRP

sl-PRS-RSRP-Meas                            ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-7f SL PRS measurement for SL PRS-RSRPP

sl-PRS-RSRPP-Meas                           ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-11 TDM-based multiplexing of SL-PRS reception from different UEs in the same slot in dedicated resource pool

sl-PRS-TDM-Multiplexing                     ENUMERATED {supported}                         OPTIONAL,

--R1 41-1-12 Comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool

sl-PRS-RxCombMultiplexing                  ENUMERATED {supported}                          OPTIONAL,

--R1 41-1-13 Reporting the additional paths for SL positioning

sl-PRS-AdditionalPathsReport                ENUMERATED {n1,n2,n4,n6,n8}                    OPTIONAL,

--R1 41-1-14 LoS/NLoS indicator for SL positioning per measurement

sl-PRS-LOS-NLOS-Indication                  ENUMERATED {hard, hard-soft}                   OPTIONAL,

-- R1 41-1-20: Supports SL PRS Rx for a band configured with SL CA

sl-PRS-RxForBandWithSL-CA                   ENUMERATED {supported}                         OPTIONAL,

-- R1 41-1-21: Supports SL PRS Tx for a band configured with SL CA

sl-PRS-TxForBandWithSL-CA                   ENUMERATED {supported}                         OPTIONAL,

...

}

SL-PRS-CommonProcCapabilityPerUE ::=     SEQUENCE {

--R1 41-1-1a    Common SL-PRS processing capability

maxNumOfActiveSL-PRS-Resources           SEQUENCE {

fr1                                      ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24}                        OPTIONAL,

fr2                                      ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64, n128}   OPTIONAL

},

maxNumOfSlotswithActiveSL-PRS-Resources  SEQUENCE {

fr1                                      ENUMERATED {n1, n2, n3, n4, n6, n8}                                   OPTIONAL,

fr2                                      ENUMERATED {n1, n2, n4, n8, n12, n16, n24, n32, n48, n64}             OPTIONAL

}

}

SL-PRS-CommonProcCapabilityPerBand ::=   SEQUENCE {

maxSL-PRS-Bandwidth                      CHOICE {

fr1                                      ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100},

fr2                                      ENUMERATED {mhz50, mhz100, mhz200, mhz400}

},

maxNumOfActiveSL-PRS-ResourcesInOneSlot  CHOICE {

fr1                                      ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24},

fr2                                      ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64, n128}

},

maxNumOfSlotsWithActiveSL-PRS-Resources  CHOICE {

fr1                                      ENUMERATED {n1, n2, n3, n4, n6, n8},

fr2                                      ENUMERATED {n1, n2, n4, n8, n12, n16, n24, n32, n48, n64}

},

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources    ENUMERATED {ms20, ms30, ms40, ms50, ms80, ms100, ms160}

}

SL-PRS-RxInDedicatedResourcePool ::=  SEQUENCE {

numOfSupportedRxPSCCH-PerSlot         ENUMERATED {value1, value2},

supportedCP-TypeFor60kHzSCS           ENUMERATED {ncp, ncpAndECP}

}

-- TAG-COMMONSL-PRS-METHODSIESPROVIDECAPABILITIES-STOP

-- ASN1STOP

CommonSL-PRS-MethodsIEsProvideCapabilities field descriptions

sl-PositioningARP-LocationProvision Indicates whether UE supports provisioning of ARP location for sidelink as assistance data.

sl-PositioningMeasReportWithARP-ID Indicates whether UE supports providing Rx ARP-ID with SL positioning measurements.

sl-PRS-AdditionalPathsReport Indicates whether UE supports RSRPP reporting for additional paths. The value indicates the maximum number of additional detected path timing reporting for K additional paths for SL positioning. UE supporting this feature shall also support at least one of sl-PRS-RSTD-Meas, sl-RTOA-Meas, sl-PRS-RxTxTimeDiffWithoutTxTimeStamp, sl-PRS-RxTxTimeDiffWithTxTimeStamp, sl-PRS-RSRPP-Meas, or sl-AoA-Meas.

sl-PRS-CommonProcCapabilityPerBand Indicates the common SL-PRS processing capability per band, and comprises the following sub-fields: - maxSL-PRS-Bandwidth: Maximum SL PRS bandwidth in MHz in a resource pool for positioning, which is supported and reported by UE for SL-PRS measurement; - maxNumOfActiveSL-PRS-ResourcesInOneSlot: Maximum number of active SL PRS resources across all configured RPs in a slot assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE; - maxNumOfSlotsWithActiveSL-PRS-Resources: Maximum number of slots with active SL PRS resources across all configured RPs assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE; - minTimeAfterEndofSlotCarryActiveSL-PRS-Resources: Minimum time after the end of a slot carrying the active SL-PRS resource(s) assuming maximum number of symbols and maximum bandwidth for a UE to finish the SL-PRS resource and the associated PSCCH processing which is supported and reported by UE; NOTE 1: An SL PRS resource is considered as active starting at the end of the last symbol of the PSCCH carrying the SCI trigger and the occupancy is released at the end of timeline indicated in minTimeAfterEndofSlotCarryActiveSL-PRS-Resources.

sl-PRS-CommonProcCapabilityPerUE Indicates the common SL-PRS processing capability, and comprises the following sub-fields: - maxNumOfActiveSL-PRS-Resources: Maximum number of active SL PRS resources across all configured RPs across all bands in a slot assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE; - maxNumOfSlotswithActiveSL-PRS-Resources: Maximum number of slots with active SL PRS resources across all configured RPs across all bands assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand.

sl-PRS-LOS-NLOS-Indication Indicates whether UE supports LoS/NLoS indicator for SL positioning per measurement. The value indicates whether the indicator is hard value or hard+soft value. UE supporting this feature shall also support at least one of sl-PRS-RSTD-Meas, sl-RTOA-Meas, sl-PRS-RxTxTimeDiffWithoutTxTimeStamp, sl-PRS-RxTxTimeDiffWithTxTimeStamp, or sl-AoA-Meas.

sl-PRS-ReportTxARP-ID Indicates whether UE supports providing Tx ARP-ID for the transmitted SL PRS.

sl-PRS-RSRP-Meas Indicates whether UE supports SL PRS measurement for SL PRS-RSRP, and is comprised of the following functional components: - Support SL PRS-RSRP measurement based on SL-PRS; - Support SL PRS-RSRP measurement reporting. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand.

sl-PRS-RSRPP-Meas Indicates whether UE supports SL PRS measurement for SL PRS-RSRPP, and is comprised of the following functional components: - Support SL PRS-RSRPP measurement based on SL-PRS; - Support SL PRS-RSRPP measurement reporting. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand.

sl-PRS-RxCombMultiplexing Indicates whether UE supports comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool. UE supporting this feature shall also support sl-PRS-RxInDedicatedResourcePool.

sl-PRS-RxForBandWithSL-CA Indicates whether UE supports SL PRS reception in a single carrier for a shared SL PRS resource pool and/or a dedicated SL PRS resource pool for a band configured with SL CA. A UE that supports this feature shall also support sl-CA-Communication-r18, defined in TS 38.331 [2] and one of sl-PRS-RxInSharedResourcePool or sl-PRS-RxInDedicatedResourcePool. NOTE 1: In a shared SL PRS resource pool in a single SL carrier: Tx power control follows the rule defined for SL CA in NR Rel-18. NOTE 2: In a dedicated SL PRS resource pool in a single SL carrier when the slots (pre)configured for the dedicated SL PRS resource pool do not collide with the slots (pre)configured for any other resource pool or S-SSB resource(s) in other carriers.

sl-PRS-RxInDedicatedResourcePool Indicates whether UE supports receiving SL-PRS in dedicated resource pool and receiving SCI format 1B. This field comprises the following sub-fields: - numOfSupportedRxPSCCH-PerSlot: Indicates the number of PSCCH UE can receive in a slot. value1 corresponds to floor (NRB /10 RBs), value2 corresponds to 2*floor (NRB /10 RBs). NRB is the number of RBs defined per channel bandwidth in TS 38.101-1 [11] Table 5.3.2-1 for FR1 and TS 38.101-2 [10] Table 5.3.2-1 for FR2. - supportedCP-TypeFor60kHzSCS: Indicates the supported CP type for 60 kHz SCS. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand.

sl-PRS-RxInSharedResourcePool Indicates whether UE supports receiving SL-PRS in shared resource pool and receiving SCI format 2D. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand and sl-Reception-r16 defined in TS 38.331 [2].

sl-PRS-TDM-Multiplexing Indicates whether UE supports TDM-based multiplexing of SL-PRS reception from different UEs in the same slot in dedicated resource pool. UE supporting this feature shall also support sl-PRS-RxInDedicatedResourcePool.

sl-PRS-TxForBandWithSL-CA Indicates whether UE supports SL PRS transmission in a single carrier for a shared SL PRS resource pool and/or a dedicated SL PRS resource pool for a band configured with SL CA. A UE that supports this feature shall also support sl-CA-Communication-r18, defined in TS 38.331 [2] and one of sl-PRS-TxInSharedResourcePool, sl-PRS-TxScheme1InDedicatedResourcePool, or sl-PRS-TxScheme2InDedicatedResourcePool. NOTE 1: In a shared SL PRS resource pool in a single SL carrier: Tx power control follows the rule defined for SL CA in NR Rel-18. NOTE 2: In a dedicated SL PRS resource pool in a single SL carrier when the slots (pre)configured for the dedicated SL PRS resource pool do not collide with the slots (pre)configured for any other resource pool or S-SSB resource(s) in other carriers.

sl-PRS-TxInSharedResourcePool Indicates whether UE supports transmitting SL-PRS in a shared resource pool, and is comprised of the following functional components: - Support transmitting SL-PRS in shared resource pool; - Support transmitting SCI format 2D; - Support downlink pathloss based open loop power control. The supported resource allocation modes are the same as for communication and signaled in sl-TransmissionMode1-r16 and sl-TransmissionMode2-r16 defined in TS 38.331 [2]. UE supporting this feature shall also support sl-TransmissionMode1-r16 or sl-TransmissionMode2-r16 defined in TS 38.331 [2], and sl-PRS-RxInSharedResourcePool.

sl-PRS-TxScheme1InDedicatedResourcePool Indicates whether UE supports transmitting SL-PRS scheme 1 in a dedicated resource pool, and is comprised of the following functional components: - Support transmitting SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool; - Support transmitting SL-PRS according to the mapping rule between PSCCH and SL-PRS; - Support transmitting SCI format 1B; - Support receiving DCI format 3_2; - Support downlink pathloss based open loop power control of SL-PRS (NOTE 1). UE supporting this feature shall also support sl-PRS-RxInDedicatedResourcePool. NOTE 1: It is not required to be supported in a band indicated with only the PC5 interface in TS 38.101-1 [11] Table 5.2E.1-1.

sl-PRS-TxScheme2InDedicatedResourcePool Indicates whether UE supports transmitting SL-PRS scheme 2 in a dedicated resource pool, and is comprised of the following functional components: - Support transmitting SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool; - Support transmitting SL-PRS according to the mapping rule between PSCCH and SL-PRS; - Support transmitting SCI format 1B. UE supporting this feature shall also support at least one of sl-PRS-TxRandomSelection-r18 or sl-PRS-TxUsingFullSensing-r18 defined in TS 38.331 [2].

#### –	CommonSL-PRS-MethodsIEsRequestAssistanceData

-- ASN1START

-- TAG-COMMONSL-PRS-METHODSIESREQUESTASSISTANCEDATA-START

CommonSL-PRS-MethodsIEsRequestAssistanceData ::= SEQUENCE {

sl-PRS-AssistanceDataInfoReq                     BIT STRING { sl-PRS-SequenceID-Req    (0),

anchorUE-LocationInfoReq (1),

arp-LocationInfoReq      (2),

sl-POS-ARP-ID-Tx-Req     (3)

}    (SIZE (1..8))                                                                            OPTIONAL,

...

}

-- TAG-COMMONSL-PRS-METHODSIESREQUESTASSISTANCEDATA-STOP

-- ASN1STOP

CommonSL-PRS-MethodsIEsRequestAssistanceData field descriptions

sl-PRS-AssistanceDataInfoReq This field indicates the SL PRS Assistance Data requested. - bit 0 indicates whether the field sl-PRS-SequenceID in IE CommonSL-PRS-MethodsIEsProvideAssistanceData is requested or not; - bit 1 indicates whether the field anchorUE-LocationInformation in IE CommonSL-PRS-MethodsIEsProvideAssistanceData is requested or not; - bit 2 indicates whether the field arp-LocationInfo in IE CommonSL-PRS-MethodsIEsProvideAssistanceData is requested or not; - bit 3 indicates whether the field sl-POS-ARP-ID-Tx in IE CommonSL-PRS-MethodsIEsProvideAssistanceData is requested or not.

#### –	CommonSL-PRS-MethodsIEsProvideAssistanceData

-- ASN1START

-- TAG-COMMONSL-PRS-METHODSIESPROVIDEASSISTANCEDDATA-START

CommonSL-PRS-MethodsIEsProvideAssistanceData ::= SEQUENCE {

sl-PRS-AssistanceDataInfo                        SEQUENCE (SIZE (1..maxNrOfUEs)) OF SL-PRS-AssistanceData                OPTIONAL,

sl-PRS-Error                                     SL-PRS-AssistanceDataError                                              OPTIONAL,

...

}

SL-PRS-AssistanceData ::= SEQUENCE {

applicationLayerID            OCTET STRING,

sl-PRS-SequenceID             INTEGER(0..4095)              OPTIONAL,  -- SL PRS sequence ID for transmitting SL-PRS

anchorUE-LocationInformation  AnchorLocationCoordinates     OPTIONAL,

arp-LocationInfo              ARP-LocationInfo              OPTIONAL,

sl-PRS-TxInfo                 SL-PRS-TxInfo                 OPTIONAL,

sl-POS-ARP-ID-Tx              SL-POS-ARP-ID-Tx-InfoList     OPTIONAL,

...

}

AnchorLocationCoordinates ::= CHOICE {

ellipsoidPoint                                      EllipsoidPoint,

ellipsoidPointWithUncertaintyEllipse                EllipsoidPointWithUncertaintyEllipse,

ellipsoidPointWithAltitude                          EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid   EllipsoidPointWithAltitudeAndUncertaintyEllipsoid

}

ARP-LocationInfo ::= SEQUENCE {

referencePoint         ReferencePoint            OPTIONAL,

arp-LocationInfoList   SEQUENCE (SIZE (1..4)) OF ARP-LocationInfoElement

}

ReferencePoint ::= CHOICE {

location2D                          EllipsoidPointWithUncertaintyEllipse,

location3D                          EllipsoidPointWithAltitudeAndUncertaintyEllipsoid

}

ARP-LocationInfoElement ::= SEQUENCE {

sl-PRS-ARP-ID               INTEGER (1..4),

arp-LocationInfo            RelativeLocation

}

RelativeLocation ::= SEQUENCE {

milliArcSecondUnits  ENUMERATED { mas0-03, mas0-3, mas3, mas30},

heightUnits          ENUMERATED { mm, cm, m, spare1},

deltaLatitude        DeltaLatitude,

deltaLongitude       DeltaLongitude,

deltaHeight          DeltaHeight,

locationUNC          LocationUncertainty                             OPTIONAL

}

DeltaLatitude ::= SEQUENCE {

deltaLatitude        INTEGER (-1024..1023),

coarseDeltaLatitude  INTEGER (0..4095)        OPTIONAL

}

DeltaLongitude ::= SEQUENCE {

deltaLongitude        INTEGER (-1024..1023),

coarseDeltaLongitude  INTEGER (0..4095)        OPTIONAL

}

DeltaHeight ::= SEQUENCE {

deltaHeight        INTEGER (-1024..1023),

coarseDeltaHeight  INTEGER (0..4095)        OPTIONAL

}

LocationUncertainty ::= SEQUENCE {

horizontalUncertainty    INTEGER (0..255),

horizontalConfidence     INTEGER (0..100),

verticalUncertainty      INTEGER (0..255),

verticalConfidence       INTEGER (0..100)

}

SL-PRS-AssistanceDataError ::= ENUMERATED { undefined, assistanceDataNotAvailable, ... }

SL-PRS-TxInfo ::=                 SEQUENCE {

sl-PRS-Priority                   INTEGER (1..8)                                                             OPTIONAL,

sl-PRS-DelayBudget                INTEGER (0..1023)                                                          OPTIONAL,

sl-PRS-Bandwidth                  ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40,

mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400,

spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}          OPTIONAL,

sl-PRS-Periodicity                ENUMERATED {ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800,

ms900, ms1000, spare6, spare5, spare4, spare3, spare2, spare1}    OPTIONAL,

sl-PRS-Transmission               ENUMERATED {true}                                                          OPTIONAL

}

-- TAG-COMMONSL-PRS-METHODSIESPROVIDEASSISTANCEDDATA-STOP

-- ASN1STOP

CommonSL-PRS-MethodsIEsProvideAssistanceData field descriptions

anchorUE-LocationInformation This field provides the location coordinates of an SL Anchor UE identified by applicationLayerID.

arp-LocationInfo This field provides the location coordinates of the ARPs of the UE identified by applicationLayerID.

applicationLayerID This field provides the application layer ID as defined in TS 23.287 [9] for which the SL-PRS-AssistanceData is applicable.

referencePoint This field provides the reference point used to define the location of ARPs provided in the arp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the sl-PRS-AssistanceDataInfo list.

sl-PRS-ARP-ID This field provides the ARP ID of an ARP. The ARP ID is used to uniquely identify an ARP associated with the UE identified by applicationLayerID.

sl-PRS-Bandwidth This field provides the bandwidth of SL-PRS transmission which is used in UEAssistanceInformation message as defined in TS 38.331 [2] and the SL-PRS resource request MAC CE as defined in TS 38.321 [15]. Value mhz5 corresponds to 5 MHz, value mhz10 corresponds to 10 MHz, and so on.

sl-PRS-DelayBudget This field provides the SL-PRS delay budget which is used in UEAssistanceInformation message as defined in TS 38.331 [2] and the resource selection as defined in TS 38.321 [15].

sl-PRS-Error This field provides SL-PRS error reasons.

sl-PRS-Periodicity This field provides the periodicity of SL-PRS which is used in UEAssistanceInformation message as defined in TS 38.331 [2]. Value ms100 corresponds to 100 ms, value ms200 corresponds to 200 ms, and so on.

sl-PRS-Priority This field provides the priority of SL-PRS which is used in UEAssistanceInformation message as defined in TS 38.331 [2] and the resource selection as defined in TS 38.321 [15]. Value 1 is the highest priority whereas value 8 is the lowest priority.

sl-PRS-SequenceID This field specifies the sequence ID used to initialize cinit value used in pseudo random generator for generation of SL PRS sequence for transmission on a given SL PRS Resource, as specified in TS 38.211 [6] for a UE identified by applicationLayerID. If UE does not receive a sequence ID via this SLPP message, the sequence ID is based on the 12 LSB bits CRC of PSCCH associated with the SL PRS.

sl-PRS-Transmission This field, if present, indicates that the UE is requested to start the SL-PRS transmission once the resource is available. If this field is absent, the UE can store the SL-PRS-TxInfo for future SL-PRS transmission (e.g., triggered by SCI from a peer UE).

#### –	CommonSL-PRS-MethodsIEsRequestLocationInformation

-- ASN1START

-- TAG-COMMONSL-PRS-METHODSIESREQUESTLOCATIONINFORMATION-START

CommonSL-PRS-MethodsIEsRequestLocationInformation ::= SEQUENCE {

sl-POS-ARP-ID-Tx-Req                                  ENUMERATED { true }                 OPTIONAL,

...

}

-- TAG-COMMONSL-PRS-METHODSIESREQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	CommonSL-PRS-MethodsIEsProvideLocationInformation

-- ASN1START

-- TAG-COMMONSL-PRS-METHODSIESPROVIDELOCATIONINFORMATION-START

CommonSL-PRS-MethodsIEsProvideLocationInformation ::= SEQUENCE {

sl-POS-ARP-ID-Tx                                      SL-POS-ARP-ID-Tx-InfoList            OPTIONAL,

sl-PRS-Error                                          SL-PRS-LocationInformationError      OPTIONAL,

...

}

SL-POS-ARP-ID-Tx-InfoList ::= SEQUENCE (SIZE (1..4)) OF SL-POS-ARP-ID-Tx-Info

SL-POS-ARP-ID-Tx-Info ::= SEQUENCE {

sl-POS-ARP-ID                          INTEGER (1..4),

sl-PRS-ResourceIdList-Tx               SEQUENCE (SIZE(1..16)) OF SL-PRS-ResourceId-Tx

}

SL-PRS-LocationInformationError ::= ENUMERATED { undefined, ... }

SL-PRS-ResourceId-Tx ::= SEQUENCE {

sl-PRS-ResourceId        INTEGER (0..16)                      OPTIONAL,

tx-TimeStamp             SL-TimeStamp

}

-- TAG-COMMONSL-PRS-METHODSIESPROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	End of SLPP-PDU-CommonSL-PRS-MethodsContents

-- ASN1START

END

-- ASN1STOP

## 6.7	SLPP PDU SL-AoA Contents

#### –	SLPP-PDU-SL-AoA-Contents

This ASN.1 segment is the start of the SLPP PDU SL-AoA Contents definitions.

-- ASN1START

-- TAG-SLPP-PDU-SL-AOA-CONTENTS-START

SLPP-PDU-SL-AoA-Contents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

LCS-GCS-Translation,

LOS-NLOS-Indicator,

PositioningModes,

SL-TimeStamp,

SL-TimingQuality,

maxNrOfUEs,

ScheduledLocationTimeSupportPerMode,

nrMaxBands

FROM

SLPP-PDU-Definitions

LocationCoordinateTypes,

VelocityTypes

FROM

SLPP-PDU-CommonContents;

-- TAG-SLPP-PDU-SL-AOA-CONTENTS-STOP

-- ASN1STOP

#### –	SL-AoA-RequestCapabilities

-- ASN1START

-- TAG-SL-AOA-REQUESTCAPABILITIES-START

SL-AoA-RequestCapabilities ::= SEQUENCE {

...

}

-- TAG-SL-AOA-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	SL-AoA-ProvideCapabilities

The IE SL-AoA-ProvideCapabilities is used to indicate the support of SL-AoA and to provide SL-AoA positioning capabilities.

-- ASN1START

-- TAG-SL-AOA-PROVIDECAPABILITIES-START

SL-AoA-ProvideCapabilities ::= SEQUENCE {

positioningModes                PositioningModes,

tenMsUnitResponseTime           PositioningModes                                  OPTIONAL,

periodicalReporting             PositioningModes                                  OPTIONAL,

scheduledLocationRequestSupported    ScheduledLocationTimeSupportPerMode          OPTIONAL,

locationCoordinateTypes              LocationCoordinateTypes                      OPTIONAL,

velocityTypes                        VelocityTypes                                OPTIONAL,

sl-AoA-CapabilityBandList       SEQUENCE (SIZE (1..nrMaxBands)) OF SL-AoA-CapabilityPerBand,

...

}

SL-AoA-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-7g SL PRS measurement for SL AoA

sl-AoA-Meas                        BIT STRING { gcs (0), lcsWithTranslation (1), lcsWithoutTranslation (2) } (SIZE (1..8))

OPTIONAL,

measurementsForMultipleARP-IDs-Rx  ENUMERATED { supported }                       OPTIONAL,

...

}

-- TAG-SL-AOA-PROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-AoA-ProvideCapabilities field descriptions

locationCoordinateTypes This parameter identifies the geographical location coordinate types that a target UE supports for SL-AoA. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE supports SL-AoA measurements for multiple SL-PRS Rx ARP-IDs.

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support periodicalReporting in IE CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-AoA mode(s) supported by the UE.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the UE supports scheduled location requests, i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the UE does not support scheduled location requests.

sl-AoA-Meas Indicates whether UE supports SL PRS measurement for SL-AoA, and is comprised of the following functional components: - Support SL AoA measurement based on SL-PRS; - Support SL AoA measurement reporting types. The value indicates the supported SL AoA measurement reporting types. The left most bit in the bitmap corresponds to GCS, the next bit in the bitmap corresponds to LCS with translation, the right most bit in the bitmap corresponds to LCS without translation. A bit in the bitmap is set to 1 if the corresponding type is supported by the UE. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand.

tenMsUnitResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'tenMilliSeconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one value at the bit position means 'tenMilliSeconds' response time unit for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support 'tenMilliSeconds' response time unit in CommonIEsRequestLocationInformation.

velocityTypes This parameter identifies the velocity types that a target UE supports for SL-AoA. TRUE indicates that a velocity type is supported and FALSE that it is not. If this field is absent, velocity reporting is not supported.

#### –	SL-AoA-RequestAssistanceData

-- ASN1START

-- TAG-SL-AOA-REQUESTASSISTANCEDATA-START

SL-AoA-RequestAssistanceData ::= SEQUENCE {

expectedSL-ElevationAoA-AndUncertainty          ENUMERATED { true}                           OPTIONAL,

...

}

-- TAG-SL-AOA-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

#### –	SL-AoA-ProvideAssistanceData

-- ASN1START

-- TAG-SL-AOA-PROVIDEASSISTANCEDATA-START

SL-AoA-ProvideAssistanceData ::= SEQUENCE {

sl-AoA-AssistanceDataInfo        SEQUENCE (SIZE (1..maxNrOfUEs)) OF SL-AoA-AssistanceData     OPTIONAL,

sl-AoA-Error                     SL-AoA-AssistanceDataError                                   OPTIONAL,

...

}

SL-AoA-AssistanceData ::= SEQUENCE {

applicationLayerID                           OCTET STRING,

expectedSL-AoA                               SEQUENCE {

expectedSL-AzimuthAoA                        INTEGER(0..3599),                  -- expected-SL-AoA-and-Uncertainty

expectedSL-AzimuthAoA-Uncertainty            INTEGER (0..60)         OPTIONAL,

expectedSL-ElevationAoA                      INTEGER(0..1800)        OPTIONAL,  -- expected-SL-AoA-and-Uncertainty

expectedSL-ElevationAoA-Uncertainty          INTEGER(0..30)          OPTIONAL

},

lcs-GCS-TranslationParameter                 LCS-GCS-Translation         OPTIONAL,

...

}

SL-AoA-AssistanceDataError ::= ENUMERATED { undefined, assistanceDataNotAvailable, ... }

-- TAG-SL-AoA-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

SL-AoA-ProvideAssistanceData field descriptions

applicationLayerID This field provides an application layer ID as defined in TS 23.287 [9] which is used to identify a UE.

expectedSL-AzimuthAoA This field specifies expected azimuth angle of arrival. Scale factor 0.1 degree; range 0 to 359.9 degrees.

expectedSL-AzimuthAoA-Uncertainty This field specifies the (single-sided) uncertainty of the expected azimuth angle of arrival. If this field is absent, it indicates maximum uncertainty (60 degrees). Scale factor 1 degree; range 0 to 60 degrees.

expectedSL-ElevationAoA This field specifies expected elevation angle of arrival. Scale factor 0.1 degree; range 0 to 180 degrees.

expectedSL-ElevationAoA-Uncertainty This field specifies expected the (single-sided) uncertainty of the expected elevation angle of arrival. If this field is absent, it indicates maximum uncertainty (30 degrees). Scale factor 1 degree; range 0 to 30 degrees.

lcs-GCS-TranslationParameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8].

sl-AoA-Error This field provides SL-AoA error reasons.

#### –	SL-AoA-RequestLocationInformation

-- ASN1START

-- TAG-SL-AOA-REQUESTLOCATIONINFORMATION-START

SL-AoA-RequestLocationInformation ::= SEQUENCE {

sl-AoA-Request                        ENUMERATED { aoa, zoa, both},

measurementReportingType              ENUMERATED { gcs, lcsWithTranslation, lcsWithoutTranslation }    OPTIONAL,

measurementsForMultipleARP-IDs-Rx     SEQUENCE {

requestedARP-IDs-Rx                   BIT STRING (SIZE (4))                                        OPTIONAL

}                                                                                                      OPTIONAL,

sl-ARP-InfoRequest                    ENUMERATED { true }                                              OPTIONAL,

sl-LOS-NLOS-IndicatorRequest          ENUMERATED { true }                                              OPTIONAL,

sl-PRS-RSRP-Request                   ENUMERATED { true }                                              OPTIONAL,

sl-RSRPP-Request                      ENUMERATED { true }                                              OPTIONAL,

sl-AdditionalPathsRequest             ENUMERATED { true }                                              OPTIONAL,

...

}

-- TAG-SL-AOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

SL-AoA-RequestLocationInformation field descriptions

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-AoA measurements for multiple SL-PRS Rx ARP-IDs.

requestedARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-AoA measurements for indicated SL-PRS Rx ARP-IDs. Bit 1 in the bit string indicates ARP-ID = 1, bit 2 indicates ARP-ID = 2, and so on.

sl-AdditionalPathsRequest This field, if present, indicates that the UE is requested to provide sl-AoA-AdditionalPathList.

sl-ARP-InfoRequest This field, if present, indicates that the UE is requested to provide sl-POS-ARP-ID-Rx.

sl-LOS-NLOS-IndicatorRequest This field, if present, indicates that the UE is requested to provide the estimated los-NLOS-Indicator.

sl-PRS-RSRP-Request This field, if present, indicates that the UE is requested to provide sl-PRS-RSRP-Result.

sl-RSRPP-Request This field, if present, indicates that the UE is requested to provide sl-RSRPP.

#### –	SL-AoA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-START

SL-AoA-ProvideLocationInformation ::= SEQUENCE {

sl-AoA-SignalMeasurementInformation   SL-AoA-SignalMeasurementInformation    OPTIONAL,

sl-AoA-Error                          SL-AoA-LocationInformationError        OPTIONAL,

...

}

SL-AoA-SignalMeasurementInformation ::= SEQUENCE {

sl-AoA-MeasList                         SEQUENCE (SIZE(1.. maxNrOfUEs)) OF SL-AoA-MeasElementPerARP-ID-Rx,

...

}

SL-AoA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-AoA-MeasElement

SL-AoA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

sl-LCS-GCS-Translation                LCS-GCS-Translation       OPTIONAL,  -- Cond FirstElement

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-AngleQuality                       MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AoA-AdditionalPathList             SL-AoA-AdditionalPathList OPTIONAL,

sl-AzimuthAoA-Result                  INTEGER (0..3599)         OPTIONAL,  -- sl-PRS-AoA

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-ElevationAoA-Result                INTEGER (0..1800)         OPTIONAL,  -- sl-PRS-AoA

...

}

SL-AoA-AdditionalPathList ::= SEQUENCE (SIZE(1..2)) OF SL-AoA-AdditionalPath

SL-AoA-AdditionalPath  ::= SEQUENCE {

sl-AngleQuality                            MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AzimuthAoA-AdditionalPathResult         INTEGER (0..3599)         OPTIONAL,  -- additionalPath-SL-PRS-AoA

sl-ElevationAoA-AdditionalPathResult       INTEGER (0..1800)         OPTIONAL,  -- additionalPath-SL-PRS-AoA

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)          OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

...

}

MeasurementAngleQuality ::= SEQUENCE {

azimuthQuality              INTEGER (0..255),

elevationQuality            INTEGER (0..255)        OPTIONAL

}

SL-AoA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FirstElement | The field is mandatory present in the first SL-AoA-MeasElement in IE SL-AoA-MeasElementPerARP-ID-Rx. Otherwise, it is not present. |

SL-AoA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-AngleQuality This field specifies the angle quality for measurement results reported. Scale factor 0.1 degree.

sl-AoA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-LCS-GCS-Translation This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8].

sl-AzimuthAoA-Result This field specifies the first path result of azimuth angle of arrival. Scale factor 0.1 degree; range 0 to 359.9 degrees.

sl-ElevationAoA-Result This field specifies the first path result of elevation angle of arrival. Scale factor 0.1 degree; range 0 to 180 degrees.

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the sidelink PRS resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-TimeStamp This field specifies the time instance at which the SL-PRS RSRP (if included) measurement is performed.

#### –	End of SLPP-PDU-SL-AoA-Contents

-- ASN1START

END

-- ASN1STOP

## 6.8	SLPP PDU SL-RTT Contents

#### –	SLPP-PDU-SL-RTT-Contents

This ASN.1 segment is the start of the SLPP PDU SL-RTT Contents definitions.

-- ASN1START

-- TAG-SLPP-PDU-SL-RTT-CONTENTS-START

SLPP-PDU-SL-RTT-Contents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

LOS-NLOS-Indicator,

PositioningModes,

SL-TimeStamp,

SL-TimingQuality,

maxNrOfUEs,

ScheduledLocationTimeSupportPerMode,

nrMaxBands

FROM

SLPP-PDU-Definitions

LocationCoordinateTypes,

VelocityTypes

FROM

SLPP-PDU-CommonContents;

-- TAG-SLPP-PDU-SL-RTT-CONTENTS-STOP

-- ASN1STOP

#### –	SL-RTT-RequestCapabilities

-- ASN1START

-- TAG-SL-RTT-REQUESTCAPABILITIES-START

SL-RTT-RequestCapabilities ::= SEQUENCE {

...

}

-- TAG-SL-RTT-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	SL-RTT-ProvideCapabilities

The IE SL-RTT-ProvideCapabilities is used to indicate the support of SL-RTT and to provide SL-RTT positioning capabilities.

-- ASN1START

-- TAG-SL-RTT-PROVIDECAPABILITIES-START

SL-RTT-ProvideCapabilities ::= SEQUENCE {

positioningModes                     PositioningModes,

tenMsUnitResponseTime                PositioningModes                            OPTIONAL,

periodicalReporting                  PositioningModes                            OPTIONAL,

scheduledLocationRequestSupported    ScheduledLocationTimeSupportPerMode         OPTIONAL,

locationCoordinateTypes              LocationCoordinateTypes                     OPTIONAL,

velocityTypes                        VelocityTypes                               OPTIONAL,

sl-RTT-CapabilityBandList            SEQUENCE (SIZE (1..nrMaxBands)) OF SL-RTT-CapabilityPerBand,

...

}

SL-RTT-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-7c SL PRS measurement for UE Rx–Tx time difference without Tx time stamp

sl-PRS-RxTxTimeDiffWithoutTxTimeStamp ENUMERATED {n1,n2,n3,n4}                        OPTIONAL,

--R1 41-1-7d SL PRS measurement for UE Rx–Tx time difference with Tx time stamp

sl-PRS-RxTxTimeDiffWithTxTimeStamp    SEQUENCE {

numOfMeasForSameSL-PRS                ENUMERATED {n1,n2,n3,n4},

maxMeasReportingForDiffSL-PRS         ENUMERATED {n1,n2,n3,n4}

}                                                                                     OPTIONAL,

measurementsForMultipleARP-IDs-Rx     ENUMERATED { supported }                        OPTIONAL,

...

}

-- TAG-SL-RTT-PROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-RTT-ProvideCapabilities field descriptions

locationCoordinateTypes This parameter identifies the geographical location coordinate types that a target UE supports for SL-RTT. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE supports SL-RTT measurements for multiple SL-PRS Rx ARP-IDs.

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support periodicalReporting in IE CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-RTT mode(s) supported by the UE.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the UE supports scheduled location requests, i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the UE does not support scheduled location requests.

sl-PRS-RxTxTimeDiffWithoutTxTimeStamp Indicates whether UE supports SL PRS measurement for UE Rx–Tx time difference without Tx time stamp, and is comprised of the following functional components: - Support UE Rx–Tx time difference measurement based on SL PRS; - Support UE Rx–Tx time difference measurement reporting without Tx time stamp. The value indicates the supported maximum number of Rx-Tx measurement reporting for different SL-PRS reception for the same pair of UEs. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand, and at least one of sl-PRS-TxInSharedResourcePool, sl-PRS-TxScheme1InDedicatedResourcePool or sl-PRS-TxScheme2InDedicatedResourcePool.

sl-PRS-RxTxTimeDiffWithTxTimeStamp Indicates whether UE supports SL PRS measurement for UE Rx–Tx time difference with Tx time stamp, and is comprised of the following functional components: - Support UE Rx–Tx time difference measurement based on SL PRS; - Support UE Rx–Tx time difference measurement reporting with Tx time stamp; This field comprises the following sub-fields: - numOfMeasForSameSL-PRS: indicates the reported number of Rx-Tx measurements for the same SL-PRS transmission (or reception) and different SL-PRS reception (or transmission) for the same pair of UEs; - maxMeasReportingForDiffSL-PRS: indicates the supported maximum number of Rx-Tx measurement reporting for different SL-PRS reception for the same pair of UEs. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand, and at least one of sl-PRS-TxInSharedResourcePool, sl-PRS-TxScheme1InDedicatedResourcePool or sl-PRS-TxScheme2InDedicatedResourcePool.

tenMsUnitResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'tenMilliSeconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one value at the bit position means 'tenMilliSeconds' response time unit for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support 'tenMilliSeconds' response time unit in CommonIEsRequestLocationInformation.

velocityTypes This parameter identifies the velocity types that a target UE supports for SL-RTT. TRUE indicates that a velocity type is supported and FALSE that it is not. If this field is absent, velocity reporting is not supported.

#### –	SL-RTT-RequestAssistanceData

-- ASN1START

-- TAG-SL-RTT-REQUESTASSISTANCEDATA-START

SL-RTT-RequestAssistanceData ::= SEQUENCE {

...

}

-- TAG-SL-RTT-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

#### –	SL-RTT-ProvideAssistanceData

-- ASN1START

-- TAG-SL-RTT-PROVIDEASSISTANCEDATA-START

SL-RTT-ProvideAssistanceData ::= SEQUENCE {

...

}

-- TAG-SL-RTT-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

#### –	SL-RTT-RequestLocationInformation

-- ASN1START

-- TAG-SL-RTT-REQUESTLOCATIONINFORMATION-START

SL-RTT-RequestLocationInformation ::= SEQUENCE {

sl-ARP-InfoRequest                    ENUMERATED { true }              OPTIONAL,

sl-LOS-NLOS-IndicatorRequest          ENUMERATED { true }              OPTIONAL,

sl-PRS-RSRP-Request                   ENUMERATED { true }              OPTIONAL,

sl-RSRPP-Request                      ENUMERATED { true }              OPTIONAL,

sl-AdditionalPathsRequest             ENUMERATED { true }              OPTIONAL,

tx-TimeInfoRequest                    ENUMERATED { true }              OPTIONAL,

multipleSL-PRS-RxTxTimeDiffRequest    SEQUENCE {

diffSL-PRS-Receptions                 ENUMERATED { n2, n3, n4 }    OPTIONAL,

diffSL-PRS-Transmissions              ENUMERATED { n2, n3, n4 }    OPTIONAL

}                                                                      OPTIONAL,

measurementsForMultipleARP-IDs-Rx     SEQUENCE {

requestedARP-IDs-Rx                   BIT STRING (SIZE (4))        OPTIONAL

}                                                                      OPTIONAL,

associatedSL-PRS-TxTimeStampRequest   ENUMERATED { true }              OPTIONAL,

...

}

-- TAG-SL-RTT-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

SL-RTT-RequestLocationInformation field descriptions

associatedSL-PRS-TxTimeStampRequest This field, if present, indicates that the UE is requested to provide the associated SL PRS transmission time stamp.

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-RTT measurements for multiple SL-PRS Rx ARP-IDs.

multipleSL-PRS-RxTxTimeDiffRequest This field, if present, indicates that the UE is requested to provide multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and up to N different SL PRS receptions (resp. transmissions) for the same pair of UE(s). Fields are as follows: - diffSL-PRS-Receptions indicates that the UE is requested to provide multiple Rx-Tx measurements for the same SL PRS transmission and up to N different SL PRS receptions. - diffSL-PRS-Transmissions indicates that the UE is requested to provide multiple Rx-Tx measurements for the same SL PRS transmission and up to N different SL PRS transmissions.

requestedARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-RTT measurements for indicated SL-PRS Rx ARP-IDs. Bit 1 in the bit string indicates ARP-ID = 1, bit 2 indicates ARP-ID = 2, and so on.

sl-AdditionalPathsRequest This field, if present, indicates that the UE is requested to provide sl-RTT-AdditionalPathList.

sl-ARP-InfoRequest This field, if present, indicates that the UE is requested to provide sl-POS-ARP-ID-Rx.

sl-RSRPP-Request This field, if present, indicates that the UE is requested to provide sl-RSRPP.

sl-LOS-NLOS-IndicatorRequest This field, if present, indicates that the UE is requested to provide the estimated los-NLOS-Indicator.

sl-PRS-RSRP-Request This field, if present, indicates that the UE is requested to provide sl-PRS-RSRP-Result.

#### –	SL-RTT-ProvideLocationInformation

-- ASN1START

-- TAG-SL-RTT-PROVIDELOCATIONINFORMATION-START

SL-RTT-ProvideLocationInformation ::= SEQUENCE {

sl-RTT-SignalMeasurementInformation   SL-RTT-SignalMeasurementInformation    OPTIONAL,

sl-RTT-Error                          SL-RTT-LocationInformationError        OPTIONAL,

...

}

SL-RTT-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-RTT-MeasElement

SL-RTT-SignalMeasurementInformation ::= SEQUENCE {

sl-RTT-MeasList                         SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-RTT-MeasElementPerARP-ID-Rx,

...

}

SL-RTT-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-RxTxTimeDiffMeasResult         SL-PRS-RxTxTimeDiffMeasResult,

...

}

SL-PRS-RxTxTimeDiffMeasResult ::= CHOICE {

single-SL-PRS-RxTxTimeDiff        SL-PRS-RxTxTimeDiffResult,

multiple-SL-PRS-RxTxTimeDiff          SEQUENCE {

sameSL-PRS-TxAndDiffSL-PRS-Rx         SEQUENCE (SIZE (2..4)) OF SL-PRS-RxTxTimeDiffResult    OPTIONAL,

sameSL-PRS-RxAndDiffSL-PRS-Tx         SEQUENCE (SIZE (2..4)) OF SL-PRS-RxTxTimeDiffResult    OPTIONAL

},

...

}

SL-PRS-RxTxTimeDiffResult ::= SEQUENCE {

sl-PRS-ResourceId             INTEGER (0..16)               OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RxTxTimeDiffResult         CHOICE {

k0                                INTEGER (0..1970049),

k1                                INTEGER (0..985025),

k2                                INTEGER (0..492513),

k3                                INTEGER (0..246257),

k4                                INTEGER (0..123129),

k5                                INTEGER (0..61565)

}                                                           OPTIONAL,  -- sl-PRS-RxTxTimeDiff

sl-PRS-RSRP-Result            INTEGER (0..126)              OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result           INTEGER (0..126)              OPTIONAL,  -- sl-PRS-RSRPP

sl-RTT-AdditionalPathList     SL-RTT-AdditionalPathList     OPTIONAL,

sl-TimeStamp                  SL-TimeStamp                  OPTIONAL,  -- sl-Timestamp

sl-TimingQuality              SL-TimingQuality              OPTIONAL,  -- sl-TimingQuality

tx-TimeInfo                   SL-TimeStamp                  OPTIONAL,  -- tx-Time-Info

...

}

SL-RTT-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-RTT-AdditionalPath

SL-RTT-AdditionalPath  ::= SEQUENCE {

sl-PRS-RxTxTimeDiffAdditionalPathResult    CHOICE {

k0                                         INTEGER (0..8191),

k1                                         INTEGER (0..4095),

k2                                         INTEGER (0..2047),

k3                                         INTEGER (0..1023),

k4                                         INTEGER (0..511),

k5                                         INTEGER (0..255)

}                                                                   OPTIONAL,  -- additionalPath-SL-PRS-Rx-Tx-TimeDiff

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)         OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                           SL-TimingQuality         OPTIONAL,  -- sl-TimingQuality

...

}

SL-RTT-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-RTT-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FirstElement | The field is mandatory present in the first SL-RTT-MeasElement in IE SL-RTT-MeasElementPerARP-ID-Rx. Otherwise, it is not present. |

SL-RTT-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the sidelink PRS resource ID used for SL positioning measurements.

sl-PRS-RxTxTimeDiffResult This field specifies SL Rx-Tx time difference measurement based on first path of arrival. The mapping of the field is defined in TS 38.133 [13].

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-RTT-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-RTT-Error This field provides SL-RTT error reasons.

sl-TimeStamp This field specifies the time instance at which the SL Rx-Tx time difference and SL-PRS RSRP (if included) measurement is performed.

tx-TimeInfo This field specifies the transmission timestamp of the SL-PRS, referred to as TUE-TX in clause 5.1.40 in TS 38.215 [16].

#### –	End of SLPP-PDU-SL-RTT-Contents

-- ASN1START

END

-- ASN1STOP

## 6.9	SLPP PDU SL-TDOA Contents

#### –	SLPP-PDU-SL-TDOA-Contents

This ASN.1 segment is the start of the SLPP PDU SL-TDOA Contents definitions.

-- ASN1START

-- TAG-SLPP-PDU-SL-TDOA-CONTENTS-START

SLPP-PDU-SL-TDOA-Contents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

LOS-NLOS-Indicator,

PositioningModes,

SL-RTD-Info,

SL-TimeStamp,

SL-TimingQuality,

maxNrOfUEs,

ScheduledLocationTimeSupportPerMode,

nrMaxBands

FROM

SLPP-PDU-Definitions

LocationCoordinateTypes,

VelocityTypes

FROM

SLPP-PDU-CommonContents;

-- TAG-SLPP-PDU-SL-TDOA-CONTENTS-STOP

-- ASN1STOP

#### –	SL-TDOA-RequestCapabilities

-- ASN1START

-- TAG-SL-TDOA-REQUESTCAPABILITIES-START

SL-TDOA-RequestCapabilities ::= SEQUENCE {

...

}

-- TAG-SL-TDOA-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	SL-TDOA-ProvideCapabilities

The IE SL-TDOA-ProvideCapabilities is used to indicate the support of SL-TDOA and to provide SL-TDOA positioning capabilities.

-- ASN1START

-- TAG-SL-TDOA-PROVIDECAPABILITIES-START

SL-TDOA-ProvideCapabilities ::= SEQUENCE {

positioningModes                     PositioningModes,

tenMsUnitResponseTime                PositioningModes                            OPTIONAL,

periodicalReporting                  PositioningModes                            OPTIONAL,

scheduledLocationRequestSupported    ScheduledLocationTimeSupportPerMode         OPTIONAL,

locationCoordinateTypes              LocationCoordinateTypes                     OPTIONAL,

velocityTypes                        VelocityTypes                               OPTIONAL,

sl-TDOA-CapabilityBandList           SEQUENCE (SIZE (1..nrMaxBands)) OF SL-TDOA-CapabilityPerBand,

...

}

SL-TDOA-CapabilityPerBand ::=      SEQUENCE {

--R1 41-1-7a    SL PRS measurement for SL-RSTD

sl-PRS-RSTD-Meas                   ENUMERATED {n1,n2,n3,n4}                      OPTIONAL,

measurementsForMultipleARP-IDs-Rx  ENUMERATED { supported }                      OPTIONAL,

...

}

-- TAG-SL-TDOA-PROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-TDOA-ProvideCapabilities field descriptions

locationCoordinateTypes This parameter identifies the geographical location coordinate types that a target UE supports for SL-TDOA. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE supports SL-TDOA measurements for multiple SL-PRS Rx ARP-IDs.

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support periodicalReporting in IE CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-TDOA mode(s) supported by the UE.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the UE supports scheduled location requests, i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the UE does not support scheduled location requests.

sl-PRS-RSTD-Meas Indicates whether UE supports SL PRS measurement for SL-RSTD, and is comprised of the following functional components: - Support SL RSTD measurement based on SL-PRS; - Support SL RSTD measurement reporting; The value indicates the supported maximum number of SL RSTD measurement reporting for different SL-PRS reception for the same pair of UEs. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand.

tenMsUnitResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'tenMilliSeconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one value at the bit position means 'tenMilliSeconds' response time unit for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support 'tenMilliSeconds' response time unit in CommonIEsRequestLocationInformation.

velocityTypes This parameter identifies the velocity types that a target UE supports for SL-TDOA. TRUE indicates that a velocity type is supported and FALSE that it is not. If this field is absent, velocity reporting is not supported.

#### –	SL-TDOA-RequestAssistanceData

-- ASN1START

-- TAG-SL-TDOA-REQUESTASSISTANCEDATA-START

SL-TDOA-RequestAssistanceData ::= SEQUENCE {

sl-RTD-InfoRequest                 ENUMERATED { true}                    OPTIONAL,

...

}

-- TAG-SL-TDOA-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

SL-TDOA-RequestAssistanceData field descriptions

sl-RTD-InfoRequest This field indicates that the SL RTD information is requested.

#### –	SL-TDOA-ProvideAssistanceData

-- ASN1START

-- TAG-SL-TDOA-PROVIDEASSISTANCEDATA-START

SL-TDOA-ProvideAssistanceData ::= SEQUENCE {

sl-RTD-Info                              SL-RTD-Info                                                   OPTIONAL,

sl-TDOA-Error                            SL-TDOA-AssistanceDataError                                   OPTIONAL,

...

}

SL-TDOA-AssistanceDataError ::= ENUMERATED { undefined, assistanceDataNotAvailable, ... }

-- TAG-SL-TDOA-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

SL-TDOA-ProvideAssistanceData field descriptions

sl-RTD-Info This field provides synchronization information of SL Anchor UEs.

sl-TDOA-Error This field provides SL-TDOA error reasons.

#### –	SL-TDOA-RequestLocationInformation

-- ASN1START

-- TAG-SL-TDOA-REQUESTLOCATIONINFORMATION-START

SL-TDOA-RequestLocationInformation ::= SEQUENCE {

sl-ARP-InfoRequest                     ENUMERATED { true }        OPTIONAL,

sl-LOS-NLOS-IndicatorRequest           ENUMERATED { true }        OPTIONAL,

sl-PRS-RSRP-Request                    ENUMERATED { true }        OPTIONAL,

sl-RSRPP-Request                       ENUMERATED { true }        OPTIONAL,

sl-AdditionalPathsRequest              ENUMERATED { true }        OPTIONAL,

measurementsForMultipleARP-IDs-Rx      SEQUENCE {

requestedARP-IDs-Rx                    BIT STRING (SIZE (4))  OPTIONAL

}                                                                 OPTIONAL,

...

}

-- TAG-SL-TDOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

SL-TDOA-RequestLocationInformation field descriptions

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TDOA measurements for multiple SL-PRS Rx ARP-IDs.

requestedARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TDOA measurements for indicated SL-PRS Rx ARP-IDs. Bit 1 in the bit string indicates ARP-ID = 1, bit 2 indicates ARP-ID = 2, and so on.

sl-AdditionalPathsRequest This field, if present, indicates that the UE is requested to provide sl-TDOA-AdditionalPathList.

sl-ARP-InfoRequest This field, if present, indicates that the UE is requested to provide sl-POS-ARP-ID-Rx.

sl-LOS-NLOS-IndicatorRequest This field, if present, indicates that the UE is requested to provide the estimated los-NLOS-Indicator.

sl-PRS-RSRP-Request This field, if present, indicates that the UE is requested to provide sl-PRS-RSRP-Result.

sl-RSRPP-Request This field, if present, indicates that the UE is requested to provide sl-RSRPP.

#### –	SL-TDOA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-START

SL-TDOA-ProvideLocationInformation ::= SEQUENCE {

sl-RSTD-ReferenceUE-Info               SEQUENCE {

applicationLayerID                     OCTET STRING

}                                                                              OPTIONAL,

sl-TDOA-SignalMeasurementInformation   SL-TDOA-SignalMeasurementInformation    OPTIONAL,

sl-TDOA-Error                          SL-TDOA-LocationInformationError        OPTIONAL,

...

}

SL-TDOA-SignalMeasurementInformation ::= SEQUENCE {

sl-TDOA-MeasList                         SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-TDOA-MeasElementPerARP-ID-Rx,

...

}

SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TDOA-MeasElement

SL-TDOA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-RSTD-Result                        CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                                OPTIONAL,  -- sl-PRS-RSTD

sl-TDOA-AdditionalPathList            SL-TDOA-AdditionalPathList OPTIONAL,

sl-TimeStamp                          SL-TimeStamp               OPTIONAL,  -- sl-Timestamp

sl-TimingQuality                      SL-TimingQuality           OPTIONAL,  -- sl-TimingQuality

...

}

SL-TDOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TDOA-AdditionalPath

SL-TDOA-AdditionalPath  ::= SEQUENCE {

sl-RSTD-AdditionalPathResult          CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                               OPTIONAL,  -- additionalPath-SL-PRS-RSTD

sl-PRS-AdditionalPathRSRPP-Result      INTEGER (0..126)         OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                       SL-TimingQuality         OPTIONAL,  -- sl-TimingQuality

...

}

SL-TDOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FirstElement | The field is mandatory present in the first SL-TDOA-MeasElement in IE SL-TDOA-MeasElementPerARP-ID-Rx. Otherwise, it is not present. |

SL-TDOA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the sidelink PRS resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-TDOA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-RSTD-Result This field specifies the SL-RSTD measurement based on first path of arrival.

sl-RSTD-ReferenceUE-Info This field indicates reference UE information for SL-PRS based RSTD measurement report.

sl-TimeStamp This field specifies the time instance at which the SL RSTD and SL-PRS RSRP (if included) measurement is performed.

#### –	End of SLPP-PDU-SL-TDOA-Contents

-- ASN1START

END

-- ASN1STOP

## 6.10	SLPP PDU SL-TOA Contents

#### –	SLPP-PDU-SL-TOA-Contents

This ASN.1 segment is the start of the SLPP PDU SL-TOA Contents definitions.

-- ASN1START

-- TAG-SLPP-PDU-SL-TOA-CONTENTS-START

SLPP-PDU-SL-TOA-Contents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

LOS-NLOS-Indicator,

PositioningModes,

SL-RTD-Info,

SL-TimeStamp,

SL-TimingQuality,

maxNrOfUEs,

ScheduledLocationTimeSupportPerMode,

nrMaxBands

FROM

SLPP-PDU-Definitions

LocationCoordinateTypes,

VelocityTypes

FROM

SLPP-PDU-CommonContents;

-- TAG-SLPP-PDU-SL-TOA-CONTENTS-STOP

-- ASN1STOP

#### –	SL-TOA-RequestCapabilities

-- ASN1START

-- TAG-SL-TOA-REQUESTCAPABILITIES-START

SL-TOA-RequestCapabilities ::= SEQUENCE {

...

}

-- TAG-SL-TOA-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	SL-TOA-ProvideCapabilities

The IE SL-TOA-ProvideCapabilities is used to indicate the support of SL-TOA and to provide SL-TOA positioning capabilities.

-- ASN1START

-- TAG-SL-TOA-PROVIDECAPABILITIES-START

SL-TOA-ProvideCapabilities ::= SEQUENCE {

positioningModes                     PositioningModes,

tenMsUnitResponseTime                PositioningModes                                  OPTIONAL,

periodicalReporting                  PositioningModes                                  OPTIONAL,

scheduledLocationRequestSupported    ScheduledLocationTimeSupportPerMode               OPTIONAL,

locationCoordinateTypes              LocationCoordinateTypes                           OPTIONAL,

velocityTypes                        VelocityTypes                                     OPTIONAL,

sl-TOA-CapabilityBandList            SEQUENCE (SIZE (1..nrMaxBands)) OF SL-TOA-CapabilityPerBand,

...

}

SL-TOA-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-7b SL PRS measurement for SL RTOA

sl-RTOA-Meas                       ENUMERATED {n1,n2,n3,n4}                      OPTIONAL,

measurementsForMultipleARP-IDs-Rx  ENUMERATED { supported }                      OPTIONAL,

...

}

-- TAG-SL-TOA-PROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-TOA-ProvideCapabilities field descriptions

locationCoordinateTypes This parameter identifies the geographical location coordinate types that a target UE supports for SL-TOA. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE supports SL-TOA measurements for multiple SL-PRS Rx ARP-IDs.

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support periodicalReporting in IE CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-TOA mode(s) supported by the UE.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the UE supports scheduled location requests, i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the UE does not support scheduled location requests.

sl-RTOA-Meas Indicates whether UE supports SL PRS measurement for SL-RTOA, and is comprised of the following functional components: - Support SL RTOA measurement based on SL-PRS; - Support SL RTOA measurement reporting. The value indicates the supported maximum number of SL RTOA measurement reporting for different SL-PRS reception for the same pair of UEs. UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand.

tenMsUnitResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'tenMilliSeconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one value at the bit position means 'tenMilliSeconds' response time unit for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support 'tenMilliSeconds' response time unit in CommonIEsRequestLocationInformation.

velocityTypes This parameter identifies the velocity types that a target UE supports for SL-TOA. TRUE indicates that a velocity type is supported and FALSE that it is not. If this field is absent, velocity reporting is not supported.

#### –	SL-TOA-RequestAssistanceData

-- ASN1START

-- TAG-SL-TOA-REQUESTASSISTANCEDATA-START

SL-TOA-RequestAssistanceData ::= SEQUENCE {

sl-RTD-InfoRequest               ENUMERATED { true}                    OPTIONAL,

...

}

-- TAG-SL-TOA-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

SL-TOA-RequestAssistanceData field descriptions

sl-RTD-InfoRequest This field indicates that the SL RTD information is requested.

#### –	SL-TOA-ProvideAssistanceData

-- ASN1START

-- TAG-SL-TOA-PROVIDEASSISTANCEDATA-START

SL-TOA-ProvideAssistanceData ::= SEQUENCE {

sl-RTD-Info                             SL-RTD-Info                                                   OPTIONAL,

sl-TOA-Error                            SL-TOA-AssistanceDataError                                    OPTIONAL,

...

}

SL-TOA-AssistanceDataError ::= ENUMERATED { undefined, assistanceDataNotAvailable, ... }

-- TAG-SL-TOA-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

SL-TOA-ProvideAssistanceData field descriptions

sl-RTD-Info This field provides synchronization information of SL Anchor UEs.

sl-TOA-Error This field provides SL-TOA error reasons.

#### –	SL-TOA-RequestLocationInformation

-- ASN1START

-- TAG-SL-TOA-REQUESTLOCATIONINFORMATION-START

SL-TOA-RequestLocationInformation ::= SEQUENCE {

sl-ARP-InfoRequest                    ENUMERATED { true }        OPTIONAL,

sl-LOS-NLOS-IndicatorRequest          ENUMERATED { true }        OPTIONAL,

sl-PRS-RSRP-Request                   ENUMERATED { true }        OPTIONAL,

sl-RSRPP-Request                      ENUMERATED { true }        OPTIONAL,

sl-AdditionalPathsRequest             ENUMERATED { true }        OPTIONAL,

measurementsForMultipleARP-IDs-Rx     SEQUENCE {

requestedARP-IDs-Rx                   BIT STRING (SIZE (4))  OPTIONAL

}                                                                OPTIONAL,

...

}

-- TAG-SL-TOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

SL-TOA-RequestLocationInformation field descriptions

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TOA measurements for multiple SL-PRS Rx ARP-IDs.

requestedARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TOA measurements for indicated SL-PRS Rx ARP-IDs. Bit 1 in the bit string indicates ARP-ID = 1, bit 2 indicates ARP-ID = 2, and so on.

sl-AdditionalPathsRequest This field, if present, indicates that the UE is requested to provide sl-TOA-AdditionalPathList.

sl-ARP-InfoRequest This field, if present, indicates that the UE is requested to provide sl-POS-ARP-ID-Rx.

sl-LOS-NLOS-IndicatorRequest This field, if present, indicates that the UE is requested to provide the estimated los-NLOS-Indicator.

sl-RSRPP-Request This field, if present, indicates that the UE is requested to provide sl-RSRPP.

sl-PRS-RSRP-Request This field, if present, indicates that the UE is requested to provide sl-PRS-RSRP-Result.

#### –	SL-TOA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-START

SL-TOA-ProvideLocationInformation ::= SEQUENCE {

sl-TOA-SignalMeasurementInformation   SL-TOA-MeasElementPerARP-ID-Rx         OPTIONAL,

sl-TOA-Error                          SL-TOA-LocationInformationError        OPTIONAL,

...

}

SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TOA-MeasElement

SL-TOA-MeasElement ::= SEQUENCE {

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-RTOA-Result                        CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                               OPTIONAL,  -- sl-PRS-RTOA

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TOA-AdditionalPathList             SL-TOA-AdditionalPathList OPTIONAL,

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-TimingQuality                      SL-TimingQuality          OPTIONAL,  -- sl-TimingQuality

...

}

SL-TOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TOA-AdditionalPath

SL-TOA-AdditionalPath  ::= SEQUENCE {

sl-RTOA-AdditionalPathResult               CHOICE {

k0                                         INTEGER (0..16351),

k1                                         INTEGER (0..8176),

k2                                         INTEGER (0..4088),

k3                                         INTEGER (0..2044),

k4                                         INTEGER (0..1022),

k5                                         INTEGER (0..511)

}                                                                OPTIONAL,  -- additionalPath-SL-PRS-RTOA

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)      OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                           SL-TimingQuality      OPTIONAL,  -- sl-TimingQuality

...

}

SL-TOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

SL-TOA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-TOA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the sidelink PRS resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-RTOA-Result This field specifies the SL-RTOA measurement based on first path of arrival.

sl-TimeStamp This field specifies the time instance at which the SL RTOA and SL-PRS RSRP (if included) measurement is performed.

#### –	End of SLPP-PDU-SL-TOA-Contents

-- ASN1START

END

-- ASN1STOP

## 6.11	Information elements related to Discovery Message

This clause specifies information elements that are transferred in Discovery Message for ranging and sidelink positioning, as specified in TS 23.304 [14].

#### –	NR-DiscoveryMessageMetaDataContents

This ASN.1 segment is the start of the NR-DiscoveryMessageMetaDataContents definitions.

-- ASN1START

-- TAG-NR-DISCOVERYMESSAGEMETADATACONTENTS-START

NR-DiscoveryMessageMetaDataContents DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-NR-DISCOVERYMESSAGEMETADATACONTENTS-STOP

-- ASN1STOP

#### –	RSPP-Metadata

The IE RSPP-Metadata includes the UE information included in Discovery Message for ranging and sidelink positioning.

-- ASN1START

-- TAG-RSPP-METADATA-START

RSPP-Metadata ::= SEQUENCE {

ue-RoleList               BIT STRING { sl-anchorUE(0), sl-ServerUE(1), sl-TargetUE(2) } (SIZE (1..8)),

knownLocationAvailable    ENUMERATED {true}                                                      OPTIONAL,

...

}

-- TAG-RSPP-METADATA-STOP

-- ASN1STOP

RSPP-Metadata field descriptions

ue-RoleList This field indicates the UE role associate with the discovery message. This is represented by a bit string, with a one value at the bit position means the particular UE role associate with the discovery message. In the case of solicitation message, this bit string is interpreted as: - bit 0 indicates whether the UE role as an SL Anchor UE is requested or not; - bit 1 indicates whether the UE role as an SL Server UE is requested or not; - bit 2 indicates whether the UE role as an SL Target UE is requested or not. Otherwise, the bit string is interpreted as: - bit 0 indicates whether the UE supports UE role as an SL Anchor UE or not; - bit 1 indicates whether the UE supports UE role as an SL Server UE or not; - bit 2 indicates whether the UE supports UE role as an SL Target UE or not.

knownLocationAvailable This field indicates whether the location of an SL Anchor UE is known or is able to be known, e.g., via Uu based positioning. The field can only be present if the bit 0 of ue-RoleList is set.

#### –	End of NR-DiscoveryMessageMetaDataContents

-- ASN1START

END

-- ASN1STOP