3GPP TSG-RAN WG2 meeting #127bis	R2-2409210

Agenda Item:	2.2

Source: 	ETSI MCC

Title:		Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

Report of 3GPP TSG RAN WG2 meeting #127

Maastricht, Netherlands

19 - 23 August, 2024

Document for: Approval

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

# Contents

Organisation of the meeting	7

Statistics/Executive Summary	7

General	8

1	Opening of the meeting	8

1.1	Call for IPR	8

1.2	Network usage conditions	8

1.3	Other	8

2	General	8

2.1	Approval of the agenda	8

2.2	Approval of the report of the previous meeting	8

2.3	Reporting from other meetings	9

2.4	Instructions	9

2.5	Others	10

3	Incoming liaisons	11

4	EUTRA Rel-17 and earlier	11

4.1	EUTRA corrections Rel-17 and earlier	11

4.2	V2X and Sidelink corrections Rel-15 and earlier	12

4.3	Positioning corrections Rel-16 and earlier	13

5	NR Rel-15 and Rel-16	13

5.1	Common	13

5.1.1	Stage 2 and Organisational	14

5.1.2	User Plane corrections	15

5.1.2.1	MAC	15

5.1.2.2	RLC PDCP SDAP BAP	15

5.1.2.3	Other	15

5.1.3	Control Plane corrections	15

5.1.3.1	NR RRC	15

5.1.3.2	UE capabilities	16

5.1.3.3	Other	18

5.2	NR V2X	18

5.3	NR Positioning Support	18

6	NR Rel-17	19

6.1	Common	19

6.1.1	Stage 2 and Organisational	20

6.1.2	User Plane corrections	20

6.1.3	Control Plane corrections	22

6.1.3.1	NR RRC	22

6.1.3.2	UE capabilities	27

6.1.3.3	Other	28

6.2	NR Sidelink relay	28

6.4	NR positioning enhancements	29

6.6	NR Sidelink enhancements	31

7	Rel-18	33

7.0	Common	33

7.0.1	UE Capabilities	33

7.0.2	Rel-18 corrections	34

7.0.2.1	RACH-less HO	34

7.0.2.2	NR network-controlled repeaters	35

7.0.2.3	NR support for UAV	35

7.0.2.4 	Mobile Terminated Small Data Transmission	36

7.0.2.5	IDC enhancements for NR and MR-DC	36

7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR	37

7.0.2.7	Timing Resiliency and URLLC Enh	37

7.0.2.8	Others	37

7.1	Void	40

7.2	Expanded and improved NR positioning	40

7.2.1	Organizational	40

7.2.2	Stage 2	42

7.2.3	SLPP corrections	44

7.2.4	LPP corrections	46

7.2.5	RRC corrections	46

7.2.6	MAC corrections	49

7.2.7	UE capabilities	52

7.2.8	Corrections to other specifications	52

7.3	Network energy savings for NR	52

7.3.1	Organizational	52

7.3.2	Other	52

7.4	Further NR mobility enhancements	54

7.4.1	Organizational	54

7.4.2	Control plane corrections	54

7.4.3	User plane corrections	64

7.5	XR Enhancements for NR	67

7.5.1	Organizational	67

7.5.2	Control plane corrections	67

7.5.3	User plane corrections	68

7.6	IoT NTN enhancements	71

7.6.1	Organizational	71

7.6.2	Corrections	72

7.7	NR NTN enhancements	75

7.7.1	Organizational	75

7.7.2	Corrections	76

7.8	Void	81

7.9	Enhanced NR Sidelink Relay	81

7.9.1	Organizational	81

7.9.2	Stage 2 corrections	81

7.9.3	RRC corrections	82

7.9.4	SRAP corrections	85

7.9.5	MAC, RLC, and PDCP corrections	85

7.9.6	UE capabilities	87

7.9.7	Idle mode corrections	87

7.10	Void	87

7.11	Enhancements of NR Multicast and Broadcast Services	87

7.11.1	Organizational	87

7.11.2	Corrections	87

7.12	Void	92

7.13	Further enhancement of data collection for SON MDT in NR and EN-DC	92

7.13.1	Organizational	92

7.13.2	Corrections	92

7.14	Enhancement on NR QoE management and optimizations for diverse services	94

7.14.1	Organizational	94

7.14.2	Corrections	94

7.15	NR Sidelink evolution	96

7.15.1	Organizational	97

7.15.2	Corrections	97

7.16	Void	100

7.17	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR	100

7.17.1	Organizational	100

7.17.3	Corrections	100

7.18	Void	103

7.19	Enhanced support of reduced capability NR devices	103

7.19.1	Organizational	103

7.19.2	Other	103

7.20	NR MIMO evolution	104

7.20.1	Organizational	104

7.20.2	Corrections	104

7.21	Further NR coverage enhancements	107

7.21.1	Organizational	107

7.21.2	Other Essential corrections	107

7.22	Void	108

7.23	Void	108

7.24	TEI18	108

7.24.1	TEI proposals by Other Groups	108

7.24.2	TEI proposals by RAN2	111

7.24.2.1	Rx XR	111

7.24.2.2	Other RAN2 TEI-18	111

7.25	R18 Other	114

7.25.1	RAN4 led items	115

7.25.2	RAN1 led items	116

7.25.3	Other	117

8	Rel-19	117

8.0	General	117

8.1	AI/ML for NR air interface	118

8.1.1	Organizational	118

8.1.2	Functionality based LCM	118

8.1.2.1	LCM for NW-sided model for Beam Management use case	119

8.1.2.2	LCM for UE-sided model  for Beam Management use case	119

8.1.2.3	LCM for Positioning use case	122

8.1.3	NW side data collection	123

8.1.4	UE side data collection	126

8.2	Ambient IoT	129

8.2.1	Organizational	129

8.2.2	Functionality aspects	130

8.2.3	A-IoT Paging	134

8.2.4	A-IoT Random Access	137

8.2.5	Topology 2 considerations	143

8.3	AI/ML for Mobility	146

8.3.1	Organizational	146

8.3.2	RRM measurement prediction	147

8.3.2.1	Simulation results for RRM measurement prediction	147

8.3.2.2	Other aspects related to RRM measurement prediction	151

8.3.3	Measurement event predictions	154

8.3.4	RLF/HO failure prediction	155

8.3.4.1	Simulation assumptions and evaluation methodology for RLF failure prediction	155

8.3.4.2	Other aspects related to RLF/HO failure prediction	159

8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	159

8.4.1	Organizational	159

8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	159

8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	162

8.4.4	Procedures for LP-WUS in RRC_CONNECTED	164

8.5	Network Energy Saving Enh.	166

8.5.1	Organizational	166

8.5.2	On-demand SSB SCell operation	166

8.5.3	On-demand SIB1	169

8.5.4	Adaptation of common signal/channel transmissions	173

8.6	Mobility Enhancement Ph4	175

8.6.1	Organizational	175

8.6.2	Inter-CU LTM	175

8.6.3	Measurement event evaluation	182

8.6.4	Measurement reporting	185

8.7	XR Enhancements Ph3	187

8.7.1	Organizational	187

8.7.1.1	Discussion on incoming LSs	188

8.7.2	Multi-modality support	190

8.7.3	RRM measurement gaps/restrictions related enhancements	194

8.7.4	Scheduling enhancements	194

8.7.5	RLC enhancements	197

8.8	NTN for NR Ph3	200

8.8.1	Organizational	200

8.8.2	Downlink coverage enhancements	202

8.8.3	Uplink Capacity/Throughput Enhancement	205

8.8.4	Support of Broadcast service	205

8.8.5	Support of regenerative payload	209

8.8.6	LTE to NR NTN mobility	211

8.9	IoT NTN Ph3	214

8.9.1	Organizational	214

8.9.2	Support of Store & Forward	214

8.9.3	Uplink Capacity Enhancement	217

8.10	SON/MDT Ph4	221

8.10.1	Organizational	221

8.10.2	MRO enhancements for Rel-18 mobility features	221

8.10.3	SON/MDT for Slicing	224

8.10.4	SON/MDT for NTN	224

8.10.5	Leftovers from Rel-18	225

8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)	226

8.11.1	Organizational	226

8.11.2	Random access in SBFD	226

8.11.3	Other aspects	228

8.12	NR MIMO Phase 5	229

8.13	NR sidelink multi-hop relay	229

8.13.1	Organizational	229

8.13.2	Relay discovery and (re)selection	232

8.13.3	Control Plane Procedures	235

8.14	Additional topological enhancements	237

9	Breakout session reports	238

9.1	Session on V2X/SL, R19 NES and MOB	238

9.2	Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS	238

9.3	Session on NR NTN and IoT NTN	238

9.4	Session on positioning and sidelink relay	238

9.5	Session on R18 MBS, R18 QoE and R19 XR	238

9.6	Session on maintenance, SON/MDT and eRedCap	238

Closing of the meeting	238

Annex A:	List of participants	239

Annex B:	List of Tdocs	239

Annex C:	Incoming liaison statements	239

Annex D:	Outgoing liaison statements	243

Annex E:	List of agreed CRs	244

Annex F:	List of email discussions during the meeting	251

Annex G:	Post-meeting email discussions	252

Guidelines for email discussions:	252

Short email discussions, Deadline Aug 29th, 18:00 UTC	253

Long email discussions, for R2-127bis, Deadline Sept. 27th, 10:00 UTC (unless otherwise stated)	257

Extra long email discussions, for R2-128, Deadline Nov. 1st, 10:00 UTC (unless stated otherwise)	258

Annex H:	History	259

# Organisation of the meeting

Meeting:	3GPP TSG RAN2#127

Meeting location:	Maastricht, Netherlands

Duration:	19 - 23.08.2024

Host:	ETSI

TSG RAN WG2 Chair	Diana Pani (InterDigital) (diana.pani@interdigital.com)

TSG RAN WG2 Vice chair:	Kyeongin Jeong (Samsung) (kyeongin.j@samsung.com)

TSG RAN WG2 Vice chair:	Erlin Zeng (CATT) (erlin.zeng@catt.cn)

TSG RAN WG2 MCC Support:	Juha Korhonen (ETSI MCC) (juha.korhonen@etsi.org)

Email reflector:	3GPP_TSG_RAN_WG2@LIST.ETSI.ORG

Technical documents:	ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_127/Docs

Next meetings:	TSG RAN2#127bis	14 - 18.10.2024, Hefei, China

TSG RAN2#128	18 - 22.11.2024, Orlando, USA

# Statistics/Executive Summary

TSG RAN2#127 was a normal face-to-face meeting, with a possibility for one-way (listen only) remote access.

There were 70 numbered email discussions during this meeting.

The topics discussed were:

-	NR Rel-15, Rel-16 and Rel-17 User Plane corrections, UE Capabilities, Rel-18 Common, Network energy savings for NR, XR Enhancements for NR, TEI18, Rel-18 Other, Rel-19 General, AI/ML for NR air interface, Ambient IoT, AI/ML for Mobility - Diana Pani (Chair)

-	NR V2X, NR Sidelink enhancements, Further NR mobility enhancements, NR Sidelink evolution, Network Energy Saving Enh, Mobility Enhancement Ph4  - Kyeongin Jeong (VC)

-	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, NR MIMO evolution, Low-power wake-up signal and receiver for NR, Evolution of NR duplex operation: Sub-band full duplex - Erlin Zeng (VC)

-	IoT NTN enhancements, NR NTN enhancements, NTN for NR Ph3, IoT NTN Ph3 - Sergio Parolari

-	Positioning corrections Rel-16 and earlier, Rel-15 and Rel-16 NR Positioning Support, NR sidelink relay, NR positioning enhancements, Expanded and improved NR positioning, Enhanced NR Sidelink Relay, NR sidelink multi-hop relay - Nathan Tenny

-	Enhancements of NR Multicast and Broadcast Services, Enhancement on NR QoE management and optimizations for diverse services, XR Enhancements Ph3 - Dawid Koziol

-	EUTRA corrections Rel-17 and earlier, NR Rel-15, Rel-16 and Rel-17 Common (except user plane), Further enhancement of data collection for SON MDT in NR and EN-DC, Enhanced support of reduced capability NR devices, Further NR coverage enhancements, SON MDT Ph4 - Mattias Bergström

The statistics from this meeting are:

-	455 f2f participants

-	1622 Tdoc numbers allocated with 1585 available contributions. (See the attached tdoc list)

-	44 incoming liaison statements, out of which 42 were noted. The remaining non-treated liaison will be treated in RAN2#127bis meeting.

-	19 outgoing liaison statements

-	70 at-meeting email discussions

-	39 email approvals/discussions scheduled after the RAN2#127 meeting (30 short, 5 long and 4 extra long, i.e., for the November meeting), see Annex G for details.

Number of CRs submitted: 408. Out of these, 146 were agreed. See Annex E for details.

# General

This meeting was an ordinary meeting and had full decision power, i.e. full decision power to make agreements and approvals according to RAN WG2 terms of reference, without any need to ratify decisions at a later RAN2 or other meeting.

# 1	Opening of the meeting

## 1.1	Call for IPR

The attention of the delegates of this Working Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were hereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (https://www.etsi.org/images/files/IPR/etsi-ipr-form.doc)

NOTE:	IPRs may be declared to the Director-General or Chairman of the SDO, but not to the RAN WG2 Chairman.

## 1.2	Network usage conditions

1/ 	To avoid email system overload, please don’t attach files and documents to emails e.g. for offline email discussions, but instead use files placed on the meeting server instead. Inbox/Drafts folder is used for meeting offline discussions.

## 1.3	Other

In accordance with the Working Procedures it is reaffirmed that: (i) compliance with all applicable antitrust and competition laws is required; (ii) timely submissions of work items in advance of TSG or WG meetings are important to allow for full and fair consideration of such matters; and (iii) the chair will conduct the meeting with strict impartiality and in the interests of 3GPP

Note on (i): In case of question please contact your legal counsel.

Note on (ii): WIDs don’t need to be submitted to the RAN2 meeting and will typically not be discussed here either.

# 2	General

## 2.1	Approval of the agenda

R2-2406201	Agenda for RAN2#127	Chairman	agenda

=>	Approved

## 2.2	Approval of the report of the previous meeting

R2-2406202	RAN2#126 Meeting Report	MCC	report

=>	Approved

## 2.3	Reporting from other meetings

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-17 maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

Rel-18 CR Handling

-	CR editors / Rapporteurs continue to support maintenance related to their respective CR / WI and are required to follow drafting rules

-	Single correction CR per spec coordinated by CR editor/rapporteurs will be agreed per feature for RAN#105

-	CR editors / Rapporteurs are to gather miscellaneous and non-controversial issues, if any, for their respective specification prior to submission deadline.  Other companies are expected to give inputs to these CRs and not have contributions on such issues.

-	Companies should give inputs on editorials and clarifications to the CR editors/rapporteurs and not have individual CRs/contributions on such issues.   Emails to CR editors/rapporteurs should follow the following naming convention when sending emails to rapporteurs:

[Pre_RAN2#127][CR xx.yyy] Clarification CRs

-	The organizational AIs for each WIs are reserved for rapporteurs only.  CR rapporteurs are expected to submit only 1 CR per spec.

-	Companies are expected to submit Tdocs with TP (not CRs).   More specifically, the Tdoc should contain description of open issues/proposal and the proposed corrections/TP in the contribution itself.   Small issues can be included in the tdoc with just short justification, same level of detail as in cover sheet.   DraftCRs can be submitted for straightforward changes instead of Tdoc (i.e. DraftCRs count toward Tdoc limit)

-	RRC ASN.1 changes should be drafted in BC way.

-	Inter-op analysis on Rel-18 CR cover pages in now required for each CR.  Companies are expected to identify inter-op analysis/impact in their tdoc for each proposed changes.   CRs rapporteurs when merging should highlight the changes that have interoperability issues.

Remaining/updated Rel-18 RRC parameters and MAC CEs

-	RRC parameters updates/corrections, including those requested by other groups, e.g. RAN1, are covered by WI-specific RRC CRs.

-	MAC CE parameters updates/corrections, including those requested by other groups, e.g. RAN1, are covered by WI-specific MAC CRs

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features/corrections are handled per WI and only a draft CR per WI is expected and will be merged with the running mega CR

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	Limit of 1 WI/SI  rapporteurs input for WI planning. The work plan is not expected to be updated/submitted every meeting, unless needed. It can include progress of other WG groups in the same Tdoc (i.e. separate Tdocs on other WG agreements are not required).

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Tdoc request/submission for RAN2#127 deadlines:

- Tdoc Submission deadline: August 9th, 2024 1000 UTC

## 2.5	Others

R2-2406203	RAN2 Handbook	MCC	discussion	Late

Withdrawn

R2-2406464	Report on RAN2 Cricket team to take on RAN4	Offline Rapporteur (ZTE)	report

=>	Noted

RAN3 Endorsed CRs to RAN2 specifications:

R2-2407836	Update of stage 2 positioning information	R3 (Nokia, CATT, Ericsson, Huawei, Samsung, Xiaomi, ZTE)	CR	Rel-18	38.305	18.2.0	0173	-	F	NR_pos_enh2-Core

=> Agreed

R2-2407837	Correction on CPA failure states and detection mechanism in stage 2	R3 (Nokia, CATT, CMCC, Qualcomm Incorporated, Huawei, ZTE Corporation, Lenovo)	CR	Rel-18	37.340	18.2.0	0400	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2407838	Correction on SPR retrieval and forwarding	R3 (Samsung, Lenovo, Cybercore, Qualcomm, Nokia, Ericsson, Huawei, ZTE, CATT)	CR	Rel-18	37.340	18.2.0	0401	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2407839	Correction on Complete Candidate Configuration	R3 (Huawei, Ericsson, LG Electronics, Samsung)	CR	Rel-18	37.340	18.2.0	0402	-	F	NR_Mob_enh2-Core

=> Agreed

R2-2407840	Missing procedure description for pre-configured SRS activation	R3 (Qualcomm Incorporated, Ericsson, Nokia, Xiaomi, Huawei, CATT)	CR	Rel-18	38.305	18.2.0	0174	-	F	NR_pos_enh2-Core

=> Agreed

R2-2407841	Correction on MIMO with 2TAs in LTM	R3 (Huawei, China Unicom, CMCC, Nokia, LG Electronics, CATT, Ericsson)	CR	Rel-18	38.300	18.2.0	0901	-	F	NR_Mob_enh2-Core

=> Agreed

R2-2407842	Further miscellaneous corrections to QMC procedure	R3 (Huawei, China Unicom, Lenovo, Ericsson, Nokia, Samsung, ZTE)	CR	Rel-18	37.340	18.2.0	0403	-	F	NR_QoE_enh-Core

=> Revised in R2-2407867

R2-2407867	Further miscellaneous corrections to QMC procedure	R3 (Huawei, China Unicom, Lenovo, Ericsson, Nokia, Samsung, ZTE)	CR	Rel-18	37.340	18.2.0	0403	1	F	NR_QoE_enh-Core

=> Agreed

# 3	Incoming liaisons

Note: LSs are moved to the respective agenda items if any.

R2-2406204	LS on RAN4 vs RAN2 Cricket Match (R4-2410764; contact: Nokia)	RAN4	LS in	Rel-19	TEI19	To:RAN2

=> Noted

R2-2406234	LS on Avoiding Cross-TSG TEI (RP-241686; contact: NEC)	RAN	LS in	To:CT, CT1, CT3, CT4, CT6, TSG SA, SA1, SA2, SA3, SA4, SA5, SA6	Cc:RAN1, RAN2, RAN3, RAN4, RAN5

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except V2X and Sidelink WIs and Positioning WIs, which are addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

scg-State

R2-2406272	Correction on the field of scg-State	Huawei, HiSilicon, Qualcomm Incorporated	CR	Rel-17	36.331	17.9.0	5034	-	F	LTE_NR_DC_enh2-Core

R2-2406273	Correction on the field of scg-State	Huawei, HiSilicon, Qualcomm Incorporated	CR	Rel-18	36.331	18.2.0	5035	-	A	LTE_NR_DC_enh2-Core

R2-2406274	Correction on the field of scg-State	Huawei, HiSilicon, Qualcomm Incorporated	CR	Rel-17	38.331	17.9.0	4864	-	F	LTE_NR_DC_enh2-Core

R2-2406275	Correction on the field of scg-State	Huawei, HiSilicon, Qualcomm Incorporated	CR	Rel-18	38.331	18.2.0	4865	-	A	LTE_NR_DC_enh2-Core

All agreed

Extended wait time

R2-2406631	extendedWaitTime correction	Peraton Labs, CISA ECD, AT&T, Verizon	CR	Rel-18	36.331	18.2.0	5041	-	F	NB_IOT-Core

-	Lenovo, MediaTek and Huawei think that this was intentional and the extended wait time is mandatory in the reject message.

Not pursued

MBMS

R2-2406646	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU	CR	Rel-14	36.331	14.17.0	5042	-	F	MBMS_LTE_enh2-Core

R2-2406647	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU	CR	Rel-15	36.331	15.22.0	5043	-	A	MBMS_LTE_enh2-Core

R2-2406648	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU	CR	Rel-16	36.331	16.16.0	5044	-	A	MBMS_LTE_enh2-Core

R2-2406649	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU	CR	Rel-17	36.331	17.9.0	5045	-	A	MBMS_LTE_enh2-Core

R2-2407210	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU	CR	Rel-18	36.331	18.2.0	5046	1	A	MBMS_LTE_enh2-Core	R2-2406650

-	Ericsson agrees that something is wrong but are not sure if this is the best way, perhaps one can add a clarification/correction. Qualcomm is OK with that approach.

-	Ericsson wonders if we can do this from a later release.

[AT127][601][MBMS] Correction to MIB-MBMS systemFrameNumber (Qualcomm)

Scope:

    - Discuss how to polish the current word rather than just removing and discuss which release this should be done from. Produce agreeable CRs

Intended outcome:

    - Agreed CR in R2-2407632 (Qualcomm)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2407796	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson	CR	Rel-14	36.331	14.17.0	5042	1	F	MBMS_LTE_enh2-Core

R2-2407797	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson	CR	Rel-15	36.331	15.22.0	5043	1	A	MBMS_LTE_enh2-Core

R2-2407798	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson	CR	Rel-16	36.331	16.16.0	5044	1	A	MBMS_LTE_enh2-Core

R2-2407799	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson	CR	Rel-17	36.331	17.19.0	5045	1	A	MBMS_LTE_enh2-Core

R2-2407632	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson	CR	Rel-18	36.331	18.2.0	5046	2	A	MBMS_LTE_enh2-Core

The 5 above are agreed

Old revisions, withdrawn

R2-2406650	Correction to MIB-MBMS systemFrameNumber field description	Qualcomm Incorporated, Samsung, ABS, SJTU	CR	Rel-18	36.331	18.2.0	5046	-	A	MBMS_LTE_enh2-Core	Revised

## 4.2	V2X and Sidelink corrections Rel-15 and earlier

REL-15 and Earlier WIs related to V2x and Sidelink are in scope but not listed explicitly (long list).

This Agenda Item is treated in the V2X and Sidelink Breakout session

Tdoc Limitation: 1 tdocs

## 4.3	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

Tdoc Limitation: 1 tdoc

R2-2406288	Correction on SBAS and GNSS ID for posSIB-r15	Huawei, HiSIlicon	CR	Rel-15	36.331	15.22.0	5036	-	F	LCS_LTE_acc_enh-Core

- Not pursued

R2-2406289	Correction on SBAS and GNSS ID for posSIB-r16	Huawei, HiSIlicon	CR	Rel-16	36.331	16.16.0	5037	-	A	LCS_LTE_acc_enh-Core

- Not pursued

R2-2406290	Correction on SBAS and GNSS ID for posSIB-r17	Huawei, HiSIlicon	CR	Rel-17	36.331	17.9.0	5038	-	A	LCS_LTE_acc_enh-Core

- Not pursued

R2-2406291	Correction on SBAS and GNSS ID for posSIB-r18	Huawei, HiSIlicon	CR	Rel-18	36.331	18.2.0	5039	-	A	LCS_LTE_acc_enh-Core

- Not pursued

Discussion:

Lenovo think the changes as such are OK, but there may be some other fields that are Need OP without procedural descriptions and maybe should be Need OR.  Ericsson think the reference to 36.355/37.355 is enough to clarify the behaviour on absence, and they think we should not take the CR going back to Rel-15 for fear of opening a Pandora’s box.

Intel agree with Ericsson and think this CR relates to how the CN sets the parameters; they see nothing broken if we leave it open and assume the CN will do the right thing.

Huawei think it is not just network behaviour and the UE should be able to know when it does not receive a correct configuration.

MediaTek think it is useful to the UE to know what configurations are not expected.

Intel think we should have a high bar for Rel-15 changes, although they agree that the change is correct.  ZTE agree with Intel that we should not start from Rel-15; they recall that we started from Rel-17 in NR and we could do the same here.

Qualcomm think either we go from Rel-15 or Rel-18.  Huawei think it is a little strange if we have different need codes in different releases.

Lenovo think we either go from Rel-15 or do nothing and rely on network implementation.

ZTE would prefer to do nothing since it is a network implementation issue.

Nokia think we could handle it in the field description.  Huawei think the current field should have a description of UE behaviour on absence to fulfil the Need OP.

Agreement:

RAN2 understand that the UE may assume the network will set the sbas-ID if and only if the gnss-ID is set to sbas.

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items NOTE: some agenda items have additional Tdoc limits.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474😉

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997;)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

MPS Priority access

R2-2406634	Alignment of mps-PriorityAccess cause in RRC Resume	Peraton Labs, CISA ECD, Verizon	CR	Rel-18	38.300	18.2.0	0882	-	F	NR_newRAT-Core

-	CATT wonders if we should fix this in earlier releases? Samsung thinks that we should change from R15 if we should fix this, but think it can be merged to rapporteur CR.

-	ZTE thinks that an impact analysis is needed and that summar of changes should be softened.

[AT127][602][Maint] mps-PriorityAccess cause in RRC resume (Peraton Labs)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2407633 (Peraton Labs)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2407775	Alignment of mps-PriorityAccess cause in RRC Resume	Peraton Labs	CR	Rel-15	38.300	15.17.0	0898	-	F	NR_newRAT-Core

R2-2407776	Alignment of mps-PriorityAccess cause in RRC Resume	Peraton Labs	CR	Rel-16	38.300	16.16.0	0899	-	A	NR_newRAT-Core

R2-2407777	Alignment of mps-PriorityAccess cause in RRC Resume	Peraton Labs	CR	Rel-17	38.300	17.9.0	0900	-	A	NR_newRAT-Core

R2-2407633	Alignment of mps-PriorityAccess cause in RRC Resume	Peraton Labs	CR	Rel-18	38.300	18.2.0	0882	1	A	NR_newRAT-Core

All 4 agreed

Transport channels

R2-2407281	Correction on Transport Channels	Philips International B.V.	CR	Rel-15	38.300	15.17.0	0889	-	F	NR_newRAT-Core

R2-2407282	Correction on Transport Channels 	Philips International B.V.	CR	Rel-16	38.300	16.16.0	0890	-	F	NR_newRAT-Core

R2-2407284	Correction on Transport Channels	Philips International B.V.	CR	Rel-17	38.300	17.9.0	0891	-	A	NR_newRAT-Core

R2-2407288	Correction on Transport Channels	Philips International B.V.	CR	Rel-18	38.300	18.2.0	0892	-	A	NR_newRAT-Core

All agreed except the R16 CR which should be cat A. Use R2-2407634 for the update of the R16 CR.

### 5.1.2	User Plane corrections

User Plane corrections will be handled in the User Plane break out session

#### 5.1.2.1	MAC

R2-2407431	Clarification on Rel-16 BFR and Rel-17 BFR	ZTE Corporation	discussion	Rel-16	NR_eMIMO-Core, NR_FeMIMO-Core

Proposal 1: RAN2 clarifies that the sr-ProhibitTimer for the SR Configuration of BFR shall be stopped if all pending SRs for the SR configuration are canceled.

-	LG doesn’t agree with ZTE

-	MEdiatek doesn’t think this is a problem as this is a Rel16.

-	Lenovo, Nokia, Ericsson also don’t think there is a problem

=>	Not supported

Proposal 2: For a BFR triggered by a BFD-RS set of a DL BWP that is deactivated, The BFR triggered by a BFD-RS set shall be cancelled if the candidate beam RS information for the BFD-RS set is contained in the enhanced BFR MAC CE.

-	Vivo thinks we should let it to UE implementation and leave the specification unchanged for Rel-17.  Qualcomm also doesn’t to have a NBC change and we can leave it to UE implantation.

Proposal 3: RAN2 confirms that, for a BFR triggered by a BFD-RS set of a DL BWP that is deactivated, the candidate beam RS selected by UE, if any, in the enhanced BFR MAC CE is from the candidate beam RS list of the current active DL BWP.

=>	Noted

Agreements:

-	RAN2 assumes that the UE reports a suitable beam information from the current active DL BWP.  Not specification change.

#### 5.1.2.2	RLC PDCP SDAP BAP

#### 5.1.2.3	Other

User plane related corrections that should be handled in User plane break out session.

### 5.1.3	Control Plane corrections

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

HO and SRB2

R2-2406336	Preconditions for MCG reconfiguration with sync	MediaTek Inc.	discussion	Rel-15	NR_newRAT-Core

R2-2406350	Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration	NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung	CR	Rel-15	38.331	15.26.0	4870	-	F	NR_newRAT-Core

R2-2406351	Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration	NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung	CR	Rel-16	38.331	16.17.0	4871	-	A	NR_newRAT-Core

R2-2406353	Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration	NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung	CR	Rel-17	38.331	17.9.0	4872	-	A	NR_newRAT-Core

R2-2406354	Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration	NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung	CR	Rel-18	38.331	18.2.0	4873	-	A	NR_newRAT-Core

The 4 above are agreed.

IDLE state changes

R2-2406799	Discussion on going to IDLE triggered by inter-RAT cell selecton or reselection	vivo	discussion	Rel-16

R2-2406800	Correction on going to IDLE triggered by inter-RAT cell selection or reselection	vivo	CR	Rel-16	38.331	16.17.0	4888	-	F	NR_newRAT-Core

R2-2406801	Correction on going to IDLE triggered by inter-RAT cell selection	vivo	CR	Rel-16	36.331	16.16.0	5047	-	F	NR_newRAT-Core

-	Samsung does not agree with this and think that it works already today. CATT thinks that a note in minutes is sufficient.

RAN2 understands that a UE implementation may go from NR CONNECTED to LTE IDLE directly, according to the proposal, but it may also go via NR IDLE, and this can be left to implementation.

SRS switching

R2-2406841	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-15	38.331	15.26.0	4893	-	F	NR_newRAT-Core

R2-2406842	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-16	38.331	16.17.0	4894	-	A	NR_newRAT-Core, TEI16

R2-2406843	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-17	38.331	17.9.0	4895	-	A	NR_newRAT-Core, TEI17

R2-2406844	Correction on IE SRS-CarrierSwitching	CATT	CR	Rel-18	38.331	18.2.0	4896	-	A	NR_newRAT-Core, TEI18

-	Ericsson and ZTE thinks there are many errors in this field description and they will send an LS. CATT wants to clarify this here now since it will help RAN1 discussions. Vivo agrees with CATTs intention. Nokia thinks something is wrong in the field description and want to fix it now. Huawei think we can remove something and refer to RAN1 specs instead.

RAN2 understands that the field description for typeA and typeB in SRS-CarrierSwitching are wrong and we will wait for input from RAN1.

Setup release

R2-2407083	Generic procedure text for SetupRelease	Ericsson	CR	Rel-15	38.331	15.26.0	4918	-	F	NR_newRAT-Core

R2-2407084	Generic procedure text for SetupRelease	Ericsson	CR	Rel-16	38.331	16.17.0	4919	-	A	NR_newRAT-Core, TEI16

R2-2407085	Generic procedure text for SetupRelease	Ericsson	CR	Rel-17	38.331	17.9.0	4920	-	A	NR_newRAT-Core, TEI17

R2-2407086	Generic procedure text for SetupRelease	Ericsson	CR	Rel-18	38.331	18.2.0	4921	-	A	NR_newRAT-Core, TEI18

Moved from 6.1.3.1

-	MediaTek thinks its clear from 5.1.2 already on how setup release is handled.

The second change (to A.3.8) is agreed and merged into R18 RRC rapporteur CR.

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

Parallel TX

R2-2407069	Parallel Tx capability discussion	Ericsson, Qualcomm Incorporated	discussion

- Huawei thinks we don’t need to differentiate. Nokia agrees with the proposal. ZTE also agrees with the proposal. Apple thinks that we should go with this proposal.

[AT127][603][Maint] Parallel Tx capability (Ericsson)

Scope:

    - Produce agreeable CRs, if possible.

Intended outcome:

    - Agreed CRs in R2-2407635 and R2-2407636 (Ericsson)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email, if possible. Otherwise we can try on Friday morning.

After offline:

Limit the following capabilities to NR SA CA:

- •              parallelTxSRS-PUCCH-PUSCH

- •              parallelTxPRACH-SRS-PUCCH-PUSCH

- •              parallelTxMsgA-SRS-PUCCH-PUSCH-r16

- •              parallelTxPUCCH-PUSCH-r17

- •              parallelTxPUCCH-PUSCH-SamePriority-r17

[Post127][604][Maint] Parallel Tx capability (Ericsson)

Scope:

    - Implement the agreement about limiting parallel Tx capabilities into a CR

    - Check whether the following intra-band capabilities can also be limited to NR SA CA, in which case this should also be implemented in the CR:

parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17

parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17

parallelTxSRS-PUCCH-PUSCH-intraBand-r17

Intended outcome:

    - Agreed CRs

Deadline:

    - Short

=> Agreed in:

R2-2407635 (38.306 CR#1153, Rel-15)

R2-2407636 (38.306 CR#1154, Rel-16)

R2-2407794 (38.306 CR#1155, Rel-17)

R2-2407795 (38.306 CR#1156, Rel-18)

Increase nrof CSI RS per MO

R2-2407297	Clarification on increasedNumberofCSIRSPerMO	Huawei, HiSilicon, vivo, MediaTek Inc., Nokia, Nokia Shanghai Bell, ZTE Corporation	CR	Rel-16	38.306	16.17.0	1138	-	F	NR_CSIRS_L3meas-Core

R2-2407298	Clarification on increasedNumberofCSIRSPerMO	Huawei, HiSilicon, vivo, MediaTek Inc., Nokia, Nokia Shanghai Bell, ZTE Corporation	CR	Rel-17	38.306	17.9.0	1139	-	A	NR_CSIRS_L3meas-Core

R2-2407299	Clarification on increasedNumberofCSIRSPerMO	Huawei, HiSilicon, vivo, MediaTek Inc., Nokia, Nokia Shanghai Bell, ZTE Corporation	CR	Rel-18	38.306	18.2.0	1140	-	A	NR_CSIRS_L3meas-Core

Agreed unseen in R2-2407637, R2-2407638, R2-2407639 but fix the styling. Everything is now style “Normal”.

ssb-AndCSI-RS-RLM

R2-2407323	Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r15)	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.25.0	1142	-	F	NR_newRAT-Core

R2-2407324	Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r16)	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.17.0	1143	-	A	NR_newRAT-Core

R2-2407325	Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r17)	ZTE Corporation, Sanechips	CR	Rel-17	38.306	17.9.0	1144	-	A	NR_newRAT-Core

R2-2407326	Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r18)	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.2.0	1145	-	A	NR_newRAT-Core

-	MediaTek think none of these are needed.

All agreed.

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

Relaxed measurements

R2-2407341	Correction to Relaxed measurement	LG Electronics	CR	Rel-16	38.304	16.10.0	0412	-	F	NR_newRAT-Core

R2-2407361	Correction to Relaxed measurement	LG Electronics	CR	Rel-17	38.304	17.9.0	0413	-	A	NR_newRAT-Core

R2-2407363	Correction to Relaxed measurement	LG Electronics	CR	Rel-18	38.304	18.2.0	0414	-	A	NR_newRAT-Core

-	ZTE wants to correct RAN2 specs rather than just refer to RAN4 specs. Vivo also does not agree with this CR, but want to send an LS to RAN4 to check with them about the issue. Nokia think people can check with their RAN4 colleagues. Vivo want an LS. Ericsson think that this is a RAN4 feature and their specs have all conditions there. MediaTek wants to get more time to check.

Postponed

## 5.2	NR V2X

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Aug 20; WID: RP-200129).

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please contact / coordinate with CR rapporteur company first for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

Tdoc Limitation: 1 tdocs

R2-2407464	MAC correction on resource selection	LG Electronics Inc.	CR	Rel-16	38.321	16.16.0	1915	-	D	5G_V2X_NRSL-Core

R2-2407472	MAC correction on resource selection	LG Electronics Inc.	CR	Rel-17	38.321	17.9.0	1916	-	A	5G_V2X_NRSL-Core

R2-2407475	MAC correction on resource selection	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1917	-	A	5G_V2X_NRSL-Core

- In cover page, typos of “reources pool” should be changed to typos of “resources pool”

- Other specs affected should be clicked for “N”

- Agreed in R2-2407588, R2-2407589, R2-2407590 with the changes above.

R2-2406699	Correction to MAC on cast type	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.16.0	1888	-	F	5G_V2X_NRSL-Core

- Agreed.

[Huawei]: We may consider to reflect it into NOTE2. Note we have similar note for HARQ enabled/disabled in Rel-16, which indicates is disabled for MAC CE. [Apple]: Prefer Huawei’s suggestion. [Session chair]: Note this change was agreed last meeting for Rel-17. It may be simpler to mimic same change for Rel-16.

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Tdoc Limitation: 1 tdoc

R2-2406295	Correction on SP positioning SRS MAC CE	Huawei, HiSIlicon	CR	Rel-16	38.321	16.16.0	1876	-	F	NR_pos-Core

- Not pursued

R2-2406296	Correction on SP positioning SRS MAC CE	Huawei, HiSIlicon	CR	Rel-17	38.321	17.9.0	1877	-	A	NR_pos-Core

- Not pursued

R2-2406297	Correction on SP positioning SRS MAC CE	Huawei, HiSIlicon	CR	Rel-18	38.321	18.2.0	1878	-	A	NR_pos-Core

- Not pursued (but editorial changes can be captured as described below)

Discussion:

vivo are OK with the editorial change, but they think the A/D change is not needed and the spec is already clear.  Huawei think the change is needed to align with other parts of the spec that capture this condition explicitly.

Samsung agree with vivo and think the second change is not needed.  CATT have the same view and think the description of the C field already makes this clear.  Ericsson agree with CATT and think the spelling mistakes should be handled in a MAC rapporteur CR if at all.

Intel think editorial changes should be done by the spec rapporteur.

CATT think we could merge the editorial changes to the Rel-18 CR.

Agreement:

Editorial changes from R2-2406297 can be merged into the Rel-18 MAC rapporteur CR.

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice -Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

Tdoc limitation: 4 Tdocs

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

R2-2406910	Discussion on PHR for mTRP PUSCH repetition	LG Electronics Inc.	discussion	Rel-17	NR_FeMIMO-Core

=>	Revised in R2-2407564

R2-2407564	Discussion on PHR for mTRP PUSCH repetition	LG Electronics Inc.	discussion	Rel-17	NR_FeMIMO-Core

Proposal 1. RAN2 decides whether/how to report one Type 3 PH value for a serving cell configured with mTRP PUSCH repetition and configured with two UL carriers.

-	Qualcomm thinks that the UE should report one Type 3.   ZTE Thinks that the problem is that the current spec doesn’t indicate how many PH fields will be present.  Qualcomm thinks that we need to revise the specification anyways.

-	ZTE and Nokia is concerned that there is no more R fields so we would need to restructure the MAC CE.   ZTE thinks that the simplest solution is to restrict the configuration.   Qualcomm thinks that maybe RAN1 needs to be involved.   Mediatek thinks that proposal 2 would solve the problem.

Proposal 2. One or two Type 1 PH values are always obtained for a serving cell configured with mTRP PUSCH repetition, if the MAC entity the serving cell belongs to is configured with twoPHRMode.

-	Qualcomm thinks that we would need a UE capability as the network wouldn’t know what the UE is reporting.

Proposal 3. Remove Type 2 PH2 field for SpCell in PHR for mTRP MAC CE.

-	MEdiatek is concerned that this is a NBC change.  ZTE explains that this is only for LTE MAC CE and it is optional.

=>	Noted

Agreements

- No Type 3 PH value is reported for a serving cell configured with mTRP PUSCH repetition, if the MAC entity the serving cell belongs to is configured with twoPHRMode.  FFS whether a UE capability is needed

- Remove Type 2 PH2 field for SpCell in PHR for mTRP MAC CE

[AT127][006][R17 UP] PHR for mTRP (LG/ZTE)

Intended outcome: discuss FFSs, agree on proposals and agree to CRs

Deadline:

R2-2407767	Report of [AT127][006][R17 UP] PHR for mTRP	LG Electronics Inc., ZTE	discussion	Rel-17	NR_FeMIMO-Core

-	Qualcomm explains that this is a NBC MAC change so a new capability may be needed.

=>	FFS whether a new capability is needed.

=>	Noted

R2-2407782	Correction on PHR for mTRP PUSCH repetition (R17)	ZTE, LG Electronics Inc., Ericsson, Samsung, CATT, Nokia, Apple	CR	Rel-18	38.321	18.2.0	1925	-	A	NR_FeMIMO-Core

=>	The CR is endorsed but will be officially approved once UE capability discussion is concluded

R2-2407432	Clarification On PHR and PHR MAC CE for mTRP	ZTE Corporation, Samsung, Nokia, CATT, Apple	discussion	Rel-17	NR_FeMIMO-Core

Proposal 2: In the case of the MAC entityEnhanced Multiple Entry PHR for multiple TRP MAC CE is generated is not configured with twoPHRMode, RAN 2 clarifies that only type 1 PH value can be obtained from PHY layer for a serving cell configured with mTRP PUSCH repetition if the MAC entity the serving cell belongs to is configured with twoPHRmode if there is no real PUSCH transmission at the slot where the PHR MAC CE is transmitted.

Proposal 3: Clarify in the subclause 6.1.3.51, one or multiple of type PH fields shall be present for the PCell.

=>	Noted

R2-2406911	Correction on PHR for mTRP PUSCH repetition	LG Electronics.	CR	Rel-17	38.321	17.9.0	1892	-	F	NR_FeMIMO-Core

=> Revised in R2-2407565

R2-2407565	Correction on PHR for mTRP PUSCH repetition	LG Electronics Inc., Ericsson	CR	Rel-17	38.321	17.9.0	1892	1	F	NR_FeMIMO-Core

=> Revised in R2-2407768

R2-2407768	Correction on PHR for mTRP PUSCH repetition	LG Electronics Inc., Ericsson, ZTE	CR	Rel-17	38.321	17.9.0	1892	2	F	NR_FeMIMO-Core

=>	The CR is endorsed but will be officially approved once UE capability discussion is concluded

R2-2406912	Correction on PHR for mTRP PUSCH repetition (R18)	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1893	-	A	NR_FeMIMO-Core

=> Revised in R2-2407566

R2-2407566	Correction on PHR for mTRP PUSCH repetition	LG Electronics Inc., Ericsson	CR	Rel-18	38.321	18.2.0	1893	1	A	NR_FeMIMO-Core

R2-2407425	Clarification on the activated TCI codepoints for Unified TCI States Activation/Deactivation MAC CE	Samsung	CR	Rel-17	38.321	17.9.0	1913	-	F	NR_FeMIMO-Core

-	LG is not sure how the UE know TCI state ID field(s) are not present.   Samsung indicates it can be done by checking the field.   Qualcomm agrees that you can know by the length of the MAC CE.

=>	RAN2 assumes that If the corresponding TCI state ID field(s) are not present, MAC entity shall ignore the Pi field.  No specification change is needed.

=>	The CR is not pursued

R2-2407426	Clarification on the activated TCI codepoints for Unified TCI States Activation/Deactivation MAC CE	Samsung	CR	Rel-18	38.321	18.2.0	1914	-	A	NR_FeMIMO-Core

R2-2407137	Correction on HARQ process	Samsung	CR	Rel-17	38.321	17.9.0	1899	-	F	NR_NTN_solutions-Core, NR_MBS-Core

-	Mediatek doesn’t agree with the change as for NTN we have the behaviour where HARQ is disabled.  This can be resolved by network implementation and can utilize the HARQ processes that are enabled.

-	Vivo also doesn’t agree and we have discussed this and current spec is enough

-	LG supports as in the previous meeting we agreed that MBS configuration would override the configuration.

-	Qualcomm explains that the change would cause problem with some RAN1 agreements, so we would have to first check with RAN1.

-	CATT agrees with CR

-	Huawei doesn’t think the change is needed as the existing sentence already covers.   Nokia also doesn’t think the change is needed.

=>	The CR is postponed (check what clarification may be needed and what the issue is)

R2-2407138	Correction on HARQ process	Samsung	CR	Rel-18	38.321	18.2.0	1900	-	A	NR_NTN_solutions-Core, NR_MBS-Core

R2-2407171	Correction on use of recommended of IAB-MT beam indication.	Ericsson	CR	Rel-17	38.321	17.9.0	1901	-	F	NR_IAB_enh-Core

=>	the CR is postponed

R2-2407172	Correction on use of recommended of IAB-MT beam indication.	Ericsson	CR	Rel-18	38.321	18.2.0	1902	-	A	NR_IAB_enh-Core

### 6.1.3	Control Plane corrections

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

MIMO – Power control

R2-2407557	Correction on power control parameters to support unified TCI state framework	Ericsson	CR	Rel-17	38.331	4964	-	F	NR_feMIMO-Core 	Late

R2-2407556	Correction on power control parameters to support unified TCI state framework	Ericsson	CR	Rel-17	38.331	4963	-	F	NR_feMIMO-Core	Late

-	Nokia does not agree and thinks there need to be more discussion. MediaTek agrees with the Ericsson CR and thinks it is needed. CATT has checked with RAN1 colleagues and agrees with the problem but want to think more how to address it, another approach is that we add specific parameters, but are not sure which parameters need to be added yet. Huawei thinks that it can work without the parameters and RAN1 said the params are not needed. Qualcomm also checked with RAN1 people and agrees with the CR but can consider to send an LS to RAN1. Docomo agrees with Qualcomm.  Samsung also checked with RAN1 colleauges and agrees with Qualcomm. Ericsson is OK to check with RAN1.

[AT127][604][FeMIMO] LS to RAN1 on power control parameters (Ericsson)

Scope:

    - Produce approvable draft LS

Intended outcome:

    - Approvable LS in R2-2407631 (Ericsson)

Deadline:

    - Wednesday session

R2-2407631 is approved in R2-2407655

[Post127][602][FeMIMO] Power control parameters to support unified TCI state framework (Ericsson)

Scope:

    - Produce agreeable CRs pending input from RAN1, if needed

Intended outcome:

    - Agreed CRs in R2-2407659 and R2-2407660 (Ericsson)

Deadline:

    - Short

=> Agreed in:

R2-2407659 (38.331 CR#4963r1, Rel-18)

R2-2407660 (38.331 CR#4964r1, Rel-17)

RedCap

R2-2406372	Corrections on the application of intraFreqReselection	Huawei, HiSilicon	CR	Rel-17	38.331	17.9.0	4874	-	F	NR_redcap-Core

R2-2406373	Corrections on the application of intraFreqReselection	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4875	-	A	NR_redcap-Core, NR_redcap_enh-Core

-	Samsung thinks the change to the IFRI field description is wrong, since a RedCap UEs look at the IFRI just that its another version of the field. Qualcomm says that they try to polish this behaviour in several specs but that was not agreeable. Huawei wants more time to discuss in a later meeting.

Postponed

RA partitioning

R2-2406411	Consideration on overlapping preamble range and empty FeatureCombination	ZTE Corporation	discussion	Rel-17	NR_newRAT-Core

On P1:

-	After a coffeebreak offline companies aligned and want to confirm P1 from the ZTE paper above.

On P3:

-

RAN2 confirm that in RedCap specific initial uplink BWP where SI-request without Msg1 repetition is enabled, the network can configure “startPreambleForThisPartition-r17” equal to 0 for a RACH partition associated with a feature (or feature combination), and this RACH partition shares the ROs of another RACH partition that not associated with any feature. No spec impact is needed.

RAN2 confirm that empty featureCombination-r17 is not supported.

To clarify in RRC spec, for SI-RequestConfig, UE applies the parameters (e.g. prach-RootSequenceIndex, msg1-SubcarrierSpacing, etc) configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization as specified in MAC spec.

R2-2407009	Correction on featureCombination and SI-RequestConfig	ZTE Corporation	CR	Rel-17	38.331	17.9.0	4911	-	F	NR_newRAT-Core

R2-2407010	Correction on featureCombination and SI-RequestConfig	ZTE Corporation	CR	Rel-18	38.331	18.2.0	4912	-	A	NR_newRAT-Core

-	Apple wants to write that “Network ensured that at least one field in the featureCombination is configured”

Change to “Network ensure that at least one field in the featureCombination is configured” in the field description for featureCombination.

Agreed unseen in R2-2407642 and R2-2407643

R2-2407642	Correction on featureCombination and SI-RequestConfig	ZTE Corporation	CR	Rel-17	38.331	17.9.0	4911	1	F	NR_newRAT-Core

R2-2407643	Correction on featureCombination and SI-RequestConfig	ZTE Corporation	CR	Rel-18	38.331	18.2.0	4912	1	A	NR_newRAT-Core

=> Coversheet revised by MCC: Rel-15 WI code only, needs TEI17 in addition

=> Revised in R2-2407879 and R2-2407880

R2-2407879	Correction on featureCombination and SI-RequestConfig	ZTE Corporation	CR	Rel-17	38.331	17.9.0	4911	2	F	NR_newRAT-Core, TEI17

=> Agreed

R2-2407880	Correction on featureCombination and SI-RequestConfig	ZTE Corporation	CR	Rel-18	38.331	18.2.0	4912	2	A	NR_newRAT-Core, TEI17

=> Agreed

R2-2407459	Clarification on configuring RACH partition in RedCap-specific initial BWP for Msg1-based SI request	LG Electronics Inc.	discussion	Rel-17	NR_redcap-Core

R2-2407173	Correction on when featureCombination is empty	Ericsson	CR	Rel-17	38.331	17.9.0	4801	1	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core	R2-2404965

R2-2407174	Correction on when featureCombination is empty	Ericsson	CR	Rel-18	38.331	18.2.0	4802	1	A	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core	R2-2404966

R2-2406927	Correction for CFRA configuration due to PRACH partitioning	Huawei, HiSilicon	CR	Rel-17	38.331	17.9.0	4899	-	F	NR_redcap-Core

R2-2406928	Correction for CFRA configuration due to PRACH partitioning	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4900	-	A	NR_redcap-Core, NR_redcap_enh-Core

-	ZTE thinks the CR is not needed because it is clear in RACH config common field description and this is just another way of achieving the same, but is incomplete. MediaTek wants to split the sentence to two, and also there should be more WI-codes on the cover page since its impacting more Wis.

Postponed

Slicing

R2-2406691	Correction on Slicing based RACH	Apple	CR	Rel-17	38.331	17.9.0	4884	-	F	NR_slice-Core

R2-2406692	Correction on Slicing based RACH	Apple	CR	Rel-18	38.331	18.2.0	4885	-	A	NR_slice-Core

-	LG thinks its clear from other parts of the spec, e.g. MAC. ZTE thinks it’s a good clarification. Vivo want to check more since its not clear from MAC specification but want to check more if RRC is clear enough.

Postponed

71 GHz

R2-2406838	Discussion on missing PRACH SCS configuration	CATT	discussion	Rel-17	NR_ext_to_71GHz-Core

R2-2406839	Correction on the missing PRACH SCS configuration	CATT	CR	Rel-17	38.331	17.9.0	4891	-	F	NR_ext_to_71GHz-Core

R2-2406840	Correction on the missing PRACH SCS configuration	CATT	CR	Rel-18	38.331	18.2.0	4892	-	F	NR_ext_to_71GHz-Core, TEI18

-	Samsung thinks this is correct, but there are interoperability issues so want a new field with a new capability. CATT thinks that if it has not been implemented then we can just agree the CR. Qualcomm thinks we can agree the CR.

Should add cover page statement saying “This CR is considered mandatory for UEs and networks implementing 71 GHz”

Both are agreed unseen in R2-2407665 and R2-2407666

SON/MDT - Mobility history info

R2-2406990	Miscellaneous corrections to mobility history information	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.9.0	4907	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2406991	Miscellaneous corrections to mobility history information	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.2.0	4908	-	A	NR_ENDC_SON_MDT_enh-Core

-	Huawei thinks the first change is an optimization and want to postpone it to R18, but are OK with change 2 and 3. Samsung would like to postpone the first change, the second and third are OK. Ericsson think that the first change is wrong since if the UE is in IDLE/INACTIVE the UE is not using DC and shouldn’t log it. Nokia agrees with Ericsson about the first change. CATT also want to postpone the discussion regarding the first change.

The first change is postponed

The second and third change are agreed unseen in R2-2407657 and R2-2407658

SON/MDT - Reestablishment at failed CHO

R2-2407114	Correction of reestablishmentCellId	Nokia	CR	Rel-17	38.331	17.9.0	4924	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2407115	Correction of reestablishmentCellId	Nokia	CR	Rel-18	38.331	18.2.0	4925	-	A	NR_ENDC_SON_MDT_enh-Core

-	Samsung thinks that the procedural text is good enough.

Not pursued

SON/MDT - RA report

R2-2407340	Corrections for RA resource related parameters in RA report	Sharp	discussion	Rel-17

-	Samsung thinks understanding 3 is correct. Huawei agrees but wonder which release we should change from? Sharp wants it from Rel-17. Ericsson thinks that current spec is not broken so want to leave it as it is. Sharp is OK to leave the spec unchanged.

RAN2 understands that understanding 3 is correct.

QoE

R2-2407089	Improvement of procedure text for QoE measurements	Ericsson	CR	Rel-17	38.331	17.9.0	4923	-	F	NR_QoE-Core

-	Huawei think it would be bad NW configuration to setup QoE but not configure SRB4. Ericsson think the spec should be clear so the UEs know what to expect. CATT think that the target will provide the needed config. Samsung think the CR is correct but expect that the NW will always configure SRB4.

Not pursued

MBS

R2-2407144	Clarification on MBS broadcast acquisition	Samsung, CATT, Nokia, LG Electronics Inc., Ericsson, Apple, Qualcomm Incorporated	CR	Rel-17	38.331	17.9.0	4926	-	F	NR_MBS-Core, NR_SmallData_INACTIVE-Core

R2-2407150	Clarification on MBS broadcast acquisition	Samsung, CATT, Nokia, LG Electronics Inc., Ericsson, Apple, Qualcomm Incorporated	CR	Rel-18	38.331	18.2.0	4927	-	A	NR_MBS-Core, NR_SmallData_INACTIVE-Core

-	MediaTek thinks this should be captured in 5.9.1.1 and not in the section for MIB acquisition. MediaTek thinks the first change is fine as it is.

[AT127][605][Maint] Example (Samsung)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2407644 and R2-2407645 (Samsung)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2407644	Clarification on MBS broadcast acquisition	Samsung	CR	Rel-17	38.331	17.9.0	4926	1	F	NR_MBS-Core, NR_SmallData_INACTIVE-Core

R2-2407645	Clarification on MBS broadcast acquisition	Samsung	CR	Rel-18	38.331	18.2.0	4927	1	A	NR_MBS-Core, NR_SmallData_INACTIVE-Core

Both above are agreed

Power saving

R2-2407371	Clarification on the case the SIB17 is absent	Google	CR	Rel-18	38.331	18.2.0	4947	-	F	NR_UE_pow_sav_enh-Core

To be merged to RRC rapp CR but change to “SIB17bis is optionally scheduled if SIB17 is not scheduled.”

CG-Config

R2-2407428	Discussion on indication of reserved SN-side NR-DC and aggregated BW resources	Nokia	discussion	Rel-17	NR_newRAT-Core, NR_eMIMO-Core, NR_BCS4-Core

R2-2407429	Correction to indication of reserved SN-side NR-DC and aggregated BW resources	Nokia	CR	Rel-17	38.331	17.9.0	4952	-	F	NR_newRAT-Core, NR_eMIMO-Core, NR_BCS4-Core

R2-2407430	Correction to indication of reserved SN-side NR-DC and aggregated BW resources	Nokia	CR	Rel-18	38.331	18.2.0	4953	-	F	NR_newRAT-Core, NR_eMIMO-Core, NR_BCS4-Core

-	ZTE agrees with the intention, but a simpler approach is that we add a sentence to clarify in which cases the SN “indicates”. Ericsson agrees with ZTE approach. CATT also likes ZTE approach. Nokia is OK with the ZTE approach.

[AT127][606][Maint] Indication of reserved SN-side resources (Nokia)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2407648 and R2-2407649 (Nokia)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2407648	Correction to indication of reserved SN-side NR-DC and aggregated BW resources	Nokia	CR	Rel-17	38.331	17.9.0	4952	1	F	NR_newRAT-Core, NR_eMIMO-Core, NR_BCS4-Core

R2-2407649	Correction to indication of reserved SN-side NR-DC and aggregated BW resources	Nokia	CR	Rel-17	38.331	17.9.0	4952	1	F	NR_newRAT-Core, NR_eMIMO-Core, NR_BCS4-Core

Both agreed

Misc

R2-2407519	Miscellaneous non-controversial corrections Set XXII	Ericsson	CR	Rel-16	38.331	16.17.0	4960	-	F	NR_newRAT-Core, TEI16

R2-2407081	Miscellaneous non-controversial corrections Set XXII	Ericsson	CR	Rel-17	38.331	17.9.0	4916	-	F	NR_newRAT-Core, TEI17

R2-2407082	Miscellaneous non-controversial corrections Set XXII	Ericsson	CR	Rel-18	38.331	18.2.0	4917	-	A	NR_newRAT-Core, TEI18

Endorsed and to be updated in a post meeting email discussion.

[Post127][601][RRC] Miscellaneous non-controversial corrections Set XXII (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2407650, R2-2407651 and R2-2407652 (Ericsson)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

    - Friday morning session

=> Agreed in:

R2-2407650 (38.331 CR#4960r1, Rel-16)

R2-2407651 (38.331 CR#4916r1, Rel-17)

R2-2407652 (38.331 CR#4917r1, Rel-18)

NTN – Parallel cell lists

R2-2407528	Clarification on parallel lists in MeasObjectNR	Samsung	CR	Rel-17	38.331	17.9.0	4961	-	F	NR_NTN_solutions-Core

R2-2407529	Clarification on parallel lists in MeasObjectNR	Samsung	CR	Rel-18	38.331	18.2.0	4962	-	F	NR_NTN_enh-Core, NR_NTN_solutions

Both Agreed

NTN - smtc4list

R2-2407300	Clarification on smtc4list-r17	Huawei, HiSilicon	CR	Rel-17	38.331	17.9.0	4943	-	F	NR_NTN_solutions-Core

R2-2407301	Clarification on smtc4list-r17	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4944	-	A	NR_NTN_solutions-Core

-	Nokia and Ericsson are not supportive. Qualcomm thinks that instead we should clarify what the UE is supposed to do in the scenarios described on the cover page.

-	Huawei think at least we need to clarify that one PCI is in at most one list. Ericsson think that from 6.1.3 everything is clear on how released lists are handled.

Postponed

NTN – Validity duration

R2-2406467	Clarification on Validity Duration	vivo, Ericsson	CR	Rel-17	38.331	17.9.0	4877	-	F	NR_NTN_solutions-Core

R2-2406468	Clarification on Validity Duration	vivo, Ericsson	CR	Rel-18	38.331	18.2.0	4878	-	A	NR_NTN_solutions-Core

The whole sentence where this change is done, should be removed instead.

[AT127][608][Maint] Clarification on Validity Duration (vivo)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2407661 and R2-2407662 (Company)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2407661	Clarification on Validity Duration	vivo	CR	Rel-17	38.331	17.9.0	4877	1	F	NR_NTN_solutions-Core

R2-2407662	Clarification on Validity Duration	vivo	CR	Rel-18	38.331	18.2.0	4878	1	A	NR_NTN_solutions-Core

Both agreed

NTN – extended k1 range

R2-2406225	LS on inconsistent issue between extended k1 range and RRC parameter DL-DataToUL-ACK-v1700 for R17 NTN R4-2409974; contact: CMCC)	RAN4	LS in	Rel-18	NR_ATG	To:RAN2	Cc:RAN1

Moved from 7.25.1

Noted

R2-2406862	Discussion on the inconsistency issue in RAN4 LS R2-2406225 for Rel-17 NR NTN	CATT	discussion	Rel-17	NR_NTN_solutions

Moved from 7.25.1

-	Qualcomm want to do a change in R18 instead, it should be simple. CATT would be OK with that. Nokia think that RAN4 asks us to do the change in R17, but could be open to do from R18 and in that case we should tell RAN4 in an LS.

Postponed.

R2-2406845	On the Inconsistency between extended k1 range and DL-DataToUL-ACK-v1700 (RAN4 LS)	Nokia	discussion	Rel-18	NR_ATG-Core

Moved from 7.25.1

Withdrawn and old revisions

R2-2407080	Miscellaneous non-controversial corrections Set XXII	Ericsson	CR	Rel-16	38.331	16.17.0	4915	-	F	NR_newRAT-Core, TEI16	Withdrawn

R2-2407500	Correction on mappingPattern for mTRP DG/CG PUSCH repetition schemes	Nokia	CR	Rel-17	38.331	17.9.0	4958	-	F	NR_FeMIMO-Core

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

FR1/FR2 diff

R2-2406817	Corrections to UE capabilities related to Rel-17 URLLC and RedCap 	Huawei, HiSilicon	discussion	Rel-17	NR_IIOT_URLLC_enh-Core, NR_redcap-Core

The TPs are agreeable.

[AT127][607][NR17] Corrections to UE capabilities related to Rel-17 URLLC and RedCap (Huawei)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2407653 and R2-2407654 (Huawei)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2407653	Corrections to UE capabilities related to Rel-17 URLLC and RedCap	Huawei, HiSilicon	CR	Rel-17	38.306	17.9.0	1149	-	F	NR_IIOT_URLLC_enh-Core, NR_redcap-Core

R2-2407654	Corrections to UE capabilities related to Rel-17 URLLC and RedCap	Huawei, HiSilicon	CR	Rel-18	38.306	18.2.0	1150	-	A 	NR_IIOT_URLLC_enh-Core, NR_redcap-Core

Both agreed

Enhanced channel raster

R2-2407076	Support of Enhanced channel raster by (e)RedCap UE Rel-17	Ericsson	discussion	Rel-17	NR_redcap-Core

Noted

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

## 6.2	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

Tdoc Limitation: 1 tdoc

R2-2406948	Correction on T300 and T302 handling due to relay selection or cell selection	Huawei, HiSilicon	CR	Rel-17	38.331	17.9.0	4905	-	F	NR_SL_relay-Core

- “if relay (re)selection or cell selection” to be replaced with “if relay (re)selection, or cell selection by a L2 U2N Remote UE,”

- “cell selection” in table to be replaced with “cell selection by a L2 U2N Remote UE”

- Agreed with this change as R2-2407724

R2-2406949	Correction on T300 and T302 handling due to relay selection or cell selection	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4906	-	A	NR_SL_relay-Core

- “if relay (re)selection or cell selection” to be replaced with “if relay (re)selection, or cell selection by a L2 U2N Remote UE,”

- “cell selection” in table to be replaced with “cell selection by a L2 U2N Remote UE”

- Agreed with this change as R2-2407725

Discussion:

Apple wonder if this would change legacy UE behaviour.  Chair thinks the affected scenario cannot occur without a relay.  Ericsson think that once the remote UE connects to the cell, it could select a relay, but a legacy UE cannot.  Huawei are OK to clarify but think it can only affect a L2 U2N remote UE.

Xiaomi agree with the intention but think it is difficult to differentiate the normal UE and remote UE; they would be OK with a coversheet clarification.

Nokia think we need clarification in the spec and the coversheet is not enough.

Chair suggests “if relay (re)selection, or cell selection by a L2 U2N remote UE, occurs”.

Qualcomm think it might be covered already by legacy procedures.

R2-2407270	RRC correction on NR SL U2N Relay UE selection and reselection procedure	Philips International B.V., NEC	CR	Rel-17	38.331	17.9.0	4938	-	F	NR_SL_relay-Core

- Change to add the sl-RemoteUE-ConfigCommon instead of the IE name

- Agreed with this change as R2-2407726

R2-2407272	RRC correction on NR SL U2N Relay UE selection and reselection procedure	Philips International B.V., NEC	CR	Rel-18	38.331	18.2.0	4939	-	A	NR_SL_relay-Core

- Change to add the sl-RemoteUE-ConfigCommon instead of the IE name

- Agreed with this change as R2-2407727

Discussion:

Huawei agree with the intention but think we could remove the parent field names instead of adding an IE name, and just say “below threshHighRemote”.

Apple think we could add an “or” case for the inactive field name.  Huawei agree this would be more clear.

R2-2407495	Corrections for SL relay measurements	ZTE Corporation, Sanechips, Apple, OPPO, Nokia	CR	Rel-17	38.331	17.9.0	4956	-	F	NR_SL_relay-Core

- Revised in R2-2407728 (change addition in NOTE to normative text)

R2-2407496	Corrections for SL relay measurements	ZTE Corporation, Sanechips, Apple, OPPO, Nokia	CR	Rel-18	38.331	18.2.0	4957	-	A	NR_SL_relay-Core

- Revised in R2-2407729

Discussion:

Huawei understand the intention is to include the serving relay case for the L3 filtering, and they see the change to the NOTE as being more like normative behaviour.  Apple think we could try to render it as normative text.

[AT127][407][Relay] Corrections for Rel-17 SL relay measurements (ZTE)

Scope: Update R2-2407495 and R2-2407496 to change the addition in the NOTE into normative text.

Intended outcome: Agreeable CRs in R2-2407728 and R2-2407729

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407728	Corrections for SL relay measurements	ZTE Corporation, Sanechips, Apple, OPPO, Nokia, Huawei, HiSilicon	CR	Rel-17	38.331	17.9.0	4956	1	F	NR_SL_relay-Core

- Agreed

R2-2407729	Corrections for SL relay measurements	ZTE Corporation, Sanechips, Apple, OPPO, Nokia, Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4957	1	A	NR_SL_relay-Core

- “the” to be added before “U2N Remote UE” in the last paragraph

- Agreed with this change as R2-2407754

Discussion:

ZTE indicate there are changes on changes.  Huawei think it is just different usernames.

## 6.4	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

Tdoc Limitation: 1 tdoc

R2-2406298	Correction on PPW for MAC spec	Huawei, HiSIlicon	CR	Rel-17	38.321	17.9.0	1879	-	F	NR_pos_enh-Core

- Not pursued

R2-2406299	Correction on PPW for MAC spec	Huawei, HiSIlicon	CR	Rel-18	38.321	18.2.0	1880	-	A	NR_pos_enh-Core

- Not pursued

Discussion:

Ericsson think the priority language is needed in the MAC spec, and we should not change it without consulting RAN1.

vivo think the current spec is correct and clear.  Nokia think the priority logic needs to be captured somewhere and we should not delete the check unless it is incorrect.  Intel agree that there is no need to remove the condition.

ZTE think the MAC condition is correct and there is no need to mention the type.

CATT agree with ZTE that type is not needed here.

Huawei think the condition is not useful and the MAC implementation does not need to check the priority, because PHY will already do it.

R2-2406788	Correction on SP SRS activation/deactivation MAC CE	ZTE Corporation, Ericsson	CR	Rel-17	38.321	17.9.0	1840	1	F	NR_pos_enh-Core	R2-2404625

- “last serving cell from where Positioning SRS configuration was received while in RRC_CONNECTED” to be replaced with “cell from which positioning SRS configuration was received”

- Agreed with this change as R2-2407716

R2-2406789	Correction on SP SRS activation/deactivation MAC CE	ZTE Corporation, Ericsson	CR	Rel-18	38.321	18.2.0	1841	1	A	NR_pos_enh-Core	R2-2404626

- Agreed with the same changes as under R2-2406788, as R2-2407717

R2-2406790	Discussion on DL MAC CE in Rel-17 and Rel-18 SP SRS in RRC_INACTIVE	ZTE Corporation, Ericsson	discussion	Rel-17	38.321	NR_pos_enh-Core

- Noted

Discussion:

Huawei think the CR is wrong because there is no “last serving cell” for the UE in RRC_INACTIVE; they assume the UE anyway will ignore the field.

vivo are fine with the intention but think the network should always set the cell ID as 0 for an inactive UE, and they think it is clear that the inactive UE should always ignore the BWP ID.

Samsung generally agree with Huawei; they think the UE in RRC_INACTIVE can ignore both the cell ID and the BWP ID, but they think there could be a clarification of the usage of the C bit in RRC_CONNECTED.

ZTE understand that the cell ID should not be directly ignored, because there is also a serving cell ID in the spatial relation field and the UE cannot interpret it.  They agree that the last serving cell ID is 0 and the gNB should always set 0 for this field, as indicated by vivo, and for the BWP ID it needs to be clarified what happens when SRS is configured outside the initial BWP.

ZTE think a change is needed to reflect the differences in how the MAC CE is used for the RRC_INACTIVE case.

Qualcomm agree with ZTE and think the current description is confusing for an inactive UE.

Samsung wonder if the BWP ID should ever be used by the UE when in RRC_INACTIVE.  ZTE understand that the spatial relation part can contain a BWP ID.

Huawei understand the intention is fine, but for RRC_INACTIVE they think the UE will just look at the UE context and not need the cell ID and BWP ID.  ZTE understand they cannot be completely ignored because they also relate to the spatial relation.  Huawei think the cell ID is not needed even for the spatial relation, because there is no cell ID in RRC_INACTIVE and the UE will just look for the configuration in the UE context.  ZTE understand that the UE will remember the cell ID, BWP ID, and reference signal configuration when in RRC_INACTIVE as part of the AS context.

ZTE wonder if Huawei’s understanding means that the gNB should always set C to 1.

Ericsson think from a signalling pov, the same MAC CE is used for connected and inactive, and we need to illustrate the meaning of the bits in a consistent way; they understand this is what the CR does.

CATT think we could just word it as “the cell from which SP-SRS configuration was received”, without mentioning “last serving cell” or “while in RRC_CONNECTED”.

Huawei understand the inactive UE should always ignore these fields.  ZTE think then the gNB should always set C to 1 and that should be clear.  Huawei agree with this interpretation.

Huawei think the SRS used for the spatial relation can be identified by resource set ID/resource ID.

Ericsson would be OK with CATT’s suggested wording.

R2-2407223	Corrections related to additional path reporting and QCL for positioning	Ericsson	CR	Rel-17	37.355	17.8.0	0513	-	F	NR_pos_enh-Core

- Not pursued

R2-2407226	Corrections related to additional path reporting and QCL for positioning	Ericsson	CR	Rel-18	37.355	18.2.0	0514	-	A	NR_pos_enh-Core

- Changes related to dl-PRS-QCL-Info to be captured in Rel-18 rapporteur CR

- This CR is not pursued

Discussion:

ZTE think the additional path change is not correct, because the additional path is calculated on the same resource.  CATT agree with ZTE; they understand that the additional measurements can relate to a different resource, but not the additional path.

Qualcomm also agree and think the CR mixes two different issues: the additional path and the resources for the additional measurements.  They think the additional path is already clearly specified, and which resources to measure is specified in the additional measurements.

vivo share Qualcomm’s view, but for the second change about the QCL they think it helps to clarify.  Qualcomm agree the QCT change is correct, but they see it as editorial and think it could be captured in the Rel-18 rapporteur CR.

Agreement:

Changes related to dl-PRS-QCL-Info from R2-2407226 will be captured in Rel-18 37.355 rapporteur CR.

## 6.6	NR Sidelink enhancements

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

Tdoc Limitation: 1 tdoc

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please contact / coordinate with CR rapporteur company first for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

R2-2406262	Correction on resource (re)selection for IUC 	OPPO	CR	Rel-17	38.321	17.9.0	1873	-	F	NR_SL_enh-Core

R2-2406263	Correction on resource (re)selection for IUC 	OPPO	CR	Rel-18	38.321	18.2.0	1874	-	A	NR_SL_enh-Core

For the first change:

[LG]: Consider the current text has no problem. [OPPO]: The removed condition can be also applicable to the three conditions. It is not exclusive condition. That’s why “the latency requirement of the Sidelink Inter-UE Coordination Information transmission (if applicable)” is added for each three cases with the removal of the exclusive condition. [LG]: Current MAC specifies two parts of resource selections. One is the resource selection for the data and another one is explicitly for IUC Information. If we remove the condition, it becomes less clear for the resource selection for IUC. [Apple]: Share the same view as LG. [OPPO]: Want to have more time to discuss with other companies. Would like to revisit it next meeting.

- Revisit it next meeting.

For the second change:

[LG]: Based on RAN1 agreement, random resource selection with sl-interUECoordinationScheme2 is not supported, so if we really want to align with RAN1, we can simply remove “random selection” in the first paragraph. [OPPO]: Have somewhat different view on RAN1 status, however it can be one of possible solutions. [OPPO, LG]: It is better to ask RAN1 if configuration of both IUC scheme 2 and random resource selection is supported or not.

- LS is sent to RAN1 to ask if configuration of both IUC scheme 2 and random resource selection is supported or not.

[AT127][107][V2X/SL] (OPPO)

Scope: Ask RAN1 if configuration of both IUC scheme 2 and random resource selection is supported or not.

Intended outcome: LS in R2-2407591

Deadline: Email approval (until end of 8/21 Wednesday) => Completed.

R2-2407591	LS on IUC Scheme-2 and Random Selection	RAN2	LSout	To: RAN1

- Approved.

R2-2406514	Correction on prioritization between SL transmission and MAC CE-triggered SR	ASUSTeK	CR	Rel-17	38.321	17.9.0	1886	-	F	NR_SL_enh-Core

R2-2406515	Correction on prioritization between SL transmission and MAC CE-triggered SR	ASUSTeK	CR	Rel-18	38.321	18.2.0	1887	-	A	NR_SL_enh-Core, NR_SL_enh2

- Noted.

[LG]: Based on RAN2 previous agreement on prioritization rule between UL MAC CE and SL, RAN2 agreed UL MAC CE is always considered as higher priority than UL threshold and it was agreed that SL is only compared to SL threshold, e.g. if SL has higher priority than SL threshold, SL is prioritized. Otherwise UL MAC CE is prioritized. The current MAC is ok. [Samsung]: Is it realistic scenario that NR-U (or IAB) and SL/V2X co-exist? If not, we may not need to consider LBT failure MAC CE and Pre-emptive BSR MAC CE. [Ericsson]: Share the view with Samsung. [CATT]: Prioritization rule was introduced in Rel-16 and we haven’t considered new MAC CEs introduced from Rel-17. It is better to have more time to think of. [OPPO]: Agree with LG, however it was for PUSCH with UL MAC CE. Cannot recall whether RAN2 discussed and made a specific decision for SR. [OPPO]: Remember SR prioritization actually inherited from URLLC. Need to double check. [CATT]: In Rel-16, we only considered BSR MAC CE then the priority of the corresponding LCH data was considered in prioritization rule. [Huawei]: Remember SR triggered by LCH data was only considered in Rel-16. We haven’t discussed other cases, e.g. SR triggered by UL MAC CE. It is good to discuss it in common session. [OPPO]: Based on companies inputs, it is not for correction. It is for new feature, which should be discussed for TEI. [Session chair]: Let’s have more time to think of and come back next meeting. It would be good to discuss in common session. Target agenda item should be discussed with RAN2 chair-lady.

R2-2407011	Correction on SL IUC and SL DRX configurations for SL enhancements	Huawei, HiSilicon	CR	Rel-17	38.331	17.9.0	4913	-	F	NR_SL_enh-Core

R2-2407012	Correction on SL IUC and SL DRX configurations for SL enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4914	-	A	NR_SL_enh-Core

- Agreed.

[Ericsson]: Support the proposal. [ZTE]: Many co-existence features are captured in stage 2. Prefer to capturing it into stage 2. [Huawei]: It is configuration aspect, so prefer RRC. [Vivo]: UE behaviour is clear in MAC so we may not need this CR. [Toyota]: It should be specified somewhere, in either stage 2 or stage 3.

R2-2407019	Correction to MAC on HARQ feedback indicator	Ericsson	CR	Rel-17	38.321	17.9.0	1895	-	F	NR_SL_enh-Core

R2-2407246	Correction to MAC on HARQ feedback indicator	Ericsson	CR	Rel-18	38.321	18.2.0	1906	-	A	NR_SL_enh-Core

- NOTE 2 will be updated to “NOTE 2:	HARQ feedback enabled/disabled indicator is set to disabled for the transmission of a MAC PDU only carrying Sidelink MAC CE(s), e.g. Sidelink CSI reporting MAC CE, Sidelink DRX Command MAC CE, Sidelink Inter-UE Coordination Request MAC CE or Sidelink Inter-UE Coordination Information MAC CE.”

- With the NOTE 2 above, agreed in R2-2407592 and R2-2407593.

R2-2407020	Correction to MAC on cast type	Ericsson	CR	Rel-18	38.321	18.2.0	1896	-	A	NR_SL_enh-Core	Withdrawn

R2-2407405	Correction on resource selection for IUC 	LG Electronics Inc.	CR	Rel-17	38.321	17.9.0	1909	-	F	NR_SL_enh-Core	Withdrawn

R2-2407412	Correction on resource selection for IUC 	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1910	-	A	NR_SL_enh-Core	Withdrawn

# 7	Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.1	UE Capabilities

UE feature list LS

R2-2406210	LS on Rel-18 RAN1 UE features list for NR after RAN1#117 (R1-2405566; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18	To:RAN2, RAN4

=>	Noted

R2-2406232	LS on RAN4 UE feature list for Rel-18 (version 5) (R4-2410748; contact: CMCC)	RAN4	LS in	Rel-19	NR_ENDC_RF_FR1_enh2, NR_channel_raster_enh, NR_FR2_multiRX_DL, NR_RRM_enh3, NR_MG_enh2, NonCol_intraB_ENDC_NR_CA, NR_HST_FR2_enh, NR_ATG, NR_demod_enh3, NR_pos_enh2, NR_MC_enh, NR_Mob_enh2, NR_NTN_enh, NR_cov_enh2, Netw_Energy_NR, 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC, NR_SL_enh2	To:RAN2	Cc:RAN1

=>	Noted

Other corrections

Corrections to AccessStratumRelease and ue-ConfigRelease

R2-2406535	Correction to UE capabilities and configuration	Google	CR	Rel-18	36.331	18.2.0	5040	-	F	IoT_NTN_enh-Core, NR_ENDC_SON_MDT_enh2-Core, LTE_UAV_enh-Core

-	Lenovo thinks that for the first two changes we should change the description to be generic like this field indicated the AS release supported by UE.

=>	Update the CR with a generic sentence

=> 	The CR is revised in R2-2407805

[AT127][001][UE cap] CR update to R2-2406535 (Google)

Intended outcome: agree to CR

Deadline:  08-23-24

R2-2407805	Correction to UE capabilities and configuration	Google	CR	Rel-18	36.331	18.2.0	5040	1	F	IoT_NTN_enh-Core, NR_ENDC_SON_MDT_enh2-Core, LTE_UAV_enh-Core

=>	The CR is agreed

Redundant pre-requisite of UE features

R2-2407327	Consideration on the Redundant Prerequisite of UE Features	ZTE Corporation, Sanechips	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: Based on the above 4 observations, RAN2 can keep the Rel-18 as it is, but send LS to RAN1/4 to ask RAN1/4 to avoid such kinds of the redundant prerequisites in the feature list table from Rel-19.

-	Intel agrees with the intention but there is no need to send an LS and for R19 we can communicate with the RAN1/RAN4 feature leads for next release

-	Ericsson thinks that this is minor and nothing really breaks

-	Samsung thinks it should be avoided and some clarification points are required and RAN1 may update the feature list.

=>	No formal action but companies can communicate internally to their delegates

=>	Noted

R2-2407328	Draft LS on the Redundant Prerequisite	ZTE Corporation, Sanechips	LS out	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN1,RAN4

=>	Not treated

[POST127][002][UE Cap] Mega CR (Intel)

Intended outcome: Agree to Mega CR

Deadline:  Sept 2nd, 2024

=> Agreed in:

R2-2407846 (38.306 CR#1159)

R2-2407847 (38.331 CR#4971)

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 5

#### 7.0.2.1	RACH-less HO

Rapporteur corrections

R2-2406300	Rapporteur MAC CR for generalized RACH-less HO/LTM [RACH-lessHO]	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1881	-	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core

=>	The CR is revised in R2-2407818 and agreed

R2-2407180	Correction on beam indication for NTN RACH-less [RACH-lessHO]	Ericsson	discussion	Rel-18	TEI18

Proposal 1	RAN2 to clarify in TS 38.331 that the field ssb-Index should not be configured when the field cg-RRC-Configuration-r18 is configured.

R2-2407181	Misc RRC corrections for RACH-less HO [RACH-lessHO]	Ericsson (Rapportuer)	CR	Rel-18	38.331	18.2.0	4932	-	F	TEI18

=> Revised in R2-2407787

R2-2407787	Misc RRC corrections for RACH-less HO [RACH-lessHO]	Ericsson	CR	Rel-18	38.331	18.2.0	4932	1	F	NR_mobile_IAB-Core, NR_NTN_enh-Core, TEI18

=>	The CR is agreed

Other corrections

CG restriction for RACH-less LTM

R2-2406493	Correction on RRC parameters for RACH-less LTM cell switch	CATT	discussion	Rel-18	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

Proposal 1: Apply the restriction of the configured grant parameters as described in the LS R2-2405997 also to CG-based RACH-less LTM cell switch, if cg-LTM-Configuration is configured.

Proposal 2: Adopt the draft CR in the Appendix.

=>	Not treated

R2-2406492	Correction on retransmission in RACH-less HO	CATT	draftCR	Rel-18	38.321	18.2.0	F	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

=>	merged in R2-2407818

[AT127][007][RACH-less] CRs  (Ericsson/Huawei)

Intended outcome: Review suggested changes/agreed on which changes are necessary and update CRs accordingly.  Agree to CRs by email

Deadline:  08-23-24

#### 7.0.2.2	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-230175)

R2-2406277	Discussion on NCR capability	Huawei, HiSilicon, Intel Corporation	discussion	Rel-18	NR_netcon_repeater-Core

=>	noted

R2-2406278	Correction on NCR capability	Huawei, HiSilicon, Intel Corporation	CR	Rel-18	38.331	18.2.0	4866	-	F	NR_netcon_repeater-Core

=>	The CR is endorsed and merged in

R2-2406279	Correction on NCR capability	Huawei, HiSilicon, Intel Corporation	CR	Rel-18	38.306	18.2.0	1133	-	F	NR_netcon_repeater-Core

=>	The CR is revised in R2-2407819 and endorsed to be merged in mega CR

R2-2406415	Introduction of waveform capability for NCR-MT	ZTE Corporation (Rapporteur), Fujitsu	CR	Rel-18	38.306	18.2.0	1136	-	F	NR_netcon_repeater-Core	Revised

R2-2406416	Introduction of waveform capability for NCR-MT	ZTE Corporation (Rapporteur), Fujitsu	CR	Rel-18	38.331	18.2.0	4876	-	F	NR_netcon_repeater-Core	Revised

R2-2407524	Introduction of waveform capability for NCR-MT	ZTE Corporation (Rapporteur), Fujitsu	CR	Rel-18	38.306	18.2.0	1136	1	F	NR_netcon_repeater-Core	R2-2406415

=>	The CR is endorsed and merged in mega CR

R2-2407525	Introduction of waveform capability for NCR-MT	ZTE Corporation (Rapporteur), Fujitsu	CR	Rel-18	38.331	18.2.0	4876	1	F	NR_netcon_repeater-Core	R2-2406416

=>	The CR is endorsed and merged in mega CR

[AT127][010][NCR] Capability CRs (ZTE/Huawei)

Intended outcome: Discuss the two CRs and merge after receiving comments and endorse by email

Deadline:  08-23-24

R2-2407814	[AT127][010][NCR] Capability CRs (ZTE/Huawei)	ZTE Corporation, Huawei	discussion	Rel-18	NR_netcon_repeater-Core

=>	Noted

Agreements

- RAN2 confirm that FG2-12(multiple NS/Pmax) is not applicable to NCR-MT, no need to send LS to RAN4, UE capability Rapp can update the TR 38.822 directly.

- RAN2 to introduce an optional DFT-S-OFDM capability for NCR-MT, CRs in R2-2407524 and R2-2407525 are endorsed and merged into UE capability mega CR.

#### 7.0.2.3	NR support for UAV

Rapporteur corrections

R2-2407193	Correction for NR UAV capabilities	Huawei, HiSilicon	CR	Rel-18	38.306	18.2.0	1137	-	F	NR_UAV-Core

-	Qualcomm thinks that the existing spec is clear enough.  Samsung agrees with the intention but perhaps we don’t need to repeat the RRC text again.

=> Revised in R2-2407784

R2-2407784	Correction for NR UAV capabilities	Huawei, HiSilicon	CR	Rel-18	38.306	18.2.0	1137	1	F	NR_UAV-Core

=>	Add period at the end of the sentence when merging the mega CR

=>	The CR is endorsed and will be merged in mega CR

R2-2407196	Correction for UAV capabilities	Huawei, HiSilicon	CR	Rel-18	36.306	18.2.0	1890	-	F	LTE_UAV_enh-Core

=>	Update to category F

=>	Remove “real” from inter-operability impact

=>	There is a mismatch between 306 capability description and ASN.1 for both V2X and A2X.   We should update the description to include band combinations and that band class is provided for each band combination.

=>	LTE V2X description should also be updated in Rel-18 with magic sentence.  Qualcomm will bring the CR

=>	The CR is revised in R2-2407747 and reviewed over email

After CB session

-	Lenovo has indicate that the description doesn’t match the ASN.1 which is per band combination and indicates more things.   Qualcomm agrees that ASN.1 has more information but we copied V2X description that also signals more information.  Should we go to each field and discussing if this field is for A2X?

-	Qualcomm is concerned that if V2X and A2X have different descriptions but should have the same behaviour this causes problem.

[POST127][011][UAV] UE capabilities (Huawei)

Intended outcome: Review updated wording for LTE CR (agreed)

Deadline:  short

=> Agreed in R2-2407747 (36.306 CR#1890r1)

Other corrections

Updated reference

R2-2406597	Miscellaneous Corrections in 38.331	Ericsson	discussion	Rel-18

Proposal 1	Update the TBD in reference to the RAN4 specification for the EUTRA-NS-PmaxList IE. Adopt the TP as in the Annex.

-	Qualcomm thinks that we can remove the table references all together as maintaining the list is complicated.  We did this for LTE specs.

=>	Noted

Agreement

=>	Fix the issue by just referring to 36.101 and clause 6.

=>	Merge/Include the agreement change is Spec rapporteur CR

#### 7.0.2.4 	Mobile Terminated Small Data Transmission

(NR_NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

R2-2406304	Rapporteur MAC CR for MT-SDT	Huawei, HiSIlicon	CR	Rel-18	38.321	18.2.0	1882	-	F	NR_MT_SDT-Core

-	Xiaomi and LG think it is already clear.

=>	Update to indicate that it is only a clarification in cover page

=>	“The SDT procedure initiated for MT-SDT cannot be performed by RA-SDT (i.e., RA-SDT is not applicable as specified in clause 5.1.1b), but can be performed either by Random Access procedure (i.e. with 2-step RA type or 4-step RA type) or by configured grant Type 1 (i.e., CG-SDT).”

=>	the CR is agreed in R2-2407580 with the changes above

#### 7.0.2.5	IDC enhancements for NR and MR-DC

(NR_IDC_enh-Core; leading WG: RAN2; REL-18; WID: RP-221281)

Corrections. For smaller corrections please contact CR editor / Rapporteur directly.

R2-2406706	Miscellaneous corrections for IDC	Xiaomi	CR	Rel-18	38.331	18.2.0	4887	-	F	NR_IDC_enh-Core

=>	Include inter-operability

=>	The CR is not pursued

R2-2407292	Discussion on handling of TDM Config for IDC	Huawei, HiSilicon	discussion	Rel-18	NR_IDC_enh-Core	Withdrawn

#### 7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR

( NR_mobile_IAB -Core; leading WG: RAN3; REL-18; WID: RP-232669)

R2-2406330	Corrections on mobile IAB terminologies	Huawei, HiSilicon (Rapporteur)	CR	Rel-18	38.340	18.0.0	0036	-	F	NR_mobile_IAB-Core

-	Qualcomm thinks this is not needed, except change number 1

=>	Agree to change 1 only

=>	The CR is agreed in R2-2407748 with the agreement above and updated cover page

R2-2407170	Misc RRC corrections for mobile IAB	Ericsson (Rapportuer)	CR	Rel-18	38.331	18.2.0	4929	-	F	NR_mobile_IAB-Core

-	Huawei and Samsung don’t think the first change is needed.  Second change can be implemented in RRC editor CRs

-	Nokia is not even sure the second change is needed.

-	Huawei thinks that only the English needs to be changed for second change “may be”

=> 	The CR is not pursued

=>	the only change is related to second change and should be updated to “may be”.

=>	merge/include this agreement in RRC spec rapporteur

#### 7.0.2.7	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

R2-2407476	Correction on UE determining the clock quality information update	Huawei, HiSilicon	CR	Rel-18	38.300	18.2.0	0895	-	F	TRS_URLLC-NR-Core

-	Nokia think there is no issue as this figure is from a single gNB perspective.

-	Vivo thinks it makes the procedure more clear.

-	Samsung doesn’t think this is needed as text in bullet 4 is describing same thing

-	Ericsson also doesn’t think it is needed

=>	the CR is not pursued

#### 7.0.2.8	Others

Rapporteur corrections

R2-2407031	Miscellaneous corrections	Samsung (Rapporteur), Ericsson	CR	Rel-17	38.321	17.9.0	1897	-	F	NR_pos_enh-Core
=> The Category is updated to D and CR is agreed in R2-2407577

R2-2407032	Miscellaneous corrections	Samsung (Rapporteur), Ericsson	CR	Rel-18	38.321	18.2.0	1898	-	F	NR_pos_enh-Core, NR_MIMO_evo_DL_UL-Core

=>	The Category is updated to D and CR is agreed in R2-247578

RAN1 parameter list

R2-2407569	Reply LS on Rel-18 higher-layers parameter list	RAN2	LS out	Rel-18	NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_pos_enh2-Core, Netw_Energy_NR, NR_cov_enh2, NR_XR_enh-Core, NR_Mob_enh2, NR_BWP_wor-Core, NR_NTN_enh, IoT_NTN_enh-Core, NR_SL_enh2-Core, NR_netcon_repeater-Core, NR_DSS_enh, NR_redcap_enh-Core, TEI18	To:RAN1	Cc:RAN3, RAN4

=>	the LS is revised in R2-2407827 and approved

R2-2407570	ASN.1 names in RAN1 parameter list Rev 1	Ericsson	discussion	NR_newRAT-Core

=>	RRC/(S)LPP editors should check the email discussion and parameter lists

=>	Approved as final list to be sent to RAN1.  The document is noted.

[AT127][002][RAN1 Parameters] Updated parameter lists (Ericsson)

Intended outcome: Agree to LS by email

Deadline:  08-23-24

ToAddModList extention

R2-2407494	Discussion on extensions of ToAddModList	Samsung	discussion	Rel-18	TEI18, NR_DualTxRx_MUSIM-Core, NR_SL_enh2

=>	 Update CG-SDT-ConfigLCH-Restriction-v1800 to CG-SDT-ConfigLCH-RestrictionExt-v1800.

Proposal 2. Discuss on whether to update gapPriority-r18 in MUSIM-GapExt-v1820 which is mandatory field, to optional field with Need R, which is NBC.

-	Huawei is not sure that we should have NBC and there are other places where it is worst than here.   Lenovo thinks that Samsung is technically correct but we can do BC changes by dummifying and introducing them correctly.

-	Vivo thinks that we don’t have this case occurring and it is not critical.

=>	we will not pursue the proposal 2

Proposal 3. Discuss on whether to update Need code (from Need M to Need R) the fields in SL-FreqConfigExt-v1800.

-	Oppo thinks that we can keep the current spec.  Mediatek thinks that there are a couple of fields that don’t allow the need code to be set to False.   Ericsson thinks that this is not an ANS.1 change it is a behaviour change.

=>	Agree to make the change for all these impacted fields

=>	Noted

[AT127][003][RRC CR] ToAddModList CR (Samsung)

Intended outcome: agree to CR

Deadline:  08-23-24

R2-2407788	Correction on extension of ToAddModList	Samsung	CR	Rel-18	38.331	18.2.0	4967	-	F	TEI18, NR_SL_enh2

=>	The CR is agreed

SI request

R2-2406814	Corrections on SI request with Msg1 repetition	Lenovo	CR	Rel-18	38.331	18.2.0	4889	-	F	NR_cov_enh2-Core, NR_pos_enh2, TEI17 (moved from 7.0.2)

=>	Philips indicates that these changes overlap with 7278

=> Check offline on whether to merge them or not

=> Revised in R2-2407774

[AT127][025][SI request] 331 CR (Lenovo)

Intended outcome: agree to CR by email

Deadline:  08-23-24

R2-2407774	Corrections on SI request with Msg1 repetition	Lenovo, Huawei, HiSilicon, Philips International B.V.	CR	Rel-18	38.331	18.2.0	4889	1	F	NR_cov_enh2-Core, NR_pos_enh2, TEI17

=>	The CR is agreed

Other corrections to be treated after we breakout in parallel session on Monday

RACH resource selection

R2-2406412	Restructure of RACH resource set selection procedure in MAC	ZTE Corporation	discussion	Rel-18

Proposal 1: RAN2 to discuss the updated TP structure (Option 1 and Option 2) and select one TP from [1] and [2].

-	LG and Vivo is positive to re-structure and slight preference to option 1 but some procedures related to LTM are not accurate

-	Huawei and Vivo think that we need to discuss some cases related to LTM and 2TA.

-	Ericsson is open to simplify the text but we should have a consensus on what to resolve and what are the issues.

-	CATT also prefers option

=>	Noted

[AT127][005][RACH resources] Restructuring of selection procedure (ZTE)

Intended outcome: Discuss the technical issues and all the cases.  Start with option 1

Deadline:  08-23-24

R2-2406413	Correction on RACH resource set selection--Option 1	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1884	-	F	NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core	Revised

R2-2406414	Correction on RACH resource set selection--Option 2	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1885	-	F	NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core	Revised

R2-2407522	Correction on RACH resource set selection---Option 1	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1884	1	F	NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core	R2-2406413

=> Revised in R2-2407813

R2-2407813	Correction on RACH resource set selection---Option 1	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1884	2	F	NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core

=>	The CR is revised in R2-2407821

R2-2407821	Correction on RACH resource set selection---Option 1	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1884	3	F	NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core

=>	The CR is agreed, but then coversheet revised by MCC: wrong CR number (1885 -> 1884)

=> Revised in R2-2407877

R2-2407877	Correction on RACH resource set selection---Option 1	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1884	4	F	NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core

=> Agreed

R2-2407523	Correction on RACH resource set selection--Option 2	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1885	1	F	NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core	R2-2406414

=>	The CR is not pursued

absoluteFrequencySSB field description

R2-2407169	Correction on absoluteFrequencySSB corresponding to CD-SSB for PCell	vivo, ZTE	CR	Rel-18	38.331	18.2.0	4928	-	F	NR_BWP_wor-Core, Netw_Energy_NR-Core, NR_redcap_enh-Core, NR_redcap-Core

=>	The CR is revised with “no interoperability issue” and agreed in R2-2407828

R18 MBS support in NTN to be treated in NTN breakout session

R2-2406338	Discussion on supporting eMBS in NTN	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core, NR_MBS_enh-Core

Proposal 1: R18 eMBS (i.e., multicast reception in RRC_INACTIVE) is supported in NTN.

-	LG is ok but don’t think that new capability is needed

=>	Noted

R2-2407266	Discussion on multicast DRX to support NTN in INACTIVE	LG Electronics Inc.	discussion	NR_MBS_enh-Core

Proposal 1. The UE in INACTIVE should extend the drx-HARQ-RTT-Timer-PTM by considering the UE-gNB RTT if the MBS downlink assignments are received from the NTN.

Proposal 2. For extension of the drx-HARQ-RTT-Timer-PTM, the new capability for the UE in INACTIVE should not be introduced.

=>	Noted

R2-2406823	Correction on multicast DRX to support NTN in RRC_INACTIVE [MBSoverNTN_RRCINACTIVE]	Nokia, Qualcomm	CR	Rel-18	38.321	18.2.0	1890	-	B	NR_MBS_enh-Core, NR_NTN_enh-Core

=>	Noted

Discussion

-	ZTE is concerned that this is a whole new use case and we don’t see any requirements from operators and there may be quite a bit of impacts that are considered in Rel-19.

-	Qualcomm points out that there are a lot of supporting companies and we introduced this in terrestrial.

-	Samsung would also like to understand the use cases.    CMCC asks if this is for transparent or regenerative.   For some scenarios like moving there may be some unidentified impacts.  Qualcomm qualifies that the complicated case is being discussed in R19.   ZTE thinks that it is questionable whether it can be supported.

-	Huawei thinks it should be simple.

Agreements

=>	The UE in INACTIVE should extend the drx-HARQ-RTT-Timer-PTM by considering the UE-gNB RTT if the MBS downlink assignments are received from the NTN

=>	We will not further optimize MBS and NTN support in Inactive

=>	No new capability is introduced

R2-2407778	Correction on DRX to support eMBS in NTN	Huawei, HiSilicon, Qualcomm, LG Electronics Inc, vivo, Ericsson, Nokia	CR	Rel-18	38.321	18.2.0	1924	-	F	NR_MBS_enh-Core, NR_NTN_enh-Core

=>	The CR is agreed

R2-2407779	Correction to support eMBS in NTN	Huawei, HiSilicon, Qualcomm, LG Electronics Inc, vivo, Ericsson, Nokia	CR	Rel-18	38.306	18.2.0	1152	-	F	NR_MBS_enh-Core, NR_NTN_enh-Core

=>	The CR is endorsed and will be merged in mega CR

## 7.1	Void

## 7.2	Expanded and improved NR positioning

(NR_pos_enh2; leading WG: RAN1; REL-18; WID: RP-232670)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.2.1	Organizational

Including incoming LSs and rapporteur inputs.

Incoming LSs with “take into account” actions and no draft reply

R2-2406208	Reply LS on SL positioning measurement (R1-2405511; contact: Huawei)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN4, RAN2

- Noted

R2-2406238	Reply LS on application layer ID (S2-2407318; contact: Xiaomi)	SA2	LS in	Rel-18	Ranging_SL	To:RAN2, CT1, CT4

- Noted

Incoming LS with “take into account” action but no consensus in source group

R2-2406213	Reply LS on DL-AoD measurements in NR-PRU-DL-Info forwarded to target UE (R1-2405586; contact: Nokia)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

- Noted

Discussion:

CATT think we should follow the RAN1 parameter list, which still includes the DL-AoD measurements.

Huawei think RAN1’s reply is a euphemism for “it is not useful”, and they think we should delete the field if there is no consensus to use it.

Qualcomm understand that we asked RAN1 if the PRU measurements apply to legacy measurements, and they said yes; they understand that the use case would not be specified, but there seems no problem in keeping it in the LPP spec.

Nokia generally agree with Huawei and think that the question was specific.

MediaTek do not really see a use case but think the most painless solution may be to follow the parameter list.

ZTE agree with MediaTek and CATT that we should follow the parameter list.

Nokia wonder if we should add a “not used in this release” indication.

Ericsson would prefer to delete the field since there is no identified use case.

ZTE consider that RAN1 did not say it is not used, but there is no consensus.  They see that we also do not have consensus in RAN2 and think we should not delete it.

Nokia think we could take one more meeting to decide if any action is needed from RAN2.  ZTE think if RAN2 delete the IE now, we will have concerns later due to the parameter list not aligning.

Other incoming LSs and related documents

R2-2406207	LS on the UE role list in RSPP-Metadata (C1-243690; contact: ZTE)	CT1	LS in	Rel-18	Ranging_SL	To:RAN2	Cc:SA2

R2-2406791	Draft reply LS on the UE role list in RSPP-Metadata	ZTE Corporation	LS out	Rel-18	NR_pos_enh2	To:CT1	Cc:SA2

- “The issue is editorial and RAN2 have already fixed it in the latest 38.355-i20:” to be replaced with “The issue has already been fixed in 38.355-i20:”

- Source to be changed to RAN2 and “Draft” to be removed from the title

- Approved with these changes as R2-2407718

Discussion:

ZTE indicate that this is already fixed in our spec.

Lenovo think it is not correctly described as an “editorial” issue.

R2-2406228	LS on synchronization source change at the transmitting anchor UE in SL positioning (R4-2410352; contact: Ericsson)	RAN4	LS in	Rel-18	NR_pos_enh2-Core	To:RAN1, RAN2

- Postponed

R2-2407228	draft LS reply on synchronization source change at the transmitting anchor UE in SL positioning 	Ericsson	discussion	Rel-18	38.355	NR_pos_enh2-Core

Discussion:

Qualcomm think we added a sync source in SLPP because of a RAN1 request, so we can inform the peer UE of a sync source change by sending a new message; they see that this is already supported.  Intel agree that this indication is possible but think we never specified that it will be done, so the UE may or may not be aware of the sync change.  Huawei think the point is that there is no spec change required from our pov; the ProvideLocationInformation can be sent unsolicited if this situation occurs.

Huawei think the sync source is missing for SL-RTT.

InterDigital understand that our specification already supports the change and what happens is up to RAN1 decision.

Ericsson indicate that if the source changes, there is no requirement today that the UE sends an update.

Intel think we could wait for RAN1 to respond.  Regarding Huawei’s comment about SL-RTT, Intel understand that it was not listed in the parameter list.  Huawei agree but note that it was in the RAN4 LS.

CATT agree we could wait and let companies coordinate offline to see if the parameter list needs to be updated.  Ericsson think we could reply that we have the signalling but have not discussed the scenario.  Intel think we cannot say that we support the scenario from a functionality perspective; we have not discussed it nor specified it.

OPPO think the update has been implicitly supported, because the sync source should reflect the current situation.  They do not think we need to decide the issue in RAN2.

Qualcomm note that our signalling does not support it for SL-RTT; they would rather wait for the RAN1 response.

Rapporteur CRs

R2-2406315	Rapporteur MAC CR for R18 positioning	Huawei, HiSIlicon	CR	Rel-18	38.321	18.2.0	1883	-	F	NR_pos_enh2	R2-2406292

[AT127][404][POS] Rel-18 positioning MAC CR update (Huawei)

Scope: Merge agreements of this meeting into R2-2406315.

Intended outcome: Agreeable CR in R2-2407722

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407722	Rapporteur MAC CR for R18 positioning	Huawei, HiSIlicon	CR	Rel-18	38.321	18.2.0	1883	1	F	NR_pos_enh2	R2-2406292

Discussion:

Huawei think some post-meeting checking is needed.

Ericsson think something is needed about the definition of the order of SRS resources for aggregation.

[Post127][404][POS] Rel-18 positioning MAC CR (Huawei)

Scope: Check and update the CR in R2-2407722

Intended outcome: Agreed CR

Deadline: Short (for RP)

=> Agreed in R2-2407854 (38.321 CR#1883r2)

R2-2406950	Miscellaneous corrections to LPP specification	CATT	CR	Rel-18	37.355	18.2.0	0512	-	F	NR_pos_enh2-Core

- Merged into R2-2407720

[AT127][402][POS] Rel-18 LPP rapporteur CR merge (CATT)

Scope: Merge agreements of this meeting into the CR in R2-2406950.

Intended outcome: Agreeable CR in R2-2407720

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407720	Corrections to LPP specification	CATT, Qualcomm Incorporated, Nokia, Ericsson	CR	Rel-18	37.355	18.2.0	0512	1	F	NR_pos_enh2-Core

- Agreed

Withdrawn/Not available

R2-2406292	Rapporteur MAC CR for R18 positioning	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4868	-	F	NR_pos_enh2	Revised

### 7.2.2	Stage 2

Impact to 38.300, 37.340, and 38.305. For each specification, a single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

This agenda item may be handled at lower priority.

R2-2406508	Corrections on TS 38.305 for time windows configuration	CATT, Nokia, NSB, Ericsson, Qualcomm Incorporated	CR	Rel-18	38.305	18.2.0	0165	1	F	NR_pos_enh2-Core	R2-2404435

- “, and to ensure…” to be removed from the NOTEs

- To be merged into a rapporteur CR

- Merged into R2-2407752

Discussion:

Huawei are fine with the idea of the NOTE, but they are a bit dubious about the wording “correlated measurements”; what is correlated is the error in the measurements, not the measurements themselves.  They would like to remove the “to ensure” part.

R2-2407227	DRX and PRS alignment for positioning	Ericsson, Intel Corporation	CR	Rel-18	38.305	18.2.0	0166	1	F	NR_pos_enh2-Core	R2-2405259

- Not pursued

Discussion:

Nokia think this is an enhancement rather than a correction, and it is not clear how the gNB gets the PRS periodicity from the LMF.  They are not sure that the gNB can perform the described behaviour.  Ericsson indicate the proposal assumes the gNB knows somehow and there are NRPPa messages that could inform it.  Qualcomm indicate this only relates to the gNB’s own PRS configuration, not neighbours.

Samsung agree with Nokia and Qualcomm that there is no neighbour information available to the gNB; they understand that RAN2 agreed only on using the OD-PRS procedure for this alignment, so if we capture something, we should capture the UE-side solution, not the gNB-side solution.

Huawei have a similar understanding to Samsung; they thought there was no specification impact because we agreed to use the UE-initiated request, but maybe this case could be reflected as well.

CATT think the previous agreement (NOTE 6 under OD-PRS transmission) already captures the UE-side case.

Ericsson think we did discuss the possibility of the gNB implementation doing something.

Huawei think the gNB may change the PRS cycle but it cannot change the DRX cycle to align with it.  Ericsson’s understanding is that the gNB may select a suitable (e)DRX cycle from multiple options.

R2-2407234	Correction of Notes for Assistance Data Transfer procedures	Philips International B.V.	discussion	Rel-18	NR_pos_enh2

- First change is not pursued

- Second change to be merged into rapporteur CR in R2-2407752

- Proposals to capture other scenarios can be seen in future if well-motivated

Proposal 1: Revise Note 2 to: "NOTE 2: Dependent on the scenario, Endpoint A may be a SL Anchor UE and Endpoint B may be a Target UE”

Discussion:

Qualcomm think the existing text is correct for delivering the capabilities/configurations of the anchor UEs to the server UE.  Chair has the same understanding.

CATT think the change is not correct because the anchor may not be connected to the server.

ZTE think a NOTE 4 could be added to capture the additional case described here.  Qualcomm think this case is covered already by NOTE 3, and they think we could consider adding more NOTEs but we should not change the existing ones.

Proposal 2: Revise Note 3 to: “NOTE 3: Dependent on the scenario, Endpoint A may be a SL-PRS receiving (Rx) UE and Endpoint B may be a SL-PRS transmitting (Tx) UE.”

Discussion:

Qualcomm think this is correcting a typo.

ZTE think NOTE 3 is confusing because of not being written in terms of UE roles.

[AT127][406][POS] Rel-18 positioning 38.305 rapporteur CR (Qualcomm)

Scope: Generate a rapporteur CR capturing agreements of this meeting.

Intended outcome: Agreeable CR in R2-2407752

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407752	Correction to LPP Measurement Time Windows and SLPP Assistance Data Transfer procedures	Qualcomm Incorporated	CR	Rel-18	38.305	18.2.0	0172	-	F	NR_pos_enh2-Core	R2-2405259

- Agreed

### 7.2.3	SLPP corrections

Impact to 38.355. A single CR with miscellaneous corrections is requested from the spec rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Email discussion summary

R2-2406374	[Post126][410] 38.355 update Open Issue list	Intel Corporation	discussion	Rel-18	NR_pos_enh2-Core

- Noted

Proposal: All open issues raised in the previous meetings have been resolved. The only potential issue is RelativeVelocityWithUncertainty which depends on SA2 discussion.

Rapporteur CR

R2-2406375	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.2.0	0005	-	F	NR_pos_enh2-Core

[AT127][401][POS] SLPP rapporteur CR update (Intel)

Scope: Email checking of agreeable updates to the CR in R2-2406375.  Take into account the question of whether NBC changes should be accepted.

Intended outcome: Agreeable CR in R2-2407719

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407719	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.2.0	0005	1	F	NR_pos_enh2-Core

- Revision number should be one digit

- Remove the duplicate reference 17

- Fix typos for “direction” in LocationCoordinateTypes

- Agreed with these changes as R2-2407753

Discussion:

Lenovo think there are still a few issues: coversheet has the wrong format for the revision number, reference 17 is a duplicate of reference 8, and in section 6.5 the new IE LocationCoordinateTypes has some typos in the field names (“driection”).

R2-2407753	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.2.0	0005	2	F	NR_pos_enh2-Core

- Agreed unseen, but issues discovered afterwards with the coversheet (“non-backwards compatible” instead of “mandatory for the concerned functionality”)

- Secretary will amend the coversheet in post-meeting processing

=> Revised in R2-2407872

R2-2407872	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.2.0	0005	3	F	NR_pos_enh2-Core

=> Agreed

Other contributions

R2-2406294	Discussion on the remaining issues for SLPP for R18 POS	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

- Noted

Proposal1: Confirm on the understanding that when the synchronization source changes, it is up to the UE’s implementation the Tx side of the SL-PRS sends another provideAssistanceInformation SLPP message to information the Rx side of the update. No spec change required.

Proposal2: Add synchronization sources as assistance data in SL-RTT-ProvideAsssitanceData for SL-RTT. Adopt the TP in Annex A.

Proposal3: Capture in the field description of SL-RTT-ProvideLocationInformation SLPP message that when the ARP for SL-PRS transmission and reception are different, the UE shall not report SL Rx-TX time difference measurement. Adopt the TP in the Annex B.

Proposal4: Support multiple per ARP measurement results for the same SL-PRS in a single report. Adopt the TP in Annex C.

Proposal5: Confirm that provideLocationInfomation can be provided in un-solicited manner and the concern raised from RAN1 is not an issue.

Discussion:

Qualcomm and Lenovo understand that the related LS was already treated and we replied to the LS and captured some of the proposals.

Intel confirm that SLPP was updated in line with the discussion last meeting.

Huawei think P3 is still open and relates to a RAN1/RAN2 misalignment.  Qualcomm think the RAN1 agreement implies that the UE does not even measure the Rx-Tx time difference in this case, so there is no question of whether to report it.  Intel have the same understanding.

R2-2406516	Clarification on RSPP metadata for direct communication request message	ASUSTeK	CR	Rel-18	38.355	18.2.0	0007	-	F	NR_pos_enh2

- Postponed

Discussion:

Lenovo indicate that stage 2 (23.586) suggests it can be in the DCR/DCA messages, but the corresponding stage 3 does not seem to match it, so they think there is a stage 2/3 misalignment in CT/SA groups.  Huawei understand from CT side that CT1 think the CR is correct.

Intel agree with Lenovo and think we should not change our spec for something that is not captured in stage 3.  If we do something, Intel think we should word our descriptions in a more general way.

Huawei agree that a better formulation would refer to the spec instead of the messages.

Lenovo think it may not be easy to write the field description in a message-agnostic way.

R2-2406809	Miscellaneous corrections on SLPP	Lenovo	draftCR	Rel-18	38.355	18.2.0	NR_pos_enh2

- To be merged into rapporteur CR (subject to offline review)

- Merged into R2-2407719

R2-2407146	Correction to SLPP PDU Common Contents	Qualcomm Incorporated	draftCR	Rel-18	38.355	18.2.0	F	NR_pos_enh2-Core

- To be merged into rapporteur CR (subject to offline review)

- Email discussion to confirm acceptability of NBC changes

- Merged into R2-2407719

Discussion:

Qualcomm clarify changes 1 and 2 are NBC.

Intel agree with the changes technically but think we need to decide if we take NBC changes.

CATT support an NBC change as it is an early stage of the release.

Ericsson would also be fine with an NBC change.

Intel wonder if the capability should be per-method.  Qualcomm understand that not all shapes are applicable to all methods; for example, relative velocity will not come from all the methods.  Qualcomm also note that it is per-method in LPP.

Qualcomm think the LPP/SLPP bar may not be as high as RRC.  Lenovo note that we already agreed an NBC change in the main session.

R2-2407369	Correction on SL-RTT-provideLocationInformation field description in TS 38.355	CATT, CICTCI	draftCR	Rel-18	38.355	18.2.0	F	NR_pos_enh2-Core

- To be merged into rapporteur CR (subject to offline review, and proponent to submit updated language for the field description)

- Merged into R2-2407719

Discussion:

Intel understand that we only need the field description if it provides some additional information.

Qualcomm think in this case it is useful to have a field description, but the text proposed here is not entirely clear: “associated” to what?

Lenovo think the language is not fully clear, and it would be better to capture the RAN1 agreement language.

CATT think the meaning of the field is not clear from other places in the spec, so the field description is needed.

ZTE understand that the feature is new in RAN1, different from the legacy way of determining the time, so they think it should be captured; they think “associated” is fine, but we could add “actual transmission timestamp”.

Withdrawn/Not available

R2-2406509	Correction on SL-RTT-provideLocationInformation field description in TS 38.355	CATT,CICTCI	CR	Rel-18	38.355	18.2.0	0006	-	F	NR_pos_enh2-Core	Withdrawn

### 7.2.4	LPP corrections

Impact to 37.355. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2407149	Editorial clean up of NR-UL-SRS-Capability	Qualcomm Incorporated	draftCR	Rel-18	37.355	18.2.0	F	NR_pos_enh2-Core

- To be merged into rapporteur CR

- Spelling/formatting corrections to be checked for parallel issues in RRC by RRC CR rapporteur

Discussion:

Lenovo think the changes are OK, but some field names are changed and they may appear also in RRC.

R2-2407230	Corrections related to carrier phase measurements	Ericsson	draftCR	Rel-18	37.355	18.2.0	F	NR_pos_enh2-Core

- Not pursued

Discussion:

CATT think there is no matching agreement in RAN1 that any carrier phase measurement should be performed in a time window.  Huawei have the same view as CATT and understand from RAN1 side that the measurement does not have to happen within a window.

Nokia find the previous comments a bit confusing, and they think the mechanism for requesting the measurements is not fully clear.

ZTE agree with Nokia that the issues of the time window and the request bitmap are separate; they think we cannot delete the request bit, and if there is a change it should come from RAN1.

Nokia think the concern underlying the CR about having the request in two places may be legitimate, but they would prefer that we remove it from the time window configuration and use the request bitmap.

R2-2407253	On-demand DL-PRS bandwidth aggregation corrections	Nokia	CR	Rel-18	37.355	18.2.0	0515	-	F	NR_pos_enh2-Core

- Changes 2 and 3 from coversheet to be merged into rapporteur CR

- Rapporteur CR will also address the undefined IE issue

Discussion:

CATT clarify that the undefined IE issue is raised in the rapporteur CR as well, but they do not see that there is any broken functionality or lack of clarity related to the other changes.

Nokia are OK to remove the first field description, but they think the clarifications to the other fields are important.

ZTE think we should not adopt the Rel-17 changes, but they think the Rel-18 change is correct because the field description does not reflect the two-level structure of the field.

### 7.2.5	RRC corrections

Impact to 38.331, except for UE capabilities. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CR

R2-2407221	Miscellaneous Positioning Corrections	Ericsson	CR	Rel-18	38.331	18.2.0	4934	-	F	NR_pos_enh2-Core

- Revised in R2-247721

[AT127][403][POS] Rel-18 positioning RRC CR update (Ericsson)

Scope: Update R2-2407221 in line with decisions of the meeting.

Intended outcome: Agreeable CR in R2-2407721, with mandatory changes in a separate CR in R2-2407769

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407721	Miscellaneous Positioning Corrections	Ericsson, Philips International, vivo, CATT	CR	Rel-18	38.331	18.2.0	4934	1	F	NR_pos_enh2-Core

Discussion:

Huawei wonder why there are two CRs; they think functional NBC need not be traced separately.  Ericsson understood that it was preferable to have the separation.  Huawei think we should be consistent across WIs.

Lenovo think it is OK to handle case by case, and here the first one is more editorial and the second has functional changes.  They also did not have time for a full review and think there may still be some ASN.1 issues.

R2-2407769	Extension of cells in Validity area and correction of SL CBR Range parameters	Ericsson, CATT, ZTE Corporation	CR	Rel-18	38.331	18.2.0	4965	-	F	NR_pos_enh2-Core

Discussion:

Ericsson indicate a comment was received from the RRC spec rapporteur that we should consider dummifying the old list.

[Post127][405][POS] Rel-18 positioning RRC CRs (Ericsson)

Scope: Check the CRs in R2-2407721 and R2-2407769, including confirming list extension mechanism for R2-2407769.

Intended outcome: Agreed CRs

Deadline: Short (for RP)

=> Agreed in:

R2-2407883 (38.331 CR#4934r3)

R2-2407884 (38.331 CR#4965r2)

=> Noted in:

R2-2407863 (Summary)

Other contributions

R2-2406405	Miscellaneous Corrections for sidelink positioning	vivo	draftCR	Rel-18	38.331	18.2.0	F	FS_NR_pos_enh2

- Changes 1, 3, 4, and 5 to be merged into rapporteur CR

- Changes 2 and 6 are postponed

Discussion:

Huawei think the intention was to introduce nothing special for dedicated pool for positioning, so they are not sure the last change is strictly needed.  vivo clarify that they understand this is not captured in any RAN1 spec, so the network restriction is needed here.

Huawei think we already discussed the condition for RRCResumeRequest, and it was not adopted before the ASN.1 freeze because there was a view that the connection needs to be established before positioning; they understand that vivo are trying to address the case of a normal resource pool, and they are not sure this needs to be addressed since the normal resource pool can be provided by system information.

vivo indicate that the CR is directed to resuming the RRC connection, separate from establishing the PC5-RRC connection.  The intention is that the UE would resume the RRC connection if it does not receive the resource pool for SL-PRS.

Huawei think establishing the PC5-RRC connection will also result in an RRCResumeRequest, and they are not sure what will happen if the UE requests repeatedly.

Ericsson think a normal UE implementation would behave as in the CR, but we may not need to make it explicit.

Huawei are not sure that the CR addresses the concern; they think we may need some input from sidelink experts on when the pools will be available, because the pool may always be there in the SIB.  vivo indicate they followed the legacy logic for the case where the frequency is valid but the normal resource pool is not provided.

Huawei’s concern is that the RRCResumeRequest does not indicate that the lack of SL-PRS resource pool is the cause, and the network may not react, causing a repeated request.  vivo think this issue is general for sidelink operations, and the UE can indicate its preference for SL-PRS transmission to the network after resuming.

Ericsson think change 6 needs to be checked with RAN1.

R2-2406793	Correction on SRS transmission in RRC_INACTIVE	ZTE Corporation	draftCR	Rel-18	38.331	18.2.0	F	NR_pos_enh2

- Constant to be replaced with maxNrOfCellsInVA-Ext-r18 (also equal to 16)

- No capability is introduced

- To be merged into rapporteur CR

- Change will be indicated as mandatory for UEs and networks implementing the concerned functionality

Discussion:

Huawei think this should be done with an NCE.

CATT agree with the intention but think the sequence should not reuse the previous constant.

Huawei think the extension mechanism may be OK but the field description should be clear that it is an NCE.  ZTE indicate this is already in the CR.

Ericsson think the new constant can be clarified in the field description.

Samsung wonder if we would need a capability.  ZTE think this may be  necessary.

Lenovo think we normally add capabilities if we introduce something at a later stage, but it is not that critical to have one for a change within Rel-18 now.

Ericsson wonder if it would be easier for RAN3 to align with RAN2, or even to have the gNB downselect from the list provided by the LMF.

CATT think 32 cells makes sense in light of the RNA size and we should follow the RAN3 agreement.  To Lenovo’s question, they understand that for interoperability we should have a capability to indicate the extension.

Ericsson have some sympathy for Lenovo’s view and think we could indicate the CR as mandatory.  They would be OK with the functional requirement but do not want to have NBC ASN.1.

Huawei are OK with not introducing a capability.  They think we should be somewhat consistent about capability introductions for functional changes.

CATT agree that we need guidance on the capability.

R2-2407273	RRC correction on NR sidelink positioning	Philips International B.V.	CR	Rel-18	38.331	18.2.0	4940	-	F	NR_pos_enh2-Core

- Change 11 is postponed

- Other changes to be merged into the rapporteur CR

Discussion:

On change 11, vivo think the intention is right but the affected field is only used for sidelink communication.  Huawei agree.

ZTE indicate that SIB23 currently only contains one frequency and SIB12 can contain multiple frequencies; they think the UE should be able to request frequencies from both.  vivo agree but think this field is related only to the frequency, not the resource pool.

Ericsson clarify that the issue is whether to remove “/positioning” in the description of sl-TxInterestedFreqList.

Huawei wonder how we support requests for multiple frequencies in CA.

ZTE think removing “/positioning” would mean the UE could not indicate interest in the shared pool.  vivo wonder how the network differentiates between communication and positioning in case the same field is used to request both.

Philips think based on the RAN1 spec the network should be able to tell.  vivo are not sure how this would work.

Ericsson think we may need to extend with an indication of whether the request for shared pool is for communication or positioning.  Huawei think this would be overdoing it, because the intention of the shared pool is to piggyback SL-PRS transmission on the data.  ZTE think there are additional configuration parameters that the network needs to send to the UE for SL-PRS.

vivo think we could use sl-PosTxInterestedFreqList with an additional mapping to SIB12.

Extension of CBR ranges (NBC and BC alternatives)

R2-2406510	Corrections on SL positioning in TS 38.331	CATT	CR	Rel-18	38.331	18.2.0	4879	-	F	NR_pos_enh2-Core

- Changes 3 and 4 are merged into the rapporteur CR

R2-2407559	Corrections for the extension of these IEs which do not support the maximum number of CBR ranges and levels for sidelink positioning in TS 38.331	CATT, Ericsson	draftCR	Rel-18	38.331	18.2.0	F	FS_NR_pos_enh2

- Dummification to replace the field names completely as usual

- No capability is introduced

- To be merged into the rapporteur CR

- Change will be indicated as mandatory for UEs and networks implementing the concerned functionality

Discussion:

Huawei think we should avoid syntax NBC changes.

Ericsson wonder if we need the capability or assume support of the extension is mandatory.  Chair understands from the previous discussion that we can accept a functional NBC but keep the syntax the same.

Intel think the field names should be changed to “dummy” / “dummyX”.

### 7.2.6	MAC corrections

Impact to 38.321. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2406293	Discussion on the remaining issues for MAC for R18 POS	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

- TP to be merged into rapporteur CR

- Noted

Proposal1: Clarify that Spatial Relation for Resource IDi indicates the spatial relation of the ith SRS resources within the first CC that is aggregated with the other SRS resource in the other CCs. Adopt the TP in Annex A.

Discussion:

Qualcomm are not sure how the mapping is intended to work (figure at the end of section 2.1 in the document).  Huawei indicate RAN1 specs should be enough to clarify it.

ZTE are OK with Huawei’s wording, but they think there are some mistaken field names in the TP.

Huawei note that the reference to 38.214 should also be added.

Samsung are also OK with the proposal but would like to specify “first activated CC”.  Huawei are OK with this change.

R2-2406376	eLCID for SL-PRS Resource Request MAC CE	Intel Corporation	draftCR	Rel-18	38.321	18.2.0	NR_pos_enh2-Core

- To be merged into rapporteur CR

Discussion:

Huawei agree the change is correct.

R2-2406404	Correction on UE behavior of SL-PRS transmission	vivo	draftCR	Rel-18	38.321	18.2.0	F	FS_NR_pos_enh2

- Add a NOTE saying that for the carriers configured in SIB12 and for which SL-PRS transmission is allowed, the UE selects one carrier for SL-PRS from among the selected candidate data carriers, and which one it selects is up to UE implementation.

- NOTE to be captured in rapporteur CR

Discussion:

ZTE do not agree with the CR; they think the changes mentioned have not been discussed in RAN2 and it would be more natural to let the UE select the shared pool frequency, and it would be enough to add a NOTE saying the UE selects a frequency among the selected data frequencies for shared pool.

Huawei agree with ZTE that the use of CBR to select the carrier has not been agreed; there was such an agreement for sidelink communication but not for positioning.  They also understand that part of the intention of using the shared pool was to minimize impact.

InterDigital agree with Huawei and think it is strange to use CBR in relation to PRS.  They think it can be handled by network implementation.

vivo wonder how the UE will know how to select the carrier.

ZTE think the UE can use the CBR to select multiple carriers for data, and then transmit shared pool SL-PRS within those frequencies; they see it as up to UE implementation which frequencies to select.

vivo think a NOTE should be added to clarify that the UE selects only one carrier to transmit SL-PRS.  Huawei wonder if we should let the SL-PRS affect the selection of carriers in CA.  vivo understand that such an effect is not the intention; the point is that the UE will select the data carriers and then select the SL-PRS carrier from among them.

ZTE are OK with adding a NOTE that reflects the RAN1 agreement; they understand it should be only for shared pool.

R2-2406792	Correction on SL pos in dedicated pool and SRS aggregation MAC CE in MAC spec	ZTE Corporation	draftCR	Rel-18	38.321	18.2.0	F	NR_pos_enh2

- Changes 1 and 2 are merged into the rapporteur CR

- Change 4 and related issues to be discussed by email

Discussion:

Huawei think changes 1 and 2 are fine and 3 is already in the rapporteur paper.  For the fourth change, it needs to be considered jointly with the discussion from earlier about whether these fields can be used in RRC_INACTIVE.

Huawei think we could handle this issue offline.

[AT127][405][POS] C field in relation to cell ID and BWP ID in RRC_INACTIVE (ZTE)

Scope: Discuss change 4 from R2-2406792 and the related issues on handling of the cell ID and BWP ID fields in RRC_INACTIVE, including Rel-17 and Rel-18 impact.

Intended outcome: Report to CB session in R2-2407723

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407723	[AT127][405][POS] C field in relation to cell ID and BWP ID in RRC_INACTIVE (ZTE)	ZTE	discussion	Rel-18	NR_pos_enh2

- Noted

Proposal 1: For (re)using MAC CE in RRC_INACTIVE, RAN2 to adopt one of the following solutions:

	Solution 1: RAN2 confirms that all of the RS types (CSI-RS, SSB, SRS, DL-PRS) can be valid spatial relation RS in RRC_INACTIVE, so gNB should be able to indicate cell ID/BWP ID directly in spatial relation field, where the indicated cell ID/BWP ID are the stored value in UE Inactive AS context.

	Solution 2: RAN2 confirms that when the (aggregated)SP-SRS activation/deactivation MAC CE is used in RRC_INACTIVE, UE should NOT use CSI-RS for spatial relation RS, and UE should ignore the cell ID/BWP ID in spatial relation field when the spatial relation RS is SRS;

	Soulution 3: RAN2 sends a LS to ask RAN1 that: whether UE can take the CSI-RS or SRS configuration provided in RRC_CONNECTED as the spatial relation RS to be applied in RRC_INACTIVE state.

Discussion:

Qualcomm are not sure about solution 1 and suspect only solution 2 is feasible; an RS configuration from connected state may not be usable in RRC_INACTIVE.  Huawei agree with Qualcomm and think the configuration should not actually be applied in RRC_INACTIVE, although it is stored in the AS context.  Huawei think there would be RRC impact from solution 1 as well.

Nokia think we should know clearly what RS can be used and the safe option is to ask RAN1.

ZTE indicate that solution 1 is based on finding no proof in RRC spec that the RS configuration from RRC_CONNECTED cannot be used in RRC_INACTIVE; it is stored in the AS context and it is not clear if it cannot be used.  They think sending an LS to RAN1 could be workable, because they understand that RAN1 have not discussed the issue.

Ericsson think it is up to gNB: The network can bring the UE to RRC_CONNECTED and provide the related configurations in the SuspendConfig, so they think solution 1 can work.

Intel also think only the inactive configuration can be used in inactive, and they are also OK to check with RAN1.

CATT checked the PHY spec and came to the same conclusion that CSI-RS only works in RRC_CONNECTED.

Huawei understand in relation to ZTE’s comment that the configuration cannot be used as a reference because it is only stored in AS context, not used in RRC_INACTIVE.  They think if we write to RAN2 we need to give some explanation of the status in RAN2.

ZTE do not intend to use CSI-RS in RRC_INACTIVE, but they think the UE can use the beam of the stored CSI-RS as a reference.

Huawei consider that only the configurations signalled in SuspendConfig can be used.

[Post127][403][POS] LS to RAN1 on CSI-RS for spatial relation in RRC_INACTIVE (ZTE)

Scope: Draft an LS to RAN1 explaining the situation in RAN2 in relation to using RS from connected configuration as reference for spatial relation in RRC_INACTIVE, and ask whether UE can take the CSI-RS or SRS configuration provided in RRC_CONNECTED as the spatial relation RS to be applied in RRC_INACTIVE state.

Intended outcome: Approved LS

Deadline: Short (not for RP)

=> Approved in R2-2407853 (LSout)

R2-2406855	SL-PRS Resource Request MAC CE in the logical channel prioritization list	Samsung	CR	Rel-18	38.321	18.2.0	1891	-	F	NR_pos_enh2

- To be merged into rapporteur CR

Discussion:

Huawei agree the change is correct.

R2-2407232	Aggregated Resource Definition and corrections	Ericsson	discussion	Rel-18	38.321	NR_pos_enh2-Core

- Noted

Proposal 2	RAN2 to agree to change from at least one to at least two in Spatial Relation for Resource IDi description.

Discussion:

Huawei wonder if it is possible to indicate just one spatial relation.  Ericsson understand that at least two carriers are needed.  Huawei think the interpretation in the document is wrong; only one spatial relation is a possible case.

Qualcomm understand that activating only one carrier of an aggregated set is valid.

R2-2407296	Corrections on SL-PRS	ASUSTeK	discussion	NR_pos_enh2

- Second change to be merged into rapporteur CR

- Intention of the third and fourth changes is agreeable, with details to be checked in the rapporteur CR discussion

Discussion:

Huawei think the second change is fine; they think the third change may be in the wrong section but need to check offline, and they think the intention is OK.

Huawei indicate for the fourth change, the priority should be indicated for each SL-PRS.  ASUSTeK agree but point out the change is for the priority of the SR.  Huawei would like to check offline.

### 7.2.7	UE capabilities

Impact to 38.306 and capability-related impact to 38.331. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2406810	Addition of capability sl-PathlossBasedOLPC-SL-RSRP-Report-r18 (FG R1 41-1-17) in UECapabilityInformation message	Lenovo	draftCR	Rel-18	38.331	18.2.0	NR_pos_enh2

- Endorsed to be captured by the capability rapporteur CR

R2-2407560	Corrections on the UE capability of indication on supporting the extension for sidelink positioning in TS 38.306	CATT, Ericsson	draftCR	Rel-18	38.306	18.2.0	F	NR_pos_enh2-Core

=> Not pursued

### 7.2.8	Corrections to other specifications

Impact to any specifications not identified above.

## 7.3	Network energy savings for NR

(Netw_Energy_NR -Core; leading WG: RAN1; REL-18; WID: RP-223540)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.3.1	Organizational

LS, workplan, email discussion etc

Spec rapporteurs are expected to submit additional contribution on open issues to conclude WI by December

R2-2406603	Network energy savings for NR miscellaneous RRC CR	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4881	-	F	Netw_Energy_NR-Core

=>	the CR is agreed

### 7.3.2	Other

SFN timing of Cell DTX/DRX

R2-2406999	Discussion on the clarification of the SFN on Cell DTXDRX	CATT	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2: RAN2 discuss which of the following cases can be used for Cell DTX/DRX:

-	Case1: using the SFN of SpCell for Cell DTX/DRX

-	Case2: using the SFN of each serving cell for Cell DTX/DRX

=>	Noted

Discussion

-	CATT, Qualcomm, Apple, Interdigital, Oppo, and Huawei think case 1 is preferable.

-	LG and ZTE think that case 2 is better as this is a serving cell feature.

-	Interdigital thinks both cases work as long as network has same understanding, but case 1 is simpler.

-	Apple thinks that case 2 would result in new UE behavior.

Agreements

=>	for cell DTX/DRX will use the SFN of SpCell for Cell DTX/DRX.  Will add a note to specify.

R2-2406668	Remaining issue on SFN timing of Cell DTX/DRX	Apple	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: RAN2 down-select between two alternative way-forward on SFN timing issue of Cell DTX/DRX:

Alt-1: RAN2 confirm asynchronized CA is not in Rel-18 scope, and thereby Cell DTX/DRX is not configured together with asynchronized CA in Rel-18.

Alt-2: Cell DTX/DRX uses the SFN of primary cell (i.e., same as UE CDRX), and NW takes the timing difference into consideration when configuring SPS/CG whose SFN is from the concerned serving cell.

=>	Noted

R2-2406602	Discussion on the SFN issues of Cell DTX/DRX	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: In case the SFN is unaligned across carriers, the UE uses the same principle as in C-DRX for on-duration calculations.

=>	Noted

Other Issues

R2-2406917	Clarification for SP CSI reporting on PUCCH	Samsung	discussion	Rel-18

Proposal 1: All the sub-configurations, if any, of a configured Semi-presistent CSI reporting on PUCCH are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. Adopt text proposal 1 as provided.

-`	LG, InterDigital and Vivo doesn’t think this is needed as the configuration contains the sub-configuration so it is implied.

=>	no need to clarify

=>	Noted

R2-2407060	NW identifying RACH for emergency call	Nokia, Nokia Shanghai Bell	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: RAN2 to confirm that at least for the case when there is dedicated SR configuration, the NW can identify RACH triggered during Cell DTX non-active period is for emergency call if there is NAS request.

-	Interdigital explains that this is already specified

Proposal 2: RAN2 to confirm if RA-SR is also prevented during Cell DTX non-active period so that the NW can identify RACH triggered during Cell DTX non-active period is only for emergency call.

-	Interdigital, Oppo, Apple doesn’t think we need to specify anything as the network has the tools to identify if there is an emergency call.   Nokia explains that the network may not know that it is for emergency call as other data arrival can trigger RACH.  Apple thinks that in message 5 the NAS container will indicate that there is an emergency call.   The spec is not broken.

-	LG supports this proposal.

=>	Noted

Agreements

-	RAN2 confirms the understanding that at least for the case when there is a SR configuration, the NW can identify RACH triggered during Cell DTX non-active period is for emergency call if there is NAS request.

R2-2407070	Handling of inter-band SSB-less configuration	Ericsson	discussion

Proposal 1	Clarify in 38.331 that the “default cell” behaviour applied upon absence of the referenceCell field is only applicable if both referenceCell and absoluteFrequencySSB are absent for an inter-band SSB-less SCell.

=>	Not treated

R2-2407128	Coexistence of Cell DTXDRX and RACH-less LTM	Rakuten Mobile, Inc	discussion	Rel-18

Proposal 1	When a serving gNB-DU expects a UE to undergo LTM cell switch, the UE is configured with an extended active duration or permitted to monitor PDCCH even during Cell DTX.

-	Huawei and Interdigital think this is a new functionality and it is late in the release.  Qualcomm thinks this is an optimization and not feasible at the moment.

-	LG thinks that this is not needed as the source cell can do different things to improve the LTM performance.

Proposal 2	RAN2 agrees that a serving gNB-DU can perform LTM cell switch even during CEL

=>	Noted

[AT127][014][NES] MAC CR (InterDigital)

Intended outcome: Agree by email to CR capturing agreements from this meed

Deadline:  08-23-24

R2-2407766	Miscellaneous MAC corrections for network energy savings	InterDigital	CR	Rel-18	38.321	18.2.0	1921	-	F	Netw_Energy_NR-Core

-	need to use the correct template next time

=>	The CR is revised R2-2407801

R2-2407801	Miscellaneous MAC corrections for network energy savings	InterDigital	CR	Rel-18	38.321	18.2.0	1921	1	F	Netw_Energy_NR-Core

=>	The CR is agreed, but then coversheet revised by MCC: filename starts with "Draft"

=> Revised in R2-2407881

R2-2407881	Miscellaneous MAC corrections for network energy savings	InterDigital	CR	Rel-18	38.321	18.2.0	1921	2	F	Netw_Energy_NR-Core

=> Agreed

## 7.4	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

Time budget: 0 TU)

Tdoc Limitation: 2 tdocs.

### 7.4.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2406217	Reply LS on intra-SN SCPAC in MN format (R3-243775; contact: ZTE)	RAN3	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

- Noted.

R2-2406227	Reply LS on LTM L1 intra and inter-frequency measurements (R4-2410303; contact: Ericsson)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2	Cc:RAN1

- Noted.

### 7.4.2	Control plane corrections

Including stage 2 and control plane (e.g. RRC) corrections (including [Post126][514][R18MobE]). A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note RRC CR rapporteur’s summary and suggestion (based on the submitted contributions) may be provided.

UE measured TA:

P4, P5, P6 and P7 in R2-2406337 (MediaTek)

Proposal 4: In clause 5.3.5.18.6, correct the procedural text so that the UE stops the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

- Noted. RRC CR rapporteur will see if anything is missed in CR preparation.

[LG]: Whether to start/stop UE-based TA measurement is up to UE implementation. [Session chair]: Based on the proposed TP, “else” includes “if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID”, then shouldn’t the UE continue UE-based TA (or the UE should be configured with UE-based TA)? [Qualcomm]: Nothing is broken even though the UE performs UE-based TA for SpCell. [Vivo]: Understand the motivation, however the TP seems not so accurate.

Proposal 5: In clause 5.3.5.18.6, correct the procedural text so that the UE does not start the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

- Intention is agreed and we will add a note. Detailed wordings can be discussed as part of RRC CR preparation (taking Qualcomm’s suggestion into account).

[Qualcomm]: We can simply add a note, e.g. UE is not expected to perform UE-based TA measurement for SpCell (instead of updating many changes in the procedure).

Proposal 6: In clause 5.3.5.18.6, remove the unnecessary procedural text to stop the UE-based TA measurements when the LTM candidate which becomes the current SpCell does not have field ltm-UE-MeasuredTA-ID.

- Noted. RRC CR rapporteur will see if anything is missed in CR preparation.

Proposal 7: RAN2 to discuss if and how to capture handling of the UE-based TA measurements for the scenario where the SpCell changes by L3 mobility procedure (handover/PSCell change) while LTM is configured.

- Not pursued.

[Qualcomm]: Upon L3 HO execution, does the UE still keep LTM configuration? [MediaTek]: Yes. We can have similar texts at L3 HO. [CATT, OPPO, LG, Apple]: Understand the intention, however thinks nothing is needed. We can count on NW implementation. We have similar situation for SCPAC and we count on NW implementation. NW configuration should be reasonable/updated accordingly.

Proposal 1: RAN2 to clarify that if there is only one TCI state in ltm-CandidateConfig for the candidate cell indicated by LTM Cell Switch Command MAC CE, the network ensures this TCI state is identical with the TCI state indicated by LTM Cell Switch Command MAC CE.

- Intention is correct. No spec change is needed.

P1a, P2, P3 in R2-2407200 (Huawei)

Proposal 1a: Confirm the understanding that the information to lower layers that the UE is configured with UE-based TA measurements for a candidate remains valid after LTM cell switch, if there is no new explicit information.

- Agreed.

Proposal 2: Capture in the LTM candidate configuration addition/modification procedures that, when ltm-UE-MeasuredTA-ID is not present for a LTM candidate configuration, the UE informs lower layers that UE-based TA measurement is not configured. (See Annex TP)

- Agreed with intention. Detailed wordings and MediaTek’s concerned case will be further discussed as part of RRC CR preparation.

[MediaTek]: It is defined as optional, M, so if not removed earlier, the information should be continued even though it is absent. [Huawei]: We have a case it is removed and then added again. In the case, it should inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

Proposal 3: Capture that when the network configures LTM candidates with ltm-UE-MeasuredTA-ID, the network ensures that the UE has a value stored in ltm-ServingCellUE-MeasuredTA-ID.

- Agreed. It can be included in the corresponding field description.

LTM Configuration:

P1 in R2-2406726 (Apple)

Proposal 1: RAN2 clarify whether the nested ltm-Config is supported or not.

- Nested ltm-Config (in both reference configuration and candidate configuration) is not supported.

- Add the clarification above into the corresponding field description.

[MediaTek]: Understand the nested LTM-config is not the intention in R18 LTM design, and it can not work in subsequent LTM. [Vivo]: Ok with the conclusion. Wonders what’s spec impact. Adding the corresponding clarification would be sufficient.

Proposal 2: If the nested ltm-Config is supported, RAN2 confirm the understanding in Observation 3 and Observation 4 on how to handle the nested ltm-Config.

Observation 3: According to current spec description, the nested ltm-Config in ltm-ReferenceConfiguration is useless.

Observation 4: According to current spec description, UE will update the ltm-Config by the nested ltm-Config in ltm-CandidateConfig at LTM cell switch execution.

Proposal 3: Confirm that that network may provide the ltm-Config without ltm-CSI-ResourceConfig.

- Agreed.

SpCell reporting:

P2 in R2-2406337 (MediaTek)

Proposal 2: In the field description of spcellInclusion, clarify that it cannot be set to True if SpCell resources are not included in LTM-CSI-ResourceConfig.

- Agreed.

[OPPO, Samsung]: Nothing is broken. The existing condition for spcellInclusion is when SpCell is included in candidate LTM configuration, which sounds already cover proposal 2. [MediaTek]: “SpCell is included in candidate LTM configuration” doesn’t mean SpCell resources are included in LTM-CSI-ResourceConfiguration. [Huawei, Vivo]: Support the proposal.

Unified TCI state type:

P3 in R2-2406337 (MediaTek)

Proposal 3: In the field description of unifiedTCI-StateType, specify that the UE applies value separate if unifiedTCI-StateType is absent in LTM-TCI-Info. In addition, change the need code of unifiedTCI-StateType to Need S.

- No change of need code.

- Add a clarification NW always signals this IE in the field description. Detailed wordings are up to RRC CR rapporteur.

[CATT]: Even with change of need code to S, consider it is NBC change. [LG]: Huawei proposed different solution, i.e. just network makes sure it is signalled. Prefer Huawei proposal. [Huawei]: In Rel17 MIMO, it was mandatory IE. NW always signalling is quite aligned with Rel17. [Qualcomm]: Share the view with Huawei. [Samsung]: LTM should be always operated based on unified TCI. In that point of view, Huawei’s option sounds better.

Early decoding:

P1 in R2-2406847 (Nokia)

Proposal 1: NW indicates which cells (e.g. by providing a list of cells) are subject to early ASN.1 decoding. This indication is included in the RRCReconfiguration message used to configure LTM.

- RAN2 understands NW indicating which cells (e.g. by providing a list of cells) for early ASN.1 decoding is not required in Rel-18.

[LG]: What was motivation to introduce this capability? In early phase, we assumed it is up to UE implementation. [Nokia]: It was RAN4 decision. [Huawei]: There is no definition regarding how fast. [Nokia]: There are numbers defined in RAN4 (either 10ms or 0ms). [Huawei]: Think NW knowledge of which candidate cell is configured for early decoding is not really useful. [Qualcomm, Samsung]: Share the view with Huawei. Sounds optimization that is not essential. [OPPO]: With Nokia proposal, which node will configure cells for early decoding, source cell or target cell? [Nokia]: Consider it should be the source cell. [LG]: UE implementation to decide which cell is enough. [Apple]: Think even NW configures for a candidate cell, cell switch may happen to the other candidate cell. Then how it works? [Nokia]: Think with RAN2 conclusion, it is good to inform RAN4. [Ericsson]: Don’t see the need to send LS to RAN4.

P5 in R2-2407200 (Huawei)

Proposal 5: The network does not indicate which LTM candidate configurations can be fast processed by the UE. Fast RRC processing for LTM candidate configurations is up to UE implementation.

Configuration for UL early sync:

R2-2407370	Discussion on PRACH occasion validation for LTM	Qualcomm Incorporated	discussion

Proposal 1: RAN2 to agree adding a new IE ltm-tdd-UL-DL-ConfigurationCommon-r18 in LTM-Candidate-r18.

- Noted. If we receive RAN1 LS this meeting, we can reflect new parameters in RRC CR (in short email discussion on RRC CR).

[Ericsson]: Technically agree with the proposal. However, it is under RAN1 discussion this meeting. And it seems more parameters are missed. It is better to wait for RAN1 inputs.

L2 reset only in inter-DU only or allowed for intra-DU also?

P5 in R2-2407050 (Samsung)

Proposal 5: RAN2 is kindly asked to confirm that L2 reset is also applicable for the intra-DU LTM case, and an LS can be sent to RAN3.

- L2 reset (RLC reestablishment and PDCP data recovery) is also applicable for the intra-DU LTM case.

[Ericsson, Huawei]: Proposal 5 is correct. Do we really need to send LS to RAN3? [Samsung]: RAN3 has already discussed it in the past. It would be good to inform our agreement officially. [Session chair]: Showing RAN2 agreement captured in the meeting reporting should be sufficient.

Measurement gap for L1 and L3 measurements:

P8 and P9 in R2-2407050 (Samsung)

Proposal 8: In NR-DC, MN considers the LTM configuration in SN (and MN) and the capabilities 39-2 and 39-3 while configuring measurement gaps.

Proposal 9: SN can inform the NR-ARFCN, physical cell-id (PCI) and SSB configuration of its LTM candidate cells to MN in CG-Config INM.

P1 in R2-2407176 (Ericsson)

Proposal 1: The existing measurement gaps framework is re-used for LTM without the need to introduce new signalling.

[Ericsson]: Understand typically if MR gap is required for L3 measurement for a given frequency, it would be also required for L1 Measurement. [Qualcomm]: It is somewhat risky assumption since they defined separate capabilities for L1 and L3. We need to carefully check. [CATT, ZTE]: Share the view with Ericsson. [Qualcomm]: Would like to have some time for check until Thursday CB session. [Apple]: Want to clarify gap is same but NW will consider LTM capability also for the gap configuration. [Ericsson]: Confirms it.

- Comeback in Thursday CB session.

- After offline discussion, P1 in R2-2407176 is agreed.

SRB5 handling during SCPAC:

P11 in R2-2407050 (Samsung)

Proposal 11: Capture the handling of SRB5 during Subsequent CPAC execution (as in TP4).

- Agreed.

[Ericsson]: Ok with the proposal 11.

discardOnPDCP and reestablishRLC in SCPAC:

P1 in R2-2406531 (OPPO)

Proposal 1: Add the restriction that the NW does not include discardOnPDCP and reestablishRLC for SRB3 in case of SCPAC in MN format.

[Ericsson, Nokia]: NW implementation can handle it. [OPPO]: We already have similar restrictions for others. [OPPO]: After real-time offline discussion with companies, they now understand the proposal better. Would like to comeback in Thursday CB session.

- Comeback in Thursday CB session.

- After offline discussion, intention is agreed. Detailed wording (up to RRC rapporteur) can be further discussed as part of RRC CR preparation.

UL Skipping and LTM cell switch:

P2 in R2-2407176 (Ericsson)

Proposal 2: If UL skipping is configured, the UE ignore the UL skipping indication for the first transmission in the target cell during an LTM cell switch procedure, i.e. the UE always performs the first transmission.

- Not pursued.

[Apple]: Even w/o new UE behaviour, we can survive with RRC reconfiguration. [Huawei]: Remember it was already discussed and no UE behaviour was introduced. [Qualcomm, CATT]: It only happens for SN change w/o SRB3. Assume it is not often happened and we can count on NW implementation.

PDCCH monitoring upon LTM Cell Switch Command:

P3 in R2-2407176 (Ericsson)

Proposal 3: RAN2 to clarify at which point, after the reception of the LTM cell switch MAC CE, the UE starts to monitor the PDCCH.

- Not pursued.

[Xiaomi]: From UE point of view, the UE will monitor PDCCH immediately after reception of LTM Cell Switch Command. [MediaTek]: Understand UE can only monitor PDCCH after RRC processing time (upon end of region 3). [Huawei]: Understand the situation is quite similar to normal RRC reconfiguration w/o consideration of LTM switch. [Apple]: Agree with Huawei. [Nokia]: Agree with Huawei and don’t see a real need to capture anything.

UE Capabilities:

R2-2406476	Report of [Post126][514][R18MobE] UE capabilities Open Issues (Intel)	Intel Corporation	report	Rel-18	NR_Mob_enh2-Core

Proposal #1 (modified): No dependency between LTM and L1 measurements is captured in RAN2 specs (306, 331, 300) for both FR1 and FR2.  Inform RAN1/4 of RAN2 decisions.

Proposal #2: BC of the Intra and Inter-frequency measurements is “Option 1: The BC granularity is BC of serving cells”.

Proposal #4:  Chose option 2+4 for LTM MCG and SCG capabilities.  That is, define capabilities per band consistent across all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands. And introduce another capability bit (per UE) to indicate inter-frequency LTM (UE indicating this capability shall also support intra-freq, (MCG or SCG) LTM i.e, the default indicated by the per band bits).

Proposal #5: Send LS to RAN1 to get clarification on whether RAN1 capabilities 45-3a/4a, 45-5/5a/6 can also be different intra and inter-frequency.

- Proposal 1, 2, 4, 5 are agreed.

[ZTE]: For FR2, it would be good to have clear RAN2 agreement.

Proposal #3: Discuss whether to introduce another capability bit which indicates that UE supports inter-frequency L1 measurements only in bands of that BC

[Huawei]: One additional bit per UE would be ok and it distinguishes two cases, i) UE supports inter-F measurements only for candidate cells in the BC that inter-F measurement capability is reported and ii) UE supports inter-F measurements for any UE’s supported bands. [ZTE]: With one bit per UE capability, we need to discuss what is default meaning. Understand default should be ii) UE supports inter-F measurements for any supported bands. [Nokia]: W/o additional bit, what will happen?

- More time for discussion seems needed. We can revisit it either CB session or next meeting.

In CB session:

[Huawei]: Additional bit is per UE. It will be applicable on the BC that is indicated where L1 inter-F measurement is supported. [MediaTek]: It is not clear how per UE bit works. Also not clear where to add the bit. [Session chair]: Suggest to revisit it next meeting (with TP).

- Will be revisited next meeting (with TP).

R2-2406477	Draft 331 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.331	18.2.0	NR_Mob_enh2-Core

R2-2406478	Draft 306 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.306	18.2.0	NR_Mob_enh2-Core

[Intel]: Propose short email discussion on the draft CRs.

[POST127][110][MOB] (Intel)

Scope: Update the draft CRs (if needed), discuss and finalize them. Prepare a LS to RAN1/4 to inform capability agreement and ask the question (see the discussion on R2-2406476).

Intended outcome: Endorsed 38.331 CR in R2-2407599 and 38.306 CR in R2-2407600. Approved LS in R2-2407601.

Deadline: Super short email discussion.

=> Endorsed in:

R2-2407859 (38.331 draftCR)

R2-2407600 (38.306 draftCR)

=> Approved in:

R2-2407601 (LSout)

R2-2406852	Discussion on LTM fast processing capabilities	Google Ireland Limited	discussion	Rel-18	38.306

Proposal 1: Clarify in the field description that maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check as specified clause 6.3 in TS 38.133.

- Agreed.

Proposal 2: Clarify the meaning of maxNumberConfigs-r18 in TS 38.306 using one of the following options:

Option 1: maxNumberConfigs-r18 represents the total number of LTMCandidateConfigs and ltm-ReferenceConfiguration for which the UE can perform early ASN.1 decoding and validity check.

Option 2 (modified): maxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133.

- Option 2 is agreed.

[Xiaomi]: Reference configuration can be included in candidate configuration. So reference configuration and candidate configuration may not be considered in separate. [MediaTek]: ASN.1 decoding and validity check for a candidate configuration already means option 2. [Apple]: What is validity check? [Google, MediaTek]: It is clear what ASN.1 decoding and validity check are according to RAN4 definition. [Intel]: Within RAN2 spec, normally when we talk about ASN.1 decoding, it already contains validity check. Question is if we need to specify validity check in RAN2 spec.

Proposal 3: maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s), and Scell(s) in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s) for MCG and SCG, that UE can store the configurations.

- Not pursued.

[MediaTek]: With the similar reason discussed for option2, we do not need to consider option3.

Proposal 4: RAN2 should discuss and clarify whether UEs not supporting LTM fast processing should indicate maxNumberStoredConfigCells-r18.

- RAN2 understands only UE supporting LTM fast processing indicates maxNumberStoredConfigCells-r18.

[MediaTek]: This UE capability information is for fast processing of LTM candidate cell. So if LTM fast processing is not supported, there is no point to report it. [Xiaomi]: Is it related to 39-6 or 39-7? [Session chair]: Think it is 39-6 based on RAN4 capability excel.

R2-2406355	Leftover LTM UE capability issues	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406418	Inter-node coordination on L1 measurement for LTM	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

[AT127][104][MOB] (MediaTek, ZTE)

Scope: Discuss proposals in R2-2406355 and R2-2406418. Make agreeable proposals based on companies’ understanding and inputs. F2F offline discussion in Brk3.

Intended outcome: Discussion summary (including TP if possible) in R2-2407584.

Deadline: Comeback in Thursday CB session. => Completed.

R2-2407584	Discussion summary on [AT127][104][MOB]	MediaTek, ZTE	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Clarify that the capability value of ltm-FastProcessingConfig-r18 indicates the maximum number across all the supported bands, rather than the current band. Adopt the TP 1 in this paper.

[Qualcomm, MediaTek]: RAN4 indicates it is per UE capability with FR1 and FR2 differentiation. [Intel]: Then is it ok to change it to per UE capability (with FR1/2 differentiation)?

- Assuming it is correct that RAN4 indicates it is per UE capability with FR1/FR2 differentiation, change this capability to per UE capability with FR1/FR2 (NBC change). TDD/FDD differentiation will be checked. Detailed ASN.1 change and wordings are up to UE capability CR rapporteur. It will be discussed as part of super short email discussion.

Proposal 2: Add separate optional markers for the three components of the feature R4 39-7 (ltm-FastUE-Processing) (NBC change). Adopt the TP 2 in this paper.

- Agreed.

Proposal 3: The signalling structure in option2 is not pursued. Keep the current signalling structure for the capabilities with per band pair per band combination granularity.

- Agreed.

Proposal 4: RAN2 people ask internally to RAN4 to see whether the capabilities R4 39-4, 39-4a and 39-5 can be indicated independently, and come back on Thursday:

•	If these capabilities can be indicated independently, adopt the TP 3 in this paper.

•	If these capabilities are always coupled, no correction is needed.

[MediaTek]: After checking, 39-4a and 39-5 are coupled but 39-4 is decoupled. Suggest to follow RAN4 guideline, i.e. to make all independent for safe.

- Make all independent (NBC change). Change will be captured in UE capability CR discussion.

Proposal 5: In order to ensure that UE capabilities on L1 measurement are not exceeded, the MN indicates the maximum number of L1 measurement resources/configurations the SN is allowed to configure for SCG LTM, including:

-	The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement;

-	The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps;

-	The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement;

-	The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement;

-	Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement;

-	Maximum number of LTM CSI report configs, including aperiodic configs, periodic configs, and semi-persistent configs, respectively;

-	Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement

- Agreed.

Proposal 6: The SN can also request the MN for new maximum values of the number of L1 measurement resources/configurations (as listed in proposal 1) that the SN can configure for SCG LTM. And it’s up to the MN whether to accommodate the SN request.

- Agreed.

Proposal 7: Regarding the capabilities across MCG and SCG per frequency layer, the MN indicates the maximum number of resources per frequency that the SN is allowed to configure. And the SN can request the MN for a new maximum value of the number that the SN can configure. I.e. similar to P1 and P2 above. The capabilities include:

-	The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps;

-	The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps;

-	The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps;

-	The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps.

- Agreed.

Proposal 8: Inter-node coordination is not supported for UE capability that defined as maximum number of resources configured in one slot, i.e. the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement.

- Agreed.

Proposal 9: For each coordinated capability above, the MN indicates a single value to the SN and vice versa. And the value is applicable to all BCs in the allowed BC list.

- Agreed.

Proposal 10:  RAN2 to adopt the TP in the Annex as baseline.

- Agreed. Further details can be discussed as part of CR preparations.

Others to be discussed as part of CR preparation:

P1, P2, and P3 in R2-2406332 (CATT)

P3 in R2-2406337 (MediaTek)

P1, P2, P3, P6, and P7 in R2-2407050 (Samsung)

CR in R2-2407175 (Ericsson)

[POST127][105][MOB] (Ericsson)

Scope: Discuss P1, P2, and P3 in R2-2406332, P3 in R2-2406337, P1, P2, P3, P6, P7 in R2-2407050 and R2-2407175. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements.

Intended outcome: Agreed RRC CR in R2-2407585. Discussion summary in R2-2407595 (if needed). If proposals are straight-forward, it is ok to directly discuss based on the CR (up to offline discussion rapporteur). => In short email discussion, no need of discussion summary.

Deadline: Comeback in Thursday CB session => Short email discussion

=> Agreed in R2-2407585 (38.331 CR#4930r1)

37.340 Correction:

R2-2406417	Miscellaneous corrections for mobility enhancements	ZTE Corporation	CR	Rel-18	37.340	18.2.0	0399	-	F	NR_Mob_enh2-Core

R2-2407091	Draft CR for subsequent CPAC corrections	Ericsson	draftCR	Rel-18	37.340	18.2.0	F	NR_Mob_enh2-Core

[POST127][106][MOB] (ZTE)

Scope: Discuss R2-2406417 and R2-2407091. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements

Intended outcome: Endorsed 37.340 CR/TP in R2-2407586. Discussion summary in R2-2407596 (if needed). If proposals are straight-forward, it is ok to directly discuss based on draft TP/CR (up to offline discussion rapporteur). => Agreed 37.340 CR in R2-2407604. In short email discussion, no need of discussion summary.

Deadline: Comeback in Thursday CB session. => Short email discussion

=> Agreed in R2-2407604 (37.340 CR#0399r2)

R2-2407586	Miscellaneous corrections for mobility enhancements	ZTE Corporation, Ericsson	CR	Rel-18	37.340	18.2.0	0399	1	F	NR_Mob_enh2-Core

38.300 Correction:

R2-2407449	TP for Stage 2 in coexistence case	Lenovo	discussion	Rel-18	NR_Mob_enh2-Core

- Noted.

[Qualcomm]: Ok with “stop” case. However not sure of “resume” case. [Lenovo]: Nothing new is proposed. All principles were already agreed. [MediaTek]: No need to specify it. We can count on UE implementation. [Lenovo]: We already captured many coexistence cases. [Nokia]: Good to capture it if it was already agreed. [Apple]: Assume it is not a realistic configuration. [CATT]: It is similar to legacy PSCell change and CHO evaluation is not stopped in the case.

Response to RAN3 LS:

R2-2407562	Discussion on RAN3 LS on intra-SN SCPAC in MN format	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: From RAN2 perspective, CG-Config and CG-CandidateList are related to different configuration of intra-SN subsequent CPAC in MN format, and should be sent together in the same SN Modification Required XnAP message.

[CATT]: Think there is no problem if two are sent in separate. Source node can combine them.

Proposal 2: From RAN2 perspective, CG-Config message and CG-CandidateList message can not be included in a single existing inter-node RRC message. RAN2 sends the reply LS to RAN3 to kindly ask RAN3 to include them in a single SN Modification Required XnAP message.

Proposal 3: RAN2 to approve the reply LS in the Annex.

[Session chair]: Revisit it next meeting since there is no consensus.

- Revisit it next meeting.

R2-2407770	L3 measurement based LTM mobility support over F1	Vodafone, BT, Qualcomm, Samsung, Apple, Vivo	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: RAN2 should ask RAN3 to introduce F1 support (F1 signalling from CU to DU) for the case where LTM is based on UE L3 measurements.

- Agreed.

R2-2407771	L3 measurement based LTM mobility support over F1	Vodafone	LS out	Rel-18	TEI18	To:RAN3

- Action should be updated to “RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.”

- Next RAN2 Meeting should be “TSG-RAN2”.

- Work item should be “NR_Mob_enh2-Core”.

- Source should be “RAN2”.

- With the changes above, approved in R2-2407606.

[Huawei]: Why not source CU sends L3 measurement to source DU? [Qualcomm]: However, we still need new message exchange from CU to DU.

Others:

S-Measure and L3 measurements on L1 LTM candidate cells (P1 in R2-2406438, Vivo)

Proposal 1: UE should always perform L3 measurement for LTM candidate cells even if s-Measure criterion is satisfied.

- Not pursued.

[Samsung]: To remove this restriction, we may need to consult RAN4. [Vivo]: With S-measure, the UE may need to perform measurements on unknown cells, which is not allowed by RAN4. [Apple]: Better to discuss it in RAN4. [MediaTek]: If the UE is located in the cell centre, L3 measurements on neighbouring cells is not triggered, then L1 measurement is not required. Don’t see any issue with s-Measure. [LG]: In LTM cell switch w/o L1 measurement report, there is no issue with s-Measure. [Nokia]: W/o s-Measure, more UE power consumption is concerned. Think the proposal is not required. [Vivo]: Without proposal, the consequence would be that the UE doesn’t perform L3 and L1 measurements if the current serving cell is better than s-Measure.

Security configuration for SCPAC and EMR reselection MR (R2-2407072, Nokia)

R2-2406332	Miscellaneous Corrections for SCPAC	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406337	Rel-18 LTM CP remaining issues	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406355	Leftover LTM UE capability issues	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406418	Inter-node coordination on L1 measurement for LTM	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406438	Discussion on the impact of s-Measure on L1 measurement	vivo	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406476	Report of [Post126][514][R18MobE] UE capabilities Open Issues (Intel)	Intel Corporation	report	Rel-18	NR_Mob_enh2-Core

R2-2406477	Draft 331 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.331	18.2.0	NR_Mob_enh2-Core

R2-2406478	Draft 306 CR on updates to UE FeMob LTM capabilities	Intel Corporation	draftCR	Rel-18	38.306	18.2.0	NR_Mob_enh2-Core

R2-2406531	Discussion on remaining issues for SCPAC	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406552	Triggering LTM Cell Switch without L1 Measurement and Reports	Google Ireland Limited	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406726	Clarification on LTM configuration	Apple	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406847	Miscellaneous Rel-18 LTM Corrections (Early decoding, TA acquisition and estimation)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406852	Discussion on LTM fast processing capabilities	Google Ireland Limited	discussion	Rel-18	38.306

R2-2407050	RRC corrections for Mobility Enhancements	Samsung	discussion

R2-2407072	correction related to security configuration for SCPAC and corrections on EMR reselection measurement reporting	Nokia	discussion

R2-2407091	Draft CR for subsequent CPAC corrections	Ericsson	draftCR	Rel-18	37.340	18.2.0	F	NR_Mob_enh2-Core

R2-2407175	Misc RRC corrections for feMob	Ericsson (Rapportuer)	CR	Rel-18	38.331	18.2.0	4930	-	F	NR_Mob_enh2-Core

R2-2407176	Remaining issues related to LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407177	Summary of RRC proposals for feMob	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core	Late

R2-2407200	RRC issues for LTM	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407370	Discussion on PRACH occasion validation for LTM	Qualcomm Incorporated	discussion

R2-2407410	Correction on LTM related procedure for UE-based TA measurement	Sharp	CR	Rel-18	38.331	18.2.0	4950	-	F	NR_Mob_enh2-Core

R2-2407449	TP for Stage 2 in coexistence case	Lenovo	discussion	Rel-18	NR_Mob_enh2-Core

### 7.4.3	User plane corrections

Including user plane (e.g. MAC) corrections. A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note MAC CR rapporteur’s summary and suggestion (based on the submitted contributions) may be provided.

Carrier selection for RACH-less LTM:

P1 in R2-2406530 (OPPO)

Proposal 1	For RACH-LESS LTM, RAN2 to discuss the way to determine UL carrier with the following three alternatives:

a.	introduce a RSRP_threshold for NUL/SUL selection;

b.	indicate NUL/SUL in LTM cell switch MAC CE;

c.	CG resource for RACH-LESS LTM execution is configured on NUL.

- Not pursued.

[ZTE]: In RACH-less LTM, CG is only configured either NUL or SUL. Not for both. Do not think we need any new. [Huawei]: Understand CG resource can be configured in both NUL and SUL. And the UE will use the first incoming CG resource. We can follow same principle. [CATT]: Share the view with ZTE. [Xiaomi]: Understands it can be configured for both, but not in the same occasion/slot.

CG retransmission timer:

P1 in R2-2406853 (NEC)

Proposal 1: The UE stops the cg-RRC-RetransmissionTimer when the uplink grant for a new transmission on the same HARQ process used for the first PUSCH transmission is received.

- Noted.

[Huawei]: Understand if LTM cell switch is considered as completed in this case, then the CG resources are no more valid so this timer is anyway stopped. [LG]: Understand it is only true when RRC reconfiguration complete is sent over RACH-less LTM switch.

MR Gap and first PUSCH:

P3 in R2-2406853 (NEC)

Proposal 3: During activated measurement gaps, the UE can transmit the first PUSCH transmission during RACH-less LTM cell switch or first PUSCH transmission during RACH-less handover.

- Noted.

[LG]: Consider it is too fancy after R18 was frozen. It is not needed. [Samsung]: Agree with LG. [LG]: Clarified it was discussed last meeting and it was agreed DRX and measurement gap are applied in LTM switch.

RACH based LTM and 2 TAs:

P1, P2, P3, and P4 in R2-2406439 (Vivo)

Proposal 1: Support the co-existence between RACH-based LTM and R18 MIMO two TA, i.e., the LTM candidate cell could be configured with R18 MIMO two TA.

Proposal 2: RAN2 to confirm the mismatch issue occurs when the TCI state ID indicated in the LTM Cell Switch MAC CE is associated to one TAG while the TA value in the RACH RAR is associated to another TAG, assuming the co-existence between RACH-based LTM and R18 MIMO two TA is supported.

Proposal 3: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH based LTM procedure are associated to different TAGs:

−	Option 2: Target DU sends a new TCI state in RACH Msg 4.

−	Option 3: If the mismatch issue occurs, UE follows the TCI state associate with the RACH based LTM procedure. Otherwise, UE follows the indicated TCI-state in the LTM cell switch command.

−	Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with indicated TCI state in LTM Cell Switch Command MAC CE during the RACH based LTM procedure.

Proposal 4: If Option 3 in Proposal 2 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

[Huawei]: The rapporteur's understanding is that it is already clear that this co-existence is supported, the mismatch can occur but may not be so likely, and as explained in option 1, the network can solve the problem by sending a PDCCH order to trigger another RACH. [Apple]: What’s the use case? [Samsung]: Target cell can have two TAs in LTM. [LG]: NW can know this situation and it can be handled by NW implementation, e.g. by PDCCH order. [Ericsson]: Agree with LG. [Vivo]: If we count on NW implementation, the consequence would be no data transmission before TA is corrected by PDCCH order.

- RAN2 understand NW can handle this situation, e.g. by PDCCH order. No need of new option.

Fall-back RACH from RACH-less LTM:

P1, P2 in R2-2406517 (ASUSTek)

Proposal 1:	For RACH-less LTM, the UE falls back to RACH-based LTM in the target Cell when the SS-RSRP of the SSB corresponding to the configured uplink grant that has the same SSB index as the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE is lower than a threshold.

Proposal 2:	Adopt text proposal below as baseline for fallback RACH for LTM.

- Not pursued.

[Huawei]: Fallback was discussed and it was not agreed. [Ericsson]: Agreed with Huawei. [ZTE]: Recall that we discussed fallback for CFRA, not for fallback for LACH-less.

Other corrections:

P2, P3, P4 in R2-2407199 (Huawei)

Proposal 2: Remove the action to stop cg-RRC-RetransmissionTimer in 5.3.1 upon reception of a DL assignment address to the UE C-RNTI or G-RNTI during RACH-less LTM cell switch.

- Not pursued.

[Xiaomi]: Understand nothing is wrong if stop it. CG resource is not used, then stopping it is natural operation. [Nokia]: Support the proposal. [LG, CATT]: Agree with Xiaomi.

Proposal 3: The UE considers RACH-less LTM cell switch as complete upon receiving PDCCH indicating a new transmission on a HARQ process for which there was a previous transmission to the target cell.

- Not pursued.

[Samsung]: It was already discussed during [108] offline and companies didn’t agree with it. [Huawei]: Confirmed P3 and P4 were treated in [108] offline.

Proposal 4: During RACH-less LTM cell switch, the UE can use all HARQ processes of the CG configured for RACH-less LTM cell switch.

[Huawei]: Based on offline discussion, it was not agreed and companies thought that there is only one HARQ process for the CG for RACH-less LTM cell switch. We can capture that understanding.

- Only one HARQ process for the CG for RACH-less LTM cell switch. Will be captured in RRC.

P2, P3, P4 in R2-2407433 (ZTE)

Others to be discussed as part of CR preparation:

R2-2406331 (CATT)

R2-2406349 (Samsung)

P5, P6 in R2-2406439 (Vivo)

R2-2407197 (Huawei)

P1 in R2-2407433 (ZTE)

P2 in R2-2406853 (NEC)

[AT127][108][MOB] (Huawei)

Scope: Discuss R2-2406331, R2-2406349, P5, P6 in R2-2406439, R2-2407197, P1 in R2-2407433, and R2-2406853. Update the CR with merging all corrections (if agreeable).

Intended outcome: Endorsed MAC CR/TP in R2-2407587. Discussion summary in R2-2407597 (if needed). If proposals are straight-forward, it is ok to directly discuss based on draft CR/TP (up to offline discussion rapporteur).

Deadline: Comeback in Thursday CB session. => Completed.

R2-2407597	[AT127][108][MOB] MAC CR discussion summary (Huawei)	Huawei	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406331 (CATT)

- 1st change is useful (add reference to the MAC CE section).

- 2nd change is not needed.

- modified 3rd change is useful: " The configured candidate cell TCI states are initially deactivated upon (re-)configuration by upper layer and after reconfiguration with sync that is not triggered by LTM."

R2-2406349 (Samsung)

Both changes are needed.

P5, P6 in R2-2406439 (Vivo)

1st proposal:

RAN2 understands that the UE performs L1 measurement report for LTM and does RACH if ordered by PDCCH for any candidate cell as in the LTM configuration, whether the candidate happens to be a deactivated SCell or not has not impact on this. No clarification in specifications is needed. This can be captured in meeting minutes.

2nd proposal: change is useful (but add "minus 1").

R2-2407197 (Huawei)

Changes are useful (editorial).

R2-2407433 (ZTE)

P1 is not needed.

No change needed for P2.

P2 in R2-2406853 (NEC)

No need (and this is not different from CG-SDT).

R2-2407199 (Huawei)

- In DL assignment reception, remove the condition on "new transmission".

- Completion is only upon UL grant for the HARQ process of the first PUSCH transmission (not of any additional transmission). No change is needed.

- Only one HARQ process can be used in the CG for RACH-less LTM cell switch. No change needed (could be captured in RRC).

- All offline discussion outcomes above are agreed.

R2-2407587	User plane corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1904	2	F	NR_Mob_enh2-Core

- Agreed.

R2-2406331	Corrections to TS 38.321 for LTM	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406349	MAC corrections for LTM	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406439	Discussion on MAC open issue for LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406517	Discussion on fallback RACH for LTM	ASUSTeK	discussion	Rel-18	38.321	NR_Mob_enh2-Core

R2-2406530	Discussion on carrier selection for RACH-LESS LTM	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

R2-2406853	Corrections of MAC issues for RACH-less LTM	NEC	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407197	User plane corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1904	-	F	NR_Mob_enh2-Core

=> Revised in R2-2407563

R2-2407563	User plane corrections for LTM	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1904	1	F	NR_Mob_enh2-Core

R2-2407198	MAC CR rapporteur summary	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core	Late

R2-2407199	Issues for RACH-less LTM cell switch	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2407433	Consideration on Remaining Issues on LTM UP Aspect	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

## 7.5	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

Time budget: 0 TU

Tdoc Limitation: 2 Tdocs

### 7.5.1	Organizational

Including LSs, any rapporteur inputs

Editorial

R2-2406925	Parameter names corrections for XR	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4898	-	D	NR_XR_enh-Core

Editorial change in parameter names to align with naming conventions from official guidelines.

=>	The CR is agreed

R2-2407789 	Corrections for XR	Qualcomm	CR	Rel-18	38.321	18.2.0	1926	-	F	NR_XR_enh-Core

=>	Interoperability analysis will be included and add Ericsson to co-sourcing companies

=>	The CR is agreed in R2-2407811 with changes above

R2-2407811	Corrections for XR	Qualcomm Incorporated, Ericsson	CR	Rel-18	38.321	18.2.0	1926	1	F	NR_XR_enh-Core

=> Agreed, but then coversheet revised by MCC: wrong revision number

=> Revised in R2-2407876

R2-2407876	Corrections for XR	Qualcomm Incorporated, Ericsson	CR	Rel-18	38.321	18.2.0	1926	2	F	NR_XR_enh-Core

=> Agreed

### 7.5.2	Control plane corrections

Including RRC and UE capabilties

Handling non-integer DRX during RRC INACTIVE state

R2-2407375	Non-integer DRX Configuration when UE enters RRC_INACTIVE	CATT	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: The non-integer DRX configuration is released when the UE enters RRC_INACTIVE state.

-	Huawei thinks that we can clarify in MAC that when the UE resumes it should reset.

-	Qualcomm thinks that this is similar to the HO case and we should to aim to have a similar behaviour.

-	Nokia doesn’t think we need to do anything.  LG agrees.

=>	Noted

### 7.5.3	User plane corrections

Including MAC, RLC and PDCP

DRX_SFN_COUNTER

Rely on NW implementation – no enhancements

R2-2406301	Remaining issues for DRX operation for XR	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: The issue of DRX initialization in case of handover can be resolved by NW implementation. There is no need for enhancement.

=>	Noted

R2-2406555	Further Consideration on DRX SFN Counter Initialization	CATT	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: Non-integer DRX configuration is not configured during HO, and PSCell Addition/PSCell change cases with reconfiguration with sync.

Proposal 2: Non-integer DRX configuration is not configured for CHO, CPAC and LTM cases.

=>	Noted

R2-2406779	Further discussion on the DRX_SFN_COUNTER initialization	OPPO	discussion	Rel-18	NR_XR_enh-Core

Proposal 1	No spec change on the DRX_SFN_COUNTER initialization for handover.

=>	Noted

Initialized upon HO complete

R2-2407061	Remaining issues for on setting DRX SFN counter during handover	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

Proposal: For CHO/LTM, DRX_SFN_COUNTER is initiated to 0 upon HO complete regardless of the drx-TimeReferenceSFN.

=>	Noted

R2-2406607	MAC Corrections for Rel-18 XR	Samsung	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: RAN2 is kindly asked to agree that the initialization of DRX_SFN_COUNTER occurs when RRCReconfigurationComplete message is sent for HO case.

Proposal 2: RAN2 is kindly asked to agree that the initialization of DRX_SFN_COUNTER occurs when RRC response message is sent by the UE for all cases.

=>	Noted

R2-2406440	Discussion on DRX_SFN_COUNTER initialization for handover case	vivo	discussion	Rel-18	38.321	NR_XR_enh-Core

Proposal 1: For handover case, the DRX_SFN_COUNTER should be initialized only after acquiring the SFN of the target SpCell .

Proposal 3: The network implementation should ensure that the non-integer DRX configuration is not provided in the CHO/CPAC/LTM candidate cell configuraitons.

=>	Noted

R2-2406909	Initialization of DRX_SFN_COUNTER at handover	ZTE Corporation, Sanechips	discussion	Rel-18

Proposal: In case of handover, the UE uses the target cell SFN as reference for initializing the DRX_SFN_COUNTER.

R2-2406918	Correction to DRX_SFN_COUNTER for handover case	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.2.0	1894	-	F	NR_XR_enh-Core

Discussion

-	Qualcomm and Lenovo thinks it can be left to network but we should capture in the field description what to expect.  ZTE doesn’t want to leave it to network implementation and we should fix it.

-	Vivo also thinks that we should capture it in the spec.

-	LG agrees with Huawei’s proposal.

-	ZTE would like to avoid the problem all together.

-	Qualcomm suggest to capture in field description - Network ensures that it provides reference SFN within one hyper frame before the UE initialize the DRX SFN context.  Vivo thinks that this is still not clear behaviour in the UE.   Nokia thinks that for CHO And LTm you may not complete the procedure within 10s.

-	Lenovo thinks that ZTE proposal is simple.  Nokia thinks that the issue is the timepoint.   Samsung agrees.

-	Huawei thinks that maybe if we change the one word in MAC to say when the configuration is applied rather than when it is received.

-	Ericsson explains that for LTM the MAC is reset already and we may end up resetting twice.

Agreements

-	We leave this up to network implementation, unless a solution that works in all scenarios (CHO/LTM/Normal HO/Reconfiguration etc) and does not require network to send an extra RRC reconfiguration message can be agreed at next meeting.

RX_NEXT handling based on PDCP SN gap report

[Change to RX_NEXT update operation]

R2-2406891	Update RX_NEXT based on PDCP SN gap report		Lenovo, CMCC	discussion	Rel-18

Proposal 1:  PDCP entity updates RX_NEXT to the COUNT value of the first PDCP SDU which is not considered as discarded and expected to be received, with COUNT value > RX_NEXT if RX_NEXT is equal to any COUNT value associated with the discarded PDCP SDUs.

=>	Noted

R2-2406394	PDCP SN gap report and updating RX_NEXT	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: SDUs considered as discarded (i.e. indicated as discarded in some received SN gap report) lead to incrementing RX_NEXT (upon reception of a SN gap report and upon being updated by a received SDU) only if, without the incrementing, the SDU with associated COUNT equal to RX_NEXT would be considered as discarded.

Proposal 2: RAN2 agree the text proposal in Annex for Proposal 1.

=>	Noted

[Add a note to disallow combining of PDCP gap reports for non-contiguously discarded PDCP SDUs, leaving details to UE implementation]

R2-2406598	Remaining Open Issues for PDCP SN Gap Reporting	Ericsson	discussion	Rel-18

Proposal 2	Update the NOTE about limiting the frequency of reporting to include “for e.g., combining of the PDCP SN gap report(s) for contiguous PDCP SDU(s) discarded”. Adopt the TP as shown in the Annex.

Proposal 3	Capture a NOTE to prevent the RX_DELIV to be updated beyond the PDCP SDU(s) yet to be received when t-Reordering expires for e.g., by not combining the PDCP SN Gap reports for non-contiguous PDCP SDU(s) discarded. Adopt the TP as in the Annex.

-	LG thinks that there is a wrong assumption that status report is transmitted earlier than the stored PDU.    Huawei agrees in general with LG.   The simplest solution for the network to just not do it.   LG thinks that you can anyways re-associated SN.  Nokia doesn’t think you can if they are ciphered.   Ericsson also thinks we can’t also reassign as we can do pre-processing.

-	Ericsson thinks that we should be cautious and solution is simple to not combine.  Nokia doesn’t think that this can help.

-	Nokia would like to capture that we can’t always re-assign SN when this case happens.

=>	We will leave this as it is

[No change to PDCP window update operation]

R2-2406777	Discussion on PDCP window update based on the SN gap report	vivo	discussion	Rel-18	NR_XR_enh-Core

Proposal: No further modification on the PDCP window update based on the SN gap report is needed and no spec impact.

Editorial stage 2 change

R2-2406926	Correction for discarding operation for XR	Huawei, HiSilicon, Nokia (Rapporteur)	draftCR	Rel-18	38.300	18.2.0	F	NR_XR_enh-Core

The CR is to clarify that the shorter discard timer is applied per PDCP SDU. Without this clarification, the text suggests that the discard timer is applied per PDU set which is not the case.

=>	Update to a CR

=>	The CR is agreed in R2-2407579 in CR 0896

DSR

R2-2406607	MAC Corrections for Rel-18 XR	Samsung	discussion	Rel-18	NR_XR_enh-Core

Proposal 3: RAN2 is kindly asked to consider clarifying that DSR and the SR for DSR can be triggered after the assembly of a MAC PDU which contains a DSR MAC CE, but before the transmission of this MAC PDU. In addition, DSR and SR for DSR can be triggered during MAC PDU assembly. Adopt TP 1.

-	Qualcomm doesn’t think this is needed as DSR is different from BSR.  Samsung thinks that is a chance as assembly is up to UE implementation so there may be a gap.   LG thinks it is possible but it clear in current spec so it is not needed.

=>	Not supported

Proposal 4: RAN2 is kindly asked to agree that, for cancelling a pending DSR, only the PDCP SDUs associated with the DSR before MAC PDU assembly are reported in the DSR MAC CE or included in the MAC PDU. Adopt TP 2.

-	LG doesn’t think there is a problem.

=>	Not supported

=>	Noted

R2-2406805	Corrections on DSR MAC CE	Langbo	draftCR	Rel-18	38.321	18.2.0	F	NR_XR_enh-Core

The CR is to clarify that if BT field is reserved, Table 6.1.3.1-2 is used to set the value of the Buffer Size field.

-	Samsung thinks it is already very clear

-	LG thinks it is needed

=>	The CR is not pursued

R2-2407046	Discussion on user plane corrections	Ericsson	discussion	Rel-18	NR_XR_enh-Core

=>	Add description of the reserved bit field for the DSR MAC CE in 38.321.

=>	Noted

Handling more than one RLC entity

R2-2406598	Remaining Open Issues for PDCP SN Gap Reporting	Ericsson	discussion	Rel-18

Proposal 1	Update the triggering conditions text to include DC and CA by adding “RLC entities” in the PDCP SN gap reporting procedure. Adopt the TP as shown in the Annex.

-	Nokia thinks that we can say “any”.  LG and Apple think that RLC is already clear.   Ericsson and Intel thinks that it is just making it consistent with other changes.

=>	The change can be added when we have a PDCP CR as an editorial

=>	Noted

Setting of FDC (First Discarded Count) field to account for out-of-sequence discard of PDCP SDUs

R2-2406504	Corrections for PDCP SN Gap Reporting	Samsung	discussion	Rel-18

Proposal 1: FDC field is set to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s) since the last transmission of the PDCP SN gap report. Adopt text proposal TP1 provided.

-	Ericsson and others haven’t changed their mind since last time.

=>	Not supported

Proposal 2: PDCP SN gap report is submitted to lower layers as the first PDCP PDU for transmission. Adopt text proposal TP2 provided.

-	Ericsson and LG think that this will introduce the problem.

=>	Not supported

=>	Noted

## 7.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.6.1	Organizational

LSs, rapporteur inputs.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2406211	LS on Rel-18 RAN1 UE features list for LTE after RAN1#117 (R1-2405569; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	IoT_NTN_enh	To:RAN2	Cc:RAN4

Noted

Rapporteur inputs

R2-2406938	Miscellaneous MAC correction for IoT NTN	MediaTek	CR	Rel-18	36.321	18.2.0	1588	-	F	IoT_NTN_enh-Core

-	QC would like to further check this

Change is agreed

Revised in R2-2407620 to include other changes agreed this week

R2-2407620	Miscellaneous MAC correction for IoT NTN	MediaTek	CR	Rel-18	36.321	18.2.0	1588	1	F	IoT_NTN_enh-Core

R2-2407303	Miscellaneous corrections to TS 36.331 for IoT NTN	Huawei, HiSilicon	CR	Rel-18	36.331	18.2.0	5049	-	F	IoT_NTN_enh-Core

Changes are endorsed

The CR is considered as a basis to include additional RRC changes agreed this week

Revised in R2-2407615

R2-2407615	Miscellaneous corrections to TS 36.331 for IoT NTN	Huawei, HiSilicon	CR	Rel-18	36.331	18.2.0	5049	1	F	IoT_NTN_enh-Core

[Post127][305][R18 IoT NTN] 36.300 CR (Ericsson)

Scope: Draft a 36.300 CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407626): short

=> Agreed in R2-2407868 (36.300 CR#1405r1)

[Post127][306][R18 IoT NTN] 36.331 CR (Huawei)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407615): short

=> Agreed in R2-2407615 (36.331 CR#5049r1)

[Post127][307][R18 IoT NTN] 36.321 CR (Mediatek)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407620): short

=> Agreed in R2-2407620 (36.321 CR#1588r1)

[Post127][308][R18 IoT NTN] 36.306 CR (Qualcomm)

Scope: Draft a 36.306 CR based on meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407627): short

=> Agreed in R2-2407627 (36.306 CR#1892)

### 7.6.2	Corrections

Corrections for all specifications.

R2-2406951	Discussion on RRC Corrections to IoT NTN	MediaTek Inc.	discussion	Rel-18	36.331	IoT_NTN_enh-Core

Proposal 1: Moving the text about T390 expiry in 5.3.3.21 to a new standalone clause.

-	HW is fine with this

-	ZTE thinks this might not be the only issue: we could also consider the restart of T390. MTK is ok to further discuss this

Agreed: text about T390 expiry in 5.3.3.21 is moved to a new standalone clause. We can consider additional changes for other similar cases (e.g. T390 restart)

Proposal 2: If T390 expires and a GNSS measurement during the available idle period is in progress, it is up to UE implementation whether to continue the current GNSS measurement using autonomous gaps or start a new GNSS measurement.

-	ZTE thinks this is not needed, this should not be up to UE implementation.

-	QC thinks that in this case the UE could just continue measuring

-	Google supports the proposal

-	Nokia thinks nothing should be captured here.

-	Samsung thinks that clarifying this would be problematic

We don’t pursue further clarifications for this.

Proposal 3: RAN2 to discuss the ASN.1 issue of EARFCN value in carrierFreqList of SIB32.

-	vivo thinks we should go for a BC change but are ok to follow the majority

-	Ericsson prefers to go for the NBC change or at least dummify the current field if we go for the BC one

-	MTK prefers option 1

-	QC think we should go for a BC change

We go for a BC change but we need to revise the proposal. Draft a TP in R2-2407791

R2-2407791	RRC Corrections to IoT NTN	MediaTek Inc.	draftCR	Rel-18	36.331 IoT_NTN_enh-Core

Changes are agreed and will be implemented in the RRC rapporteur CR

R2-2406329	Correction on SIB33	CATT	discussion	Rel-18	36.331	IoT_NTN_enh-Core

Change#1:Under the field description of epochTime of SIB33, clarify that if epochTime is absent in an NTN cell, the UE uses epoch time of the serving cell, but if it is absent in a TN cell, the UE uses the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB33(-NB) is transmitted

- Ericsson suggest to change “UE uses” to “epochTime is”

Change is agreed with the following addition to the epochTime field description: “If this field is absent in TN cell, the epochTime is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB33(-NB) is transmitted.” Align the wording (“UE uses” to “epochTime is”) also in the NR NTN RRC CR

For validity duration we check in the post meeting CR review

Change#2: Under field description of systemInfoValueTag in SystemInformationBlockType1/MasterInformationBlock-NB, exclude that systemInfoValueTag is used for SIB33/SIB33-NB

Agreed

R2-2406450	Miscellaneous RRC Correction for IoT-NTN	vivo	draftCR	Rel-18	36.331	18.2.0	IoT_NTN_enh-Core

1. Add the description that UE stops the timer T318 if it is running in case of HO.

-	Samsung thinks in case this is a Rel-17 issue but thinks the current behaviour was intended. Nokia agrees with Samsung and thinks there is no need for further optimizations.

2. Add the description that UE shall stop the periodic reporting in case the leave condition of the event D1/D2 is satisfied.

Agreed

3. Add clarification on referenceLocation2 separately for event D1 and cond event D1.

Agreed

4. In clause 7.3.1, clarify the timer T310 is stopped upon expiry of t-Service or being out of the current serving cell coverage in discontinuous coverage scenario.

Agreed

R2-2407166	Further discussion on T390 stop and GNSS validity report during C-DRX inactive time	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: RAN2 to specify the UE stops timer T390 upon sending the GNSS validity duration report. A text proposal is provided below.

- MTK thinks we already discussed this and we shouldn’t discuss again and in any case it’s impossible to completely align between the NW and the UE. QC agrees

- Ericsson agrees with Nokia

- Google thinks the mismatch can be avoided by UE and also NW implementation, e.g. implementing some grace periods. As a last resort a release indication can be sent to the UE

- Nokia thinks the time mismatch can be up to 2 second and then a real issue

Not pursued

Proposal 2: The note on UE behavior for GNSS acquisition in an idle/C-DRX inactive period is updated to reflect the reporting of the remaining GNSS validity duration is mandatory. A text proposal is provided below.

- QC thinks we already discussed this. Nokia disagrees

- Ericsson supports this

- HW thinks we can go for p2

- MTK don’t think that always sending RACH is correct but could accept p1 in Ericsson paper

- ZTE thinks we could clarify that the UE should report leaving to UE

We modify the note as follows:

- NOTE: When in RRC_CONNECTED, if the UE autonomously starts and completes GNSS acquisition using available idle periods, the UE reports the remaining validity duration but it is up to UE implementation whether to stop timeAlignmentTimer and initiate a Random Access procedure.

R2-2407254	Various corrections for IoT NTN Rel-18	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

- Stage 2

Proposal 1: Agree Stage 2 text proposal in Section 5.1 on measuring NTN cells in a terrestrial network.

-	ZTE thinks no description is needed in Stage2

-	Ericsson thinks that some minimal description is beneficial and we can work on the TP

Check in the post meeting Stage 2 CR review

Proposal 2: If none of the satellite IDs for a specific frequency in SIB3/SIB5 are present in SIB33, it is up to UE implementation to measure the frequency.

-	HW thinks the current text is sufficiently clear

Proposal 3: Agree Stage 2 correction text proposal.

- Capabilities

Proposal 4: Introduce non-signalled UE capability for a UE to measure an NTN cell using NTN assistance information in idle mode.

-	ZTE thinks we don’t have NTN-related non-signalled UE capabilities in Rel-17, e.g. for SIB32 acquisition

Proposal 5: Introduce a signalled UE capability for a UE to measure an NTN cell using NTN assistance information in connected mode.

-	Samsung think that it’s essential to have a capability for this, at least for Connected mode.

-	QC supports the need for a capability for this, both for the provision of information via dedicated signalling and via SIB33

-	HW thinks it could be ok to introduce a capability for the provision of information via dedicated signalling

We introduce a signalled UE capability indicating whether the UE can be configured via dedicated signalling with NTN assistance information (satellite ID or ephemeris info in MO) to measure an NTN cell in connected mode.

RAN2 understands that in any case there are no mandatory requirements for a UE to keep an updated version of SIB33 in Connected mode. Come back in the next meeting to check whether we can add another capability for a UE capable of keeping an updated version of SIB33 in Connected mode

Proposal 6: The UE feature to acquire SIB33 in a TN cell indicates that UE is capable of performing measurements of NTN cells using NTN assistance information in a terrestrial cell.

Proposal 7: If P4-P5 is agreed, the capabilities indicate capability to measure NTN cell both in an NTN cell and a terrestrial cell.

- RRC

Proposal 8: Dedicated ephemeris element in measurement object is not allowed for connected mode RRM in IoT NTN. Capture this in 36.331.

- Idle mode

Proposal 9: IoT NTN UE shall disregard NTN distance-based measurement when t-Service is triggered, as in NR NTN.

Proposal 10: Agree 36.304 correction text proposal.

R2-2407553	R18 IoT NTN GNSS extension	Ericsson	discussion	Rel-18

Proposal 1	Add “In this case, the UE may compensate TA (see TS 36.211 [7] clause 8) to avoid time misalignment due to changes in the GNSS position.” to the NOTE in MAC spec section 5.4.10.

-	Nokia thinks would make things complex. ZTE also agree

Proposal 2	Consider the text proposal below.

-	Nokia, vivo, MTK this this is useful

Include the TP (or a revision of it) in the post meeting CR review

R2-2406642	Clarification of note on UL and DL parallel scheduling for NB-IoT	Qualcomm Incorporated	CR	Rel-18	36.321	18.2.0	1587	-	F	IoT_NTN_enh-Core

-	Nokia thinks the CR is not needed. Vivo agrees

Change is agreed in principle but the text should be reworded, e.g. as: “For NB-IoT when HARQ RTT timer has been started for a HARQ process, …”

Continue the discussion in the post meeting CR review

The note 4 in section 5.7 is clarified that it is only applicable for the case HARQ feedback is enabled.

R2-2407302	Clarification on scenario support	Huawei, HiSilicon	CR	Rel-18	36.306	18.2.0	1891	-	F	IoT_NTN_enh-Core

Modify the description of ntn-ScenarioSupport-r17 so that it indicates whether the UE supports NTN features other than the GNSS enhancement and HARQ enhancement in GSO or NGSO scenario, to clarify it covers the supported scenarios of all Rel-17 NTN features and all other Rel-18 NTN features except for GNSS/HARQ enhancements.

-	QC thinks we don’t necessarily need this CR or in case we can clarify the description of Rel-18 capabilities

Not pursued

RAN2 understands that R18 IoT NTN features other than GNSS/HARQ enhancements refer only to ntn-ScenatioSupport-17

RAN2 understands that GNSS/HARQ enhancements scenario support need to be set consistently with ntn-ScenatioSupport-17 (discuss how to add this clarification in the field description during the CR review)

R2-2407538	GNSS correction for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.2.0	5051	-	F	IoT_NTN_enh-Core

Even for the GNSS measurement during C-DRX, UE is required to trigger remaining GNSS measurement validity duration report when it receives indication that a new GNSS position becomes valid.

-	Nokia agrees with the intention but doesn’t think the CR is needed

Not pursued

## 7.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-232669)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.7.1	Organizational

LSs, rapporteur inputs.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2406215	Reply LS on Reference Point for SSB-TimeOffset (R1-2405719; contact: Apple)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN4

We stick to the UL time synchronization reference point as the Reference point for SSB-TimeOffset

Noted

R2-2406229	Reply LS on reference point for SSB-TimeOffset (R4-2410381; contact: Apple)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN1

Noted

Rapporteur inputs

R2-2407239	Rapporteur Input to Rel-18 NR NTN	Ericsson	CR	Rel-18	38.331	18.2.0	4935	-	D	NR_NTN_enh-Core

Change is endorsed

The CR is considered as a basis to include additional RRC changes agreed this week (changing the category to F)

Revised in R2-2407611

R2-2407611	Miscellaneous corrections to Rel-18 NR NTN	Ericsson	CR	Rel-18	38.331	18.2.0	4935	1	F	NR_NTN_enh-Core

[Post127][301][R18 NR NTN] 38.300 CR (Thales)

Scope: Draft a 38.300 based on meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407621): short

=> Agreed in R2-2407621 (38.300 CR#0902)

[Post127][302][R18 NR NTN] 38.331 CR (Ericsson)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407611): short

=> Agreed in R2-2407611 (38.331 CR#4935r1)

[Post127][303][R18 NR NTN] 38.321 CR (Interdigital)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407628): short

=> Agreed in R2-2407628 (38.321 CR#1928)

[Post127][304][R18 NR NTN] UE Caps CRs (Intel)

Scope: Draft CRs based on meeting agreements

Intended outcome: Endorsed CRs

Deadline for draftCRs (in R2-2407624 and R2-2407625): very short

=> Endorsed in:

R2-2407624 (38.306)

R2-2407625 (38.331)

### 7.7.2	Corrections

Corrections for all specifications.

Reference Point for SSB-TimeOffset

R2-2406727	Clarification on Reference Point for SSB-TimeOffset	Apple	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Adopt the UL RP as the reference point of SSB-TimeOffset.

Proposal 2: Capture Proposal 1 in Chairman notes, and no spec change is needed.

R2-2407482	Clarification on the ssb-TimeOffset	Google	draftCR	Rel-18	38.331	18.2.0	F	NR_NTN_enh-Core

ssb-TimeOffset

Indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point. It is given in number of subframes. The value is provided based on the assumption that the propagation delay difference from the uplink time synchronization reference point to UE between the source and target satellite equals to 0 ms, and UE can adjust the actual time offset based on the actual propagation delay difference.

-	LG agrees with Google that this needs to be clarified in the spec

-	Samsung thinks we could merge the proposals from QC and Google

-	HW think this change is not needed but also the QC proposal needs a revision, e.g. swapping target with source

-	Apple also thinks this is not needed

R2-2406641	Discussion on soft satellite switch with re-sync	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Based on LS response, keep the reference point of ssb-TimeOffset at ULSRP and clarify the field description of ssb-TimeOffset that the source satellite and target satellite may have its own different ULSRP as below:

Indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point between a SSB burst of the target cell at its uplink time synchronization reference point and the SSB burst of the source cell at its uplink time synchronization reference point. It is given in number of subframes.

Continue in offline 301 to discuss a clarification for ssb-TimeOffset based on the text proposal in p1

Proposal 2	Confirm that there is no impact to running timers (e.g., HARQ RTT timer) due to cell timing change after soft satellite switch with resync. Consider updating NOTE 1 in section 3.1 by adding another example as “due to soft satellite switch with resynchronization” in addition to “due to BWP switching”.

-	Ericsson and Apple confirm the understanding but think we don’t need any further clarification

Continue the discussion in offline 301

[AT127][301][R18 NR NTN] corrections for satellite switch with re-sync (Qualcomm)

Scope: Continue the discussion on p1 and p2 in R2-2406641

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Thursday 2024-08-22 20:00

Deadline for rapporteur's summary (in R2-2407612):  Friday 2024-08-23 08:00

R2-2407612	Report of [AT127][301][R18 NR NTN] corrections for satellite switch with re-sync	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	The field description of ssb-TimeOffset is updated as “Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes”.

Agreed

Proposal 2	Confirm that there is no impact to running timers (e.g., HARQ RTT timer) due to possible cell timing change after soft satellite switch with resync. No further clarification is needed.

Agreed

Misc changes

R2-2406328	Discussion on the absence of validity duration in SIB19 broadcast by a TN serving cell	CATT	discussion	NR_NTN_enh-Core

Proposal 1: RAN2 undoes the previous agreement for the case of TN broadcast SIB19 "If the validity duration indicated in SIB19 in TN serving cell is absent, the UE follows R17 behaviour (left to UE implementation) (no spec impact)".

Proposal 2: RAN2 down-selects the following options on the UE behaviour for the case that SIB19 is broadcast by a TN serving cell:

-	Option 1: When SIB19 is broadcast in TN serving cell, ntn-UlSyncValidityDuration is mandatory present for neighbour NTN cell;

-	Option 2: Clarify that UE determines the validity duration of neighbour cell satellite assistance information, if not configured, based on UE implementation.

-	Lenovo thinks that some clarification is needed and thinks that Option 1 has more impacts

-	Samsung thinks that the whole reacquisition of SIB19 would be up to UE implementation

-	Nokia supports option 2

We go for option 2 and continue in offline 302 on the actual wording of the change (starting from the TP in Annex 2)

Proposal 3: Based on proposal 1, if option 1 is agreed, adopt the TP in Annex A; if option 2 is agreed, adopt the TP in Annex B.

[AT127][302][R18 NR NTN] absence of validity duration in SIB19 (CATT)

Scope: Continue the discussion on the actual wording of the change according to option 2 for p2 (starting from the TP in Annex 2)

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Thursday 2024-08-22 20:00

Deadline for rapporteur's summary (in R2-2407613):  Friday 2024-08-23 08:00

R2-2407613	Report of [AT127][302][R18 NR NTN] absence of validity duration in SIB19	CATT	discussion	Rel-18	NR_NTN_enh-Core

Proposal: RAN2 adopts the following change in the field description of ntn-UlSyncValidityDuration:

If this field is absent in ntn-Config provided via NTN-NeighCellConfig or SatSwitchWithReSync in an NTN cell, the UE uses validity duration from the serving cell assistance information. If this field is absent in ntn-Config provided via NTN-NeighCellConfig in a TN cell, how the UE sets validity duration is left to UE implementation.

Agreed

R2-2406846	Addressing Various Release 18 NTN Issues	Nokia	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Pursue the changes in section 16.14.3.2.2 of TS 38.300 aimed at removing the location-based trigger for CHO in NTN when satellite hard switching is considered (in line with R2-2404890).

Proposal 2: RAN2 adopts gNB as the reference point of ssb-TimeOffset for both soft and hard satellite switch without PCI change scenarios.

Proposal 3: Agree the NR RRC changes in Annex A.

Proposal 4: Do not change the granularity of softSatelliteSwitchResyncNTN-r18 or hardSatelliteSwitchResyncNTN-r18.

Agreed (softSatelliteSwitchResyncNTN-r18 and hardSatelliteSwitchResyncNTN-r18 remain defined per UE)

Proposal 5: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support parallelMeasurementWithoutRestriction.

-	HW thinks we already discussed this, sent an LS to RAN4 and we don’t need to discuss this again

-	Ericsson thinks that after agreeing p4 we cannot consider p5, unless we make parallelMeasurementWithoutRestriction mandatory for all the bands

We only come back to this discussion if further information is received by RAN4

Proposal 6: RAN2 is asked to reconsider the decision made at RAN2#125 regarding PDD during satellite switching.

We only come back to this discussion if further information is received by RAN4

Proposal 7: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support serviceLinkPropDelayDiffReporting-r17.

Proposal 8: If the decision regarding PDD is revised, confirm with RAN4 that service link propagation delay difference (PDD) helps in reducing the scheduling restriction duration during satellite soft-switching with resynchronization.

Proposal 9: If the LS to RAN4 is sent, RAN2 asks how early such measurements, calculations and reporting should be done.

Proposal 10: Adopt the 38.331 and 38.306 Text Proposals in the Annex B and C, respectively.

R2-2407238	Remaining open issues for NR NTN enhancements	Ericsson	discussion	NR_NTN_enh-Core

Proposal 1	Satellite switch with resync related capabilities (softSatelliteSwitchResyncNTN and hardSatelliteSwitchResyncNTN) are defined per UE.

Proposal 2	Adopt the following clarification to Stage 2 related to the independent configuration of time or location-based CHO trigger conditions from RSRP-based trigger conditions.

-	HW agrees we could fix this but the current proposal is also misleading

-	vivo thinks the current spec is ok

-	QC also thinks we can live with the existing text (saying “at least...”)

-	Oppo and Nokia think that some clarification is needed

RAN2 agrees that a correction to the existing Stage 2 description is needed and continue in offline 303 to discuss the actual change

[AT127][303][R18 NR NTN] Stage 2 text for trigger conditions (Oppo)

Scope: Continue the discussion on the actual wording of the change for the Stage 2 description on trigger conditions without RSRP-based trigger conditions.

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Thursday 2024-08-22 20:00

Deadline for rapporteur's summary (in R2-2407614):  Friday 2024-08-23 08:00

R2-2407614	Report of [AT127][303][R18 NR NTN] Stage 2 text for trigger conditions	Oppo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 adopt the following wording for the stage-2 change.

Time-based or location-based trigger conditions may be configured independently from the RRM measurement-based trigger conditions for CHO in NTN in some scenarios, e.g., when the service discontinuity gap time length is zero or negligible. at least hard satellite switch case where the service discontinuity gap time length is assumed to be zero or negligible. However, if the service discontinuity gap time length is not zero or negligible, then a time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12].

Agreed

R2-2407255	Miscellaneous corrections to NR NTN Rel-18	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Modify NOTE 3 in Table 6.2.1-2c in TS 38.321 as follows…

-	vivo thinks the current spec is ok

-	Ericsson and ZTE agree with Samsung proposal to clarify the spec

Agreed

Proposal 2-1: Allow idle/inactive UE capable of time-based measurement initiation to skip neighbor cell measurement before t-Service if the current serving cell broadcasts satellite switch with resynchronization.

-	QC agrees with P2-1 but would like to discuss the wording offline

-	LG thinks this is not needed

-	Apple supports this

-	CATT wonders if we are introducing a new UE behavior (i.e. measurement relaxation). HW also thinks this is an enhancement rather than a relaxation

-	ZTE thinks we don’t need to add further clarification. Nokia agrees

-	ZTE thinks there is no requirement for this in RAN4 and if we do this we would add more work to RAN4. CMCC agrees

-	vivo also think we can wait for RAN4 in case

Proposal 2-2: If Proposal 2-1 is agreed, adopt the TP below…

Proposal 3: Introduce a capability to indicate UE support of dedicated ephemeris and epoch time in measurement object configuration for connected mode RRM measurements.

-	vivo thinks we don’t need a specific capability for this and we can link this to the support of event D2. Xiaomi agrees

-	Ericsson support this

-	QC supports

-	HW thinks we don’t need any capability for this

-	Samsung thinks the support for this is not only related to D2 support

The UE will indicate the support for this via a new independent capability.

Proposal 4-1: For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but without corresponding satellite information in the measurement configuration or in SIB19.

-	HW supports the proposal and the TP

-	QC wonders how the UE discriminates TN cell from NTN cell. HW thinks we should no further discuss this aspect

Agreed

Proposal 4-2: For proposal 4-1, adopt TP as follows:

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but not included in SIB19 without corresponding satellite information in measurement configuration or in SIB19.

-	Toyota suggest to add a comma before “but”:

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration, but not included in SIB19 without corresponding satellite information in measurement configuration or in SIB19.

Agreed

Proposal 5: The UE feature to acquire SIB19 in a TN cell indicates that UE is capable of performing measurements of NTN cells using NTN assistance information in a terrestrial cell.

Proposal 6: A new non-signalled UE capability to receive SIB19 in TN cell in idle/inactive mode (i.e UE capable of measuring NTN cell using NTN assistance information in TN cell) is introduced.

-	QC supports p6 but not p7. Also there are no RAN4 requirement for connected mode. Apple agrees

-	vivo agrees with QC: for connected mode we can rely on HO

-	DT/Inmarsat think this should also be supported in Connected mode

Proposal 7: A new signalled UE capability to receive SIB19 in TN cell in connected mode (i.e UE capable of measuring NTN cell using NTN assistance information in TN cell) is introduced.

-	Samsung understands that it’s difficult to agree on this at this stage given the lack of RAN4 requirements for this

-	Ericsson supports the proposal

-	MTK thinks that if we have a new UE behaviour we should have a new capability. Nokia agrees

-	Apple thinks we cannot have a capability without requirements. QC agrees

Consider the following options:

Option 1: A new signalled UE capability to receive SIB19 in TN cell in connected mode is introduced (without any relation to RRM requirements)

Option 1a: IOT bit to indicate the capability to receive SIB19 in TN cell in connected mode is introduced???

Option 2: RAN2 understands that a Rel-18 UE supporting NTN needs to support SIB19 acquisition in a TN cell???

Option 3: A new signalled UE capability to measure NTN cells in a TN cell in connected mode???

We go for option 1: A new R18 signalled UE capability to receive SIB19 in TN cell in connected mode is introduced (without any relation to RRM requirements) (can come back to further clarify the capability description in the future)

R2-2406992	Miscellaneous corrections to epochTime	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.2.0	4909	-	F	NR_NTN_enh-Core

-	Ericsson agrees with the first change (but this requires a Rel17 change as well) and the second but not the third.

First and second change are agreed and merged into the R18 NR NTN rapporteur CR

The change to R17 spec for the first change will be merged into the overall R17 RRC rapporteur CR

The intention of the third change is agreed but we need to work on the actual change in the post meeting RRC CR review

UE capabilities corrections

R2-2406451	Remaining Issues on FR2-NTN Support	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Introduce a new R18 UE capability without signalling for enhanced FR2-NTN bands RRM requirements.

-	HW support this

-	QC thinks this should be discussed and RAN4 and they should request this

-	Ericsson wonders if we can just update the field with FR2 of this idle mode capability. Vivo thinks this would not be so clear

-	Samsung thinks this depends on RAN4

-	QC prefers to wait

Come back in the next meeting to check whether we Introduce a new R18 UE capability without signalling for enhanced FR2-NTN bands RRM requirements

Proposal 2: RAN2 sends an LS to check with RAN4 on the introduction of the new R18 UE capability.s

Proposal 3: For 38.306 and 38.331, it is RAN2's understanding that NTN bands within FR2-NTN can be regarded as a FR2-1 band.

-	HW thinks this is not needed

Proposal 4: If proposal 2 is agreed, RAN2 sends LS to check with RAN4 whether the understanding is correct.

Proposal 5: Updating the following description of the NTN related UE capabilities to indicate the support of NTN FDD-FR2 bands:

-	configuredUL-GrantType1-v1650;

-	configuredUL-GrantType2-v1650;

-	enhancedSkipUplinkTxConfigured-v1660;

-	enhancedSkipUplinkTxDynamic-v1660;

-	pusch-RepetitionMultiSlots-v1650;

-	pusch-RepetitionTypeA-v16c0;

-	rlm-Relaxation-r17;

-	bfd-Relaxation-r17.

37.335 corrections

R2-2406280	Correction on network verification of UE location	Huawei, HiSilicon, CATT	CR	Rel-18	37.355	18.2.0	0511	-	F	NR_NTN_enh-Core

-	QC thinks the UE would perform the measurement by default, without the need for an explicit request so this would be unnecessary

-	CATT indicates that RAN1 has already decide to include this parameter so we need to avoid the mismatch

-	ZTE also supports

-	Toyota suggests to remove “IE” in tow instances of the new field description

Revised in R2-2407629 to remove “IE”

R2-2407629	Correction on network verification of UE location	Huawei, HiSilicon, CATT	CR	Rel-18	37.355	18.2.0	0511	1	F	NR_NTN_enh-Core

Agreed, but then coversheet revised by MCC: spec number is missing a dot (i.e., 37355)

=> Revised in R2-2407869

R2-2407869	Correction on network verification of UE location	Huawei, HiSilicon, CATT	CR	Rel-18	37.355	18.2.0	0511	2	F	NR_NTN_enh-Core

=> Agreed

Draft a corresponding LS to RAN1 in R2-2407630

R2-2407630	LS on correction on network verification of UE location	CATT	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1

[Post127][309][R18 NR NTN] LS to RAN1 (CATT)

Scope: Draft a LS to RAN1 on correction on network verification of UE location

Intended outcome: Approved LS

Deadline for approved LS (in R2-2407630): short

=> Approved in R2-2407630 (LSout), but then coversheet revised by MCC in R2-2407887 (attachment was missing from the zip file)

## 7.8	Void

## 7.9	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Time budget: 0TU

Tdoc Limitation: 2 tdocs

### 7.9.1	Organizational

Including incoming LSs and rapporteur inputs.

### 7.9.2	Stage 2 corrections

Impact to 38.300. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CR

R2-2407059	draft_(Rel-18)_38.300 relay stage 2 CR_rapp	LG Electronics France	draftCR	Rel-18	38.300	18.2.0	NR_SL_relay_enh-Core

- Revised in R2-2407772

[AT127][411][Relay] Rel-18 relay stage 2 rapporteur CR (LG)

Scope: Check the rapporteur CR in R2-2407059

Intended outcome: Agreeable CR in R2-2407772

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407772	Corrections on NR sidelink relay enhancements	LG Electronics (Rapporteur)	CR	Rel-18	38.300	18.2.0	0897	-	F	NR_SL_relay_enh-Core

- “Other core specifications” should not be ticked on coversheet

- Typo fix in step 5 of figure 16.12.6.1-2 (“acknowlege”)

- Agreed with these changes as R2-2407756

R2-2407756	Corrections on NR sidelink relay enhancements	LG Electronics (Rapporteur)	CR	Rel-18	38.300	18.2.0	0897	1	F	NR_SL_relay_enh-Core

=> Agreed, but then coversheet revised by MCC: wrong revision number

=> Revised in R2-2407873

R2-2407873	Corrections on NR sidelink relay enhancements	LG Electronics (Rapporteur)	CR	Rel-18	38.300	18.2.0	0897	2	F	NR_SL_relay_enh-Core

=> Agreed

Other contributions

R2-2406698	Corrections for U2U relay	ZTE Corporation, Sanechips	CR	Rel-18	38.300	18.2.0	0883	-	F	NR_SL_relay_enh-Core

R2-2407267	Correction on NR SL Multi-path relay operation	Philips International B.V.	CR	Rel-18	38.300	18.2.0	0888	-	F	NR_SL_relay_enh-Core

### 7.9.3	RRC corrections

Impact to 38.331, except for capability-related issues (see agenda item 7.9.7). A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues where no clear conclusion was reached in [Post125][417] can be discussed based on contributions.

R2-2406368	Correction for U2N remote UE's serving cell during path switch	OPPO	draftCR	Rel-18	38.331	18.2.0	F	NR_SL_relay_enh-Core

- To be checked as part of the rapporteur CR discussion

Discussion:

Huawei agree there is an error in the current specification, but they think the network should make sure the PCI in the PCell configuration is the same one as the relay UE’s serving cell.  They think the same situation occurs for multi-path.  So they understand that the bullet is actually redundant and could be removed.

Ericsson think the point is that you maintain the indirect path and don’t update anything.  Huawei understand it is in PCellConfigCommon.  Ericsson think we could point to this IE.

ZTE think the “else” branch is needed for the case that sl-PathSwitch is configured and the indirect path is also included.  Huawei indicate that in the multi-path case, the indirect path is considered as a secondary path, and the network will perform reconfiguration with sync under the PCellConfigCommon and this indicates to the UE which cell is the target.  They think it can be checked in the RRC rapporteur CR update.

R2-2406369	Discussion on SLRB index in SUI for L2 U2U relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

- Noted

Proposal 1	For per-SLRB QoS report, allow L2 U2U Relay UE to set different SLRB index, remove the restriction that “sl-RemoteUE-SLRB-Identity is set to the same value as the SLRB-PC5-ConfigIndex received in RRCReconfigurationSidelink message from the L2 U2U Remote UE for the same end-to-end SLRB”, the text proposal is as annex.

Discussion:

Apple have the opposite proposal in their paper and think the same ID should be used.  They see that it is different from legacy operation where the ID comes from the network, and having different IDs would introduce additional steps for the implementation.

LG do not think the restriction is needed.

R2-2406556	Clarification on the filtering of SL-RSRP for U2U relay	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

- Noted

Proposal 1：For U2U relay, suggest RAN2 to confirm that it is unclear which filtering parameter is used for SL-RSRP for the cases other than integrated discovery.

Proposal 2:  If Proposal 1 is confirmed, suggest RAN2 to perform down-selection between the following two options:

	Option 1: Use sd-FilterCoefficientU2U;

	Option 2: Introduce new parameter sl-FilterCoefficientU2U.

Discussion:

Apple prefer option 2.  Huawei agree, but they would like to clarify the UE behaviour; a UE that does not implement the new parameter will apply the sd-FilterCoefficientU2U, or it is up to UE implementation.  Also, if the network does not provide the new parameter, we should clarify if the UE uses the sd-FilterCoefficient.

Nokia understand that the current spec forces the UE to use the sd-FilterCoefficient, and the question is whether we need a different coefficient to account for the difference between power-controlled and non-power-controlled messages.  If the SL-RSRP is power-controlled they think option 2 is the best way forward.

Qualcomm think we could leave it to UE implementation if it does not receive the new parameter; they are not sure it makes sense to use discovery parameters for communication.

Toyota think if we leave it to UE implementation the behaviour is not predictable.

Agreements:

Introduce a new parameter sl-FilterCoefficientU2U, in a BC way (may require extending a parent IE).

If the UE does not receive the sl-FilterCoefficientU2U, it uses the sd-FilterCoefficientU2U (this covers UEs that do not implement the new parameter and networks that do not send it).

To be captured in rapporteur CR.

R2-2406557	Correction on the indirect path failure reporting condition	CATT	draftCR	Rel-18	38.331	18.2.0	F	NR_SL_relay_enh-Core

- To be merged into rapporteur CR

R2-2406599	Remaining Open Issues in 38.331	Ericsson	discussion	Rel-18

- Noted (companies can check stage 2 to see if something further is needed)

Proposal 1	Add a NOTE stating that the assignment of the local IDs is up to U2U relay UE implementation. Adopt the TP as in the Annex.

Discussion:

Apple think it is not necessary to capture it as a NOTE; they think it is already clear based on the SRAP spec how the IDs are assigned.  ZTE agree with Apple; it is up to relay UE implementation, but they do not see a need to capture it.

Ericsson think it could be captured in stage 2, but it should be in the specs somewhere.  Apple think it is already captured in stage 2 that the relay UE assigns the IDs.  Lenovo checked the stage 2 spec and they agree with Apple.

R2-2406679	Discussion on remaining RRC issues for NR Sidelink relay enhancements	Apple	discussion	Rel-18	NR_SL_relay_enh-Core

- Noted

Proposal 1	RAN2 concludes that L2 U2U Relay UE shall set the same value of SLRB index in SUI from what it received from remote UE. No more spec change is needed for this issue.

Proposal 2	Specify the SRAP mapping configuration procedure for end-to-end SL-DRBs in clause 5.8.9.11 and 5.3.5.16 for IDLE/INACIVE/OCC case and CONNECTED case, respectively and remove corresponding text in 5.8.9.1a.2.

Discussion:

Huawei understand Apple’s view but think the UE behaviour is already complex.  They can double-check the specification but think we need a clear suggested change to evaluate.

Nokia have a bit different understanding from Apple about how the cases should be aligned.

Proposal 3	RAN2 discuss whether/how to consolidate some procedure texts in 5.8.9.1a.1, 5.8.9.1a.2, 5.8.9.1a.5, 5.8.9.1b, 5.8.9.7 into 5.8.9.1.9 (Reception of RRCReconfigurationCompleteSidelink).

R2-2406680	Miscellaneous corrections on TS 38.331	Apple	CR	Rel-18	38.331	18.2.0	4883	-	F	NR_SL_relay_enh-Core

- To be merged into the rapporteur CR

- Change 2 is subject to agreement during finalizing the CR

Discussion:

ZTE think the first change is not needed (the others are fine); they think it is clear that the configuration is derived from the sl-RLC-BearerConfig.  Apple clarify that the issue is that the IE name appears in two places.  ZTE understand the field names are not identical.  Huawei agree that the same IE name occurs twice, so they agree to adding the parent field name.

OPPO understand for the second change that the previous condition can cover both DRB release triggered by dedicated configuration and RLF, but the change excludes the RLF case and they wonder how the UE can derive the configuration after RLF.  Apple think the wording can be improved, but the point is to capture the missing case.  OPPO understand that the dedicated configuration is used also in the RLF case and the current specification is correct.  Apple think this is covered by existing text under RRCReconfiguration.

Huawei think there is a common understanding on the second change, but maybe the reason for change is not completely clear.  They think it would be OK to check the wording in the rapporteur CR discussion.  OPPO think another bullet would be needed for the RLF case.

R2-2406697	Corrections for SL relay	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.2.0	4886	-	F	NR_SL_relay_enh-Core

- To be merged into the rapporteur CR (with the exception of the second change)

- Change on T400 to be considered in interoperability analysis

Discussion:

ZTE indicate the second change may not be needed.

Apple think change 4 is OK and does not directly conflict with the Apple CR previously discussed.

Chair notes that the extended t400-u2u-r18 should have its own set of double square brackets.  Nokia think that if this parameter is missing, it is not clear what the expected UE behaviour is.  Huawei think the current description for T400 does not differentiate, so a UE that does not have the new timer will apply the existing T400.  OPPO are concerned in case there is a mismatch between UEs where T400 can be different for source and target UEs.  Chair thinks this can cause early RLF on one side of the connection; Huawei agree but think this is not worse than the current situation.

Philips think there is another field in the RemoteUEInformationSidelink where -r18 should be removed.  This can be handled in rapporteur CR.

R2-2406946	Miscellaneous corrections for SL relay enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4904	-	F	NR_SL_relay_enh-Core

- Revised in R2-2407790

Discussion:

Huawei indicate that one change was deemed unnecessary during offline discussion and will not be taken into the update.

[AT127][408][Relay] Rel-18 relay RRC rapporteur CR update (Huawei)

Scope: Merge decisions of this meeting into R2-2406946.

Intended outcome: Agreeable CR in R2-2407790

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407790	Miscellaneous corrections for SL relay enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4904	1	F	NR_SL_relay_enh-Core

- Changes on changes to be corrected

- sl-FilterCoefficientU2U field to be broken out into a separate field description table

- Agreed as R2-2407755

Discussion:

Huawei indicate some late comments were received, but they think these issues can be handled next meeting.  They also indicate there are changes on changes.

Nokia indicate there is a field that should have a separate field description table.

R2-2407116	Correction in U2U relay sidelink DRB addition/modification	Nokia	discussion	Rel-18	NR_SL_relay_enh-Core

- Noted

Proposal 1: The RLC channel related operations should be specified for UEs in RRC_CONNECTED in clause 5.8.9.1a.2.2.

Discussion:

Apple think this change impacts legacy Rel-17 operation.  Nokia see the point and think we can just note that something needs to be done, and further discuss towards next meeting what to do about it.

R2-2407268	RRC correction on NR SL U2U relay operation	Philips International B.V.	CR	Rel-18	38.331	18.2.0	4937	-	F	NR_SL_relay_enh-Core

- To be merged into rapporteur CR

Discussion:

Huawei think on the fifth change, it relates to coexistence of U2N/U2U.  Philips understand that the potential to trigger the SUI to request U2U Tx resources was missing.

R2-2407411	Discussion on U2U relay related issues	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

- Change from TP on proposal 2 to be merged into rapporteur CR

- Noted

Proposal 1. In SUI, Relay UE is allowed to use a Uu-SLRB-ConfigIndex value different from PC5-SLRB-ConfigIndex value provided by source remote UE.

Proposal 2. To clarify time unit of split PDB, RAN2 add the field description of sl-SplitPacketDelayBudget-r18 as shown in TP#1

### 7.9.4	SRAP corrections

Impact to 38.351. A single CR with miscellaneous corrections is requested from the specification rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

### 7.9.5	MAC, RLC, and PDCP corrections

Impact to 38.321, 38.322, and 38.323. A single CR for each specification with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2406600	Remaining Open Issues in 38.323	Ericsson	discussion	Rel-18

- Noted

Proposal 1	Remove the ‘/’ punctuation mark between SRAP entity and N3C. Adopt the TP as in the Annex.

Proposal 2	Include the text ‘RLC entities deactivated for PDCP duplication’ in the duplicate PDU discard section when PDCP duplication is deactivated.

Discussion:

Nokia think the slash in the first change was intended to mean one or the other.  For the second change, they think there should be no RLC entity which is kept activated for duplication, so the change may not be needed.

Huawei agree with Nokia on the first change and have no strong view on the second.

Qualcomm agree with Nokia that the second change is not needed.

InterDigital have no strong view on the first change and think it is just a style issue; on the second change, they agree with Nokia and Qualcomm that it is not needed.

Ericsson understand that a slash by default means “and/or”; on the second change, they wonder if it is possible to have CA and multi-path together, and deactivate only the RLC entities related to multi-path.  Nokia think this scenario is possible, but the “if” that Ericsson have touched is related to the case that all RLC entities under the DRB are deactivated.

R2-2406947	Correction on data transmission and data valume calculation in MP	Huawei, HiSilicon	draftCR	Rel-18	38.323	18.2.0	NR_SL_relay_enh-Core

- To be adapted to rapporteur CR

- “either” to be added to the sentence in the first change

- Merged into R2-2407765

Discussion:

Huawei indicated that the second change is not needed.  InterDigital have the same understanding and think it would impact legacy operation; they are OK with the first change and with the intention of the third change, but for the third change they want to check that legacy operation is not broken.

Nokia think the first change puts “and” and “or” in one sentence, and “either…or” would be clearer.

[AT127][409][Relay] Rel-18 relay PDCP rapporteur CR (InterDigital)

Scope: Develop a rapporteur CR incorporating decisions of this meeting (first and third changes from R2-2406947).

Intended outcome: Agreeable CR in R2-2407765

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407765	Correction on data transmission and data volume calculation in MP	InterDigital	CR	Rel-18	38.323	18.2.0	0140	-	F	NR_SL_relay_enh-Core

- Agreed

R2-2407293	RLC correction for multi-path relay with N3C	Huawei, HiSilicon	CR	Rel-18	38.322	18.1.0	0057	-	F	NR_SL_relay_enh-Core

- Merged into R2-2407751

Discussion:

Xiaomi think the intention is OK but the wording is not correct, because it is not the upper layer of any RLC entity.  Huawei agree the wording can be polished.

Apple have doubts about the second part and wonder if we need to clarify it.  They are OK with polishing of the wording.

Nokia agree with Xiaomi and think no clarification may be needed.  Huawei think the description of the N3C case is missing today.

Samsung think for the N3C case, the change is correct but the wording can be improved.

Nokia agree that there is something missing, but they think it will be difficult to find good wording.

Ericsson have the same concern as Nokia; they thought the N3C case should have no impact on the RLC spec.  Huawei think it is parallel to the other notes.

Samsung understand that the intention is to clarify what the RLC spec means, so the intention seems reasonable for the N3C case.

Nokia think the definition of RLC channel is clear, but we have this note to clarify the case that something other than PDCP is on top of RLC.  They think the intention is to clarify the different upper-layer cases, not to clarify the RLC channel terminology.

[AT127][410][Relay] Rel-18 relay RLC rapporteur CR (Xiaomi)

Scope: Develop a rapporteur CR taking into account the proposals of R2-2407293, and attempt to reach agreeable wording.

Intended outcome: Agreeable CR in R2-2407751

Deadline: Wednesday 2024-08-21 1900 CET

R2-2407751	RLC correction for multi-path relay with N3C	Xiaomi	CR	Rel-18	38.322	18.1.0	nnnn	-	F	NR_SL_relay_enh-Core

- Postponed

Discussion:

Xiaomi indicate there is still some discussion on the normative text.

Apple think there are issues with the existing NOTEs and the description in the new note does not cover all cases.

Huawei think we should not open the other two NOTEs now and the CR is correct.

OPPO have some sympathy with Apple’s concern regarding whether the RLC channel can be referred to as a relay RLC channel.  They would like some more time to check.

Nokia think the N3C part needs to be added as normative text and we could postpone.

Apple would like to discuss with the spec rapporteur and would be OK to postpone to next meeting.

Xiaomi agree it would be better to postpone.

Ericsson think some clarification is needed about exactly what we are trying to capture in the NOTE; they think the mapping to RLC channels is already clear and we should be careful what is captured in stage 3.

Huawei think the RLC spec today does not refer clearly to the behaviour with the N3C layer and the definition/clarification is needed.  Ericsson understand why something is needed in the PDCP spec where there are procedural changes for N3C, but here there is a one-to-one mapping of channels and they do not see ambiguity, especially considering that N3C is outside 3GPP scope.

### 7.9.6	UE capabilities

Impact to 38.306 and capability-related impact to 38.331. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2407103	Discussion on MP relay capabilities	Qualcomm Incorporated	discussion

- Noted

Proposal 1: Agreement 2 does not impact Agreement 1 and RAN2 confirms to introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

Discussion:

Qualcomm clarify that this means no change to the current specification.

### 7.9.7	Idle mode corrections

Impact to 38.304. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

## 7.10	Void

## 7.11	Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-231829)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.11.1	Organizational

LS in, rapporteur input

R2-2407750	Reply LS on the MBS broadcast service continuity and MBS session identification

Noted

R2-2407477	Miscellaneous correction on eMBS	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4955	-	F	NR_MBS_enh-Core

-	Nokia, QCM thinks the changes are not essential and we don’t need this CR.

[AT127][505][MBS] RRC CR (Huawei)

Scope: Update the RRC CR with the agreements from the meeting, discuss the related FFSes/wording improvements

Intended outcome: Agreeable RRC CR in R2-2407736

Deadline: CR available for offline approval: Friday 2024-08-23 0900

### 7.11.2	Corrections

Corrections for all specifications

R2-2406333	Corrections on UE behavior in Multicast MCCH-Less Cell	CATT, CBN, China Broadnet	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1: Upon receiving group paging which indicates to allow the inactive multicast reception, if multicast MCCH is not present, UE initiates RRC resume if it was not configured to receive multicast in RRC_CONNECTED. TP in Annex 1 is adopted.

Proposal 2: Upon receiving group paging which indicates to allow inactive multicast reception when UE is receiving multicast in RRC_INACTIVE, UE checks whether the selected or reselected cell is multicast MCCH-less cell before reading multicast MCCH. TP in Annex 2 is adopted.

Proposal 3: Upon receiving RRCRelease, UE checks whether multicast MCCH is present before monitoring the multicast MCCH-RNTI, if UE selected to the same cell as the one receiving the active session in RRC_CONNECTED. TP in Annex 3 is adopted.

DISCUSSION on P1:

- Nokia think the case in P1 is an error case.

- QCM agrees with the intent of P1, but wording can be improved.

- Huawei thinks that the situation in P1 can be avoided.

Upon receiving group paging which indicates to allow the inactive multicast reception, if multicast MCCH is not present, UE initiates RRC resume if it was not configured to receive multicast in RRC_CONNECTED. FFS the exact change (as part of offline#505)

DISCUSSION on P2:

- Huawei, vivo thinks the existing description already covers this case.

- Vivo thinks clarifying P1 is sufficient.

- QCM does not think current specs cover P2. Ericsson agrees with QCM.

- Nokia thinks this is an error case, QCM disagrees.

- Huawei thinks the UE will check this already during cell reselection.

Upon receiving group paging which indicates to allow inactive multicast reception when UE is receiving multicast in RRC_INACTIVE, UE checks whether the selected or reselected cell is multicast MCCH-less cell before reading multicast MCCH. FFS whether this is already covered by the current specs. (offline#505)

DISCUSSION on P3:

- QCM, Samsung supports the clarification. The wording can be simplified.

Upon receiving RRCRelease, UE checks whether multicast MCCH is present before monitoring the multicast MCCH-RNTI, if UE selected to the same cell as the one receiving the active session in RRC_CONNECTED.

R2-2406507	Corrections for Multicast Reception	Samsung	discussion	Rel-18

Proposal 1: RAN2 to discuss the issues for error data handling and decide to adopt either TP 1A or TP 1B.

a)	Error data handling should be limited to G-RNTI for "MBS multicast in RRC_INACTIVE" (TP 1A, TP 1B)

b)	Avoid redundancy between demultiplexing and error data handling procedures (TP 1B)

Proposal 2: Replace the term non-ServingCellMII used in clause 5.3.5.3 by “nonServingCellMII”. Adopt TP2.

Proposal 3: Add in the definition of PDSCH-ConfigBroadcast an additional term “multicast MCCH”. Adopt TP3.

DISCUSSION on P1:

- Apple thinks the intention is OK.

- Xiaomi thinks the clarification is needed.

- LGE thinks the current text is more generic, but it is still OK. It is already this is for suspended MRB.

- QCM slightly prefers to clarify.

RAN2 to adopt TP 1A to clarify that	error data handling should be limited to G-RNTI for "MBS multicast in RRC_INACTIVE"

[AT127][506][MBS] MAC CR (Samsung/Apple)

Scope: Draft MAC CR according to the agreements

Intended outcome: Agreeable MAC CR in R2-2407737

Deadline: CR available for offline approval: Friday 2024-08-23 0900

R2-2407737	Correction for error data handling for MBS	Samsung, Apple	CR	Rel-18	38.321	18.2.0	1923	-	F	NR_MBS_enh-Core

The CR is agreed

Replace the term non-ServingCellMII used in clause 5.3.5.3 by “nonServingCellMII”.

- DISCUSSION on P3:

- Huawei thinks that MCCH covers both MC and BC MCCH.

- QCM does not think this is clear as we use MC/BC MCCH in a lot of places.

- Huawei thinks we need to check all other specs and other places.

- QCM indicates that MC MCCH is a different logical channel than (legacy) MCCH.

Check whether we need to update MCCH to BC/MC MCCH. We come back next meeting.

R2-2406661	Data losing avoiding for multicast reception in RRC_INACTIVE	Sharp	discussion

Proposal 1 When UE initiates RRC resume procedure only for mt-SDT, it should start monitoring G-RNTI(s) of joined MBS session(s) indicated by the TMGI(s) included in the paging message.

DISCUSSION:

- Xiaomi thinks the network may send two Paging messages to avoid this issue.

- Sharp thinks that this is for RRC INACTIVE, so the gNB may not have information about UE being configured to receive MC in INACTIVE.

- Ericsson think the network may move the UE to RRC CONNECTED in response to MT-SDT resume request

- CATT thinks the issue is valid.

- Xiaomi thinks it is a corner case.

Offline to check if this is agreeable and how to capture in specs:

    - When UE initiates RRC resume procedure only for mt-SDT, it should start monitoring G-RNTI(s) of joined MBS session(s) indicated by the TMGI(s) included in the paging message. FFS how to capture in specs.  (Offline#507 Sharp)

[AT127][507][MBS] MBS and MT-SDT co-existence (Sharp)

Scope: Continue discussion on R2-2406661 to check whether/what needs to be captured in specifications

Intended outcome: Report with TP in R2-2407738

Deadline:  Report available for CB session on Thursday

R2-2407738	Summary of [AT127][507][MBS] MBS and MT-SDT co-existence (Sharp)	Sharp

When UE initiates RRC resume procedure with resumeCause set to mt-SDT, it should start monitoring G-RNTI(s) of joined MBS session(s) indicated by the TMGI(s) included in the paging message. FFS if there is spec impact (discuss in post-meeting e-mail discussion).

R2-2406953	[N103] [N105] Control plane aspects of multicast reception in RRC_INACTIVE state	Nokia	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1: When the UE receives stop monitoring G-RNTI indication in RRC release, it stops monitoring for data in the current cell and stop monitoring G-RNTI even after cell reselection to a cell that does not contain SIB24/MCCH (or PTM configuration).

Proposal 2: When the UE does not receive stop monitoring G-RNTI in RRC release, but only in MCCH, it stops monitoring data only in the cell where such indication is provided; and goes back to RRC_CONNECTED even after cell reselection to a cell that does not contain SIB24/MCCH (or PTM configuration).

DISCUSSION:

- Ericsson agrees with P1.

- Samsung thinks that P1 was already agreed.

- Nokia clarifies that in their view this is not captured in specs.

- Huawei thinks it is captured

Offline (Nokia) to check whether P1/P2 need to be reflected in specifications or current specifications is already correct. (CB Thursday)

After offline:

- Nokia thinks there is still a problem and we need to clarify the UE behaviour. Proposal to discuss: “When the UE which was configured to receive MBS multicast in INACTIVE reselects to a new cell where there is no SIB24, it should trigger RRC resume.”

- CATT indicates that in case the UE was indicated to stop G-RNTI in the previous cell, it should not resume.

- Samsung thinks UE should always resume to make sure it has the latest information about the ongoing sessions.

- Ericsson supports the proposal from Nokia. If we don’t agree then the UE can get stuck. Clarifies that the session status may be different in different cells.

- Huawei is not sure with this as the UE should wait for Paging. Missing Paging would be a corner case.

- ZTE thinks the proposal is reasonable.

When the UE which was configured to receive MBS multicast in INACTIVE reselects to a new cell where there is no SIB24, it should trigger RRC resume. Include the change in the post-meeting e-mail discussion for RRC.

R2-2407395	Validity of PTM configuration in RRCRelease	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1	Clarify that inactivePTM-Config IE is always included in multicastConfigInactive IE.

Proposal 3	RAN2 to discuss if it needs to clarified when the UE can use the PTM configuration received in RRCRelease.

DISCUSSION on P1:

- CATT thinks current specs is clear.

- Huawei thinks P1 is contradicting previous agreements and current specs.

- Nokia agrees with CATT/Huawei that how the current specification is done is the intention.

P1 is not pursued

DISCUSSION on P3:

- Qualcomm does not think we need to change anything.

- Xiaomi thinks it is not clear what happens with the stored configuration after the UE reselects the cell.

P3 is postponed, it needs to be better clarified what changes in specifications are needed

R2-2407474	Correction on broadcast reception for eRedcap UE	Huawei, HiSilicon	CR	Rel-18	38.300	18.2.0	0894	-	F	NR_MBS_enh-Core

CR is agreed

- Nokia thinks the changes make it more difficult to read. Nokia thinks we can simplify.

- Huawei, vivo clarify that the way in the CR is aligned with what RedCap session decided to do.

- ZTE thinks current wording is OK and aligned with NGAP.

- Qualcomm thinks the proposal is OK, perhaps NGAP should also be corrected.

R2-2407526	RedCap UE's Multicast reception in RRC_INACTIVE - not a good idea	ZTE Corporation, Sanechips	discussion	Rel-18	NR_MBS_enh-Core

Noted

MBS TEI18

NOTE: Include TEI identifiers in agreed CRs.

R2-2406281	Correction on the capabilities on PTM retransmission	Huawei, HiSilicon, Intel Corporation, Nokia, vivo	CR	Rel-18	38.306	18.2.0	1134	-	F	NR_MBS_enh-Core, TEI18

R2-2406282	Correction on the capabilities on PTM retransmission	Huawei, HiSilicon, Intel Corporation, Nokia, vivo	CR	Rel-18	38.331	18.2.0	4867	-	F	NR_MBS_enh-Core, TEI18

Add TEI18 identifier in the CR titles

To be discussed with UE capability rapporteur how to handle the clash with CRs agreed due to NTN support in MBS

The CRs are revised in R2-2407809 and R2-2407810

- Chair: After discussion in the Main session, it was concluded that the clash can be handled during CR implementation phase

R2-2407809	Correction on the capabilities on PTM retransmission [PTM_ReTx_Mcast_HARQ_Disb]	Huawei, HiSilicon, Intel Corporation, Nokia, vivo, Samsung	CR	Rel-18	38.306	18.2.0	1134	1	F	NR_MBS_enh-Core, TEI18

The CR is agreed, but then coversheet revised by MCC: spec number is missing a dot (i.e., 38306)

=> Revised in R2-2407874

R2-2407874	Correction on the capabilities on PTM retransmission [PTM_ReTx_Mcast_HARQ_Disb]	Huawei, HiSilicon, Intel Corporation, Nokia, vivo, Samsung	CR	Rel-18	38.306	18.2.0	1134	2	F	NR_MBS_enh-Core, TEI18

=> Agreed

R2-2407810	Correction on the capabilities on PTM retransmission [PTM_ReTx_Mcast_HARQ_Disb]	Huawei, HiSilicon, Intel Corporation, Nokia, vivo, Samsung	CR	Rel-18	38.331	18.2.0	4867	1	F	NR_MBS_enh-Core, TEI18

The CR is agreed, but then coversheet revised by MCC: spec number is missing a dot (i.e., 38331)

=> Revised in R2-2407875

R2-2407875	Correction on the capabilities on PTM retransmission [PTM_ReTx_Mcast_HARQ_Disb]	Huawei, HiSilicon, Intel Corporation, Nokia, vivo, Samsung	CR	Rel-18	38.331	18.2.0	4867	2	F	NR_MBS_enh-Core, TEI18

=> Agreed

R2-2406345	Correction on PTM Retransmission Capability	Samsung	CR	Rel-18	38.306	18.2.0	1135	-	F	NR_NTN_solutions-Core, NR_MBS-Core

The CR is agreed and merged into R2-2406281

DISCUSSION:

- Huawei thinks this was discussed in the common session last meeting and the understanding was that this is already clear form other capability descriptions.

- Nokia thinks this CR captures properly what’s been agreed.

R2-2407527	Search space configuration for RedCap UE’s MBS broadcast reception	ZTE Corporation, Sanechips	discussion	Rel-18	TEI18

Postponed to the next meeting to try to come up with an acceptable wording (e.g. a guideline for the NW rather than requirement)

- ZTE thinks the intention seems to be agreeable, but the exact change can be discussed further.

- Ericsson agrees it is good to capture this use case and the current wording is OK.

- Nokia does not see the need for this implementation guideline, but is OK if majority wants it. Huawei agrees with Nokia. Previous sentence already covers it for the most part, but the NW may not be able to ensure what is proposed to be captured.

- CATT agrees with Nokia and Huawei. Even if it happens, the UE will attempt to decode.

- QCM prefers network to follow the guideline that is proposed..

## 7.12	Void

## 7.13	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

Includes LS in’s related to AI/ML for NG-RAN

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.13.1	Organizational

Ls in and Rapporteur input. WI/Spec Rapporteur(s) are invited to provide updated open issues lists that need to be handled.

R2-2406218	Reply LS on MDT for NPN (R3-243892; contact: Ericsson)	RAN3	LS in	Rel-18	NR_ENDC_SON_MDT_enh2-Core	To:RAN2	Cc:SA2, SA5

Noted

R2-2406219	Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics (R3-243941; contact: Nokia)	RAN3	LS in	Rel-18	NR_AIML_NGRAN-Core	To:SA5	Cc:SA2, CT3, CT4, RAN2

Noted

R2-2407117	Stage 2 alignments with stage 3 specification	Nokia, NEC	CR	Rel-18	37.320	18.2.0	0136	-	F	NR_ENDC_SON_MDT_enh2-Core

-	ZTE think that the second change is not correct since CHO recovery is not a clearly defined operation, current spec wording is correct. NEC think CHO recovery is used in 38.300.

Agreed

R2-2407338	Miscellaneous corrections on R18 SONMDT for 36.331	Huawei, HiSilicon	CR	Rel-18	36.331	18.2.0	5050	-	F	NR_ENDC_SON_MDT_enh2-Core

Moved from 7.13.2

Could put it in an editorial CR if there is a recommendation that we should write “included” rather than “set to true” and there is risk of misunderstanding. Should check with spec rapporteur

After check with the LTE RRC specification rapporteur, this will be handled by him in CR implementation

### 7.13.2	Corrections

R2-2406997	Miscellaneous SON corrections	ZTE Corporation, Sanechips,	CR	Rel-18	38.331	18.2.0	4910	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Google agrees with the first change, but the second change does not cover all cases. Ericsson thinks the CR is not needed. Samsung agrees with Ericsson and even think that with this CR there would be some issues created, e.g. when the field should be released. ZTE would be OK for, regarding the second change, that we instead clarify that it is the source cell. Ericsson, Nokia, Qualcomm think it is a corner case and think current spec is OK.

Not pursued. For the second change, even if it is a valid case, we think it is a rare case. Our understanding is that the UE logs the source cell.

R2-2407000	 Corrections on SPR configuration	CATT	draftCR	Rel-18	38.331	18.2.0	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung thinks that it is possible for inter-SN so the CR is not correct.

Postponed

R2-2407038	RRC Corrections for SON/MDT	Samsung, Ericsson	discussion

Proposal 1: If attemptCondReconfig or attemptLTM-Switch are configured while initiating RRC Reestablishment procedure, UE keeps the successPSCell-Config configured by the PSCell. RIL S254 is closed.

Proposal 2: UE keeps the successHO-Config and successPSCell-Config during full configuration.

On P1:

-	ZTE is OK with P1. Qualcomm doesn’t see the need for this.

On P2:

-	ZTE thinks that the target can provide the field. Qualcomm doesn’t see the need for this, it is an enhancement. Samsung this there will be a misalignment between the UE and the NW. Ericsson think this proposal is needed to avoid misalignment. Huawei agrees this is needed.

Try to find agreeable wording for a TP offline and merge with other changes to RRC, if any.

[AT127][609][SONMDT] RRC Corrections for SON/MDT (Samsung)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2407663 (Samsung)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2407663	Rel-18 SONMDT Corrections	Ericsson, Nokia, Samsung	CR	Rel-18	38.331	18.2.0	4968	-	F	NR_ENDC_SON_MDT_enh2-Core

Agreed, but then coversheet revised by MCC: Spec number is missing a dot (i.e., 38331)

=> Revised in R2-2407871

R2-2407871	Rel-18 SONMDT Corrections	Ericsson, Nokia, Samsung	CR	Rel-18	38.331	18.2.0	4968	1	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2407118	RAN2 impacts of RAN3 reply LS on MDT for NPN (R2-2406218/R3-243892)	Nokia	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

The TP is agreeable and to be merged in the CR above.

R2-2407217	Addressing SONMDT issues	Ericsson	discussion	NR_ENDC_SON_MDT_enh2-Core

Proposal 2	RAN2 capture in the RRC spec that plmn-identityList is not included in case of MDT collection in SNPNs. E268 is closed.

Proposal 3	RAN2 agree that similar to the procedural text for CEF in public network UE compares the registered snpn-identity with the snpn- identity logged in VarConnEstFailReport when appending the VarConnEstFailReport to the VarConnEstFailReportlist.

Proposal 4	RAN2 agree to fix the missing correction in [R2-2405968] (changing VarConnEstFailReportlist to VarConnEstFailReport) in the next version of the RRC TS.

Proposal 5	RAN2 agree to the draft CRs in the annex of the paper.

RAN2 capture in the RRC spec that plmn-identityList is not included in case of MDT collection in SNPNs. E268 is closed.

RAN2 agree that similar to the procedural text for CEF in public network UE compares the registered snpn-identity with the snpn- identity logged in VarConnEstFailReport when appending the VarConnEstFailReport to the VarConnEstFailReportlist.

RAN2 agree to fix the missing correction in [R2-2405968] (changing VarConnEstFailReportlist to VarConnEstFailReport) in the next version of the RRC TS.

R2-2407337	Correction to 38.306 on SON reports	Huawei, HiSilicon	CR	Rel-18	38.306	18.2.0	1146	-	F	NR_ENDC_SON_MDT_enh2-Core

Agreed

R2-2407367	SCG failure information in fast MCG recovery MRO	Sharp	discussion	Rel-18

Proposal 1: remove “beamFailure” from the values of scg-FailureCause.

Proposal 2: Discuss whether UE records SCG failure information for fast MCG recovery failure in RLF report upon SCG failures due to synchReconfigFailureSCG, scg-ReconfigFailure, srb3-IntegrityFailure during fast MCG recovery procedure.

Proposal 3: Discuss how to capture case 8~10 in RRC specification if case 8~10 are valid cases for SCG failure during fast MCG recovery procedure.

On P1:

-	Qualcomm does not agree since they think the cause value can happen in a deactivated state before the SCG failure happened. Sharp disagrees and think the procedural text says that in this case the UE uses another cause. Qualcomm is not convinced. Nokia think that even if the cause is not used in procedural text, the ASN.1 can be left as is.

On P2:

-	Qualcomm think these are corner cases and do not need to be addressed. Ericsson agrees with Qualcomm. LG agrees with Qualcomm.

On P3:

-

Noted. We consider the cases 8-10 to be corner cases.

R2-2407373	Correction on the successPSCell-Config handling	Google	CR	Rel-18	38.331	18.2.0	4948	-	F	NR_ENDC_SON_MDT_enh2-Core

Noted.

## 7.14	Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.14.1	Organizational

LSs and rapporteur inputs

R2-2407088	Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson	CR	Rel-18	38.331	18.2.0	4922	-	F	NR_QoE_enh-Core

- Samsung indicates appLayerBufferLevelList has granularity of 10ms, so the change in the CR is not correct.

Remove “ms” from “If the buffer level is larger than the maximum value of 30000 ms (5 minutes), the UE reports 30000.”

Other changes are agreeable

CR to be updated with the agreements from the meeting

[AT127][503][QoE] RRC CR (Ericsson)

Scope: Update the RRC CR with the agreements from the meeting

Intended outcome: Agreeable RRC CR in R2-2407734

Deadline: CR available for offline approval: Friday 2024-08-23 0900

R2-2407734 Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson, CATT, ZTE, Google, Huawei, Samsung	CR	Rel-18	38.331	18.2.0	4922	1	F	NR_QoE_enh-Core

The CR is agreed

### 7.14.2	Corrections

Corrections to all specifications.

R2-2407001	The correction for RRC spec for R18 QoE	CATT	draftCR	Rel-18	38.331	18.2.0	F	NR_QoE_enh-Core

- Huawei thinks first change is not needed as the added condition is always true.

- CATT clarifies that maybe this can be reflected in the field description. ZTE is OK to clarify in the field description.

- Nokia thinks 1st change is not needed. Ericsson agrees.

- Ericsson thinks 2nd change is incorrect.

1st change is not needed

Discuss 2nd change offline (offline#503, Ericsson)

Change 4 is agreed and will be merged into rapp CR

R2-2406998	Consideration on QoE configuration release during inter-RAT mobility	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1: Specify in LTE RRC specs that the UE should release the IDLE/INACTIVE states QoE configurations/reports stored in the AS layer, if any, upon successful RRC connection establishment; and inform the application layer to release the corresponding QoE configuration stored.

Proposal 2: Send LS to inform SA4 and CT1 that RAN2 has agreed to update LTE RRC specs that upon successful RRC Connection establishment procedure in LTE UE will release the NR QoE configurations/reports stored in both the AS layer and the application layer, and ask them to check if their specs requires any updates.

Proposal 3: Add in the description of AppLayerIdleInactiveConfig in RRC specs that this parameter is also applicable to connected states.

DISCUSSION on P1:

- Samsung thinks that this is a relevant issue, but we should not impact LTE.

- QCM also thinks this is a relevant issue, but the UE may also go back to NR. Perhaps it is best to specify this upon UE entering RRC IDLE in NR specs.

- Ericsson acks the problem, perhaps offline is needed to decide how to solve it.

- Huawei thinks another scenario is when UE is in NR RRC IDLE state and then the UE moves to LTE cell for connection establishment. OK to discuss offline.

Offline on the issue of improperly stored IDLE/INACTIVE QOE configs and how to solve it (ZTE)

[AT127][504][QoE] Release of QoE configurations (ZTE)

Scope: Discuss whether/what changes are needed for release of QoE configurations as per R2-2406998 and/or R2-2407090

Intended outcome: Report in R2-2407735 with the agreeable TPs

Deadline: Report available for CB session on Thursday

DISCUSSION on P3:

- Ericsson thinks some rewording may be needed

Discuss offline (offline#503)

R2-2407735	Draft Report of [AT127][504][QoE] Release of QoE configurations (ZTE)	ZTE Corporation

Proposal 1: UE shall release its stored NR QoE configuration and possible QoE reports for below cases:

	Case 1: UE reselects to EUTRA after during RRC connection reestablishment and establish connections to EUTRA

	Case 2: UE is in NR RRC IDLE state and then the UE moves to LTE cell for connection establishment.

Proposal 2: RAN2 agrees on the corrections provided in R2-2407090. If not agreeable, RAN2 captures in  chairman’s note that RAN2 understands based on existing specs, UE will release all LTE QoE configurations when HO from LTE/5GC to NR.

- Qualcomm thinks another case to consider is when the UE in RRC ILDE/INACTIVE reselects to EUTRA.

- ZTE thinks we need to check this case further.

UE shall release its stored NR QoE configuration and possible QoE reports/variables for below cases:

    - Case 1: UE reselects to EUTRA after during RRC connection reestablishment and establish connections to EUTRA

    - Case 2: UE is in NR RRC IDLE state and then the UE moves to LTE cell.

We intend to correct this from Rel-18. FFS where (LTE or NR specs) and how to address this in specs.

DISCUSSION on P2:

- Huawei thinks that this is already covered.

- Ericsson thinks there may be a fault in the specifications, because it is unclear whether certain actions are executed.

- Lenovo also thinks what we already have is generic and should apply.

R2-2407090 is postponed to check if specs already covers properly the needed behaviour

R2-2407168	Miscellaneous Stage-2 corrections on R18 QoE	Nokia, Nokia Shanghai Bell,China Unicom	CR	Rel-18	38.300	18.2.0	0886	-	F	NR_QoE_enh-Core

-	Ericsson think 1st change is worded strangely, so we may improve the text.

Change “The QoE Measurement Collection deactivation permanently stops all or some of the QoE measurement collection(s) configured at a UE” to “The QoE Measurement Collection deactivation permanently stops all or some of the QoE measurement collection configurations at a UE”

With this change the CR is agreed unseen in R2-2407731

R2-2407336	Correction on area scope checking for MBS QoE	Huawei, HiSilicon	draftCR	Rel-18	38.331	18.2.0	F	NR_QoE_enh-Core

-	Ericsson agrees with the second change. 1st change is OK, but it causes another issue, so perhaps it is not needed. QCM agrees with Ericsson.

-	Samsung also does not think the first change is not needed.

First change is not needed

2nd change is agreed

R2-2407339	Correction on application layer measurement report re-submittion	Google	CR	Rel-18	38.331	18.2.0	4946	-	F	NR_QoE_enh-Core

-	Ericsson agrees we need to correct this.

The change is agreed

=> Merged to R2-2407734

R2-2407090	Release of QoE measurements at successful handover from LTE	Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-18	36.331	18.2.0	5048	-	F	NR_QoE_enh-Core

Moved from 7.0.2.8

- QCM thinks the CR is OK, but wonders whether there is an issue with the other direction.

- Huawei thinks that in the current specifications it is already clear.

- ZTE thinks the procedure mentioned by Huawei is only executed in the failure case.

Offline (discuss as part of offline#504)

## 7.15	NR Sidelink evolution

(NR_SL_enh2; leading WG: RAN1; REL-18; WID: RP-230077)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.15.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2406209	Reply LS on Sidelink Feature Co-configuration (R1-2405530; contact: OPPO)	RAN1	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2

- Noted.

- MAC CR rapporteur will update the corresponding MAC changes (if needed).

### 7.15.2	Corrections

Including corrections to all specifications. A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. A contribution can include multiple TPs. Note RRC and MAC CR rapporteurs’ summary and suggestion (based on the submitted contributions) may be provided.

TX carrier selection for SL MAC CE:

P1 in R2-2406746 (LG)

Proposal 1. Carrier selection of SL IUC Request MAC CE, Condition based SL IUC Information MAC CE and SL DRX command MAC CE uses the same procedure as the carrier selection procedure of logical channel data.

P1 in R2-2407131 (Nokia)

Proposal 1: Enhancements to Tx carrier (re-)selection for SL IUC Request MAC CE, Condition based SL IUC Information MAC CE should not be considered.

[Session chair]: TX carrier is part of resource selection. How the UE performs TX carrier for MAC CE according to the current MAC? [Huawei]: MAC will wait until grant is created due to other data transmission. Should be ok since those MAC CEs are not delay sensitive. [LG]: In Rel-16 and Rel-17, MAC generates a new grant for MAC CE. It doesn’t make a sense that MAC should wait until a grant is made for LCH data transmission in Rel-18. [Ericsson, Apple, NEC]: Share the view with LG.

- P1 in R2-2406746 is agreed. Detailed wordings will be discussed in MAC CR preparation.

Legacy carrier selection in TX carrier selection:

R2-2406700	Correction on legacy carrier selection	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.2.0	1889	-	F	NR_SL_enh2-Core

[CATT]: if the upper layer indicates single carrier, is it natural to assume no need to perform TX carrier selection among multiple carriers? [Session chair]: Suggest offline discussion among interested companies, e.g. whether the current MAC works w/o change? Or with adding the new sentence (to make it clearer), any real issue or further spec impacts?

- Comeback in Thursday CB session.

In CB session:

R2-2407800	Discussion on legacy carrier selection	ZTE Corporation, Sanechips	Discussion	NR_SL_enh2-Core

Proposal1: If PDCP duplicated is enabled, regarding the LCH carring QoS flow of Tx profile = BC, RAN2 is suggested to discuss following options on how to use the legacy carrier:

| Not ignore CBR threshold | Network set the CBR threshold of legacy carrier to 100%, specifically for the dedicated prio of the LCH carrying QoS flows of TxProfile = BC | (Option1a)Current spec works well | (Option1)Spec change is not needed |
| --- | --- | --- | --- |
| Ignore CBR threshold | single carrier branch& multiple carrier branch is performed per LCH channel, RLC with legacy carrier will perform single carrier branch | (Option1b)Current spec works well |  |
|  |  | (Option2)determination of single/multiple carrier operation is now per UE, not per LCH, spec change is needed to capture this understanding | Spec change is needed |
|  | Clairfy in 5.22.1.11 that UE will select legacy carrier directly, if indicated by upper layer to use | (Option3)by a Note or Normative Text |  |

[OPPO]: Think option1a should be sufficient. Assume backward compatible data and non-backward compatible data would be set as different QoS flows. [Xiaomi]: Don’t think option2 is correct. Resource selection that will trigger TX carrier selection occurs once LCH data is arrived. It is not per UE operation. [Qualcomm]: Agree with Xiaomi. Don’t think MAC is broken. [ZTE]: Triggering is per LCH, however when to compare whether it’s single carrier or multiple carriers, it is defined per UE. [IDC, LG, Apple, Vivo]: Prefer Note based option. [OPPO]: It is ok with adding a note. However, in the example TP, it is too general. It needs to be more precise according to RAN2 previous agreement. [ZTE, IDC]: Example TP was made to be aligned with what is specified in RRC. [Nokia]: Prefer original proposal (normative text in TX carrier selection).

- Add a note to clarify the intention according to what RAN2 agreed into MAC. Detailed wording will be discussed as part of MAC CR preparation.

TP on IUC/DRX in Co-Ex:

R2-2406584	Discussion on MAC corrections when supporting IUC and DRX in Co-Ex	LG Electronics France	discussion	Rel-18	38.321	NR_SL_enh2

[Apple, Ericsson]: Looks quite complicated. Can we consider another structure to capture it? [OPPO, Vivo, Huawei]: Thinks this change is clear and it is the best we can do. Support the change. [Vivo]: Would like to have more time to check more carefully (e.g. in short email discussion). [Huawei]: Ok to agree with it now. [Apple]: Will prepare simpler MAC change until Thursday CB session.

- Apple’s MAC correction is provided in R2-2407594

- Comeback in Thursday CB session. Will compare two versions and select one of two.

R2-2407594	TP for reosuece selection when supporting both IUC and Co-Ex	Apple, Ericsson	discussion	Rel-18	38.321	NR_SL_enh2

[LG]: Don’t think with this TP, all UE behaviours in various cases are really readable. [OPPO]: Think both works and prefer simpler version (Apple). [Qualcomm]: IUC is performed in UC only while Co-Ex supports all cast types. Not sure if this TP covers all cases. [Session chair]: For scheme 2, it seems IUC only case is now removed.

- R2-2407594 is baseline for IUC + Co-Ex correction.

- For scheme 2, IUC only case needs to be included.

- Further details will be discussed as part of MAC CR preparation.

Correction on MCSt:

R2-2406316	Correction on selection of resources for MCSt	CATT, CICTCI	draftCR	Rel-18	38.321	18.2.0	F	NR_SL_enh2

- Intention (to specify selection of multi-shots resources for MCSt approach 2) is agreed.

- Detailed wordings can be further discussed in MAC CR preparation.

[LG]: Understand “one transmission opportunity” already covers multiple slots if a number of consecutive slots is provided by L1. [CATT]: One transmission opportunity has been an old term, which indicates single slot from Rel-16. [Qualcomm]: Understand the intention. With this change, do we need additional specification efforts to define a relationship between a grant and a number of consecutive slots, e.g. a number of consecutive slots will be considered as single grant or multiple grants? How does it impact HARQ processes and retransmissions? etc. [Vivo]: Support the proposal. On Qualcomm’s question, think no more specification effort is needed. Think a number of consecutive slots will be considered as single grant. [CATT]: Suggest to agree with intention. We can further discuss detailed wordings or other aspects during MAC CR preparation.

Correction on prioritization:

R2-2406518	Discussion on SR prioritization regarding SL transmission	ASUSTeK	discussion	Rel-18	38.321	NR_SL_enh2

- Noted.

[Session chair]: For the first proposal that is related to SL-PRS, it would be better to discuss it in positioning session. [OPPO]: Current MAC specifies a collision with two channels and it is clear a collision with three channels would bring the problem as described. Then NW should avoid this collision by implementation. [LG]: Although understand the intention, also agree with OPPO. We can rely on NW implementation to avoid this issue. [ASUSTek]: Not clear NW can always avoid this issue. If cannot avoid the issue, we need to define UE behaviour.

Correction on LCP restriction:

R2-2406806	Correction to per-LCH carrier set restriction in LCP	Ericsson	draftCR	Rel-18	38.321	18.2.0	NR_SL_enh2

- Agreed.

[Huawei]: RAN2 agreed this information is for LCP. We should keep it. [OPPO]: sl-AllowedCarriers is not the only one to be considered. It will be further filtered out based on other factors, e.g. UE capabilities, etc. [OPPO]: Allowed carriers is the term that the upper layer indicates to MAC after all filtering out. Note the allowed carriers are already considered in LCP.

Others on MAC:

R2-2406596	Miscellaneous corrections for SL evolution	Huawei, HiSilicon	discussion	Rel-18	NR_SL_enh2-Core

R2-2407381	MAC corrections on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1907	-	F	NR_SL_enh2

- Correction 1 was agreed.

[POST127][101][V2X/SL] (LG)

Scope: Discuss changes in R2-2407381 (only correction 1) and capture all agreements made in RAN2#127. => It is extended to short email discussion. Capture all RAN2#127 agreements.

Intended outcome: MAC CR in R2-2407581. => Agreed MAC CR in R2-2407603.

Deadline: Short offline discussion.

=> Agreed in R2-2407865 (38.321 CR#1922r2), but then coversheet revised by MCC: Wrong spec version (18.1.0 -> 18.2.0)

=> Revised in R2-2407878

R2-2407878	MAC corrections on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1907	3	F	NR_SL_enh2

=> Agreed

R2-2407581	MAC corrections on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.2.0	1922	-	F	NR_SL_enh2 => Will revised to R2-2407603

R2-2406265	Miscellaneous correction on R18 SL Evolution	OPPO	CR	Rel-18	38.331	18.2.0	4863	-	F	NR_SL_enh2

- Agreed.

R2-2407372	Correction on setuprelease type sidelink fields handling	Google	CR	Rel-18	38.331	18.2.0	4821	1	F	NR_SL_enh2	R2-2405322

- Not pursued.

[AT127][102][V2X/SL] (OPPO)

Scope: Discuss changes in R2-2406265 and R2-2407372, capture all agreements made in RAN2#127 and update RRC CR with merging all corrections (if agreeable).

Intended outcome: RRC CR in R2-2407582.

Deadline: Comeback in Thursday CB session. => Completed.

In CB session:

[OPPO]: During offline discussion, most companies didn’t agree with R2-2407372. We can only agree with R2-2406265 (no merged CR in R2-2407582). [Session chair]: Ask any need to extend it to short email discussion to capture any further RAN2 agreement. [OPPO]: No.

R2-2407388	Rapporteur Stage 2 Corrections for NR Sidelink Evolution	InterDigital France R&D, SAS, NEC	CR	Rel-18	38.300	18.2.0	0893	-	F	NR_SL_enh2

R2-2406554	TP for SL CA in TS38300	NEC	discussion	NR_SL_enh2

[AT127][103][V2X/SL] (IDC)

Scope: Discuss changes in R2-2407388, R2-2406554 and update CR with merging all corrections (if agreeable)

Intended outcome: 38.300 CR in R2-2407583.

Deadline: Comeback in Thursday CB session. => Completed.

R2-2407583	Rapporteur Stage 2 Corrections for NR Sidelink Evolution	InterDigital, NEC	CR	Rel-18	38.300	18.2.0	0893	-	F	NR_SL_enh2

- Agreed.

R2-2406264	Discussion on LS R1-2405530	OPPO	discussion	Rel-18	NR_SL_enh2

R2-2406746	Discussion on carrier selection for SL MAC CE(s)	LG Electronics Inc.	discussion	Rel-18	38.321	NR_SL_enh2

R2-2407131	Carrier reselection for IUC	Nokia	discussion

## 7.16	Void

## 7.17	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-233071)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 7.17.1	Organizational

Incoming LS, Rapporteur input, etc.

Corrections to TS 38.300.

R2-2407186	Corrections to TS 38.300 for R18 MUSIM	China Telecom, Samsung	CR	Rel-18	38.300	18.2.0	0887	-	F	NR_DualTxRx_MUSIM-Core

Agreed, but then coversheet revised by MCC: “Proposed change affects” and “Source to TSG”  are empty

=> Revised in R2-2407870

R2-2407870	Corrections to TS 38.300 for R18 MUSIM	China Telecom, Samsung	CR	Rel-18	38.300	18.2.0	0887	1	F	NR_DualTxRx_MUSIM-Core

=> Agreed

### 7.17.3	Corrections

R2-2406715	Discussion on configuration for temporary capability restriction	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Discussions

- Xiaomi do not think this is critical and think it is a signalling optimization. Nokia agree with this view.

- Samsung think Option 1 is ok. QC share this view.

- LG E understand the intention of this CR, and think Option 2 is better as Option 1 is NBC. ZTE agree with Option 2.

- Vivo think NW has the way to configure properly and think it is not so critical as this stage.

- Intel and Ericsson think changing need code as in Option 1 does not create NBC issue.

Discussions in CB

-	HW explains based on offline discussions option 1 seems agreeable.

Option 1 in P1 is agreed. The change will be taken into account in Rapp’s CR.

R2-2407077	Further discussion on MUSIM capability restriction signalling	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Discussions

P1

- QC, ZTE agree.

- Xiaomi think the functionality in P1 is already there in the spec.

- Huawei think it is a valid issue and think it is useful to clarify in the spec.

- Samsung think there is no such restriction that UE cannot include multiple entry for the same band. Samsung support P1.

- Huawei wonders whether we also need some clarification for the avoided bands. Xiaomi think we may need some changes. Samsung think we do not need any change for that part.

Band entry (MUSIM-CapabilityRestrictedBandParameters-r18 with same musim-bandEntryIndex ) that appear more than once in musim-AffectedBandsList indicate that the UE supports intra-band non-contiguous CA on the band.

Chair: companies can check if any change for avoided band is needed.

P2

- Nokia think more clarification is needed.

- QC think this proposal is fine.

- Samsung think this clarification better fit in the description of the candidate band list.

- HW think it is needed that UE can indicate PCell in this case but the intention is to release SCell.

Intention of P2 is agreeable, can discuss further how to capture (e.g., whether the clarification is reflected in other places in the spec.)

Discussions in CB

-	Ericsson think based on offline discussions we can go with the proposal 2 in the contribution. QC agree.

P2 is agreed. The change will be taken into account in Rapp’s CR.

R2-2407104	Clarification on DAPS Handover for Dual TX/RX MUSIM operation	Nokia	discussion	Rel-18

Noted

Discussion

- HW do not think this we need to capture this restriction and think it is up to NW/UE implementation.

- Xiaomi think this does not impact DAPS feature and think it can be left to implementation.

- Nokia think with such combination there may be need for more coordinates btw the NW nodes and think those are not discussed sufficiently. Samsung wonders what is the R3 impact if we allow such combination.

- CATT also think it is up to NW implementation and we do not need further spec impact.

R2-2407321	Further Clarification on the Reconfiguration Failure Processing	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Discussions

- Samsung think if we agree the changes there may be some duplication and we should simplify.

?? the proposed changes are agreed, and the texts ‘in case the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation’ in the next sentence is removed.

Discussions in the CB

-	ZTE have updated TP based on offline discussions.

The following changes are agreed. Will be reflected in Rapp’s CR.

NOTE 2: If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the MUSIM case (i.e. that the UE is configured to provide MUSIM assistance information for temporary capability restriction and is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation). For the MUSIM case, the UE does not apply above failure handling in case the UE is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation, and it is up to UE implementation how to apply RRCReconfiguration message. If UE does not perform RRC reconfiguration failure in this case, UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.7.4 and still considers the configuration resulting from RRCReconfiguration message as the current configuration and baseline for delta configuration for future reconfigurations.

R2-2407515	Remaining issues on MUSIM	Samsung Electronics Czech	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Discussion

P1

- Ericsson think if there is issue in the spec then we should better reflect in the spec. Samsung think this is to help the implementation.

- Vivo think the spec is ok, so no change is needed. HW also think the spec is clear and is OK to capture the texts in the minutes. Xiaomi agree no spec change is needed. LG E agrees.

- QC think it is the intended behaviour and think it is OK to add a note.

RAN2 understands that upon T348 expiry, the UE still considers the latest configuration as the current configuration but does not perform any required behaviors about dedicated configuration associated with musim-CapRestriction. No spec impact is needed

P2

- LG E think current spec is fine so no need to change.

Discussions in CB

-	Samsung explains that based on offline companies have common understanding but this hasn’t been explicitly captured as agreements.

RAN2 confirms that UE include the musim-GapKeepPreference only if musim-GapPriorityPreferenceList is included as specified in 38.331. No spec change.

Proposal 3: If the measurement gap requirements received in the musim-needForGapsInfoNR is included in the AS-Context, source gNB does not include the needForInterruptionInfoNR and needForGapNCSG-InfoNR (as in TP).

Discussions in CB

-	Nokia think whether to include the fields is up to NW implementation so no change needed. Ericsson think it is not essential to capture. QC also think we may not need to change. Vivo agree.

P3 is not pursued.

Post meeting email disc

[Post127][204][MUSIM] CR for TS 38.331 (vivo)

Scope: Update and review the CR for TS 38.331

Intended outcome: Agreed CR

Deadline: Short

=> Agreed in R2-2407864 (38.331 CR#4970r1)

## 7.18	Void

## 7.19	Enhanced support of reduced capability NR devices

(NR_redcap_enh-Core; leading WG: RAN1; REL-18; WID: RP-232671)

WI is declared 100% complete

Time budget: 0 TU

Tdoc Limitation: 1 Tdocs

### 7.19.1	Organizational

Incoming LSs, CR rapporteur’s miscellaneous non-controversial corrections, etc.

LS

R2-2406205	Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502 (C1-243517; contact: Huawei)	CT1	LS in	Rel-18	NR_redcap_enh-Core	To:SA2	Cc:CT4, RAN2, RAN3

Noted

Misc CRs

R2-2407314	Miscellaneous corrections on TS 38.304 for eRedCap	Huawei, HiSilicon	CR	Rel-18	38.304	18.2.0	0411	-	F	NR_redcap_enh-Core, NR_redcap-Core

-	Qualcomm think we should do a general cleanup of the part of the spec impacting barring. Ericsson agrees with Qualcomm.

Agreed

Withdrawn

R2-2407554	Miscellaneous corrections on TS 38.321 for eRedCap	vivo (Rapporteur)	CR	Rel-18	38.321	18.2.0	1920	-	F	NR_redcap_enh-Core	Withdrawn

### 7.19.2	Other

Critical corrections, if any.

R2-2406442	Discussion on 2-step RACH for eRedCap	vivo, Guangdong Genius	discussion	Rel-18	NR_redcap_enh-Core

-	LG think that current spec is correct. NEC agrees with LG. QC think that current spec (MAC and RRC) are clear and correct. Huawei supports vivos proposal and think that eRedCap UEs cannot use RedCap 2-step RA resources. Vivo want to confirm that current spec is clear and correct but would like to align companies understanding.

RAN2 confirms that eRedCap UE can use 2-step RedCap resources when there is no set associated with eRedCap. No change to the spec is expected due to this confirmation.

R2-2407315	Corrections on some features application to (e)RedCap UE	Huawei, HiSilicon	draftCR	Rel-18	38.306	18.2.0	NR_redcap_enh-Core, NR_redcap-Core

Agreed in R2-2407656

R2-2407656	Corrections on some features application to (e)RedCap UE	Huawei, HiSilicon	CR	Rel-18	38.306	18.2.0	1151	-	F	NR_redcap_enh-Core, NR_redcap-Core

=> Agreed

Withdrawn

R2-2407534	MAC correction for eRedCap	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.2.0	1919	-	F	NR_redcap_enh-Core

## 7.20	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

Time budget: 0TU

Tdoc Limitation: 1 tdoc

### 7.20.1	Organizational

Incoming LS, Rapporteur input, etc.

Stage 2 corrections.

R2-2406214	Reply to RAN2 LS on type 3 PH value for the serving cell configured with mTRP (R1-2405619; contact: MediaTek)	RAN1	LS in	Rel-18	NR_FeMIMO-Core, NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2406489	Remaining issues on PHR for mTRP STx2P	Samsung	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

=> Moved to 7.20.2

### 7.20.2	Corrections

PHR aspects

R2-2406915	Remaining issue on STx2P PHR	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Discussion

P1

- CATT understand that R1 already discuss this in this week and agree that type 1 PHR is associated with the 1st TCI state. So CATT do not see a need to LS R1.

- HW think the proposed principle is fine and think we need to make it clear in the spec.

- QC think R1 already agreed so perhaps no need to ask R1.

- LG E think R1 agreement is for a different case when the MAC entity is not configured with twoPHR.  Samsung think the same principle applies.

R2-2407202	Correction on PHR for STx2P in NR-DC	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1905	-	F	NR_MIMO_evo_DL_UL-Core

Noted, exact change on this issue will be discussed in Rapp’s MAC CR.

R2-2407434	Further Considerations on Harmonizing the PHR for Different Features	ZTE Corporation	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1:	Two values of PCMAX,f,c,k are obtained from physical layer for the serving cell configured with STx2P from the other MAC entity if the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE is generated by this MAC entity and the other MAC entity is configured with twoPHRmode.

Discussion

P1

- LG E agree with the intention. HW not sure about the intention.

Proposal 2:	RAN2 confirm that the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE is generated If this MAC entity is configured with twoPHRmode and at least one Serving Cell of both MAC entities is configured with STx2P.

Proposal 3:	RAN2 confirm that the Enhanced Multiple Entry PHR for multiple TRP MAC CE is generated if this MAC entity is configured with twoPHRmode and at least one Serving cell of both MAC entities is configured with mTRP PUSCH repetition and none of serving cells of both MAC entities is configured with STx2P.

Chair: the proposals seem not well understood; it seems more offline discussions will be helpful.

Discussions in CB on P1-3 and necessary changes.

-	ZTE explains that there have been offline and P2-3 can be skipped. And ZTE suggest to postpone P1.

-	LG E suggest to address P1 in the CR discussion after the meeting. ZTE think there may be many changes. Samsung suggest to postpone.

P2 and P3 are not pursued.

P1 is postponed.

R2-2406489	Remaining issues on PHR for mTRP STx2P	Samsung	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Discussion

P1

- Xiaomi, ZTE, LG E, Ericsson agree.

- Oppo also fine with this proposal but think there is other case, and think we can perhaps check. ZTE think there is no need to send a LS.

In PHR procedure, type 3 PH is not reported for serving cell configured with multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN. Remove type 3 PH at two places in the above MAC procedural text.

P2-1

- OPPO think this is R1 discussion.

- Vivo think we could agree in R2 and think P2-1 is agreeable. Ericsson agree and think we can change the MAC accordingly.

- HW agree with the proposal. ZTE also agree, and think if R1 has different agreement then we can revisit.

- LG E can accept but want to send LS to R1 to confirm.

The following is agreed and the exact change to MAC spec can be further checked. Can revisit if R1 agreed otherwise.

For mTRP STx2P, if twoPHRmode is not configured,

- if there is at least a real PUSCH transmission,

  - if the first TCI state is applied for a real PUSCH transmission, UE obtains the type 1 PH and the Pcmax associated with the first TCI state;

  - else UE obtains the type 1 PH and the Pcmax associated with the second TCI state.

- else if there is no real PUSCH transmission, UE obtains the type 1 PH and the Pcmax associated with the first TCI state.

Other changes

R2-2406488	Correction to MIMO Evolution	Samsung	draftCR	Rel-18	38.331	18.2.0	NR_MIMO_evo_DL_UL-Core

1.	For mappingPattern-r17 in IE ConfiguredGrantConfig and IE PUSCH-Config, change the presence condition SRSsets to optional presence when two SRS sets are configured.

2.	In the field description of n-TimingAdvanceOffset2 clarify that N_TA-Offset2 is applied only for inter-cell multi-DCI multi-TRP operation with two TA, i.e, for PDCCH order CFRA towards the additional PCI and for all UL transmission associated to tag2.

Discussion

P1

- CATT think change 1 has Rel-17 impact so perhaps we should focus on R18 change. HW, QC agree with this view.

It is clarified in the spec that for Rel-18 mappingPattern-r17 in IE ConfiguredGrantConfig and IE PUSCH-Config is absent, no changes to Rel-17 is needed. Detailed changes to the RRC spec can be checked further.

R2-2406519	Discussion on remaining issue for 8Tx in MAC specification	ASUSTeK	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Discussion

- ZTE think we do not need to add the note, and think UE will generate two TBs according to the UL grant.

- CATT think the issue is valid, and note is useful.

- LG E not sure whether the wording ‘for the HARQ process’ is correct.

Discussions in CB

-	Asustek reports that in offline there are common understanding that some note may be needed, exact changes FFS.

-	In addition, in the offline discussion, some companies pointed out that HARQ modelling (i.e., in LTE there are two HARQ process ID for two UL grants for the case of UL spatial multiplexing, but in NR there is only one) discussions are needed. Asustek think more time is needed, also may need to check with R1. ZTE think the potential issue was found in this meeting can think in R2 we can analyse the impact. Samsung think we need more time to check if any issue in R2 spec, can discuss based on company contribution.

-	Samsung not ready to send LS to R1.

Postponed

R2-2406574	Correction on simultaneousU-TCI-UpdateListx	CATT	draftCR	Rel-18	38.331	18.2.0	NR_MIMO_evo_DL_UL-Core

Discussion

- Ericsson think the intention is OK but exact changes need further checking.

Postponed

R2-2406778	Correction on unified TCI state for SRS	vivo	draftCR	Rel-18	38.331	18.2.0	F	NR_MIMO_evo_DL_UL-Core

Discussion

- Ericsson think the changes are needed.

Endorsed, will merge into the RRC Rapp’s CR

R2-2406807	Random Access problem for SpCell with two TAGs	Langbo	discussion	Rel-18	38.321	NR_MIMO_evo_DL_UL-Core

Discussion

- LG E has different understanding and think no change is needed. Samsung, QC share this view and think this is a minor optimization.

Not pursued

R2-2406808	Clarification on the codebook type request in the UE capability enquiry	Nokia Corporation	discussion	NR_MIMO_evo_DL_UL-Core

Discussion in CB

-	CATT think the problem in the paper is valid, so agree with the proposals. Samsung agree.

-	HW think no changes are needed, since the new codebooks are introduced in later release.

-	QC think the codebook type request is introduced in Rel-16. HW understand the codebook type request impact Rel-16 codebook. Samsung understand that it only applies to Rel-15.

-	DCM wonders whether this belongs to other agenda item for earlier releases.

Postponed

[AT127][201][MIMOevo] Proposals for RRC changes (Ericsson)

Intended outcome: Review the proposed RRC changes from the company contributions, taking into account the agreements so far. Summary/proposals in R2-2407701 for discussion in the CB session.

Deadline: before Thursday CB session

R2-2407701 Summary for [AT127][201][MIMOevo] Proposals for RRC changes Ericsson

P1 is agreed.

P2 is agreed.

Post meeting email disc

[Post127][202][MIMOevo] CR for TS 38.321 (Samsung)

Scope: Update and review the CR for TS 38.321.

Intended outcome: Agreed CR

Deadline:  Short

=> Agreed in R2-2407843 (38.321 CR#1927)

[Post127][203][MIMOevo] CR for TS 38.331 (Ericsson)

Scope: Update and review the CR for TS 38.331.

Intended outcome: Agreed CR

Deadline:  Short

=> Agreed in R2-2407866 (38.331 CR#4969r1)

## 7.21	Further NR coverage enhancements

(NR_cov_enh2-Core; leading WG: RAN1; REL-18; WID: RP-221858)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.21.1	Organizational

Incoming LSs, Rapporteur input etc.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

R2-2407419	Miscellaneous corrections for further NR coverage enhancements in RRC	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4951	-	F	NR_cov_enh2-Core

Second changes (IE name to field name) is agreeable.

Incorporate first change from R2-2407275

Incorporate R2-2407278

Agreed unseen in R2-2407640

R2-2407517	Miscellaneous corrections on further NR Coverage enhancements in MAC	ZTE Corporation	CR	Rel-18	38.321	18.2.0	1918	-	F	NR_cov_enh2-Core

Agreeable and to be updated to capture changes from R2-2407420. Agreed unseen in R2-2407641

### 7.21.2	Other Essential corrections

R2-2406811	Addition of missing prerequisite in the description of capability dynamicWaveformSwitchIntraCA-r18	Lenovo	draftCR	Rel-18	38.306	18.2.0	NR_cov_enh2-Core

Endorsed to be merged with mega capability CR

R2-2406922	Conditional Presence for absence of field preambleTransMax-Msg1-Repetition	Ericsson	CR	Rel-18	38.331	18.2.0	4897	-	F	NR_cov_enh2-Core

Not pursued

R2-2407275	RRC Correction on Msg1 based SI request with Msg1 Repetition	Philips International B.V.	CR	Rel-18	38.331	18.2.0	4941	-	F	NR_cov_enh2-Core

First change is agreeable, to be merged to an update of R2-2407419

R2-2407278	Correction on posSIB(s) acquisition for Msg1 based SI request with Msg1 Repetition [TEI17 SI-SCHEDULING]	Philips International B.V., Ericsson	CR	Rel-18	38.331	18.2.0	4942	-	F	TEI17, NR_cov_enh2-Core

The change is agreeable to be merged to an update of R2-2407419

R2-2407420	Clarifications on initilization of RACH parameters during repetition fallback	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1911	-	F	NR_cov_enh2-Core

First change and option 1 in the CR is agreeable to be merged to update of R2-2407517.

## 7.22	Void

## 7.23	Void

## 7.24	TEI18

Specific items may be allocated to a breakout session for treatment.   Essential corrections only.  No new proposals will be treated.

Time budget: 1 TU

### 7.24.1	TEI proposals by Other Groups

Items initiated by other groups that is/has been communicated by LS, where the other group indicate this is TEI18. (Specific other-group-WIs should use the R18 Other Agenda Item below).

NOTE: Include TEI identifiers in agreed CRs.

SDT related topics

Tdocs related to RAN3 LS:

R2-2406458	SDT signalling optimization for partial context transfer	ZTE Corporation, Sanechips	discussion

Proposal 1: Capture in RAN2 specs that the network should not send RRCSetup if DL data is sent to the UE

Proposal 2: Reply to RAN3 as follows:

-	A MAC Reset is needed if RRCSetup procedure is initiated in the middle of the SDT session and this needs new UE capability and is hence not compatible with Rel-17 operation.

-	Network based solution would result in data loss since UE will discard any pending UL data and setup a new RRC connection.

-	Network based solution would result in additional signalling to reestablish AS context and AS security context both of which are released upon receiving RRCSetup

-	Hence RAN2 agreed that the network should not send RRCSetup message after sending any DL data during SDT

=>	Noted

R2-2406660	Impacts on RAN2 for supporting of SDT signalling optimization for partial context transfer	Sharp	discussion

=>	Noted

R2-2406929	Impact of SDT signalling optimization for partial context transfer on RAN2	Huawei, HiSilicon	discussion	Rel-18	TEI18

Proposal 1: RAN2 to discuss if it is acceptable for the UE not to perform MAC reset upon RRCSetup reception during an ongoing SDT procedure when some data has already been exchanged.

Proposal 2: In the reply LS to RAN3, RAN2 should mention the potential issue on the residual data in MAC and clarify whether or not it can be handled by the network implementation (depending on the outcome of RAN2 discussion).

=>	Noted

R2-2407229	Discussion on LS to RAN3	Ericsson, Nokia	discussion	Rel-18	TEI18, NR_MT_SDT-Core

=>	Moved from 7.24.2.2

Discussion

-	Nokia doesn’t think that a reset is needed.   Xiaomi thinks that if we do a MAC reset this would cause backward compatibility problems.

-	LG indicates that today the MAC is not reset and if there is some data in the buffer this may cause some desynch issues.

-	Intel indicates that there will be some impact if we have to introduce a MAC reset and support ZTE.

-	Qualcomm would prefer to not introduce a new mechanism a MAC reset and not sure how the network would handle this issue.

-	CATT asks if a MAC reset is acceptable to RAN2.

-	Apple thinks that this would be either a UE impact or NW impact.

-	Vivo thinks that the network can handle this.

-	Intel thinks that we should also include service continuity.   Nokia doesn’t think this is a big issue.

Agreements on the LS in addition to what is already agreed in the LS from last meeting

1	Acknowledge that there is an issue for the cases that there is some data in the buffer which causes dy-synch issues.  RAN2 discussed how to solve the issue and identified two possible ways to handle this: potentially a MAC reset may be needed or NW implementation can handle it.   If a MAC reset is needed, this would be a non-backward compatible change (i.e. not desirable), and some companies are not sure whether NW implementation can handle this issue.

R2-2407501	Reply LS on SDT signalling optimization for partial context transfer	ZTE Corporation, Sanechips	LS out	Rel-18	TEI18	To:RAN3

=>	The LS is revised in R2-2407746

[AT127][008][SDT] LS to RAN3 (ZTE)

Intended outcome: agree to LS

Deadline:  08-23-24

R2-2407746	Reply LS on SDT signalling optimization for partial context transfer	ZTE Corporation, Sanechips	LS out	Rel-18	TEI18	To:RAN3

=>	Update actions to RAN3 instead of RAN2 and update date of next meeting

=>	The LS is approved in R2-2407816 with changes above

R2-2407816	Reply LS on SDT signalling optimization for partial context transfer	RAN2	LS out	Rel-18	TEI18	To:RAN3

=> Approved

Paging Monitoring for CG-SDT (moved from 7.24.2)

R2-2406465	Paging monitoring for extended CG period [CG-SDT-Enh]	ZTE Corporation, Sanechips, Mediatek, Qualcomm, Ericsson	CR	Rel-18	38.331	18.2.0	4788	1	F	TEI18	R2-2404546

=>	The CR is revised and reviewed over email

R2-2406930	Correction for Paging monitoring during SDT [CG-SDT-Enh]	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4901	-	F	TEI18

=>	The CR is not pursued

Discussion

-	LG thinks that we should have a common behaviour regardless of the extended CG periodicity configured.   The CR from Huawei just to make an exception for RedCap is better.  Qualcomm explains that the behavior doesn’t change from legacy, only when extended periodicity is configured.

-	MEdiatek doesn’t understand why we can’t have new behaviour for the new feature.

-	Vivo thinks that ZTEs CR addresses all issues.

-	Huawei doesn’t want to link to a capability

-	Intel suggests to have it for RedCap but link it to extended CG

-	Huawei is only ok to change it only for RedCap

-	Sony prefers to do it for all UEs, as ZTE proposes

=>	Rel-17 behaviour will not change for RA-SDT and CG-SDT. Baseline wording from ZTE CR (ZTE)

R2-2407803	Paging monitoring for extended CG period [CG-SDT-Enh]	ZTE Corporation, Sanechips, Mediatek, Qualcomm, Ericsson	CR	Rel-18	38.331	18.2.0	4788	2	F	TEI18

=>	The CR is agreed

Other

R2-2406206	Reply LS on 5GS missing CBC support for shared networks (C1-243686; contact: Ericsson)	CT1	LS in	Rel-18	TEI18	To:SA2	Cc:RAN2, RAN3, SA1

=>	Noted

R2-2406243	Reply LS Mitigation of Downgrade attacks (S3-242367; contact: Apple)	SA3	LS in	Rel-18	TEI18	To:CT1	Cc:RAN2

=>	Noted

Intra-band EN-DC channel spacing

R2-2406231	LS on intra-band EN-DC channel spacing (R4-2410693; contact: Huawei)	RAN4	LS in	Rel-18	TEI18	To:RAN2

=>	Noted

R2-2406833	Discussion on LS on intra-band EN-DC channel spacing	CATT	discussion	Rel-18	TEI18

Proposal 1: Instead of introducing new UE capabilities, update the field descriptions of the following UE capabilities from R18, including intraBandENDC-Support, intrabandENDC-Support-UL, intraBandENDC-Support-v1790 and intraBandENDC-Support-UL-v1790, to add restriction that a UE only indicates ‘both’ if the corresponding UE capability is supported.

Proposal 2: Send a reply LS to R4 and explain RAN2 approach.

=>	Noted

R2-2407466	Discussion on intra-band EN-DC channel spacing	Huawei, HiSilicon	discussion	Rel-18	TEI18

Proposal 1: Introduce new capability to indicate support of intra-band non-contiguous (NG)EN-DC with nominal channel spacing.

Proposal 2: The new capability is in per-BC level, which is applicable for all the DL and/or UL intra-band EN-DC component(s) with non-contiguous capability in the BC.

Proposal 3: The UE indicating support of the new capability shall indicate support of “non-contiguous” in at least one among intrabandENDC-Support/intrabandENDC-SupportUL/intrabandENDC-Support-DL-v1790/ intrabandENDC-Support-UL-v1790.

=>	Noted

Discussion

-	ZTE thinks that it is not clear whether such UEs support contiguous or not.  Qualcomm thinks that this has been very clear in RAN4 that for this nominal gap we would need a new capability.

=>	Wait for further RAN4 discussions before deciding what to do

Not treated

R2-2406261	Discussion on LS R4-2410693	OPPO	discussion	Rel-18	TEI18

R2-2406834	Capability update for intraBandENDC-Support	CATT	draftCR	Rel-18	38.306	18.2.0	F	TEI18

R2-2406835	DRAFT Reply LS on intra-band EN-DC channel spacing	CATT	LS out	Rel-18	TEI18	To:RAN4

R2-2407329	Consideration on the  intra-band EN-DC Channel Spacing	ZTE Corporation, Sanechips	discussion	Rel-18	TEI18

R2-2407330	Draft Reply LS on the intra-band EN-DC Channel Spacing	ZTE Corporation, Sanechips	LS out	Rel-18	TEI18	To:RAN4

R2-2407467	Introduction of new capability for intra-band EN-DC channel spacing	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4954	-	B	TEI18

R2-2407468	Introduction of new capability for intra-band EN-DC channel spacing	Huawei, HiSilicon	CR	Rel-18	38.306	18.2.0	1147	-	B	TEI18

R2-2407469	Reply LS on intra-band EN-DC channel spacing	Huawei, HiSilicon	LS out	Rel-18	TEI18	To:RAN4

### 7.24.2	TEI proposals by RAN2

Items initiated in RAN2 for NR and LTE.

No contributions should be submitted under 7.24.2.  They should be submitted under 7.24.x

Tdoc limitation: 1 tdoc, limitation applicable to new proposals.  No new Cat. B proposals expected for this meeting

#### 7.24.2.1	Rx XR

Contributions on signaling support for ‘2Rx non-REDCAP XR devices’ as per RP-234015.  Co-source contributions are highly encouraged.

NOTE: Include TEI identifiers in agreed CRs.

R2-2406933	Cell barring depending on the UE band for 2Rx XR UE	Huawei, HiSilicon, Xiaomi Communications, Ericsson, LG Electronics		discussion	Rel-18	TEI18

Proposal 1: RAN2 to confirm that it is possible for the UE to be 2Rx XR UE only in some bands while being a regular UE in other bands.

-	Apple and Qualcomm thinks that the plenary agreed that it is a band specific capability.  ZTE confirms.  ZTE doesn’t think we should modify the band selection because of this.

-	Vodafone this also depends on whether the operator will support this scenario.

-	CMCC thinks that the current implementation is better.

=>	No change to current specification is needed

=>	Noted

#### 7.24.2.2	Other RAN2 TEI-18

Contributions should focus only critical issues/corrections for already agreed TEI-18 topics.   Co-sourcing of such proposals is encouraged.   Contributions on items that were explicitly downprioritized from Rel-18 WIs should not be brought as TEI18.  No new Cat. B proposals expected for this meeting

NOTE: Include TEI identifiers in agreed CRs.

Emergency call

R2-2406441	Discussion on issues for emergency call with barringExemptEmergencyCall	vivo, China Telecom, China Unicom, Guangdong Genius	discussion	Rel-18	TEI18

-	Vodafone thinks that this scenario is not used as the PLMNs are not talking.

-	Samsung and Qualcomm thinks this is a valid issue but there are a number of other cases and we can have a generic solution.

=>	attempt to capture this in a generic way

=>	Noted

R2-2406511	Discussion on cell barring and barring exemption	Qualcomm Incorporated	discussion	Rel-18	TEI18

Proposal 1:	Clarify that an acceptable cell is not a barred cell without any exception.

.  Proposal 2:	Clarify that after the barring exemption, the UE is still required to check the cell reservations.

Proposal 3:	Clarify that after the barring exemption, the UE is no longer required to select another cell on the same frequency or other frequency.

=>	Noted

Discussion

-	Vodafone thinks that we should go through all possible cases.

-	LG supports the proposals from Qualcomm.

-	DT thinks that we should be extremely carefully of touching legacy behavior/text.   ZTE and Apple also doesn’t think we need to change anything with regards to existing behaviour

Agreements

-	Clarify that an acceptable cell is not a barred cell without any exception.

-	Start with ZTE CR and see if anything additional is needed to be clarified and remove anything not needed

[AT127][012][Emergency calls] Acceptable cells (ZTE)

Intended outcome: Review CR for acceptable cells behaviour.  Attempt to find a generic way, if possible.

Deadline:  08-23-24

R2-2407804	Correction to barring exemption for (e)Redcap and XR 2RX UEs [EM_Call_Exemption]	ZTE Corporation, Sanechips	CR	Rel-18	38.304	18.2.0	0409	1	F	TEI18

-	Nokia is concerned that “The RedCap UE that supports only 1Rx branch may treat this cell as an acceptable cell.” this is a ‘may’.   Qualcomm thinks that this is an optional feature and should be without capability signaling.

-	Huawei and Nokia think the problem is that it is not testable so we can link it to a UE capability.

-	LG doesn’t understand why the “may” doesn’t work as we have multiple cases in idle mode spec.

=>	Introduce an optional capability without signaling and link the description to the capability with a “shall”

=>	the CR is revised in R2-2407820and will be reviewed by email in [012].

R2-2407820	Correction to barring exemption for (e)Redcap and XR 2RX UEs [EM_Call_Exemption]	ZTE Corporation, Sanechips	CR	Rel-18	38.304	18.2.0	0409	2	F	TEI18

=>	The CR is agreed

R2-2407505	Introduction of a UE capability for the barring exemption for emergency call [EM_Call_Exemption]	Huawei, HiSilicon	CR	Rel-18	38.306	18.2.0	1148	-	B	TEI18

-	Nokia and ZTE thinks that this is not needed and all UEs supporting redcap, XR, etc. should support it.

-	Apple doesn’t think it is need

-	Qualcomm thinks that this only applies to some UEs.

-	Huawei thinks that we should not make it mandatory for eRedcap as it is supposed to be low complexity and it is quite late in the release.   BT asks if it supports voice then why can’t it do emergency calls.

-	Qualcomm agreed to the feature with the assumption that it was optional and we wouldn’t agree to this feature.

-	Vodafone thinks we can work offline for the description and we highlight that it is desirable for all UEs to support this.

=> 	It is recommended that UEs supporting voice/emergency calls support this feature

=>	The CR is agreed

R2-2406461	Open issues for emergency calls for 2RX UEs	ZTE Corporation, Sanechips	discussion

=>	Nothing specific for NES will need to be done for Rel-18

=>	Noted

Not treated

R2-2406462	Correction to barring exemption for (e)Redcap and XR 2RX UEs [EM_Call_Exemption]	ZTE Corporation, Sanechips	CR	Rel-18	38.304	18.2.0	0409	-	F	TEI18

R2-2406512	Clarifications on cell barring [EM_Call_Exemption]	Qualcomm Incorporated	CR	Rel-18	38.331	18.2.0	4880	-	F	TEI18

R2-2406513	Clarifications on cell barring [EM_Call_Exemption]	Qualcomm Incorporated	CR	Rel-18	38.304	18.2.0	0410	-	F	TEI18

R2-2406931	Introduction of barring exemption for emergency call [EM_Call_Exemption]	Huawei, HiSilicon	CR	Rel-18	38.300	18.2.0	0884	-	B	TEI18

-	Qualcomm thinks we should contain all changes in one section.  Nokia thinks we can simplify the description.

=>	Need to update the text to simplify and improve

[AT127][013][Emergency Calls] 38.300 CR (Huawei)

Intended outcome: Agree to CR by email

Deadline:  08-23-24

R2-2407829	Introduction of barring exemption for emergency call [EM_Call_Exemption]	Huawei, HiSilicon	CR	Rel-18	38.300	18.2.0	0884	1	B	TEI18

=> Agreed

R2-2406932	Corrections on the barring exemption for emergency call [EM_Call_Exemption]	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4902	-	F	TEI18	Withdrawn

[meas_report_enh]

R2-2406859	Discussion on TEI18 enhancement to measurement report 	Samsung Electronics	discussion	Rel-18	TEI18

Proposal 1: When the event leaving condition is fulfilled, the UE stores the concerned cell(s) in cellsMetLeavingCond only if enteringLeavingReport is configured in the corresponding reportConfig. The TP1 in annex can be considered for the correction.

-	Ericsson and LG support

Proposal 2: The UE stores each of the concerned cell(s) in cellsMetLeavingCond only if the cell has been reported to the gNB before. The TP2 in annex can be considered for the correction.

-	LG thinks this is a rare occurring event so we can leave it up to implementation.   Samsung things this is not a corner case

-	Huawei thinks the intention is correct but the proposal is not.

-	Qualcomm and Ericsson would like to avoid further optimization

=>	Not supported

Proposal 3: The UE can set the first cell in reportedBestNeighbourCell only if measResultNeighCells is not empty and the UE can set the second cell in reportedBestNeighbourCell only if there is more than one entry in measResultNeighCells. The TP3 in annex can be considered for the correction.

-	Ericsson doesn’t understand why we need to specify anything.  If the UE doesn’t have anything it shouldn’t report.  We agreed to not over specify.

=>	Noted

Agreements

=>	When the event leaving condition is fulfilled, the UE stores the concerned cell(s) in cellsMetLeavingCond only if enteringLeavingReport is configured in the corresponding reportConfig. The TP1 in annex can be considered for the correction.

=>	Add “if available” to both cases where n1 and n2.

=> Revised in R2-2407749

R2-2407749	Discussion on TEI18 enhancement to measurement report 	Samsung Electronics	discussion	Rel-18	TEI18

[AT127][015][TEI18] Measurement Reporting (Samsung)

Intended outcome: Agree to CR capturing agreements

Deadline:  08-23-24

R2-2407783	Correction on enhancements to measurement report [meas_report_enh]	Samsung	CR	Rel-18	38.331	18.2.0	4966	-	F	TEI18

=>	The CR is agreed

[CIO_in_ReportConfig]

R2-2407493	Discussion on remaining issues on cell individual offset	Samsung, Ericsson	discussion	Rel-18	TEI18

Proposal 1. RAN2 to discuss whether to introduce CIO in reportConfig for conditional handover.

-	CATT and Huawei don’t think we should introduce for CHO.

=>	Not supported

Proposal 2. RAN2 to discuss whether to introduce CIO in reportConfig for handover from LTE (i.e., LTE to NR handover, or LTE to LTE handover).

-	Qualcomm would like to avoid it as LTE Deployments are stable.

=>	Noted

R2-2407182	Correction on frequency introduced for CIO [CIO_in_ReportConfig]	Ericsson, Samsung, NTT Docomo, INC., KDDI, Qualcomm Incorporated	CR	Rel-18	38.331	18.2.0	4933	-	F	TEI18

-	Mediatek suggestion to capture: this new feature doesn’t apply to CSI-RS measurement.  Ericsson thinks we can then capture in some field description.   Huawei thinks that this is already clear from current signaling.

=>	The CR is postponed

Enterleavingreport

R2-2407510	Correction to enteringLeavingReport condition	LG Electronics France	discussion	Rel-18	TEI18

=>	Noted

R2-2407513	Correction to EnteringLeavingReport condition	LG Electronics	CR	Rel-18	38.331	18.2.0	4959	-	F	TEI18

-	Qualcomm thinks that these are optimizations and we need to keep the feature simple

R2-2407822	Correction to EnteringLeavingReport condition	LG Electronics, Ericsson, Qualcomm Inc., ZTE, Apple, MediaTek Inc., Samsung	CR	Rel-18	38.331	4959	2	F	TEI18

=>	Update with TEI identifier and add space to second change.

=>	The CR is agreed in R2-2407833 with changes above

R2-2407833	Correction to EnteringLeavingReport condition	LG Electronics, Ericsson, Qualcomm Inc., ZTE, Apple, MediaTek Inc., Samsung	CR	Rel-18	38.331	4959	3	F	TEI18

=> Agreed, but then coversheet revised by MCC: the CR title is missing TEI identifier "[meas_report_enh]"

=> Revised in R2-2407882

R2-2407882	Correction to EnteringLeavingReport condition [meas_report_enh]	LG Electronics, Ericsson, Qualcomm Inc., ZTE, Apple, MediaTek Inc., Samsung	CR	Rel-18	38.331	4959	4	F	TEI18

=> Agreed

NES/NCR

Not treated

R2-2407521	NES for NCR	LG Uplus	discussion	Rel-18

Treated in MBS breakout session

NOTE: Include TEI identifiers in agreed CRs.

R2-2406281 Correction on the capabilities on PTM retransmission	Huawei, HiSilicon, Intel Corporation, Nokia, vivo	CR	Rel-18	38.306	18.2.0	1134	-	F	NR_MBS_enh-Core, TEI18

R2-2406282 Correction on the capabilities on PTM retransmission	Huawei, HiSilicon, Intel Corporation, Nokia, vivo	CR	Rel-18	38.331	18.2.0	4867	-	F	NR_MBS_enh-Core, TEI18

R2-2406345 Correction on PTM Retransmission Capability	Samsung	CR	Rel-18 38.306 18.2.0	1135	-	F	NR_NTN_solutions-Core, NR_MBS-Core

R2-2407527 Search space configuration for RedCap UE’s MBS broadcast reception	ZTE Corporation, Sanechips	discussion	Rel-18	TEI18

## 7.25	R18 Other

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-18 specific WIs/SIs that has no RAN WI.

Clarification CRs should be discussed with spec rapporteurs of the topic prior to submission.

Time budget: 1 TU

Tdoc Limitation: -

### 7.25.1	RAN4 led items

R2-2406226	Reply LS to RAN2 on RRM enhancements for NR FR2 HST (R4-2410285; contact: Samsung)	RAN4	LS in	Rel-18	NR_HST_FR2_enh	To:RAN2

=>	Noted

R2-2407247	Clarification on highSpeedMeasFlagFR2 for HST FR2 RRM	Samsung	CR	Rel-18	38.331	18.2.0	4936	-	F	NR_HST_FR2_enh

-	Huawei doesn’t think the CR is aligned with the LS.  Samsung can clarify that it is intra-band

=>	The CR is revised in R2-2407792

[AT127][016][HST] 331 CR (Samsung)

Intended outcome: Review revised CR and agree by email

Deadline:  08-23-24

R2-2407792	Clarification on highSpeedMeasFlagFR2 for HST FR2 RRM	Samsung	CR	Rel-18	38.331	18.2.0	4936	1	F	NR_HST_FR2_enh

=>	The CR is agreed

Not treated

R2-2407335	Correction on the configuration parameter highSpeedMeasFlagFR2 for FR2 HST	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4945	-	F	NR_HST_FR2_enh

R2-2407379	Discussion on RRM enhancements for NR FR2 HST	ZTE Corporation, Sanechips	discussion	Rel-18	NR_HST_FR2_enh

R2-2406230	Reply LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1 (R4-2410600; contact: Qualcomm)	RAN4	LS in	Rel-18	NR_FR1_lessthan_5MHz_BW-Core	To:RAN1, RAN2

=>	Noted

R2-2406339	Correction on inter-frequency configuration for less than 5MHz	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4869	-	F	NR_FR1_lessthan_5MHz_BW

-	Qualcomm explains that it was not included on purpose as some companies wanted to sue the whole list for some cases.

=>	check if we can indicate 12 or 20 PRB in SIB1

=>	The CR is postponed

R2-2407322	Correction on asymmetricBandwidthCombinationSet for the 3M	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.2.0	1141	-	F	NR_FR1_lessthan_5MHz_BW-Core

=>	simplify text: “if defined for the band in the TS 38.101-1 [2]. If the field is absent the UE supports asymmetric channel bandwidth combination set 0, if defined for the band in the TS 38.101-1 [2]”

=>	The CR is endorsed in R2-2407761 and will be merged in mega CR

R2-2406604	Clarification of offsetThresholdTA-r18 for NR ATG	Huawei, HiSilicon, CMCC	CR	Rel-18	38.331	18.2.0	4882	-	F	NR_ATG-Core

=>	Noted

R2-2407380	Discussion on ATG	ZTE Corporation, Sanechips	discussion	Rel-18	NR_ATG-Core

=>	Noted

R2-2407530	Clarifications for ATG timing advance reporting procedures	Samsung	discussion	Rel-18	NR_ATG-Core

=>	Noted

[AT127][024][ATG] Clarification CR (Samsung)

Intended outcome: agree to CR

Deadline:  08-23-24

R2-2407806	Report of [AT127][024][ATG] Clarification on SCS for Timing Advance reporting	Samsung	discussion	Rel-18	NR_ATG

=>	The issue is postponed to next meeting

=>	Noted

R2-2406836	Dummify the capability bit multiRx-FR2-Preference-r18	CATT, Apple, Intel Corporation	draftCR	Rel-18	38.331	18.2.0	F	NR_FR2_multiRX_DL-Core

=>	Update cover page with no interoperability

=>	The CR is endorsed in R2-2407762 with changes above and merged in mega CR

R2-2406837	Dummify the capability bit multiRx-FR2-Preference-r19	CATT, Apple, Intel Corporation	draftCR	Rel-18	38.306	18.2.0	F	NR_FR2_multiRX_DL-Core

=>	Delete  (i.e. not supporting simultaneous reception with different QCL-typeD)

=>	Update cover page with no interoperability

=>	The CR is endorsed in R2-2407763 with changes above and merged in mega CR

R2-2406944	Introduction of unknown SCell activation enhancement	Huawei, HiSilicon, Apple	CR	Rel-18	38.300	18.2.0	0885	-	B	NR_RRM_enh3

=>	Revise to simplify text and references to RAN4

[AT127][017][SCell act] Stage 2 CR  (Huawei)

Intended outcome: Agree to CR by email

Deadline:  08-23-24

R2-2407815	Introduction of unknown SCell activation enhancement	Huawei, HiSilicon, Apple	CR	Rel-18	38.300	18.2.0	0885	1	B	NR_RRM_enh3

=>	The CR is agreed

R2-2407178	MAC correction on fast unknown SCell activation	Ericsson	CR	Rel-18	38.321	18.2.0	1903	-	F	NR_RRM_enh3

=>	The CR is not pursued

R2-2407179	RRC correction on fast unknown SCell activation	Ericsson	CR	Rel-18	38.331	18.2.0	4931	-	F	NR_RRM_enh3

-	Samsung, Apple, Huawei, think that it is clear already that one SCell can trigger it

=>	RAN2 understanding is that if at least one of the activated indicated SCell(s) fulfills the measurement requirement then measurement report can be triggered

=>	The CR is not pursued

R2-2407399	Miscellaneous corrections for fast unknown SCell activation	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4949	-	F	NR_RRM_enh3

=>	The CR is not pursued

R2-2407400	Miscellaneous corrections for fast unknown SCell activation	Huawei, HiSilicon	CR	Rel-18	38.321	18.2.0	1908	-	F	NR_RRM_enh3

=>	The CR is not pursued

### 7.25.2	RAN1 led items

E.g. UL Tx Switching, MC enhancements, DSS

R2-2406317	Correction on PDCCH monitoring for Multi-cell scheduling	CATT	draftCR	Rel-18	38.321	18.2.0	F	NR_MC_enh-Core	Withdrawn

R2-2406815	Removal of term ‘legacy’ from the description of dci-FormatsMC	Lenovo	CR	Rel-18	38.331	18.2.0	4890	-	F	NR_MC_enh-Core

=>	The CR is merged with R2-2406945

R2-2406945	Miscellaneous corrections for Multi-Carrier enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.2.0	4903	-	F	NR_MC_enh-Core

=>	The CR is merged with 6815 and revised in R2-2407764

[AT127][018][MC] 331 CR (Huawei)

Intended outcome: Agree to revised CR by email

Deadline:  08-23-24

R2-2407764	Miscellaneous corrections for Multi-Carrier enhancements	Huawei, HiSilicon, Lenovo, NTT DOCOMO INC.	CR	Rel-18	38.331	18.2.0	4903	1	F	NR_MC_enh-Core

=>	The CR is agreed

R2-2407424	Clarification on deactivated or dormant cells in multi-cell scheduling	Samsung, ZTE Corporation, Sanechips, NTT DOCOMO, OPPO	CR	Rel-18	38.321	18.2.0	1912	-	F	NR_MC_enh-Core

-	Qualcomm and LG don’t think this is needed.  CATT thinks that the change is correct and aligned with RAN1 behaviour.

=>	RAN2 understands that current specifications supports UE to monitor the PDCCH for multicell scheduling with deactivated/dormant SCell

=>	The CR is not pursued

R2-2407463	Reply LS on UE capability for multi-carrier enhancements	NTT DOCOMO, INC., vivo	LS out	Rel-18	NR_MC_enh-Core	To:RAN4	Cc:RAN1

=>	The LS is approved

### 7.25.3	Other

RAN3, SA2, SA3, CT1 led items and others, e.g. eNPN, Slicing, NTN self evaluation issues, etc.

# 8	Rel-19

## 8.0	General

This AI is reserved for Rel-19 LSs from other WGs.  No contributions are expected on these LSs for this meeting

ASN.1 input

R2-2407087	On new ASN.1 review process	Ericsson	discussion	Rel-19	NR_newRAT-Core, TEI19

Proposal 1: RAN2 to discuss if the review file should be split into smaller files to allow for more delegates to access in parallel.

Proposal 2: RAN2 to discuss the need for automated file lock mechanism for ASN.1 review.

Proposal 3: RAN2 to ask MCC to delete “StyleRefs” from 3GPP specifications.

Proposal 4: RAN2 to discuss the following challenges of the new ASN.1 review process: - Commercial tools need to be selected and be available to all companies. - Availability of scripts and adaptions of the selected commercial tools to all companies need to be ensured. - RAN2 delegates need to learn new tools and process.

Proposal 5: RAN2 to discuss if new ASN.1 review tools and process [1] should be developed.

Proposal 6: RAN2 to discuss whether for 6g consider providing the RRC ASN.1 on file server separated from the procedure texts in Word document.

=>	Noted

R2-2407190	ASN.1 Review Modernization	Nokia Corporation	discussion	Rel-19	R2-2405470

Proposal 1: Initiate a discussion on how to modernize the ASN.1 and procedural text review procedure using Git version control. (moved from 7.0.2.8)

=>	Noted

R2-2407786	Report of [AT127][004][ASN.1 Modernization] F2F offline (Nokia)	Nokia	discussion	Rel-19

=>	Conduct an email discussion with a deadline of 4 November 2024 to discuss requirements of the end of release ASN.1 review process.

=>	Noted

[POST127][004][ASN.1 Modernization] requirements (Nokia)

Intended outcome: discuss requirements of the end of release ASN.1 review process

Deadline:  November deadline

VMR

R2-2406223	Reply LS on FS_VMR_Ph2 solution impacts to RAN (R3-243961; contact: Qualcomm)	RAN3	LS in	Rel-19	FS_VMR_Ph2	To:SA2	Cc:RAN2

=>	Noted

R2-2406239	LS on questions regarding FS_VMR_Ph2 (S2-24073455; contact: Samsung)	SA2	LS in	Rel-19	FS_VMR_Ph2	To:RAN3	Cc:RAN2

=>	Noted

NTZ

R2-2406235	LS on NTZ solution impacts to RAN (S2-2407355; contact: LGE & Ericsson)	SA2	LS in	Rel-19	FS_UAS_Ph3	To:RAN	Cc:RAN2, RAN3

=>	Noted

R2-2406233	Reply LS RP-240891 on NTZ solution impacts to RAN (RP-241668; contact: InterDigital)	RAN	LS in	Rel-19	FS_UAS_Ph3	To:SA2	Cc:RAN1, RAN2, RAN3, SA, ETSI MSG TFES

=>	Noted

## 8.1	AI/ML for NR air interface

(NR_AIML_air-Core; leading WG: RAN1; REL-19; WID: RP-240774)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

R2-2406224	LS on terminology definitions for AI-ML in NG-RAN (R3-243969; contact: Ericsson)	RAN3	LS in	Rel-18	NR_AIML_NGRAN-Core	To:SA5, RAN2	Cc:RAN1, RAN, SA

=>	Noted

### 8.1.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2406212	Reply LS on data collection to enable ML model training and inference in 5GC for Direct AI/ML based positioning (R1-2405578; contact: Ericsson)	RAN1	LS in	Rel-19	FS_AIML_CN	To:SA2	Cc:RAN2, RAN3

=>	Noted

R2-2406236	LS on data collection to enable ML model training and inference in 5GC for Direct AI/ML based positioning (S2-2405833; contact: vivo)	SA2	LS in	Rel-19	FS_AIML_CN	To:RAN1, RAN2	Cc:RAN3

=>	Noted

R2-2407252	On RAN3 inputs about AIML Terminology	Ericsson	discussion	NR_AIML_air-Core

-	Huawei thinks that RAN1 should discuss this as they are the leading group

-	Xiaomi thinks that we can agree to parts of the terminology

=>	Noted

### 8.1.2	Functionality based LCM

Contributions should focus on general understanding of LCM procedure (except for data collection and model transfer/delivery), what is required to enable the UE to perform different steps of the LCM procedure, what is the granularity of functionality, dependencies with RAN1 and what is needed from RAN1 to progress in RAN2

Contributions should be submitted in 8.1.2.x and aspects related to data collections should be submitted in data collection section

Two-sided model discussions are out of scope of this AI

Model identification and model transfer/delivery is out of scope of this AI and will not be discussed in RAN2#127 given further RAN1 progress is required.  Work post RAN2#127 depends on RAN 105 outcome.

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

LCM related to NW-sided model for beam management use case only

No contributions expected for this meeting, waiting for further RAN1 progress

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Including functionality identification, additional conditions and further reporting of applicable functionalities

Including outcome of email discussion [POST126][032][AI/ML PHY] LCM (Intel/Samsung)

R2-2407165	Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung) – Phase 1	Samsung	discussion	Rel-19	NR_AIML_air-Core

Easy agreements:

Proposal 1: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

Proposal 3: Applicable functionalities refers to functionalities that the UE is ready to apply for inference.

-	This is too simple and what is ready

-	Lenovo also thinks that this is also dependent on when inference information is provided

Proposal 5: Activated functionalities refers to functionalities already enabled for performing inference.

For discussion:

Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

=>	Noted

R2-2406381	Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

-	Huawei thinks this is a good approach and there is no mention of reactive and proactive.   WE should wait for RAN1 for the FFS

-	Samsung thinks that we should identify the important questions for RAN1 so we can make progress.  Apple agrees and there are some divergence between RAN1 and RAN2 understanding.  Nokia agrees but we should have an email discussion

-	CMCC thinks that if we were to inference configuration in step 3 there will be singaling overhead so we should only provide the inference configuration in step 5.  Qualcomm thinks that this is too early to discuss.

-	ZTE thinks that the step 5 shouldn’t be dotted lines.

Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

-	Xiaomi doesn’t think that the FFS is needed.  Intel explains that this will keep all proposals consistent and we can discuss the FFSs after RAN1 progress.

-	Qualcomm asks if we are changing the applicable functionality definition and whether the applicable functionality can be determined without NW-side additional conditions.  Samsung indicates that this is why we have to ask RAN1 as there are different interpretations.   Interdigital thinks that the confusion is who decides/checks but for a functionality to be used the three conditions must be fulfilled.

-	Intel thinks that we can add another FFS.

Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

-	Vivo would like to have a unified solution rather than have two procedures UAI and RRC reconfigurationcomplete.  Nokia thinks that even in legacy NR we can provide UAI in reconfiguration complete.

-	Oppo thinks that bullet 1 and 2 are the same.  Intel explains that they are different and that bullet 2 is also referring to the case where the network requests the UAI.

-	Futurwei thinks that there may be other UE capabilities to take into account (i.e. memory).  Intel explains that in email discussions companies didn’t want to expose this internal information and the UE can use this information internally to determine applicability.

-	LG asks whether we will consider the cases where the network requests but the applicable functionality hasn’t changed.  Should the UE report anyways.    Xiaomi thinks that if the network asks the UE should always report.   Apple explains that that logic for UAI has always been that the UE may report, so it doesn’t have to.

Proposal 5: Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

-	Nokia thinks that we need to ask RAN1 on what is required.

Proposal 6: The applicable functionality may be activated by receiving its configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

-	ZTE thinks that the second FFS is not reasonable as the initial state should be deactivate.  Qualcomm agrees.

-	NEC thinks that not all applicable functionality will be activated.   Lenovo thinks that the network can configure which functianility should be activated.  Qualcomm thinks that only one would be activated.

=>	 Noted

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

[POST127][019][AI PHY] LS to RAN1(Intel)

Intended outcome: Agree to list of questions to ask RAN1 to enable RAN2 progress.

Deadline:  two weeks

=> Available in R2-2407848 (LS out)

Postponed

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

R2-2407365	Functionality definition and applicability related report	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

Proposal 1. Before (i) idefining configured functionality and (ii)clarifying relationship between configured functionality and applicable functionality, RAN2 to wait RAN1 input to address below ambiguities:

- Ambiguity on relation between NW sided additional condition and full inference configuration: For the network-sided additional conditions, whether they are partial/separate configuration of full inference configuration (At least for proactive reporting)

- Ambiguity on timing for full inference configuration: Whether full inference configuration needs to be provided to UE before applicable functionality reporting (At least for reactive reporting)

- Ambiguity on range of full inference configuration: Beam related configuration like set A and set B, other CSI framework related configuration, or outside of CSI framework (For both proactive and reactive reporting)

R2-2407189	Applicable functionality reporting	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 1:	Postpone conclusion on applicable functionality reporting procedure until RAN1 progresses on: 1) definition of NW-side additional conditions; 2) relationship of Associated ID and inference configuration information; and 3) required configuration(s) for applicable functionality determination.

R2-2406259	Reporting of applicable functionalities	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2406260	Other LCM for UE-sided model  for Beam Management use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2406335	LCM for UE-sided model for Beam Management use case	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2406381	Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2406382	Beam management UE-sided model LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2406387	Discussion on LCM for UE-sided model for Beam Management	vivo	discussion	Rel-18	NR_AIML_air-Core

R2-2406537	Discussion on the LCM for UE-sided Model for Beam Management	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2406564	Discussion on LCM for UE-sided model	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2406578	Availability and Applicability Reporting for UE-side Functionality	MediaTek Inc.	discussion

R2-2406613	Some aspects for model inference	Sony	discussion	Rel-19	NR_AIML_air-Core

R2-2406643	On LCM for UE-sided models	Qualcomm Incorporated 	discussion	Rel-19

R2-2406672	Further discussion on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2406701	Discussion on LCM for UE-sided model for BM	Xiaomi	discussion

R2-2406758	Discussion on LCM for UE-sided model for Beam Management	Spreadtrum Communications	discussion	Rel-19

R2-2406827	Discussion on LCM for UE-sided model for Beam Management use case	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2406876	LCM for UE-sided model in beam management	Lenovo	discussion	Rel-19

R2-2406934	Discussion on LCM for UE-sided model  for Beam Management use case	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

R2-2406963	Discussion on LCM for UE-sided model for BM	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2407066	LCM for UE-side model for Beam Management	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2407154	Discussion on LCM for UE-sided model for Beam Management	Sharp	discussion

R2-2407164	Remaining issues in LTM for UE-sided model	Samsung	discussion	Rel-19	NR_AIML_air-Core

R2-2407251	LCM for UE-side models for beam management	Ericsson	discussion	NR_AIML_air-Core

R2-2407435	Further Discussion on LCM for UE side Model	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2407457	LCM for UE-sided model for beam management use case	TCL	discussion

R2-2407471	Considerations for the Five Functionalities Conditions 	Kyocera 	discussion

R2-2407485	Discussion on Functionality-based LCM of UE-sided Model for Beam Management Use Cases	CEWiT	discussion	Rel-19	NR_AIML_air-Core

R2-2407488	Discussion on LCM for UE-Side Models (Beam Management)	Futurewei	discussion	Rel-19

#### 8.1.2.3	LCM for Positioning use case

No Contributions expected for this meeting, waiting for further RAN1 progress.

should focus on UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases

Not treated

R2-2406334	Lifecycle management for positioning use-cases	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2407456	Discussion on Dynamic Capability Report for Positioning	Sharp	discussion

=> Withdrawn

### 8.1.3	NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18

Starting/stopping of data collection:

R2-2406383	Enhancements for NW-side model training data collection	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Network controls the time duration of how UE performs training data collection in below two options:

Option 1: Network monitors a timer by implementation and releases training data collection configuration upon completion,

Option 2: Network configures a time window/timer of when UE performs training data collection via RRC signaling.

-	Oppo thinks that option 2 would make UE

Proposal 2: UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.

Proposal 3: When UE reaches its buffer limitation, following two options can be considered up to UE’s implementation:

1) stop measurements and reports the logged measurement reports to network based on network configuration

2) continuously perform training data collection measurement, releases the most oldest data in the buffer, then reports to network based on network configuration

=>	Noted

R2-2406573	Enhanced MDT for Data collection for Network Side Model Training  	MediaTek Inc.	discussion

Proposal 3: As the baseline approach, the UE initiates the collection and logging of L1 measurement results once it receives the measurement configuration for AI/ML-enabled features/FGs.

-	Qualcomm thinks that gNB will provide different RS configuration for the UE to measure so they can be activated/deactivated by the NW.   IT should not be activated based on configuration.

-	Nokia thinks that the network should be in charge of activation/deactivation.

-	ZTE thinks that baseline should be based on configuration and it should be FFS whether it can be further activated/deactivated.

-	Apple is not sure about the gain of having dynamic activation/deactivation.

-	Lenovo thinks that can be multiple configuration and the UE would measure when certain conditions are met.

Proposal 4: Other criteria to control UE data collection and logging for model training are considered, including the UE memory, power state and link quality.

=>	Noted

Discussions

-	LG thinks that memory can be a consideration but the link quality is not necessary as the network can control the stopping

-	ZTE explains that MDT memory and IDC can stop the logging

-	Huawei wonders if we need a standardized solution

-	Oppo thinks we need to differentiate for the case

Power state

-	Apple, ZTE, Lenovo would like to consider power state from the UE .  Qualcomm thinks it can be very vague if we specify it.  Nokia thinks that this isn’t the highest priority.  Mediatek thinks we need to consider as the UE needs to stay in connected mode long time.  Vivo agrees with MEdiatek.   Qualcomm agrees that the network shouldn’t keep the UE in connected just to do data collection, so we should consider it.   Xiaomi, CATT and Oppo would like to consider but is not sure how we would specify it.

Event based/on-demand reporting:

R2-2406673	Further discussion on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: Support the following event reporting of logged L1 measurement:

- AS buffer based event reporting (e.g. when UE’s AS buffer to store L1 measurement logging is full)

- Radio condition based event reporting (e.g. reuse legacy A1/A2 event)

-	Interdigital asks what is the use case for reporting for radio condition based.  It may make sense for logging purposes but not reporting.

Proposal 7: Support NW requested reporting of logged L1 measurement by reusing RRC message pair UEInformationRequest / UEInformationResponse.

=>	Noted

R2-2407366	NW side data collection	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

Proposal 2. To report data, to consider report type in RRC: periodic, event-based, on-demand request

Proposal 3. To discuss whether there need additional condition to report with respect to UE’s memory size. The straight forward way is to report collected data when the number of data from lower layer is larger than a threshold.

=>	Noted

R2-2406258	Data Collection for Network Side Model Training	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: For data collection for NW-sided BM model training, postpone the discussion on whether to introduce event-based reporting.

R2-2407040	Discussion on NW-side data collection 	Samsung R&D Institute UK	discussion

Proposal 3c. RAN2 will not pursue event-based reporting.

Discussion on AS buffer based event reporting

-	Qualcomm doesn’t thinks that makes sense it should just be based on NW request.   Qualcomm thinks that the UE would always result in a failure.    LG thinks that event based report can be beneficial.

-	Ericsson thinks that instead of reporting it can send an availability reporting.  Qualcomm, Interdigital, CATT, Apple and ZTE think it is a good idea.   Samsung doesn’t think this is urgent so periodic reporting is sufficient.

-	ZTE thinks that the AS buffer based is a good solution.

-	Vivo thinks that periodic reporting would be enough.

-	Lenovo thinks that buffer based makes sense as instead of losing data we can provide the data to network and keep logging after.

-	Huawei thinks that periodic is enough and what are the extra benefits.

-	Interdigital thinks that if we have event based we don’t need periodic reporting.

-	Nokia thinks that if we have buffer based event reporting we should report right away rather than sending availability indication as a separate message.

-	Apple is concerned that the network doesn’t know that the UE has stopped logging.

-	Oppo thinks that event based may cause congestion in the network.   Ericsson thinks that is why we need availability indication.  LG and Mediatek agrees with Ericsson.

-	Xiaomi this is only useful when the UE buffer is full.

Discussion on demand reporting

-	Interdigital thinks that this should be base line.  LG supports and if we have availability indication this needs to be supported.  Ericsson thinks this is only linked to availability indications.    ZTE doesn’t thinks this should be linked to availability, the NW can request the logged data at any time.    Samsung asks what is the difference between aperiodic and on-demand.   Nokia doesn’t think that availability is needed.

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Handling of large logs:

R2-2406388	Discussion on NW side data collection framework	vivo	discussion	Rel-18	NR_AIML_air-Core

Proposal 6.	When a single RRC message is not sufficient to report logged data, RRC message segmentation is used to report logged data.

=>	Noted

R2-2407248	NW-side data collection for beam management use cases	Ericsson	discussion	NR_AIML_air-Core

Proposal 2: The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements).

Proposal 3: In case all the collected data do not fit into a single RRC message, RAN2 to discuss:

a.	Whether and how the UE should inform the network about remaining collected data to be transmitted.

b.	When the UE should transmit remaining collected data, e.g. in following RRC message(s) based on SRB priority, upon gNB request, at the next periodic transmission occasion, etc.

=>	noted

R2-2406702	Discussion on NW side data collection	Xiaomi	discussion

Proposal 12: Up to UE implementation to send multiple RRC message to carry data, if single RRC message is not sufficient to carry all data.

=>	Noted

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

Discussion on SRBs used

-	Qualcomm asks which SRB we would send it on.   Interdigital thinks that it shouldn’t be SRB1. LG thinks we can follow MDT over SRB2.   Qualcomm thinks that even SRB2 is priority SRB and this is not urgent.    Nokia thinks that it should be low priority SRB.  Oppo thinks that if we use low priority SRB, periodic reporting is useless.

Not treated

R2-2406498	AIML MDT-based data collection for NW-side model 	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2406503	NW side data collection	TCL	discussion	Rel-19

R2-2406538	Discussion on NW side Data Collection for Beam Management	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2406539	Discussion on NW side Data Collection for Positioning	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2406644	On Network Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

R2-2406776	Discussion on NW side data collection for AI-ML based positioning accuracy enhancement	Baicells	discussion

R2-2406828	Consideration on NW side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2406877	Data collection for NW-sided model training in beam management	Lenovo	discussion	Rel-19

R2-2406878	Data collection for NW-sided model training in positioning	Lenovo	discussion	Rel-19

R2-2406964	Discussion on NW side data collection	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2407067	Data Collection for Training of NW-side models	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2407243	Data Collection for Network-Sided Model Training	InterDigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2407331	Discussion on NW-sided data collection for training	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

R2-2407417	Discussion on  NW-side Data Collection for Positioning	Sharp	discussion

R2-2407436	Further Consideration on NW side Data Collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2407489	Discussion on Data Collection for NW-side Model Training	Futurewei	discussion

R2-2407535	Discussion on Network Side Data Collection	Rakuten Mobile, Inc	discussion	Rel-19

### 8.1.4	UE side data collection

Study part of WID - Contributions should focus on the mechanisms identified for data collection for UE side model training during rel-18

Including outcome of [POST126][034][AIML PHY] TP for data collection  (Ericsson)

Contributions should focus to conclude remaining FFSs in table and any potential recommendations for RANP 105.

Input for TR regarding UE data collection:

R2-2407209	RAN2 inputs to TR 38.843	Ericsson	draftCR	Rel-19	38.843	18.0.0	F	NR_AIML_air-Core

=>	The draft CR is updated in R2-2407807

R2-2407802	RAN2 inputs to TR 38.843	Ericsson	draftCR	Rel-19	38.843	18.0.0	F	NR_AIML_air-Core

R2-2407807	RAN2 inputs to TR 38.843	Ericsson	draftCR	Rel-19	38.843	18.0.0	F	NR_AIML_air-Core

=>	The TP is endorsed and will be sent to RAN1 to be merged in final TR CR

R2-2407773	Report of [AT127][021][AIML PHY] UE side data collection	InterDigital Inc. (Rapporteur)	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Full controllability to be defined as below:

MNO can control the management of the data collection and transfer, without a need for an SLA.

-	Samsung thinks that data collection should not be part of this discussion.

-	BT and DT

Proposal 2: Visibility of data content signifies that the MNO can be aware of, access, and comprehend the transferred data, without a need for an SLA. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to anonymise the collected data).

Option 1b controllability

-	Mediatek thinks that it is absolutely true that RAN doesn’t have any controllability.

Proposal 3: RAN2 to discuss the need for partial visibility (for solutions 1b/2/3).

-	Qualcomm asks if it is possible to achieve partial visibility.   Nokia explains that there are many ways to do partial visibility, it can be done via a container and an IE indicating the container.  Xiaomi thinks that partial visibility should not be removed as this would allow the UE to transmit proprietary information.

-	Apple think that partial visibility is not discussed before in 3GPP.   Ericsson and Huawei asks what is partial visibility, if it proprietary information then the UE can send it over the top.   CMCC and Verizon agrees.

-	BT thinks that we have spoken about visibility for a while now and it is possible, even if it is for the first time.  It should not be precluded.   Samsung also things that there are some benefits.

=>	Noted

Agreements:

-	update note above table to add controllability can be achieved via SLA:

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility and any level controllability could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

-	update note1 in table as follows:

Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection without the need of SLA. This includes at least initiating, terminating, and fully managing data transfer.  FFS if further refinements/modifications to this definition are needed.

-	Update Note 2

- 	Note 2: Visibility of data content signifies that the MNO can at least, be aware of, access, and comprehend the data during transfer without the need of SLA. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data).

-	Table modified by chair during online time uploaded in R2-2407785 is agreable and will be reviewed over email for editorial corrections

[AT127][028][AI PHY] TR update (Ericsson)

Intended outcome: final review of UE data collection, endorsement of draft CR (R2-2407807) , and LS to RAN1 (R2-2407808)  to provide the endorse CR for TR

Deadline:  08-23-24

R2-2407785	Data Collection Table modified online	Chairlady	discussion	Rel-19	NR_AIML_air-Core

=>	The content are agreeable and will be reviewed over email

=>	Noted

R2-2407808	LS to RAN1 on TR 38.843 update	Ericsson	LS out	Rel-19	NR_AIML_air-Core	To:RAN1

=>	The LS is approved

=> Revised in R2-2407832 (attachment was missing from R2-2407808

R2-2407832	LS to RAN1 on TR 38.843 update	Ericsson	LS out	Rel-19	NR_AIML_air-Core	To:RAN1

=> Approved

R2-2406965	Discussion on UE side data collection	CMCC, China Unicom, NTT DOCOMO, ZTE, OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: RAN2 to revise the full controllability definition as follows:

Note 1: Full controllability on data collection means all the procedures as well as the control info, e.g. data collection configuration, involved in the procedures for data collection are understandable and managed by MNO, MNO can do adjustment anytime without any SLA with third party.

Proposal 2: RAN2 to revise the definition of full visibility as follows:

Note 2: Full visibility of data content signifies the capability of the MNO to decode and process, e.g. modify/use, the data using 3GPP spec and/or standardized spec out of 3GPP without SLA with third party.

=>	Not treated

R2-2407250	UE-side Data Collection	Ericsson	discussion	NR_AIML_air-Core

Proposal 1	Clarify in the TR that for option 1b/2/3, the final termination entity can be either the Server for data collection for UE-side model training (inside the MNO), or the OTT server (outside the MNO).

Proposal 2	Clarify in the TR that the full controllability of the UE-side data collection implies the MNO´s ability to manage both the data collection process (including e.g. the ability to allow/reject/initiate/terminate/pause the data collection process) and the data transfer (as already agreed).

Proposal 3	From RAN2 point of view, AIML-specific controllability full or partial should be possible in all the options (except option 1a). The level of controllability should depend on the MNO decision. RAN2 consult SA2 to discuss the details of how controllability can be achieved for the various options.

Proposal 4	Standardized full or partial visibility should be possible in all the options (except option 1a). RAN2 consult SA2 to discuss the details of at least option 1b/2 and 3 (if UP solution is adopted).

Proposal 5	No standardized visibility should be considered only for option 1a.

=>	Not treated

R2-2406829	Consideration on UE side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Remove “FFS: UP tunnel” in the aspect of UP/CP tunnel for option 2/3.

Proposal 2: Remove “FFS: LPP” in the aspect of UP/CP tunnel for option 2.

Proposal 3: If Proposal 1 is agreed, remove FFS: the protocol layer for UP tunnel in option 2/3.

Proposal 4: Capture No standardized visibility for Visibility of data content in MNO and Data format for option 1b.

Proposal 5: Partial/partially visibility can be defined as: the MNO is aware of the procedure/PDUs data collection procedure, but has limited access/comprehension to some elements of the data content.

Proposal 6: For option 2, Partial visibility for partially standardized data content in MNO and Data format is captured into TR 38.843.

Proposal 7: For option 2, FFS Opt C) No standardized visibility in the TR 38.843 is removed.

Proposal 8: For option 3, Partial visibility for partially standardized data content in MNO and Data format can be captured into TR 38.843.

Proposal 9: For option 3, FFS Opt C) No standardized visibility in the TR 38.843 is removed.

=>	Not treated

Requirements:

R2-2407413	Data collection for the training of a UE-sided model	InterDigital Inc., Ericsson, Nokia, T-Mobile USA Inc., BT PLC., Deutsche Telekom, Verizon, NTT Docomo, Charter Communications Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: The network should have full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data collection and data transfer.

=>	Noted

R2-2406674	Further discussion on UE-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 11: Remove the FFS on “need for control” in MNO concern because “Controllability of MNO on data transfer” is one evaluation aspect of UE-sided data collection solution but it is not a concern.

Proposal 14: Before making conclusion of option 1b/2/3, RAN2 discuss why UE-sided data collection needs to be enhanced with MNO controllability for training data transfer and visibility of data content, including any blocking issue of solution 1a for RAN1 identified AI/ML use cases and how much system benefit the enhancement can achieve.

=>	Not treated

Recommendations:

Not treated

R2-2406965	Discussion on UE side data collection	CMCC, China Unicom, NTT DOCOMO, ZTE, OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 15: From RAN2 point of view, Option 2 and Option3 is recommended for data collection for UE-sided model training.

Proposal 16: Even if Option 2 and Option 3 is recommended by RAN2, data privacy issue will be addressed for Option 2 and Option 3 when designing the solution details.

R2-2406407	Data collection for UE side model training	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: For UE side data collection, it is recommended to select Option 2 with partial visibility for partially standardized data content, with standardized fields for data visibility and container for UE/chipset proprietary information.

R2-2407332	Discussion on UE-sided data collection for training	Huawei, HiSilicon, CAICT	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: For the study objective of UE-sided model training data, we recommend to only adopt Option 1a as the candidate solution.

R2-2406389	Discussion on UE side data collection	vivo	discussion	Rel-18	NR_AIML_air-Core

Proposal 11.	No down-selection of options for UE-sided data collection is expected in RAN2.

Proposal 12.	RANP to further discuss the options based on the analysis of RAN2.

[AT127][021][AI PHY] UE Data Collection ()

Intended outcome: face to face discussion on table FFS

Deadline:  08-23-24

R2-2406340	Remaining issues for Data Collection for UE side Model training	NEC	discussion

R2-2406384	Discussion on UE-sided model training data collection	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2406572	Discussion on the FFS Issues of UE-Side Data Collection	MediaTek Inc.	discussion

R2-2406645	On UE Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

R2-2407068	Data Collection for Training of UE-side models	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2407437	Further Consideration on UE side Data Collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2407490	Discussion on Data Collection for UE-side Model Training	Futurewei	discussion	Rel-19

R2-2407503	Remaining FFSs on data collection options for UE-side model training	Samsung	discussion	Rel-19	NR_AIML_air-Core

## 8.2	Ambient IoT

(FS_Ambient_IoT_solutions,leading WG: RAN1; REL-19; SID: RP-240826)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.2.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2406237	LS on Clarification of requirements for Ambient IoT (S2-2407231; contact: Ericsson)	SA2	LS in	Rel-19	FS_Ambient_IoT_solutions	To:SA1, RAN1, SA3	Cc:RAN2, RAN3

=>	Noted

R2-2406271	TP for TR 38.769 update	Huawei, CMCC, T-Mobile USA	discussion	Rel-19	FS_Ambient_IoT_solutions

=>	The TP is endorsed as baseline for further updates

=>	The TP should be updated to follow 3GPP drafting rules

[POST127][022][AIoT] TP for TR (Huawei)

Intended outcome: Updated TP based on agreements from RAN2 127

Deadline:  long

### 8.2.2	Functionality aspects

Contributions should focus on the functionalities required for A-IoT devices, including security related aspects, need of resource allocation/scheduling, segmentation/reassembly (pending RAN1 progress), QoS handling, higher layer repetition, message size/status indicationetc.?

Segmentation

R2-2406654	Ambient IoT transport block size and MAC layer segmentation	MediaTek Inc., Samsung, vivo, ZTE Corporation, Sanechips, Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: Indicate to RAN1 that RAN2 can define MAC PDU sizes to correspond to the capacity of the PHY layer, and RAN2 will accordingly follow RAN1 on the definition of the TB size.

Proposal 2: Ask SA2 what data block sizes are expected to be delivered from upper layers to AIoT AS layers.

Proposal 3: Inquire of RAN1 regarding the conditions under which different TB sizes may be used in both the D2R and R2D directions.

-	Nokia supports thinks that we should add SA3 in cc.  Lenovo also supports.

-	Lenovo thinks that for coverage purposes the TB sizes may be smaller.

-	CATT supports RAN1 LS but doubts that SA2 can give us a clear answer on this so perhaps LS to SA2 is not needed.

-	CMCC and Intel thinks that RAN1 is anyways discussing this so it doesn’t help.

-	Vodafone thinks that application layer can support segmentation, so we can ask SA2 is they can support segmentation.

-	CCMC thinks that from RAN2 point of view there is no need for segmentation.

-	ZTE thinks that whether segmentation is needed depends on many aspects.   But upper layers cannot do segmentation based on physical layer conditions and TBS.   WE should understand the functionality need and if we want to do segmentation it should be done in lower layers.  Ericsson agrees.

-	Interdigital thinks that we can provide information to RAN1 to help us make a decision.

-	Telit thinks we should ask about typical size.

-	Qualcomm and vivo thinks that if we were to do segmentation it should be done in L2 as it is simpler than doing it in RAN1

=>	Noted

Agreements

-	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size.

[AT127][023][AIoT] LS to RAN1/SA2 (Mediatek)

Intended outcome: Approval of LS

Deadline:  08-23-24

R2-2407830	Draft LS on data block sizes for Ambient IoT	MediaTek	LS out	Rel-19	FS_Ambient_IoT_solutions	To:RAN1, SA2

=>	The LS is approved in R2-2407831

R2-2406942	Views on the segmentation of A-IoT	Huawei, HiSilicon, Wiliot Ltd., Quanray, Telit Communications S.p.A., Nordic Semiconductor ASA, China Unicom, China Telecom, China Southern Power Grid, NTT DOCOMO INC., LG Electronics Inc., Spreadtrum Communications, Xiaomi, ASUSTeK, Kyocera, ETRI, CAICT, NEC	discussion	Rel-19

Proposal 1:	For A-IoT device, RAN2 assumes that there is no need of L2 AS segmentation functionality. (This assumption can be revisited, if needed, based on other WGs further progress.)

-	Qualcomm and Samsung thinks that we are jumping to a conclusion prematurely before even getting an answer from RAN1.  Samsung thinks that we can define a simple segmentation if needed.  ZTE agrees and we can have segmentation with no SN and it can be needed.

-	LG thinks that segmentation will cause complexity.  CATT doesn’t think that there will be a complexity as we can do buffer free segmentation.  Xiaomi thinks that you will always need a buffer for segmentation.

=>	Noted

Charging Impacts

R2-2406590	Energy Status report	Vodafone, Qualcomm, Nokia, Interdigital, Xiaomi, Samsung, Deutsche Telekom	discussion	Rel-19

Proposal 1: It is proposed to include energy status report to be delivered from the UE to the reader in case there the device does not have anergy for the follow up messages

Proposal 2: It is proposed to discuss if one bit of information is sufficient for any use case highlighted above or there is a need to represent the energy status in a different way (e.g. sufficient energy for sending of a particular number of bits or an expected time when the device will be ready for subsequent operations or any others)

-	ZTE thinks that some information may be useful on the reader side but wonders if the reader can get this information based on device type.  Vodafone thinks that we have a lot of things that we are studying but there may be device type that can’t provide the information but for others it can be useful.

-	Lenovo and Nokia are supportive.   Lenovo is not convinced that 1 bit indication would be sufficient.

-	Intel asks what is the reader action when it receives this indication and whether the intention is for reader to keep the status.   Qualcomm explains that the reader would then wait until the device has sufficient energy.

-	Huawei thinks that we can study but we shouldn’t make any agreements on the need to use it or not.

-	Futurewei thinks that this shouldn’t be a standalone message.   RAN1 is not going in this direction and may specify states.

=>	Noted

Message Size/Status Indication

R2-2406769	Discussion on functionalities required for A-IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3	RAN2 to support device message size/status indication.

=>	Noted

R2-2406977	Discussion on functionality for A-IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6: Message size or status reporting is not considered for A-IoT is this release.

=>	Noted

R2-2406608	Discussions on functionalities required for AIoT	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: RAN2 is kindly asked to agree that the device can provide the message status information, e.g., number of bits for transmission, time length for D2R transmission, to the reader.

=>	Noted

Discussions

-	LG doesn’t support message size reporting as the message size should be known to the reader so the reader can schedule accordingly.  Intel thinks that there is also dependency on RAN1 discussion.   Xiaomi thinks that this would only be needed if segmentation is needed.

-	ZTE thinks that we say that it is useful for the reader to know the message size.    Qualcomm thinks that we are talking about the remaining size.

-	Interdigital thinks that for segmentation it will be needed and also for command.

-	Vodafone asks if this is a one shot indication.   ZTE confirms

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

Visibility of AIOT Information

R2-2406751	Discussion on the functionalities required for Ambient IoT	Spreadtrum Communications	discussion	Rel-19

Proposal 11:  The following information is visible to the reader：

-	The service type of A-IoT (i.e. inventory and/or command);

-	The service is targeted on single or multiple devices;

-	The number of devices, if the service is targeted on multiple devices.

=>	Noted

R2-2406390	Discussion on functionality aspects for Ambient IoT	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

Proposal 6	The reader executes the AS procedure in response to the service request from CN based on the necessary information derived from the received service request, wherein the necessary information may comprise:

-	the service type information, e.g. inventory, read/write command.

-	the ID of the target device.

-	the population of target devices.

-	D2R data/message size.

-	periodicity to execute the service request.

-	QoS requirement (e.g. e2e latency and target successful inventory ratio)

=>	Noted

R2-2406709	Functionality for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8:	RAN2 assumes the AS layer has some visibility of the command or command type carried by a R2D PDU (e.g., for the reader to determine the transmission behaviour from a device).

Proposal 9:	RAN2 assumes that commands (e.g., read/write/inventory/disable) themselves are carried over the AIOT interface as upper layer data.

=>	Noted

Discussion

On need for service type

-	Xiaomi doesn’t think this is necessary.

-	Lenovo thinks that we should also be told whether it is for multiple UEs.

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

NOT treated

Temporary Device ID (need, assignment, duration)

R2-2406737	Discussion on functionality aspects of Ambient IoT	China Telecom 	discussion

Proposal 4: Whether to introduce temporary ID to identify A-IoT devices needs further discussion.

R2-2406769	Discussion on functionalities required for A-IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4	A device identifier is used to identify a device during communication with the reader.

Proposal 5	For contention-based random access, the random ID transmitted in Msg1 can be used as the device identifier after the devices receives Msg2 which completes contention resolution.

Proposal 6	For contention-free access, device identifier is allocated by the reader, e.g. in AIoT paging message.

Proposal 7	Device identifier can be updated, e.g. based on the reader’s request.

R2-2406656	Ambient IoT data transmission after initial access	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: Data can be transferred between the reader and device in the R2D direction as long as the device’s AS identifier (random number or value assigned by the reader) remains valid.

R2-2406818	Functionality aspects for A-IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 14	Study whether a requirement on how long the AS ID can be kept in memory is feasible and reliable.

L2 Repetitions

R2-2406459	A-IoT Functionality	ZTE Corporation, Sanechips	discussion

Proposal 7: MAC layer should support a mechanism where the upper layer packet/segment is retransmitted when it is not received successfully

R2-2406751	Discussion on the functionalities required for Ambient IoT	Spreadtrum Communications	discussion	Rel-19

Proposal 7: R2D repetition can be up to reader implementation.

Proposal 8: D2R repetition can be supported by reader indication or device implementation.

R2-2406818	Functionality aspects for A-IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6	RAN2 to study repetitions mechanism in the hypothesis that PHY layer repetitions or just one transmission is not enough to satisfy a success probability requirement at cell edge.

Proposal 7	RAN2 to consider multiple RACH attempts as a mechanism to implement repetitions.

Protocol Stack

R2-2406540	Discussions on Functionality Aspect of Ambient IoT	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: Only A-IoT MAC layer is needed as AS L2 protocol.

Proposal 2: A-IoT device may be configured via A-IoT MAC layer, using MAC CE-like message.

R2-2407319	Views on Functionality Aspects of Ambient IoT	Qualcomm Incorporated	discussion	FS_Ambient_IoT_solutions

Proposal 8:  	A new simplified AIoT L2 protocol will be introduced.

Proposal 9:  	Simplified AIoT protocol stack is unified for AIoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

QoS

R2-2406608	Discussions on functionalities required for AIoT	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1-2: RAN2 is kindly asked to consider the inventory latency reduction when designing inventory procedure.

Proposal 1-3: RAN2 is kindly asked to discuss whether the inventory + command and command only procedure have the latency requirement or not.

R2-2406362	Discussion on protocol stack for ambient IOT	Xiaomi	discussion	Rel-19

R2-2406377	Required functions for A-IoT	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406453	Considerations for functionality aspects	Semtech Neuchatel SA	discussion

R2-2406482	Discussion on the Functionality Aspects for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406505	Ambient-IoT Functionality Aspects	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406582	Inventory procedure without permanent device ID	VODAFONE Group Plc	discussion	Rel-19

R2-2406614	Considerations on functionality aspects for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406666	Funcitonal Aspects of A-IoT devices	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406812	Considerations on functionality aspects for Ambient IoT	Lenovo	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406941	A-IoT functionalities	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407063	Discussion on security aspects for Ambient IoT	NTT DOCOMO, INC.	discussion	Rel-19

R2-2407126	Discussion on security related aspects of Ambient IoT devices	T-Mobile USA Inc., Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions	Revised

R2-2407132	AIoT Functionality aspects	Nokia	discussion

R2-2407231	Discussion on security related aspects of Ambient IoT devices	T-Mobile USA Inc., Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions	R2-2407126

R2-2407290	Discussion on functionality aspects of ambient IoT	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407342	Discussions on functionalities required for A-IoT Communication	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407445	Functionalities for Ambient IoT	Kyocera	discussion	Rel-19

R2-2407504	Discussions on functionality aspects for AIoT	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

### 8.2.3	A-IoT Paging

Contributions should focus on paging aspects and content required for Ambient IoT for the different identified procedures (i.e. inventory, inventory + command, command only), including monitoring of DL message and determination of transmission/access occasion.

Paging Triggers/Retransmission

R2-2407244	Discussion on DL messages for Ambient IoT UEs	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2	The gNB/intermediate UE should be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN.

Proposal 3	gNB/intermediate UE provides an ID in the paging message to associate it with a CN request. Details of the ID is FFS.

Proposal 4	If paging message indicates an ID different that the one indicated in the previous paging message, i.e., paging messages are associated with separate requests from the CN, the device performs the procedure.

Proposal 5	If paging message is associated with the e.g., same request from the CN, when compared with the previous paging message, the device needs to perform the procedure only if it has not completed the procedure before in one of the earlier rounds that are associated with the same request from the CN. Otherwise, the device can ignore the A-IoT paging message.

=>	Noted

R2-2406426	Discussion on A-IOT paging procedure	Xiaomi	discussion

Proposal 8: RAN2 to discuss and support the BS reader to trigger A-IOT paging procedure periodically according to the configuration from CN.

-	Apple, Oppo, and Vivo think this should be discussed in SA

-	Nokia doesn’t thinks the CN should be in control of the radio resources

=>	Noted

Discussion

-	CATT ask what is the use case for subsequent paging, if multiple devices failed to answer this paging, it is due to contention conflict and we need to make enhancements to RA.

-	Ericsson explains that it is not due coverage but setting the q number properly.   Huawei thinks that this is useful and one additional use case is that some devices turn on after the first paging.  LG, Docomo, Apple, agrees with Ericsson.  Apple explains that the device could have also run out of energy.

-	FUturewei asks whether this is for all use cases.  MEdiatek and Intel explain that it is use case agnostic.

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

Definition of the access occasion

R2-2407533	Discussion on Paging procedure for Ambient IoT	Philips International B.V.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3: Reader may transmit access occasion announcement message.

=>	Noted

R2-2406747	Discussion on paging procedure of A-IoT	Spreadtrum Communications	discussion	Rel-19

Proposal 1: The boundary of the access occasion can be defined/indicated by the reader sending paging repetition or other message.

=>	Noted

Discussion

-	Qualcomm and Docomo thinks that RAN1 is discussing this so we should wait.   Huawei thinks that from RAN2 perspective we need to understand message.   Samsung indication from reader to device to indicate boundary of access occasions.

-	Apple, Ericsson, Qualcomm thinks that the paging message would indicate it.

=>	wait for further RAN1 progress on indication of the start of access occasion.

Not treated

Determination of the access occasion in CBRA case

R2-2407508	Discussions on AIoT paging	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 9. For an AIoT paging message that triggers a random access procedure, the reader should schedule multiple D2R access occasions. An intended device generates an RN within the range of the RNs as indicated in the AIoT paging message, and then uses the RN to determine whether and in which D2R access occasion it can send its response.

R2-2406710	Paging Related Aspects for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3:	A paging message for contention-based access includes parameters that map the device ID to one or more corresponding resources (e.g., access occasions) in a deterministic way.

Proposal 4:	A device performing contention-based access may randomly select among the resources determined from the device ID to select the specific resource used for random access initiation (i.e., MSG1).  FFS on any resource selection restrictions (e.g., defined by RAN1).

R2-2407136	Discussion on A-IoT paging message	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 5.	RAN2 wait for RAN1 further progress on details of resources to transmit A-IoT Msg1.

Determination of the access occasion in CFRA case (single device and multiple devices)

R2-2406960	Discussion on A-IoT paging	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5: There may be no explicit indication of D2R transmission resources when paging a single device.

R2-2407508	Discussions on AIoT paging	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8. For an AIoT paging message that triggers a non-RA procedure, if the AIoT paging message is intended for multiple devices, the reader should schedule multiple D2R access occasions, one unique D2R access occasion for each intended device. The intended devices determine their respective D2R access occasions based on a one-to-one mapping between the ranks among the scheduled D2R access occasions and the ranks among the intended devices.

R2-2407283	Discussion on A-IoT paging	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: Given that A-IoT paging message including multiple IDs of A-IoT devices could indicate information for resource determination in the RA procedure, RAN2 should confirm the need for A-IoT paging message to include multiple IDs of A-IoT devices without waiting for the SA2 discussion.

R2-2406856	Ambient-IoT Paging	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 10:	For contention free RA configuration when A-IoT paging message contains multiple IDs of A-IoT devices, mapping relationship between the IDs of A-IoT device and the random access resources is implicitly indicated by the device ID list.

Access type

R2-2407343	Discussion on A-IoT Paging	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4: RA type, e.g., 2-step/3-step CBRA or CFRA, is provided implicitly or explicitly in A-IoT paging message.

Service type

R2-2406541	Discussions on AIoT paging	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: Paging type, e.g., for inventory-only, command-only or inventory and command is included in the AIoT paging message.

R2-2406426	Discussion on A-IOT paging procedure	Xiaomi	discussion

Proposal 4: No need to introduce paging type indication to indicate the use case triggered the A-IOT paging procedure, e.g. inventory or command or inventory+command.

Additional contents/indications in the paging message

R2-2406879	Discussion on paging procedure for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 8: RAN2 to consider the feasibility of the scenario that one A-IoT device receives the A-IoT paging messages from multiple readers.

R2-2406960	Discussion on A-IoT paging	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8: To improve the efficiency of A-IoT paging, an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging.

R2-2406378	Consideration on initial trigger message	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406391	Discussion on AIoT Paging	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

R2-2406483	Discussion on Paging for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406501	Ambient IoT device paging	TCL	discussion	Rel-19

R2-2406520	Discussion on Ambient IoT paging message	ASUSTeK	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406609	Discussion on A-IoT paging	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406615	Considerations on paging for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406651	Paging for Ambient IoT	Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406655	Procedures and signalling for ambient IoT paging	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406681	Discussion on Ambient IoT Paging	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406718	Discussion on paging procedure for Ambient IoT	OPPO	discussion	Rel-18	FS_Ambient_IoT_solutions

R2-2406738	Discussion on Ambient IoT Paging	China Telecom 	discussion

R2-2407021	Discussion on Paging for A-IoT	Transsion Holdings	discussion	Rel-19

R2-2407148	Further consideration on paging procedure for AIoT	ZTE Corporation, Sanechips	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407203	Ambient IoT identifiers for A-IoT paging	Panasonic	discussion

R2-2407212	Paging procedures for Ambient IoT	Nokia France	discussion

R2-2407222	Discussion on Paging aspects for Ambient-IoT	Continental Automotive	discussion

R2-2407261	Discussion on A-IoT paging	Sharp	discussion

R2-2407444	Consideration of paging for Ambient IoT	Kyocera	discussion	Rel-19

R2-2407547	Discussion on A-IOT Paging related aspects	Rakuten Mobile, Inc	discussion	Rel-19

### 8.2.4	A-IoT Random Access

Contributions should focus on A-IoT random access steps for both 2-step and 3-steps RA, content required for the different procedures, and any additional aspects related to CFRA and CBRA procedures.  Failure handling (i.e. msg 3 or paging) will be treated in this AI.

3-step CBRA

MSG1 size

R2-2406484	Discussion on the Random Access for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: With a large enough ID size and slot-ALOHA, it is feasible to use a fixed ID size to generate the random ID for contention resolution. RAN2 to discuss whether the ID size in A-IoT Msg1 can be indicated by the reader.

=>	Noted

R2-2406987	Further consideration on Ambient IoT random access	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7a: A-IoT reader can configure ID length of 3-step CBRA Msg1 to device, can be captured in A-IoT paging message.

=>	Noted

Discussion

-	Vodafone thinks that it should be a fixed size and not complicate further with configurability.   Apple thinks that this is adding complexity.

-	Ericsson thinks that the important part is that both reader and device know the message size.

-	Qualcomm think that it can be known at the deployment level.

MSG1 ID selection

R2-2406484	Discussion on the Random Access for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: The random ID used in A-IoT Msg1 for “3-step” CBRA is randomly generated by the device within the ID size.

=>	Noted

R2-2406987	Further consideration on Ambient IoT random access	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6: RAN2 to agree to use random ID only in 3-step CBRA Msg1, which may be generated based on device ID.

=>	Noted

Discussions

-	Ericsson thinks that we should keep both options randomly generated or based on device ID.  LG thinks that we should randomly generate.  Docomo thinks that if we go based on device ID we would have to ask SA3 if this is causing any security issues.   CMCC thinks that it would be based on time as well.   Qualcomm, Mediatek, Huawei, ZTE and Apple thinks that we don’t need to specify which option at this stage.  This is up to UE implementation.

MSG2 scheduling

R2-2406682	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7	RAN2 discuss whether resource scheduling for Msg3 transmission is indicated in Msg2 or not.

-	Option 1: Msg3 transmission resource is allocated together with the Msg1 Tx resource.

-	Option 2: Msg3 transmission resource is explicitly indicated in Msg2.

-	Apple prefers option 2.    Xiaomi and LG thinks that this is RAN1 scope so we can wait.

-	Vodafone asks if this would be optional, meaning that the reader can but it doesn’t have to.  ZTE thinks that we should at least support msg3 scheduling as a possibility.    Huawei thinks this was agreed in RAN1 that R2D will schedule D2R.  Qualcomm thinks that this is specific to RAN2 RA and we should agree.  Samsung explains that it is agreed in RAN1.   Intel explains that RAN1 is discussing whether this is scheduled in PHY or MAC.

-	MEdiatek thinks we should be careful with sayng can be explicitly as adding optionality will cause complexity.  ZTE agrees but since option 1 is still being discuss we can’t preclude it.

=>	RAN2 assumes that Msg3 transmission resource can explicitly be indicated in Msg2 (based on RAN1 agreements/design).  Wait for RAN1 for further details.

=>	Noted

MSG3 contents

R2-2407385	Discussion on random access for ambient IoT	Google Ireland Limited	discussion	FS_Ambient_IoT_solutions

Proposal 5: Msg3 can contain a buffer status, power status, or device capability information.

=>	Noted

R2-2406711	Random Access Procedure for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5:	For contention-based random access, MSG3 may contain a control element (e.g., MAC CE) to indicate the device message size/status to the reader.

=>	Noted

Discussions on UE capability

-	Intel thinks that it is too early to discuss UE capability.  Vodafone that we can restrict the discussion to device type.   We should be careful to not increase the message3 size.   Nokia agrees.  Qualcomm agrees with size limitation but shouldn’t we also consider msg1?

-	Lenovo thinks that RAN1 should discuss this.

-	Docomo would like to understand whether this is sent by AS or NAS and this also depends on security aspects as well.  Ericsson thinks that CN capability report would be more suitable.

-	LG doesn’t think we need UE capability as all devices can do RA regardless of capability.

-	ZTE thinks that there is also the possibility for the CN to indicate it directly to the reader, so we can minimize the air interface.  CMCC agrees as CN has subscriber information and can share the device type with the reader.   Also it is not clear whether we need different treatment would depend on further discussions.

-	Intel thinks that the coverage is different and that’s why we would need to know to select topology type for example.

-	ZTE asks if we should ask SA2 if this is possible.  Huawei thinks that we would need to understand the justification.  Interdigital explains that RAN1 is discussing device type having different resource capabilities.

-	Huawei thinks that we can also differentiate based on device type.

Subesequent R2D message

R2-2407344	Discussion on A-IoT random access	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: For 3-step CBRA, the subsequent R2D transmission after Msg3 could be used for the following one or both potential cases:

Confirm the failure/success reception of Msg3.

Scheduling/transmission for the following higher layer data.

=>	Noted

R2-2406899	Random access procedure for Ambient IoT	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6: The reader should send a failure indication message to the device if it can't receive the A-IoT Msg3 after sending A-IoT Msg2.

=>	Noted

R2-2406711	Random Access Procedure for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7:	Absence of the subsequent R2D message (after access procedure) can be interpreted by the device to mean at least successful data transmission and no additional command reception.

=>	Noted

Discussions on subsequent transmission after msg3.

-	Docomo asks if the reader can determine whether there was a failure.  Intel thinks that there is a case where the reader knows that it hasn’t received but doesn’t know it happens.

-	Intel thinks that even the device sends a failure indication the device doesn’t know what to do.

-	Xiaomi thinks that there is a case where it can be useful to configure the random value again.

-	LG thinks that this would be useful to resolve the collision between device.

-	Huawei thinks that the reader can indicate the failure to the device and the device can re-attempt access.

-	Ericsson thinks that in some cases it is needed and in some cases it is not needed, so we should study cases it may be needed.

-	MEdiatek thinks that if there was data in msg3 we should acknowledge it, but not necessarily a failure indication for msg3.

-	Lenovo thinks it is necessary.

-	Vodafone is not sure that msg3 contains data, just device ID.

-	Interdigital thinks that the device has already completion contention so it would be beneficial for the reader to indicate so it doesn’t have to trigger another message.   ZTE agrees.

-	Qualcomm thinks that there are different use cases and in some cases it is needed.   R2D should indicate whether subsequent acknowledgement should be expected by device.

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

2-step CBRA

MSG1 contents

R2-2406682	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5	Random ID is not included along with the Device ID in A-IoT Msg1.

=>	Noted

R2-2406716	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6:	For 2-step CBRA, the random ID is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.

=>	Noted

Discussion

-	Huawei thinks that if we don’t have random ID in msg1 then what do send back in msg2, as device ID being echoed is too large.

-	Samsung thinks that both options are possible, but for option 1 we would have to discuss partial ID or full ID for msg2 and that is more complex.

-	CMCC prefers Apple as otherwise what is the difference between 2 and 3 step.

-	Docomo thinks that this is dependent on subsequent tranmsion and Intel thinks we need to understand whether we need msg2.  The randomID can be useful for command only case.

-	CATT thinks that we need the ID to resolve contention and random ID is not preferred as the msg2 can echo back msg1.

-	Qualcomm thinks that msg2 is similar to msg4 (i.e. subsequent transmission).  We should have a unified solution with the 3 step RA.  Vodafone thinks that we should resolve contention based on random number.

-	Huawei thinks that for 2-step RA msg2 is needed.   Mediatek thinks that it is important for the AS to have an AS device to address the device.  ZTE thinks it is important to simplify the devices and including random number will be good.

-	Intel explains that there are cases where msg2 is not needed.  Inventory only cases – device ID sent to reader and if you don’t receive it you can trigger the device to send the ID again. For command – it may be needed

-	Apple doesn’t see the complexity of supporting different design as the UE would only support either 2-step or 4-step.   Vodafone thinks that logistically this is difficult to differentiate between devices.   Williot agrees that there can devices that only support 2 step RA.

-	ZTE thinks that the difference between 2 and 3 step is just the reader indicating to the UE simply send random ID or send data as well.

CFRA procedure

R2-2406716	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 18:	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).

-	Mediatek wonders if we have to confirmed whether the first message needs to be protected.   ZTE is concerned that we would go away from the unification, as even for subsequent transmission we would need an ID.  Huawei explains that random ID is not needed for contention but maybe for scheduling purposes it would need to be further discussed.    Intel also thinks that random ID is needed for subsequent transmissions and context.

-	Xiaomi thinks an ID is always needed, so we should try to specify a common procedure.

-	LG thinks that the ID Can be fixed in paging message.

-	CATT Thinks that the CFRA will give resource so that’s sufficient.

-	Qualcomm asks why we use random ID if we have an ID in the reader used for scheduling the initial CFRA message.    ZTE asks how long would this ID be and would we want to address the UE with 100bits device all the time.   Intel explains that you would need to include the ID again from R2D and that is a large ID.    Mediatek thinks that there are also security issues of including the device ID for scheduling purposes.   The random ID is needed for subsequent scheduling and we should unify the procedure.

-	Nokia thinks that we need an ID but it doesn’t need to be generated by the device.    ZTE thinks it is possible for reading to allocate but we are branching it.

-	CATT thinks that storing the ID would cause complexity.

-	Apple thinks that we would need to maintain some context and random ID could have collision problem.

-	MEdiatek thinks that the reader can provide the AS device ID.  WE can also consider providing this device ID for CBRA so these id is used for subsequent.

-	Vodafone thinks it should be only one approach.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

[POST127][033][AIoT] Random Access (Huawei)

Intended outcome: Discuss Failure/success indication aspects and FFS for CBRA and on FFS on AS ID for scheduling purposes

Deadline:  long

Not treated

MSG2

R2-2406484	Discussion on the Random Access for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5: The reader confirms the reception of data transmission in A-IoT Msg1 by including the device ID information in A-IoT Msg2. FFS including the whole device ID or partial device ID.

R2-2407265	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

Proposal 2. For 2-step CBRA, the A-IOT device transmits the A-IOT device ID and random ID in Msg1. After that, if the A-IOT device receives the same random ID in Msg2, the A-IOT device considers that the contention resolution is successfully completed

Others

Determination of type of access

R2-2406987	Further consideration on Ambient IoT random access	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2c: Reader can indicate CBRA or CFRA for device in A-IoT paging message.

R2-2407317	Views on Random Access Aspects of Ambient IoT	Qualcomm Incorporated	discussion	FS_Ambient_IoT_solutions

Proposal 4: Reader may indicate the type of AIoT access (e.g., 2-step or ‘4-step’) to AIoT devices in the initial trigger message.

Access failure detection

R2-2407344	Discussion on A-IoT random access	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7: RAN2 to study the failure detection from perspective of both reader and device.

R2-2406880	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 6: Device detects contention resolution failure if the device does not receive matched ID from the reader in its access occasion.

R2-2406682	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8	RAN2 study two possible solutions for the device to determine random access failure: 1) timer-based; 2) DL-message based.

Re-access following access failure

R2-2406786	Discussion on UL multiple access	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 12	For handling contention resolution failure and access failure, RAN2 to study the three options:

a.	Option 1: a device which experiences contention-failure or access failure, re-accesses in the same round.

b.	Option 2: a device which experiences contention-failure or access failure, re-accesses in the next round.

c.	Option 3: the round length is adaptive. The round can be adjusted by increasing its length or terminating earlier upon detection of too high collision. More time occasions are added in current round or in the new round (if current one terminated).

Power ramping

R2-2406392	Random Access Procedure for A-IoT Device	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

Proposal 13. Whether to support power ramping for Msg1 transmission is up to RAN1 conclusion and does not affect the RAN2 RACH scheme design.

R2-2406341	Random Access for Ambient IoT device	NEC	discussion

R2-2406361	Discussion on access procedure for ambient IOT	Xiaomi	discussion	Rel-19

R2-2406379	Consideration on A-IoT Random access	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406454	Considerations for Random Access	Semtech Neuchatel SA	discussion

R2-2406460	Unified random-access procedure for A-IoT	ZTE Corporation, Sanechips	discussion

R2-2406502	Random Access for Ambient IOT	TCL	discussion	Rel-19

R2-2406521	Discussion on Ambient IoT access message	ASUSTeK	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406542	Discussions on AIoT Random Access	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406616	Considerations on random access aspects for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406752	Discussion on random access of Ambient IoT	Spreadtrum Communications	discussion	Rel-19

R2-2406764	Further discussions on A-IoT random access	ETRI	discussion	Rel-19

R2-2406770	Discussion on random access for A-IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407022	Discussion on Random Access for A-IoT	Transsion Holdings	discussion	Rel-19

R2-2407207	Discussion on A-IoT random access procedure	NTT DOCOMO, INC.	discussion	Rel-19

R2-2407220	Random access for Ambient IoT	Nokia France	discussion

R2-2407262	Discussion on A-IoT random access	Sharp	discussion

R2-2407443	Consideration of random access of Ambient IoT	Kyocera	discussion	Rel-19

R2-2407458	Further discussion on Ambient IoT random access	Samsung Electronics Czech	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407509	Discussions on AIoT random access	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407536	Discussion on Random Access procedure for Ambient IoT	Philips International B.V.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407542	Discussion on Failure Handling 	Rakuten Mobile, Inc	discussion	Rel-19

### 8.2.5	Topology 2 considerations

Contributions should focus on topology 2 related aspects between gNB and reader.

General Work Guidelines

R2-2406380	Topology 2 considerations	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

=>	Noted

R2-2406881	Considerations on Topology 2 for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 1: A-IoT Uu interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

=>	Noted

Discussion

-	Mediatek, CATT, LG, Samsung, Apple, Oppo, Vivo and Qualcomm think these proposals are good.   Apple thinks that we should send an LS to RAN3.

-	Samsung thinks that first proposal should just be an understanding.   LG thinks we should aim to have low impact.

UE Authorization

R2-2406712	Topology 2 for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5:	Intermediate UE authorization is performed by upper layers without RAN2 impact.

-	Apple thinks that with NCR there was RAN2 impacts.

-	Vodafone thinks that the UE reader has to be authorized by the network.

-	CATT explains that SA3 has identified this as a key issue and are working to fix this issue and from RAN2 perspective there are no identified impact.

-	Samsung thinks whether the gNB needs to be aware that this a UE reader or normal UE.  Intel thinks that we should wait as RAN3 is also discussing this.  Mediatek thinks that this is a UE capability.   Qualcomm thinks that it would be a capability but it also depends on authorization.

=>	Noted

Topology 2 architecture (Upper layer solution or RRC solution)

R2-2406485	Discussion on Topology2 for Ambient IoT	CATT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: RAN2 assumes the transmission between UE reader and RAN node for inventory/command service can be supported by legacy mechanism, i.e., via NAS message or PDU session, which does not bring stage-3 impact on Uu. RAN2 may come back when the network architecture and protocol stack are determined by SA2.

=>	Noted

R2-2406785	Topology 2 and DO-A for Ambient IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2	RAN2 assume RRC sublayer at UE reader handles A-IoT related information.

=>	Noted

R2-2406380	Topology 2 considerations	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5: RAN2 should wait for SA2 further inputs before making decision on the protocol stack and how the intermediate node (i.e., reader) obtain the AIoT service related information since they are tightly related to SA2 discussion.

=>	Noted

R2-2406943	A-IoT Topology 2 issues	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3:	For Topology 2 protocol stacks, RAN2 considers the upper layer based solution (i.e. A-IoT device's upper layer data is carried by the UE reader's upper layer) and the RRC layer based solution for further study.

Proposal 4:	In upper layer based solution, the data forwarding for inventory (and command) has minor RAN2 impact, which can be left to SA2 study.

Proposal 5:	In RRC based solution, the data forwarding for inventory (and command) includes the additional NR Uu impact as follows, and can be captured in TR for Topology 2:

	BS sends the A-IoT related information (e.g. paging identifier) to intermediate UE via Uu RRC message based on received A-IoT service request.

	The device’s upper layer data is forwarded between the intermediate UE and BS, via Uu RRC messages.

=>	Noted

Discussions

-	Intel thinks that SA2 is still discussing but between CP and UP solution from CATT there are no impact for now, but we can wait for further progress

-	Apple, Quacomm, CATT, LG, Oppo, Lenovo, Intel, Mediatek, Interdigital etc is not sure we need yet another RRC solutions.

-	Ericsson thinks that RRC options is being considered in SA2 and can’t be excluded.   Qualcomm explains that it is a NAS option carried by RRC.     Huawei explains that this is captured in SA2 solutions.  ZTE thinks that we shouldn’t fully leave it SA2 and we should at least agree that the gNB has visibility and is in control of the radio resources.

-	CMCC and ZTE thinks that RRC solution should be kept.   ZTE Thinks that RRC is more efficient from an air interface resource.

-	Vivo thinks that we should be a little bit more open, but for progress sake we can focus.

-	Mediatek suggests that we follow what SA2 decides.

-	Xiaomi thinks that we should study other AS solutions like MAC layer.

Resource Allocation

R2-2406610	Discussion on A-IoT Topology 2	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: RAN2 is kindly asked to discuss the resource allocation for topology 2 by considering the following two options:

	Option 1: resource allocation via gNB directly

	Option 2: resource allocation via the intermediate node based on the resource pool allocated by gNB.

=>	Noted

R2-2406652	Topology 2 related aspects	Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1:  	Both in-coverage and out-of-coverage scenarios are studied.

Proposal 3:  	SL-mode-2-like resource allocation is further studied for resource allocation for the UE Reader. SL-mode-1-like resource allocation is not considered.

=>	Noted

R2-2406712	Topology 2 for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2:	Resources for AIOT operations in topology 2 between the intermediate UE and the device are determined/controlled by the gNB (in either option 1 or option 2).

Proposal 3:	For option 1, the gNB sends the inventory and/or command trigger and corresponding resources to the intermediate UE (e.g., using RRC signalling).

Proposal 4:	For option 2, signalling (e.g., RRC) between the intermediate UE and the gNB is required to allow the UE to inform the gNB of a triggered AIOT operation and to provide appropriate operation-related information necessary for AIOT resource allocation.

=>	Noted

Discussion on in-coverage/out-of-coverage

-	Ericsson thinks out coverage shouldn’t be considered and the RRC should always be used.   The reader should always be in connected mode.

-	Qualcomm and Mediatek thinks that it should be NW is in control.  Mediatek thinks that pre-configuration should be considered.

-	CMCC thinks we should support only in-coverage as out of coerage is topology 4 and base line should be gNB direct scheduling (mode 1).  Vodafone thinks that it can be quite often that it becomes out of coverage especially if you are in cell edge.  If the device becomes out of coverage, the reader doesn’t stop.

-	Apple thinks that out-of-coverage is out of scope.  We should consider something like SL mode 1 and mode 2.

-	LG, Huawei and Intel doesn’t think we should consider out-of-coverage.   Huawei thinks that idle is also complicated and pre-configured should be discussed in SA2.

-	Qualcomm thinks that if we if we exclude idle mode then we may develop a solution that won’t work for both.

-	Samsung thinks that we should study connected and in-coverage.

-	Mediatek explains that the scope of the SI covers in-coverage and we shouldn’t exclude the case that Vodafone mentioned on the device going out of coverage while connected.   Vivo thinks that the reader would be configured only in-coverage.

-	T-mobile reminds everyone that RF ID is our competion and they can be read anywhere.

-	Oppo thinks that we should take SL as a baseline.

-	ZTE thinks that coverage is important as that has implication on cell planning and that would require reserving frequencies.

-	Mediatek thinks that as well forward, we will consider extendibility of temporary with full gNB control of resources.

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Not treated

RRC State

R2-2406652	Topology 2 related aspects	Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2:  	The UE Reader can be in any RRC State while performing R2D/D2R activities.

R2-2407074	RAN2 aspects for Topology 2	Nokia	discussion

Proposal 5: For topology 2, RAN2 to study AIoT operation with reader UE in RRC_CONNECTED and RRC_INACTIVE in the time between initial paging till last response. FFS on RRC_IDLE.

R2-2406748	Discussion on topology 2 issues	Spreadtrum Communications	discussion	Rel-19

Proposal 6: In R19, the intermediate UE should be in RRC_Connected state only.

Reader Selection

R2-2407074	RAN2 aspects for Topology 2	Nokia	discussion

Proposal 4: RAN2 to study both options for reader selection before down selecting in WI:

•	Option 1: Readers are selected based on actual radio conditions. FFS whether this includes reader assistance information based on previous device status, keep alive messaging, or discovery procedure.

•	Option 2: Readers are selected by core network based on prior topology knowledge without considering actual radio conditions. FFS fallback options.

R2-2407112	Discussion on Topology 2 related aspects for Ambient IoT	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: For the selection of UE reader, RAN2 wait for more progress in RAN3/SA2 and further discuss the potential impacts if identified by RAN3/SA2.

Intermediate Node Mobility

R2-2407442	Topology 2 aspects of Ambient IoT	Kyocera	discussion	Rel-19

Proposal 6	RAN2 should agree the mobility of UE acting as Reader is supported. The details will be discussed in a normative phase.

R2-2406393	Discussion on Topology 2 related aspects	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

R2-2406406	Considerations on TP2 related aspects between BS and UE reader	Xiaomi	discussion	Rel-19

R2-2406522	Consideration on resource and scenario in topology 2	ASUSTeK	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406543	Discussions on topology 2 related aspects	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406565	Discussion on topology 2 related aspects between gNB and reader	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406657	A-IoT Architecture	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406667	Discussion on Topology 2 for A-IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406720	Discussion on topology 2 for Ambient IOT	OPPO	discussion	Rel-18	FS_Ambient_IoT_solutions

R2-2406785	Topology 2 and DO-A for Ambient IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2406961	Discussions on topology 2 for A-IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407151	Consideration on TP2 for AIoT	ZTE Corporation, Sanechips	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407291	Discussion on topology 2 considerations of ambient IoT	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2407450	Discussion on topology 2 specific issues for Ambient IoT	NTT DOCOMO, INC.	discussion	Rel-19

## 8.3	AI/ML for Mobility

(FS_NR_AIML_Mob; leading WG: RAN2; REL-19; SID: RP-240082)

Time budget: 2 TUs

Tdoc Limitation: 3 tdocs

### 8.3.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2406309	Text proposal of 38.744	OPPO	draft TR	Rel-19	38.744	0.0.3	FS_NR_AIML_Mob

=>	The TR is endorsed and will be used as baseline for further updates

### 8.3.2	RRM measurement prediction

#### 8.3.2.1	Simulation results for RRM measurement prediction

Contributions should focus on initial simulation results on the agreed on prioritized scenarios and agreed assumptions.   Any other results should be submitted in AI 8.3.2.2

Observations from initial simulation results:

R2-2406422	Evaluation on RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

=>	Noted

For FR1 to FR1 intra-frequency temporal domain case B:

Observation 1: With the increase of MRRT from 33.3% to 60%, the average RSRP difference increases at most 1.27dB, and the prediction accuracy for temporal domain measurement prediction case B decreases.

Observation 2: Regarding temporal domain measurement prediction B, even with the same MRRT, skipping pattern (e.g. [400ms,400ms] or [600ms,600ms]) can also impact the prediction accuracy, their average prediction error difference achieves up to 0.6dB.

Observation 3: Regarding temporal domain measurement prediction B, the prediction accuracy of temporal domain measurement prediction with spatial consistency option A is better than that with option B.

Observation 4: As UE speed decreases from 90km/h to 30km/h, the prediction accuracy of temporal domain measurement prediction Case B improves. The average and maximum RSRP difference decrease at most 32.2% and 33.3%, respectively.

Observation 5: For temporal domain measurement prediction case B, even under the same condition, different AI model can bring different prediction accuracy.

Observation 6: The prediction accuracy of temporal domain measurement prediction case B on the best cell is similar as that on the sub-best cell. And the difference between the average RSRP difference for the best cell and for the sub-best cell keeps within 0.14dB.

For FR2 to FR2 intra-frequency temporal domain case A:

Observation 7: With the same observation window length, the prediction accuracy is decreased with the increase of predicted time, and the average RSRP difference increases at most 127.1%.

Observation 8: The prediction accuracy of temporal domain measurement prediction Case A with RRM sub use case 1 and RRM sub use case 2 are similar, the maximum difference is less than 0.5dB.

-	Huawei asks if L1 filtering was done for this case.   L1 filtering may have an impact on the results and should clarify if they do L1 filtering and how they do it.   MEdiatek explains that L1 filtering should be used as baseline.    Mediatek has seen a different outcome that use case 2 is better than case 1.

-	Samsung also agrees that companies should provide L1 filtering method.   Samsung has a similar observation for case 2 showing better results.

-	Vivo thinks that we should provide the real RSRP change.   And case 2 and 3 show better performance.

-	Mediatek asks how to capture all these observations.   ZTE thinks that we should at least capture where we have consensus on the trends.

-	Qualcomm doesn’t see a good explanation for the discrepancy between companies for case 1 and 2.

-	Apple wonders why would we capture this observations as this is UE implementation.  Huawei explains that its good to understand for NW sided model.

-	Xiaomi also doesn’t the need to compare the performance of these usecases.

-	Ericsson thinks that it is too early to capture results as the assumptions haven’t been fully aligned.  Oppo agrees that we should capture only once simulations are stable

-	Oppo thinks that the main cause of the discrepancies are due to how to do the L1 filtering.

-	Huawei thinks that we need to discuss how the model is trained.  Mediatek thinks that this would be a lot of work.  Qualcomm also doesn’t think it matters.

-	CATT thinks that we should discuss the observations by email.

-	ZTE thinks that we should also discuss L3 filtering.  MEdiatek thinks that it is totally UE implementation and there isn’t much point.  ZTE thinks that L3 is specified in RAN4 and we should clarify and companies may have different values.   Oppo agrees with ZTE and there is difference on the frequency

-	Qualcomm thinks that we should add comparison with sample and hold.   MEdiatek agrees that we should have a reasonable benchmark.  Samsung agrees as we agreed to not do any calibration.   Oppo thinks that it is not as simple as sample and hold as there are quite a bit of details on how to do it.  Apple doesn’t think it is needed and same and hold is not a reasonable benchmark.   Mediatek thinks that we need to a benchmark to compare.    Apple thinks that we agreed to compare them with the actual measurements.   Nokia agrees with Qualcomm.   Ericsson agrees with Apple and we have ground truth, real measurements and we can determine the accuracy.

-	Xiaomi thinks that there is a problem with sample and hold as we do filtering so you are adding additional problems.   And the sample you hold is not actual ground truth.   CATT also is not interested in same and hold.

-	Samsung thinks that sample and hold is only applicable to temporal predication and case B

Observation 9: The observation 4~6 for temporal domain measurement prediction case B are applicable for temporal domain prediction case A.

For FR1 to FR1 frequency domain prediction:

Observation 10: The prediction accuracy of full correlation inter-freq shadowing fading model is better than that of no correlation model.

Observation 11: The maximum average RSRP difference of frequency domain prediction with no correlation inter-freq shadowing fading model is only 1.4dB, which is acceptable.

Observation 12: The prediction accuracy of frequency domain measurement prediction is not impacted by UE speed.

For FR2 to FR2 intra-frequency spatial domain measurement prediction

Observation 13: With the MRRS increase from 50% to 75%, the prediction accuracy of spatial domain prediction degrades largely. The average RSRP difference increases 183%.

Observation 14: The prediction accuracy of spatial domain measurement prediction with RRM sub use case 1 is better than that with RRM sub use case 3.

R2-2406579	Evaluation and Simulation Results for AIML RRM Prediction	MediaTek Inc.	discussion

=>	Noted

Preliminary Results for Temporal domain prediction on serving cell and neighbouring cells (Case 2)

Observation 1: The correlation between the discrepancy in predicted versus actual values and the overall system performance does not inherently follow a linear or proportional pattern. Minor prediction errors may not substantially affect the overall system performance, but when the error surpass a certain threshold, the system performance may precipitously decline.

Observation 2: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in speed leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

Observation 3: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), an increase in the measurement reduction ration leads to a decrease in prediction accuracy and an increase in the cell-level RSRP difference.

Observation 4:  For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), among all the sub-cases, the L3 to L3 prediction exhibits the smallest cell-level RSRP difference compared to both the L1 to L1 and L1 to L3 prediction.

Observation 5: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort can be achieved, without any degradation in mobility performance compared with legacy L3 HO in the metrics such as HOF, Ping-pong, data interruption time, and average TOS.

Observation 6: For Case 2 (FR1 to FR1 intra-frequency temporal domain case B), at least for 50% reduction in reference signal and UE measurement effort, cluster-specific model has better system level results than cell-specific model.

Preliminary Results for Frequency domain prediction on serving cell and neighbouring cell (Case 3)

Observation 7: In Case 3, it demonstrates that reducing the frequency domain measurement effort and the need for measurement gaps results in better AI/ML prediction performance under the same beam pattern settings compared to different ones. This suggests that the consistency of beam patterns is essential for AI to achieve accurate predictions.

Consider same beam pattern or explore different beam pattern

-	Vivo thinks that we should stick to the same beam pattern.  ZTE We can consider different beam patterns

-	ZTE thinks that we should explore the non-collocated case.

=>	Stick to the same beam pattern

Need to further discuss simulation assumptions

- UE RX and TX numbers

- For temporal domain prediction to consider some candidates for prediction and observation window

- L1 filtering

- L3 filtering (clarify how L3 filtering is done – e.g. sliding window/non-sliding window)  [CB to see if we can agree to non-sliding window]

- How is model trained and what is used for inference

- Beam level  Filter co-efficient

R2-2406310	Discussion on simulation result of RRM measurement	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: RAN2 to focus on predicting cells with high signal strength, e.g., the serving cell and/or cells with high RSRP values.

-	Samsung agrees.   MediaTek asks how do you know which one is high signal strength if you haven’t predicted yet.   Oppo explains that it is current RSRP not predicted.   CATT thinks that companies are free to report which cells were selected as input and don’t need to restrict.

-	NTT Docomo supports this proposal.   Xiaomi agrees with intention but we need to define how we determine which one is high and in practice the cell’s RSRP changes dynamically.     Ericsson thinks that this is the case we are interested.

-	Apple and Nokia think that this is what companies will do anyways.

Proposal 2: Inter-frequency prediction considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell. Only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h.

-	Ericsson has seen a difference with speeds when distance between frequencies is large.   Nokia agrees with the direction but we can consider more speeds.

-	Samsung asks if this would mean that we simulate both prediction.

Proposal 3: For intra-frequency temporal domain case B prediction, no need to align detailed patterns and time instances chosen for AI/ML model input and output as long as the basic assumptions, e.g., measurement reduction rate, are aligned.

-	Nokia thinks that at least companies should provide details.

Proposal 4: Companies to use case-specific spreadsheets to collect and report simulation assumptions and results as what TR 38.843 presented.

Agreements

=>	companies are encouraged to considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell, but only low to high is expected.

=>	For the agreed frequencies for inter-frequence case, only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h.  Companies can consider other speeds for other frequencies if they chose to simulate them.

=>	For temporal domain case B prediction the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s).

[AT127][026][AI Mob] Simulation assumptions (OPPO)

Intended outcome: discuss simulation assumptions

Deadline:  08-22-24

[POST127][027][AI Mob] Simulation table (Mediatek )

Intended outcome: Agree how to capture simulation results

Deadline:  two weeks

=> Available in R2-2407886

R2-2407781	Summary of [AT127][026][AI Mob] Simulation assumptions (OPPO)	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

3: In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

-	Xiaomi is concerned that this doesn’t work for case B.

=>	Noted

Agreements 1 To keep two filtering options on the table and up to company to report. 2 One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS. 3 In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement 4 continue to discuss following issues in the post email discussion: 1, Further clarification of intra-frequency of temporal domain case A and case B. 2, The set of observation vs prediction window parameters for intra-frequency temporal domain case A and case B 3, The number of TX and RX for FR1 and FR2 4, Filtering co-efficient for beam level prediction

[POST127][030][AI mobility] RRM simulation assumptions (Oppo)

Intended outcome: agree to simulation assumptions

Deadline:  short

=> Available in:

R2-2407849 (Disc, first phase)

R2-2407885 (Disc, second phase)

Not treated

R2-2406401	Simulation results for RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406664	RRM measurement prediction results	Apple	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406665	Field data for RRM measurement prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob	Late

R2-2406703	Discussion on RRM prediction simulation result	Xiaomi	discussion

R2-2406816	Simulation results for RRM measurement prediction	Qualcomm Incorporated	discussion	Rel-19

R2-2406824	Discussions on simulation results for RRM measurement prediction	NTT DOCOMO, INC.	discussion

R2-2406830	Simulation results of intra-frequency RRM Measurement Prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406831	Simulation results of inter-frequency RRM Measurement Prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406860	Discussion on the simulation results for RRM measurement prediction	Samsung Electronics	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406935	Simulation results for RRM measurement prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406936	Discussion on other aspects related to RRM prediction	Huawei, HiSilicon, China Telecom	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406975	Initial simulation results for RRM measurement prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407092	Simulation results for RRM measurement inter-frequency predictions	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407219	Simulation results for RRM measurement temporal prediction	Ericsson	discussion	FS_NR_AIML_Mob

R2-2407376	Initial simulation results for RRM measurement predictions	InterDigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407451	Simulation results for RRM measurement prediction	Indian Institute of Tech (M), IIT Kanpur	discussion	Rel-19

R2-2407479	Measurement Reduction based on RRM Measurement Prediction	Nokia, Nokia Shanghai Bell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407484	Intial simulation results RRM Measurement prediciton	CEWiT	discussion	Rel-19	FS_NR_AIML_Mob

#### 8.3.2.2	Other aspects related to RRM measurement prediction

Further input on remaining issues related to RRM measurement prediction.   Any simulation results on non-prioritized scenarios and/or not yet agreed assumptions should be submitted in this AI.

Beam level prediction:

R2-2406423	Discussion on RRM measuremnet prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: To support L3 beam level measurement prediction, and consider the following cases:

Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results.

Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

Proposal 2: Companies are encouraged to provide simulation results for comparing the prediction accuracy of case 1~3.

Proposal 3: Considering large simulation overhead, not all agreed simulation scenario in cell level prediction should be considered in beam level prediction, we can select some of them.

Proposal 4: The simulation for beam level prediction should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A and FR2 to FR2 intra-frequency spatial domain prediction.

Proposal 5: For L3 beam level measurement prediction, consider average L3 beam level RSRP difference and Top-K/1 beam prediction accuracy as KPI.

Proposal 6: RAN2 to discuss and decide which understanding for layer 3 filtering period is correct in the AI-mobility simulation.

=>	Noted

R2-2406402	Other aspects related to RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 9: For RRM measurement prediction, besides Case1-3, a new sub-use case (Case 4) is considered: To directly predict cell-level results and beam-level results based on beam-level results and optional cell-level results.

Proposal 11: For spatial domain prediction, RAN2 to consider how to derive cell-level quality based on predicted beam qualities and measured beam qualities.

=>	Noted

R2-2406966	Discussion on other aspects related to RRM measurement prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: The temporal domain, spatial domain and frequency domain can be applied for L3 beam level results prediction.

=>	Noted

Discussions

-	Huawei agrees with all cases thinks we should agree to the filter co-efficient.  Ericsson thinks that the workload could be high so the companies can select.   Apple, Nokia and CATT thinks that this isn’t the highest priority.

-	ZTE is proposing to focus on FR2 cases.  Docomo and Samsung agrees to focus on FR2.

-	CATT Thinks that it should only be applied to case 1 and 3.

-	Nokia thinks that we should first finish L3 results.

-	Qualcomm thinks that it should be inter-cell and what is the benefit of doing this now.   The higher priority should be other cases like event prediction.  ZTE explains that this is still intra-cell prediction.

-	Interdigital thinks that for case 1 we should compare the accuracy of beam level prediction and cell level prediction.

Discussions on system level performance results

-	Apple is open to look at something reasonable like handover failure rate to understand how measurement accuracy impacts it.  Huawei agrees but we can do it after.

Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

Cluster based approach:

R2-2406385	Discussion on AI/ML based RRM measurement prediction	China Telecom	discussion	Rel-19\

=>	Noted

R2-2406580	Cluster-based Approach for RRM Prediction and Other Aspects 	MediaTek Inc.	discussion

Proposal 3: RAN2 support to study the cluster-based approaches in each RRM prediction use case.

=>	Noted

Discussion

-	Docomo agrees that we should consider cluster-based approach but don’t need to go into details.  Interdigital thinks that this is an implementation details and we just need to agree that it can be allowed, but how companies do it doesn’t have to be specified.

-	Lenovo thinks we need to discuss which cells should be part of the cluster and how much correlation.

-	Ericsson thinks that due to workload we should just leave it up to companies if they do it.

-	ZTE thinks that for NW side model it would impact data collection.  Ok to simulate but we have to clarify how many cells we consider in simulation, we should consider only 3.  Nokia doesn’t think this is needed for NW side model.

-	Huawei thinks we need some reasonable assumption and exclude inter-frequency.

-	Apple thinks that using multiple measurement as input this is a already allowed.  The output part seems to be model generalization.  Nokia agrees with apple.

-	CATT this is up to companies to select number of input and output

=>	it is up to companies to select the number of cells for input and output (companies should clarify what they are using for cluster-based approach).  Cluster-based approach evaluation is optional and lower priority for now.

Input/Output:

Not treated

R2-2406499	AIML mobility RRM measurement prediction 	NEC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407287	Discussion on RRM Measurement Prediction Framework	Meta Ireland	discussion

Proposal 1:	For RRM prediction case 1 and 3, model input should consider filtered L1-RSRP with standardized and configurable filter coefficients.

Proposal 2:	For RRM prediction case 1, the model output should be filtered L1 RSRP and the data for model training (set A of beams) should apply the same L1 filter as the model input (set B of beams)

FFS: If L3 filtering of model output needs to be aligned for consistency.

Proposal 3:	For RRM prediction case 2/3, the model output may be filtered L3 RSRP.

R2-2407359	Discussion on AI aided RRM measurement prediction	HONOR	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: Discuss following model input/output for RRM measurement prediction:

Model input:

- Measured RSRP for SSB/CSI-RS (FFS L1 or L3 measurement)

- Pattern information:

- Time instances

- SetA/SetB pattern, i.e., Measured serving/neighbor cells vs predicted intra/inter-frequency neighbor cells

- prediction window

Model output:

- Predicted L3 RSRP for time instances

- Predicted average L3 RSRP of the configured prediction window

- Predicted L3 RSRP for time instances for Top-K neighbor cells

KPIs:

R2-2406402	Other aspects related to RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: System-level performance evaluation is needed to reflect the impact on mobility performance after RRM measurement prediction is introduced for both study goals.

Proposal 3: The system-level performance KPIs for RRM measurement prediction include Ping-pong HO rate, short ToS rate, HOF rate, RLF frequency, handover interruption.

R2-2407480	Mobility Optimization based on RRM Measurement Prediction	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: RAN2 should consider at least a selection of the following common mobility related KPIs as discussed in TR 36.839 as a starting point for the metrics evaluated in each use case:

•	HO performance: total number of HOs, unnecessary HOs, Ping-Pong HOs, etc.

•	Failure performance: total number of HO failure, RLF, percentage of failure (RLF+HOF)/(HO+ RLF+HOF) in %, etc.

•	HO timing performance: HO interruption time, Time of outage (ToO), time of staying (in a cell, in a beam, in a UE panel), etc.

•	Measurement performance: measurement reduction, measurement accuracy (absolute and relative), CDF of measurement difference, etc.

•	QoS related KPIs such as downlink/uplink throughput, latency

Proposal 3: RAN2 should discuss incorporating system-level simulations into the study to better understand what actions should be taken by the network in response to predictions, as well as the effects that prediction errors and the distribution of predictions have on the system KPIs.

R2-2406966	Discussion on other aspects related to RRM measurement prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 6: RAN2 consider the performance evaluation metrics/KPIs (Ping-pong HO rate, HO failure rate, RLF rate, short stay rate, HO successful rate), and discuss the baseline threshold for evaluation, such as the HO successful rate above a threshold (e.g. 95%) or the degrading of HO successful rate below a threshold (e.g. 3%).

R2-2406308	Discussion on cluster based RRM measurement prediction	BJTU	discussion

R2-2406311	Discussion on open issue of RRM measurement use case	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406500	AI/ML RRM measurement prediction	TCL	discussion	Rel-19

R2-2406568	Discussion on RRM Measurement Prediction	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2406704	Discussion on cell and beam RRM prediction	Xiaomi	discussion

R2-2406759	Discussion on other aspects related to RRM measurement prediction	Spreadtrum Communications	discussion	Rel-19

R2-2406825	Discussion on other aspects related to RRM measurement prediction	NTT DOCOMO, INC.	discussion

R2-2406861	Discussion on temporal domain RRM measurement prediction	Samsung Electronics	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406924	Discussion on RRM Measurement Prediction	Sharp	discussion	Rel-19

R2-2407113	AI-ML based Inter-frequency measurement prediction	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407130	RRM measurement prediction	Lenovo	discussion

R2-2407377	Other aspects of RRM measurement predictions	InterDigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

### 8.3.3	Measurement event predictions

Contributions should focus on measurement event prediction use cases/scenarios to focus during the study and relevant performance metrics/KPIs to evaluate

This AI will not be treated this meeting

Not treated

R2-2406344	Target scenarios for measurement event prediction	NEC	discussion

R2-2406796	Further Discussion on AI based Measurement Event Prediction	Continental Automotive	discussion	Rel-19

R2-2407360	Discussion on AI aided measurement events prediction	HONOR	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407541	Discussion on measurement event predictions	III	discussion

### 8.3.4	RLF/HO failure prediction

#### 8.3.4.1	Simulation assumptions and evaluation methodology for RLF failure prediction

Contributions should focus on discussing RLF specific methodology and simulation assumptions (addressing the differences or additional aspects from RRM predicution asssumptions).

FFS issue on RLF definition and relevant metrics discussions should be submitted to this AI.

No evaluations/simulation results expected for this meeting

Simulation parameters and assumptions:

R2-2406937	Discussion on simulation assumptions and evaluation methodology for RLF failure prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

=>	Noted

Proposal 1: Reuse the simulation assumptions listed in Table 1 as much as possible.

=>	Agree to use the table as a baseline.  Companies can chose the UE speeds and UE distribution they use and report it.

-	Vivo thinks that the UE speed should be discussed and highspeed should be considered

-	Samsung would like to add traffic model as that has impact on SINR.  Oppo thinks that the real question is how we simulate interference, we should have some simple.  Samsung doesn’t want to make anything complex and perhaps we can consider full buffer.

-	Qualcomm asks if we should explore blockage models.  Mediatek thinks that would be a huge effort.

-	Mediatek thinks that in addition to full buffer we should assume that all the cells are fully loaded (e.g. 100% loaded).  Interdigital agrees and thinks we should assume no HO.

-	CATT thinks that this is linked whether we consider HO failure.

-	Samsung thinks that we have to train the model with HO, it doesn’t make sense we can train a model assuming no HO and then we use it for HO performance KPIs.

-	Qualcomm thinks that simple model for beam schedule

-	ZTE thinks that companies should chose whether to simulate blockage.

Proposal 2: The recommended values in TR 36.839 can be considered as the baseline, including:

    - Monitoring windows of Qout and Qin are 200ms and 100ms, respectively;

    - The sample period is 10ms;

    - The values of Qout and Qin are -8dB and -6 dB, respectively.

-	Catt and apple thinks that the sampling period it should be aligned and be 20ms

Proposal 3: For RLF prediction, RAN2 can start to evaluate the simulations using indirect prediction as a baseline.

-	Apple thinks we should used direct as a baseline or both.  Samsung and Qualcomm think it is reasonable to start with indirect first given that we have already RRM results.  For direct it is unclear and it would increase overhead without much gain in prediction.  Apple thinks that we have to use SINR so we can’t reuse and direct would be much simpler.   Nokia agrees with Apple.

-	Huawei explains that indirect SINR as an input and output SINR and calculate whether RLF can happen with direct you can predict RLF.  Mediatek thinks that interference prediction is very difficult so not sure if we can really do if we use SINR.   Qualcomm agrees with mediatek.

-	Interdigital for direct one, some inputs are N310 and N311 and T310 or different combination values.  Ericsson thinks that this should be the current values.

-	ZTE thinks that the timer and counter are used as input for the indirect case.  For direct case there are other magic inputs (e.g. trajectory etc).   It is a bit hard to model direct case in simulation.

-	Nokia thinks that the only difference if we are using predicted SIRN or actual SINR.   Samsung thinks this is UE implementation and we shouldn’t spend time discussing inputs.   The only difference is that for indirect we would use the RRM prediction results for direct we would have to use another model.

-	Xiaomi, Ericsson and Oppo think that the output the is not the same.  For indirect output is time instance the RLF occurs.   Huawei agrees that the output is not same, probability of RLF in direct case and other case we predict SINR.

-	Interdigital thinks that the simulation assumptions would be different for direct case.

-	Apple asks how we are going to use the time instance the RLF occurs.   Ericsson thinks that the UE can sent a report to indicate that RLF is about to happen but we can discuss that later.

-	Nokia asks whether we will get a confidence score for indirect.  Samsung thinks that confidence level doesn’t make sense.  Apple thinks that we can’t have prediction without confidence.   Qualcomm and Lenovo agrees we should have a confidence score for the predicted time instance.

-	Oppo and CMCC thinks confidence is not need as we don’t have it for RRM.

-	Huawei thinks that we are mixing KPI with output.  For indirect we would evaluate if the RLF would happened at that instance and if we predicted it correctly.   ZTE thinks the time difference of true time RLF and predicted RLF would be the KPI.

-	Qualcomm asks how useful is this KPI.  Xiaomi thinks this is an important KPI to understand how accurate our prediction is.   Interdigital indicates that time instance doesn’t make sense as one measurement (i.e. ho failure happens after 5mins) can bias the results.   ZTE thinks that even if the UE reports confident can we trust the UE.   Nokia thinks this would be a RAN4 requirement.

-	Docomo thinks we should have missed prediction rate and false detection.  CATT also agrees if the time difference is larger than a threshold this would be marked as false detection.

-	Ericsson thinks that we should have the SINR difference for indirect and for direct we can have false detection.

-	Docomo doesn’t think that probability is a KPI, it is an outpu.

[POST127][029][AI mobility] RLF Simulation Assumption (Oppo)

Intended outcome: Agreeable simulation assumption

Deadline:  Super long (November meeting)

Agreements

-	Both direct and indirect are allowed.  Companies should indicate what they used and what inputs they are using

-	Output for indirect: predicted SINR.  Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

-	Output for direct: probability of RLF within an window

-	Companies should report the prediction time window they have used in the simulations

Agreements on KPI

For indirect

-	SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction.

Direct

-	missed RLF detection, false RLF detection, F1 score.

-	Whether the RLF will happen based on the following methodology.   Above a probability threshold we assume that RLF will happen.   This is then compared with true RLF.

-	For the time being we don’t need HO procedure simulation in RLF simulation

-	FFS  full buffer and assumption that all the cells are fully loaded. We will not simulate traffic.

-	Simulation results are not expected before February

R2-2406832	Simulation assumptions and evaluation methodology for RLF prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: The UE will be re-dropped during simulation when RLF occurs.

Proposal 3: RAN2 to discuss how to handle HO procedure in RLF simulation:

-	Option 1: Simple HO procedure;

-	Option 2: No HO procedure simulation

-	Option 3: Adjusting RLF parameters with HO procedure

=>	Noted

Not treated

R2-2406312	Discussion on RLF use case	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 11: A sliding window scheme is adopted for both observation window and prediction window

Proposal 12: RAN2 is kindly requested to discuss the two alternatives when a real RLF occurs during the simulation. We slightly prefer option 1 due to its simplicity

Option 1: the simulation stops for this UE and this trajectory

Option 2: the simulation continues after a fixed duration (to simulate RRC re-established). The serving cell may be changed and count on the best measurement result of recovery time point.

Proposal 14: the assumption of T310 and N311 should be set as such that RLF could be a bit easy to happen and a bit hard to cancel to facilitate training and inference

Proposal 15: RAN2 is kindly asked to discuss the detailed values of T310 and N331 so that they can be aligned among companies. Our recommendation: T310 = 100ms, N311=2.

HO

Proposal 8: A hypothetical HO procedure should run in parallel with model for RLF prediction to change PCell.

Proposal 9: The handover parameter i.e. event A3 offset, hysteresis and TTT timer are aligned for simulation

Proposal 16: The recommended value for handover parameters i.e. event A3 offset, hysteresis , TTT timer, and handover preparation and execution time (HPET), the recommended value are: 5db, 0db, 480ms and 80ms

R2-2407211	Simulation assumptions and evaluation methodology for RLF prediction	InterDigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: For the RLF prediction use case, the following simulation parameters can be considered along with the other RRM prediction use case evaluation parameters:

- UE Distribution: A combination of options 1 and 2 in Question 2.3.1.3-2 [4] can be considered, i.e., x% of UEs are uniformly dropped in the cell and (100-x)% of UEs are dropped in circular disk at cell edge.

- UE Speed: Consider higher speed UEs (e.g., 60 kmh, 90 kmh).

- UE Trajectory: Consider option 3 in Question 2.3.1.1-1 [4] (i.e., “random direction, straight line trajectory”), with inter-cell handover or beam switching enabled for fair comparison with non-AI baseline.

Input/output:

R2-2406581	Simulation assumptions and evaluation methodology for RLF failure prediction	MediaTek Inc.	discussion

Proposal 4: For direct prediction, AI predicts whether RLF events will happen in a given prediction region, i.e. a temporal window within which AI is used to predict the likelihood of an RLF event occurring. The predicted distance and the length of the prediction region could be FFS according to the application and other configurations, e.g., T310 timer.

Proposal 5: For direct prediction, the AI model input is SINR of all cells and AI model output is the probability (or YES/NO flag) of any RLF event occurring at the given prediction region.

Proposal 6: For indirect prediction, AI predicts the serving cell SINR. RLF can be detected based on the continuous predicted SINR. The predicted distance is FFS according to the applications and other configurations, e.g., T310 timer. The AI model input is the SINR of all cells and AI model output is the predicted serving cell SINR.

R2-2407093	AI/ML based RLF predictions	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1	Measured and/or predicted SINR of the cell and running values of T310, N310 and N311 can be used as input to direct RLF prediction model.

Proposal 2	Additional input to the direct RLF prediction model is FFS.

Proposal 3	Expected time of RLF is one output from direct RLF predictions.

Proposal 4	The output of indirect RLF predictions is the predicted time of the RLF and the RLF probability.

R2-2406813	RLF prediction and RLF prediction reporting	Qualcomm Incorporated	discussion	Rel-19

Proposal 1. In simulation evaluation, the AI/ML model for Indirect RLF prediction, which is based on the temporal domain source cell predictions of RRM measurements, produces as output one or more of the following:

- Prediction regarding whether RLF will occur in a certain window of time into the future (the prediction window).

- Predicted time of occurrence of RLF.

- Predicted times of occurrence of RLF and associated probabilities.

E.g., Probability of occurrence of RLF is 10% 100ms into the future, 20% 150ms into the future, etc.

KPIs:

R2-2406976	Discussion on Simulation Assumption and Methodology for RLF prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 8: The following performance metrics/KPIs could be considered for RLF prediction:

    - The performance metrics/KPIs defined for RRM measurement prediction can be reused for indirect RLF prediction, e.g. the SINR difference to the actual measurement

    - The prediction accuracy for the occurrence of RLF, e.g. Precision, Recall or F1-score

R2-2406705	RLF  prediction simulation assumption and evaluation methodology	Xiaomi	discussion

Proposal 10: If the AI output is RLF/no RLF within a time window, the positive and negative can be defined by whether real RLF/no RLF occurs within the time window.

Proposal 11: If the output is RLF/no RLF at a time instance, e.g. indirect prediction, the positive and negative can be defined whether real RLF/no RLF occurs at the time instance.

Proposal 12: Time difference between real RLF and predict RLF can also be used to further evaluate the accuracy.

R2-2406663	On RLF prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 5: consider additional KPIs (beyond RLF prediction accuracy and prediction window) once it has been established that AI/ML is capable of predicting RLF with sufficiently high accuracy and sufficiently long prediction window.

R2-2406343	Simulation assumption for RLF prediction	NEC	discussion

R2-2406403	Simulation assumptions and evaluation methodology for RLF prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406424	Discussion on simulation assumption for RLF prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2406826	Discussions on simulation assumptions and EVM for RLF predictions	NTT DOCOMO, INC.	discussion

R2-2406885	Discussion on RLF Prediction	Lenovo	discussion	Rel-19

R2-2407071	AI-ML based RLF/HO failure prediction	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407289	Evaluation Assumptions for RLF/HO Failure Prediction	Meta Ireland	discussion	Rel-19

R2-2407389	Discussion on simulation assumption for RLF prediction	KDDI Corporation	discussion	Rel-19

R2-2407481	RLF Prediction Aspects	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407492	Discussion on simulation assumptions and evaluation methodology for RLF prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

R2-2407514	RLF prediction result for indirect case	LG Electronics France	discussion	FS_NR_AIML_Mob

#### 8.3.4.2	Other aspects related to RLF/HO failure prediction

No contributions expected for this meeting

Including definition of RLF and HO prediction sub use cases, scenarios, metrics/KPIs, prioritizations etc.

R2-2406313	Discussion on RLF use case	OPPO	discussion	Rel-19	FS_NR_AIML_Mob	Withdrawn

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID: RP-240801)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, including workplan, etc.

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

LP-WUS configuration

(Whether dedicated configration is needed or not)

R2-2406787	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion

Proposal 1: RAN2 confirms that the network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE.

Proposal 3: The network indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality, the detailed signalling is FFS.

Discussion

P1

- OPPO agree with the intention. OPPO understand that P1 means no signalling for this but NW implementation can achieve sth.

- Xiaomi, LG E, CATT think no need to discuss P1 as it was already agreed in SI phase.

- Sony see some benefit for NW to know this info.

- Nokia think NW has some way to know in some cases.

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

P3

- Xiaomi do not think this is needed as UE can know from SI.

- ZTE support the idea of having per UE signalling. LG E think per UE activation/deactivation is useful, for example for some UE with high traffic the power saving gain from monitoring lpwus is limited.

R2-2407240	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

Proposal 1: Support LP-WUS configuration via system information (e.g., cell-specific). Do not support dedicated LP-WUS configuration

Noted

Discussion

P1

- Vodafone think we need to have the possibility of using dedicated signalling to override the cell level configuration.

- QC want to first understand what dedicates signalling needs to be introduced.

- NEC think at least entry/exist condition can be dedicated configuration.

- Samsung support P1, since this is for idle/inactive and think there is no need to introduce new configuration in RRCRelease.

- Nokia think dedicated signalling is useful.

- Vivo think we can postpone the discussion and we can conclude after more discussion on what needs to be provided.

(Whether configuration of entry/exit condtion is mandatory or not)

R2-2407156	LP-WUS operation in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

Proposal 1	The configuration of the entry/exit condition of LP-WUS monitoring is mandatory present in the LP-WUS related configuration.

R2-2407096	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Proposal 2	If the gNB does not configure the conditions, it is left to UE implementation to determine whether to enter or exit LP-WUS mode.

R2-2407396	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core

Proposal 1	For OFDM-based WUR the entry/exit condition is optional.

Proposal 2	For OOK-based WUR the entry/exit condition is mandatory.

Proposal 3	Separate entry/exit conditions can be configured for OFDM-based and OOK-based WUR.

Discussions on the above proposals

- HW do not think the configuration should be mandatory, and think if not configured it can rely on default values. CATT agree, for PEI the condition is not configured, and think Ericssion proposal is good compromise way to go.

- Xiaomi support LG E P1, and think LP-SS/LP-WUS may not be full coverage of the cell. Nokia support LG E proposal and also ok with Ericsson proposal. Lenovo, NEC also agree with LG E P1.

- LG E think even for OFDM-based LR we need to consider inter freq case. NEC agree.

- Lenovo agree with P3 in Ericsson paper and think it is already a working assumption in R1.

- ZTE want to the condition is optional and think HW proposal is a good compromise.

- Samsung prefer to have the condition optional, and think we need to discuss whether different types of LR can be differentiated. OPPO do not think the condition is mandatory and do not need default value.

- Vodafone think in some case NW do not really know the coverage and think different UE may have different measurement. Vodafone prefer to have separate conditions for different LR types.

- NEC support P3 from Ericsson paper.

- Ericsson think LPWUS should not impact system performance.

Discussion on P3 from R2-2407396

- Nokia suggest to put FFS on detailed signalling. Vivo think it is already R1 assumption, and suggest to add ‘if NW support both types of LR’.

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

LP-WUS entry/exit condition

R2-2407013	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

Proposal 4: LP-WUS monitoring entry condition includes both MR (cell coverage) and LR (LP-WUS coverage) measurements.

Proposal 5: LP-WUS monitoring exit condition is evaluated based on LR measurement.

Proposal 6: RAN2 assumes that RAN1 defines LR measurements quantities for LP-WUS.

R2-2406730	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

Proposal 2: The entry condition for LP-WUS monitoring is not based on measurement from LR.

R2-2406427	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

Proposal 3: Regarding whether any measurement via LR is needed for entry condition and whether any measurement via MR is needed for exit condition, RAN2 to wait for RAN1/RAN4 progress.

Discussion on P4 in R2-2407013 and P2 in R2-2406730

- NEC do not think LR results are needed for entry condition. HW, Lenovo agree.

- Sony think LR results are needed for entry condition, since there might be different rx sensitivities at the UEs. QC agree.

- Spredtrum think it depends on NW configuration. IDT share this view. IDT, OPPO think R1 has similar understanding and think we need to check the R1 progress. HW agree. Ericsson think we can conclude in R2.

- Ericsson think LR measurement is useful in some cases, and think if we only allow MR based results then NW may be forced to configure higher thresholds. ZTE also think LR results are needed. LG E, CATT, Xiaomi agree.

- Vodafone think the entry condition is mainly for the case when UE is in very good coverage, so LR results may not be so useful, and think Pingpong can be handled by offset btw entry/exit thresholds. Samsung share this view. HW agree.

- Samsung, Apple think we can postpone.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

Proposal 4: The metric for serving cell quality measured by MR for entry condition includes RSRP and optional RSRQ.

Proposal 5: RAN2 assumes the metric for serving cell quality measured by LR for exit condition includes RSRP and optional RSRQ. An LS is sent to RAN1/RAN4 for confirmation.

Discussion

P4 and P5 in R2-2406427

- IDT, Nokia fine with P4 and P5.

- Xiaomi think these metrics should be decided in R1/4. Vivo think these metrics have already agreed there will be LP-RSRP/Q kind of measurements, and think R2 can decide to use them.

- Nokia, Ericsson, ZTE, NEC do not see a need to send LS.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

LP-WUS Subgrouping

R2-2407543	Discussion on Procedure and configuration in RRC_IDLE-INACTIVE	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

Proposal 1: 	In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups differently in order to reduce the false paging rate.

R2-2406772	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

Proposal 8	For LP-WUS, UE can be assigned by CN a different subgrouping ID than the subgrouping ID for PEI.

Proposal 9	For LP-WUS, RAN can configure a different subgrouping number used in UE ID-based subgrouping than the subgrouping ID for PEI.

Discussions

- HW, NEC, Xiaomi agree with DCM P1. NEC think this same principle may also apply to CN based.

- HW think ‘different subgrouping number’ in OPPO P8 is not so clear. OPPO clarifies this means the formula may be the same but exact parameters can be different.

- Xiaomi think we can further discuss details of the formula.

- LG E think it is possible to achieve the same effect configure different # of subgroups.

- Ericsson think we need to first discuss in R2 whether PEI and LPWUS are used together for a UE, and think it is controlled by the NW. CATT and HW think there is already agreement in R1.

- ?? In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups independently and separately (if UE is configured to use both PEI and LP-WUS and if UE support both) in order to reduce the false paging rate. FFS on detailed configurations.

R2-2406447	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2406495	LP-WUS procedure in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2406575	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2406585	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2406617	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2406753	Discussion on LP-WUS operation in IDLE/INACTIVE mode	Spreadtrum Communications	discussion	Rel-19

R2-2406802	Discussion on entry exit conditions for LP-WUS monitoring	Sharp	discussion

R2-2406900	LP-WUS Operation in RRC_IDLE/INACTIVE	China Telecom	discussion	Rel-19	NR_LPWUS-Core

R2-2406985	LP-WUS operation in IDLE/INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2407127	Procedure and Configuration of LP-WUS in RRC Idle/ Inactive	Lenovo	discussion	NR_LPWUS-Core

R2-2407310	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2407357	Procedure of LP-WUS in RRC_IDLE and INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

### 8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

General aspects

R2-2406496	LP-WUS RRM measurement 	NEC	discussion	Rel-19	NR_LPWUS-Core

Proposal-1: Only LP-WUS capable UE is applicable to LP-WUS specific measurement offloading/relaxation, i.e., it is not applicable to normal UE.

Proposal-2: RAN2 to confirm that non-LPWUS-capable UE is not the target of this WI, e.g. the RRM measurement relaxation is applicable only for LPWUS-capable UE.

Proposal-3: RAN2 to confirm that as long as the UE is in LP-WUS monitoring mode, the UE always need to use LR to perform measurement on serving cell.

Proposal-4: RAN2 is suggested to further consider fully offloading case (no any measurement by MR) and relaxed case b (both relaxed serving cell and neighbour cell measurement by MR) identified in RAN4.

Discussion

P1, P2

- QC think R4 is discussing the scenarios, so R2 do not need to discuss this. CATT think in R4 the high level assumption is the same as the proposals.

- HW, Apple, Nokia, Samsung support P1, P2.

- Xiaomi not sure about the relaxation case and want to wait for R4.

- Vivo explain there are two cases agreed in R4, full offloading and serving cell relx with LR measurements, so we only need to focus on these cases (no need to agree on P1, P2.).

- Ericsson think it is already clear based on WI scope.

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

Serving cell measurment offloading (i.e., there is no serving cell measurement by MR)

R2-2406754	Discussion on RRM measurement relaxation and offloading in IDLE/INACTIVE mode	Spreadtrum Communications	discussion	Rel-19

Proposal 1: UE serving cell RRM measurement can be offloaded from MR to LR with the follows conditions:

	Option 1: when signal quality of serving cell is greater than a threshold.

	Option 2: when entry condition of using LP-WUS is satisfied.

R2-2406803	Discussion on RRM measurement offloading and relaxation	Sharp	discussion

Proposal 1: The entry/exit condition(s) for LP-WUS monitoring is also applied for serving cell RRM measurement offloading from MR to LR.

Discussion

- Xiaomi support Sharp proposal to use the same conditions for LPWUS monitoring and RRM measurement offloading. LG E, CATT, QC, Nokia think we do not need to link these two together. QC think there are different UE capabilities.

- Apple support P1, and think it is only one of the condition, other aspects e.g., neighboring cell measurement can also be consider. Ericsson can accept separate conditions but think the power saving gain is very limited if the conditions are different for LPWUS monitoring and measurement offloading. ZTE share the understanding from Ericsson.

R2-2406970	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

Proposal 2: Serving cell measurement relaxation/offloading of MR cannot affect the performance of cell (re)selection by UE.

Proposal 3: The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered if it is on at this time.

Proposal 4: The RSRP threshold can be considered in conjunction with the threshold for LP-WUS monitoring. That is, the two thresholds can be the same or offset by a predetermined value.

Proposal 5: The exit condition is based on the LR measurement results.

Proposal 6: According to the 'Fully offloading case' agreed by RAN4, the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements.

Proposal 7: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 8: RAN2 discuss how to deal with the measurement loss during the hundreds of milliseconds required to restart MR.

Discussion in CB on P3-5 in R2-2406970

- NEC think the conditions for serving cell meas offloading should be tighter than relx.

- Lenovo, OPPO, Samsung, CATT generally fine with P3 and P5.

- OPPO wonders what are the metrics for the thresholds.

- Samsung want to keep it open for other possible conditions. Nokia not sure what are those.

- Apple wonders what is the impact with P3 on neighbouring cell measurement, and think it is possible that it is not new condition but it reuse the exiting condition. Nokia not sure whether existing conditions can be used.

- Ericsson do not agree to add ‘FFS to other conditions’.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

R2-2406285	RRM measurement relaxation and offloading in RRC_IDLE or RRC_INACTIVE	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2406428	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2406448	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2406576	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2406586	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2406618	Discussion on RRM aspects for  LP-WUS/WUR	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2406731	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core	Late

R2-2406739	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE mode	China Telecom	discussion

R2-2406767	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2406882	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

R2-2407014	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2407098	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2407157	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2407241	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2407311	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2407397	LP-WUS and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core

### 8.4.4	Procedures for LP-WUS in RRC_CONNECTED

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring.

Understanding of Option 1-2-1 and 1-2-2

Discussion

-	Rapporteur suggest that R1 already made agreements during the week and think we should take into account those progress.

R2-2406587	Discussing on LP-WUS monitoring for RRC_Connected	Xiaomi Communications	discussion

Proposal 1	For Option 1-2-2, PDCCH monitoring not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS will not be supported.

R2-2406449	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Proposal 2: Option 1-2 is supported, e.g. UE can monitor LP-WUS with a LP-WUS cycle during the non-active time of C-DRX cycle if LP-WUS for RRC_CONNECTED state is configured and activated.

Proposal 2a: UE starts drx-OnDurationTimer after a timeOffset when it detects LP-WUS.

Proposal 2b: UE determines the LP-WUS monitoring occasion with the similar formula as that to determine the start of drx-onDurationTimer, e.g. the UE monitors LP-WUS during the non-active time of C-DRX cycle if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP_WUS-cycle)) = 0

R2-2406577	Analysis on LP-WUS for RRC_CONNECTED Mode	CATT	discussion	Rel-19	NR_LPWUS-Core

Option 1-2-1:

Proposal 6: RAN2 confirms the following understanding for Option 1-2-1:

-	The UE is triggered to perform PDCCH monitor based on LP-WUS indication during the time outside legacy DRX active time;

-	 UE behaviors related to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

Option 1-2-2:

Proposal 8a: The UE is triggered to perform PDCCH monitor based on LP-WUS indication during the time outside legacy DRX active time and the time during legacy drx-onDuration in option 1-2-2.

Proposal 8b: For option 1-2-2, UE behaviors related to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL are not impacted by LP-WUS.

Proposal 9a: For option 1-2-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. The timer can be new defined timer or legacy drx-inactivityTimer.

Proposal 9b: For option 1-2-2, the timer is started at a configured time offset after LP-WUS. The range of time offset is left for RAN1.

Proposal 9c: For option 1-2-2, (long) DRX command MAC CE can be used to stop the timer. The other indications, e.g. DCI or LP-WUS, are dependent on RAN1.

Proposal 10: For option 1-2-2, whether to enhance CSI reporting during legacy drx-onDurationTimer depends on RAN1.

Proposal 11: For option 1-2-2, whether the legacy drx-onDurationTimer is started or not can wait for further inputs from RAN1, especially progress on CSI reporting during drx-onDurationTimer.

Discussion

- Nokia pointed out that in R1 Option 1-2-1 and 1-2-2 are merged. Nokia suggest to discuss whether UE monitor LPWUS when onDurationTimer is running.

- Nokia suggest to disucss CSI measurement as well. Ericsson also think we need to consider CSI measurement. LG E has concern on 1-2-2 in terms of CSI measurement, as it consumes UE power, and it depends on how DRX configuration is configured. QC think CSI measurement in connected state needs no enhancement, and there is no discussion on it in R4.

- Lenovo think R2 can discuss the timer for PDCCH monitoring trigger by LP-WUS. Apple agree.

- ZTE suggest to discuss P9a.

P9a

-	ZTE think there may be three options. Nokia think we do not consider onDurationTimer, slightly prefer a new timer. Ericsson think with Option 1-2 we introduce a new timer, and think if UE receives PDCCH when this new timer is running, UE starts the legacy drx-inactivityTimer.

-	LG E suggest think we can just say new timer or existing timer. Xiaomi agrees.

-	QC think new timer is clear solution as that avoids impact to legacy procedure.

-	vivo suggest we confirm in R2 that a new timer is introduced. Lenovo, Apple agrees.  ZTE not ready for that and want to keep the wording legacy timer in the bullet.  Xiaomi suggest to first clarify the UE behaviour when the new timer expires. Nokia think if the timer expires, UE go back to monitoring LPWUS. Apple agree with this understanding.

-	HW think we can use legacy timer.

P9b

-	HW suggest to remove ‘configured’.

-	Nokia wonders whether the offset is the wake up time needed for UE. CATT think this proposal is only about the procedure.

For option 1-2,

    - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

    - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

R2-2406429	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2406497	LP-WUS procedure in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2406619	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2406717	Discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2406732	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2406768	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2406901	Discussion on LP-WUS in RRC_CONNECTED	China Telecom	discussion	Rel-19	NR_LPWUS-Core

R2-2406978	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2407097	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2407134	LP-WUS in RRC Connected Mode	Lenovo	discussion	NR_LPWUS-Core

R2-2407242	Discussion on LP-WUS operation in RRC_CONNECTED mode	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2407286	Discussion on Procedures for UE MR PDCCH monitoring triggered by LP-WUS in RRC_CONNECTED Mode	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2407312	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

R2-2407358	Discussion on LP-WUS in RRC_CONNECTED	HONOR	discussion	Rel-19	NR_LPWUS-Core

R2-2407398	LP-WUS in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core

R2-2407406	LP-WUS in RRC_CONNECTED	Nokia	discussion	NR_LPWUS-Core

R2-2407512	Discussion on LP-WUS in RRC_CONNECTED	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID:  RP-241650 )

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Including incoming LSs and rapporteur inputs.

- R19 NES CR rapporteurs:

RRC: Ericsson

MAC: IDC

38.300: Huawei

38.304: Apple

38.306: Intel

### 8.5.2	On-demand SSB SCell operation

RAN2 spec impacts and high-level solutions.

Scenarios on OD-SSB:

P1 in R2-2406425 (Xiaomi)

Proposal 1: RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

- Agreed.

OD-SSB transmisson MAC CE:

P4, P5 in R2-2406721 (Vivo)

Proposal 4: RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- Agreed.

[Xiaomi]: Understand RRC can be used for activation/deactivation even after initial activation. [Huawei]: Agree with Xiaomi. [Apple]: Agree with P4. It is similar to what we already have for cell DTX activation. [OPPO]: Agree with Apple. If RRC is used for dynamic activation/deactivation purpose, RRC and MAC are introduced for the same purpose. In general, MAC CE is used for dynamic activation/deactivation. [Nokia]: In initial configuration, 1bit would be included. However, that can be changed/reconfigured later. We don’t need any restriction.

Proposal 5: MAC CE based OD-SSB transmission indication is used to activate the OD-SSB when the SCell is in deactivated (i.e., Scenarios 2) or indicate the activation state of OD-SSB when activating the SCell via MAC CE (i.e., Scenarios 2A).

P2, P3 in R2-2406669 (Apple)

Proposal 2: Following RAN1 agreement, configuration on OD-SSB transmission pattern (e.g. frequency, SSB position and transmission periodicity) doesn’t need to be included in MAC-CE, i.e., it is sufficient to only include 1bit indication of activation / deactivation of OD-SSB transmission on the corresponding SCell in MAC-CE.

- Noted. We need to see more RAN1 progress.

[IDC]: RAN1 agreed transmission periodicity is included in MAC CE. [Xiaomi]: Multiple SSB transmission patterns is under RAN1 discussion. We may need an indication which transmission pattern is used in MAC CE. [Session chair]: Let’s wait for more RAN1 progress.

Proposal 3: RAN2 down select between the following two alternatives of new OD-SSB MAC-CE design:

•	Alt-1: The new MAC-CE is fixed size only to indicate OD-SSB transmission, and NW send both new MAC-CE and legacy SCell activation MAC-CE for Scenario 2A.

o	The Bi field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Bi field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

•	Alt-2: The new MAC-CE is variable size to indicate either only OD-SSB transmission (scenario 2), or joint OD-SSB transmission and SCell activation / deactivation (scenario 2A).

o	The first Oct (i.e. Oct1) is mandatory present and the meaning of Bi field is same as Alt-1.

o	The second Oct (i.e. Oct2) is optional present. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated (i.e. same as legacy SCell activation / deactivation MAC-CE).

[Nokia, NEC]: Minimum need is to define a new MAC CE for OD-SSB transmission indication. [Xiaomi]: For scenario 2, we need the new MAC CE. For scenario 2A, we may consider a merged MAC CE for both the new MAC CE and SCell activation/deactivation MAC CE. [Huawei]: Agree with Nokia and NEC. For scenario 2A, prefer not to introducing additional MAC CE. Prefer having common solution for both 2 and 2A. [IDC, Samsung]: Agree with Huawei. [Ericsson]: Scell index will be included. If we merge OD-SSB transmission indication and SCell activation/deactivation, it can save overhead. [Huawei]: Scell index is not needed for OD-SSB transmission indication. Scell is implicitly associated based on the 1bit location, so no additional overhead as Ericsson mentioned.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

P1 in R2-2406347 (Samsung)

Proposal 1: RAN2 to discuss whether on-demand SSB measurements for L3 measurement (i.e. RRM) are used for

•	Option 1: Case #1 only

•	Option 2: both Case #1 and Case #2

[Apple]: Support case 2. In case 2, would like to include/consider both OD-SSB and periodic SSB. For case 1, extra spec impact will be introduced. [Huawei]: We first decide case 1 since observe that companies have different understanding on option 2. Understanding A) With OD-SSB, periodic SSB’s periodicity is changed. Understanding B) Additional OD-SSB is signalled in addition to periodic SSB. We need to understand which understanding is correct. [OPPO]: Share the view with Huawei. We need to first secure case 1. [Nokia]: Case 2 is more similar to what we already have. Should consider case 2.

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

P2 in R2-2407414 (Sharp)

Proposal 2 (modified): For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- Agreed.

[Xiaomi]: For case 2, the UE anyway needs to perform measurements. How does the UE know whether it is case 1 or case 2. If not known, the UE needs to perform measurements anyway. [Fujitsu]: Agree with intention. [Huawei, Vodafone]: Agree with proposal 2. Otherwise the MR is useless. [Apple]: Agree with P2. The UE can know whether case 1 or case 2 based on the configuration.

Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

[Huawei]: NW configures TTT and NW knows when to start OD-SSB. The problem may not happen. [Apple]: Similar issue happens for measurement on CSI-RS, e.g. when CSI-RS is deconfigured in the middle of TTT running. [Xiaomi]: Similar issue already exists in legacy measurement.

P8 in R2-2406669 (Apple)

Proposal 8: RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

- Agreed.

[OPPO]: Instead of SMTC adaptation, we can consider the UE just measure according to OD-SSB pattern that is configured by NW. [Apple]: SMTC is per frequency (not per cell). OD-SSB transmission pattern is only for the SCell. [Xiaomi]: We may not need any new one, e.g. NW may always send OD-SSB only during SMTC measurement duration.

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

R2-2406266	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406347	On-demand SSB SCell Operation	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406425	Discussion on on-demand SSB	Xiaomi	discussion

R2-2406444	On-demand SSB SCell operation in connected mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406469	RAN2 impacts to enable on-demand SSB SCell	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406620	On-demand SSB Scell operation discussion	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406669	Discussion on RAN2 work of on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406721	Discussion on on-demand SSB SCell operation	vivo	discussion	Rel-19

R2-2406749	Discussion on on-demand SSB Scell operation	Spreadtrum Communications	discussion	Rel-19

R2-2406889	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-19

R2-2406895	Discussion on on-demand SSB	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406954	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406979	Discussion on on-demand SSB	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407002	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407039	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407123	Discussion on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407158	On-demand SSB SCell operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407161	Discussion on On-demand SSB SCell Operation	Qualcomm 	discussion

R2-2407185	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407271	Discussion on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407304	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407414	Discussion on on-demand SSB SCell operation	Sharp	discussion

### 8.5.3	On-demand SIB1

Leftover issues from RAN2#126 and remaining essential issues for study (e.g. need of WUS configuration acquisition from NES cell (if needed, what’s use cases and how it works)?, need of SIB1 validity (if needed, what’s use case and how it works), whether/how to support case 3 defined in RAN1 (e.g. reuse/enhance on-demand SIB request procedure)?, etc.). Note study of OD-SIB1 needs to be concluded

Support of RAN1 case 3 (option 2 + B + Y):

P2 in R2-2406955 (Nokia)

Proposal 2: RAN2 to consider Case 2, i.e. scenario 1a, only for normative work.

P8 in R2-2406980 (CMCC)

Proposal 8: For the scenario of acquiring on-demand SIB1 with only Cell A involved, the following agreements made for Scenario 1a can be reused:

1.	Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

2.	For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

3.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

[Huawei]: Understand the support of case 3 is not complicated. We can reuse the existing mechanism. Understand case 3 is better in energy saving point of view. Support case 3. [Vodafone]: Both case 2 and case 3 supports anchor scenario. It is not good to have multiple options to handle same scenario. [ZTE]: Share the same view as Nokia. Case2 was already decided in earlier time, so we don’t need to introduce another option for the same scenario. [LG, ITRI]: Think case 2 is the baseline, however case 3 can give maximum energy saving gain. Support to have case 3. We can reuse many of case 2 for case 3. Do not expect much specification efforts. [Rakuten]: In case 2, SIB1 is always acquired from a NES cell while in case 3, SIB1 is acquired from a NES cell and cell A. Prefer simple option (case 2). [Xiaomi]: Cell A may need to provide multiple SIB1s, which make case 3 more complicated. [Fujitsu]: Support Nokia proposal. [Session chair]: Let’s check companies views.

- Case 2 only: Vodafone, ZTE, Vivo, Rakuten, Fujitsu, CATT, Google, Ericsson, Xiaomi, IDC, Nokia, DT (12)

- Case 2 and case 3: Sony, Samsung, Huawei, ITRI, LG, Qualcomm, Lenovo, KDDI, DCM, CMCC (10)

[Apple]: We can make a conclusion that case 2 is baseline and case 3 is feasible, but final conclusion can be left to RAN1. [Nokia]: What’s RAN1 impact on RAN1. Case 3 needs more RAN2/3 works. [ZTE]: Based on our RAN1 colleagues, RAN1 waits for RAN2 decision for case3. Suggest to follow slight majority companies’ views. [Ericsson]: Concluding just feasible for case 3 is not sufficient for RAN discussion. Please note we have spent most of time for OD-SIB1. We still have many works for other scopes. [Vodafone]: Concern the information overhead of inter-node information exchange in case3. [Lenovo]: Expect SIB1 does not change often.

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

P5, P6 in R2-2406470 (Intel)

Proposal 5: To re-use validity time concept (which is currently defined for other SIBs) for OD-SIB1. I.e., when Rel-19 NES UE acquires OD-SIB1, this UE uses it as a valid stored version.

Proposal 6: Legacy notification of SI change can be reused for OD-SIB1. NOTE: network is excluded to trigger a notification of SI change due to changes of the H-SFN value.

P3, P4 in R2-2406955 (Nokia)

Proposal 3:  We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

Proposal 4: Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

- P3 and P4 in R2-2406955 are agreed.

P2 in R2-2406670 (Apple)

Proposal 2: SIB1 validity mechanism is supported, i.e., the UE can know when the acquired OD-SIB1 is invalid. Its details can be discussed in normative phase.

[Samsung]: A new timer is not needed for optimization. Current 3 hours existing timer works with value tag and it is not applied to SIB1. [Session chair]: Ask if validity timer means 3 hours existing one or newly introduced timer for SIB1 validity? [LG]: Do we talk about 1) while the UE stays in cell A or 2) while the UE camps on NES cell. [Nokia]: Confirms current 3 hours timer does not work for SIB1. [Nokia]: Understand most companies think that existing mechanism is applied to SIB1.

WUS configuration of NES cell from NES cell:

P2 in R2-2407041 (Ericsson)

Proposal 2: Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

- P2 in R2-2407041 is agreed.

P2, P3 in R2-2406780 (OPPO)

Proposal 2: RAN2 confirms that acquiring the UL WUS configuration from NES Cell is unnecessary.

Proposal 3: The outdated UL WUS configuration stored by the NES UE causes the failure of SIB1 acquisition and eventually requires the UE to go back to Cell A for the latest UL WUS configuration.

[OPPO]: Question is if NES cell only provides its own WUS configuration or if NES cell can provide neighbouring cells’ WUS configuration. [Rakuten]: If NES cell does not do that, the UE needs to back to cell A, which is too much for the UE. [Qualcomm]: W/o WUS configuration of neighbouring NES cells, the UE always need to back cell A whenever cell reselection happens to another NES cell. [Huawei]: Support Ericsson proposal and share the view with Qualcomm.

MSG3 based on OD-SIB1 REQ:

P5 in R2-2406659 (Fujitsu)

Proposal 5: Msg3 based SIB1 request is triggered if dedicated RA resources for SIB1 request is not configured. Details will be discussed in a normative phase.

P3 in R2-2406605 (Huawei)

Proposal 3: Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

- P3 in R2-2406605 is agreed.

[Apple]: Support Huawei’s proposal. We have only single SIB1, so there is no motivation to use MSG3 based OD-SIB1 procedure. If we allow MSG3 based OD-SIB1, it will increase WUS configuration overheads as well. [ZTE]: With MSG3 based OD-SIB1, NES cell needs to monitor MSG3 and send MSG4. MSG3 based OD-SIB1 is not needed. [LG]: Since case 3 seems ruled out, no motivation for MSG3 based OD-SIB1 REQ. [NEC]: WUS contains dedicated RA, so no need for MSG3-based OD-SIB1 REQ.

Access restriction for legacy UEs:

P11 in R2-2406346 (Samsung)

Proposal 11: RAN2 to discuss how to handle Legacy UEs (i.e. UEs not supporting on-demand SIB1).

•	Option 1: Legacy UEs bar the on-demand SIB1 cell based on cellBarred bit set to barred in MIB.

•	Option 2: Legacy UEs bar the on-demand SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

•	Option 3: Network includes cells supporting on demand SIB1 to list of excluded cells

- Above example options can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

[Nokia]: With option1, we need an additional barring bit for NES UE. [Google]: For Rel18 legacy UEs (e.g. RedCap UE), with option1, the UE still needs to check SIB1 to determine whether the cell is barred or not. [Lenovo]: When Kssb has a value indicating there is no SIB1, combined with SIB1-PDCCH CORESET and search space, the UE may try to other frequency. Operator needs to carefully set the configuration. [BT]: With option1, legacy UE may not get an emergency call. Want barring but allowing emergency call for legacy UE in NES cell. [KDDI]: Concern if the legacy UE in the market actually bars the cell according to option2 and option3. [Vodafone]: Legacy UE cannot have even emergency call if the UE cannot get SIB1 in NES cell.

Impacts to RRC connected UEs:

P4 in R2-2406670 (Apple)

Proposal 4: OD-SIB1 doesn’t impact both legacy and Rel-19 RRC_CONNECTED UEs (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

P2 in R2-2406359 (Xiaomi)

Proposal 2 (modified): RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

- P2 in R2-2406359 is agreed.

[Nokia]: With the original proposal 2, it sounds like always no impact to RRC connected UE. There may be some cases. Think this conclusion somewhat risky.

SI conclusion:

P6 in R2-2406670 (Apple)

Proposal 6 (modified): RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

- Agreed.

Agreements on OD-SIB1

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

R2-2406346	On-demand SIB1	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406359	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

R2-2406445	Remaining issues of on-demand SIB1 in idle and inactive mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406470	Remaining details to enable on-demand SIB1	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406569	The procedure for the on-demand SIB1 transmission	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406605	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406621	On-demand SIB1 for IDLE/INACTIVE UEs	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406622	Further study on Case 3 in on-demand SIB1	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406653	Discussion on On-demand SIB1 for RAR	KDDI Corporation	discussion	Rel-19

R2-2406659	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu Limited	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406670	Finalize study of on-demand SIB1	Apple Inc, BT Plc	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406722	Discussion on  on-demand SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19

R2-2406780	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406804	Discussion on on-demand SIB1	Sharp	discussion

R2-2406896	Discussion on left issues of on-demand SIB1	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406955	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406980	Discussion on on-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407003	Consideration on on-demand SIB1 issues	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407041	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407042	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407043	Discussion on On-demand SIB1 procedure and UL WUS configuration	NEC	discussion

R2-2407051	Detection and access of NES cells with OD-SIB1	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407159	 On-demand transmission of SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407183	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407351	Further discussion on on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407438	Remaining essential issues for study	Lenovo	discussion	Netw_Energy_NR-Core

R2-2407455	Discussion on on-demand SIB1	NTT DOCOMO INC..	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407499	On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2407540	On-demand SIB1 for Idle/Inactive mode UEs	III	discussion

R2-2407162	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

### 8.5.4	Adaptation of common signal/channel transmissions

Further consideration of adapation of paging occasions in time domain, legacy UE impact (including barring aspect for paging adaptation), configuration aspect for paging adaptation, RAN2 spec impact and solutions for RACH adaptation and SSB (with consideration of RAN1 progress), etc.

Paging adaptation:

[POST127][109][NES] (OPPO)

Scope: For each direction (N extension vs PF bundling): 1) List proposed options, 2) Discuss to understand each option better, 3) Down select options, 4) Discuss pros and cons for each direction or down-selected options (with the consideration of UE impacts, system impacts and specification job). F2F offline discussion in Brk3 room. => It is extended to long email discussion to continue detailed analysis of pros and cons for each direction.

Intended outcome: Discussion summary in R2-2407598. => Updated discussion summary

Deadline: Comeback in Thursday CB session. => Long email discussion.

R2-2407598	Summary of [109]	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1	Option-a) is about the bundling of PF for R19 NES UEs.

Proposal 3	RAN2 observe that the option-a) and option-b) can be designed to configure the POs at same time position.

- P1 and P3 are agreed.

Proposal 2	For option-a), R2 further discuss whether to allow gaps between bundled PFs.

- Can be discussed as part of long email discussion.

Agreements on paging adaptation

- Option-a) is about the bundling of PF for R19 NES UEs.

- RAN2 observe that the option-a) and option-b) can be designed to configure the POs at same time position.

R2-2406270	Discussion on PO confinement options	OPPO, Samsung, ZTE, Huawei, HiSilicon, Qualcomm	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406348	Adaptation of common signal channel transmissions	Samsung Electronics Co., Ltd	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406360	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19

R2-2406446	Further consideration on paging occasion adaptation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406471	RAN2 impacts to enable adaptation of paging and RACH in time	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406523	Discussion on paging adaptation	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2405428

R2-2406544	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406671	Further discussion on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406723	Discussion on adaptation on common signal transmissions	vivo	discussion	Rel-19

R2-2406750	Discussion on adaptation of common signal channel transmissions	Spreadtrum Communications	discussion	Rel-19

R2-2406866	Discussion on the paging occasion adaptation for NES cell	ITRI	discussion	Netw_Energy_NR_enh-Core

R2-2406890	Paging statistics from field and PRACH adaptation	Lenovo	discussion	Rel-19

R2-2406897	Discussion of adaption of paging occasions	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406956	Common signal aspects of NES WI	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2406981	Discussion on adaptation of common signal channel transmissions	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407004	Consideration on adaptation of common signalchannel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407048	PRACH and paging adaptation	NEC	discussion

R2-2407163	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated	discussion

R2-2407184	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407245	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2405290

R2-2407305	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407352	Discussion on adaptation of common channel transmissions	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2407440	Discussion on RACH adaptation 	SHARP	discussion	Rel-19

R2-2407454	Discussion on Adaptation of paging occasions	NTT DOCOMO INC..	discussion	Rel-19	Netw_Energy_NR_enh

R2-2407486	Adaptation of Common Signals and Channels for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2407520	Discussion on common signal and channel adaptation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh

R2-2407531	Adaptation of common signal/channel transmissions	III	discussion

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-241515)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Including incoming LSs, WI rapporteur inputs, etc.

R2-2406244	Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases (S3-242400; contact: Apple)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN3

- Noted.

R2-2406693	Important topics for resolution for Rel-19 Mob Enh WI (Rapporteur)	Apple Inc, China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

- R19 MOB CR rapporteurs:

RRC on all of inter-CU and conditional LTM: Ericsson

RRC on L1 event-triggered MR: Huawei

MAC: Vivo

37.340: China Telecom

38.300: Apple

38.323 (if needed): ZTE

### 8.6.2	Inter-CU LTM

Remaining open issues and further details of LTM preparation phase (e.g. single or multiple reference configuration(s)?, details of RRC signaling structure, etc.), early sync phase, execution phase (e.g. support of mixture of intra-CU and inter-CU, etc.), and LTM cell switch completion phase. Initial discussion on inter-CU LTM with DC (high-level spec impacts and solutions).

Single or multiple reference configuration(s)?

P1 in R2-2406694 (Apple)

Proposal 1: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations are supported.

- P1 in R2-2406694 is agreed.

P1 in R2-2407269 (LG)

Proposal 1. Multiple reference configurations are provided for inter-CU LTM, where each reference configuration is CU specific.

[Samsung]: Multiple reference configuration gives more flexibility. NW still can achieve single reference configuration, which is up to NW. [Huawei]: Assume it does not happen often and saved bits are not significant. It is considered as optimization for some cases, which is not essential. [ZTE]: With multiple reference configurations, it simplifies inter-CU signalling. [Xiaomi]: With multiple reference configurations, it increases UE complexity and it requires more UE memory capability. [Ericsson]: Single reference configuration works as baseline. [Nokia]: With single reference configuration, the reference configuration setup procedure needs to be repeated whenever any CU reconfigures some configuration. [Session chair]: Check companies’ positions. If there is no clear majority companies, we can stick to what we already have (single reference configuration).

- Multiple reference configurations: LG, Samsung, CMCC, Nokia, Fujitsu, Rakuten, ITRI, KDDI, ZTE, BT, IDC, KT, ETRI (13)

- Single reference configuration: Google, Honor, CATT, Lenovo, Vivo, Apple, OPPO, Spredtrum, Xiaomi, NEC, Intel, MediaTek, Qualcomm, China Telecom, Ericsson, Sharp (16)

CSI resource and report configuration:

P3, P4 in R2-2406419 (ZTE)

Proposal 3: The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

Proposal 4: The source CU is responsible to generate the common CSI resource configuration.

- P3 and P4 in R2-2406419 are agreed.

P5 in R2-2407269 (LG)

Proposal 5. As in Rel-18 LTM configuration, CSI resources for LTM L1 measurement for all LTM candidate cells are provided in the form of a single list.

Inter-node RRC message for inter-CU LTM:

P5 in R2-2407421 (Samsung)

Proposal 5: RAN2 is kindly asked to discuss the LTM candidate configuration (ltm-candidate IE) transfer from candidate gNB-CU to the source gNB-CU during the LTM cell preparation procedure by considering the following two options:

	Option 1: Define a new inter-node RRC message for LTM candidate configuration

	Option 2: Define new XnAP IEs to include other information in the LTM candidate configuration

- Noted.

[NEC, Ericsson]: Think we can discuss it later. [Samsung]: Handover request/response procedure is performed per each candidate cell. [Vodafone]: Understand it is under RAN3 discussion. [Session chair]: Let’s wait for more time to think of and check RAN3 discussion status.

Early DL sync:

P3 in R2-2406305 (CATT)

Proposal 3: For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

- Agreed.

Who decides inter-CU LTM switch?

P3 in R2-2406694 (Apple)

Proposal 3 (modified): For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

- Agreed.

[NEC]: Understand source CU initiates inter-CU LTM preparation, however source DU triggers inter-CU LTM cell switch. [LG]: With the agreement that LTM and L1 measurement are decoupled, without L1 measurement, can it be the source CU the node to trigger inter-CU LTM cell switch? [Nokia]: Understands source CU decides it however source DU sends the MAC CE. [Vodafone]: Decoupling between inter-CU LTM and L1 measurement will be continued in Rel-19? [Samsung, OPPO]: Whether source CU triggers inter-CU LTM to source DU is under RAN3 discussion. We may need to wait for further RAN3 progress. [Vivo]: Understand it is RAN2 issue. [Rakuten]: The proposal is which node triggers the MAC CE. No one disagree with source DU. Regarding who decides inter-CU LTM, we may wait for RAN3 progress. [Nokia]: Propose to send a LS to RAN3 to inform RAN2 agreements. [Ericsson]: LTM is based on L1 measurement as baseline. LTM based on L3 measurement is done by NW implementation. [Ericsson, Rakuten]: No need to send a LS to RAN3. [BT]: Think for inter-CU case, if we want to have L3 measurement based inter-CU LTM, we need to standardize the corresponding CU-DU interface to support L3 measurement based LTM. [Vodafone]: We can agree with proposal 3 and we can make further agreement such as “if L3 measurement is used for inter-CU LTM, source CU decides inter-CU LTM cell switch and source DU still triggers the MAC CE.” Also propose to send a LS to RAN3. [Session chair]: Seems for additional agreement Vodafone proposed, there is no consensus right now. Ask Vodafone to have offline discussion with interested companies and if consensus can be made, we can handle the proposal and LS in CB session.

In CB session:

Corresponding proposal and LS are handled in Rel-18 Mobility session (R2-2407770, R2-2407771)

Indication of inter-CU LTM (RLC and PDCP re-establishment)

P4 in R2-2407201 (Huawei)

Proposal 4 (modified): Introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update.

P7, P8 in R2-2406819 (Xiaomi)

Proposal 7: If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. For the issue how to indicate that the security key update is required, we can wait for SA3’s progress on the security of inter-CU LTM.

Proposal 8: If the security key update is not required, reuse the Rel-18 L2 handling mechanism and the Rel-18 NoResetID is reused to determine whether to perform RLC re-establishment and PDCP data recovery when the UE performs the LTM cell switch procedure.

[CATT]: Prefer separate Rel-19 ID. It is also compatible to conditional LTM. [MediaTek]: Based on the current spec, re-establishment and security key change are always coupled. [LG]: Share the view with MediaTek. [OPPO, Vivo]: Support Huawei’s proposal. [Xiaomi]: Conditional LTM could be restricted to intra-CU LTM. [Nokia]: For recovery case, Rel-19 ID is helpful. [Session chair]: Let’s assume recovery is done to different CU, doesn’t the UE need to change a security key anyway? Wonders if security key update can be commonly applicable to all cases. [IDC]: Although we need to wait for SA3 response for security aspect (NCC change), it sounds reasonable to configure re-establishment control by RRC.

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch

P4 in R2-2406919 (Ericsson)

Proposal 4 (modified): As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

- Agreed.

[ZTE]: For DC case, proposal 4 may not work. The UE may need to release LTM candidate configuration. It is better to clarify it is for non-DC inter-CU LTM now.

Handling of DRBs in subsequent inter-CU LTM:

P6 in R2-2407421 (Samsung)

Proposal 6: RAN2 is kindly asked to discuss whether different LTM candidate cells can have different admission results (i.e., different LTM candidate cells accept different PDU sessions).

	Option 1: After accessing to a LTM candidate cell, UE automatically releases the PDU session, which has been released before. This introduces specification impact (RAN2).

	Option 2: The inter-CU LTM candidate cell is prepared only if all LTM candidate cells have the same admission result. This seems to be the principle applied in intra-CU LTM.

[Ericsson]: In Rel-18, it is not a real issue since all candidate cells are under the same CU. [Samsung]: Seems first we need to have clear understanding on the concerned scenario in Rel-18 (including whether it exists in Rel-18 and how it was resolved). Then we can further discuss possible way-forward for Rel-19. [Session chair]: Suggest to have offline discussion among interested companies. Will comeback in Thursday CB session. [ZTE]: Why not directly discuss it in RAN3? [Samsung]: The issue is in the UE side, it is reasonable to discuss it in RAN2.

- Comeback at Thursday CB session.

R2-2407793	Summary on offline discussion on DRB handling in subsequent LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 assumes that different LTM candidate cells with different admission results towards PDU sessions/DRBs is a valid case for both intra-CU and inter-CU LTM.

Proposal 2: For the solutions, RAN2 is kindly asked to discuss whether this issue can be firstly discussed in RAN2. If not, a post-email can be requested for the LS to RAN3.

[Huawei]: It is general issue, not specific to LTM. [Ericsson]: Agree with Huawei. And NW can control to avoid this situation, e.g. NW may reconfigure after the first LTM cell switch or NW may include only cells that have same PDU sessions support into LTM candidates. [Intel]: Agree with Huawei and Ericsson. [Samsung]: Some companies understand all candidate have same admission control and others understands each candidate has different admission control. [Lenovo]: Agree with Huawei. [Rakuten]: For inter-CU LTM, yes NW can control it, but source CU and candidate CUs need to exchange information for LTM candidate reconfiguration/release, which would be new to RAN3. [Ericsson]: It is supported by the existing inter-node procedures. [Apple]: Does NW control means if a candidate does not support all DRBs, the source CU will not perform cell switch to that candidate? [Ericsson]: Yes.

- RAN2 understand NW implementation can handle it.

LTM and L3 HO:

P9 in R2-2406819 (Xiaomi)

Proposal 9 (modified): L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

	When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

	The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

	For the execution order between CHO and LTM, Rel-18 principle is applied.

- Agreed.

[Apple]: Hope NW provides correct LTM configuration. [Nokia]: It would be good to change the last bullet to clarify Rel-18 principle.

Inter-CU SCG LTM:

P8, P9, P10, P11 in R2-2406305 (CATT)

Proposal 8 (modified): Inter-CU SCG LTM preparation can be initiated by source SN.

- Agreed.

Proposal 9 (modified): The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- Agreed.

Proposal 10: For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- Agreed.

Proposal 11 (modified): RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Agreed.

[Xiaomi]: It may be discussed in SA3. Prefer waiting for SA3 response. [Nokia]: It would be good to inform SA3 by LS. Previous LS didn’t include it. [LG]: New SA3 WI already include DC case. Prefer waiting for SA3 response. [Apple]: If the complete message is sent to target SN, SCPAC option may not work. We may need new mechanism. We first need to discuss signaling flows (including whether complete message is sent to MN or target SN). [OPPO]: In Rel-18, it is sent to MN with sk-counter, then MN forwards it to SN. It should be the baseline for Rel-19. [ZTE]: With P9, it means the complete message is sent to MN. It is ok with P11.

For P8:

[Vivo]: If L3 measurement is used for LTM, think MN can also initiated preparation. [Xiaomi]: Agree with Vivo. MN initiated preparation is already applicable to SCPAC. [CATT]: It is not reasonable directly inherit everything from SCPAC. The intended scenario is different, e.g. for LTM, it is mainly for mobility. [Ericsson, Huawei]: Agree with CATT.

For P9:

[Nokia]: It depends on whether there is SRB3 or not. It would be ok when SRB3 is not there, however it is not clear when SRB3 is there.

[POST127][111][MOB] (CATT)

Scope: Prepare a LS to SA3 to inform RAN2 agreements to ask them to take it into account for their works.

Intended outcome: Approved LS in R2-2407602.

Deadline: Short email discussion.

=> Approved in R2-2407602 (LSout)

P8 in R2-2406919 (Ericsson)

Proposal 8: Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

- Agreed.

Inter-CU MCG LTM:

P9, P10, P11, P12 in R2-2406419 (ZTE)

Proposal 9: RAN2 to clarify that inter-CU MCG LTM with SCG unchanged just means that PSCell is not changed during inter-CU LTM execution, but the other change related to SCG is allowed, e.g. radio bearer type and termination change.

[Apple]: The simplest option would be to agree that SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). [Huawei]: Think SCG release is the simplest option.

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

Proposal 10: Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- Agreed.

Proposal 11 (modified): For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

- Agreed.

[CATT]: Think there is no need to add a new sk-Counter since MN-key will be anyway changed due to inter-CU MN LTM, then it will make change of SN-key. There is no security key issue even with the same sk-Counter. [Nokia]: Agree with CATT. [Huawei]: To avoid any confusion, we can simply clarify legacy Rel-15 RRC reconfiguration with sync procedure is applied.

Proposal 12: 	RAN2 to discuss how to handle the co-existence case of inter-CU MCG LTM with SCG unchanged and inter-CU MCG LTM with SCG release with the following options:

•	Option 1: The NW ensures that the UE does not switch to an LTM candidate cell with SCG configuration if the UE has released DC prior to the LTM cell switch. Additional Xn interaction is required.

•	Option 2: The UE does not apply the SCG configuration (if any) in the target LTM candidate configuration if the NW triggers the LTM cell switch to an LTM candidate cell with SCG configuration but the UE has released DC prior to the LTM cell switch.

•	Option 3: The NW can trigger LTM cell switch to any configured LTM candidate cells, regardless of whether the target candidate cell has the SCG configuration and whether the UE is in NR-DC currently. I.e. inter-CU MCG LTM with SCG addition is supported, this may be considered as out of the current WI scope.

- Noted.

[Ericsson]: We agreed that “SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration)”. With this agreement, we do not need any new option more. [Samsung]: This issue is similar to the issue raised in R2-2407421. Suggest we should first clarify the scenario in R2-2407421 and then comeback if needed. [ZTE]: Difference is that for PDU session case, it impacts to CN also while SCG release/reconfiguration does not. Solution can be different. [MediaTek]: Option2 means the UE does not follow NW configuration. Would like to avoid option 2.

Coexistence of MCG LTM and SCG LTM:

P15 in R2-2406419 (ZTE)

Proposal 15: 	RAN2 to discuss the following options for the coexistence on different types of LTM candidates:

•	Option 1: The coexistence of Intra-MN MCG LTM and Inter-SN SCG LTM, and the coexistence of Inter-MN MCG LTM and Inter-SN SCG LTM are not supported, i.e. not configured simultaneously.

•	Option 2: The coexistence of Intra-MN MCG LTM and Inter-SN SCG LTM, and the coexistence of Inter-MN MCG LTM and Inter-SN SCG LTM are supported, i.e. can be configured simultaneously. But the execution of one type of LTM shall trigger the UE to release the other type of LTM candidate configurations autonomously.

P8 in R2-2406694 (Apple)

Proposal 8: If MN has inter-CU LTM candidates configured, then the UE can be configured with intra-CU LTM in SN, but if the LTM switch in MN is inter-CU, then the UE should not have SN configured after the LTM switch.

R2-2407201	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: RAN2 works on a solution to resolve the data rate drop issue after cell switch.

- Noted.

[Qualcomm]: Understand it is also applicable to intra-LTM (not Rel-19 LTM specific issue). [Nokia]: Recall it was discussed in TEI-18 in earlier time. [Apple]: It is clear that this enhancement is not within this WID scope. [ZTE]: Observed this issue in the legacy L3 HO. Support the proposal 3. [Sony]: UE can perform aperiodic CSI report after HO/LTM. With it, can it address the concern? [Xiaomi]: In RACH based HO/LTM, the UE can report CSI in MSG3. With it, can it address this concern. [CMCC]: Support the proposal. Understand it is under RAN1 discussion. We may wait for RAN1 progress. [Futurewei]: Think it is severer issue in inter-CU LTM. [China Unicom]: Support the proposal. [LG]: Agree with Apple, it is not within this WID scopes.

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

R2-2406305	Discussion on inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406356	Further discussion on Inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406386	Discussion on inter-CU LTM	ETRI	discussion	Rel-19

R2-2406419	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406430	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406532	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406623	LTM for Inter-CU	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2406658	Discussion on Inter-CU LTM	InterDigital, Inc.	discussion	Rel-19

R2-2406694	View on open issues in inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406743	Discussion on inter-CU LTM	KT Corp.	discussion

R2-2406775	Discussion on Inter CU LTM 	Lekha Wireless Solutions	discussion	Rel-19

R2-2406819	Discussion on Inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406820	Initial considerations for inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2406854	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406863	Cell switch command for subsequent inter-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2406867	Discussion on the reference configuration for inter-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2406919	Important aspects regarding inter-CU LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406982	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407023	Further detailed discussion on supporting inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2407033	Security impacts of Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407073	On Inter-CU LTM Open Issues	Nokia	discussion

R2-2407107	Radio Resource aspects for intra-CU and inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407108	Discussion on Inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407133	Fast LTM recovery in DC scenarios	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407155	RACH-less LTM completion in inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407201	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407269	Discussion on inter-CU LTM	LG Electronics	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407320	Discussion on subsequent inter-CU or inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407348	Further discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407374	Inter-gNB LTM with no change of RRC/PDCP anchor	Qualcomm Incorporated, NTT DOCOMO, Vodafone, Bharti Airtel (India), Sony	discussion

R2-2407407	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407421	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407439	Discussion on inter-CU LTM 	Kyocera 	discussion	Rel-19

R2-2407441	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407448	Discussion on Inter-CU LTM	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2407465	Discussion on inter-CU LTM	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407478	RRC Modelling for Inter-CU LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2407483	LTM enhancements for Inter-CU mobility	CEWiT	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	Measurement event evaluation

Remaining open issues and further details of measurement event evaluation (e.g. need of event LTM1 (if needed, what’s use case)? , what beam(s) of the serving cell and neighboring cell is used for event evaluation?, Need of cell level measurement result for event evaluation (if needed, what’s use case and how it works)? Further details on event configuration signaling design, e.g. how to associate with resource configuration,, etc.)

Need of event LTM1:

P1 in R2-2406286 (Huawei)

Proposal1: Support Event LTM1 (Beam of serving cell becomes better than absolute threshold).

P1 in R2-2406733 (Qualcomm)

Proposal 1: Event LTM1 is not defined.

- P1 in R2-2406733 is agreed.

[IDC]: With the intention proposed in R2-2406286, LTM1 is not needed since we can just reuse A1. [ZTE]: LTM1 can be still used for CSI measurement reconfiguration purpose. [Ericsson, LG]: Agree with Huawei and ZTE. [Nokia, Samsung]: Agree with Qualcomm. Do not think NW will use LTM1 (instead of A1) for the intention. [Samsung]: Original intention of event triggered L1 LTM MR is to support UE mobility. For other purposes, we use A events. LTM1 is not essential. [Xiaomi]: Agree with Samsung. Note we already agreed the main intentions of LTM events.

Serving cell’s beam for L1 MR event evaluation:

P2 in R2-2406431 (Vivo)

Proposal 2: Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Proposal 2 in R2-2406431 is agreed.

P2 in R2-2406968 (CMCC)

Proposal 2: The best beam of the serving cell and neighboring cell measured by the UE should be used for event evaluation.

[Fujitsu]: Similar discussion happened in RAN1 (RAN1 had offline discussion and will come back to RAN1 online session). Assume it is RAN1 issue. Note RAN1 clarified all possible options. [Apple]: Think we need to continue each WG job. If controversial, it can be further discussed in RAN. [Apple]: In MIMO, the serving beam is compared to the neighbouring beam. And in most of cases, current beam would be the best beam. It will be good to follow same principle for mobility. [MediaTek]: Prefer CMCC proposal, however we need a restriction, i.e. should be configured by LTM configuration. [Nokia]: What matters is NW indicated beam (regardless of best or worst).

- Option1: Current beam (indicated beam by TCI state): Samsung, Apple, Vivo, Xiaomi, Qualcomm, Spreadtrum, Ericsson, Fujitsu, Rakuten, NEC, Huawei, Honor, Nokia, Lenovo, Panasonic, Sharp, IDC (17)

- Option2: Best beam: 9 companies

Candidate cell’s beam for L1 MR event evaluation:

P3 in R2-2406851 (Nokia)

Proposal 3: For the beam level event evaluation for LTM 3/4/5 the measurement resources listed in the R18 resource configuration are used for evaluating beam level events.

P4, P5 in R2-2406286 (Huawei)

Proposal4: For LTM events involving beam of the neighboring cell as the triggering quantity (e.g., Event LTM A3/5), only LTM candidate cells are applicable.

Proposal5: For LTM events with beam-level measurement of candidate cell as triggering quantity, the applicable beam of the candidate cell is the best beam of that cell.

[MediaTek]: Best beam of the candidate beams is not clear since the UE cannot measure all candidate beams. [Apple]: Agree with MediaTek. For Nokia proposal, if any beam among the beams configured by LTM RS configuration meets the event, it triggers MR. [CATT]: Agree with Nokia. Note we agreed beam-level measurement, which is more aligned with Nokia proposal. [Ericsson]: Understand best beam among all candidate RSs, not all possible RSs.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

Need of applying cell level measurement for L1 MR event evaluation:

P5 in R2-2406728 (Apple)

Proposal 5 (modified): Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- P5 in R2-2406728 is agreed.

P4 in R2-2407506 (LG)

Proposal 4: Evaluation of Event LTM based on consolidated result of multiple RSs of a serving cell and/or multiple RSs of a candidate cell is supported.

[Spreadtrum]: Think for cell level measurements, L3 measurement can be reused. [Samsung]: Share the view with Apple. If need to address LG’s concern, we can allow MR includes multiple beams (like N best beams). [Xiaomi]: Agree with Spreadtrum. [MediaTek, ZTE]: Let’s assume a beam meets event and TTT starts, during TTT if another beam that belongs to the same candidate cell meets the event, what will happen? [Vivo]: Understand each beam that meet the event has its own TTT. Support P5 in R2-2406728. [Lenovo]: For cell level L1 measured result, we can just reuse the legacy way. [Huawei]: Support cell level measurement is applied to LTM event. [CATT]: Cell level measurement is not essential for LTM L1 event evaluation. We can rely on e.g. L3 measurement if needed. However, prefer putting FFS for conditional LTM case.

LTM L1 event-triggered MR configuration:

P10, P11 in R2-2406728 (Apple)

Proposal 10: R19 LTM event triggered measurement configuration can be taken as baseline:

-	LTM measurement resource configuration is provided in LTM-config.

-	Event triggered report config is provided in serving cell config.

- P10 in R2-2406728 is agreed.

[Xiaomi, Huawei]: If MR is reported via MAC CE, there would be some issue. [Nokia]: Directly agreeing with Figure 4 is too detailed now. Prefer removing figure 4 from the conclusion.

Proposal 11: FFS on whether the event configuration can be shared across all candidate cells.

P12 in R2-2407506 (LG)

Proposal 12: Common RS pool introduced for Rel-18 LTM is extended to support configuring CSI RS.

Layer to perform event evaluation:

P7 in R2-2407446 (Kyocera)

Proposal 7: RAN2 should agree that MAC layer handles the event evaluation and measurement report triggering.

- Agreed.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

R2-2406287	Discussion on event triggered report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406306	Measurement Event Evaluation	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406357	Discussion on measurement and signalling design	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406420	Discussion on measurement event evaluation	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406431	Discussion on LTM measurement event evaluation	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406524	Discussion on beam for evaluation of LTM event-triggered reporting	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406533	Open issues for event triggered  L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406545	Open issues of measurement event evaluation for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406707	Discussion on the measurement event evaluation for LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406728	LTM measurement event evaluation and configuration	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406733	Measurement enhancements for LTM	Qualcomm Incorporated	discussion

R2-2406756	Discussion on measurement event evaluation for L1 measurement event	Spreadtrum Communications	discussion	Rel-19

R2-2406851	Further View on Measurement-related Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4	R2-2405149

R2-2406886	L1 Measurement enhancements	Lenovo	discussion	Rel-19

R2-2406908	Discussion on measurement event evaluation for LTM 	Lekha Wireless Solutions	discussion	Rel-19

R2-2406920	Important aspects regarding event triggered L1 measurements	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406968	Discussion on measurement event evaluation	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407024	Discussion on event triggered L1 measurement configuration for LTM	Transsion Holdings	discussion	Rel-19

R2-2407109	Discussion on measurement event evaluation for LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407124	Event evaluation of L1 measurement reporting	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407141	Event triggered L1 measurement evaluation for LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407160	Event based L1 measurements triggered LTM candidate cell addition/release	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407195	Reference resource configuration for L1 measurement event	Panasonic	discussion

R2-2407349	Discussion on measurement event evaluation	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407393	Discussion on measurement event evaluation	KDDI Corporation	discussion	Rel-19

R2-2407408	Discussion issues on related to measurement event evaluation	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407422	Remaining Issues for Measurement Event Evaluation	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407446	Measurement event evaluation for LTM enhancement	Kyocera	discussion	Rel-19

R2-2407470	Discussion on event triggered L1 measurement	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407506	Event LTM - Event variant, new event, RS determination, configuration	LG Electronics France	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.4	Measurement reporting

Remaining open issues and further details of measurement reporting procedure (e.g. what information in the measurement report? MAC CE or UCI for measurement report?, need of measurement reporting once leaving condition is met, etc.)

MAC or UCI for MR?

P1 in R2-2406921 (Ericsson)

Proposal 1: Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

- P1 in R2-2406921 is agreed.

P1 in R2-2407423 (Samsung)

Proposal 1: RAN2 confirm the RAN1 agreements using UCI L1 measurement reporting to align the event based L1 measurement reporting is used for LTM.

[Samsung]: UCI-based L1 measurement reporting was agreed for MIMO and it would be good to have common framework also for L1 LTM. [Session chair]: Check companies’ views.

- UCI: 3 companies (Apple, Samsung, Xiaomi)

- MAC CE: 24 companies

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Need of MR when leaving condition is met:

P3 in R2-2406432 (Vivo)

Proposal 3: It is configurable by gNB whether a UE initiates L1 measurement reporting when leaving condition is satisfied (e.g. reportOnLeave-like parameter in event configuration).

Need of event-triggered periodic MR:

P4 in R2-2406432 (Vivo)

Proposal 4: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report (e.g. reportAmount-like parameter in event configuration).

P5 in R2-2406358 (MediaTek)

Proposal 5: The event triggered L1 MR is a one-shot report. Multiple reporting for one trigger event is not introduced. (This does not preclude multiple report by multiple trigger events configuration).

Information in MR:

P1 in R2-2407285 (NTTDCM)

Proposal 1: Event-triggered L1 measurement includes below contents;

・Beam information (i.e., SSBRI or CRI)

・Beam quantity (up to RAN1)

・Triggered event information (e.g., ReportConfigID)

P2 in R2-2407423 (Samsung)

Proposal 2: Event triggered L1 measurement reporting for LTM includes following information:

-	L1 RSRP of the current beam

-	L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition)

-	DL RS resource index (SSB or CSI-RS)

R2-2406286	Discussion on measurement event evaluation	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406307	Measurement Reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406358	Discussion on measurement reporting of event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406421	Discussion on measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406432	Discussion on event-triggered L1 measurement reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406534	Discussion on the UL signalling for L1 reproting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406546	Open issues of measurement reporting procedure for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406708	Event-based measurement reporting procedure for L1 measurement	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406729	LTM event triggered measurement reporting	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406757	Discussion on measurement reporting for L1 measurement event	Spreadtrum Communications	discussion	Rel-19

R2-2406921	Discussion on which layer reports the event triggered L1 measurements	Ericsson, Honor, Huawei, InterDigital Inc., MediaTek Inc., NEC, Nokia, NTT DOCOMO, Oppo, T-Mobile USA, Vivo, ZTE corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2406969	Discussion on measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407025	Discussion on L1 measurement reporting for LTM	Transsion Holdings	discussion	Rel-19

R2-2407110	Discussion on measurement reporting for LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407125	Details of event triggered L1 measurement report	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407142	Event triggered L1 measurement reporting for LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407208	Event triggered reporting enhancements for LTM	Panasonic	discussion	Rel-19

R2-2407285	Discussion on Event-triggered L1 measurement reporting	NTT DOCOMO, INC.	discussion	Rel-19

R2-2407350	Discussion on measurement reporting	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407394	Discussion on event triggered L1 measurement reporting	KDDI Corporation	discussion

R2-2407409	Discussion issues on related to measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407423	Support of Event Triggered L1 Measurement Reporting	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2407447	Measurement reporting procedures for LTM enhancements	Kyocera	discussion	Rel-19

R2-2407507	Event LTM - Report triggering, report contents and transmission procedure	LG Electronics France	discussion	Rel-19	NR_Mob_Ph4-Core

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240791)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.7.1	Organizational

LS, Rapporteur input, including workplan, etc.

Rapporteur input

R2-2406395	XR Workplan	Nokia, Qualcomm (Rapporteurs)	Work Plan	Rel-19	NR_XR_Ph3-Core

R2-2406396	XR Agreements	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406397	SA2 Overview	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406398	RAN3 Overview	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

4 Tdocs above noted

- Nokia reminds we start RRM meas gaps discussion in October.

LS in

R2-2406216	LS on UE assistance information (R1-2405736; contact: Nokia)	RAN1	LS in	Rel-19	NR_XR_Ph3-Core	To:RAN4	Cc:RAN2

Noted

R2-2406221	LS on UL PSI based PDU discarding in NR-DC (R3-243957; contact: Qualcomm)	RAN3	LS in	Rel-19	NR_XR_Ph3-Core	To:RAN2

Noted

R2-2406222	Response LS on FS_XRM Ph2 (R3-243958; contact: Lenovo)	RAN3	LS in	Rel-19	FS_XRM_Ph2	To:SA2	Cc:RAN2, SA4

Noted

R2-2406241	LS on FS_XRM Ph2 (S2-2407351; contact: vivo)	SA2	LS in	Rel-19	FS_XRM_Ph2	To:SA4, RAN2, RAN3

Noted

R2-2406242	LS Reply on FS_XRM Ph2 (S4-241370; contact: Huawei)	SA4	LS in	Rel-19	FS_XRM_Ph2, FS_5G_RTP_Ph2	To:SA2	Cc:RAN2, RAN3

- Intel thinks there are some replies we should consider in RAN2 work. Intel encourages companies to evaluate how this can be considered from RAN2 point of view.

Noted

#### 8.7.1.1	Discussion on incoming LSs

Discussion on RAN2 replies to SA2 LS on FS_XRM Ph2 (S2-2407351) and RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957)

Reply to RAN3 LS on PSI based discarding in NR-DC

R2-2406254	Discussion on reply to RAN3 LS on PSI-based PDU discard in NR-DC	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1. 	UE activates PSI-based SDU discard on a UL split-bearer only after it has received MAC CEs activating the discard from both MN and SN.

Proposal 2.	UE deactivates PSI-based SDU discard on a UL split-bearer once it has received a MAC CE deactivating the discard from either MN or SN.

Proposal 3.	After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the primary path that activates SDU discard, UE applies a smaller ul-DataSplitThreshold.

Proposal 4.	After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the secondary path that activates SDU discard, UE applies a larger ul-DataSplitThreshold.

Proposal 5.	UE applies the legacy ul-DataSplitThreshold if both primary and secondary paths are in congestion state.

Proposal 6.	Adopt the above enhancements (Proposal 1~5) as Rel-18 corrections too.

Proposal 7.	Include the above enhancements, if agreed, in the reply LS to RAN3.

R2-2407216	Discussion on incoming LSs	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: 	Reply to RAN3 with the following:

•	The likelihood of the PDCP entity in the UE receiving inconsistent commands from the MN and SN nodes is low.

•	PSI level that triggers discarding at PDCP is up to UE implementation, resulting in little impact in the unlikely event of inconsistent commands from the MN and SN nodes.

•	RAN2 does not think any UE behaviour needs to be specified for handling uplink PSI based SDU discard for split bearer.

DISCUSSION:

- ZTE indicates that it was a decision in RANP that this objective should not impact RAN2. RAN3 can do coordination at NW side. We should follow WID and just reply what we have now.

- Apple agrees with P1 from QCM as it would allow optimizing the behaviour, but is OK to follow majority.

- OPPO thinks NW coordination can be done, but in R2 we can just keep our specs as they are.

- CATT, Xiaomi, vivo agrees with ZTE and OPPO.

- Xiaomi indicates the same behaviour is for PDCP duplication.

- QCM indicates R3 does not believe in coordination between the nodes, that is why they sent the LS.

- CMCC thinks we can tell R3 that there may be coordination needed in the network.

- Nokia does not like NW coordination solution.

- Ericsson thinks no NW coordination is needed. It works OK as it is now.

- Huawei think we just reply with the current behaviour. For R18, we should not change anything.

- Intel agrees we should not touch R18, but there is some valid point in R3 LS.

- QCM indicates that the current behaviour is not optimal and there is a simple way to improve.

For the reply LS to RAN3:

We just reply how this works in R2 specs

Whether to apply NW side solution to improve is up to RAN3

[AT127][501][XR] LS to RAN3 (Qualcomm)

Scope: Reply LS to RAN3

Intended outcome: Approved LS to RAN3 in R2-2407780

Deadline: Agreeable LS available for offline approval: Friday 2024-08-23 0900

R2-2407780	Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC	RAN2	LS out	Rel-19	NR_XR_Ph3-Core	To:RAN3

Approved

Reply to SA2 LS on XRM Ph2

R2-2406433	Discussion on LS from SA2 on FS_XRM Ph2	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: RAN2 thinks indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed.

Proposal 2: RAN2 thinks the time to next burst is not useful for RAN resource scheduling.

R2-2406675	On Responses to SA2 and RAN3 LS for XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: RAN2 should reply to SA2 that, in-band signaling for dynamic changes of traffic periodicity may be useful in terms of UE power saving, if the application server can provide the information and if NG-RAN can use such information properly.

Proposal 2: RAN2 should reply to SA2 that, the information of the time to next burst may be useful. However, RAN3 is better positioned to evaluate whether jitter can be compensated by the NG-RAN when such information is used.

DISCUSSION on whether dynamic periodicity indication and/or time to next burst (TTNB) is useful:

- Mediatek agrees with vivo, thinks no additional info is needed.

- CMCC thinks periodicity is semi-static, not dynamic, so the benefit is unclear. TTNB is useful for scheduling.

- Lenovo thinks TTNB was discussed in the past.

- Nokia thinks there is a benefit for power saving from TTNB, but if periodicity is dynamic it can be handled with TTNB.

- NEC thinks dynamic periodicity is not needed. TTNB is useful.

- Ericsson thinks dynamic periodicity is more efficient than CP based solution. TTNB is similar, but in case there is jitter, this information may not be always reliable.

- Meta is in general supportive of dynamic periodicity as the periodicity can change dynamically. Xiaomi agrees.

- Samsung believes TSCAI is enough for periodicity. TTNB does not consider jitter, so it may not be useful for the gNB.

- LGE thinks whether this is useful depends on how dynamic changes are expected.

- Huawei thinks we need to focus on whether this is useful. If periodicity is very dynamic then TTNB can handle it. If it does not, then existing solution is enough.

- ZTE, vivo think that the periodicity changes should not be too frequent. If they are very frequent, then it should be handled with TTNB.

- ZTE asks whether this is for both DL and UL? Thinks it is more useful for UL.

From RAN2 periodicity can already be provided to gNB via TSCAI and/or UAI, which is sufficient for infrequent periodicity changes.

RAN2 does not have consensus on whether additional indication for dynamic changes of the periodicity are needed.

RAN2 thinks TTNB may be useful for the NW scheduling for DL, provided it is provided in advance and is reliable and accurate at RAN.

[AT127][502][XR] Reply LS to SA2 (vivo)

Scope: Reply LS to SA2

Intended outcome: Approved LS to SA2 in R2-2407733

Deadline:  Agreeable LS available for offline approval: Friday 2024-08-23 0900

R2-2407733	Reply LS on FS_XRM Ph2	RAN2	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2	Cc: RAN3, SA4

Approved

R2-2406253	Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC	Qualcomm Incorporated	LS out	Rel-19	NR_XR_Ph3-Core	to:RAN3

R2-2406255	Discussion on reply LS to SA2 on FS_XRM Ph2	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406303	Discussion on incoming LSs	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406399	Periodicity and Time to Next Burst	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406408	Discussion on SA2 and RAN3 LSs for Rel-19 XR	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406434	Discussion on LS from RAN3 on UL PSI based PDU discarding in NR-DC	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406457	Discussion on LSs for XR	ZTE Corporation, Sanechips	discussion

R2-2406472	RAN2 views and responses to LSs from SA2, RAN3 and  SA4	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406480	Discussion on XRM and UL PSI-based PDU Discard	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406558	Discussion on SA2 and RAN3 LSs	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406566	Discussion on SA2 LS on FS_XRM Ph2 and RAN3 LS on UL PSI based PDU discarding in NR-DC	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406624	Views on LSs for SA2 and RAN3	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2406781	Discussion on the LS from SA2 and RAN3	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406783	Discussion on incoming LSs	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406892	Discussion on RAN2 Replies to LS	Lenovo	discussion	Rel-19

R2-2406913	Discussion on SA2 and RAN3 LSs for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407044	Discussion on LSs from SA2 and RAN3	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407276	Discussion on SA2 and RAN3 LSs on Rel-19 XR	Meta	discussion

R2-2407383	Discussion on LS on FS_XRM Ph2 and UL PSI based PDU discarding in NR-DC	CMCC	discussion	Rel-18	NR_XR_Ph3-Core

### 8.7.2	Multi-modality support

Objective: Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption.

Including aspects such as:

- potential enhancements based on multi-modal information awareness depending on traffic direction (UL/DL)

- can the multi-modal information be provided from the UE

- other enhancements for multi-modal traffic not strictly related to multi-modality awareness, e.g. power saving, scheduling

Multi-modal info from the UE

R2-2406625	Need for MMSID and DRB mapping	Sony	discussion	Rel-19	NR_XR_Ph3

Proposal 1: RAN2 assumes that multi modal service ID is received in RAN from the core network i.e. there is no need for UE to provide this information to the gNB.

R2-2406916	Further aspects of multi-modality support in RAN	Samsung R&D Institute UK	discussion

Proposal 3. Regardless of SA2 decision, RAN2 to consider extending the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

Proposal 4. If SA2 decides that CN-based solution on multi-modal awareness for RAN will not be considered in Rel-19 XR, RAN2 to discuss the UAI extension from Proposal 3 as an alternative for downlink QoS flows MM treatment in Rel-19 XR.

DISCUSSION:

- OPPO thinks UE can provide this information considering the lack of info from SA2. Support P3 from Samsung. For DL, see no need to discussion RAN2.

- Xiaomi thinks for DL SA2 agreed to provide MMSID, but for UL it should be from the UE.

- LGE thinks for UL UE has better knowledge about the association as it is determined by app layer.

- CMCC thinks that for UL UE can get this information from APP layer and provide this information to gNB.

- Lenovo thinks we should make the decision based on SA2 reply. ZTE agrees. If SA2 does not reply, then we need to rely on UAI. They think both UL and DL can be available from the UE, if needed.

- Qualcomm agrees with Lenovo.

- Vivo agrees with Samsung. Vivo can think make a decision for UL part in RAN2, we cannot wait for SA2.

- Nokia tends to agree with Lenovo, they understand the situation in SA2 is a bit different.

- Huawei thinks it is OK to include MMSID in UAI. In Rel-18 we have both CN solution and UAI. There are cases where this can only come form the UE.

- Fujitsu agrees with Huawei and Samsung, it is similar for UAI in Rel-18.

- Meta also thinks that UL is independent from SA2.

- Apple thinks SA2 can technically provide this information, but they want to understand the reason we need it.

- Ericsson thinks we need to discuss how we use this information.

Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

FFS whether this can be applied to DL.

How is multi-modal information used

R2-2406559	Discussion on Multi-Modality	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5: Scheduling/LCP enhancements can be considered for multi-modality.

Proposal 6: Admission control enhancement can be considered for multi-modality.

DISCUSSION:

- NEC supports proposals 5 and 6.

- Xiaomi thinks SA2 discussed about integrated handling and is not sure we should consider it. For P5, Xiaomi thinks this can be left up to gNB implementation. See no need to enhance LCP.

- Mediatek thinks that enhancing LCP is too complex.

- Qualcomm is concerned about P5 as it has huge impact on UE complexity. WID already clarifies that the impact should be limited.

- vivo supports the proposals and the exact solution depends on exact information we get. We can take complexity into consideration.

- Spreadtrum thinks delay-aware LCP can be reused for sync requirement.

- Nokia has concerns on the complexity with P5. This can be handled by gNB scheduling.

- Huawei thinks admission control is one use case for multi-modality information. Even with no spec impact, gNB can consider this info.

- Ericsson thinks a conclusion in SA2 was that application layer can handle joint admission control for multiple flows. Ericsson thinks benefits of LCP enhancements are unclear.

- ZTE thinks what we can do depends on what information we have. ZTE thinks the more information we get, the more things we can do.

- CMCC thinks that LCP enhancements can be used to properly multiplex data into TB.

- Google thinks P6 is a low-hanging fruit with no RAN2 spec impact. For P5 it is not clear what we can do, it depends on the information we can get.

- Lenovo thinks for LCP the main question is whether we consider synchronization thresholds.

- Apple asks how MAC can know which packets are associated together, this is too complex.

- Ericsson asks if multi-modal information is MMSID or sth else.

- ZTE thinks another use of MMSID is for QoS flow to DRB mapping.

RAN2 considers that based on multi-modal information:

    - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.

- The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

R2-2406589	Discussion on Multi-modality support for XR traffic	Xiaomi Communications	discussion

Proposal 2   QoS flow level synchronization is prioritized over packet/frame level synchronization. It is gNB implementation on how to achieve QoS flow level synchronization.

Proposal 3   For multi-modal QoS requirements, LCP enhancement will not be considered until the requirement of multi-modal QoS is clear enough.

Proposal 4   PDU set based discarding across PDU sets/QoS flows should not be considered until we get requirement from SA2.

DISCUSSION on P2:

- Lenovo thinks that for UL we don’t do any LCP enhancements.

- CMCC would like to still consider UL LCP enhancements.

- Lenovo indicates that whether the gNB can do packet or flow level synchronization depends on what information we can get from SA2. ZTE, OPPO agree.

- OPPO wonders if we can have flow level sync for UL.

- Ericsson asks how flow level synchronization can be achieved.

- LGE clarifies that gNB may e.g. map flows to the same DRB to make sure they are sent together.

For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns.

For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

R2-2407135	Multi-modality support for XR	Google Ireland Limited	discussion

Proposal 3: Awareness of synchronization requirements would enable the RAN to make informed scheduling decisions, improving the quality and reliability of multi-modal XR applications.

Proposal 4: Study the optimization of PDU-Set discard and prioritization across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows and the unmet synchronization requirements.

DISCUSSION on PDU set discard enhancements:

- QCM thinks that joint discard would require packet level synchronization and only app layer knows this.

- LGE think inter-QoS flow discard should also be excluded due to complexity.

- Vivo supports P4 from Google. Think that we can still consider flow level synchronization to support this.

- MTK agrees with Qualcomm and LGE.

- Samsung is supportive of the proposal and agrees with vivo. It is less complex than changing LCP.

- Apple thinks this also depends on what information we can get from SA2. It is not clear that we can drop packets from different flows. OPPO share views with Apple. OPPO suggests to check with SA2/SA4.

- Nokia agrees with Qualcomm. It will be complex, especially in case of traffic in different DRBs.

- Sharp indicates that this mechanism is performed in upper layers, so this is less complex than LCP. OK to ask SA2/SA4 about whether the required information can be available.

- QCM indicates that the main complexity issue is with mapping packets which need to be synchronized.

- Ericsson agrees with QCM, Nokia and wonders about the gains.

- LGE is concerned about the complexity, especially in cross-DRB scenario.

- Ericsson thinks this is too complex and gains were not shown.

- Futurewei thinks there are two levels of information to be considered – dependency and synchronization.

RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

DRX enhancements

R2-2407045	Discussion on Multi-Modality	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2	Support multiple active DRX configurations to limit the delay and optimize power saving of UEs with multi-flow XR services.

Proposal 3	Support independent configuration parameters for the secondary DRX group.

DISCUSSION on P2:

- Qualcomm thinks that power savings are not very big while the complexity is significant for the UE.

- Nokia indicates that there is an active time where the UE is awake so can be scheduled at that times. No need to introduce additional complexity.

- Spreadtrum thinks this enhancement is not needed.

- Xiaomi sees some benefits if some flows are integer while some are non-integer. Think we can have a single DRX cycle with multiple on-durations.

- Huawei agrees with QCM, Nokia. We can handle multi-modal flows with single DRX.

- LGE thinks single DRX is not enough to align with multiple periodicity, so supports having this.

- Sharp agrees with complexity issue mentioned by Qualcomm. Think SPS/CG can be used as well as this does not affect DRX cycle.

- Samsung thinks multiple DRX is useful.

- ZTE thinks with multi-modal traffic UE will anyway be active for the most time. This seems to be an unnecessary optimization.

- Ericsson thinks that SPS/CG may not always be used, e.g. in the view of jitter.

- Apple supports Ericsson.

Not support multiple active DRX configurations

Ericsson would like to focus on the following proposal: “Proposal: Support independent configuration of drx-StartOffset for the secondary DRX group”

DISCUSSION:

- Qualcomm is OK to consider drx-StartOffset, but not other parameters, e.g. DRX timers, cycle.

- Huawei agrees with QCM. Thinks the reason to introduce was to support FR2 and different SCS. Asks what’s the use case.

- Ericsson clarifies that additional offset helps to handle jitter.

- Lenovo is not sure about the usefulness as this would mean that we map different traffic to different FR.

- Mediatek thinks if it is supposed to handle jitter, then it is complex.

- LGE also does not see then need and feels single DRX configuration is sufficient.

- Apple also does not see the relation to multi-modality.

Not support independent configuration of drx-StartOffset for the secondary DRX group

Scheduling enhancements

R2-2406302	Discussion on multi-modal XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: Scheduling enhancements are needed to support multi-modal XR services with haptic data from the haptic KPI and the network capacity point of view.

Noted

DISCUSSION:

- Ericsson appreciates the simulations, but they do not show what the capacity is for haptic traffic. Ericsson is not sure what enhancements can be done for the traffic with such tight requirements.

- Qualcomm thinks the simulated case is not likely in the near future. Qualcomm thinks that Rel-18 enhancements can be already used for this kind of traffic, also URLLC enhancements are relevant. No need for anything additional.

- Apple indicates that this would have RAN1 impact and we should not involve them. Agrees with QCM.

- Vivo supports Huawei observation and proposal. Some further enhancements are needed for this kind of requirements.

- OPPO share view with QCM and Apple. Not sure about additional solutions, RAN1 should be involved.

- Xiaomi doubts the requirements are valid.

- Sony thinks that we had simulations in R18 and it showed some gains of resource sharing, but this is already possible.

- Nokia thinks that shared CG is already possible by implementation.

- Sharp thinks some enhancements might be needed, but we already have enough to discuss.

- Ericsson indicates it is too late.

R2-2406370	Discussion on Multi-modality for XR	TCL	discussion	Rel-19

R2-2406435	Discussion on Multi-modality	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406463	Multi-modality assistance information for RAN awareness	ZTE Corporation, Sanechips	discussion

R2-2406473	UE/RAN enhancements considering multi-modal awareness	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406525	Discussion on DRX enhancements for multi-modality	ASUSTeK	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406547	Discussions on Multi-modality XR	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406567	Potential enhancements based on multi-modal information awareness	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406595	Enhancements for support of Multi-Modal XR applications 	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406662	Discussion on Multi-Modality	Sharp	discussion

R2-2406676	Views on Support of Multi-Modality Services in Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406740	Discussion on XR multi-modality	China Telecom 	discussion

R2-2406760	Discussion on enhancements for XR Multi-modality	Spreadtrum Communications	discussion	Rel-19

R2-2406782	Discussion on the multi-modality support	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406864	Discussion on scheduling enhancements for multi-modal traffic	ITRI	discussion	NR_XR_Ph3-Core

R2-2406914	Discussion on Multi-modal support for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406988	Further discussion on multi-modality support for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407213	Multi-modality support for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407225	Discussion on multi-modality	MediaTek Inc.	discussion	Rel-19

R2-2407277	Discussion on Multi-Modality XR	Meta	discussion

R2-2407356	Discussion on multi-modality support	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407404	Multi-modality support	Nokia	discussion	NR_XR_Ph3-Core

R2-2407516	Primary use cases for multi-modality support in RAN	III	discussion	NR_XR_Ph3-Core

### 8.7.3	RRM measurement gaps/restrictions related enhancements

Objective: Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc).

This agenda item will not be treated during RAN2#127 and no contributions should be submitted for this AI for this meeting.

### 8.7.4	Scheduling enhancements

Objective: For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

Including aspects such as:

- further details of the additional priority for LCH with dealy-critical data

- whether/how to enhance LCP restrictions

- further details of DSR with multiple pairs of remaining time and buffer size, e.g. does PSI need to be included, whether/how is DSR triggering impacted etc.

LCP restrictions

R2-2407354	Discussion on Scheduling enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: LCH mapping restrictions configured to LCH can be relaxed in case of the LCH with delay-critical data.

Proposal 4: RAN2 evaluates which LCH mapping restrictions can be relaxed.

R2-2406741	Discussion on XR scheduling enhancements	China Telecom	discussion

Proposal 2: The new LCP restriction only allows LCH with delay-critical data to have priority over LCH with non-delay critical data in resource allocation.

R2-2406784	Scheduling enhancements for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

Observation 1. There is little benefit to enhance LCP restriction without RAN1 impact.

Proposal 2. RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

DISCUSSION on whether enhance LCP relaxation or LCP restrictions:

- Xiaomi thinks LCP relaxation can work, but the problem some of these are for URLLC so they cannot be relaxed. For new LCP restriction, dynamic priority can serve the same purpose.

- Fujitsu thinks LCP relaxation can be combined with dynamic prioritization and supports this. New LCP restriction solution has some issues.

- Ericsson thinks that relaxation is complex and brings issues, e.g. it impacts the scheduler in NW. IDT, Nokia, LGE agrees, the NW may just not configure such restrictions.

- QCM thinks there are use cases where enhancements are useful. They are currently used to separate different types of traffic, e.g. voice and data. For delay critical data it makes sense to relax the restrictions.

- Lenovo thinks neither is useful.

- Apple thinks that in case we relax LCP restrictions, the grant may not fit the data. Apple agrees with Samsung.

- Huawei supports relaxation of LCP restrictions to have more transmit opportunities for delay critical data. We can discuss which restrictions can be relaxed and it should be controlled by the NW.

- MTK indicates that the NW will provide proper grant based on DSR.

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

Enhanced DSR contents

R2-2407047	Discussion on scheduling enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	Network should be able to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

Proposal 2	Any data in front of the queue with longer remaining time than the data behind in the queue should report the lowest remaining time and total buffer size.

Proposal 4	Two importance levels to be included in the DSR and the importance level is indicated in the DSR format using a new I-bit instead of the previous R-bit.

R2-2407062	Scheduling Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 7: Include low-importance data ahead of delay critical data in the buffer size calculation for DSR .

R2-2406989	Further discussion on scheduling enhancement on XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 7: RAN2 to agree that PSI is not reported in the DSR.

Proposal 9: Non-delay-critical data is reported in BSR as legacy.

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

Proposal 2	Any data in front of the queue with longer remaining time than the data behind in the queue should report the lowest remaining time and total buffer size.

Proposal 7: Include low-importance data ahead of delay critical data in the buffer size calculation for DSR .

DISCUSSION on whether to include (some) non-delay critical data in the DSR:

- QCM thinks the packets in the buffer will always be sorted properly.

- Apple does not think there is a need to discuss this in Rel-19 with multiple reporting thresholds.

- Xiaomi does not see the issue, it can be solved by UE implementation.

- Ericsson indicates we have many solutions assuming the data may not always be ordered and that the UE may sometimes send non-delay critical before delay critical as we agreed not to do intra-LCH prioritization.

- Nokia thinks that maybe non-delay critical term is problematic, the point is we should not have delay critical data blocked by less important data.

- Mediatek agrees with the intention, but how to capture in specs may be a problem.

- Intel thinks that with PSI discard, it does not have to be reported. But if it is not configured, this may be useful.

- Lenovo thinks UE implementation may solve this issue.

- LGE thinks we have a clear definition of delay critical data, so the question is whether we need to update it. LGE does not think we need to do it. It is infrequent case.

- vivo thinks that with multiple thresholds, we may have to report non-delay critical data, depending on how delay critical data is specified considering multiple thresholds.

- LGE thinks that DC data is the data below the shortest DSR threshold.

- Qualcomm thinks we just should improve the granularity of the reported information.

For enhanced DSR:

    - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

    - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

    - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

DISCUSSION on PSI reporting in DSR:

- QCM does not think importance needs to be reported.

- Ericsson thinks that perhaps we need to clarify the above FFS points first, before discussing PSI inclusion in DSR.

R2-2406594	Enhanced uplink scheduling for XR	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6: RAN2 should discuss enhancements to the intra-UE prioritization procedure, e.g. considering the remaining delay budget when determining the priority of an UL grant (prioritized/deprioritized UL grant).

DISCUSSION:

- LGE, Xiaomi, vivo thinks additional priority can be considered in intra-UE prioritization.

- QCM thinks this is a stage-3 detail.

- Nokia thinks there may be no spec impact

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

R2-2406256	Discussion on delay-aware scheduling	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406269	Discussion on scheduling enhancements for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406371	Discussion on delay-aware LCP enhancement	TCL	discussion

R2-2406436	Discussion on scheduling enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406455	Scheduling enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2406474	Scheduling enhancements using delay related information	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406479	Discussion on additional priority for delay-critical data	SHARP Corporation	discussion	NR_XR_Ph3-Core

R2-2406548	Discussions on enhancement of the LCP for delay-critical data	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406560	Consideration on XR-specific scheduling enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406588	Discussion on scheduling enhancements of XR traffic	Xiaomi Communications	discussion

R2-2406626	UL Scheduling enhancements for XR	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2406677	Views on Delay-Aware Operations for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406761	Discussion on XR scheduling enhancements	Spreadtrum Communications	discussion	Rel-19

R2-2406797	Delay-aware scheduling enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406798	Considerations on delay-sensitive scheduling for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406858	Discussion on DSR enhancement	TCL	discussion	Rel-19

R2-2406923	Discussion on additional priority for delay aware LCP	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406939	Discussion on Delay status report	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407214	Scheduling enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407274	Discussion on scheduling enhancements for XR	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407279	Discussion on Scheduling Enhancement for XR	Meta	discussion

R2-2407384	Discussion on LCP enhancement for XR	Google Ireland Limited	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407392	Discussion on UL scheduling enhancements	MediaTek Inc.	discussion	Rel-19

R2-2407460	Discussion on Scheduling enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407518	Discussion on XR scheduling enhancements	III	discussion	NR_XR_Ph3-Core

R2-2407539	Discussion on UL related Scheduling Enhancements for XR	Rakuten Mobile, Inc	discussion	Rel-19

### 8.7.5	RLC enhancements

Objective: RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

Including aspects such as:

- how to avoid unnecessary retransmissions, e.g. details of Tx and Rx approaches, pros and cons comparison.

- how to  ensure timely RLC retransmissions for XR, e.g.

  - can existing mechanisms be reused or do we need enhancements?

  - what kind of enhancements are needed, e.g. autonomous retransmission, retransmission based on enhanced status report, retransmission based on enhanced polling.

  - details and pros and cons of different solutions.

Unnecessary retransmissions – Rx and Tx approach clarifications

R2-2406400	RLC AM enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: the RAN2#126 agreement on proper advancing of the window for RX-initiated approach for avoiding unnecessary retransmissions also applies to the TX-initiated approach i.e. “RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter” applies to both RX- and TX-initiated approaches.

Proposal 2: RAN2 acknowledge that in the TX-initiated approach for avoiding unnecessary retransmissions, the mandatory delivery of an SDU is only exchanged for a mandatory delivery of its discard indication.

Proposal 3: For indicating abandoned RLC SDUs from RLC receiver to transmitter, in order that the transmitting PDCP reliably knows how high-numbered PDCP SDUs it can proceed to transmit, RAN2 select between:

A)	RLC ACK, combined with regular PDCP status reporting to keep the PDCP transmitter reliably informed of successful delivery; or

B)	A new explicit RLC indication separate from ACK, of SDUs abandoned by the receiver.

Proposal 4: in the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer like t-Reordering at PDCP or t-Reassembly in RLC UM.

DISCUSSION on P1:

- OPPO thinks Tx side can advance the window without the enhancement on the Rx side.

- Lenovo agrees with the principle proposed in P1. The receiver needs to send the indication to Tx side for the Tx side to update the window.

- Samsung thinks P1 just means that Tx window will be update as in legacy.

- QCM thinks in Tx approach, Tx side can advance its window automatically.

- OPPO does not see the point of waiting for Rx window to advance before advancing Tx window.

- Huawei supports the idea of keeping the windows aligned, but it can be achieved via different means, not only SR.

- Ericsson also supports the idea, but it can be achieved in different ways.

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.

- DISCUSISON on P3:

- LGE thinks this problem will happen with proper configurations. Treordering should ensure PDCP window advances properly. LGE suggests to skip the proposal.

DISCUSSION on P4:

- Sony is OK with the proposal, but SR should be more frequent.

- Lenovo is also OK the proposal.

- Huawei would like to consider counter based solution.

In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer.

Unnecessary retransmissions – Rx vs Tx approach

R2-2406481	Analysis of RLC AM Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	RAN2 to adopt TX-initiated mechanism to avoid unnecessary retransmission.

Proposal 2	Discard Indication from PDCP triggers the indication, when the PDCP SDU is already transmitted in the lower layer (i.e., MAC).

R2-2406857	Discussion on RLC AM enhancement	TCL	discussion	Rel-19

Proposal 1: It is suggested RAN2 to choose the Rx initiated approach as the baseline to further discuss the solution for Unnecessary retransmissions.

R2-2407015	RLC AM enhancement	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3 RAN2 consider independent Rx and Tx approach, where

•	Tx side stops to retransmit an obsolete SDUs based on the discard indication as for Tx initiated approach

•	Rx side stops to receive an obsolete SDU based on local timer and variable as for Rx initiated approach

DISCUSSION:

- Apple thinks we need to look at two aspects: 1. How to stop unnecessary reTx immediately, 2. How do we sync the windows between Rx and Tx. Tx is better for bullet 1, for bullet 2 we need some signalling from Rx. Apple prefers Tx approach, but combined approach is also acceptable.

- Intel agrees with Apple.

- Lenovo agrees that in terms of latency Tx approach is better. The concern with Rx approach is with dropping the report.

- OPPO also prefers Tx approach to avoid additional gaps in PDCP.

- QCM would like to support NEC’s proposal as it has least impact to specifications.

- Mediatek shares views with Apple, it has less impact on capacity.

- Ericsson is concerned about the impact on specifications.

- Samsung supports Tx approach.

- Xiaomi agrees with Ericsson, if the timer is set based on discard timer than there should be no latency.

- LGE agrees with Xiaomi and Ericsson.

In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where

    - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach

    - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

Timely RLC retransmissions

R2-2407368	Discussion on details of RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer for transmission.

Proposal 7. Retransmission based on enhanced status report is not supported.

Proposal 8. If remaining time of a RLC SDU in the transmitting window becomes below a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Proposal 10. RLC retransmission based on HARQ NACK is not supported.

R2-2406561	Consideration on XR-specific RLC enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: Not to support the RLC autonomous retransmission for R19 XR.

Proposal 2: Enhanced status report for UL can be left to gNB implementation, no spec impact.

Proposal 3: For UL XR traffic, the network can configure a new set of shorter value for pollPDU, pollByte, t-PollRetransmit to trigger the polling for timely RLC retransmission. When to apply the shorter value can be further studied.

R2-2406367	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3	For the faster polling in RLC AM, RAN2 discuss to rely on proper configuration (the existing value range or introducing smaller values) of related parameters (i.e., pollByte, pollPDU, t-PollRetransmit).

Proposal 4	For autonomous retransmission without feedback, RAN2 to discuss relying on the legacy t-PollRetransmit expiry triggered retransmission with proper t-PollRetransmit configuration.

Proposal 5	For autonomous retransmission without feedback, if new trigger condition besides PollRetransmit expiry is needed, the trigger of autonomous retransmission should be based on the remaining delay budget, e.g., based on a configured remaining delay threshold.

R2-2406257	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406364	Considerations on RLC re-transmission related enhancements for XR 	KDDI Corporation	discussion

R2-2406409	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406437	Discussion on RLC enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406443	RLC Enhancements for XR	Samsung	discussion	Rel-19

R2-2406456	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2406475	RLC AM enhancements for XR traffic	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406549	Discussions on RLC enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406601	Further Discussions on RLC AM Enhancements	Ericsson	discussion	Rel-19

R2-2406627	RLC AM enhancements	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2406678	Views on RLC-AM Enhancements for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406734	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406742	Discussion on RLC enhancements for XR	China Telecom	discussion

R2-2406762	Discussion on timely RLC retransmission(s)	Spreadtrum Communications	discussion	Rel-19

R2-2406893	AM RLC enhancement	Lenovo	discussion	Rel-19

R2-2406940	Discussion on RLC AM Enhancements	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

R2-2406984	Discussion on the RLC Enhancements for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407215	RLC enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407280	Discussion on RLC AM Enhancements for XR	Meta	discussion

R2-2407355	Discussion on RLC enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2407391	Discussion on RLC enhancements on small packet delay budget scenario	MediaTek Inc.	discussion	Rel-19

R2-2407511	Discussions on RLC enhancements for Rel-19 XR	Futurewei	discussion	Rel-19	NR_XR_Ph3-Core

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-241667)

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240924)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, etc.

For the LTE_TN_NR_NTN_mob WI, including initial draft CRs from the WI spec rapporteurs: draft 36.300 CR (Samsung), draft 36.331 CR (CATT), draft 36.306 CR (Vivo).

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2406220	Reply LS on Support of Regenerative-based Satellite Access (R3-243954; contact: ZTE)	RAN3	LS in	Rel-19	NR_NTN_Ph3-Core	To:SA2	Cc:RAN2

Noted

Rapporteur inputs (for LTE_TN_NR_NTN_mob WI)

(moved here from 8.8.6)

R2-2407259	Stage 2 Running CR for E-UTRAN to NR NTN mobility	Samsung	draftCR	Rel-19	36.300	18.2.0	LTE_TN_NR_NTN_mob-Core

Check the CR in offline 304

Revised in R2-2407616

R2-2407616	Stage 2 Running CR for E-UTRAN to NR NTN mobility	Samsung	draftCR	Rel-19	36.300	18.2.0	LTE_TN_NR_NTN_mob-Core

Content is endorsed. Can further check in the next meeting whether to put the change elsewhere

[AT127][304][LTE_TN_NR_NTN_mob] Stage 2 CR (Samsung)

Scope: Check the Stage 2 running CR for E-UTRAN to NR NTN mobility

Intended outcome: Endorsable CR

Deadline for companies' feedback:  Thursday 2024-08-22 20:00

Deadline for final CR (in R2-2407616):  Friday 2024-08-23 08:00

R2-2406318	Open issue list for LTE_TN_NR_NTN_mob WI	CATT	discussion	LTE_TN_NR_NTN_mob

[Issue 1] Down-selection on the signalling options on how to signal the NR NTN satellite assistance information:

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

[Issue 2] Whether ntn-PolarizationDL for the NR NTN cell is mandatory or optional.

[Issue 3] Whether to reuse existing SatelliteId-r18 or introduce separate IE for NR satellite ID.

[Issue 4] Whether to reuse existing maxSat-r17 as the maximum number of NR satellites configured (taking into account the signalling option finally agreed).

[Issue 5] Confirmation of the working assumption "We don’t introduce multiple SMTCs in LTE".

[Issue 6] Confirmation of the working assumption "NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell".

Noted

R2-2406319	Introduction of LTE TN to NR NTN IDLE mode mobility (Option 1)	CATT	draftCR	Rel-19	36.331	18.2.0	B	LTE_TN_NR_NTN_mob

R2-2406320	Introduction of LTE TN to NR NTN IDLE mode mobility (Option 2)	CATT	draftCR	Rel-19	36.331	18.2.0	B	LTE_TN_NR_NTN_mob

Update the CR in offline 305

Revised in R2-2407617

R2-2407617	Introduction of LTE TN to NR NTN IDLE mode mobility (Option 2)	CATT	draftCR	Rel-19	36.331	18.2.0	B	LTE_TN_NR_NTN_mob

Endorsed as a baseline. Continue the discussion in the next meeting both on the ASN.1 structure and the field description

[AT127][305][LTE_TN_NR_NTN_mob] RRC CR (CATT)

Scope: Update the RRC CR for E-UTRAN to NR NTN mobility based on meeting agreements

Intended outcome: Endorsable CR

Deadline for companies' feedback:  Thursday 2024-08-22 20:00

Deadline for final CR (in R2-2407617):  Friday 2024-08-23 08:00

R2-2407812	Summary of [AT127][305][LTE_TN_NR_NTN_mob] RRC CR	CATT	disc	Rel-19		LTE_TN_NR_NTN_mob

Proposal 1: RAN2 agrees signaling Alternative 1 to indicate ephemeris data for a satellite supporting both NR NTN and IoT NTN, i.e. introducing a Satellite reference ID that refers to the ephemeris data of an IoT NTN satellite in the existing list neighSatelliteInfoList-r18, and using the "CHOICE" signaling structure including this Satellite reference ID and explicit ephemeris data configuration as the two choices.

Come back in the next meeting

Proposal 2: RAN2 sticks to the agreement that only duplication of ephemeris data needs to be avoided in the case of same satellite supporting both NR NTN and IoT NTN.

Come back in the next meeting

R2-2406321	Introduction of LTE TN to NR NTN IDLE mode mobility (Option 3)	CATT	draftCR	Rel-19	36.331	18.2.0	B	LTE_TN_NR_NTN_mob

R2-2406250	Introduction of LTE TN to NR NTN Mobility UE Capability	vivo	draftCR	Rel-19	36.306	18.2.0	LTE_TN_NR_NTN_mob-Core

-	Nokia supports the capability but thinks we can modify the wording referring to idle and inactive at the beginning of the sentence: “it is optional for a UE in idle/inactive...”

-	QC thinks we should refer to the support of measurements: “… a UE in idle/inactive supports measurement for inter-RAT...”)

-	ZTE think we could add a reference to 304

Update the CR in offline 306 taking into account the comments above

Revised in R2-2407618

R2-2407618	Introduction of LTE TN to NR NTN Mobility UE Capability	vivo	draftCR	Rel-19	36.306	18.2.0	LTE_TN_NR_NTN_mob-Core

Endorsed

[AT127][306][LTE_TN_NR_NTN_mob] UE capability CR (vivo)

Scope: Update the UE capability CR for E-UTRAN to NR NTN mobility based on meeting agreements

Intended outcome: Endorsable CR

Deadline for companies' feedback:  Thursday 2024-08-22 20:00

Deadline for final CR (in R2-2407618):  Friday 2024-08-23 08:00

WI CR Rapporteurs:

38.300:	Thales

38.304:	ZTE

38.321:	Interdigital

38.331:	Ericsson

38.306:	Apple

### 8.8.2	Downlink coverage enhancements

Contributions should focus on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

RAN2 impacts due to SSB periodicity extension

R2-2406591	Discussion on the impact of SSB extension and cell DTXDRX for NTN	Beijing Xiaomi Mobile Software	discussion	Rel-19

Observation 1: From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact. If the SSB periodicity is larger than 160ms, ssb-PeriodicityServingCell , measurement gap periodicity, SMTC periodcity, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset needs to be enhanced to consider the SSB periodicity higher than 160ms.

From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

Observation 2: From RAN2 point of view, if the SSB periodicity is not larger than 160ms, there is no impact to legacy UE. If the SSB periodicity is larger than 160ms, the performance of RLF/HO for legacy UE will be degraded and requires RAN4 evaluation. Also the reception of MIB/SIB1 will be degraded.

-	Apple agrees in principle but think this is in RAN4 scope. Vivo agrees

-	Fraunhofer thinks it’s early to say there is no impact below 160ms. CMCC thinks that the first sentence for RAN2 aspects

Observation 3: From RAN2 point of view, for cell level DTX/DRX, DRX retransmission, SR, and random access procedure will be impacted for legacy UE.

R2-2407306	Discussion on DL coverage enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 waits for RAN1 input before further discussion on cell level and beam level Cell DTX/DRX pattern.

Proposal 2: If RAN1 decides to support 320ms for SSB periodicity, RAN2 need to extend the ssb-periodicityServingCell, SSB-MTC, SSB-MTC4-r17 and mgrp to support 320ms.

R2-2407129	Idle mode considerations for downlink coverage enhancements	Nokia, Nokia Shanghai Bell	discussion

Proposal 1: Until RAN1 reply has been received, only SSB periodicity extension should be studied in RAN2.

Observation 1: SSB periodicity of 320 will only provide a 96.8% coverage ratio, with little to no DL resources for UE communication.

Observation 2: SSB periodicity of 640 will provide a 100% coverage ratio, with DL resources available for UE communication.

Proposal 2: RAN2 to study a SSB periodicity of up to 640 ms.

Observation 3: Adjusting the SMTC window and measurement gaps e.g. in case of changing SSB frequency will increase the signalling overhead.

Proposal 3: Serving cell provides neighbor cell activity such as SSB pattern information. FFS on state information i.e. whether to include information on when cells and/or beams are on or off. FFS on how to signal i.e. on/off pattern and/or duration, or delta signaling for patterns in case of dynamic configuration.

Proposal 4: Irrespectively of the selected RAN2 solution, RAN2 should strive for a low signaling overhead solution.

Proposal 5: Due to the nature of the configuration complexity, RAN2 include in the study to allow UE implementation to read SSB occasions.

R2-2407532	Downlink coverage enhancement SMTC impacts	Sequans Communications	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: SSBs needs to be time multiplexed across the satellite footprint over up to 80ms or 320ms range

Observation 2: Existing SMTCs framework is limited to up to 4 SMTCs per frequency layer

Proposal 1: RAN2 to study SMTC impact of SSB time multiplexing across the satellite footprint

RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

Proposal 2: Consider extending the NTN SMTC list / define a new list to signal at least all neighbor SMTCs

Observation 3: The UE should ideally consider only SMTCs corresponding to the closest neighbor cells

Proposal 3: Consider broadcast of neighbor cells reference location to allow location-based UE SMTC selection

Observation 4: In connected, NW may not have UE location information needed to configure SMTCs to UE

Observation 5: R18 location-based CHO without associated measurement avoids SMTCs configuration, but adds complexity and may increase handover interruption time

Proposal 4: Consider location-based UE SMTC selection also in connected

Cell/Beam DTX / impact to legacy UEs

R2-2406490	Discussion on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: The existing NTN bar bit can be used to bar legacy UE from accessing a cell operating with DL coverage enhancement.

If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

Proposal 2: Consider to introduce a new bar bit indicate the barring for UE supporting DL coverage enhancement.

Proposal 3: If beams in a cell are in the same state at any time, cell DTX/DRX pattern can be considered, with Rel-18 NES cell DTX/DRX as a baseline.

Proposal 4: If beams in a cell can be in different states at any time, consider to support configuration of multiple DTX/DRX patterns and dynamical activation by NW indication.

R2-2406952	Discussion on Downlink Coverage Enhancements	CSCN	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 should consider the impact on SSB periodicity extension, e.g. cell search/research, RA process, paging and measurement.

Proposal 2: RAN2 assumes beam level DTX is preferred for downlink coverage enhancements.

Proposal 3: RAN2 needs to wait for reply from RAN1 to determine whether cell/beam level DRX is needed.

R2-2406993	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Introduce cell DTX for beam power sharing for all RRC states, and RAN2 further studies UE behaviors adaption, e.g., on RACH/measurements/RLM, and etc., during (de)activated time of the cell DTX.

Proposal 2: Similar to NES, barred bit can be introduced to prevent UE not supporting cell DTX for beam power sharing (e.g., legacy UEs) from accessing current cell.

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

R2-2406246	Further Considerations on DL Coverage Enhancements	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406324	Discussion on Downlink Coverage Enhancements	CATT	discussion	NR_NTN_Ph3-Core

R2-2406550	Discussions on cell DTX during satellite dynamic power sharing	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406571	Discussion on the DL coverage enhancement	Google	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406638	Discussion on cell DTX	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406685	DL coverage enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406765	Discussion on DL coverage enhancement for NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406870	On beam-level DL coverage enhancement in NTN	Lenovo	discussion	Rel-19

R2-2406894	Consideration on downlink coverage enhancement	NEC Corporation	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406902	Beam management of NR NTN coverage enhancement	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407187	Downlink coverage enhancement for NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407345	Discussion on downlink coverage enhancement	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407382	RAN2 Impact on DL coverage enhancements	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407401	Discussion on NTN downlink coverage enhancements	NERCDTV	discussion

R2-2407462	Discussion on downlink coverage enhancement	LG Electronics Inc.	discussion	Rel-19

R2-2407551	DL coverage enhancements	Ericsson	discussion	Rel-19

Withdrawn?

R2-2407544	Views on DL Coverage Enhancements for NR-NTN	Inmarsat, Viasat	discussion	Rel-19	NR_NTN_Ph3-Core	Late

### 8.8.3	Uplink Capacity/Throughput Enhancement

No contributions are expected for this AI at this meeting.

Withdrawn?

R2-2407545	Views on UL Capacity Enhancements for NR-NTN	Inmarsat, Viasat	discussion	Rel-19	NR_NTN_Ph3-Core	Late

### 8.8.4	Support of Broadcast service

Contributions should address the signaling of the intended service area of a broadcast service.

R2-2406719	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-18	NR_NTN_Ph3-Core

- Different MBS services in different areas of a NTN cell

Observation 1: if one cell covers a large area, different MBS sessions may be available in different sub-areas.

Proposal 1: RAN2 to agree that different MBS sessions could be associated with different MBS broadcast intended service areas, and such information should be indicated in the system information.

Observation 2: for the NTN network whose satellite footprint consists of one cell or multiple cells, and each of them covers a big area, regarding location dependent broadcast service, more than one contents for the same MBS session could be distributed in each cell.

Proposal 2: RAN2 to agree that, for the location dependent broadcast service, the area session ID of each distinguished content of the MBS session and the associated area range should be provided in the MBS configuration information.

Proposal 3: RAN2 to agree that, to support MBS services/contents in cell area of more than one NTN cells (or portions thereof), only the service area covering portions of cells need to be explicitly indicated.

- Signalling details

Observation 3: It seems that SIB20 informing of the intended service area information does not align with its current functionality carrying the both of the MCCH and MTCH scheduling information.

Observation 4: providing a MCCH configurations per MBS session or, in a further step, per different area session for each location dependent MBS session in the SIB20 will let the UE interested in more than one MBS services needs to monitor more than one MCCHs simultaneously, which brings additional UE operation complexity.

Observation 5: the granularity of the mapping of FSAIs included the SIB21 is only the MBMS session, but not area session.

Proposal 4: RAN2 to agree to embed the intended area information in the MBSBroadcastConfiguration IE transmitted on the MCCH.

-	Apple thinks that use of SIB20 is better in terms of power saving

-	vivo thinks the possible benefit in terms of power saving is limited and not a justification to go for SIB20

-	ZTE thinks legacy UE in any case will not interpret the new information

-	Thales agrees with Apple that power saving is important. QC agrees. Interdigital also agrees as there is a benefit if the UE knows the information in advance.

-	LGE thinks the UE should anyway decode the MCCH

-	Ericsson thinks that if the area information changes then if included in SIB20 all the UEs will be affected (or this case should be excluded from the update procedure)

-	HW prefers to have it in MBSBroadcastConfiguration and thinks that proponents of SIB20 have different views on the details, and in some cases their proposals still require acquiring MCCH

-	LGE prefers using MBSBroadcastConfiguration. CMCC also.

-	Samsung supports Apple view.

-	Fujitsu agrees with CATT

Continue in offline 307 to discuss the use of SIB20 vs MBSBroadcastConfiguration in MCCH

Proposal 5: RAN2 to agree that ETWS in the NTN network could be associated with only certain geographical areas, and such areas could be indicated in SIB6, if the satellite footprint consists of one cell or multiple cells, and each of them covering large area.

Proposal 6: RAN2 to agree to reuse the coverage area information included in the SIB25 to provide the information of the intended area for the MBS services and/or MBS session content for the NTN network in the SIB.

[AT127][307][R19 NR NTN] service area information (Apple)

Scope: Continue the discussion on the options to provide service area information: SIB20 vs MBSBroadcastConfiguration in MCCH

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Thursday 2024-08-22 20:00

Deadline for rapporteur's summary (in R2-2407619):  Friday 2024-08-23 08:00

R2-2407619	Report of [AT127][307][R19 NR NTN] service area information	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Discuss online whether the expected UE behavior as below can be agreed.

- When UE is not in any intended service area of its interested broadcast services, UE may not need to (re)acquire up-to-date MCCH.

Proposal 2: FFS on solutions.

RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

R2-2406687	Broadcast service support over NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

- Signalling details

Proposal 1: It is possible that the broadcast transmission is limited to the intended service area only. And no special handling is needed if UE cannot receive the service inside the intended service area.

Proposal 2: The intended broadcast service area can be defined by a geographical area represented by referenceLocation and radius.

-	ZTE thinks we could also consider polygon shapes on top of this. LGE also think we could support other approaches but could live with the majority view.

-	Thales think the notion of geographical areas, covering both circles and polygons, is already there and we could adopt something similar also for this. Apple thinks we could rely on the TMGI for differentiating among different countries

-	Nokia thinks we don’t need to be too accurate for this

The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Proposal 3: RAN2 to discuss whether to provision multiple intended service areas for one MBS broadcast session in a cell.

Proposal 4: Use SIB20 to provision intended service area for serving cell.

- UE behaviour

Proposal 5: It’s up to UE implementation whether to establish/release MRB when UE enters/leaves the intended service area. In details:

•	The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated upon entering an intended service area within an NTN cell providing an MBS broadcast service the UE is interested in.

•	The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated upon leaving an intended service area within the NTN cell providing an MBS broadcast service the UE is interested in.

Proposal 6: If UE outside of intended service area receives the broadcast service, RAN group does not enforce any rule for UE to discard the broadcast data (instead it could be left to application layer to handle).

- Service continuity

Proposal 7: Service continuity can be enhanced in two places by considering intended service area.

•	In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session.

•	In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected).

R2-2406635	MBS broadcast service area information	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

- Signalling details

Proposal 1	Similar to TN coverage area list, the geographical area list can also be provided as list of reference locations with corresponding cell radius.

Proposal 2	Each MBS broadcast service area in the list additionally includes one or more associated MBS broadcast frequencies or MBS FSAIs. Decide whether to include only MBS broadcast frequency or only MBS FSAIs.

Proposal 3	Extend existing SIB21 for broadcasting the list of MBS broadcast service areas.

- UE behaviour

Proposal 4	In TS 38.331 section 5.9.3.2, “upon entering intended service area” is added as additional condition where the UE may initiate the broadcast MRB establishment procedure.

Proposal 5	In TS 38.331 section 5.9.3.2, “upon leaving intended service area” is added as additional condition where the UE may initiate the broadcast MRB release procedure.

Proposal 6	In TS 38.304 for cell reselection evaluation, UE may prioritize the frequency if the UE is in the geographical Area associated with the frequency and UE is interested in or receiving the MBS service in that frequency.

R2-2407236	Support for broadcast services in NR NTN	Ericsson	discussion	NR_NTN_Ph3-Core

- UE behaviour

Proposal 1	A UE does not establish MRB(s) associated with an MBS service limited to an intended service area when it is not located within the area.

Proposal 2	A UE releases its established MRB(s) associated with an MBS service limited to an intended service area when exiting the area.

- Signalling details

Proposal 3	Information about intended broadcast service area is signalled in MBSBroadcastConfiguration.

- ETWS support

Proposal 4	Send an LS to SA1 to clarify the requirements for ETWS primary notification and the possibility to include the intended warning area in NTN.

-	QC agrees we need to do it if we want to do something about ETWS

-	LGE/vivo also support LS

-	Samsung agrees to send the LS but we should ask about requirements without hinting to a precise solution

Send an LS to SA1 considering the TP in R2-2407236 but modifying the action based on the comments in the meeting.

R2-2407622	LS on requirements for ETWS primary notification	Ericsson	LS 	Rel-19	NR_NTN_Ph3-Core	To:SA1

-	Samsung thinks we should only ask about requirements to support geofencing information for ETWS in a NTN cell

Revised in R2-2407823

R2-2407823	LS on requirements for ETWS primary notification	Ericsson	LS out	Rel-19	NR_NTN_Ph3-Core	To:SA1

Approved

R2-2406491	Discussion on MBS Broadcast Service Area information	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

- Signalling details

Proposal 1: The format of a circle center with a radius is used to indicate a fixed geographic area for an MBS intended service area.

Proposal 2: The format of a circle center with a radius and a reference time is used to indicate a moving geographic area for an MBS intended service area. UE derives the real-time center location based on the corresponding satellite ephemeris.

Proposal 3: A list of geographic areas with each area identified by an area ID is included in SIB20.

Proposal 5: The legacy SIB modification procedure is applied to update the geographic area information in SIB20.

Proposal 4: The mapping between area ID and MBS session ID is included in MBSBroadcastConfiguration message in MCCH, where multiple area IDs can map to an MBS session ID.

Proposal 6: The legacy MCCH modification procedure is applied to update the mapping between area ID and MBS session ID.

- UE behaviour

Proposal 7: For an MBS broadcast service intended for a certain area, UE should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

-	LGE thinks that in this case we should not use a geographical area represented by circles

-	QC thinks this would anyway not be applicable for legacy R18 UEs

-	Toyota thinks a “should” does not enforce anything for the UE. Samsung agrees and thinks we can put this in Stage 2

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

Proposal 8: For an MBS broadcast service intended for a certain area, broadcast MRB establishment procedure may be initiated when UE is inside the intended area; broadcast MRB release procedure may be initiated when UE leaves the intended area.

-	Toyota thinks that we should clarify that the procedure is initiated by the UE, e.g.:

Proposal 8a: For an MBS broadcast service intended for a certain area, the UE may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

-	Nokia thinks we either put some requirements or we better leave it to UE implementation.

-	Ericsson think the way this is currently specified takes into account the TN case only but the NTN case is different and we should have some requirements for the UE

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

For an MBS broadcast service intended for a certain area, the UE may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

R2-2406247	Further Discussion on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406267	Discussion on MBS service in NTN system	CAICT	discussion

R2-2406323	Discussion on support of broadcast service in NR NTN	CATT	discussion	NR_NTN_Ph3-Core

R2-2406352	Signalling for the support of MBS broadcast service in NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406551	Discussions on supporting broadcast intending to serve partial cell	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406583	MBService area indication & geoblocking	PANASONIC	discussion

R2-2406606	Discussion on MBS Broadcast service area signaling	THALES	discussion	Rel-19	NR_NTN_Ph3-Core	Late

R2-2406628	Broadcast service area signaling	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406849	On How To Support MBS in Rel-19 NR NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3

R2-2406865	Discussions on handling MRB(s) associated with intended service areas	ITRI	discussion	NR_NTN_Ph3-Core

R2-2406871	On broadcast service area indication in NTN	Lenovo	discussion	Rel-19

R2-2406903	Signaling design of service area in NR NTN	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406958	Discussions on MBS in Rel-19 NTN	TOYOTA Info Technology Center	discussion

R2-2406971	Discussion on MBS broadcast service for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406994	Consideration on broadcast service ehancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407049	Further details on intended service area for MBS and ETWS	NEC	discussion

R2-2407053	Discussion on the support of broadcast service	Xiaomi	discussion

R2-2407188	Support for broadcast service in NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407263	Considerations on MBS in NTN	Continental Automotive	discussion	Rel-19

R2-2407307	Discussion on MBS broadcast over NTN	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407346	Discussion on the support of broadcast service	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407415	UE behaviour for MBS related procedures	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407416	Discussion on MBS service support for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407418	Discussion on support of broadcast service in NTN	LG Electronics France	discussion	Rel-19	38.331	NR_NTN_Ph3-Core

R2-2407453	Support of broadcast service in NTN	NERCDTV	discussion

R2-2407473	Discussion on support of broadcast service	ITL	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407497	Discussion on Support of MBS Broadcasting over NTN access 	TCL	discussion

### 8.8.5	Support of regenerative payload

Contributions should focus on the needed updates for Stage 2 description and on whether any existing essential features would be affected - and potentially need any modifications - in a regenerative payload architecture.

R2-2406972	Considerations on regenerative payload	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

- Common TA

Observation 1: Common TA and kmac are already support value zero in current specification.

Proposal 1: For regenerative payload, introduce negative values for ta-Common-r17 in TS 38.331 and up to network configuration, then additional clarification is not needed for both common TA and kmac in stage 2 specification.

-	Apple suggested to fix this to 0 and thinks we need to check with RAN1 in case we want to set this to negative values. LG agrees

-	Ericsson wonders how to handle legacy UEs in this case. CMCC thinks could apply at least for R19

-	CATT thinks we should confirm the understanding that legacy UEs are supported in regenerative payload.

-	vivo does not see the strong need for this and in case this would require stage3 updates

-	CATT thinks that legacy UEs could be configured with value 0 via existing parameter and a new R19 parameter should be added

-	Ericsson thinks we should have done it from the beginning but now we should not do it in order not to have different behaviour for different UEs

RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

-	QC thinks this is not needed. Ericsson agrees

-	IDC thinks this is not needed and it would be bad for some other UEs

-	Xiaomi thinks that if we have different values for legacy and R19 UEs then the NW does not know which value the UE applies. HW thinks the NW could use different values for different UEs

Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

R2-2407623	LS on common TA in a regenerative payload scenario	CMCC	LSout	Rel-19	NR_NTN_Ph3-Core	To:RAN1, RAN4

Approved

- RACH-less HO

Proposal 2: For regenerative payload, existing RACH-less HO could be applied directly, no further enhancement is needed.

-	vivo thinks we only need a Stage 2 clarification that inter-gNB RACH-less HO in supported. Lenovo agrees

-	Ericsson agrees that no further enhancement is needed

-	Nokia is ok not to have enhancement but also don’t want to indicate that everything is supported.

RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

-	HW thinks the current Stage 2 description restrict to intra-gNB case is a mistake that has to be corrected. Ericsson think this was by design

R2-2406686	Features support in regenerative payload architecture	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Do not optimize UE power saving for SDT procedure in regenerative payload architecture.

Agreed

Proposal 2: Handover preparation over Xn/ISL needs to be enhanced to support RACH-less HO. Send an LS to RAN3.

Draft LS can be found in Annex.

-	Lenovo is ok to inform RAN3

Proposal 3: Capture in the spec that common TA and Kmac shall/should be set to zero for regenerative payload architecture.

R2-2406850	Addressing Potential Issues for NTN over Regenerative Architecture	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3

- RACH-less HO

Observation 9: Target cell may underestimate/overestimate the appropriate time to transmit the UL assignment over PDCCH, which may lead to a waste of resources and increased power consumption at the UE’s side.

Observation 10: In regenerative architecture, target gNB has no means to know when the source gNB will stop serving the area and when the UE will access the cell.

Observation 11: In case of combining RACH-less HO and time-based CHO, the target gNB can better estimate when to transmit the UL assignment since it is aware of the T1-T2 time window.

Observation 12: The RACH less handover feature works for regenerative architecture, while few optimisations are possible, like informing the target gNB when the UE is supposed to be present in the target cell.

Proposal 4: RAN2 to discuss means to reduce target gNB’s uncertainty during RACH-less handover for regenerative architecture. baseline for the satellite switching with regenerative payload.

R2-2407054	Discussion on the support of regenerative payload	Xiaomi	discussion

- RAN paging

Observation 1: The UE cannot receive RAN paging if there is no ISL between the serving cell and the last serving cell of the UE.

Proposal 1: Consider the following two solutions to address RAN paging:

•	The RAN notification area only includes cells deployed with the serving satellite.

•	The UE performs a RAN update when there is no ISL between the serving cell and the last serving cell of the UE.

-	Lenovo wonders in the UE needs to know about the presence of ISL

-	Apple thinks we don’t have to worry whether ISL is available or not

-	vivo thinks this is in RAN3 scope and we should not discuss this unless asked by Ran3

-	Xiaomi is ok to discuss this in RAN3

Agreements:

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

3.	RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

R2-2406248	Discussion on RACH-less Handover with Regeneration Payload	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406268	Discussion on NTN regenerative payload	CAICT	discussion

R2-2406322	Further discussion on regenerative payload	CATT	discussion	NR_NTN_Ph3-Core

R2-2406629	Satellite switch with re-sync in regenerative payload	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406636	Discussion on regenerative payload	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 4	After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message.

-	Apple wonders if this is a NW implementation issue

Come back in the next meeting

R2-2406744	Discussion on regenerative payload	KT Corp.	discussion

R2-2406773	Discussion on satellite switch with resynch for regenerative payload	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406872	UE location verification in NTN regenerative architecture	Lenovo	discussion	Rel-19

R2-2406904	Consideration of essential features supporting in regenerative payload	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406995	Consideration on support of regenerative payload	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407016	mobility with regenerative payload	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407026	Discussion on RACH-less handover for regenerative payload	Transsion Holdings	discussion	Rel-19

R2-2407260	Regenerative payload for NR NTN	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407264	Considerations on RACH-less handover for regenerative payload	Continental Automotive	discussion	Rel-19	Revised

R2-2407308	Discussion on regenerative payload	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407347	Discussion on regenerative payload	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407452	Discussion on time-based measurement initiation for regenerative payload	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407498	Discussion on support of NTN regenerative payload architecture 	TCL	discussion	Rel-19

R2-2407548	Considerations on RACH-less handover for regenerative payload	Continental Automotive	discussion	Rel-19	R2-2407264

R2-2407550	Regenerative payload	Ericsson	discussion	Rel-19

### 8.8.6	LTE to NR NTN mobility

Contributions should focus on the remaining issues for the support of idle mode mobility between LTE and NR NTN.

R2-2406325	Conclusion on remaining open issues for LTE TN to NR NTN IDLE mode mobility	CATT	discussion	LTE_TN_NR_NTN_mob

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

Observation 1: Signaling Option 1 seems to need some changes specific to eMTC in order to specify the applicability of the new SIBXY introduced.

Proposal 1: RAN2 selects one of the 3 signaling options taking into account the corresponding draft CRs for each option provided in [2][3][4].

We don’t consider option 1

-	QC and LG support option 2. Lenovo also supports

-	Samsung thinks option 3 is the natural way of extending and this could allow to signal both NR NTN and IoT NTN if they are on the same satellite, without repeating the ephemeris. Lenovo wonders if this a valid scenario. Nokia wonders the same

-	CATT indicates that in the current CR there is a distinction between NR NTN and IoT NTN satellites.

-	Inmarsat thinks that the scenario of a satellite supporting both IoT NTN and NR NTN is absolutely realistic

We go for option 2 (can come back to this if we find serious issue with this)

Proposal 2: The ntn-PolarizationalDL is optional.

Agreed

Proposal 3: RAN2 down-selects following alternatives to identify the NR satellite assistance information:

-	Alternative 1: Reuse the SatelliteId-r18 to identify either an NR satellite or an IoT NTN satellite;

-	Alternative 2: Define NR specific satellite ID, e.g. SatelliteId-NR-r19, to identify NR satellite specifically.

-	Samsung thinks we can go for Alt1 unless we find problems

We go for Alt1

We consider a solution that avoids repeating the ephemeris for a satellite which provides both IoT NTN and NR NTN cells

Proposal 4: If signaling Option1 [2] or Option 2 [3] is adopted, maxSat-r17 (4) is reused for the maximum number of NR satellites.

-	QC thinks we should consider the possibility to signal more satellites, e.g. 8

-	LGE thinks we could stick to 4. ZTE agrees

-	Nokia thinks that the existing number of satellite is sufficient

Agreed

Proposal 5: RAN2 confirms the working assumption "We don’t introduce multiple SMTCs in LTE".

RAN2 will not do further work to introduce multiple SMTCs in LTE

Proposal 6: RAN2 confirms below working assumption and confirms no Spec impact to TS 36.304:

-	NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

-	LGE supports this

Agreed

R2-2407258	E-UTRAN TN to NR NTN mobility	Samsung	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

Proposal 1: RAN2 confirms E-UTRAN TN to NR NTN RRC_INACTIVE mobility is supported, with the understanding that UE moves to RRC idle upon selecting the NR NTN cell. No spec change.

-	vivo agrees but maybe this could be clarified in a capability description (if we have a capability)

RAN2 confirms that measurements of NR NTN cells for a UE in E-UTRAN TN RRC_INACTIVE are supported, with the understanding that UE moves to RRC idle upon selecting the NR NTN cell

Proposal 2: Confirm that SIB33 and its ephemeris is used to broadcast NR NTN neighbour cell ephemeris where NeighSatelliteInfo is extended as a parallel list. No new SIB is introduced.

Proposal 3: If NR NTN TA Common parameters in extended NeighSatelliteInfo-v1900 are not present, the UE uses the nta-CommonParameters-r18 in NeighSatelliteInfo-r18.

Proposal 4: RAN2 to consider the benefits on power saving by introducing a distance-based measurement initiation of NR NTN cells in a terrestrial E-UTRA cell. Distance is between stationary reference location and a UE.

-	LGE thinks in case this is only for the serving cell, but there is no need for this in LTE

-	HW does not support this and think this would expose the eNB location information

R2-2406637	Idle mode mobility from LTE to NR NTN	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Define new neighbor satellite list neighSatelliteInfoList-r19 with larger size maxSat-r19 (=8) in SIB33.

Proposal 2	Confirm that multiple satellites associated with the same frequency can have different SMTC offsets.

R2-2406905	Remaining issues of LTE TN to NR NTN mobility	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 select Option2 for NR satellite assistance information: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Proposal 2: LTE TN informs the available time duration for NR satellite assistance information. UE only reselects the neighbour NR NTN cell in the available time window.

R2-2406249	Further Discussion on LTE TN to NR NTN Mobility	vivo	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

Proposal 5: Introduce a new UE capability without signalling for LTE TN to NR NTN mobility.

Agreed

Agreements:

1.	Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

2.	The ntn-PolarizationalDL is optional.

3.	Reuse the SatelliteId-r18 to identify either an NR satellite or an IoT NTN satellite

4.	Consider a solution that avoids repeating the ephemeris for a satellite which provides both IoT NTN and NR NTN cells

5.	maxSat-r17 (4) is reused for the maximum number of NR satellites.

6.	RAN2 will not do further work to introduce multiple SMTCs in LTE

7.	NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

8.	RAN2 confirms that measurements of NR NTN cells for a UE in E-UTRAN TN RRC_INACTIVE are supported, with the understanding that UE moves to RRC idle upon selecting the NR NTN cell

9.	Introduce a new UE capability without signalling for LTE TN to NR NTN mobility.

R2-2406745	Discussion on LTE to NR NTN mobility	InterDigital, Inc.	discussion	Rel-19

R2-2406774	Discussion on LTE to NR NTN idle mode mobility	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2406848	On the Remaining Issues for E-UTRA TN to NR NTN Mobility in IDLE mode	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3

R2-2406873	NR satellite assistance information provisioning for LTE	Lenovo	discussion	Rel-19

R2-2406973	Considerations on LTE to NR mobility	CMCC	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2406996	Consideration on  idle mode mobility between LTE TN and NR NTN	ZTE Corporation, Sanechips	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2407017	LTE TN to NR NTN Idle Mode Mobility	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2407036	Discussion on cell reselection from E-UTRA TN to NR NTN	MediaTek Inc.	discussion	LTE_TN_NR_NTN_mob-Core

R2-2407055	Discussion on the cell reselection from EUTRA TN to NR NTN	Xiaomi	discussion

R2-2407235	E-UTRAN TN to NR-NTN mobility	Ericsson	discussion	LTE_TN_NR_NTN_mob

R2-2407309	Discussion on LTE TN to NR NTN mobility	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Withdrawn?

R2-2407549	Views on LTE to NR-NTN Mobility	Inmarsat, Viasat	discussion	LTE_TN_NR_NTN_mob	Late

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-241624)

Time budget: 1 TU

Tdoc Limitation: 2 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2406240	LS on FS_5GSAT_Ph3_ARCH conclusions (S2-2407350; contact: OPPO)	SA2	LS in	Rel-19	FS_5GSAT_Ph3_ARCH	To:SA3, SA3-LI	Cc:RAN2

Noted

R2-2406245	Reply LS on FS_5GSAT_Ph3_ARCH conclusions (s3i240477; contact: Tencastle)	SA3-LI	LS in	Rel-19	FS_5GSAT_Ph3_ARCH	To:SA2	Cc:SA3, RAN2

Noted

WI CR Rapporteurs:

36.300:	Ericsson

36.304:	Nokia

36.321:	MediaTek

36.331:	Huawei

36.306:	Qualcomm

### 8.9.2	Support of Store & Forward

Contributions should focus on possible impacts to the radio interface.

R2-2407237	Support for store and forward in IoT NTN	Ericsson	discussion	IoT_NTN_Ph3-Core

- Architecture

Proposal 1	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion. This reverts the previous agreement to consider a scenario where only the full eNB is onboard the satellite.

-	ZTE thinks previous agreement were conditional to the support of eNB only and don’t need to be undone

RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

Proposal 2	RAN2 to address general scenarios that may be common for both architectures.

Proposal 3	Consider both single satellite pass and multiple satellite pass scenarios.

RAN2 will consider both single satellite pass and multiple satellite pass scenarios

Proposal 4	Include both MO and MT data within scope.

RAN2 will consider both MO and MT data within scope

- S&F Indication

Proposal 5	UE is informed whether its serving satellite is currently operating in S&F mode (without feeder link connectivity) via System Information broadcast.

-	Oppo understands that this would be a dynamic indication but wonders if should rather have a static indication

-	Nokia thinks the presence of the field could also realize a static indication

-	MTK thinks that at least we need a static indication that the NW supports this functionality.

-	Telit thinks we need a dynamic indication as this impacts the UE behaviour.

-	Google agrees a dynamic indication would be needed

-	QC thinks we could remove the part in brackets at least for now: S&F operation could happened without feeder link connectivity or with feeder link connectivity but MME still operating in S&F mode (if this will be possible scenario))

-	Telit and CATT don’t think a static indication is needed

UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

Proposal 6	UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

-	Ericsson thinks this could be realized with the introduction of a t-service like parameter

-	QC is not sure how this would be used. Samsung agrees the UE behaviour is not clear. CATT agrees

-	ZTE is interested in this proposal

-	Sequans thinks we need an absolute time. Ericsson confirms this is the intention

-	IDC thinks this could be useful

-	Telit thinks that if the timer is shorter than t-service this would rather bring additional problems to the NW/system behaviour

-	Google thinks an indication could be beneficial also for R19 UUs not supporting S&F operation.

Come back to this in the next meeting

- Legacy UEs

Proposal 7	Send an LS to SA2 to ask whether UEs that do not support S&F enhancements (legacy or Rel-19) should be barred from a S&F network.

- Info on neighbour satellites

Proposal 8	The UE may be informed of the S&F support status of neighbour and upcoming satellites.

R2-2407075	Radio-Interface Impacts for IoT-NTN SF Operations	Nokia, Nokia Shanghai Bell	discussion

Proposal 1: RAN2 to revisit the signallng steps agreed in RAN2-126 to reflect the modified CN architecture options agreed in SA2.  RAN2 to consider the modified steps provided in section 2.1 as starting point for the signaling procedure changes.

Proposal 2: RAN2 to discuss whether further indication is needed in system-information to indicate the MME Split option.

Proposal 3:  RAN2 to confirm multi-satellite operation as basis for further analysis.

Proposal 4:  RAN2 to discuss the options for indication of mode transition (from SF to Normal and vice-versa) towards UE.

Proposal 5:  RAN2 to investigate access control and scheduling related changes for S&F operation.

Proposal 6:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

Proposal 7:  RAN2 to investigate the security context exchange across IoT-NTN nodes in CIoT-UP SF operation and the initial analysis on security impacts for this scenario.

R2-2406689	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: It is feasible for legacy IoT NTN UE to operate in full CN onboard satellite deployment.

-	QC thinks that at initial Attach the MME would take time to check the credentials and would reject the request. Legacy UEs could not handle this. vivo agrees and thinks we should sned an LS to SA2

-	Telit thinks we need to consider also the MVNO case and whether roaming agreements are available

-	MTK agrees and thinks we need to send a LS to SA2

-	ZTE thinks that in a possible LS to SA2 we would need to convey the RAN2 view on what is possible from AS perspective.

-	ZTE does not think we need a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs

-	ZTE really hope we can avoid introducing unnecessary bits in MIB/SIB for new barring options.

RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

Proposal 2: RAN2 to discuss whether cellBarred-NTN bit in SIB1 should be always set to barred to forbid legacy UE camping on the S&F mode satellite with partial CN.

Proposal 3: If legacy IoT NTN UE can camp on S&F mode satellite, a new Rel-19 barring bit is not justified.

Proposal 4: S&F indication comprises of starting time point and duration of network S&F mode.

Proposal 5: For RRC connected UE, UE does not automatically go to idle state due to network switching into S&F mode. Network configuration can make sure that, only DRB(s) with tolerant latency are maintained when satellite switches to S&F mode.

Proposal 6: RRC idle UE determines when to monitor paging messages based on current and past feeder link status.

- If camping satellite had once recovered feeder link, UE monitors paging message and perform data reception.

- If camping satellite had never recovered feeder link, UE may skip monitoring paging message.

R2-2407256	On SA2 progress and RAN2 aspects of Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2: A UE capable of S&F can be assumed to be discontinuous coverage-capable.

Proposal 3: S&F information provided by NAS can be used for discontinuous coverage operation.

R2-2406536	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot, Thales	discussion	Rel-19	R2-2404979	Revised

- Revised in R2-2407487

R2-2407487	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot, Thales	discussion	Rel-19	R2-2406536

- S&F Indication

Proposal 1: UE should receive an indication whether S&F satellite operation is activated in an NTN cell or not.

Proposal 2: If the S&F satellite operation mode is indicated to be as not activated, or no S&F satellite operation mode indication is provided (e.g. in a pre-Rel-19 NTN cell), the UE should assume that the NTN cell is operating in RT mode (i.e. default/normal mode).

- Legacy UEs

Proposal 3: Given that a pre-Rel-19 UE will not be able to decode the above proposed S&F indicator, it needs to be discussed whether and, in case, how pre-Rel-19 UEs could be barred from accessing to an NTN cell operating in S&F mode upon operator policy.

- Additional indications

Proposal 4: For S&F Satellite operation expected maximum delivery time should be indicated to the UE.

Proposal 5: Satellite should be able to indicate (broadcast) the duration during which  S&F or RT operation are respectively available or unavailable.

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

R2-2406251	RAN2 Aspect for S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406283	RAN2 aspects of the Store and Forward satellite operation	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406326	Discussion on support of store and forward operation	CATT	discussion	IoT_NTN_Ph3-Core

R2-2406526	Discussion on information for Store & Forward	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406570	Discussion on the S&F indication	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406639	Support of S&F mode operation	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406771	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406821	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2405132

R2-2406874	Store and Forward support in IoT NTN	Lenovo	discussion	Rel-19

R2-2406906	The design of radio interface for IoT NTN Store & Forward	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406967	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407018	Support  of Store and Forward	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407056	Discussion on the support of store and forward	Xiaomi	discussion

R2-2407152	Further consideration on S&F operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2404882

R2-2407233	AS Security for Store & Forward Satellite Operation	SHARP Corporation	discussion

R2-2407353	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407491	Consideration on S&F operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2407537	Support of Store & Forward	Sequans Communications	discussion	Rel-19	IoT_NTN_Ph3-Core

Withdrawn

R2-2407027	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

### 8.9.3	Uplink Capacity Enhancement

Contributions should focus on the possible enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

R2-2407502	Discussion on DSA and CRDSA Performance	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot	discussion	Rel-19

Observation 1: The DSA solution is improving the Msg3-EDT throughput by about a factor of 4.5 without any impacts on the current receiver implementation at the network, and minimal specification impact overall.

Observation 2: Observation 2: The CRDSA solution is improving the Msg3-EDT throughput by about a factor from 20 to 40, depending on the selected configuration.

Proposal 1: RAN2 to specify the support of Msg#3-EDT replicas.

Proposal 2: RAN2 to specify the indexing strategy of the Msg#3-EDT replicas, when multiple copies are transmitted.

-	vivo thinks we could ask RAN1 if OCC could also be used.

WI objective:

- Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

  - Msg3 transmission without msg1/ Random Access Response (RAR)

  - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

  - Study and specify RRM requirement, if identified [RAN4]

CB Thursday to check whether we can agree:

- to specify a CB-msg3 EDT-like mechanism (or we rather look for improvements for 4-step RA)

- in case we go for a CB-msg3 EDT-like mechanism, whether a DSA mechanisms can be considered (rather than the current SA mechanism)

- in case we go for a CB-msg3 EDT-like mechanism, whether a CRDSA mechanism can also be considered (with the understanding that this will inevitably require RAN1 involvement, i.e. an LS to RAN1 or discussion in the plenary)

or we postpone the whole discussion to the next meeting…

-	session chair thinks a CB-msg3 EDT-like mechanism would fulfil the WI objective even if, at least in terms of usage of UL resources, there may be a load threshold when CB-msg3 EDT-like mechanisms do not bring additional benefit of top of a 4-step RA mechanism

RAN2 will continue working on a CB-msg3 EDT-like mechanism

RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

-	session chair thinks companies not intending to suggest other schemes than SA/DSA/CRDSA are not requested to bring new simulation results

R2-2406640	Discussion on EDT enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	For the first objective, the enhancement of Msg3 (RRC connection request/UL data) transmission over contention-based common PUSCH resource is considered.

Proposal 2	Similar to PRACH resource pool, a pool of contention-based common PUSCH resources per CE level is considered.

Proposal 3	The RACH-less EDT establishment cause can be mo-Data or mo-Signalling or mo-ExceptionData or delayTolerantAccess or mo-VoiceCall.

Proposal 4	A new RNTI to monitor response is derived from the resource of the contention-based common PUSCH occasion where Msg3 transmission occurs.

Proposal 5	For the first objective, confirm that, for Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Proposal 6	For the first objective, Support enhanced random access techniques, such as Contention Resolution Diversity Slotted Aloha (CRDSA), for improved random-access performance of msg3 transmission without NPRACH.

Proposal 7	For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

Proposal 8	For the second objective, multicast Msg4 (multi-user Msg4 multiplexing) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

R2-2407552	UL capacity enhancements objectives for IoT NTN	Ericsson	discussion	Rel-19

- Revised in R2-2407555

R2-2407555	UL capacity enhancements objectives for IoT NTN	Ericsson	discussion	Rel-19

Proposal 1	The Rel-19 enhancement of EDT without Msg1 and Msg2 transmission is referred to as preamble-less EDT.

Proposal 2	As there are no capacity benefits for the preamble-less EDT enhancements compared to 4-step RA, there is no reason to introduces preamble-less EDT – therefore RAN2 shall focus on efficient delivery of msg4 for EDT.

-	ESA and Inmarsat wonder if the shown results are correct.

Proposal 3	In case preamble-less EDT is agreed beneficial, System Information is used to provide preamble-less EDT cell specific PUSCH resources for Msg3 transmission. FFS on signalling details.

Proposal 4	In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

Proposal 5	In case preamble-less EDT is agreed beneficial, Preamble-less EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold and largest TBS for Msg3 transmission.

Proposal 6	In case preamble-less EDT is agreed beneficial, Preamble-less EDT does not require a running TAT for Msg3 transmission.

Proposal 7	In case preamble-less EDT is agreed beneficial, After a failure in Msg3 transmission, the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power.

Proposal 8	In case preamble-less EDT is agreed beneficial, After a number of failed Msg 3 transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

Proposal 9	In case preamble-less EDT is agreed beneficial, The RNTI used to schedule Msg4 transmission is derived based on the PUSCH time-frequency resource(s) used for contention based Msg3 EDT transmission.

Proposal 10	Introduce multiplexing of several UE Contention Resolution Identities in Msg4.

Proposal 11	In case preamble-less EDT is agreed beneficial, For DSA and CRDSA, RAN2 shall first discuss whether it is feasible to integrate them with repetition.

This aspect can be considered in the evaluation in the next meeting

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

R2-2406327	Consideration on the feasibility of RAN2 scope for UL capacity enhancements	CATT	discussion	IoT_NTN_Ph3-Core

Observation 0: RAN2 needs to address the issue on how gNB handles collisions of multiple UEs' CB-Msg3 transmissions on the same PUSCH resources, in order for successful decoding of each UE's transmission. Otherwise, the severe collision degrades resource efficiency, resulting in the feasibility of such CB Msg3 transmission skeptical, especially on whether such CB-Msg3 transmission can really improve UL capability.

Observation 1: OCC allows multiple UEs to transmit at the same time to improve the uplink capability for Msg3-EDT transmission without msg1/RAR, but it has rigorous requirement on symbol/slot alignment which involves RAN1 justification. Also, multiplexing more than 4 UEs are not supported for NPUSCH format 1, as determined for this release by RAN1.

Observation 2: 2-step RACH may realize Msg2 overhead saving purpose, and may be able to resolve the contention resolution/TA issues as well. However, it has never been introduced in LTE Specs, and whether it is in/out of WI is questionable.

Observation 3: Using DM-RS may be one way to differentiate transmissions from different UEs on the same PUSCH resource. But this needs to be done together with MU-MIMO which requires effective tracking of different UEs' channel status at the gNB, and is thus difficult to be realized for IDLE UEs.

Observation 4: (CR)DSA relies on the repetition/retransmission with different resource patterns in the time domain to handle collisions among different UEs. However it may involve non-trivial discussions/Spec impacts on RAN1/RAN2 and the benefit to UL capability improvement may need to be justified first.

Proposal 0: RAN2 waits for RAN1/RAN4 response before carrying out any further discussions on TA related issue for CB-Msg3 transmission.

Proposal 1: RAN2 discusses the feasibility of CB-Msg3 transmission and justifies how the gNB handles the transmission collision on the same PUSCH resource by different UEs, by taking into account the following candidates and the UL capability gain they can bring:

- Code-domain collision handling, e.g. via OCC, 2-Step RACH, DMRS.

- Time domain collision handling, e.g. (CR)DSA.

R2-2407153	Further consideration on uplink capacity enhancements in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2404884

Proposal 1: It’s suggested not to support CRDSA in R19 IoT NTN.

Proposal 2a: If contention-based Msg3 transmission is supported, when the camping cell of the UE changes, the UE doesn’t need to stop using this function.

Proposal 2b: If RAN1 and RAN4 can confirm that RRC Idle UE with a pre-compensated TA can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2, the timing alignment validation condition in legacy PUR (including both PUR TA timer-related criteria and RSRP-related criteria) would not be needed for contention-based Msg3 transmission in R19 IoT NTN.

Proposal 3a: With reference to legacy PUR configuration, RAN2 can take the following resources types as start point for the discussion on resources configuration for contention-based Msg3 transmission:

- Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of NPUSCH occasions

- Frequency domain resources, e.g., Carrier, SubCarrier

- TBS

- NPDCCH search space window

- OCC resource

Proposal 3b: RAN2 is suggested to discuss/evaluate the following alternatives for configuring resources for contention-based Msg3 transmission:

•	Alt1: To provide a totally shared resource via SIB.

•	Alt2: To provide (partially) shared or dedicated resources via dedicated signaling. It can be allowed to configure some different resources for different UEs while the other resources may be same for all the UEs.

•	Alt3: To provide multiple sets of shared resources via the SIB and NW can indicate index of the resource set for a specific UE via the dedicated signaling.

Proposal 4: The existing types of “Msg4” for normal PUR process, e.g., Physical layer L1 ACK, Timing Advance Command MAC CE, and RRCEarlyDataComplete/RRCConnectionRelease/RRCConnectionSetup/ RRCConnectionResume, need to be supported for contention-based Msg3 transmission scheme. The very simple Physical layer L1 ACK can be used in suitable cases to achieve the most efficient Msg4 transmission.

Proposal 5: RAN is suggested to discuss the following alternatives for scheduling of Msg4:

•	Alt1: Common PUR-RNTI configured in the shared resources configuration.

•	Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

•	Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE for Msg3 transmission.

Proposal 6: The contention resolution scheme in legacy random access procedure can be reused for contention-based Msg3 transmission, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 7: RAN2 is suggested to discuss the following other issues:

•	Issue#1: How to handle the coexistence of dedicated PUR and contention-based Msg3 transmission

•	Issue#2: Whether a backoff scheme is needed when Msg3 transmission collision occurs

R2-2407140	Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

Proposal 2: Confirm that it is already possible with the existing PUR feature to terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 3: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Proposal 4: Confirm that it is already possible with the existing PUR feature for UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Proposal 5: Confirm that the existing PUR feature may not be optimal in an NTN deployment as there is currently no mechanism for the eNB to determine whether UE expects a downlink application layer response unless the UE moves to RRC_CONNECTED in every cell to perform dedicated PUR configuration.

Proposal 6: RAN2 to consider whether the following enhancements are beneficial:

1)	Introducing network signalling so that PUR configuration request information to be transferred across cells.

2)	Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter (whether UE expects a downlink application layer response)

3)	Enabling EDT termination without any downlink ACK, or using a common ACK for multiple transmissions or UEs

R2-2406252	Further Discussion on EDT Enhancement for IoT-NTN	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406284	Way forward for RAN2 discussion on UL capacity enhancement	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406592	Discussion on uplink capacity enhancements for IOT NTN	Beijing Xiaomi Mobile Software	discussion	Rel-19

R2-2406593	Performance of Advanced Random Access Protocols	DLR	discussion	Rel-19

R2-2406688	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406763	Uplink Capacity Enhancement for EDT transaction	Spreadtrum Communications	discussion	Rel-19

R2-2406766	Discussion on enhanced EDT for IoT NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406868	Consideration on UL capacity enhancement for IoT-NTN	NEC Corporation.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406869	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2405133

R2-2406875	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2406907	Contention-based Msg3-EDT for IoT NTN capacity enhancement	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406974	Considerations on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407028	Discussion on uplink capacity enhancement	Transsion Holdings	discussion	Rel-19

R2-2407121	Discussion on Contention Resolution Diversity Slotted ALOHA	TOYOTA Info Technology Center	other	Rel-19	IoT_NTN_Ph3-Core

R2-2407139	Msg3 transmission without msg1/RAR	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407167	Consideration on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407257	Procedures for uplink capacity enhancements for IoT NTN	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Withdrawn?

R2-2407546	Views on UL Capacity Enhancements for IoT-NTN	Inmarsat, Viasat	discussion	Rel-19	NR_NTN_Ph3-Core	Late

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM, CHO with candidate SCGs, subsequent CPAC

LTM

R2-2407333	Discussion on MRO enhancements for Rel-18 mobility features	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1-P7

Proposal 1: only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, no specification impact for timeConnFailure and reconnectCellId is foreseen.

Proposal 2: introduce a new field in RLF report to indicate the LTM recovery cell.

Proposal 3: RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch.

Proposal 4: RAN2 include in the RLF report whether the network provides the TA in the cell switch command.

Proposal 5: RAN2 include in the RLF report the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

Proposal 6: In case the UE is configured with LTM config, the RA-based access could be one triggering condition for the SHR report.

Proposal 7: In case the UE is configured with LTM config, the HO interruption time could be one triggering condition for the SHR report.

For P3:

-	Vivo think the NW can know this already. Huawei think that it may be implicitly known.

P4:

-	Lenovo think the UE context makes this clear. Qualcomm wonders what the use is? LG thinks there are good use for this. Samsung think it is not useful. ZTE supports the proposal. Ericsson think we can leave TA-related details to a later discussion.

P5:

-	Qualcomm think the NW should know this. Huawei think network can know but the PDCCH order is L1 signal so its complex for the NW to know this. Samsung is not supportive.

P6:

-	vivo think the SHR is for near failure but RACH based is not near failure, hence is not supportive. Nokia likes P6 since RA is an indication that LTM almost failed. CATT want to first confirm that the legacy conditions can be reused. CATT is supportive of P6 though. Samsung think that T304 condition is sufficient and NW can set good thresholds. Qualcomm agrees with Samsung. ZTE agrees with Samsung and Qualcomm.

P7:

-	LG and vivo thinks this is like T304 threshold.

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

introduce a new field in RLF report to indicate the LTM recovery cell id.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report

R2-2407064	Discussion on MRO enhancement for LTM	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

Proposal 2: Define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

Proposal 3: RAN2 to discuss whether to log the following timing information in SON reports (e.g. RLF report) for LTM:

T3: the lapsed time between UE obtains the target cell TA measured by itself and the moment UE receives the cell switch command

T3’: the elapsed time between UE sends preamble according to PDDCH order and the moment UE receives the cell switch command

Proposal 4: RAN2 to discuss whether to log interruption time in SON reports (e.g. SHR) for LTM. The interruption time is defined as the elapsed time between UE receives cell switch command and the moment UE completes the first DL/UL reception/ transmission on the indicated beam of the target cell.

Proposal 5: Define and log ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not.

Proposal 6: If available, log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

On P1:

-

On P2:

-	Qualcomm is not sure about the usage, long/short time does not tell you much, could have to do with UE mobility. Ericsson think that it is available for CHO so they want it for LTM too. Lenovo think CHO is different since for CHO the UE triggers the CHO, while for LTM it’s the NW that is in control and NW can know. Ericsson think here we talk about failure. CMCC want to limit it to failure cases. Qualcomm is not happy even if limited to failure. Vivo think this is useless.

On P3:

-	vivo think we can skip this as it has with TA info to do. CMCC think we should do it for UE based TA acquisition. CMCC think the info is good to know if the sync info is valid or poor and resulted in a failure. Ericsson agrees with CMCC. Samsung does not understand what is the point of this. Qualcomm think the info can be deduced from info if the UE did RA or RA-less. ZTE agrees with the proposal since a failure may be due to poor sync and the NW wants to know this. LG agrees with ZTE.

On P4:

-	ZTE think we that we have UP interruption and that is perhaps good enough. Lenovo think the is not interested in this.vivo think T304 threshold is good enough. LG agrees with vivo. Nokia think this is not needed.

On P5:

-	Samsung think it is useful but may be known from other info, want to wait to see.

On P6:

-	vivo are OK with this but think it exists already.

Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

We will not log interruption time in SON reports (e.g. SHR) for LTM.

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

CHO with candidate SCGs

R2-2407052	MRO enhancements for Rel-18 mobility features	Samsung	discussion

Focus on P3-1 and P3-2

Proposal 3-1: UE includes following information in RLF report/SCGFailureInformation and SHR/SPR:

a.	Identifier of fulfilled execution conditions for CHO and CPC

b.	Time between fulfilment of CHO/CPC conditions and the occurrence of failure/near failure.

c.	Measurement results of PCells and PSCells which are candidates for CHO with SCG.

Proposal 3-2: UE includes conditional reconfiguration identifier of the fulfilled condition in RLF report/SHR.

-	Lenovo does not think a) is needed. LG are OK with a) but to focus on RLF report for now and need more time for the other messages. Qualcomm also do not understand what info a) mean and also only want to do RLF report. ZTE are OK with b and c, but want to think more about which messages. CMCC explains that for a) there are different sets of conditions. Qualcomm wonders about the details about a). Huawei think that we can focus on RLF report for now.

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

R2-2407005	Discussion on MRO Enhancements for Mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P5-P7

Proposal 5: For bullet “CHO with candidate SCG” in MRO enhancement for mobility, also consider the R18 leftover scenario of “CHO with target SCG”.

Proposal 6: RAN2 to consider how to associate the MCG failure/near failure and SCG failure/near failure information with the following two options:

Option 1: UE records the MCG failure/near failure and SCG failure/near failure information in the same report;

Option 2: UE records the MCG failure/near failure and SCG failure/near failure information in different reports, and the association information is needed in the two reports.

Proposal 7: The timing different threshold between PCell execution condition being fulfilled and PSCell execution condition being fulfilled is considered as the trigger condition for successful CHO with candidate SCGs execution.

Subsequent CPAC

R2-2406959	Discussion on MRO enhancements for R18 mobility features	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P12

Proposal 12: Include the subsequent CPAC related information in MHI, e.g. the visited SCPAC candidate PSCell(s), the time stays in the candidate PSCell(s), the PSCell(s) measurement result with the indication about the PSCell(s) is for SCPAC, etc.

-	LG is OK with the proposal, but this seems to be existing already. Qualcomm agrees. Lenovo wants to check the spec in more detail. Session chair thinks that companies seem to need more time to check the spec and we can come back to this next meeting.

R2-2406883	Discussion on MRO for R18 mobility	Lenovo	discussion	Rel-19

Focus on P11 and P12

Proposal 11: An indication concerning the SCG Failure Information message is for an initial CPAC execution or a following subsequent CPC execution may be included in the SCG Failure Information message.

Proposal 12: RAN2 to discuss whether SPR can be reused for subsequent CPAC.

R2-2406527	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407065	Discussion on MRO enhancement for CHO with candidate SCGs	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2407094	MRO enhancements for LTM	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407029	MRO for Rel-18 mobility	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407095	MRO for CHO with candidate SCG(s)	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407099	MRO enhancement for SON and MDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2407105	MRO for R18 Mobility	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407119	MRO for LTM	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407120	MRO for CHO with candidate SCG	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407122	Configuring UE based TA acquisition for LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2407191	SON/MDT reports for LTM 	Kyocera	discussion

R2-2407218	SON support for MRO	Ericsson	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2407362	Discussion on MRO enhancement for R18 mobility features	Sharp	discussion

R2-2407386	MRO for Rel-18 mobility features	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

### 8.10.3	SON/MDT for Slicing

No contributions are expected and this AI will not be treated in RAN2#127, in wait for RAN3 progresses

### 8.10.4	SON/MDT for NTN

No contributions are expected and this AI will not be treated in RAN2#127, in wait for RAN3 progresses

R2-2407106	Logging for Unchanged PCI Mobility in NTN	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

### 8.10.5	Leftovers from Rel-18

RACH optimization for SDT, MHI Enhancement for SCG Deactivation/Activation, MRO for MR-DC SCG failure

SCG act/deact info

R2-2406986	MHI Enhancement for SCG Deactivation/Activation	CMCC, CATT, Ericsson, ZTE, Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal: Include in MHI the information of SCG activation/deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation.

R2-2407100	SON and MDT Rel-18 leftover issues	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Focus on P3

Proposal 3	Send LS to RAN3 to confirm the feasibility of network based solution for collecting percentage of time spent in the PSCell in activated state or deactivated state.

-	Qualcomm think the NW can figure this out. Nokia agrees. CMCC think the IDLE/INACTIVE cases can only be logged by the UE. ZTE agrees with CMCC that the MHI can log thinks which the UHI cannot. Nokia is not sure about the point of having this information for IDLE. ZTE think it is not about act/deact in IDLE, it is about MHI that the UE shall indicate time of stay. Huawei think there are some unclarities and want more time.

It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

[Post127][603][SONMDT] SCG act/deact info (CMCC)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approved LS in R2-2407664 (CMCC)

Deadline:

    - Short

=> Approved in R2-2407664 (LSout)

MR-DC SCG failure

R2-2406884	Discussion on MRO for MR-DC SCG failure	Lenovo	discussion	Rel-19

Proposal 1: To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure and RA info.

Proposal 2: To support MRO for SCG failure in EN-DC, RAN2 to discuss whether to introduce a new inter-node message or reuse the CG-ConfigInfo inter-node message to forward SCG failure information from MN to SN.

To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure.

SDT

R2-2407037	SON/MDT enhancements for leftover topics from R18	Samsung	discussion

Focus on P3 and P4

Proposal 3: If the UE evaluates if it should perform SDT, UE logs sdt-RSRP-Threshold.

Proposal 4: UE logs the following failure causes of SDT to the network:

•	if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

•	if random access problem indication is received from MCG MAC while SDT procedure is ongoing.

•	if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

•	if integrity check failure indication is received from lower layers while SDT procedure is ongoing.

•	if T319a expires.

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

R2-2407006	Consideration on leftovers from Rel-18 SONMDT	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407030	Rel-18 leftovers for SON MDT	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407249	On Rel.18 leftovers	Ericsson	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2407334	Discussion on support of the Rel-18 leftovers	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2407364	Discussion on R18 leftovers for SON MDT	Sharp	discussion

R2-2407387	RACH optimization for SDT	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP‑241614)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, including workplan, etc..

R2-2406314	RAN2 workplan for Rel-19 Evolution of NR duplex operation	Huawei, HiSilicon	Work Plan	Rel-19	NR_duplex_evo-Core

Noted

### 8.11.2	Random access in SBFD

RAN2 impacts to support SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

R2-2406452	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Proposal 1: RAN2 to discuss for which triggering events, random access procedure in SBFD symbols is supported.

Proposal 2: RAN2 to confirm that the network provides the RACH configuration for SBFD via SIB. Whether to provide the RACH configuration for SBFD via dedicated RRC signalling is pending on RAN1’s progress.

Proposal 3: RAN2 to confirm that RACH configuration for SBFD can be either based on the legacy RACH parameters or a new set of RACH parameters, and to discuss the signalling design for both options.

Proposal 4: RAN2 to consider the criterion for the SBFD-aware UE to select between the PRACH resource in SBFD symbols and the PRACH resource in non-SBFD symbols.

Proposal 5: RAN2 to discuss new fall-back behaviour if UE fails on the PRACH resource in SBFD symbols.

Discussion

P1

- ZTE agree with P1, and suggest to make it a working assumption.

- LG E suggest to exclude SI request case.

- CATT, Xiaomi, NEC, QC think all those events can be supported, do not see strong reason to drop any of them.

- Apple think for P1 we also need to consider RACH resource.

- Ericsson think the main benefit of SBFD is to reduce the latency, so we may look at those latency sensitive cases. Nokia share this view.

- Nokia think we first agree on the cases for UEs in connected state.

- Nokia and LG E suggest to focus on connected state use case.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

P2

- Xiaomi wonders this is for connected UE or also for idle/inactive. HW clarifies that it is for both.

- ZTE think dedicated signalling can be confirmed as well. Ericsson think main benefit is for connected mode, so also suggest to confirm dedicated singaling is supported.

RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

P3

- LG E think it is ok since it confirms R1 agreement.

- Apple not sure if this implies NW can configure two configurations for a UE at the same time, and think only one is configured in a cell.

- HW clarifies that P3 is to confirm what R1 agreed from R2 perspective, nothing new.

- Ericsson, Goolge think we do not need to confirm this in R2.

P4

- ZTE not sure whether such selection is before or after selection of ‘configuration’. CATT ok to discuss. Xiaomi think such selection is up to UE’s implementation.

- Samsung think we do not discuss selection criteria in R2, we can wait for R1.

- Ericsson think at some point we need to discuss this, this cannot be completely left to UE implementation.

- Nokia wonders whether P4 is only for the configuration option 2 in R1.

P5

- Xiaomi think it introduces complexity and would like to leave it to UE implementation.

R2-2407078	Discussion on subband full duplex (SBFD) RA operation	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Discussion

P1

- CATT, Google, Nokia support P1.

- Xiaomi want to make it a working assumption.

- Vivo want to understand what the common design means.

- LG E think for CFRA we need dedicated signalling, and think we could confirm this for CBRA.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

P6

- LG E, Sony think it too early to discuss P6. Xiaomi think P6 is related to many other aspects that we need to discuss further, so prefer to skip for it right now.

P7

- Nokia think this is not needed for configuration Option 1.

- ZTE, Sony think this is only related to UE sending RACH using legacy RA resource but not SBFD resource. CATT has different view and think UE may indicate in Msg3.

- Xiaomi think this can wait for R1.

- QC think early indication is needed.

- Ericsson think considering the existing features, there is no much room/tool for extra early indication. Samsung share this understanding and wonders how idle UEs do this.

- Apple agree with ZTE for msg1, but for legacy RO case, there is technical issue to handle.

- LG E has slight preference of introducing early indication for msg1.

- HW also think it is ok to wait for R1.

- ZTE suggest to add msg1-based as an example. CATT think the key point is whether SBFD is treated as a feature in RA.

FFS whether/how early indication is used during a SBFD RA procedure.

P8

- Google think 2step is also covered.

- HW, Nokia think in R1 2step is FFS, so in R2 we just discuss 4step.

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

R2-2406342	Random Access for SBFD Operation	NEC	discussion

R2-2406363	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19

R2-2406486	Discussion on SBFD random access operation	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2406630	SBFD configuration and supporting Random access	Sony	discussion	Rel-19	NR_duplex_evo-Core

R2-2406690	Framework to support RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

R2-2406724	Discussion on random access in SBFD	vivo	discussion	Rel-19

R2-2406794	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2406822	Random Access Aspect of SBFD	Nokia Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2406962	Discussion on random access in SBFD	CMCC	discussion	Rel-19	NR_duplex_evo-Core

R2-2407143	Random Access in Sub-Band Full Duplex	Google Ireland Limited	discussion

R2-2407192	Views on random access for SBFD 	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2407313	Random access in SBFD	Samsung	discussion	Rel-19

R2-2407461	Discussion on Random Access procedure for SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

### 8.11.3	Other aspects

Other RAN2 impacts with SBFD if not covered by the previous agenda items.

R2-2407427	SBFD Overall and Support of Cross Link Interference	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Proposal 1: Source gNB request SBFD support and configuration (semi-static cell-specific SBFD time/frequency configuration) to other gNBs in Xn Setup Request or NG-RAN node Configuration Update message

Proposal 2: Semi-static cell-specific SBFD time/frequency configuration is provided by SIB1.

Proposal 3: RAN2 wait the RAN1 decision on support of the SBFD configuration by the dedicated RRC configuration.

Proposal 4: For the support gNB-to-gNB CLI handling, RAN2 wait the RAN1 parameter list on UL resource muting for PUSCH i.e. only RRC signalling support is expected.

Proposal 5: For L1 based UE-to-UE CLI measurement and reporting, the legacy CSI resource and report structure is reused and need to check how to exchange the information on the activation/deactivation of SRS at aggressor gNB.

Discussion on P2 and P3

- LG E suggest to add ‘at least in P2’, HW do not see a need.

- Ericsson think P3 is not clear, think dedicated signalling is not needed. CATT agree.

- Charter support P2.

- CATT think no need for dedicated signalling.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

R2-2406410	SBFD resource indication and CLI handling	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2406466	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2406487	Discussion on other aspects of SBFD	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2406725	Discussion on other issues in SBFD	vivo	discussion	Rel-19

R2-2406795	Discussion on CLI measurement in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2406957	Other aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core

R2-2406983	Discussion on SBFD related issues	CMCC	discussion	Rel-19	NR_duplex_evo-Core

R2-2407079	Non-RA aspects for subband full duplex (SBFD) operation	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2407194	Other aspects of SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-240087)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

This WI will not be treated in RAN2#127, therefore no contribution is expected under agenda item 8.12.

## 8.13	NR sidelink multi-hop relay

(NR_SL_relay_enh2; leading WG: RAN2; REL-19; WID: RP-241609)

Time budget: 1 TU

Tdoc Limitation: 2 tdocs

### 8.13.1	Organizational

LSs and rapporteur input, including workplan, etc.

Workplan

R2-2407145	Work plan for NR sidelink multi-hop relay	LG Electronics, InterDigital	Work Plan	Rel-19	NR_SL_relay_multihop, NR_SL_relay_multihop-Core

- Noted

CR assignments:

38.300	LG

38.331	Huawei (multi-hop), CATT (service continuity)

Huawei (merge)

38.304	MediaTek

38.321	InterDigital

38.322	Xiaomi

38.323	Ericsson

38.351	OPPO

UE capability	Samsung (38.306 and 38.331)

Discussion:

Nokia think there is a general need for guidelines/openness on assigning CR rapporteurs.

InterDigital note that it is a difficult assignment process and there is an attempt to have some continuity with previous releases.

Apple agree the process could be more open.

Nokia think we should have one rapporteur for the RRC spec, because service continuity here is not really a different functionality and the merge will be difficult.  LG think we can follow the Rel-18 precedent and the merge worked well there.

Nokia think the objectives here are much less separate and a single proposal may implicate both objectives.

InterDigital agree it may not be perfect, but having one company with responsibility for the merge will be the point where we centralize the work.

Nokia think the concern is about the openness of the discussion, and they would like more open discussion in the future.  Chair will raise the issue offline with the group chair.

LG think we need early CR allocation considering the schedule of the item.

Toyota agree with Nokia’s concern on openness.

Other organizational documents

R2-2407147	Terminologies and Scenarios for SL multihop relay	LG Electronics Inc.	discussion	Rel-19	NR_SL_relay_multihop, NR_SL_relay_multihop-Core

- Noted

[Terminology]

Proposal 1: According to the WID, the last relay UE is defined as the relay UE directly connected to gNB on multi-hop relay. Alternatively, the root relay UE can be used with WID revision.’

Discussion:

Huawei think using “U2N relay UE” might be better.  NEC have the same view, considering that the terminology should align with previous releases in case there is a legacy remote UE.

Ericsson agree with Huawei and NEC and think the functionality of the “last” relay UE is the same as a U2N relay UE.  They also understand that SA2 are using the term “U2N relay UE”.

Samsung think the U2N relay functionality can be reused, but we may need terminology to differentiate the “last” relay UE from the intermediate UEs.

vivo think “U2N relay UE” is simpler and there is not a risk of confusion.

LG understand SA2 initially used “U2N relay UE”, but they think SA2 have now reconsidered terminology and this term now applies also to the intermediate UEs.  LG understand we need a clear term to differentiate the UE with a direct connection to the gNB for discussion, but we can discuss if we capture this term in the specs.

InterDigital think we should not spend too much time on terminology, but they see Huawei’s comment as reasonable and think we should be aligned with SA2 terminology.

NEC think this issue affects whether we need a new UE type in the discovery message (as an SA2 issue), and they would like to avoid forcing a new type with our terminology.  NIST indicate that SA2 are discussing the terminology this meeting, and the CR is not yet final but the merged baseline introduces the “intermediate” terminology from P2.

LG think SA2 are considering L3 as well as L2, and the L3 modelling is different.  Here they understand that every intermediate relay UE has to support U2N.

Proposal 2: The intermediate relay UE is defined as the relay UE not directly connected to gNB on multi-hop relay. That is, all relay UEs except the last relay UE on multi-hop relay are called intermediate relay UEs.

Proposal 3: According to the WID, the first relay UE (or the first intermediate relay UE) is defined as the intermediate relay UE directly connected to the remote UE on multi-hop relay.

Proposal 4: All of the last relay UE and intermediate relay UEs on a multi-hop indirect path are L2 U2N relay UEs.

Discussion:

Chair suggestion:

- Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion

- Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion

- Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion

- FFS if we will capture the requirements according to these terms when implementing CRs.

- RAN2 intend to align with SA2 terminology when it is stable

Apple have a serious concern with the “intermediate” terminology, because they do not think it has to duplicate all the U2N functionality; it depends on proposals yet to be seen.

Toyota think we should avoid divergence between stage 2 (inc. SA2) and stage 3 spec terminology.

Kyocera wonder how the terminology applies to a single-hop case with a relay that is multi-hop-capable.

NEC think we may not model the intermediate UEs as being relay UEs at all.

Qualcomm wonder if we need a special term for the second intermediate (relay) UE.

Samsung wonder if we need terminology for the links between relay UEs.

Agreements:

n-hop relaying involves n relay UEs by definition.

Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion

Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion

Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion

FFS if we will capture the requirements according to these terms when implementing CRs.

FFS link terminology.

RAN2 intend to align with SA2 terminology when it is stable

Proposal 5: If necessary, RAN2 can send a LS to other WGs to align terminologies across WGs

[Scenarios]

Proposal 6: A candidate last relay UE can be in RRC_IDLE or RRC_INACTIVE, but not outside network coverage.

Proposal 7: Discuss whether some or all of intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage.

Discussion:

Lenovo think on P6, if the last relay UE is in idle/inactive, it would require that all intermediate relay UEs would be in idle/inactive.

Samsung agree with P6 in principle but wonder how the discovering UE will distinguish whether a candidate is a candidate for last relay UE or intermediate.  Qualcomm think we do not need to make such a distinction from AS layer, but SA2 may distinguish.  vivo understand that it should be possible to determine from the discovery message.

vivo think on P7, the intermediate relay UEs can be in or out of coverage, and SA2 have already established this.

OPPO agree with the intention of P6 but wonder if we need to specify “candidate”, since after the multi-hop connection the last relay UE will still be in whatever RRC state.

Huawei have some concern about P7 if all the intermediate relay UEs are within network coverage, because the benefits of coverage extension would not be clear.  Chair thinks there could be an intermediate relay UE at the edge of coverage.

InterDigital think we should not limit the coverage scenario too much, especially as the relays may be moving around.

Apple think inter-cell cases could be complicated and it may be premature to discuss them.

Agreements:

A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage.  FFS if acting as a relay requires it to transition to RRC_CONNECTED.

Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage.

Proposal 8: Discuss whether some or all of intermediate relay UEs inside network coverage can be RRC_IDLE, RRC_INACTIVE or ‘indirect’ RRC_CONNECTED for the multi-hop indirect path.

Discussion:

LG think there are related proposals in the control plane agenda item.  They understand that the plenary discussion excluded multi-hop U2U operation, so if we want to support two additional hops, using a U2U model would force us to support multi-hop U2U; thus they think we cannot support a solution under this WI that is based on the U2U design.

Huawei have the same understanding as LG and think we need to focus on the U2N model.

ZTE think the proposal is not intended to discuss the model, but they have concern about the “indirect RRC_CONNECTED” part; they consider that there is no such state, and if we scope the states in this way, RRC_IDLE and RRC_INACTIVE could also be “indirect”.  LG agree there is no “indirect RRC_CONNECTED” state as such, but they mean to capture RRC_CONNECTED for a UE with an indirect connection.

vivo think the issue is whether the intermediate UEs can be seen/controlled by the network, and they agree with Huawei and LG and think that each intermediate UE should be visible to the network.

Qualcomm think we should discuss it under the control plane.

Ericsson echo the understanding of LG but think this can be further discussed under control plane.

Apple think we can borrow some functionality from U2U without specifying U2U multi-hop.

Proposal 9: Discuss whether the last relay UE and intermediate relay UEs on a multi-hop indirect path can be in different cell coverages or should be in the same cell coverage.

R2-2407378	Discussion on Working Assumptions for Multi-hop Relay Mechanisms	InterDigital France R&D, SAS, LG Electronics, FirstNet, Ericsson, AT&T, Qualcomm, Samsung	discussion

- Revised in R2-2407390

R2-2407390	Discussion on Working Assumptions for Multi-hop Relay Mechanisms	InterDigital France R&D, SAS, LG Electronics, FirstNet, Ericsson, AT&T, Qualcomm, Samsung	discussion

- Noted

Proposal 1	RAN2 to focus on developing mechanisms aiming to minimize the impact of hop count on the developed multi-hop relay mechanisms.

Proposal 2	RAN2 to develop mechanism from the onset of Rel19 work which are applicable to at least 2 additional hop relays.

Discussion:

Nokia understand the intention of P2 is not to develop two-hop mechanisms from the beginning but to design in flexibility for a unified mechanism.

Huawei agree with the proposals and think we should aim for a hop-agnostic design.

Samsung think the intention of the second agreement is that we should avoid mechanisms that are limited to two additional hops.  Qualcomm understand that the intention is to consider two additional hops from the beginning.

LG think the solution can be hop-agnostic and we could design for extra hops if it works.

AT&T think we might dimension the design to accommodate a fixed number of hops, and their goal is for it to be agnostic.

Agreements:

RAN2 intend to minimize the impact of hop count on the multi-hop relay mechanisms.

RAN2 intend to design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19.

### 8.13.2	Relay discovery and (re)selection

Enhancements to relay dscovery and (re)selection to support one additional hop relay (remote UE  first relay UE  last relay UE  gNB). Extensibility to a second additional hop in this WI is considered as a design criterion.

R2-2407101	Discovery and Relay (re)selection for multi-hop U2N relay	Qualcomm Incorporated	discussion

- Noted

[Discovery basic assumptions]

Proposal 4	Taking the existing discovery mechanisms as baseline for Multi-hop U2N Discovery message

- Reuse existing AS discovery protocol to transmit Multi-hop U2N Discovery message

- Reuse SL-SRB4 transmit Multi-hop U2N Discovery message.

- Reuse existing resource pools defined for discovery message transmission and reception

- Both of resource allocation mode 1 or mode 2 can be supported as today.

- Configuration can be provided by SIB/dedicated message or pre-configured as today

Discussion:

Samsung wonder if this is only between the remote UE and the relay UE or also between two relay UEs.  Ericsson understand that mode 1 implies we are considering also relay UEs, because the remote UE OOC cannot support mode 1.

Huawei are OK with the proposals and think we could add that both models A and B can be supported.

Kyocera wonder if the different UEs in the chain would use different resource pools.  Apple think there is no requirement to align the configuration.  LG think this was resolved by the previous release when we discussed resource pool coordination: It is up to gNB to coordinate.

Agreements:

From RAN2 perspective, models A and B can both be supported.

Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying.

Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying.

Reuse existing resource pools defined for discovery message transmission and reception

Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE.

Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18.

R2-2407294	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

- Noted

[Discovery conditions/thresholds]

Proposal 3: The condition for the Remote UE to perform the discovery for multi-hop Relay can be  one of the following, i.e., if the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could  not find  a serving cell,  the Remote UE can perform discovery procedure.

Proposal 4: Consider the following conditions for the Intermediate Relay UE to perform the discovery.

1.	The PC5 link quality of the Intermediate Relay UE with U2N Relay or another Intermediate Relay UE (for > 2 hops) is above a threshold

2.	The hop number between the Intermediate Relay UE and the gNB is below a threshold

3.	The number of the UE served by the U2N Relay UE of the Intermediate Relay UE is below a threshold.

Discussion:

Samsung understand that P3 is to reuse existing mechanisms and we do not need to list all cases.

vivo think P3 is OK, and we can further discuss if it is the same or a different threshold.  They also think there should be a Uu threshold for the intermediate relay UEs, which could also be based on the legacy operation.

LG think P3 can be discussed along with P6, and they wonder about the definition of “number of UEs served” in item 3 of P4.

NEC think the intermediate relay UE should be a last relay UE if the cell RSRP is above a threshold.

Ericsson disagree with all the proposed bullets for the intermediate relay UE.

InterDigital think we could agree to hop count from RAN2 perspective and see what SA2 do.

OPPO agree with the first bullet and think the intermediate relay UE should not be in cell centre.  In inter-cell cases, they understand that it could cause interference to its own serving cell.  For the hop number, they think it can be considered from SA2 perspective.

AT&T would like to defer the decisions on intermediate relays; they think there will be unusual situations that may affect the criteria.

vivo wonder if we can reuse legacy criteria for the last relay UE.  NEC think we do not need the high threshold, because a relay at cell centre may be needed when the surrounding intermediate relay UEs are in bad coverage.

CATT wonder about the relation to discovery models.

OPPO support the legacy conditions for the last relay UE, and they think there is no need to differentiate multi-hop from single-hop discovery in this setting.

Samsung understand the first two proposed agreements just align with legacy design.

ZTE think on the second proposed agreement, the two thresholds may not both be configured.

Agreements:

If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

FFS discovery conditions for the intermediate relay UEs.

[(Re)selection triggers]

Proposal 6: The Relay selection for multi-hop relay can follow the Release 17 design that U2N Remote UE triggers U2N Relay selection in following cases:

•	The RSRP measurement of the cell on which U2N Remote UE camps is below a configured threshold

•	Indicated by upper layer of the U2N Remote UE.

•	PC5 link quality between the current serving Intermediate Relay UE and remote UE is below a configured threshold.

•	Upon receiving the notification message from the serving Intermediate Relay UE.

•	The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

•	The U2N Remote UE detects the PC5 RLF

•	Indicated by upper layer of the U2N Remote UE.

Proposal 8: The serving Intermediate Relay UE sends the notification message to the U2N Remote UE when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens, or upon receiving the notification message from the U2N Relay that indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE

Email discussion towards RAN2#128 on discovery and (re)selection:

[Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

Scope:

  - Discovery message initiating/forwarding condition at intermediate relay UE

  - PC5 AS conditions for discovery at the last relay UE

  - Reselection triggers for all UEs

  - (Re)selection criteria for all UEs (considering discovery models A/B and whether criteria beyond the first hop are considered)

  - Whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last relay UE via one (physically same) intermediate relay UE

Intended outcome: Report to RAN2#128

Deadline: Very long (for RAN2#128)

R2-2406553	Discussion on multi-hop U2N relay discovery and relay selection	NEC	discussion		NR_SL_relay_multihop

[Suitability criteria]

Proposal 10	A U2N intermediate UE is considered suitable by a U2N remote UE in terms of radio criteria if the PC5 link quality measured by U2N remote UE towards the U2N intermediate UE exceeds configured threshold.

R2-2406714	Discovery and relay (re)selection for multi-hop relay 	InterDigital France R&D, SAS	discussion

[(Re)selection criteria]

Proposal 7:	RAN2 defines rules to select among multiple relays which consider both the number of hops to the gNB and the sidelink RSRP to the parent node.

Proposal 8:	(Re)selection should prioritize a relay with the next hop already established via an existing PC5-RRC connection.

R2-2406365	Discovery and relay (re)selection for multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406528	Discussions on relay discovery for multi-hop U2N Relay	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2406562	Discussion on multi-hop discovery and (re)selection	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406611	Initial considerations on relay discovery and (re)selection	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406632	Multi-hop relay selection/re-selection	Sony	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406683	Relay discovery and selection for Multi-hop UE-to-NW Relay	Apple	discussion	Rel-19	DUMMY

R2-2406695	Discussion on discovery and (re)selection for support of multi-hop SL Relay	ZTE Corporation, Sanechips	discussion	NR_SL_relay_multihop-Core

R2-2406887	Relay discovery and (re)selection in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2406898	Discussion on multi-hop relay discovery and reselection	China Telecom	discussion	Rel-19

R2-2407007	Discussion on multi-hop U2N Relay discovery and (re)selection	vivo	discussion	Rel-19

R2-2407035	discussion on discovery and relay (re)selection	Ericsson, FirstNet, AT&T	discussion	Rel-19

R2-2407057	Discussion on Relay discovery and selection	LG Electronics France	discussion	Rel-19

R2-2407204	Discussion on relay discovery and (re)selection for NR sidelink multi-hop relay	TOYOTA InfoTechnology Center	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2407205	Discovery and (re)selection under multihop relay	Kyocera	discussion

R2-2407316	Relay discovery and reselection for multi-hop relay	Nokia	discussion	NR_SL_relay_multihop

R2-2407402	discussion on Relay discovery and (re)selection for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Withdrawn/Not available

R2-2406735	Discussion on multi-hop relay discovery and (re)selection	vivo	discussion

- Withdrawn

R2-2407224	Discussion on Working Assumptions for Multi-hop Relay Mechanisms	FirstNet, Ericsson, AT&T, LG Electronics, InterDigital, Qualcomm	discussion	Rel-19

=> Withdrawn

### 8.13.3	Control Plane Procedures

Contributions should focus on control plane procedures and can include QoS handling to support additional hops.  NOTE: No service continuity aspects should be discussed in contributions for this meeting.

R2-2407008	Discussion on multi-hop U2N Relay Control Plane Procedures	vivo	discussion	Rel-19

- Noted

[PC5-RRC connections]

Proposal 2	RAN2 to agree the basic assumption for Multi-hop U2N relay path setup (see Figure 1):

-	From U2N Relay UE perspective:

A per-hop PC5 connection with U2N intermediate Relay UE, and

A per-hop Uu connection with serving gNB.

-	From Intermediate U2N Relay UE perspective:

A per-hop PC5 connection with U2N Relay UE, and

A per-hop PC5 connection with U2N Remote UE, and

An end-to-end Uu connection with serving gNB.

-	From U2N Remote UE perspective:

A per-hop PC5 RRC connection with U2N Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB.

Discussion:

vivo clarify that they assume for ongoing services, the whole chain of UEs must be in RRC_CONNECTED.

Apple think this is specific to the single-extra-hop case and the terminology needs to be adjusted.  They also to not see why the intermediate relay UE needs a connection with the gNB.

Ericsson think we cannot assume RRC_CONNECTED now.

Qualcomm think there is ambiguity about which gNB serves the intermediate relay UE, and the Uu connections are obvious if we assume the UEs are in RRC_CONNECTED.

Kyocera wonder if we have to be concerned about when the PC5 connections are established.

Huawei think we can consider the baseline as being all UEs in RRC_CONNECTED.

Qualcomm wonder about the case that an intermediate relay UE connects to multiple other relay UEs served by different cells; who governs the state of the intermediate relay?

Samsung understand that the serving gNBs should be the same for all UEs.

Agreements:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

[Stacks]

Proposal 4	RAN2 to agree the Multi-hop CP protocol stack as shown in Figure 2.  [E2E Uu-PDCP and above, hop-by-hop RLC and below]

Proposal 5	RAN2 to agree the Multi-hop UP protocol stack as shown in Figure 3.  [E2E Uu-PDCP and above, hop-by-hop RLC and below]

Discussion:

LG note the intermediate relay UE may have only one PC5-SRAP entity.

Agreements:

The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

[RRC connection establishment]

Proposal 6	RAN2 to agree the general procedure of RRC connection establishment for U2N Remote UE via one additional hops relay (illustrated in Figure 4) as the discussion start point. The procedure includes:

-	U2N Remote UE, Intermediate U2N Relay UE and U2N Relay UE performing discovery and PC5 Connection Establishment;

-	U2N Remote UE sends the first RRCSetupRequest to gNB, which triggers Intermediate U2N Relay UE and/or U2N Relay UE to enter RRC_CONNECTED if they are in RRC_IDLE/INACTIVE;

-	U2N Remote UE and gNB complete connection establishment, security establishment, RRC reconfiguration and so on.

Email discussion towards RAN2#128 on control plane:

[Post127][402][Relay] Multi-hop relay control plane (InterDigital)

Scope:

- Describe different solutions (from company contributions) for multihop U2N relay UE by at least describing:

  - Connection establishment procedures

  - Assumptions on RRC state(s) of intermediate UEs and last relay UE

  - Assumptions on controlling gNB/cell of each relay UE

  - How the remote and intermediate relay UEs obtain their configurations in each solution

  - How to meet QoS requirement e2e

- Evaluate the feasibility and pros/cons of the different solutions towards downscoping to a single solution

Intended outcome: Report to RAN2#128

Deadline: Very long (for RAN2#128)

R2-2406713	Scenarios, QoS Handling, and Control Plane Procedures for Multi-hop	InterDigital France R&D, SAS	discussion

[Paging and SI]

Proposal 13:	Paging and system information monitoring and forwarding at the U2N relay UE re-use the rules from Rel17 using PC5-RRC signalling on the corresponding dedicated PC5-RRC connection with each intermediate relay UE/remote UE.

R2-2406612	Initial considerations on CP and UP aspects for R19 multi-hop relay	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

[RLF handling]

Proposal 3-2: RAN2 is kindly asked to agree that the Uu RLF of last relay UE can trigger the notification message transmission or PC5 link release toward the connected first relay UE.

Proposal 3-3: RAN2 is kindly asked to agree that the PC5 RLF between first relay UE and last relay UE can trigger the RRC connection re-establishment of the first relay UE.

R2-2406366	Control plane procedures of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406494	Discussion on control plane procedures for multi-hop relays	MediaTek Inc.	discussion	Rel-19

R2-2406506	Considerations on Control Plane of Multi-hop Relay	NEC	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406529	Discussions on the L2 Intermediate U2N Relay in multi-hop L2 U2N Relay	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2406563	E2E Connection Setup and QoS Split for Multi-hop Relay	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406633	Control plane procedure for multi-hop U2N relay	Sony	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2406684	Control Plane Design for Multi-hop UE-to-NW Relay	Apple	discussion	Rel-19	DUMMY

R2-2406696	Discussion on architecture and control plane procedures for support of multi-hop SL relay	ZTE Corporation, Sanechips	discussion	NR_SL_relay_multihop-Core

R2-2406755	Discussion on QoS handling for NR sidelink multi-hop relay	Spreadtrum Communications	discussion	Rel-19

R2-2406888	Control plane in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2407034	discussion on control plane procedure	Ericsson, FirstNet, AT&T	discussion	Rel-19

R2-2407058	Discussion on Control Plane Procedure	LG Electronics France	discussion	Rel-19

R2-2407102	Control procedure for multi-hop L2 based U2N relay	Qualcomm Incorporated	discussion

R2-2407111	Discussion on control plane procedures for NR sidelink multi-hop relay	China Telecom	discussion	Rel-19

R2-2407206	Control Plane under multihop L2 U2N relaying	Kyocera	discussion

R2-2407295	Control plane procedures for multi-hop relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2407318	Control plane procedure for multi-hop relay	Nokia	discussion	NR_SL_relay_multihop

R2-2407403	discussion on C-plane procedure for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Withdrawn/Not available

R2-2406736	Discussion on Multi-hop Control Plane Procedures	vivo	discussion

- Withdrawn

## 8.14	Additional topological enhancements

(FS_WAB_5GFemto_NR; leading WG: RAN3; REL-19; SID RP-241264)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

This WI will not be treated in RAN2#127, therefore no contribution is expected under agenda item 8.12.

# 9	Breakout session reports

No documents shall be submitted to this AI or its sub-AIs. It is only for at-meeting-generated contents.

## 9.1	Session on V2X/SL, R19 NES and MOB

R2-2407571	Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)	Report

1. ltm-FastProcessingConfig-r18 is corrected to per UE capability (see R2-2407584).

2. fr1-r18, fr2-r18, fr1-AndFR2-r18 in ltm-FastUE-Processing-r18 are corrected to OPTIONAL (see R2-2407584)

3. pdcch-RACH-AffectedBands-r18, pdcch-RACH-SwitchingTimeList-r18, pdcch-RACH-PrepTime-r18 are corrected to all independent (see R2-2407584)

-	Qualcomm thinks we should avoid NBC especially for UE capability and SIB as it is not isolated to the feature.

=>	These changes will be done in a BC way

=>	Approved

## 9.2	Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

R2-2407572	Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS	Vice Chairman (CATT)	Report

=>	Approved

## 9.3	Session on NR NTN and IoT NTN

R2-2407573	Report from session on NR NTN and IoT NTN	Session chair (ZTE)	Report

=>	Approved

## 9.4	Session on positioning and sidelink relay

R2-2407574	Report from session on positioning and sidelink relay	Session chair (MediaTek)	Report

=>	guidance is that NBC change for SLPP

=>	For the RRC CR in R2-2407769 discuss the option over email discussion

=>	Approved

## 9.5	Session on R18 MBS, R18 QoE and R19 XR

R2-2407575	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)	Report

=>	Approved

## 9.6	Session on maintenance, SON/MDT and eRedCap

R2-2407576	Report from session on maintenance, CovEnh, SON/MDT and eRedCap	Session chair (Ericsson)	Report

=>	Approved

# Closing of the meeting

The meeting was closed by the chair at 13:30 CEST on Friday, 23rd of August.

# Annex A:	List of participants

RAN2#127 participants list is attached to this report.

Total number of participants: 455

# Annex B:	List of Tdocs

The list of tdocs from RAN2#127 is attached to this report.

Total of 1621 tdoc numbers were allocated of which 1584 tdocs were made available.

# Annex C:	Incoming liaison statements

| TDoc | Title | Source | Status | Rel | Related WIs | To | Cc | Original LS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2406204 | LS on RAN4 vs RAN2 Cricket Match (R4-2410764; contact: Nokia) | RAN4 | noted | Rel-19 | TEI19 | RAN2 |  | R4-2410764 |
| R2-2406205 | Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502 (C1-243517; contact: Huawei) | CT1 | noted | Rel-18 | NR_redcap_enh-Core | SA2 | CT4, RAN2, RAN3 | C1-243517 |
| R2-2406206 | Reply LS on 5GS missing CBC support for shared networks (C1-243686; contact: Ericsson) | CT1 | noted | Rel-18 | TEI18 | SA2 | RAN2, RAN3, SA1 | C1-243686 |
| R2-2406207 | LS on the UE role list in RSPP-Metadata (C1-243690; contact: ZTE) | CT1 | noted | Rel-18 | Ranging_SL | RAN2 | SA2 | C1-243690 |
| R2-2406208 | Reply LS on SL positioning measurement (R1-2405511; contact: Huawei) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN4, RAN2 |  | R1-2405511 |
| R2-2406209 | Reply LS on Sidelink Feature Co-configuration (R1-2405530; contact: OPPO) | RAN1 | noted | Rel-18 | NR_SL_enh2-Core | RAN2 |  | R1-2405530 |
| R2-2406210 | LS on Rel-18 RAN1 UE features list for NR after RAN1#117 (R1-2405566; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18 | RAN2, RAN4 |  | R1-2405566 |
| R2-2406211 | LS on Rel-18 RAN1 UE features list for LTE after RAN1#117 (R1-2405569; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-18 | IoT_NTN_enh | RAN2 | RAN4 | R1-2405569 |
| R2-2406212 | Reply LS on data collection to enable ML model training and inference in 5GC for Direct AI/ML based positioning (R1-2405578; contact: Ericsson) | RAN1 | noted | Rel-19 | FS_AIML_CN | SA2 | RAN2, RAN3 | R1-2405578 |
| R2-2406213 | Reply LS on DL-AoD measurements in NR-PRU-DL-Info forwarded to target UE (R1-2405586; contact: Nokia) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2405586 |
| R2-2406214 | Reply to RAN2 LS on type 3 PH value for the serving cell configured with mTRP (R1-2405619; contact: MediaTek) | RAN1 | noted | Rel-18 | NR_FeMIMO-Core, NR_MIMO_evo_DL_UL-Core | RAN2 |  | R1-2405619 |
| R2-2406215 | Reply LS on Reference Point for SSB-TimeOffset (R1-2405719; contact: Apple) | RAN1 | noted | Rel-18 | NR_NTN_enh-Core | RAN2 | RAN4 | R1-2405719 |
| R2-2406216 | LS on UE assistance information (R1-2405736; contact: Nokia) | RAN1 | noted | Rel-19 | NR_XR_Ph3-Core | RAN4 | RAN2 | R1-2405736 |
| R2-2406217 | Reply LS on intra-SN SCPAC in MN format (R3-243775; contact: ZTE) | RAN3 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 |  | R3-243775 |
| R2-2406218 | Reply LS on MDT for NPN (R3-243892; contact: Ericsson) | RAN3 | noted | Rel-18 | NR_ENDC_SON_MDT_enh2-Core | RAN2 | SA2, SA5 | R3-243892 |
| R2-2406219 | Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics (R3-243941; contact: Nokia) | RAN3 | noted | Rel-18 | NR_AIML_NGRAN-Core | SA5 | SA2, CT3, CT4, RAN2 | R3-243941 |
| R2-2406220 | Reply LS on Support of Regenerative-based Satellite Access (R3-243954; contact: ZTE) | RAN3 | noted | Rel-19 | NR_NTN_Ph3-Core | SA2 | RAN2 | R3-243954 |
| R2-2406221 | LS on UL PSI based PDU discarding in NR-DC (R3-243957; contact: Qualcomm) | RAN3 | noted | Rel-19 | NR_XR_Ph3-Core | RAN2 |  | R3-243957 |
| R2-2406222 | Response LS on FS_XRM Ph2 (R3-243958; contact: Lenovo) | RAN3 | noted | Rel-19 | FS_XRM_Ph2 | SA2 | RAN2, SA4 | R3-243958 |
| R2-2406223 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (R3-243961; contact: Qualcomm) | RAN3 | noted | Rel-19 | FS_VMR_Ph2 | SA2 | RAN2 | R3-243961 |
| R2-2406224 | LS on terminology definitions for AI-ML in NG-RAN (R3-243969; contact: Ericsson) | RAN3 | noted | Rel-18 | NR_AIML_NGRAN-Core | SA5, RAN2 | RAN1, RAN, SA | R3-243969 |
| R2-2406225 | LS on inconsistent issue between extended k1 range and RRC parameter DL-DataToUL-ACK-v1700 for R17 NTN (R4-2409974; contact: CMCC) | RAN4 | noted | Rel-18 | NR_ATG | RAN2 | RAN1 | R4-2409974 |
| R2-2406226 | Reply LS to RAN2 on RRM enhancements for NR FR2 HST (R4-2410285; contact: Samsung) | RAN4 | noted | Rel-18 | NR_HST_FR2_enh | RAN2 |  | R4-2410285 |
| R2-2406227 | Reply LS on LTM L1 intra and inter-frequency measurements (R4-2410303; contact: Ericsson) | RAN4 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 | RAN1 | R4-2410303 |
| R2-2406228 | LS on synchronization source change at the transmitting anchor UE in SL positioning (R4-2410352; contact: Ericsson) | RAN4 | postponed | Rel-18 | NR_pos_enh2-Core | RAN1, RAN2 |  | R4-2410352 |
| R2-2406229 | Reply LS on reference point for SSB-TimeOffset (R4-2410381; contact: Apple) | RAN4 | noted | Rel-18 | NR_NTN_enh-Core | RAN2 | RAN1 | R4-2410381 |
| R2-2406230 | Reply LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1 (R4-2410600; contact: Qualcomm) | RAN4 | noted | Rel-18 | NR_FR1_lessthan_5MHz_BW-Core | RAN1, RAN2 |  | R4-2410600 |
| R2-2406231 | LS on intra-band EN-DC channel spacing (R4-2410693; contact: Huawei) | RAN4 | noted | Rel-18 | TEI18 | RAN2 |  | R4-2410693 |
| R2-2406232 | LS on RAN4 UE feature list for Rel-18 (version 5) (R4-2410748; contact: CMCC) | RAN4 | noted | Rel-19 | NR_ENDC_RF_FR1_enh2, NR_channel_raster_enh, NR_FR2_multiRX_DL, NR_RRM_enh3, NR_MG_enh2, NonCol_intraB_ENDC_NR_CA, NR_HST_FR2_enh, NR_ATG, NR_demod_enh3, NR_pos_enh2, NR_MC_enh, NR_Mob_enh2, NR_NTN_enh, NR_cov_enh2, Netw_Energy_NR, 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core | RAN2 | RAN1 | R4-2410748 |
| R2-2406233 | Reply LS RP-240891 on NTZ solution impacts to RAN (RP-241668; contact: InterDigital) | RAN | noted | Rel-19 | FS_UAS_Ph3 | SA2 | RAN1, RAN2, RAN3, SA, ETSI MSG TFES | RP-241668 |
| R2-2406234 | LS on Avoiding Cross-TSG TEI (RP-241686; contact: NEC) | RAN | available |  |  | CT, CT1, CT3, CT4, CT6, TSG SA, SA1, SA2, SA3, SA4, SA5, SA6 | RAN1, RAN2, RAN3, RAN4, RAN5 | RP-241686 |
| R2-2406235 | LS on NTZ solution impacts to RAN (S2-2407355; contact: LGE & Ericsson) | SA2 | noted | Rel-19 | FS_UAS_Ph3 | RAN | RAN2, RAN3 | S2-2407355 |
| R2-2406236 | LS on data collection to enable ML model training and inference in 5GC for Direct AI/ML based positioning (S2-2405833; contact: vivo) | SA2 | noted | Rel-19 | FS_AIML_CN | RAN1, RAN2 | RAN3 | S2-2405833 |
| R2-2406237 | LS on Clarification of requirements for Ambient IoT (S2-2407231; contact: Ericsson) | SA2 | noted | Rel-19 | FS_Ambient_IoT_solutions | SA1, RAN1, SA3 | RAN2, RAN3 | S2-2407231 |
| R2-2406238 | Reply LS on application layer ID (S2-2407318; contact: Xiaomi) | SA2 | noted | Rel-18 | Ranging_SL | RAN2, CT1, CT4 |  | S2-2407318 |
| R2-2406239 | LS on questions regarding FS_VMR_Ph2 (S2-24073455; contact: Samsung) | SA2 | noted | Rel-19 | FS_VMR_Ph2 | RAN3 | RAN2 | S2-2407345 |
| R2-2406240 | LS on FS_5GSAT_Ph3_ARCH conclusions (S2-2407350; contact: OPPO) | SA2 | noted | Rel-19 | FS_5GSAT_Ph3_ARCH | SA3, SA3-LI | RAN2 | S2-2407350 |
| R2-2406241 | LS on FS_XRM Ph2 (S2-2407351; contact: vivo) | SA2 | noted | Rel-19 | FS_XRM_Ph2 | SA4, RAN2, RAN3 |  | S2-2407351 |
| R2-2406242 | LS Reply on FS_XRM Ph2 (S4-241370; contact: Huawei) | SA4 | noted | Rel-19 | FS_XRM_Ph2, FS_5G_RTP_Ph2 | SA2 | RAN2, RAN3 | S4-241370 |
| R2-2406243 | Reply LS Mitigation of Downgrade attacks (S3-242367; contact: Apple) | SA3 | noted | Rel-18 | TEI18 | CT1 | RAN2 | S3-242367 |
| R2-2406244 | Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases (S3-242400; contact: Apple) | SA3 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN2 | RAN3 | S3-242400 |
| R2-2406245 | Reply LS on FS_5GSAT_Ph3_ARCH conclusions (s3i240477; contact: Tencastle) | SA3-LI | noted | Rel-19 | FS_5GSAT_Ph3_ARCH | SA2 | SA3, RAN2 | s3i240477 |
| R2-2407750 | Reply LS on the MBS broadcast service continuity and MBS session identification (S4-241257; contact: Qualcomm) | SA4 | noted | Rel-17 | NR_MBS-Core, 5MBP3 | RAN2 | RAN3, SA2 | S4-241257 |
| R2-2407826 | LS on mandated support of the enhanced channel raster by RedCap UEs from Rel-17 (R4-2414409; contact: Ericsson) | RAN4 | noted | Rel-18 | NR_channel_raster_enh | RAN2 |  | R4-2414409 |

44 incoming LS, of which 42 LS were noted. The remaining non-treated LSin will be treated in RAN2#127bis.

# Annex D:	Outgoing liaison statements

| TDoc | Title | Rel | Related WIs | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2407463 | Reply LS on UE capability for multi-carrier enhancements | Rel-18 | NR_MC_enh-Core | RAN4 | RAN1 |
| R2-2407591 | LS on IUC Scheme-2 and Random Selection | Rel-17 | NR_SL_enh-Core | RAN1 |  |
| R2-2407601 | LS on LTM L1 intra and inter-frequency measurements capabilities | Rel-18 | NR_Mob_enh2-Core | RAN1, RAN4 |  |
| R2-2407602 | LS on security key update of inter-CU SCG LTM | Rel-19 | NR_Mob_Ph4-Core | SA3 | RAN3 |
| R2-2407606 | L3 measurement based LTM support over F1 | Rel-18 | NR_Mob_enh2-Core | RAN3 |  |
| R2-2407623 | LS on common TA in a regenerative payload scenario | Rel-19 | NR_NTN_Ph3-Core | RAN1, RAN4 |  |
| R2-2407655 | LS on power control parameters for unified TCI state framework | Rel-17 | NR_FeMIMO-Core | RAN1 |  |
| R2-2407664 | LS on MHI enhancement solution for SCG deactivation/activation | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN3 |  |
| R2-2407718 | Reply LS on the UE role list in RSPP-Metadata | Rel-18 | NR_pos_enh2-Core | CT1 | SA2 |
| R2-2407733 | Reply LS on FS_XRM Ph2 | Rel-19 | NR_XR_Ph3-Core | SA2 | RAN3, SA4 |
| R2-2407780 | Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC | Rel-19 | NR_XR_Ph3-Core | RAN3 |  |
| R2-2407816 | Reply LS on SDT signalling optimization for partial context transfer | Rel-18 | TEI18 | RAN3 |  |
| R2-2407823 | LS on requirements for ETWS primary notification | Rel-19 | NR_NTN_Ph3-Core | SA1 |  |
| R2-2407827 | Reply LS on Rel-18 higher-layers parameter list | Rel-18 | TEI18, NR_DSS_enh, NR_cov_enh2, NR_Mob_enh2, IoT_NTN_enh-Core, NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_SL_enh2-Core, NR_NTN_enh, NR_netcon_repeater-Core, NR_redcap_enh-Core, Netw_Energy_NR, NR_pos_enh2-Core, NR_XR_enh-Core, NR_BWP_wor-Core | RAN1 | RAN3, RAN4 |
| R2-2407831 | LS on data block sizes for Ambient IoT | Rel-19 | FS_Ambient_IoT_solutions | RAN1, SA2 |  |
| R2-2407832 | RAN2 inputs to TR 38.843 | Rel-18 | NR_AIML_air-Core | RAN1 |  |
| R2-2407853 | LS on CSI-RS/SRS for spatial relation in RRC_INACTIVE | Rel-18 | NR_pos_enh2-Core | RAN1 |  |
| R2-2407887 | LS on correction on network verification of UE location | Rel-18 | NR_NTN_enh-Core | RAN1 |  |

19 outgoing LS.

# Annex E:	List of agreed CRs

| TDoc | Title | Source | Rel | Spec | Related WIs | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2406265 | Miscellaneous correction on R18 SL Evolution | OPPO | Rel-18 | 38.331 | NR_SL_enh2-Core | 4863 |  | F |
| R2-2406272 | Correction on the field of scg-State | Huawei, HiSilicon, Qualcomm Incorporated | Rel-17 | 36.331 | LTE_NR_DC_enh2-Core | 5034 |  | F |
| R2-2406273 | Correction on the field of scg-State | Huawei, HiSilicon, Qualcomm Incorporated | Rel-18 | 36.331 | LTE_NR_DC_enh2-Core | 5035 |  | A |
| R2-2406274 | Correction on the field of scg-State | Huawei, HiSilicon, Qualcomm Incorporated | Rel-17 | 38.331 | LTE_NR_DC_enh2-Core | 4864 |  | F |
| R2-2406275 | Correction on the field of scg-State | Huawei, HiSilicon, Qualcomm Incorporated | Rel-18 | 38.331 | LTE_NR_DC_enh2-Core | 4865 |  | A |
| R2-2406350 | Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration | NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung | Rel-15 | 38.331 | NR_newRAT-Core | 4870 |  | F |
| R2-2406351 | Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration | NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung | Rel-16 | 38.331 | NR_newRAT-Core | 4871 |  | A |
| R2-2406353 | Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration | NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung | Rel-17 | 38.331 | NR_newRAT-Core | 4872 |  | A |
| R2-2406354 | Correction on the way to include ReconfigurationWithSync in masterCellGroup in RRCReconfiguration | NTTDOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson, Samsung | Rel-18 | 38.331 | NR_newRAT-Core | 4873 |  | A |
| R2-2406603 | Network energy savings for NR miscellaneous RRC CR | Huawei, HiSilicon | Rel-18 | 38.331 | Netw_Energy_NR-Core | 4881 |  | F |
| R2-2406699 | Correction to MAC on cast type | ZTE Corporation, Sanechips | Rel-16 | 38.321 | 5G_V2X_NRSL-Core | 1888 |  | F |
| R2-2406925 | Parameter names corrections for XR | Huawei, HiSilicon | Rel-18 | 38.331 | NR_XR_enh-Core | 4898 |  | D |
| R2-2407011 | Correction on SL IUC and SL DRX configurations for SL enhancements | Huawei, HiSilicon | Rel-17 | 38.331 | NR_SL_enh-Core | 4913 |  | F |
| R2-2407012 | Correction on SL IUC and SL DRX configurations for SL enhancements | Huawei, HiSilicon | Rel-18 | 38.331 | NR_SL_enh-Core | 4914 |  | A |
| R2-2407117 | Stage 2 alignments with stage 3 specification | Nokia, NEC | Rel-18 | 37.320 | NR_ENDC_SON_MDT_enh2-Core | 0136 |  | F |
| R2-2407281 | Correction on Transport Channels | Philips International B.V. | Rel-15 | 38.300 | NR_newRAT-Core | 0889 |  | F |
| R2-2407284 | Correction on Transport Channels | Philips International B.V. | Rel-17 | 38.300 | NR_newRAT-Core | 0891 |  | A |
| R2-2407288 | Correction on Transport Channels | Philips International B.V. | Rel-18 | 38.300 | NR_newRAT-Core | 0892 |  | A |
| R2-2407314 | Miscellaneous corrections on TS 38.304 for eRedCap | Huawei, HiSilicon | Rel-18 | 38.304 | NR_redcap_enh-Core, NR_redcap-Core | 0411 |  | F |
| R2-2407323 | Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r15) | ZTE Corporation, Sanechips | Rel-15 | 38.306 | NR_newRAT-Core | 1142 |  | F |
| R2-2407324 | Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r16) | ZTE Corporation, Sanechips | Rel-16 | 38.306 | NR_newRAT-Core | 1143 |  | A |
| R2-2407325 | Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r17) | ZTE Corporation, Sanechips | Rel-17 | 38.306 | NR_newRAT-Core | 1144 |  | A |
| R2-2407326 | Clarification on the Prerequisite of the ssb-AndCSI-RS-RLM (r18) | ZTE Corporation, Sanechips | Rel-18 | 38.306 | NR_newRAT-Core | 1145 |  | A |
| R2-2407337 | Correction to 38.306 on SON reports | Huawei, HiSilicon | Rel-18 | 38.306 | NR_ENDC_SON_MDT_enh2-Core | 1146 |  | F |
| R2-2407474 | Correction on broadcast reception for eRedcap UE | Huawei, HiSilicon | Rel-18 | 38.300 | NR_MBS_enh-Core | 0894 |  | F |
| R2-2407505 | Introduction of a UE capability for the barring exemption for emergency call [EM_Call_Exemption] | Huawei, HiSilicon | Rel-18 | 38.306 | TEI18 | 1148 |  | B |
| R2-2407528 | Clarification on parallel lists in MeasObjectNR | Samsung | Rel-17 | 38.331 | NR_NTN_solutions-Core | 4961 |  | F |
| R2-2407529 | Clarification on parallel lists in MeasObjectNR | Samsung | Rel-18 | 38.331 | NR_NTN_enh-Core, NR_NTN_solutions | 4962 |  | F |
| R2-2407577 | Miscellaneous corrections | Samsung (Rapporteur), Ericsson | Rel-17 | 38.321 | NR_pos_enh-Core | 1897 | 1 | D |
| R2-2407578 | Miscellaneous corrections | Samsung (Rapporteur), Ericsson | Rel-18 | 38.321 | NR_pos_enh-Core, NR_MIMO_evo_DL_UL-Core | 1898 | 1 | D |
| R2-2407579 | Correction for discarding operation for XR | Huawei, HiSilicon, Nokia (Rapporteur) | Rel-18 | 38.300 | NR_XR_enh-Core | 0896 |  | F |
| R2-2407580 | Rapporteur MAC CR for MT-SDT | Huawei, HiSIlicon | Rel-18 | 38.321 | NR_MT_SDT-Core | 1882 | 1 | F |
| R2-2407583 | Rapporteur Stage 2 Corrections for NR Sidelink Evolution | InterDigital, NEC | Rel-18 | 38.300 | NR_SL_enh2 | 0893 | 1 | F |
| R2-2407585 | Misc RRC corrections for feMob | Ericsson | Rel-18 | 38.331 | NR_Mob_enh2-Core | 4930 | 1 | F |
| R2-2407587 | User plane corrections for LTM | Huawei, HiSilicon | Rel-18 | 38.321 | NR_Mob_enh2-Core | 1904 | 2 | F |
| R2-2407588 | MAC correction on resource selection | LG | Rel-16 | 38.321 | 5G_V2X_NRSL-Core | 1915 | 1 | D |
| R2-2407589 | MAC correction on resource selection | LG | Rel-17 | 38.321 | 5G_V2X_NRSL-Core | 1916 | 1 | A |
| R2-2407590 | MAC correction on resource selection | LG | Rel-18 | 38.321 | 5G_V2X_NRSL-Core | 1917 | 1 | A |
| R2-2407592 | Correction to MAC on HARQ feedback indicator | Ericsson | Rel-17 | 38.321 | NR_SL_enh-Core | 1895 | 1 | F |
| R2-2407593 | Correction to MAC on HARQ feedback indicator | Ericsson | Rel-18 | 38.321 | NR_SL_enh-Core | 1906 | 1 | A |
| R2-2407604 | Miscellaneous corrections for mobility enhancements | ZTE Corporation, Ericsson | Rel-18 | 37.340 | NR_Mob_enh2-Core | 0399 | 2 | F |
| R2-2407611 | Miscellaneous corrections to Rel-18 NR NTN | Ericsson | Rel-18 | 38.331 | NR_NTN_enh-Core | 4935 | 1 | F |
| R2-2407615 | Miscellaneous corrections to TS 36.331 for IoT NTN | Huawei, HiSilicon | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5049 | 1 | F |
| R2-2407620 | Miscellaneous MAC correction for IoT NTN | MediaTek | Rel-18 | 36.321 | IoT_NTN_enh-Core | 1588 | 1 | F |
| R2-2407621 | Stage-2 corrections for NR NTN | THALES | Rel-18 | 38.300 | NR_NTN_enh-Core | 0902 |  | F |
| R2-2407627 | Corrections of UE Capabilities in IoT NTN | Qualcomm Inc. | Rel-18 | 36.306 | IoT_NTN_enh-Core | 1892 |  | F |
| R2-2407628 | Corrections for PUCCH repetition for Msg4 HARQ-ACK | InterDigital | Rel-18 | 38.321 | NR_NTN_enh-Core | 1928 |  | F |
| R2-2407632 | Correction to MIB-MBMS systemFrameNumber field description | Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson | Rel-18 | 36.331 | MBMS_LTE_enh2-Core | 5046 | 2 | A |
| R2-2407633 | Alignment of mps-PriorityAccess cause in RRC resume | Peraton Labs, CISA ECD, Verizon | Rel-18 | 38.300 | NR_newRAT-Core | 0882 | 1 | A |
| R2-2407634 | Correction on Transport Channels | Philips International B.V. | Rel-16 | 38.300 | NR_newRAT-Core | 0890 | 1 | A |
| R2-2407635 | Correction to Parallel Tx capabilities | Ericsson, Qualcomm Incorporated | Rel-15 | 38.306 | NR_newRAT-Core | 1153 |  | F |
| R2-2407636 | Correction to Parallel Tx capabilities | Ericsson, Qualcomm Incorporated | Rel-16 | 38.306 | NR_newRAT-Core | 1154 |  | A |
| R2-2407637 | Clarification on increasedNumberofCSIRSPerMO | Huawei, HiSilicon, vivo, MediaTek Inc., Nokia, Nokia Shanghai Bell, ZTE Corporation | Rel-16 | 38.306 | NR_CSIRS_L3meas-Core | 1138 | 1 | F |
| R2-2407638 | Clarification on increasedNumberofCSIRSPerMO | Huawei, HiSilicon, vivo, MediaTek Inc., Nokia, Nokia Shanghai Bell, ZTE Corporation | Rel-17 | 38.306 | NR_CSIRS_L3meas-Core | 1139 | 1 | A |
| R2-2407639 | Clarification on increasedNumberofCSIRSPerMO | Huawei, HiSilicon, vivo, MediaTek Inc., Nokia, Nokia Shanghai Bell, ZTE Corporation | Rel-18 | 38.306 | NR_CSIRS_L3meas-Core | 1140 | 1 | A |
| R2-2407640 | Miscellaneous corrections for further NR coverage enhancements in RRC | Huawei, HiSilicon, Philips International B.V., Ericsson, Lenovo | Rel-18 | 38.331 | NR_cov_enh2-Core | 4951 | 1 | F |
| R2-2407641 | Miscellaneous corrections on further NR Coverage enhancements in MAC | ZTE Corporation | Rel-18 | 38.321 | NR_cov_enh2-Core | 1918 | 1 | F |
| R2-2407644 | Clarification on MBS broadcast acquisition | Samsung, CATT, Nokia, LG Electronics Inc., Ericsson, Apple, Qualcomm Incorporated | Rel-17 | 38.331 | NR_MBS-Core, NR_SmallData_INACTIVE-Core | 4926 | 1 | F |
| R2-2407645 | Clarification on MBS broadcast acquisition | Samsung, CATT, Nokia, LG Electronics Inc., Ericsson, Apple, Qualcomm Incorporated | Rel-18 | 38.331 | NR_MBS-Core, NR_SmallData_INACTIVE-Core | 4927 | 1 | A |
| R2-2407648 | Correction to indication of reserved SN-side NR-DC and aggregated BW resources | Nokia | Rel-17 | 38.331 | NR_newRAT-Core, NR_eMIMO-Core, NR_BCS4-Core | 4952 | 1 | F |
| R2-2407649 | Correction to indication of reserved SN-side NR-DC and aggregated BW resources | Nokia | Rel-18 | 38.331 | NR_newRAT-Core, NR_eMIMO-Core, NR_BCS4-Core | 4953 | 1 | A |
| R2-2407650 | Miscellaneous non-controversial corrections Set XXII | Ericsson | Rel-16 | 38.331 | NR_newRAT-Core, TEI16 | 4960 | 1 | F |
| R2-2407651 | Miscellaneous non-controversial corrections Set XXII | Ericsson | Rel-17 | 38.331 | NR_newRAT-Core, TEI17 | 4916 | 1 | F |
| R2-2407652 | Miscellaneous non-controversial corrections Set XXII | Ericsson | Rel-18 | 38.331 | NR_newRAT-Core, TEI18 | 4917 | 1 | F |
| R2-2407653 | Corrections to UE capabilities related to Rel-17 URLLC and RedCap | Huawei, HiSilicon | Rel-17 | 38.306 | NR_IIOT_URLLC_enh-Core, NR_redcap-Core | 1149 |  | F |
| R2-2407654 | Corrections to UE capabilities related to Rel-17 URLLC and RedCap | Huawei, HiSilicon | Rel-18 | 38.306 | NR_IIOT_URLLC_enh-Core, NR_redcap-Core | 1150 |  | A |
| R2-2407656 | Corrections on some features application to (e)RedCap UE | Huawei, HiSilicon | Rel-18 | 38.306 | NR_redcap_enh-Core, NR_redcap-Core | 1151 |  | F |
| R2-2407657 | Miscellaneous corrections on mobility history information | ZTE Corporation, Sanechips | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 4907 | 1 | F |
| R2-2407658 | Miscellaneous corrections on mobility history information | ZTE Corporation, Sanechips | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 4908 | 1 | A |
| R2-2407659 | Correction on power control parameters to support unified TCI state framework | Ericsson | Rel-18 | 38.331 | NR_FeMIMO-Core | 4963 | 1 | A |
| R2-2407660 | Correction on power control parameters to support unified TCI state framework | Ericsson | Rel-17 | 38.331 | NR_FeMIMO-Core | 4964 | 1 | F |
| R2-2407661 | Clarification on Validity Duration | vivo, Ericsson | Rel-17 | 38.331 | NR_NTN_solutions-Core | 4877 | 1 | F |
| R2-2407662 | Clarification on Validity Duration | vivo, Ericsson | Rel-18 | 38.331 | NR_NTN_solutions-Core | 4878 | 1 | A |
| R2-2407665 | Correction on the missing PRACH SCS configuration | CATT | Rel-17 | 38.331 | NR_ext_to_71GHz-Core | 4891 | 1 | F |
| R2-2407666 | Correction on the missing PRACH SCS configuration | CATT | Rel-18 | 38.331 | NR_ext_to_71GHz-Core, TEI18 | 4892 | 1 | F |
| R2-2407716 | Correction on SP SRS activation/deactivation MAC CE | ZTE Corporation, Ericsson | Rel-17 | 38.321 | NR_pos_enh-Core | 1840 | 2 | F |
| R2-2407717 | Correction on SP SRS activation/deactivation MAC CE | ZTE Corporation, Ericsson | Rel-18 | 38.321 | NR_pos_enh-Core | 1841 | 2 | A |
| R2-2407720 | Corrections to LPP specification | CATT, Ericsson, Qualcomm, Nokia | Rel-18 | 37.355 | NR_pos_enh2-Core | 0512 | 1 | F |
| R2-2407724 | Correction on T300 and T302 handling due to relay selection or cell selection | Huawei, HiSilicon | Rel-17 | 38.331 | NR_SL_relay-Core | 4905 | 1 | F |
| R2-2407725 | Correction on T300 and T302 handling due to relay selection or cell selection | Huawei, HiSilicon | Rel-18 | 38.331 | NR_SL_relay-Core | 4906 | 1 | A |
| R2-2407726 | RRC correction on NR SL U2N Relay UE selection and reselection procedure | Philips International B.V., NEC | Rel-17 | 38.331 | NR_SL_relay-Core | 4938 | 1 | F |
| R2-2407727 | RRC correction on NR SL U2N Relay UE selection and reselection procedure | Philips International B.V., NEC | Rel-18 | 38.331 | NR_SL_relay-Core | 4939 | 1 | A |
| R2-2407728 | Corrections for SL relay measurements | ZTE Corporation, Sanechips, Apple, OPPO, Nokia, Huawei, HiSilicon | Rel-17 | 38.331 | NR_SL_relay-Core | 4956 | 1 | F |
| R2-2407731 | Miscellaneous Stage-2 corrections on R18 QoE | Nokia, Nokia Shanghai Bell,China Unicom | Rel-18 | 38.300 | NR_QoE_enh-Core | 0886 | 1 | F |
| R2-2407734 | Correction of Enhancement on NR QoE management and optimizations for diverse services | Ericsson, CATT, ZTE, Google, Huawei, Samsung | Rel-18 | 38.331 | NR_QoE_enh-Core | 4922 | 1 | F |
| R2-2407736 | Miscellaneous corrections on eMBS | Huawei, HiSilicon, CATT, Samsung, Nokia | Rel-18 | 38.331 | NR_MBS_enh-Core | 4955 | 1 | F |
| R2-2407737 | Correction for error data handling for MBS | Samsung, Apple | Rel-18 | 38.321 | NR_MBS_enh-Core | 1923 |  | F |
| R2-2407747 | Correction for UAV capabilities | Huawei, HiSilicon, Nokia, Qualcomm, Lenovo, Samsung | Rel-18 | 36.306 | LTE_UAV_enh-Core | 1890 | 1 | F |
| R2-2407748 | Corrections on mobile IAB terminologies | Huawei, HiSilicon (Rapporteur) | Rel-18 | 38.340 | NR_mobile_IAB-Core | 0036 | 1 | F |
| R2-2407752 | Correction to LPP Measurement Time Windows and SLPP Assistance Data Transfer procedures | Qualcomm Incorporated | Rel-18 | 38.305 | NR_pos_enh2-Core | 0172 |  | F |
| R2-2407754 | Corrections for SL relay measurements | ZTE Corporation, Sanechips, Apple, OPPO, Nokia, Huawei, HiSilicon | Rel-18 | 38.331 | NR_SL_relay-Core | 4957 | 2 | A |
| R2-2407755 | Miscellaneous corrections for SL relay enhancements | Huawei, HiSilicon, OPPO, CATT, Apple, ZTE Corporation, Sanechips, Nokia, Philips International B.V., Sharp | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 4904 | 2 | F |
| R2-2407764 | Miscellaneous corrections for Multi-Carrier enhancements | Huawei, HiSilicon, Lenovo, NTT DOCOMO INC. | Rel-18 | 38.331 | NR_MC_enh-Core | 4903 | 1 | F |
| R2-2407765 | Correction on data transmission and data volume calculation in MP | InterDigital, Huawei, HiSilicon | Rel-18 | 38.323 | NR_SL_relay_enh-Core | 0140 |  | F |
| R2-2407774 | Corrections on SI request with Msg1 repetition | Lenovo, Huawei, HiSilicon, Philips International B.V. | Rel-18 | 38.331 | NR_cov_enh2-Core, NR_pos_enh2, TEI17 | 4889 | 1 | F |
| R2-2407775 | Alignment of mps-PriorityAccess cause in RRC Resume | Peraton Labs, CISA ECD, Verizon | Rel-15 | 38.300 | NR_newRAT-Core | 0898 |  | F |
| R2-2407776 | Alignment of mps-PriorityAccess cause in RRC Resume | Peraton Labs, CISA ECD, Verizon | Rel-16 | 38.300 | NR_newRAT-Core | 0899 |  | A |
| R2-2407777 | Alignment of mps-PriorityAccess cause in RRC Resume | Peraton Labs, CISA ECD, Verizon | Rel-17 | 38.300 | NR_newRAT-Core | 0900 |  | A |
| R2-2407778 | Correction on DRX to support eMBS in NTN | Huawei, HiSilicon, Qualcomm, LG Electronics Inc, vivo, Ericsson, Nokia | Rel-18 | 38.321 | NR_MBS_enh-Core, NR_NTN_enh-Core | 1924 |  | F |
| R2-2407783 | Correction on enhancements to measurement report [meas_report_enh] | Samsung | Rel-18 | 38.331 | TEI18 | 4966 |  | F |
| R2-2407787 | Misc RRC corrections for RACH-less HO [RACH-lessHO] | Ericsson | Rel-18 | 38.331 | TEI18, NR_NTN_enh-Core, NR_mobile_IAB-Core | 4932 | 1 | F |
| R2-2407788 | Correction on extension of ToAddModList | Samsung | Rel-18 | 38.331 | TEI18, NR_SL_enh2 | 4967 |  | F |
| R2-2407792 | Clarification on highSpeedMeasFlagFR2 for HST FR2 RRM | Samsung | Rel-18 | 38.331 | NR_HST_FR2_enh | 4936 | 1 | F |
| R2-2407794 | Correction to Parallel Tx capabilities | Ericsson, Qualcomm Incorporated | Rel-17 | 38.306 | NR_newRAT-Core | 1155 |  | A |
| R2-2407795 | Correction to Parallel Tx capabilities | Ericsson, Qualcomm Incorporated | Rel-18 | 38.306 | NR_newRAT-Core | 1156 |  | A |
| R2-2407796 | Correction to MIB-MBMS systemFrameNumber field description | Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson | Rel-14 | 36.331 | MBMS_LTE_enh2-Core | 5042 | 1 | F |
| R2-2407797 | Correction to MIB-MBMS systemFrameNumber field description | Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson | Rel-15 | 36.331 | MBMS_LTE_enh2-Core | 5043 | 1 | A |
| R2-2407798 | Correction to MIB-MBMS systemFrameNumber field description | Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson | Rel-16 | 36.331 | MBMS_LTE_enh2-Core | 5044 | 1 | A |
| R2-2407799 | Correction to MIB-MBMS systemFrameNumber field description | Qualcomm Incorporated, Samsung, ABS, SJTU, Ericsson | Rel-17 | 36.331 | MBMS_LTE_enh2-Core | 5045 | 1 | A |
| R2-2407803 | Paging monitoring for extended CG period [CG-SDT-Enh] | ZTE Corporation, Sanechips, Mediatek, Qualcomm, Ericsson | Rel-18 | 38.331 | TEI18 | 4788 | 2 | F |
| R2-2407805 | Correction to UE capabilities and configuration | Google | Rel-18 | 36.331 | IoT_NTN_enh-Core, NR_ENDC_SON_MDT_enh2-Core, LTE_UAV_enh-Core | 5040 | 1 | F |
| R2-2407815 | Introduction of unknown SCell activation enhancement | Huawei, HiSilicon, Apple | Rel-18 | 38.300 | NR_RRM_enh3 | 0885 | 1 | B |
| R2-2407818 | Rapporteur MAC CR for generalized RACH-less HO/LTM [RACH-lessHO] | Huawei, HiSilicon | Rel-18 | 38.321 | TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core | 1881 | 1 | F |
| R2-2407820 | Correction to barring exemption for (e)Redcap and XR 2RX UEs [EM_Call_Exemption] | ZTE Corporation, Sanechips | Rel-18 | 38.304 | TEI18 | 0409 | 2 | F |
| R2-2407828 | Correction on absoluteFrequencySSB corresponding to CD-SSB for PCell | vivo, ZTE | Rel-18 | 38.331 | NR_BWP_wor-Core, Netw_Energy_NR-Core, NR_redcap_enh-Core, NR_redcap-Core | 4928 | 1 | F |
| R2-2407829 | Introduction of barring exemption for emergency call [EM_Call_Exemption] | Huawei, HiSilicon | Rel-18 | 38.300 | TEI18 | 0884 | 1 | B |
| R2-2407836 | Update of stage 2 positioning information | R3 (Nokia, CATT, Ericsson, Huawei, Samsung, Xiaomi, ZTE) | Rel-18 | 38.305 | NR_pos_enh2-Core | 0173 |  | F |
| R2-2407837 | Correction on CPA failure states and detection mechanism in stage 2 | R3 (Nokia, CATT, CMCC, Qualcomm Incorporated, Huawei, ZTE Corporation, Lenovo) | Rel-18 | 37.340 | NR_ENDC_SON_MDT_enh2-Core | 0400 |  | F |
| R2-2407838 | Correction on SPR retrieval and forwarding | R3 (Samsung, Lenovo, Cybercore, Qualcomm, Nokia, Ericsson, Huawei, ZTE, CATT) | Rel-18 | 37.340 | NR_ENDC_SON_MDT_enh2-Core | 0401 |  | F |
| R2-2407839 | Correction on Complete Candidate Configuration | R3 (Huawei, Ericsson, LG Electronics, Samsung) | Rel-18 | 37.340 | NR_Mob_enh2-Core | 0402 |  | F |
| R2-2407840 | Missing procedure description for pre-configured SRS activation | R3 (Qualcomm Incorporated, Ericsson, Nokia, Xiaomi, Huawei, CATT) | Rel-18 | 38.305 | NR_pos_enh2-Core | 0174 |  | F |
| R2-2407841 | Correction on MIMO with 2TAs in LTM | R3 (Huawei, China Unicom, CMCC, Nokia, LG Electronics, CATT, Ericsson) | Rel-18 | 38.300 | NR_Mob_enh2-Core | 0901 |  | F |
| R2-2407843 | Correction to PHR for multi-TRP multi-panel scheme in MIMO Evolution | Samsung | Rel-18 | 38.321 | NR_MIMO_evo_DL_UL-Core | 1927 |  | F |
| R2-2407846 | Updates and Introduction of UE capabilities for Rel-18 WIs | Intel Corporation | Rel-18 | 38.306 | NR_MIMO_evo_DL_UL-Core, NR_MC_enh-Core, NR_NTN_enh-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_UAV-Core, NR_FR2_multiRX_DL-Core, NR_MBS_enh-Core, NR_netcon_repeater-Core, NR_cov_enh2-Core, NR_Mob_enh2-Core | 1159 |  | B |
| R2-2407847 | Updates and Introduction of UE capabilities for Rel-18 WIs | Intel Corporation | Rel-18 | 38.331 | NR_MIMO_evo_DL_UL-Core, NR_MC_enh-Core, NR_FR2_multiRX_DL-Core, NR_netcon_repeater-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_pos_enh2-Core | 4971 |  | B |
| R2-2407854 | Rapporteur MAC CR for R18 positioning | Huawei, HiSilicon | Rel-18 | 38.321 | NR_pos_enh2-Core | 1883 | 2 | F |
| R2-2407864 | Correction on NR MUSIM enhancements | vivo | Rel-18 | 38.331 | NR_DualTxRx_MUSIM-Core | 4970 | 1 | F |
| R2-2407866 | Correction on MIMOevo | Ericsson | Rel-18 | 38.331 | NR_MIMO_evo_DL_UL-Core | 4969 | 1 | F |
| R2-2407867 | Further miscellaneous corrections to QMC procedure | R3 (Huawei, China Unicom, Lenovo, Ericsson, Nokia, Samsung, ZTE) | Rel-18 | 37.340 | NR_QoE_enh-Core | 0403 | 1 | F |
| R2-2407868 | Correction for IoT NTN neighbour cell measurements and TN to NTN mobility | Ericsson, Samsung | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1405 | 1 | F |
| R2-2407869 | Correction on network verification of UE location | Huawei, HiSilicon, CATT | Rel-18 | 37.355 | NR_NTN_enh-Core | 0511 | 2 | F |
| R2-2407870 | Corrections to TS 38.300 for R18 MUSIM | China Telecom, Samsung | Rel-18 | 38.300 | NR_DualTxRx_MUSIM-Core | 0887 | 1 | F |
| R2-2407871 | Rel-18 SONMDT Corrections | Ericsson, Nokia, Samsung | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 4968 | 1 | F |
| R2-2407872 | Miscellaneous corrections to SLPP specification | Intel Corporation | Rel-18 | 38.355 | NR_pos_enh2-Core | 0005 | 3 | F |
| R2-2407873 | Corrections on NR sidelink relay enhancements | LG Electronics (Rapporteur) | Rel-18 | 38.300 | NR_SL_relay_enh-Core | 0897 | 2 | F |
| R2-2407874 | Correction on the capabilities on PTM retransmission [PTM_ReTx_Mcast_HARQ_Disb] | Huawei, HiSilicon, Intel Corporation, Nokia, vivo, Samsung | Rel-18 | 38.306 | NR_MBS_enh-Core, TEI18 | 1134 | 2 | F |
| R2-2407875 | Correction on the capabilities on PTM retransmission [PTM_ReTx_Mcast_HARQ_Disb] | Huawei, HiSilicon, Intel Corporation, Nokia, vivo, Samsung | Rel-18 | 38.331 | NR_MBS_enh-Core, TEI18 | 4867 | 2 | F |
| R2-2407876 | Corrections for XR | Qualcomm Incorporated, Ericsson | Rel-18 | 38.321 | NR_XR_enh-Core | 1926 | 2 | F |
| R2-2407877 | Correction on RACH resource set selection---Option 1 | ZTE Corporation | Rel-18 | 38.321 | NR_cov_enh2, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, NR_redcap_enh-Core | 1884 | 4 | F |
| R2-2407878 | MAC correction on Release-18 Sidelink evolution | LG | Rel-18 | 38.321 | NR_SL_enh2 | 1922 | 3 | F |
| R2-2407879 | Correction on featureCombination and SI-RequestConfig | ZTE Corporation, Ericsson, LG Electronics Inc. | Rel-17 | 38.331 | NR_newRAT-Core, TEI17 | 4911 | 2 | F |
| R2-2407880 | Correction on featureCombination and SI-RequestConfig | ZTE Corporation, Ericsson, LG Electronics Inc. | Rel-18 | 38.331 | NR_newRAT-Core, TEI17 | 4912 | 2 | A |
| R2-2407881 | Miscellaneous MAC corrections for network energy savings | InterDigital | Rel-18 | 38.321 | Netw_Energy_NR-Core | 1921 | 2 | F |
| R2-2407882 | Correction to EnteringLeavingReport condition [meas_report_enh] | LG Electronics, Ericsson, Qualcomm Inc., ZTE, Apple, MediaTek Inc., Samsung | Rel-18 | 38.331 | TEI18 | 4959 | 4 | F |
| R2-2407883 | Miscellaneous Positioning Corrections | Ericsson, Philips International B.V., vivo, CATT | Rel-18 | 38.331 | NR_pos_enh2-Core | 4934 | 3 | F |
| R2-2407884 | Extension of cells in Validity area | Ericsson, CATT, ZTE Corporation | Rel-18 | 38.331 | NR_pos_enh2-Core | 4965 | 2 | F |

146 Agreed CRs.

# Annex F:	List of email discussions during the meeting

Discussions during the R2-127 meeting:

List of AT meeting email discussions

[AT127][001][UE cap] CR update to R2-2406535 (Google)

[AT127][002][RAN1 Parameters] Updated parameter lists (Ericsson)

[AT127][003][RRC CR] ToAddModList CR (Samsung)

[AT127][005][RACH resources] Restructuring of selection procedure (ZTE)

[AT127][006][R17 UP] PHR for mTRP (LG, ZTE)

[AT127][007][RACH-less] CRs (Ericsson-Huawei)

[AT127][008][SDT] LS to RAN3 (ZTE)

[AT127][009][SDT] Extended CG correction (ZTE)

[AT127][010][NCR] Capability CRs (ZTE, Huawei)

[AT127][011][UAV] UE capabilities (Huawei)

[AT127][012][Emergency calls] Acceptable cells (ZTE)

[AT127][013][Emergency Calls] 38.300 CR (Huawei)

[AT127][014][NES] MAC CR (InterDigital)

[AT127][015][TEI18] Measurement Reporting (Samsung)

[AT127][016][HST] 331 CR (Samsung)

[AT127][017][SCell act] Stage 2 CR  (Huawei)

[AT127][018][MC] 331 CR (Huawei)

[AT127][021][AIML PHY] UE side data collection (Interdigital)

[AT127][023][AIoT] LS to RAN1-SA2 (MediaTek)

[AT127][024][ATG] Clarification on SCS for Timing Advance Reporting (Samsung)

[AT127][025][SI request] 331 CR (Lenovo)

[AT127][026][AI Mob] Simulation assumptions (OPPO)

[AT127][028][AI PHY] table (Ericsson)

[AT127][102][V2XSL] R18 SLe2 RRC CR (OPPO)

[AT127][103][V2XSL](InterDigital)

[AT127][104][MOB] (MediaTek, ZTE)

[AT127][105][MOB] (Ericsson)

[AT127][106][MOB] (ZTE)

[AT127][107][V2XSL] LS to R1 (OPPO)

[AT127][108][MOB] R18 MAC CR

[AT127][109][NES] Paging Enh (OPPO)

[AT127][110][V2XSL] R18 SLe2 MAC CR (LG)

[AT127][201][MIMOevo] Proposals for RRC changes (Ericsson)

[AT127][301][R18 NR NTN] satellite switch with re-sync (Qualcomm)

[AT127][302][R18 NR NTN] absence of validity duration in SIB19 (CATT)

[AT127][303][R18 NR NTN] Stage 2 text for trigger conditions (OPPO)

[AT127][304][LTE_TN_NR_NTN_mob] Stage 2 CR (Samsung)

[AT127][305][LTE_TN_NR_NTN_mob] RRC CR (CATT)

[AT127][306][LTE_TN_NR_NTN_mob] UE capability CR (vivo)

[AT127][307][R19 NR NTN] service area information (Apple)

[AT127][401][POS] SLPP rapporteur CR update (Intel)

[AT127][402][POS] Rel-18 LPP rapporteur CR merge (CATT)

[AT127][403][POS] Rel-18 positioning RRC CR update (Ericsson)

[AT127][404][POS] Rel-18 positioning MAC CR update (Huawei)

[AT127][405][POS] C field in relation to cell ID and BWP ID in RRC_INACTIVE (ZTE)

[AT127][406][POS] Rel-18 positioning 38.305 rapporteur CR (Qualcomm)

[AT127][407][Relay] Corrections for Rel-17 SL relay measurements (ZTE)

[AT127][408][Relay] Rel-18 relay RRC rapporteur CR update (Huawei)

[AT127][409][Relay] Rel-18 relay PDCP rapporteur CR (InterDigital)

[AT127][410][Relay] Rel-18 relay RLC rapporteur CR (Xiaomi)

[AT127][411][Relay] Rel-18 relay stage 2 rapporteur CR (LG)

[AT127][501][XR] LS to RAN3 (QCM)

[AT127][502][XR] Reply LS to SA2 (vivo)

[AT127][503][QoE] RRC CR (Ericsson)

[AT127][504][QoE] Release of QoE Configurations (ZTE)

[AT127][506][MBS] MAC CR (Samsung, Apple)

[AT127][507][MBS] MBS and MT-SDT co-existence (Sharp)

[AT127][601][MBMS] MIB-MBMS systemFrameNumber (Qualcomm)

[AT127][602][Maint] mps-PriorityAccess cause in RRC resume (Peraton Labs)

[AT127][603][Maint] Parallel Tx capability (Ericsson)

[AT127][604][feMIMO] LS on RAN1 power control parameters (Ericsson)

[AT127][605][Maint] Clarification on MBS broadcast acquisition (Samsung)

[AT127][606][Maint] Indication of reserved SN-side resources (Nokia)

[AT127][607][NR17] Corrections to UE capabilities related to Rel-17 URLLC and RedCap  (Huawei)

[AT127][608][Maint] Clarification on Validity Duration (vivo)

[AT127][609][SONMDT] RRC Corrections for SONMDT (Samsung)

[AT127][CB] Correction on use of recommended of IAB-MT beam indication (Ericsson)

[AT127][CB][R19 NR NTN] LS on common TA in a regenerative payload scenario(CMCC)

[NR R18] RAN1 parameter list

[R18MobE] MAC rapporteur CR

70 at-meeting offline discussions.

# Annex G:	Post-meeting email discussions

# Guidelines for email discussions:

General guidelines for SHORT email discussions, to be concluded approved endorsed at current meeting.

- Aim to have the final version of the agreed documents provided by the rapporteur at or shortly after the deadline.

- Please provide comments on the first version of the document in good time before the deadline. This allows the rapporteur to make an update addressing all companies' comments and there still be time for a quick round of comments on the update.

- If you have provided comments in the discussion then please indicate to the rapporteur if you are ok with the update provided (preferably via reflector). This avoids the rapporteur having to wait before they can conclude that their update is acceptable to you.

- Rapporteurs, if not already available, please request your tdoc number from the Secretary when you initiate your email discussion and then provide the final version as soon as you are confident that it is agreeable. You do not need to wait for a reminder from chair, session chair or Secretary before sending the final version.

- To avoid any confusion, Secretary, chair, or session chair will send an email to confirm the final status of the document.

For emails discussion to the next meeting (long):

- Rapporteurs, feel free to set an intermediate deadline for companies to provide initial comments, so that the conclusions and proposals can be prepared and distributed before the final deadline.

- Participants, please respect any intermediate deadline indicated by the rapporteur, and preferably provide your feedback as soon as possible.

39 email approvals/discussions scheduled after the RAN2#127 meeting (30 short, 5 long and 4 extra long, i.e., for the November meeting).

Deadlines

Aug. 29th 	1800 UTC	Deadline short email discussions

Sept. 27th 	1000 UTC	Deadline long email discussions

Oct. 4th	1000 UTC	Submission Deadline RAN2#127bis

Weekends are inactive periods.

It is recommended to not send emails or update files on the server during inactive periods while It is not strictly prohibited. Rapporteurs may kick-off discussions during inactive period. However, no intermediate deadlines and no interactive discussion, no decision making may occur during the inactive period. It shall be possible for a delegate to stay away from reflector and 3GPP server during the inactive period, and still fully participate. Rapporteur announcements during the inactive period, if any, or other updates, can be taken into account after the inactive period.

# Short email discussions, Deadline Aug 29th, 18:00 UTC

Please request R2-127 TDoc numbers for the following email discussions from MCC if not already allocated. Approval / endorsement will be declared at or shortly after the deadline.

[POST127][000][Organizational] Email discussion list

Intended outcome: provide email discussion list

[POST127][002][UE Cap] Mega CR (Intel)

Intended outcome: Agree to Mega CR

Deadline:  Sept 2nd, 2024

=> Agreed in:

R2-2407846 (38.306 CR#1159)

R2-2407847 (38.331 CR#4971)

[POST127][011][UAV] UE capabilities (Huawei)

Intended outcome: Review updated wording for LTE CR (to be agreed)

Deadline:  short

=> Agreed in R2-2407747 (36.306 CR#1890r1)

[POST127][030][AI mobility] RRM simulation assumptions (Oppo)

Intended outcome: agree to simulation assumptions

Deadline:  short

=> Available in:

R2-2407849 (Disc, first phase)

R2-2407885 (Disc, second phase)

[POST127][032][Channel Raster] Capability (Ericsson)

Intended outcome:

Deadline:  short

=> Reserved in:

R2-2407834 (38.306 CR#1157)

R2-2407835 (38.306 CR#1158)

=> Postponed

[POST127][101][V2X/SL] (LG)

Scope: Discuss changes in R2-2407381 (only correction 1) and capture all agreements made in RAN2#127. => It is extended to short email discussion. Capture all RAN2#127 agreements.

Intended outcome: MAC CR in R2-2407581. => Agreed MAC CR in R2-2407603.

Deadline: Short offline discussion.

=> Agreed in R2-2407865 (38.321 CR#1922r2)

[POST127][105][MOB] (Ericsson)

Scope: Discuss P1, P2, and P3 in R2-2406332, P3 in R2-2406337, P1, P2, P3, P6, P7 in R2-2407050 and R2-2407175. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements.

Intended outcome: Agreed RRC CR in R2-2407585. Discussion summary in R2-2407595 (if needed). If proposals are straight-forward, it is ok to directly discuss based on the CR (up to offline discussion rapporteur). => In short email discussion, no need of discussion summary.

Deadline: Short email discussion

=> Agreed in:

R2-2407585 (38.331 CR#4930r1)

=> Reserved in:

R2-2407595 (Discussion summary)

[POST127][106][MOB] (ZTE)

Scope: Discuss R2-2406417 and R2-2407091. Update the CR with merging all corrections (if agreeable). => In CB session, this offline discussion is extended to short email discussion and the updated CR will capture all RAN2#127 agreements

Intended outcome: Endorsed 37.340 CR/TP in R2-2407586. Discussion summary in R2-2407596 (if needed). If proposals are straight-forward, it is ok to directly discuss based on draft TP/CR (up to offline discussion rapporteur). => Agreed 37.340 CR in R2-2407604. In short email discussion, no need of discussion summary.

Deadline: Short email discussion

=> Agreed in:

R2-2407604 (37.340 CR#0399r2)

=> Reserved in:

R2-2407596 (Discussion summary)

[POST127][110][MOB] (Intel)

Scope: Update the draft CRs (if needed), discuss and finalize them. Prepare a LS to RAN1/4 to inform capability agreement and ask the question (see the discussion on R2-2406476).

Intended outcome: Endorsed 38.331 CR in R2-2407599 and 38.306 CR in R2-2407600. Approved LS in R2-2407601.

Deadline: Aug. 27th, 2024

=> Endorsed in:

R2-2407859 (38.331 draftCR)

R2-2407600 (38.306 draftCR)

=> Approved in:

R2-2407601 (LSout)

[POST127][111][MOB] (CATT)

Scope: Prepare a LS to SA3 to inform RAN2 agreements to ask them to take it into account for their works.

Intended outcome: Approved LS in R2-2407602.

Deadline: Short email discussion.

=> Approved in R2-2407602 (LSout)

[Post127][202][MIMOevo] CR for TS 38.321 (Samsung)

Scope: Update and review the CR for TS 38.321.

Intended outcome: Agreed CR

Deadline:  Short

=> Agreed in R2-2407843 (38.321 CR#1927)

[Post127][203][MIMOevo] CR for TS 38.331 (Ericsson)

Scope: Update and review the CR for TS 38.331.

Intended outcome: Agreed CR

Deadline:  Short

=> Agreed in R2-2407866 (38.331 CR#4969r1)

[Post127][204][MUSIM] CR for TS 38.331 (vivo)

Scope: Update and review the CR for TS 38.331

Intended outcome: Agreed CR

Deadline: Short

=> Agreed in R2-2407864 (38.331 CR#4970r1)

[Post127][301][R18 NR NTN] 38.300 CR (Thales)

Scope: Draft a 38.300 based on meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407621): short

=> Agreed in R2-2407621 (38.300 CR#0902)

[Post127][302][R18 NR NTN] 38.331 CR (Ericsson)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407611): short

=> Agreed in R2-2407611 (38.331 CR#4935r1)

[Post127][303][R18 NR NTN] 38.321 CR (Interdigital)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407628): short

=> Agreed in R2-2407628 (38.321 CR#1928)

[Post127][304][R18 NR NTN] UE Caps CRs (Intel)

Scope: Draft CRs based on meeting agreements

Intended outcome: Endorsed CRs

Deadline for draftCRs (in R2-2407624 and R2-2407625): Aug. 27th, 2024

=> Endorsed in:

R2-2407624 (38.306)

R2-2407625 (38.331)

[Post127][305][R18 IoT NTN] 36.300 CR (Ericsson)

Scope: Draft a 36.300 CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407626): short

=> Agreed in R2-2407868 (36.300 CR#1405r1)

[Post127][306][R18 IoT NTN] 36.331 CR (Huawei)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407615): short

=> Agreed in R2-2407615 (36.331 CR#5049r1)

[Post127][307][R18 IoT NTN] 36.321 CR (Mediatek)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407620): short

=> Agreed in R2-2407620 (36.321 CR#1588r1)

[Post127][308][R18 IoT NTN] 36.306 CR (Qualcomm)

Scope: Draft a 36.306 CR based on meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2407627): short

=> Agreed in R2-2407627 (36.306 CR#1892)

[Post127][309][R18 NR NTN] LS to RAN1 (CATT)

Scope: Draft a LS to RAN1 on correction on network verification of UE location

Intended outcome: Approved LS

Deadline for approved LS (in R2-2407630): short

=> Approved in R2-2407630 (LSout), but then coversheet revised by MCC in R2-2407887 (attachment was missing from the zip file)

[Post127][403][POS] LS to RAN1 on CSI-RS for spatial relation in RRC_INACTIVE (ZTE)

Scope: Draft an LS to RAN1 explaining the situation in RAN2 in relation to using RS from connected configuration as reference for spatial relation in RRC_INACTIVE, and ask whether UE can take the CSI-RS or SRS configuration provided in RRC_CONNECTED as the spatial relation RS to be applied in RRC_INACTIVE state.

Intended outcome: Approved LS

Deadline: Short (not for RP)

=> Approved in R2-2407853 (LSout)

[Post127][404][POS] Rel-18 positioning MAC CR (Huawei)

Scope: Check and update the CR in R2-2407722

Intended outcome: Agreed CR

Deadline: Short (for RP)

=> Agreed in R2-2407854 (38.321 CR#1883r2)

[Post127][405][POS] Rel-18 positioning RRC CRs (Ericsson)

Scope: Check the CRs in R2-2407721 and R2-2407769, including confirming list extension mechanism for R2-2407769.

Intended outcome: Agreed CRs

Deadline: Short (for RP)

=> Agreed in:

R2-2407883 (38.331 CR#4934r3)

R2-2407884 (38.331 CR#4965r2)

=> Noted in:

R2-2407863 (Summary)

[POST127][505][MBS] RRC CR (Huawei)

Scope: Update the RRC CR with the agreements from the meeting, discuss the related FFSes.

Intended outcome: Agreeable RRC CR in R2-2407736

Deadline: Short

=> Agreed in R2-2407736 (38.331 CR#4955r1)

=> Noted in R2-2407860 (Report)

[Post127][601][RRC] Miscellaneous non-controversial corrections Set XXII (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2407650, R2-2407651 and R2-2407652 (Ericsson)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

    - Friday morning session

=> Agreed in:

R2-2407650 (38.331 CR#4960r1, Rel-16)

R2-2407651 (38.331 CR#4916r1, Rel-17)

R2-2407652 (38.331 CR#4917r1, Rel-18)

[Post127][602][FeMIMO] Power control parameters to support unified TCI state framework (Ericsson)

Scope:

    - Produce agreeable CRs pending input from RAN1, if needed

Intended outcome:

    - Agreed CRs in R2-2407659 and R2-2407660 (Ericsson)

Deadline:

    - Short

=> Agreed in:

R2-2407659 (38.331 CR#4963r1, Rel-18)

R2-2407660 (38.331 CR#4964r1, Rel-17)

[Post127][603][SONMDT] SCG act/deact info (CMCC)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approved LS in R2-2407664 (CMCC)

Deadline:

    - Short

=> Approved in R2-2407664 (LSout)

[Post127][604][Maint] Parallel Tx capability (Ericsson)

Scope:

    - Implement the agreement about limiting parallel Tx capabilities into a CR

    - Check whether the following intra-band capabilities can also be limited to NR SA CA, in which case this should also be implemented in the CR:

parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17

parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17

parallelTxSRS-PUCCH-PUSCH-intraBand-r17

Intended outcome:

    - Agreed CRs

Deadline:

    - Short

=> Agreed in:

R2-2407635 (38.306 CR#1153, Rel-15)

R2-2407636 (38.306 CR#1154, Rel-16)

R2-2407794 (38.306 CR#1155, Rel-17)

R2-2407795 (38.306 CR#1156, Rel-18)

# Long email discussions, for R2-127bis, Deadline Sept. 27th, 10:00 UTC (unless otherwise stated)

Please request R2-127bis TDoc numbers for the following email discussions by 3GU according to normal tdoc submission procedure.

[POST127][019][AI PHY] LS to RAN1(Intel)

Intended outcome: Agree to list of questions to ask RAN1 to enable RAN2 progress.

Deadline:  two weeks

=> Available in R2-2407848 (LS out)

[POST127][022][AIoT] TP for TR (Huawei)

Intended outcome: Updated TP based on agreements from RAN2 127

Deadline:  long

[POST127][027][AI Mob] Simulation table (Mediatek )

Intended outcome: Agree how to capture simulation results

Deadline:  two weeks

=> Available in R2-2407886

[POST127][033][AIoT] Random Access (Huawei)

Intended outcome: Discuss Failure/success indication aspects and FFS for CBRA and on FFS on AS ID for scheduling purposes

Deadline:  long

[POST127][109][NES] (OPPO)

Scope: For each direction (N extension vs PF bundling): 1) List proposed options, 2) Discuss to understand each option better, 3) Down select options, 4) Discuss pros and cons for each direction or down-selected options (with the consideration of UE impacts, system impacts and specification job). F2F offline discussion in Brk3 room. => It is extended to long email discussion to continue detailed analysis of pros and cons for each direction.

Intended outcome: Discussion summary in R2-2407598. => Updated discussion summary

Deadline: Long email discussion.

# Extra long email discussions, for R2-128, Deadline Nov. 1st, 10:00 UTC (unless stated otherwise)

Please request R2-128 TDoc numbers for the following email discussions by 3GU according to normal tdoc submission procedure

Companies are encouraged to only trigger these discussions after the short email discussion/RAN plenary rush

[POST127][004][ASN.1 Modernization] requirements (Nokia)

Intended outcome: discuss requirements of the end of release ASN.1 review process

Deadline:  extra long

[POST127][029][AI mobility] RLF Simulation Assumption (Oppo)

Intended outcome: Agreeable simulation assumption

Deadline:  extra long

[Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

Scope:

  - Discovery message initiating/forwarding condition at intermediate relay UE

  - PC5 AS conditions for discovery at the last relay UE

  - Reselection triggers for all UEs

  - (Re)selection criteria for all UEs (considering discovery models A/B and whether criteria beyond the first hop are considered)

  - Whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last relay UE via one (physically same) intermediate relay UE

Intended outcome: Report to RAN2#128

Deadline: extra long

[Post127][402][Relay] Multi-hop relay control plane (InterDigital)

Scope:

- Describe different solutions (from company contributions) for multihop U2N relay UE by at least describing:

  - Connection establishment procedures

  - Assumptions on RRC state(s) of intermediate UEs and last relay UE

  - Assumptions on controlling gNB/cell of each relay UE

  - How the remote and intermediate relay UEs obtain their configurations in each solution

  - How to meet QoS requirement e2e

- Evaluate the feasibility and pros/cons of the different solutions towards downscoping to a single solution

Intended outcome: Report to RAN2#128

Deadline: extra long

# Annex H:	History

| Document history |  |  |
| --- | --- | --- |
| Date | TSG RAN WG2 Tdoc | Subject |
| 09.09.2024 |  | First draft version |
| 30.09.2024 |  | Second draft version |
| 04.10.2024 | R2-2407902 | Version submitted for approval |
| 09.10.2024 | R2-2409202 | Revised version submitted for approval |
| 13.10.2024 | R2-2409210 | Revised version submitted for approval |
| Author: Dr. Juha Korhonen ETSI Mobile Competence Centre (MCC) Tel. +33 (0)4 92 94 42 00 email: Juha.Korhonen@etsi.org |  |  |