3GPP TSG-RAN WG2 Meeting #127bis	R2-2408339

Hefei, China, 14 - 18 Oct, 2024

Agenda Item:	8.8.5

Source:	Huawei, HiSilicon, Turkcell

Title:	Discussion on the RA-SDT in Regenerative payload

Document for: Discussion and Decision

# 1	Introduction

In the last meeting the following agreements were achieved for the support of regenerative payload in NR NTN [1].

Agreements:

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

3.	RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

And the discussion of SDT in regenerative architecture is still expected to be open in this meeting.

Proposal 4 After early contention resolution, i.e., after sending HARQ feedback of the message containing only contention resolution MAC CE, a UE waits UE-gNB RTT to monitor the PDCCH for further downlink message. - Apple wonders if this is a NW implementation issue - Come back in the next meeting

In this paper we provide our further views on the SDT in regenerative architecture.

# 2	Discussion

In previous meetings, we explained our concern for the scheme that UE considers the feeder link RTT for the PDCCH monitoring after sending the contention resolution MAC CE HARQ feedback: it is unclear how to set the offset value between MAC CE and Msg4. And the uncertainty of feeder link RTT not only lead to the difficulty for setting the PDCCH waiting time, but also brings impact to the SDT failure detection timer.

But according to the agreement for the last meeting, the proposal that “UE waits UE-gNB RTT to monitor the PDCCH” is to be continued discussed in this meeting. It seems the feeder link RTT is not expected to be considered any more.

When it comes to the UE-gNB RTT (service link RTT only), the case for the regenerative architecture is nothing different from that for the transparent architecture, and we doubt any enhancement specific to regenerative payload is needed. Besides, the current RA-SDT procedure does not restrict that the NW can only schedule subsequent transmission after HARQ ACK for CR MAC CE is received, it is up to NW implementation.

Proposal: For the issue of UE waiting for a UE-gNB RTT before monitoring the PDCCH in regenerative payload architecture, nothing is different from that for the transparent payload architecture.

# 3	Conclusion

This contribution analyses the RA-SDT in regenerative payload, and proposes the following:

Proposal: For the issue of UE waiting for a UE-gNB RTT before monitoring the PDCCH in regenerative payload architecture, nothing is different from that for the transparent payload architecture.

# 4	References

- R2-2407573, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 127.