# 3GPP TSG-RAN WG2 Meeting #127bis		                  							R2-2408672

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.4

Source:	Robert Bosch GmbH

Title:	Discussions on Failure Detection/Recovery in Ambient IoT Devices

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN2 discussed the ambient IoT random access and reached the following agreement [1]:

Agreement

Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

# Failure Detection and Recovery

When an Ambient IoT (AIoT) device sends msg1 of the random access mechanism, it needs to wait for a limited amount of time to receive msg2 from a reader which contains its random ID transmitted in the msg1. The waiting time can be calculated and configured in a device based on the number of slots passed from the transmission of msg1 since in the slotted-ALOHA mechanism a device needs to recognize the beginning of time slots and can use a counter to calculate its waiting time for the reception of msg2.

Observation 1: AIoT devices need to recognize the start of time slots and be synchronized in a slotted-ALOHA mechanism.

Proposal 1: The waiting time of a device for reception of msg2 can be configured based on a number of slots. If an AIoT device does not receive any message after a configured number of slots, it detects a failure of msg1.

In another case, a device may receive a msg2 which does not contain its random ID or device ID. In this case, the device will recognize the loss in the contention and should try again with a new msg1.

Observation 2: Not reception of any msg2 after a limited number of slots can be interpreted as failure of msg1 or failure of msg2.

Proposal 2: A device needs to be configured to behave differently if no msg2 is received or a msg2 is received with a different random/device ID than the ID of the device.

Along with the transmission of msg2 by a reader to a AIoT device, the reader can also schedule time slots for the transmission of msg3 or further messages in the device. Upon the reception of msgs2, the device will send msg3 and further messages in the scheduled time slots. If the msg3 or any further message by the device which are sent in the scheduled time slots fail, there could be two possible options:

Proposal 3: In the case of failure of msg3 or further message i.e., D2R messages, a reader can:

Option 1: Re-schedule a new time slot for the AIoT device and send it to the device to indicate the failure of D2R messages.

Option 2: Send an indication such as a NACK to ask the device to re-start its transmission or the random access procedure.

In case of several repeated D2R failures, the reader may realize a bad channel condition or a problem with the device for the transmission of messages. In this case, the corresponding network node should be notified about this problem to avoid further triggering of the AIoT device.

Observation 3: Several repeated failures of D2R transmissions may be a sign of bad channel condition, low energy level of a device or etc.

Proposal 4: A reader should notify the corresponding network node i.e., core network component about the repeated D2R transmission failures to be considered in the future triggering of the AIoT device i.e., paging the device.

# Conclusions

Based on the discussion in the contribution, we have the following observation and proposals:

Observation 1: AIoT devices need to recognize the start of time slots and be synchronized in a slotted-ALOHA mechanism.

Observation 2: Not reception of any msg2 after a limited number of slots can be interpreted as failure of msg1 or failure of msg2.

Observation 3: Several repeated failures of D2R transmissions may be a sign of bad channel condition, low energy level of a device or etc.

Proposal 1: The waiting time of a device for reception of msg2 can be configured based on a number of slots. If an AIoT device does not receive any message after a configured number of slots, it detects a failure of msg1.

Proposal 2: A device needs to be configured to behave differently if no msg2 is received or a msg2 is received with a different random/device ID than the ID of the device.

Proposal 3: In the case of failure of msg3 or further message i.e., D2R messages, a reader can:

Option 1: Re-schedule a new time slot for the AIoT device and send it to the device to indicate the failure of D2R messages.

Option 2: Send an indication such as a NACK to ask the device to re-start its transmission or the random access procedure.

Proposal 4: A reader should notify the corresponding network node i.e., core network component about the repeated D2R transmission failures to be considered in the future triggering of the AIoT device i.e., paging the device.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #127”, August 2024.