3GPP TSG-RAN WG2 Meeting #127bis	R2-2409199

Hefei, China, 14th – 18th October 2024

Agenda Item:	8.16

Source:	Ericsson

Title:	LPP Impacts for B2b Signal addition

Document for:	Discussion and Decision

# Introduction

RAN Plenary#105 agreed to have a new WI “RP-242413, New WID on Introduction of BDS B2b Signal in A-GNSS”.

BDS has been supported in 3GPP since Rel-12 and over years it has been a Global navigation satellite systems primarily with the extension of new BDS signals, constellations etc.

As such the impact of stage2 for this WI is straight-forward; that is simply to add reference of the new ICD; the extension however to LPP requires some careful analysis.

A draft LPP CR has been provided in R2-2408036. We observe that one change as compared to the CR in R2-2408036 would be needed and is discussed below.

# LPP Impacts Ionospheric Model

As specified in ICD: 7.7 Ionospheric Delay Correction Model Parameters

7.7.1 Parameters Description

The BeiDou Global Ionospheric delay correction Model (BDGIM) contains nine parameters which are used to correct the effect of ionospheric delay for the single frequency user. Descriptions of these parameters are shown in Table 7-8.

In LPP, the Ionospheric delay model is depicted by KlobucharModel2Parametere with the 9 parameters alpha1 to 9 as below.

#### 6.5.2.2	GNSS Assistance Data Elements

#### –	KlobucharModel2Parameter

-- ASN1START

KlobucharModel2Parameter-r16 ::= SEQUENCE {

alfa1-r16			INTEGER (0..1023),

alfa2-r16			INTEGER (-128..127),

alfa3-r16			INTEGER (0..255),

alfa4-r16			INTEGER (0..255),

alfa5-r16			INTEGER (0..255),

alfa6-r16			INTEGER (-128..127),

alfa7-r16			INTEGER (-128..127),

alfa8-r16			INTEGER (-128..127),

alfa9-r16			INTEGER (-128..127),

...

}

-- ASN1STOP

Hence, there is no need to introduce new parameters C1 to C9 and same parameter alpha1 to alpha 9 can be reused.

The Ionospheric delay model can be represented by existing KlobucharModel2Parameter fields alpha1 to alpha9. This approach varies with what is currently captured in draft CR R2-2408036

Agree to LPP ASN.1 for KlobucharModel2Parameter as shown in Annex.

## Analysis of other parameters

## EOP (Earth Orientation Parameter)

The ICD provides EOP detail as below

The LPP captures the above parameter, and no change is expected except the reference for B2b.

Similarly for clock model and navigation model the existing IE can be reused and correct reference for B2b applicability needs to be added.

The EOP parameters and clock and navigation models can be reused for B2b addition with main impact to update the reference.

# 3	Conclusion

Observation 1	The Ionospheric delay model can be represented by existing KlobucharModel2Parameter fields alpha1 to alpha9. This approach varies with what is currently captured in draft CR R2-2408036

Observation 2	The EOP parameters and clock and navigation models can be reused for B2b addition with main impact to update the reference.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree to LPP ASN.1 for KlobucharModel2Parameter as shown in Annex.

# Annex

#### 6.5.2.2	GNSS Assistance Data Elements

#### –	KlobucharModel2Parameter

-- ASN1START

KlobucharModel2Parameter-r16 ::= SEQUENCE {

alfa1-r16			INTEGER (0..1023),

alfa2-r16			INTEGER (-128..127),

alfa3-r16			INTEGER (0..255),

alfa4-r16			INTEGER (0..255),

alfa5-r16			INTEGER (0..255),

alfa6-r16			INTEGER (-128..127),

alfa7-r16			INTEGER (-128..127),

alfa8-r16			INTEGER (-128..127),

alfa9-r16			INTEGER (-128..127),

...

}

-- ASN1STOP

KlobucharModel2Parameter field descriptions

alfa1 This field specifies the 1 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

alfa2 This field specifies the 2 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

alfa3 This field specifies the 3 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

alfa4 This field specifies the 4 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

alfa5 This field specifies the  parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor -2-3 TECU.

alfa6 This field specifies the 6 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

alfa7 This field specifies the 7 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

alfa8 This field specifies the 8 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

alfa9 This field specifies the 9 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49] or in 7.7.1 in [XX]. Scale factor 2-3 TECU.

# 4	References

[1] http://www.beidou.gov.cn/xt/gfxz/202008/P020230516558683155109.pdf