3GPP TSG-RAN WG2 Meeting #128	R2-2409617

Orlando, USA, Nov. 18th – 22nd, 2024

| Agenda Item: | 8.6.4 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Discussion on conditional LTM |
| Document for: | Discussion and Decision |

1.	Introduction

In RAN2#127bis meeting, the followings are agreed for conditional LTM (CLTM):

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this paper, we discuss the detailed procedure for CLTM. To be specific, we analyze the impact of NW-UE interaction between preparation and cell switch execution. Further, we discuss the detail of CLTM cell switch execution in terms of procedure and execution condition.

2.	Discussion

2.1.	NW-UE interaction between preparation and cell switch execution

Similar to CHO, CLTM may be a procedure for an LTM cell switch that is executed by the UE when one or more LTM cell switch execution conditions are met, where the execution conditions are provided to the UE within the candidate cell configurations in advance. Since Rel-18 LTM is a pre-configuration (i.e., candidate cell configuration) based mobility, Rel-18 LTM signaling procedure can be a baseline for discussion on CLTM signaling procedure.

All information required for handover in CHO is included in the RRCReconfiguration message to configure CHO. Hence, it is sufficient that the UE starts to evaluate the execution conditions for CHO candidate cells upon receiving CHO configuration via RRC. Based on the CHO procedure, we can design a CLTM procedure as CLTM without network(NW)-UE interaction between preparation and cell switch execution.

On the other hand, all information required for handover (i.e., cell switch) in LTM is seperately included in the RRCReconfiguration message to configure LTM and LTM Cell Switch Command (CSC) MAC CE. For example, the CFRA resource is included in the LTM CSC MAC CE for CFRA-based LTM cell switch execution or the beam information and TA information are included in the LTM CSC MAC CE for RACH-less LTM cell switch. This means that, for LTM cell switch, the UE needs the integrated information between RRC LTM configuration and LTM CSC MAC CE.

For Rel-19 CLTM, some companies may think it is necessary to introduce a new MAC CE indicating e.g. beam information or TA information for RACH-less CLTM cell switch and/or a new MAC CE indicating the CFRA resource for CFRA-based CLTM cell switch. For the purpose of RACH-less/CFRA-based CLTM cell switch, in our view, it is sufficient to reuse Rel-18 LTM CSC MAC CE. Further, the Rel-18 LTM procedure is reused as much as possible because it is good to minimize the specification work for CLTM. Based on the Rel-18 LTM procedure, we can design a CLTM procedure as CLTM with NW-UE interaction between preparation and cell switch execution.

Observation 1. Based on the CHO and Rel-18 LTM procedures, a CLTM procedure can be designed in two kinds as follows: 1) CLTM without NW-UE interaction between preparation and cell switch execution (see Figure 1); and 2) CLTM with NW-UE interaction between preparation and cell switch execution (see Figure 2)

- CLTM without NW-UE interaction between preparation and cell switch execution (see Figure 1): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE starts to evaluate the execution conditions for CLTM candidate cells upon receiving RRC CLTM configuration. If an execution condition is met, the UE performs the LTM cell switch.

- CLTM with NW-UE interaction between preparation and cell switch execution (see Figure 2): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE may perform early DL/UL synchronization to one or more CLTM candidate cells and performs L1 measurement/reporting of CLTM candidate cells. The network sends a CLTM CSC MAC CE. The UE starts to evaluate execution conditions for CLTM candidate cells upon receiving CLTM CSC MAC CE. If an execution condition is met, the UE performs the LTM cell switch.

Figure 1. CLTM without NW-UE interaction between preparation and cell switch execution

Figure 2. CLTM with NW-UE interaction between preparation and cell switch execution

In case of CLTM without NW-UE interaction, the network should reserve relevant resources (e.g., RA preamble) provided in the CLTM configuration after CLTM configuration until reconfiguration of CLTM or CLTM release. Since CLTM supports subsequent mobility, the resource reservation time may become too long, which may result in poor resource utilization. On the contrary, the network can reserve the resources for cell switch (e.g., RA preamble or CG) at an appropriate time in case of CLTM with NW-UE interaction, which may lead to prevent waste of resources.

Moreover, signaling overhead may occur between the network and the UE due to RRC reconfiguration to dynamically control resources for cell switch during subsequent mobility. On the other hand, with CLTM with NW-UE interaction, resources for cell switch can be controlled by the LTM CSC MAC CE, which is lighter than RRC messages. Hence, CLTM with NW-UE interaction between preparation and cell switch execution is a good way to support subsequent mobility.

Observation 2. Between preparation and cell switch execution, NW-UE interaction would be beneficial to both resource utilization and signaling overhead during subsequent mobility.

As described in Annex 5.1, RACH-less cell switch is beneficial for CLTM in terms of mobility latency. In CLTM without the NW-UE interaction, there may be several difficulties to support RACH-less cell switch. First, the information required for RACH-less cell switch (e.g., beam indication and TA information) may need to be frequently changed because of UE movement. For example, the network frequently should indicate beam indication by the RRC reconfiguration procedure for RACH-less CLTM cell switch when the UE moves fast, which can lead to signaling overhead and large interruption. Second, the opportunity of RACH-less cell switch may be reduced, i.e., most of CLTM cell switches may be executed with the RA procedure because the information about RACH-less cell switch included in RRC CLTM configuration becomes invalid at CLTM cell switch execution, which can reduce the usefulness of CLTM. Third, as mentioned in the observation 2, the reservation time of UL resources for RACH-less cell switch, which is configured in RRC CLTM configuration, may become too long to result in poor resource utilization.

Basically, configurations that change dynamically, such as the best beam, are better controlled by MAC rather than RRC because MAC signal is lighter and faster than RRC signal. For this reason, dynamic information about LTM cell switch (e.g., beam indication, TA information, RA preamble, etc.) is included in the LTM CSC MAC CE in Rel-18 LTM. In this context, it seems beneficial to provide the dynamic information about LTM cell switch into the LTM CSC MAC CE. In other words, frequent RRC reconfiguration can be avoided by utilizing LTM CSC MAC CE to dynamically reconfigure the information for LTM cell switch.

Observation 3. In terms of RACH-less cell switch, CLTM without the NW-UE interaction may have the following difficulties: frequent RRC reconfiguration for updating information about RACH-less cell switch; reduced usefulness of CLTM; and poor resource utilization. The difficulties can be addressed by the NW-UE interaction.

In CLTM with the NW-UE interaction, most of the existing Rel-18 LTM procedures can be reused for CLTM, i.e., preparation, early DL/UL synchronization, LTM cell switch execution, and LTM cell switch completion. Consequently, specification work can be minimized. Additionally, the existing Rel-18 LTM RRC signaling model can be reused because of similarity between LTM and CLTM. Based on the similarity, the RRC signaling for CLTM configuration may be designed by adding new information for CLTM on top of the RRC signaling for Rel-18 LTM. Then, the network can configure CLTM based on the existing LTM configuration by delta configuration and the network can flexibly indicate to the UE LTM or CLTM cell switch toward a candidate cell based on a single LTM candidate cell configuration, which result in flexibility for the network and signaling overhead reduction. Further, the UE can save its capability for LTM configuration. In other words, the UE may not need to store the CLTM configuration separately from the LTM configuration. As a result, the UE complexity can be reduced.

Observation 4. CLTM with the NW-UE interaction has the following advantages: 1) less specification work; 2) flexibility for the network; 3) signaling overhead reduction; and 4) UE complexity reduction.

Based on the observations 2, 3, and 4, we suggest that the signaling procedure for CLTM with NW-UE interaction between preparation and cell switch execution given in Figure 2 as a baseline for discussion on CLTM. In other words, the network and UE interact with each other for early DL/UL synchronization and/or L1/L3 measurement reporting between CLTM preparation and CLTM cell switch execution.

Proposal 1. In conditional LTM, the network and UE interact with each other for early DL/UL synchronization and/or L1/L3 measurement reporting between preparation and cell switch execution. The signaling procedure given in Figure 2 is the baseline for CLTM.

In the previous meeting, it was agreed that “RACH-less Conditional intra-CU LTM is supported”. For the early DL synchronization, it was agreed that “Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM”, i.e., the Rel-18 LTM early DL synchronization procedure is reused for Rel-19 CLTM.  However, it is FFS whether to reuse the Rel-18 LTM early UL synchronization procedure is reused for Rel-19 CLTM. In our view, the Rel-18 UL early synchronization can be reused. Namely, the UE can acquire the information of UL synchronization of one or more candidate cells between CLTM preparation and CLTM cell switch execution, i.e., the UL synchronization is acquired by RA based early TA or UE based TA.

Proposal 2. Rel-18 early UL synchronization procedure is used for early UL synchronization to candidate cells for CLTM.

Now, we need to discuss how to provide to the UE with the UL synchronization acquired by RA based early TA. In RAN2#126 meeting, some companies think RAR based option may be needed for CLTM. However, the RAR based option produces many issues, e.g., as follows:

- FFS which cell sends RAR to the UE, i.e., the current serving cell or the candidate cell

- FFS the UE needs to evaluate the validity of candidate cell TA by, e.g., TA timer

- FFS the UE action if the candidate cell TA becomes invalid

The issues listed above seems controversial so discussion on RAR based early TA for CLTM may take a lot of time to conclude. Further, RAN1 may need to be involved in order to resolve the issues, which makes it difficult to reach a conclusion.

Observation 5. RAR based early TA for CLTM may take a lot of time to conclude because it can produce many controversial issues.

To save the time for discussion, it seems like a good idea to reuse the Rel-18 way of providing to the UE with the TA of candidate cell, i.e., the TA of candidate cell is included in LTM CSC MAC CE.

Proposal 3. In CLTM, RAR based option for RA based early TA is not pursued, i.e., TA is included in the LTM Cell Switch Command MAC CE as in Rel-18 LTM.

If the Rel-18 way is reused for provision of candidate cell TA (i.e., Proposal 3 is agreed), the following question is: how can the UE distinguish whether the LTM CSC MAC CE is for CLTM or not? It is simple and straightforward that LTM CSC MAC CE includes the flag indicating whether or not the LTM cell switch is conditional LTM. For example, one reserved bit in Rel-18 LTM CSC MAC CE can be used for indicating whether or not the LTM cell switch is conditional LTM.

Proposal 4. LTM Cell Switch Command MAC CE includes the flag indicating whether or not the LTM cell switch is conditional LTM.

2.2.	CLTM execution procedure

If the LTM CSC MAC CE indicates non-conditional LTM (i.e., Rel-18 LTM), then the UE performs the LTM cell switch according to Rel-18 LTM cell switch execution procedure. If the LTM CSC MAC CE indicates conditional LTM, then the UE starts to evaluate the execution condition(s) associated with the candidate cell indicated by the LTM CSC MAC CE. If an execution condition is met, the UE performs the LTM cell switch.

Proposal 5. If the LTM Cell Switch Command MAC CE indicates CLTM, then the UE starts to evaluate the execution condition(s) associated with the candidate cell indicated by the LTM Cell Switch Command MAC CE.

After receiving the LTM CSC MAC CE indicating CLTM, the UE may not execute CLTM cell switch because, e.g., the beam quality of candidate cell becomes poor. In this case, the TA and beam indication included in the LTM CSC MAC CE become invalid after long time. If the UE executes the CLTM cell switch based on the invalid TA and beam indication, the UE fails the CLTM cell switch and consequently experiences large interruption due to connection re-establishment. Hence, it is an important issue for the UE how long TA and beam indication included in the LTM CSC MAC CE are valid. We think it is straightforward and simple to use the validity timer (e.g., TA timer). For example, the UE starts to evaluate the validity of TA and beam indication based on the configured timer value upon receiving the LTM CSC MAC CE indicating CLTM.

Proposal 6. If the LTM Cell Switch Command MAC CE for CLTM includes TA and beam indication, the UE evaluates the validity of TA and beam indication included in the LTM Cell Switch Command MAC CE based on the timer (e.g., TA timer), where the timer value is configured by the network.

If the execution condition is met while the timer is running, then the UE executes RACH-less LTM cell switch based on the LTM CSC MAC CE. If the timer expires before cell switch execution, the UE considers the TA and beam indication are invalid. In this case, the network can evaluate the validity because the network knows the configured timer value and the start timing of the timer (i.e., LTM CSC MAC CE). For this reason, the UE does not need to indicate to the network that the timer has expired.

Proposal 7. If the execution condition is met while the timer is running, then the UE executes RACH-less LTM cell switch based on the TA and beam indication included in the LTM Cell Switch Command MAC CE.

Proposal 8. If the timer expires before cell switch execution, the UE considers the TA and beam indication are invalid. The UE does not indicate to the network that the timer has expired.

If the execution condition is met after the timer expiry, the UE executes the LTM cell switch based on the preconfiguration, i.e., LTM candidate cell configuration provided via RRC in the preparation phase. For example, if CFRA resources are configured in the candidate cell configuration, the UE executes the LTM cell switch based on the CFRA configuration when the execution condition is met after the validity timer of TA and beam indication has expired.

Proposal 9. If the execution condition is met after the timer expiry, the UE executes the LTM cell switch based on the preconfiguration (i.e., LTM candidate cell configuration provided via RRC at preparation).

Based on the agreements made in the previous meeting, it is an open issue whether or not DG-based first UL transmission on target cell is supported for CLTM (i.e., FFS whether to support DG-based RACH-less CLTM).

Consider the case where beam information is not indicated to the UE for DG-based RACH-less CLTM. In this case, a candidate cell should reserve DG resources associated with its all beams for first UL transmission of the UE until the UE arrival is detected. This is because the candidate cell does not know which beam the UE will select to perform PDCCH monitoring for receiving DG scheduling information. Consequently, DG-based RACH-less CLTM may result in under-utilization of network resources. From this observation, in our view, RAN2 hesitated to decide whether to support DG-based RACH-less CLTM at the previous meeting.

In Rel-18 LTM, the candidate cell can start to reserve the DG resource associated with a specific beam based on the LTM CSC MAC CE. For example, the source cell sends the LTM CSC MAC CE including beam indication and TA, and notifies the candidate cell of the LTM cell switch trigger via Cell Switch Notification message. Assume that the CG resources are not configured in the candidate cell configuration. Then, the candidate cell can start to reserve the DG resource associated with the indicated beam upon the notification. In this manner, the network can avoid wasting UL resources in Rel-18 DG-based RACH-less LTM.

Based on the CLTM procedure described in Section 2.1, similar to Rel-18 LTM (i.e., mentioned above), the network can avoid UL resource waste in Rel-19 DG-based RACH-less CLTM. For instance, the source cell sends the LTM CSC MAC CE including beam indication, TA, and CLTM flag, and notifies the candidate cell of the CLTM cell switch trigger via Cell Switch Notification message. Assume that the CG resources are not configured in the candidate cell configuration. Then, the candidate cell can start to reserve the DG resource associated with the indicated beam upon the notification. Hence, we argue that DG-based first UL transmission on target cell is supported for CLTM. The network can start to reserve the corresponding DG resource based on the LTM CSC MAC CE.

Proposal 10. DG-based first UL transmission on target cell is supported for CLTM. The network can start to reserve the corresponding DG resource based on the LTM Cell Switch Command MAC CE.

2.3.	CLTM execution condition

Event evaluation based on multiple beams of candidate cell

RAN2 agreed that for network-triggered LTM, a single beam quality is used for event evaluation. It is FFS if event evaluation based on quality related to multiple beams for conditional LTM is supported.

The motivation of considering quality of multiple beams of candidate cell for event evaluation is to trigger cell switch based on more representative quality of a concerned candidate cell, rather than relying on a single good beam (or limited number of good beams). If the event evaluation based on the consolidated results of multiple RSs is supported, the UE can avoid triggering CLTM to candidate cell where there is a limited number of good beams (e.g., only one good beam) in the candidate cell. This event evaluation based on multiple beams of candidate cell is more than necessary for CLTM, since network has less control in triggering the cell switch.

Observation 6: Event evaluation based on multiple beams of candidate cell is more than necessary for CLTM to avoid cell switching to a candidate cell with a limited number of good beams (e.g., single good beam only), since network has less control in triggering the cell switch in CLTM.

There are two approaches to consider the quality of multiple beams of a concerned candidate cell.

Approach 1) One way of considering quality of multiple beams for CLTM is simply to calculate a consolidated value of the candidate cell beams and use the average value for evaluation CLTM execution condition.

If we go for the approach1, we can somehow reuse the cell quality derivation method as specified in 5.5.3.3a of 38.331, where beam quality threshold and the number of beams to be averaged are used to derive the cell quality.

Approach 2) Another way is that, instead of UE calculating the consolidated quality, network configures the minimum number of beams, say N as minimum number of good beams required for cell switch, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively. For example, if the value N set to 2 is configured for LTM event 3 for a candidate cell, the UE considers that the event is met if two or more beam of the candidate cell are offset better than the serving beam. In our view, the practical value range of N is {1, 2, 3, 4}, considering both FR1 and FR2.

In our view, the latter approach is simpler since consolidation of beam quality of multiple beams is not needed and it only needs to take single-beam based event evaluation (which RAN2 agreed to use for event triggered LTM report) for multiple beams into account. The approach2 can take a good balance between dynamic cell switching based on beam-based evaluation and too frequent cell switching.

Proposal 11: RAN2 to consider at least one of the following approaches for event evaluation based on multiple beams:

- Approach1) UE derives a consolidated quality of multiple beams of candidate cell based on a method similar to the cell quality derivation as specified in 5.5.3.3a of 38.331 and use the consolidated result for evaluation CLTM execution condition.

- Approach2) Network configures the minimum number of good candidate cell beams for CLTM event (event 3/4/5), say N. If N is configured for Event CLTM, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.  The value range of N is {1, 2, 3, 4}.

Use of CHO execution conditions as CLTM execution condition

It is FFS whether CHO execution conditions can be used as CLTM execution condition. We support using CHO execution conditions for CLTM execution conditions, whenever applicable. We note that the UE capability of L1 candidate cell beam reporting for LTM is already optional for UE supporting LTM, and for these UEs, L3 measurement reporting would be used for triggering LTM. Applying the same logic to CLTM, we believe that beam based event evaluation capability need not be mandatory for supporting CLTM. Along this, if UE does not support beam based event evaluation but supports CLTM, existing CHO execution conditions shall be used as CLTM execution conditions instead.

Proposal 12: Beam-based event evaluation is not mandatory for UE supporting CLTM.

Proposal 13: CHO execution conditions can be configured as CLTM execution conditions.

3.	Conclusion

In this paper, we have made the following observations and proposals:

NW-UE interaction between preparation and cell switch execution

Observation 1. Based on the CHO and Rel-18 LTM procedures, a CLTM procedure can be designed in two kinds as follows: 1) CLTM without NW-UE interaction between preparation and cell switch execution (see Figure 1); and 2) CLTM with NW-UE interaction between preparation and cell switch execution (see Figure 2)

- CLTM without NW-UE interaction between preparation and cell switch execution (see Figure 1): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE starts to evaluate the execution conditions for CLTM candidate cells upon receiving RRC CLTM configuration. If an execution condition is met, the UE performs the LTM cell switch.

- CLTM with NW-UE interaction between preparation and cell switch execution (see Figure 2): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE may perform early DL/UL synchronization to one or more CLTM candidate cells and performs L1 measurement/reporting of CLTM candidate cells. The network sends a CLTM CSC MAC CE. The UE starts to evaluate execution conditions for CLTM candidate cells upon receiving CLTM CSC MAC CE. If an execution condition is met, the UE performs the LTM cell switch.

Observation 2. Between preparation and cell switch execution, NW-UE interaction would be beneficial to both resource utilization and signaling overhead during subsequent mobility.

Observation 3. In terms of RACH-less cell switch, CLTM without the NW-UE interaction may have the following difficulties: frequent RRC reconfiguration for updating information about RACH-less cell switch; reduced usefulness of CLTM; and poor resource utilization. The difficulties can be addressed by the NW-UE interaction.

Observation 4. CLTM with the NW-UE interaction has the following advantages: 1) less specification work; 2) flexibility for the network; 3) signaling overhead reduction; and 4) UE complexity reduction.

Proposal 1. In conditional LTM, the network and UE interact with each other for early DL/UL synchronization and/or L1/L3 measurement reporting between preparation and cell switch execution. The signaling procedure given in Figure 2 is the baseline for CLTM.

Proposal 2. Rel-18 early UL synchronization procedure is used for early UL synchronization to candidate cells for CLTM.

Observation 5. RAR based early TA for CLTM may take a lot of time to conclude because it can produce many controversial issues.

Proposal 3. In CLTM, RAR based option for RA based early TA is not pursued, i.e., TA is included in the LTM Cell Switch Command MAC CE as in Rel-18 LTM.

Proposal 4. LTM Cell Switch Command MAC CE includes the flag indicating whether or not the LTM cell switch is conditional LTM.

CLTM execution procedure

Proposal 5. If the LTM Cell Switch Command MAC CE indicates CLTM, then the UE starts to evaluate the execution condition(s) associated with the candidate cell indicated by the LTM Cell Switch Command MAC CE.

Proposal 6. If the LTM Cell Switch Command MAC CE for CLTM includes TA and beam indication, the UE evaluates the validity of TA and beam indication included in the LTM Cell Switch Command MAC CE based on the timer (e.g., TA timer), where the timer value is configured by the network.

Proposal 7. If the execution condition is met while the timer is running, then the UE executes RACH-less LTM cell switch based on the TA and beam indication included in the LTM Cell Switch Command MAC CE.

Proposal 8. If the timer expires before cell switch execution, the UE considers the TA and beam indication are invalid. The UE does not indicate to the network that the timer has expired.

Proposal 9. If the execution condition is met after the timer expiry, the UE executes the LTM cell switch based on the preconfiguration (i.e., LTM candidate cell configuration provided via RRC at preparation).

Proposal 10. DG-based first UL transmission on target cell is supported for CLTM. The network can start to reserve the corresponding DG resource based on the LTM Cell Switch Command MAC CE.

Event evaluation based on multiple beams of candidate cell

Observation 6: Event evaluation based on multiple beams of candidate cell is more than necessary for CLTM to avoid cell switching to a candidate cell with a limited number of good beams (e.g., single good beam only), since network has less control in triggering the cell switch in CLTM

Proposal 11: RAN2 to consider at least one of the following approaches for event evaluation based on multiple beams:

- Approach 1) UE derives a consolidated quality of multiple beams of candidate cell based on a method similar to the cell quality derivation as specified in 5.5.3.3a of 38.331 and use the consolidated result for evaluation CLTM execution condition.

- Approach 2) Network configures the minimum number of good candidate cell beams for CLTM event (event 3/4/5), say N. If N is configured for Event CLTM, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.  The value range of N is {1, 2, 3, 4}.

Use of CHO execution conditions as CLTM execution condition

Proposal 12: Beam-based event evaluation is not mandatory for UE supporting CLTM.

Proposal 13: CHO execution conditions can be configured as CLTM execution conditions.

4.	References

[1] TS 38.300 NR and NG-RAN Overall Description (Release 18)

[2] TS 38.133 Requirements for support of radio resource management (Release 18)

5.	Annex

5.1.	Potential gains from RACH-less cell switch in CLTM

In [1], the mobility latency charts for RACH-based LTM and RACH-less LTM are presented as follows (i.e., Figure G-2 and Figure G-3).

Figure G-2: Mobility Latency for RACH-based LTM

Figure G-3: Mobility Latency for RACH-less LTM

Basically, in terms of mobility latency, the main difference between RACH-based LTM and RACH-less LTM comes from the latency due to random access. In [2], the LTM cell switch delay is expressed as

DLTM = Tcmd + TLTM-interrupt,

where Tcmd = THARQ + 3ms, and TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. Based on the mobility latency charts, Tfirst-RS, TRS-proc, and TLTM-IU can be reduced by RACH-less LTM cell switch, where Tfirst-RS and TRS-proc are respectively the time to the first SSB transmission on the target cell after Tcmd and the time for SSB processing, and TLTM-IU is the interruption uncertainty during LTM cell switch. Note that TRS-proc = 2 ms for RACH-based LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [2] and it can be up to 160 ms. Further, in case of RACH-based LTM cell switch, TRAR is added to the mobility latency as shown in Figure G-2, where the worst case TRAR can be calculated by multiplying the maximum value of RAR window size (i.e., 10 ms) and the maximum number of RA preamble transmission (i.e., 200), i.e., TRAR can be up to 2 seconds. Finally, the increased mobility latency due to RACH is expressed as

ΔRACH = DLTM(RACH-based) - DLTM(RACH-less) = Tfirst-RS + TRS-proc + TLTM-IU + TRAR,

where ΔRACH can be larger than 2 seconds based on the analysis.

Observation A1. RACH-based LTM has less benefit than RACH-less LTM in terms of mobility latency.

In Rel-18, LTM is introduced in order to reduce the latency, overhead, and interruption time. For instance, L2 reset skipping for intra-DU LTM cell switch and RACH-less LTM cell switch based on early DL/UL synchronization are introduced in order to reduce latency and interruption. This means that the conditional LTM (CLTM) has smaller latency/interruption than CHO. Compared to CHO, RACH-based conditional LTM (CLTM) shows the latency gain due to skipping L2 reset in intra-DU LTM case. For CHO and LTM, the maximum value of UE processing delay (i.e., TProcessing or TLTM-Processing) is 20 ms for intra frequency case and 40 ms for inter frequency case [2]. This implies that the latency gain from skipping L2 reset is within 20 ms for intra frequency case and 40 ms for inter frequency case, respectively. In our view, the latency gain from skipping L2 reset is marginal compared to the latency gain from skipping RACH because the latency gain from skipping RACH is up to 2 seconds in the worst case. Hence, RACH-based CLTM may have a marginal latency gain compared to the CHO.

Observation A2. The potential latency gain of RACH-based CLTM compared to the CHO is marginal.

Based on the observations A1 and A2, we observe that the gains from CLTM can be further increased by supporting RACH-less cell switch.

Observation A3. The gains from CLTM can be further increased by supporting RACH-less cell switch.