3GPP TSG-RAN2 #128	R2-2409630

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda item:	8.6.3 (NR_Mob_Ph4-Core)

Source:	LG Electronics Inc.

Title: 	Event triggered LTM report

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss issues related to event triggered LTM measurement report.

2. Discussion

2.1 Event evaluation and report triggering

2.1.1 TimeToTrigger

TTT granularity

In RAN2#127bis, RAN2 briefly discussed how TTT should work for beam-based event evaluation, but the decision was postponed.

Two options are on the table.

- Option1: TTT per cell [R2-2408525, ZTE]

- Option2: TTT per beam .

Taking LTM event3 as example, the two options can be understood as follows:

In option1, if a candidate beam satisfies entering condition of the event, TTT is initiated only if there is no other running TTT for the event. That is, only the first-entering beam can initiate TTT. Once TTT is initiated, the TTT keeps running as long as there is at least one beam satisfying the entering condition of the event, even after the first-entering beam satisfies the leaving condition. TTT stops running only if no beam satisfying the entering condition of the event remains.

We note that, in option1, immature report may be triggered by beam that has not satisfied the event for the entire duration of TTT as shown in the Figure 1. In this example, report based on TTT per cell is triggered by beam2 but beam2 has only satisfied half of the TTT. Note that the whole purpose of TTT is to ensure that the reported result is robust enough based on evaluation with the TTT, but the option1 is against the purpose. Since the beam2 cannot be considered to be a robust good beam, triggering a cell switch to the cell only relying on the beam2 is a risky mobility decision. Immature mobility decision cannot be justified by fast mobility decision.

Figure 1 in [R2-2408525]. An example of the TTT evaluation (per beam vs. per cell)

Observation 1: TTT per cell may lead to immature beam measurement reporting, because measurement reporting can be triggered by beam that has only satisfied a partial duration of TTT.

In option2, whenever a beam satisfies entering condition of an event, the corresponding TTT is initiated. Measurement reporting is only triggered by beam that has satisfied the entering condition for the whole duration of TTT, which is in line with the whole purpose of TTT. Therefore, the beam measurement report provides the robust beam result that is useful for robust mobility decision.

Based on the above discussion, we believe that TTT per beam should be adopted for beam based measurement reporting for LTM, and measurement reporting should be triggered by beam that satisfies the entering condition for the whole duration of TTT.

Proposal 1: TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

TTT operation upon change of the current serving beam

For every Event LTM2,3, and 5, the current beam quality is involved.

We fist discuss Event LTM3. In Event LTM3, candidate cell beam is compared with the current serving beam. Reminding that the whole purpose of TTT is used to filter out a robust good beam by checking whether the concern beam satisfies the condition for the whole TTT duration, Event LTM3 should be considered to be fulfilled only if the concerned candidate cell beam is better than the current serving beam for the whole TTT duration with no uncertainty.

If the serving beam is changed while TTT for Event LTM3 is running, two options are possible

- Option1: Reset TTT

- Option2: Do not reset TTT

  - Option2a: Keep TTT if the new serving beam satisfies the entering condition, and reset otherwise.

  - Option2b: Keep TTT unconditionally

In our view, option1 is simplest and sufficient, provided that TTT for event triggered LTM report is relatively short, and hence the chance of the current beam while TTT is running is not high.

Option2a is not a wrong approach, but it unnecessarily complicates UE behaviours since UE should determine whether to keep TTT running or not based on the initial evaluation result of the new serving beam quality. Option2b may result in immature or misleading LTM report due to the non-reset TTT, which then triggers unnecessary cell switch even if the new serving beam may be sufficient good enough to avoid any need of cell switch. In short, we think option2 is an over optimization.

Now we move onto Event LTM2, and Event LTM5. The common part of these two events is that the current serving beam is evaluated with a threshold. For these event, the above options (option1 and option2a, 2b) are equally applicable for the serving beam evaluation, and we believe that option1 is simplest and sufficient.

Based on the discussion for Event LTM2,3 and 5, we conclude that it is sufficient to reset TTT upon serving beam change while TTT is running

Note that Event LTM4 evaluates candidate cell beam (entering/leaving) and is not affected by the change of the current serving beam. So TTT operation related to Event LTM4 should be independent of the change of the current serving beam.

Proposal 2: For Event LTM 2, 3,5, if current beam is changed, TTT is reset. Event LTM4 is independent of change of current serving beam.

Handling of subsequent LTM reporting

In case TTT per beam is applied, it may happen that two or multiple beams satisfy the entering condition of the same event at slight different times. Consequently, corresponding TTTs may expire at slightly different times. Then, L1 measurement reports are triggered successively at slight different times, as shown in the Figure 2.

Figure 2. Successive LTM MRs triggered by the same report config

Note that this successive reporting is not a unique issue for event triggered LTM MR and such successive reporting can already occur in event triggered L3 MR in case two different cells of the same measurement object satisfy the entry condition of the same event at different times (say, cell A first and then cell B) and they keep satisfying the entry condition for TTT respectively. In that case, the cell A triggers an initial measurement reporting upon expiry of TTT for cell A and the cell B triggers a subsequent measurement reporting upon expiry of TTT for cell B. If the second measurement reporting is triggered, the numberOfReportsSent is reset but the existing cellsTriggeredList is not reset but updated by the cell B (i.e., cellB is included into the cellsTriggeredList).

If we decide that L1 Event LTM report follows event triggered L3 MR approach, the subsequent LTM report triggering may be frequent. For example, if the acquisition times of beam measurement results for different beams are slightly different, each beam will trigger reporting separately. If two different beams satisfy the entering condition at slight different times, each beam will trigger reporting separately. Higher dynamics of beam quality also contributes to more frequent subsequent LTM reporting, compared to L3 cell quality where exponential weighted moving average smoothens the short-term quality variation.

To reduce unnecessary/successive LTM report, some company proposed a prohibit timer approach. We think there are other approaches that are worth considering, which can be further discussed if RAN2 sees it beneficial to reduce unnecessary subsequent/successive event triggered LTM report.

Proposal 3: To discuss the issue of successive event triggered LTM report triggered by two different beams satisfying the entering condition of the same event at slightly different times, and decide either of the two approaches:

- a) to consider some restrictions to avoid unnecessary successive reporting (FFS details);

- b) to let each beam trigger separate measurement reporting without any restriction, as similar to the existing L3 event triggered MR.

2.1.2 Multi-beam restriction for report triggering

For selecting a cell switch candidate, network may be interested in a candidate cell that can provide at least N good beams. For example, network wants to trigger cell switch to a cell only if the cell can offer at least two good beams. In this case, if UE sends a first Event LTM report triggered by a single good beam, network has to wait more to receive subsequent reports until the subsequent report includes sufficient number of good beams. In this case, the first Event LTM report is considered redundant from network point of view, because the subsequent report including the sufficient good beams is only necessary for triggering a cell switch to the cell.

To avoid such redundant/unnecessary reporting, network can configure a restriction parameter N for configured Event LTMs, so that the event is considered to be met if at least N triggering beams satisfy the entry condition of the Event LTM. For example with N=2, in case of Event LTM3, UE triggers reporting if at least 2 beams of a candidate cell is offset better than the serving cell beam.

Proposal 4: Network can configure N for Event LTMs. If N is configured, the event is considered to be met if at least N beams satisfy the entry condition of the event. N can be configured as one of {1,2,3,4} and default value of N is one.

2.1.3 Activation/deactivation of configured Event LTM report conditions

When a candidate cell becomes a new serving cell, UE applies the candidate cell’s configuration including Event LTM report configuration, as serving cell configuration. If the Event LTM report configuration includes several (or large number of) LTM events/conditions, satisfaction of entry/leaving conditions of those events would happen frequently, and this will result in large LTM report signaling overhead. In contrast, if the Event LTM report configuration includes minimal LTM events/conditions, immediate LTM report signaling overhead would be small, but the new serving cell may need to configure UE with additional LTM events/conditions to trigger proper LTM reports via reconfiguration, which also increases signaling overhead. Furthermore, for LTM cell switching, network may need to update Event LTM report conditions more frequently to adapt to changes in beam conditions, and if we consider subsequent cell switching, the resulting update of Event LTM report conditions could be more frequent. Considering the potential UL LTM report signaling overhead and DL LTM report condition reconfiguration overhead, we think dynamic activation or deactivation of LTM Events via L1/L2 signaling would be beneficial. Network can configure several LTM report conditions and activate some of them as per their demand dynamically.

Proposal 5: Network can activate or deactivate some of the configured Event LTM report conditions via MAC CE.

2.2 Contents of MR MAC CE

2.2.1 Construction of N beams in LTM MR MAC CE

RAN2#127bis made the following agreements.

Agreements on L1 event triggered MR 13. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. 14. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration.

Given that the MR MAC CE can include up to N beams, we should decide which N beams should be included in the report. In our view, it is beneficial for MR MAC CE to include the following information.

- Triggered beam information, up to K triggered beams as essential information

  - The triggering beam is the beam that has fulfilled the event and thus triggered the MR

- Non-triggered beam information, up to L non-triggered beams of the candidate cell(s)

  - This information is useful for network to figure out the presence of reasonably good candidate cell beams

- Serving cell activated beam information, up to M best activated beams of the special cell

  - this information is useful for network to determine whether cell switch is better or beam switching without cell switch is better

In the above, the K, L, M is non-negative integer and not larger than N, and configured by network. K+L+M may be larger than N, and in this case, the UE may need to fill up the beams with some priority order. For example, UE first fills up the triggered beams up to K beams, and then if the number of triggered beams is less than K, UE can further fill up the non-triggered best beams up to L beams, and then if there is remaining room, UE can further fill the best activated beams up to M beams.

Proposal 6: The N beams comprises

- a) up to K triggered beam(s) (those fulfilling the event),

- b) up to L non-triggering best beams of the candidate cell(s), after filling up the triggering beams

- c) up to M best activated beam(s) of the serving cell, after filling up the triggering beams and non-triggering beams

- where K, L, and M are configured by network. K,L, and M are not greater than N, and K+L+M may be larger than N.

Proposal 7a: For non-triggering beams in MR MAC CE triggered by Event LTM2, UE can include up to L best beams for all candidate cells.

Proposal 7b: For non-triggering beams in MR MAC CE triggered by Event LTM3/4/5, UE can include up to L best beams of the candidate cell.

Proposal 8: For serving cell activated beam information in MR MAC CE triggered by Event LTM2/3/4/5, UE can includes up to M best activated beams of the special cell.

2.2.2 L3 measurement results in LTM MR MAC CE

Even for beam-based cell switch decision at network side, L3 cell quality of the serving/candidate cell is a good side information for the decision making.

Regarding the L3 measurement result of the special cell, since L3 measurement result of the special cell is always available, UE can easily include L3 RSRP of the special cell in the LTM MR MAC CE, if there is a room available in the UL grant.

Regarding the L3 measurement result of the candidate cell, if the candidate cell beam satisfies Event LTM, it is quite likely that L3 measurement result of the candidate cell is available. Therefore, UE can also include L3 RSRP of the special cell to assist network cell switch decision, if there is a room available in the UL grant.

Network should be able to configure UE with whether to include the L3 RSRP of the special cell and/or candidate cell, so that it can figure out the required grant size of the MR MAC CE, when triggered.

Proposal 9a: MR MAC CE triggered by Event LTM2 can include L3 measurement results of the special cell, based on network configuration (via include/notInclude flag).

Proposal 9b: MR MAC CE triggered by Event LTM3/4/5 can include L3 measurement results of the special cell and the candidate cell, based on network configuration (via include/notInclude flag).

2.2.3 Number of dedicated SRs for LTM MR MAC CE

RAN2 agreed in RAN2#127bis to introduce dedicated SR for LTM MR MAC CE, and details are FFS. In the following, we discuss how many SRs should be defined for LTM MR MAC CE.

If the maximum size of the LTM MR MAC CE is not much different across different LTM MR MAC CEs, a single dedicated SR would be sufficient and network can determine the max size of the LTM MR MAC CE based on RRC configuration controlling the content of the MAC CE and then determine the corresponding UL grant.

But if maximum size of the MR MAC CE can be very different across different MR MAC CEs and network wants to minimize the truncated part of the contents of the MR MAC CE, it is more efficient to define multiple dedicated SRs corresponding to different max sizes. For example, one may consider introducing two dedicated SRs such that one is applicable for LTM MR MAC CE triggered by Event LTM2 (i.e., short LTM MR MAC CE) and the other is applicable for LTM MR MAC CE triggered by Event 3/4/5 (i.e., long LTM MR MAC CE).

In our view, more than one dedicated SR is not justified, because a) the essential information that must be included in the LTM MR MAC CE is limited to triggered beams which can be only up to several beams (and extra information for network assistance can be provided on a best-efforts basis depending on UL grant), and b) network can control the max size of the essential information and max size of the extra information via RRC configuration, as proposed in our proposal 6 to 9. So we believe that a single dedicated SR for LTM MR MAC CE is sufficient.

Proposal 10: Introduce a single dedicated SR for LTM MR MAC CE.

3. Conclusion

Our proposals are listed below.

TTT granularity

Observation 1: TTT per cell may lead to immature beam measurement reporting, because measurement reporting can be triggered by beam that has only satisfied a partial duration of TTT.

Proposal 1: TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

TTT operation upon change of the current serving beam

Proposal 2: For Event LTM 2, 3,5, if current beam is changed, TTT is reset. Event LTM4 is independent of change of current serving beam.

Handling of subsequent LTM reporting

Proposal 3: To discuss the issue of successive event triggered LTM report triggered by two different beams satisfying the entering condition of the same event at slightly different times, and decide either of the two approaches:

- a) to consider some restrictions to avoid unnecessary successive reporting (FFS details);

- b) to let each beam trigger separate measurement reporting without any restriction, as similar to the existing L3 event triggered MR.

Multi-beam restriction for report triggering

Proposal 4: Network can configure N for Event LTMs. If N is configured, the event is considered to be met if at least N beams satisfy the entry condition of the event. N can be configured as one of {1,2,3,4} and default value of N is one.

Activation or deactivation of configured Event LTM report conditions

Proposal 5: Network can activate or deactivate some of the configured Event LTM report conditions via MAC CE.

Construction of N beams in LTM MR MAC CE

Proposal 6: The N beams comprises

- a) up to K triggered beam(s) (those fulfilling the event),

- b) up to L non-triggering best beams of the candidate cell(s), after filling up the triggering beams

- c) up to M best activated beam(s) of the serving cell, after filling up the triggering beams and non-triggering beams

- where K, L, and M are configured by network. K,L, and M are not greater than N, and K+L+M may be larger than N.

Proposal 7a: For non-triggering beams in MR MAC CE triggered by Event LTM2, UE can include up to L best beams across all candidate cells.

Proposal 7b: For non-triggering beams in MR MAC CE triggered by Event LTM3/4/5, UE can include up to L best beams of the candidate cell.

Proposal 8: For serving cell activated beam information in MR MAC CE triggered by Event LTM2/3/4/5, UE can includes up to M best activated beams of the special cell.

L3 measurement results in LTM MR MAC CE

Proposal 9a: MR MAC CE triggered by Event LTM2 can include L3 measurement results of the special cell, based on network configuration (via include/notInclude flag).

Proposal 9b: MR MAC CE triggered by Event LTM3/4/5 can include L3 measurement results of the special cell and the candidate cell, based on network configuration (via include/notInclude flag).

Number of dedicated SRs for LTM MR MAC CE

Proposal 10: Introduce a single dedicated SR for LTM MR MAC CE.