3GPP TSG-RAN WG2 Meeting#128	R2-2409705

Orlando, USA, 18th – 22nd Nov 2024

Source:	vivo

Title:	Discussion on RAN2 issues of AI/ML enhanced positioning

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

RAN2 127bis meeting has made progress on functionality-based LCM of UE-sided model for AI enhanced positioning as follows [1].

Agreements 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In this paper, we mainly focus on the leftover issues for AI-enhanced UE-based DL positioning in terms of RAN2 from the following perspectives:

- Applicable functionality reporting

- Consistency for training and inference

- Signaling of performance monitoring/functionality management

- AI positioning method

- Discussion

## Applicable functionality reporting

According to RAN2 agreed procedure, Step 4 is illustrated as “UE reports the applicable functionality to the LMF by LPP provide capabilities message.” And it is for further study on whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step 4 is sent reactively or proactively).

From our understanding, after the target UE has provided the supported functionalities to the LMF, the UE is aware that LMF is interested in the applicable functionalities for further management. Furthermore, the applicability of functionality varies in whether NW-sided additional condition is applicable for the current scenario, or UE-sided condition can be supported for AI functionality. In this sense, with the provided NW-sided conditions, UE is highly counted on for judging whether the functionality is applicable in a proactive manner without additional trigger from LMF.

- UE reports the applicable functionality to LMF proactively on condition that UE receives the request of supported functionalities and NW-side additional condition.

## Consistency for training and inference

RAN1#118bis meeting came to the agreement that on-demand PRS procedure can be used for training data collection [2].

Conclusion For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

For Case 1, since UE owns AI/ML model/functionality, it has the knowledge of DL PRS configurations during training stage. When it comes to the stage of inference, to ensure the consistency of training and inference, UE may request for suitable PRS configurations via UE-initiated on-demand PRS scheme. In this way, when assigning DL PRS configurations for inference, LMF can forward such preference to gNB, and consistency can be ensured.

- On-demand PRS scheme can be utilized to ensure consistency of NW-side configuration between training and inference.

The current on-demand PRS scheme is designed to ensure the positioning QoS, therefore only considering certain characteristics (e.g., bandwidth, periodicity) of the PRS, without requesting specific cells to transmit PRS. However, from our understanding, except for PRS configuration itself, the consistency should be ensured by the same PRS transmission cells. Therefore, the target UE should send the area where the interested PRS is applicable to LMF. With this priori knowledge, LMF is able to attach area and PRS configurations to specific NW-side additional condition. Furthermore, LMF may request the RAN-node near the UE and match with the condition(s) to perform DL positioning accordingly.

- Include the cell information in on-demand PRS request to ensure consistency during training and inference.

## Signaling related to performance monitoring/functionality management

At the previous RAN1 meetings, the following agreements/conclusions were made regarding the performance monitoring of AI/ML positioning Case 1.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Summarized from the RAN1 agreement, the performance monitoring and functionality management can be realized in the following options for positioning use case 1:

- Option 1: UE performs monitoring metric calculation and functionality management

- Option 2: LMF performs monitoring metric calculation and UE performs functionality management

- Option 3: UE performs monitoring metric calculation and LMF performs functionality management

- Option 4: LMF performs monitoring metric calculation and functionality management

For Option 1, with no LMF perception of which AI/ML functionality is enabled, UE autonomously decides to enable AI/ML based positioning and performs functionality management, UE may report AI/ML output of location prediction as the legacy location estimate. However, since RAN2 has agreed that “’UE-autonomous, UE’s decision is not reported to the network’ is not considered for Rel-19”, no further discussion on Option 1 is needed in WI phase.

As to Option 2, for the LMF-involved positioning (i.e., not UE autonomous positioning) procedure, we tend to believe that UE cannot perform AI functionality management under LMF decision on positioning method, where AI functionality is designed to be a positioning method on the trend. That is, if the target UE receives the request of location information attached to positioning method correspond to a specific functionality (FFS the granularity), it should consider the functionality is activated, while those not requested are deactivated. Therefore, Option 2 is excluded naturally.

In this sense, functionality management can only be performed by LMF in terms of R19 scope and positioning system layout.

- LMF is responsible for functionality management based on performance monitoring results calculated by target UE or LMF.

For Option 3, functionality management is realized by LMF requesting UE with AI-enabled location information just as illustrated in Option 2.

- Functionality management (e.g., activation/deactivation) is realized implicitly by whether LMF requesting UE for location information with the positioning method related to the AI functionality, i.e., UE considers the functionality is activated when the corresponding method is requested for UE to provide location estimate.

Furthermore, based on RAN1 input, LMF may provide some assistance information to UE to achieve performance monitoring metric calculation at UE, e.g., ground truth label of the target UE (Option A-1), position calculation assistance data (Option A-2), PRU measurement or PRU location (Option A-3). UE can also obtain label from PRU in 3GPP-transparent way (Option A-4), which has no specification impact and is outside RAN2 scope. Logically speaking, if LMF configures UE to perform performance monitoring, or UE itself has intention to perform performance monitoring, the information can be provided via LPP Assistance Data Transfer procedure in both proactive/reactive way.

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring metric calculation at target UE, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data.

Furthermore, when UE plays the role of performance monitoring metric calculation, it is of possibility that the performance of AI functionality deteriorates and becomes unable to fulfill the requested positioning QoS, e.g., horizontalAccuracy. In this scenario, UE should feedback a positioning error indication to LMF to help with LMF managing the functionalities, e.g., LMF re-sends a new request for location information of another applicable functionality. However, the current error cause can not indicate this case and a new cause value can be considered, e.g., “the requested functionality inapplicable”.

#### – NR-DL-TDOA-TargetDeviceErrorCauses The IE NR-DL-TDOA-TargetDeviceErrorCauses is used by the target device to provide NR DL-TDOA error reasons to the location server. -- ASN1START NR-DL-TDOA-TargetDeviceErrorCauses-r16 ::= SEQUENCE { cause-r16 ENUMERATED { undefined, assistance-data-missing, unableToMeasureAnyTRP, attemptedButUnableToMeasureSomeNeighbourTRPs, thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA, locationCalculationAssistanceDataMissing, ... }, ..., [[ remoteUE-Indication-r18 ENUMERATED {true} OPTIONAL -- Cond NR ]] } -- ASN1STOP

- If the performance of AI-based positioning cannot fulfill the positioning QoS requested by LMF, the target UE should indicate an error of the AI functionality and provide no inference result in LPP Provide Location Information, setting the cause as the bad performance of AI functionality.

For Option 4, based on RAN1 input, to achieve performance monitoring metric calculation at LMF, the model output based on either target UE’s measurement (Option B-1) or PRU’s measurement (Optionb B-2) is needed from UE.

Since the model output is location estimate, which is already the location information request by LMF as in the legacy positioning session, this can directly reuse LPP Provide Location Information to transmit. And for the PRU measurement part, LPP Provide Assistance Data can be reused.

- To enable performance monitoring metric calculation at LMF, target UE should provide model inference output based on the measurement of target UE or PRU.

- To enable performance monitoring metric calculation at LMF, LMF obtains model inference output from target UE via LPP Request/Provide Location Information.

On the basis of the above analysis, LMF or UE is able to perform performance monitoring metric calculation. LMF can be of long-term decision while UE can make real-time to decide whether AI/ML positioning accuracy is trustworthy. However, RAN1 has been parallel discussing entities to perform performance monitoring, we would like RAN2 to wait for RAN1 progress on whether both options should be introduced in specification.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring metric calculation.

## AI positioning method

In consideration to the specialty of positioning protocol organization, the reporting and request of information/assistance data are organized in a per positioning method way. That is, except for some commonly used IEs which are defined outside the scope of any method but referred in each method, other IEs are designed method specifically. What is more, with regards to introduction of AI/ML based positioning, RAN1’s parallel discussion about what kind of assistance information of existing positioning method is needed explicitly/implicitly for AI enhanced positioning [2]. If most existing assistance data are not needed or new information is needed, the AI/ML-based positioning can be a separate new method.

Agreement For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable. Information; UE-based DL-TdoA; UE-based DL-AoD. 1; Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement. 2; Timing relative to the serving (reference) TRP of candidate NR TRPs. 3; DL-PRS configuration of candidate NR TRPs. 4; Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation. 5; SSB information of the TRPs (the time/frequency occupancy of SSBs). 6; Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 8; Fine Timing relative to the serving (reference) TRP of candidate NR TRPs. 9; PRS-only TP indication. 10; The association information of DL-PRS resources with TRP Tx TEG ID. 11; LOS/NLOS indicators. 12; On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation. 13; Validity Area of the Assistance Data. 14; PRU measurements together with the location information of the PRU. 15; Data facilitating the integrity results determination of the calculated location. 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). 17; Expected Angle Assistance information. 18; PRS priority list.

- RAN1 parallelly studies whether the existing assistance data is needed for AI/ML-based positioning explicitly or implicitly. If most existing assistance data can not be directly reused or new information is needed, the AI/ML-based positioning can be introduced as a separate new method.

Besides, from the perspective of forward compatibility, AI can be not always attached to the current existing positioning method. That is, the input data needed for AI/ML-based positioning can be individual from existing positioning methods onwards. In the future, if new measurements and assistance data are required, it is easy to add them into a separate positioning method dedicated for AI. In this sense, we prefer to introduce AI/ML-based positioning as a new positioning method rather than merge it with the existing positioning methods.

- Introduce AI/ML-based positioning as a new positioning method for a clear signaling structure and forward compatibility.

- Conclusion

In summary, we make the following proposals on functionality-based LCM for AI/ML enhanced positioning:

Applicable functionality reporting

- UE reports the applicable functionality to LMF proactively on condition that UE receives the request of supported functionalities and NW-side additional condition.

Consistency for training and inference

- On-demand PRS scheme can be utilized to ensure consistency of NW-side configuration between training and inference.

- Include the cell information in on-demand PRS request to ensure consistency during training and inference.

Signaling of Performance monitoring/functionality management

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring metric calculation at LMF, target UE should provide model inference output based on the measurement of target UE or PRU.

- LMF is responsible for functionality management based on performance monitoring results calculated by target UE or LMF.

- Functionality management (e.g., activation/deactivation) is realized implicitly by whether LMF requesting UE for location information with the positioning method related to the AI functionality, i.e., UE considers the functionality is activated when the corresponding method is requested for UE to provide location estimate.

- To enable performance monitoring metric calculation at target UE, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data.

- If the performance of AI-based positioning cannot fulfill the positioning QoS requested by LMF, the target UE should indicate an error of the AI functionality and provide no inference result in LPP Provide Location Information, setting the cause as the bad performance of AI functionality.

- To enable performance monitoring metric calculation at LMF, LMF obtains model inference output from target UE via LPP Request/Provide Location Information.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring metric calculation.

AI positioning method

- RAN1 parallelly studies whether the existing assistance data is needed for AI/ML-based positioning explicitly or implicitly. If most existing assistance data cannot be direcly reused or new information is needed, the AI/ML-based positioning can be introduced as a separate new method.

- Introduce AI/ML based positioning as a new positioning method for a clear signaling structure and forward compatibility.

- Reference

- RAN2#127bis meeting Chair’s Notes

- RAN1#118bis meeting Chair’s Notes