3GPP TSG-RAN WG2 #128	R2-2409729

Orlando, US, November 18-22, 2024

Agenda Item:	7.15.2

Source:	NEC

Title:	TP for SL enh in TS 38.321

Document for:	Discussion, Decision

Introduction

In this contribution, we will discuss some remaining issues in current spec for SL further enhancement.

Discussion

According to RAN2’s understanding, the procedure of triggering for carrier selection shall be, currently UE is configured with one or more carriers, when there is available data on specific sidelink logical channel, UE shall check whether there is selected sidelink grant on the carriers configured for the sidelink logical channel. If there is no sidelink grant on any carriers allowed for the sidelink logical channel, UE shall reselect other carriers which satisfies CBR condition. Otherwise, if there is sidelink grant on some carriers, makred as carrier set-A allowed for the sidelink logical channel, and the CBR keeping condition is/are satisfied, UE shall use the selected grant on the carriers within carrier set-A, otherwise, if none of the carriers within carrier set-A fulfils CBR keeping condition, UE shall select other carrier which fulfil CBR reselection condition.

It can be observed that the carrier set-A can be one or multiple carriers, which means UE is possible to be configured with one or multiple carriers before triggering of carrier selection.

UE is possible to be configured with one or multiple carriers before triggering of carrier selection.

However, according to current specification, which is shown in below:

If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, the MAC entity shall: 1> if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]): 2> for each carrier configured by upper layers associated with the concerned sidelink logical channel: 3> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: NOTE 1: In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation. 4> consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel when the carrier satisfies all the following conditions; 5> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 6> the carrier includes at least one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 5> else: 6> the carrier includes any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 1> else: 2> if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel, for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1: 3> select the carrier and the associated pool of resources. 2> else: 3> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: 4> consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the sidelink logical channel is allowed when the carrier satisfies all the following conditions; 5> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 6> the carrier includes at least one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 5> else: 6> the carrier includes any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured.

UE can only be configured with one carrier before triggering of carrier selection, which is mis-aligned with RAN2 understanding.

Therefore, it is suggested to allow that UE can be configured with multiple carriers before triggering of carrier selection.

It is suggested to allow that UE can be configured with multiple carriers before triggering of carrier selection.

Correspondingly, it is suggested to adopt the corresponding TP1.

It is suggested to adopt the corresponding TP1.

Secondly, according to current specification, under the condition when there is no selected grant on any available carriers, UE will traversal all carriers configured by upper layers, which is shown in below yellow highlighted sentence.

#### 5.22.1.11 TX carrier (re-)selection The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 38.214 [7] if CBR measurement results are available, or the corresponding sl-defaultTxConfigIndex configured by upper layers if CBR measurement results are not available. If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, the MAC entity shall: 1> if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]): 2> for each carrier configured by upper layers associated with the concerned sidelink logical channel: 3> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: NOTE 1: In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation. 4> consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel when the carrier satisfies all the following conditions; 5> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 6> the carrier includes at least one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 5> else: 6> the carrier includes any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 1> else: 2> if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel, for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1: 3> select the carrier and the associated pool of resources. 2> else: 3> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: 4> consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the sidelink logical channel is allowed when the carrier satisfies all the following conditions; 5> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 6> the carrier includes at least one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 5> else: 6> the carrier includes any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. The MAC entity shall: 1> if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection: 2> if Tx carrier (re-)selection is triggered, for each sidelink logical channel allowed on the carrier where data is available: 3> select one or more carrier(s) and associated pool(s) of resources among the candidate carriers with increasing order of CBR from the lowest CBR. NOTE 2: It is left to UE implementation how many carriers to select based on UE capability. NOTE 3: It is left to UE implementation to determine the sidelink logical channels among the sidelink logical channels where data is available and that are allowed on the carrier for which Tx carrier (re-) selection is triggered.

Yet under the condition when there is selected grant on available carriers, but none of the carrier satisfies CBR keeping condition, UE will not traversal all carriers configured by upper layers, which is not correct. So it is suggested to allow UE to traversal all carriers configured by upper layers for the associated sidelink logical channel under the condition there is selected grant on available carriers but none of the carrier satisfies CBR keeping condition.

It is suggested to allow UE to traversal all carriers configured by upper layers for the associated sidelink logical channel under the condition there is selected grant on available carriers but none of the carrier satisfies CBR keeping condition.

So it is suggested to adopt corresponding TP2.

It is suggested to adopt the corresponding TP2.

Conclusion

In this contribution, we have discussed the remaining issue in TS 38.321 for sidelink further enhancement, the brunch of observation and proposal are given in the following:

Observation 1	UE is possible to be configured with one or multiple carriers before triggering of carrier selection.

Proposal 1	It is suggested to allow that UE can be configured with multiple carriers before triggering of carrier selection.	2

Proposal 2	It is suggested to adopt the corresponding TP1.	2

Proposal 3	It is suggested to allow UE to traversal all carriers configured by upper layers for the associated sidelink logical channel under the condition there is selected grant on available carriers but none of the carrier satisfies CBR keeping condition.	4

Proposal 4	It is suggested to adopt the corresponding TP2.	4

Reference

- TP1 - 5.22.1.11 TX carrier (re-)selection The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 38.214 [7] if CBR measurement results are available, or the corresponding sl-defaultTxConfigIndex configured by upper layers if CBR measurement results are not available. If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, the MAC entity shall: 1> if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]): 2> for each carrier configured by upper layers associated with the concerned sidelink logical channel: 3> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: NOTE 1: In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation. 4> consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel when the carrier satisfies all the following conditions; 5> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 6> the carrier includes at least one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 5> else: 6> the carrier includes any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 1> else: 2> for each carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 287[19]): 3> if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel, for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1: 4> select the carrier and the associated pool of resources.

- TP2 - 5.22.1.11 TX carrier (re-)selection The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 38.214 [7] if CBR measurement results are available, or the corresponding sl-defaultTxConfigIndex configured by upper layers if CBR measurement results are not available. If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, the MAC entity shall: 1> if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]): 2> for each carrier configured by upper layers associated with the concerned sidelink logical channel: 3> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: NOTE 1: In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation. 4> consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel when the carrier satisfies all the following conditions; 5> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 6> the carrier includes at least one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 5> else: 6> the carrier includes any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 1> else: 2> if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel, for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1: 3> select the carrier and the associated pool of resources. 2> else: 3> for each carrier configured by upper layers associated with the concerned sidelink logical channel: 4> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: 5> consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the sidelink logical channel is allowed when the carrier satisfies all the following conditions; 6> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 7> the carrier includes at least one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 6> else: 7> the carrier includes any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. The MAC entity shall: 1> if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection: 2> if Tx carrier (re-)selection is triggered, for each sidelink logical channel allowed on the carrier where data is available: 3> select one or more carrier(s) and associated pool(s) of resources among the candidate carriers with increasing order of CBR from the lowest CBR. NOTE 2: It is left to UE implementation how many carriers to select based on UE capability. NOTE 3: It is left to UE implementation to determine the sidelink logical channels among the sidelink logical channels where data is available and that are allowed on the carrier for which Tx carrier (re-) selection is triggered.