3GPP TSG-RAN WG2 #128	R2-2409733

Orlando, USA, November 2024

Agenda Item:	8.7.5

Source:	OPPO

Title:	Discussion on RLC re-transmission related enhancements

Document for:	Discussion, Agreement

Introduction

For the RLC retransmission related enhancements, RAN2 has made some down selection of the enhancement options for both timely RLC retransmission and avoiding unnecessary retransmission. This paper will further discuss details for each aspect.

Discussion

Unnecessary retransmissions

To avoid unnecessary retransmissions, the following agreements are made:

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

For the agreed combined approach, the Tx side and Rx side stops transmission and reception independently. At the Rx side, it is agreed that the abandonment of SDU is based on a local timer, while it is not clear how the Tx side decides the retransmission of SDU is not needed. In the current specification, the SDU discarding at the RLC Tx side is based on the indication from the PDCP entity. PDCP decides whether a packet is outdated based on discardTimer or discardTimerForLowImportance. It is straightforward to follow the current principle that the SDU discard at Tx RLC is based on the indication from PDCP layer.

TS 38.322 ## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU. TS 38.323 - 5.3 SDU discard When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

RAN2 confirm SDU discard at TX side is based on the indication from PDCP as the current specification.

The next question is how to achieve the synchronization between the Tx side and Rx side, e.g., for a specific number (e.g., N) of SDUs,

- If Tx side decides not to transmit the SDUs, and Rx side still waits, there will be: 1) N * Rx_timer delay, the delay is more serious if a set of SDUs are discarded at the same time (in PDU set case); 2) How to handle the NACK feedback of the discarded SDU at Tx side needs to be discussed.

- If Tx side is going to transmit the SDUs while the Rx side decides to abandon, there will be data loss.

The un-sync decision of SDU discarding at Tx and Rx side may cause 1) additional delay especially in PDU set discarding; 2) special NACK SR handling for the discarded SDU at Tx side; 3) additional data loss.

Besides, according to the WID objective, the enhancement should support unnecessary retransmission avoidance, and the Rx side cannot identify whether the SDU to be abandoned is a new or retransmission. Or is a PDCP control PDU which may be essential.

Rx side cannot identify whether the SDU to be abandoned is a new or retransmission or an essential PDCP control PDU.

Therefore, it is necessary to have the indication from the Tx to Rx side to sync up the SDU discarding decision.

For the “combined” approach for avoiding unnecessary RLC retransmissions, the indication from Tx to Rx side is needed.

For the detailed format of the indication from Tx to Rx side, the following 2 approaches can be considered:

Alt-1: Use control PDU to indicate the SN of the discarded SDU at the Tx side, i.e., similar to PDCP SN gap report;

Alt-2: Use data PDU with no Data field, i.e., only header with SN number to indicate the discard information.

Alt-1 is similar to R18 PDCP SN gap report and Alt-2 is simpler considering specification impact.

For the indication from Tx to Rx side, RAN2 selects between the following options:

- Option 1: Use control PDU to indicate the SN of the discarded SDU at Tx side.

- Option 2: Use data PDU with no Data field, i.e., only header with SN number to indicate the discard information.

Timely RLC retransmission(s)

For timely retransmissions, it is agreed that we will focus on autonomous retransmission and polling enhancements.

Exclude enhanced status reporting. Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

For the comparison between polling enhancements and autonomous, as guided by the agreement, the key point is to understand how each option affects the capacity and packet delay.

From the capacity aspect, polling only requires the Tx side to set the “P” field to “1” but doesn’t introduce additional signaling overhead.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall: - set the P field of the AMD PDU to "1"; - set PDU_WITHOUT_POLL to 0; - set BYTE_WITHOUT_POLL to 0.

For autonomous retransmission, since there will be additional data transmissions compared to retransmission based on NACK feedback, there is additional signaling overhead.

In terms of signaling overhead, polling only requires the transmitting side to set the "P" field to "1", which does not introduce any additional signaling overhead. On the other hand, autonomous retransmission results in more data transmission, thereby increasing the signaling overhead.

From packet delay aspect, for a single PDU, autonomous retransmission seems better than polling since the retransmission can be performed without waiting for the status report. While for multiple PDUs (e.g., PDUs in a PDU set or for a service) the whole delay for autonomous retransmission may be worse than enhanced polling since autonomous retransmission occupies additional resources, impacts the follow-up data transmission, which may cause further congestion and result in increased overall latency for a service.

For delay, autonomous retransmission may cause more delay than polling for multiple PDUs (e.g., in a PDU set) transmission.

Therefore, the enhancement to polling is preferred considering both capacity and latency aspects.

For timely RLC retransmission, RAN2 to adopt polling enhancement.

The polling from UE for the UL traffic is triggered based on the following conditions:

-	If PDU_WITHOUT_POLL >= pollPDU;

-	If BYTE_WITHOUT_POLL >= pollByte:

-	Upon expiry of t-PollRetransmit

So timely polling can be achieved by the following options

Option-1: Introducing new polling trigger condition besides the above ones, e.g., the UE can trigger polling if the delay budget of the packet is below a threshold;

Option-2: Reuse the above existing polling trigger conditions but have separate configurations for small remaining delay budget data and normal data, i.e., UE can use the configuration with smaller value if there is small remaining delay budget data.

For the polling enhancement in RLC AM, RAN2 discuss the following two options:

- Option 1：Define new polling trigger condition: UE triggers polling if the remaining delay budget of the packet is below a threshold;

- Option 2: Define separate configurations for small remaining delay budget data and normal data (smaller value for small remaining delay budget data), UE decides which configuration to use based on data status.

SA2 LS in S2-2410999

For the FEC Awareness at RAN, SA2 asked two further questions regarding the feasibility of the use of content ratio information for discarding of DL PDUs during congestion for both RLC AM and UM mode.

Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791. With respect to RLC UM, RAN indicated in R2-2405781 that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer." After sending the previous LS in S2-2405625, SA2 clarified the principles for potential use of content ratio information for discarding of PDUs during congestion as listed above. Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

For AM mode, in legacy, i.e., before R19 RLC AM enhancement on the unnecessary retransmission avoiding, NG-RAN can determine reliably whether a PDU has been successfully delivered over RLC AM mode. While in R19, since the agreed SDU discarding at the Rx side based on the local timer, it is infeasible for the NG-RAN to know reliably the reception status.

In R19 RLC AM enhancement to avoid unnecessary retransmission, due to the SDU discarding at the Rx side (i.e., UE for DL data), NG-RAN cannot determine reliably whether a PDU has been successfully delivered.

Besides, for the timely retransmission enhancement for RLC AM mode in R19, there is a conclusion that the enhancement is focused on UL data, and the FEC based discarding is for DL-PDUs, there seems no dependencies between timely retransmission enhancement and FEC based discarding.

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

The enhancement for timely retransmission in RLC AM in RAN2 focused on UL traffic.

Therefore, it is suggested that RAN2 reply to SA2 that the use of the content ratio information for discarding DL PDUs during congestion for  RLC AM data bearer is only feasible without R19 RAN-level enhancements of RLC Acknowledged Mode (RLC AM), i.e., it is not applicable when considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791.

For Q1 in S2-2410999, RAN2 reply to SA2 that the use of the content ratio information for discarding DL PDUs during congestion for RLC AM data bearer is not applicable when considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget, as per RP-240791.

For UM mode in Question 2, RAN2 has concluded and replied that it is not possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer. There should be no change to the RAN2 understanding, and RAN2 should stick to the previous conclusion.

For Q2 in S2-2410999, RAN2 stick to the previous reply that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer."

Conclusion

We have the following observations:

Observation 1	The un-sync decision of SDU discarding at Tx and Rx side may cause 1) additional delay especially in PDU set discarding; 2) special NACK SR handling for the discarded SDU at Tx side; 3) additional data loss.

Observation 2	Rx side cannot identify whether the SDU to be abandoned is a new or retransmission or an essential PDCP control PDU.

Observation 3	In terms of signaling overhead, polling only requires the transmitting side to set the "P" field to "1", which does not introduce any additional signaling overhead. On the other hand, autonomous retransmission results in more data transmission, thereby icreasing the signaling overhead.

Observation 4	For delay, autonomous retransmission may cause more delay than polling for multiple PDUs (e.g., in a PDU set) transmission.

Observation 5	In R19 RLC AM enhancement to avoid unnecessary retransmission, due to the SDU discarding at the Rx side (i.e., UE for DL data), NG-RAN cannot determine reliably whether a PDU has been successfully delivered.

Observation 6	The enhancement for timely retransmission in RLC AM in RAN2 focused on UL traffic.

We have the following proposals:

Proposal 1	RAN2 confirm SDU discard at TX side is based on the indication from PDCP as the current specification.

Proposal 2	For the “combined” approach for avoiding unnecessary RLC retransmissions, the indication from Tx to Rx side is needed.

Proposal 3	For the indication from Tx to Rx side, RAN2 selects between the following options:

Option 1: Use control PDU to indicate the SN of the discarded SDU at Tx side.

Option 2: Use data PDU with no Data field, i.e., only header with SN number to indicate the discard information.

Proposal 4	For timely RLC retransmission, RAN2 to adopt polling enhancement.

Proposal 5	For the polling enhancement in RLC AM, RAN2 discuss the following two options:

Option 1：Define new polling trigger condition: UE triggers polling if the remaining delay budget of the packet is below a threshold;

Option 2: Define separate configurations for small remaining delay budget data and normal data (smaller value for small remaining delay budget data), UE decides which configuration to use based on data status.

Proposal 6	For Q1 in S2-2410999, RAN2 reply to SA2 that the use of the content ratio information for discarding DL PDUs during congestion for RLC AM data bearer is not applicable when considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget, as per RP-240791.

Proposal 7	For Q2 in S2-2410999, RAN2 stick to the previous reply that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer."