3GPP TSG-RAN WG2 Meeting #128		R2-2409764

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreements were made for inter-CU LTM [1]:

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

In this paper, we first discuss the remaining open issues for inter-CU LTM in the SA scenario. Then we discuss the scenarios and issues of inter-CU LTM with DC.

- Discussion

  - Remaining issues for Inter-CU LTM in SA

Security key handling for subsequent inter-CU LTM

In the SA3 LS [2], it mentions that “the rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signaling to be used by the UE for key derivation at the next inter-CU LTM cell switch”.

Actually, the solution preferred by SA3 in the LS is not consistent with RAN2’s desire to avoid RRC Reconfiguration during subsequent LTM, as we clarified in the WID [3].  In our view, if RRCReconfiguration is needed after each LTM cell switch, then this does not constitute a subsequent LTM cell switch because the LTM cell switch cannot be performed before receiving the new RRCReconfiguration. There will be no much gain for reserving the LTM candidate cell configurations after an LTM cell switch since the network needs to reconfigure the UE anyway. Therefore, RAN2 should confirm that avoiding RRCReconfiguration is necessary for subsequent LTM during LTM cell switch.

Proposal 1: RAN2 confirms that avoiding RRCReconfiguration is necessary for subsequent LTM during LTM cell switch.

If Proposal 1 is agreed, we believe there are two options that RAN2 could consider regarding the security key handling issue. The first option is to inform SA3 that avoiding RRCReconfiguration is essential for subsequent LTM and request SA3 to further explore solutions that can support subsequent LTM without RRC Reconfiguration. The second option is to decide not to support subsequent inter-CU LTM in Rel-19 and to consider this feature in future releases.

Proposal 2: RAN2 discusses the following two options for the security key handling of subsequent inter-CU LTM:

- Option 1:  Reply LS to inform SA3 that avoiding RRCReconfiguration is necessary for subsequent LTM and request SA3 to further explore solutions that can support subsequent LTM without the need for RRCReconfiguration.

- Option 2: RAN2 confirms that subsequent inter-CU LTM is not supported in Rel-19.

UE-based TA measurement

In the last meeting, whether UE-based TA measurement can be supported for inter-CU LTM was discussed. Some companies believe that RAN4 has already defined the UE requirement for UE-based TA measurement and it applies to both intra-CU and inter-CU scenarios. From our understanding, UE-based TA measurement is supported in synchronized cases in Rel-18 according to the reply LS by RAN4 [4]. Therefore, when the serving cell and candidate cells in Rel-19 from different CUs are synchronized, UE-based TA measurement is supported obviously and the Rel-18 mechanism can be reused.

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 3: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

However, for Rel-19 inter-CU LTM, as the serving cell and the candidate cells may belong to different CUs and are more likely to be unsynchronized, it seems highly beneficial to support UE-based TA measurement for unsynchronized scenarios. The decision on whether UE-based TA measurement for unsynchronized cases can be supported and how to support it should be made by RAN1 and RAN4. Therefore, an LS should be sent to RAN1 and RAN4 to inquire about the feasibility and approach for supporting UE-based TA when the serving cell and the candidate cell are unsynchronized.

Proposal 4: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 5: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

CFRA resource in LTM Cell Switch Command MAC CE

For RACH-based LTM, reserving specific CFRA resources for each UE via pre-configured RRC configuration will result in a huge waste of CFRA resources for the candidate cells. To resolve the issue in Rel-18, CFRA resources are supported to be indicated in the LTM cell switch command, which means the CFRA resource can be allocated when the S-DU determines to trigger LTM and prevent the waste of RACH resources.

To address the potential waste of CFRA resources caused by inter-CU LTM, we propose supporting the inclusion of CFRA resources in the LTM Cell Switch Command MAC CE for inter-CU LTM.

Proposal 6: CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

  - Discussion on inter-CU LTM with DC

2.2.1 Inter-CU SCG LTM

In the last meeting, RAN2 made some agreements about inter-CU SCG LTM:

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

According to the agreements, the detailed signaling procedure is provided below, and the highlighted part includes the issues that need to be further discussed for inter-CU SCG LTM.

Figure 1. Inter-CU SCG LTM signalling procedure

LTM preparation

1.   The source SN initiates the inter-SN LTM procedure by sending the SN Change Required message, which contains an LTM indication. The message also contains candidate node ID(s) and may include an SCG reference configuration (to support delta configuration), and contains the measurement results related to the candidate SN(s), and the number of PSCells that can be prepared by each candidate SN.

2/3  The MN requests each candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure(s), indicating the request is for LTM, and provides the measurement results related to the candidate SN received from the source SN. The message also contains the target candidate cell ID, the LTM configuration ID of the candidate PSCells, the LTM configuration ID mapping list, and the CSI resource configuration. The MN may request PRACH resources from the candidate SN(s). The MN also provides the upper limit for the number of PSCells that can be prepared by each candidate SN and provides a list of KSN and associated sk-Counter values for each candidate SN. The candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN may also propose data forwarding to the MN or other candidate SN(s) for inter-SN LTM. The candidate SN may include an indication that the SCG radio resource configuration of a prepared PSCell is a complete configuration, i.e. that it is not a delta configuration with respect to the SCG reference configuration.

3a.   For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in

the Xn-U Address Indication message.

4. The MN sends a SN Modification Request message to the source SN including the information related to early sync, the LTM configuration IDs, and the CSI resource config for the prepared target candidate cell(s) in other SN(s). The source SN responds with a SN Modification Response message including the updated candidate PSCell configuration(e.g., the updated CSI report configuration) and the common CSI resource configuration.

5.  The MN sends a SN Modification Request message to the candidate SN(s) including the information related to early sync, the LTM configuration IDs, and the common CSI resource configuration. The candidate SN(s) respond with a SN Modification Response message including the updated candidate PSCell configuration(e.g., the updated CSI report configuration).

6.     The MN sends to the UE an RRCReconfiguration message including the LTM candidate configuration, i.e. a list of RRCReconfiguration* messages for the execution of subsequent LTM, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from one of the candidate SN(s) in steps 2 and 3, and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN. The RRCReconfiguration message also includes a security update configuration and may also include a reference configuration.

7.   The UE applies the RRCReconfiguration message received in step 6, stores the LTM configuration, and replies to the MN with an RRCReconfigurationComplete message, which can include an NR RRCReconfigurationComplete*** message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

8/9. If an SN RRC response message is included, the MN informs the source SN with the SN RRCReconfigurationComplete*** message via the SN Change Confirm message.

The MN sends the SN Change Confirm message towards the source SN to indicate that LTM is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding address(es), the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding. In case multiple candidate SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN.

Issue #1: Which node determines the LTM candidate PSCells for inter-CU SCG LTM?

In the LTM preparation procedure above, the S-SN sends the measurement results of candidate PSCell(s) to the MN and the MN forwards these measurement results to the candidate SN(s). Subsequently, the candidate SN(s) decides on its LTM candidate cells. This procedure is more like the legacy SN-initiated inter-SN change, wherein the T-SN determines the target PSCell, rather than inter-SN CPAC, where the S-SN propose candidate PSCells to the T-SN and the T-SN can only choose to accept or reject these candidates. Unlike CPAC, there are no execution conditions related to each LTM candidate cell. Therefore, the source SN does not need to decide on the LTM candidate cells and can just send the measurement results to the T-SN and for the T-SN to make the decision.

Proposal 7: Source SN sends measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN.

Issue #2: Whether MCG part can be included in the inter-CU SCG LTM candidate configuration?

Similar to S-CPAC, although the LTM candidate configuration is intended for inter-CU SCG LTM, the corresponding MCG configuration may also need to be updated when applying the LTM candidate configuration. For instance, the allocation of radio bearers between the MN and SN could change after the inter-CU SCG LTM. If this occurs, the MCG configuration may require reconfiguration as well.

In the last meeting, it was agreed that the LTM reference configurations of inter-CU SCG LTM can include both SCG configuration and possible corresponding MCG configuration.

Observation 2: The LTM reference configurations of inter-CU SCG LTM can include both SCG configuration and possible corresponding MCG configuration

In our view, similar to S-CPAC, the LTM candidate configuration can also include both SCG configuration and possible corresponding MCG configuration.

Proposal 8: The LTM candidate configurations of inter-CU SCG LTM can include both SCG configuration and possible corresponding MCG configuration, similar to S-CPAC.

Issue #3: Whether the combined LTM candidate configuration must contain both MCG and SCG part configurations?

In the last meeting, there was an FFS about whether the network should ensure that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

From our perspective, network implementation can ensure the validity of LTM candidate configurations even if neither the reference nor the candidate configuration for inter-CU SCG LTM includes the MCG part configurations. For instance, the network could ensure that the current MCG configuration remains unchanged during the time when the inter-CU SCG LTM configuration is provided to UE and the time when UE executes inter-CU SCG LTM, or it could reconfigure the inter-CU SCG LTM candidate configurations if there is a change in the current MCG configuration. In any case, the network should ensure that the combined configuration applied by the UE during an inter-CU SCG LTM cell switch is valid, especially when there is an absence of MCG part configuration in both the reference and candidate configuration for inter-CU SCG LTM.

Proposal 9: The network implementation should ensure the validity of the combined configuration applied by UE during the inter-CU SCG LTM cell switch when the MCG part configuration is absent from both the reference and candidate configuration for inter-CU SCG LTM.

Early sync

10. The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command.

11. Early UL synchronization to the target candidate cell(s) may be performed as indicated by the source SN.

12. The candidate SN sends the TA value, the associated CFRA resource information, the candidate cell ID, the source gNB-CU ID, and the source gNB-DU ID in the SN-MN TA INFORMATION TRANSFER message to the MN.

13. The MN forwards the TA value, and the associated CFRA resource information to the source SN in the MN-SN TA INFORMATION TRANSFER message.

For the early sync procedure, the source SN triggers the UE to perform early RACH to the candidate SN by PDCCH order. After the candidate SN acquires the TA value, the candidate SN sends the TA value to the MN, and the MN then forwards the TA value to the source SN.

Proposal 10: After acquiring the TA value via the CFRA triggered by the PDCCH order of the source SN, the candidate SN sends the TA value to the MN, and the MN then forwards the TA value to the source SN.

LTM execution

14. The UE sends the L1 measurement results to the source SN.

15. The source SN sends the Cell Switch command to the UE.

16/17. The source SN sends the SN-MN CELL SWITCH NOTIFICATION message to the MN to indicate the initiation of the Cell Switch command to the UE, for which the message includes the target cell ID and the TCI state ID.

18.  If applicable, the MN triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the target PSCell, to start late data forwarding.

19/20.  If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer message to MN, which the MN sends then to the SN of the target PSCell if needed.

21.  If applicable, data forwarding from the source SN takes place. It may be initiated as early as the the source SN receives the early data forwarding address in step 9.

22.  The UE applies RRCReconfiguration* message corresponding to the target candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the target candidate PSCell, and information enabling the MN to identify the SN of the target candidate PSCell. The RRCReconfigurationComplete* message may also include the sk-Counter value associated with the selected candidate PSCell if a new sk-Counter value is selected.

23.  The MN informs the SN of the selected candidate PSCell (i.e. the selected candidate SN) that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message. If the sk-Counter value is received by the RRCReconfigurationComplete* message, the MN also indicates the received sk-Counter value to the SN.

24.   The UE performs the random access procedure towards the target cell if UE does not have valid TA of the target cell.

For inter-CU SCG LTM, to help the target SN schedule the UE, the cell switch notification should be sent from the source SN to the target SN via MN after LTM cell switch execution is triggered.

Proposal 10: Source SN notifies the MN after LTM cell switch is triggered, and the MN forwards the cell switch notification to the target SN.

L2 handling

As we agreed in the last meeting, UE determines the L2 reset operation based on the Rel-19 IDs and the Rel-18 L2 reset IDs for non-DC case.

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

For DC case (i.e., inter-CU SCG LTM), there are two types of DRB: one involves a change in the termination point and/or a security key update, while the other does not.

In our view, for DRBs with termination point change/security key update, the L2 reset operation in Rel-18 S-CPAC can be reused, which means that the UE will perform PDCP re-establishment, RLC re-establishment, and SCG MAC reset when performing inter-CU SCG LTM.

RAN2#125 Agreements

- P2 For DRB, UE performs L2 reset according to the change of termination point for SCPAC execution.

-	UE performs PDCP re-establishment, RLC re-establishment and MAC reset if there is change of termination point for the bearer;

-	UE performs PDCP data recovery, RLC re-establishment and MAC reset if there is no change of termination point for the bearer.

RAN2#126 Agreements

- MCG MAC is not reset at SCPAC execution and the MN needs to use different LCID values for the MCG RLC bearer serving a radio bearer when the radio bearer is terminated in the MN and in the SN. Remove “reset MCG MAC” in section 5.3.5.13.8 for Subsequent CPAC execution.

Proposal 11: For DRBs with termination point change or security key update, UE follows the same L2 reset behavior for inter-CU SCG LTM as in Rel-18 S-CPAC, i.e., PDCP re-establishment, RLC re-establishment, and SCG MAC reset.

For DRBs without termination point change/security key update (i.e., there is no termination point change for DRBs and the new Rel-19 ID of serving cell and target cell have the same values), the process aligns with the Rel-18 intra-CU SCG LTM and the L2 reset behavior can be reused. For example, when the ltm-ServingCellNoResetID and ltm-NoResetID have different value, UE performs SCG RLC re-establishment and PDCP data recovery for AM DRB if the associated RLC entity is released or re-established.

Proposal 12: For DRBs without termination point change or security key update, UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and determines whether SCG RLC re-establishment and PDCP data recovery(only for AM DRB) should be performed based on the L2 reset mechanism as in Rel-18 LTM.

Regarding SRBs, the L2 reset mechanism during inter-CU SCG LTM can directly follow the Rel-18 S-CPAC mechanism. As an illustration, for SRB3, the UE decides whether to perform PDCP-reestablishment or PDCP SDU discard based on the Rel-19 IDs. For SRB1/SRB2, UE relies on NW indication to perform L2 reset.

RAN2#125 Agreements

- P3 For SRB3, cell set ID (mapping to SN) is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for SCPAC execution.

- P4	For SRB1/SRB2, UE relies on NW indication to perform L2 reset.

Proposal 13: For SRBs, UE follows the same L2 reset operation as defined in Rel-18 CPAC during inter-CU SCG LTM, i.e., Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3, and UE relies on NW indication to perform L2 reset for SRB1/SRB2.

LTM cell switch completion

25.  If the UE has performed an RA procedure in step 7, the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

26.  If applicable, the MN triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

27.  If applicable, a PDU Session path update procedure is triggered by the MN.

Proposal 14: RAN2 takes the procedure illustrated in Figure 1 as baseline for inter-CU SCG LTM signaling procedure.

2.2.2 Inter-CU MCG LTM with SN released or unchanged

In RAN2#127 meeting, the following agreements were made for the case of inter-CU MCG LTM with SN released or unchanged:

RAN2#127 Agreements

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

First, what “SN unchanged” means should be clarified. In our view, the following options can be considered:

- Option 1: The CU of SCG is unchanged (e.g., inter-CU MCG LTM with intra-CU PSCell change is supported)

- Option 2: The SCG is unchanged (e.g., the PSCell and SCell(s) of the current SCG are unchanged)

For option 1, the CU of current SCG remains unchanged when performing inter-CU MCG LTM, which means that intra-SN PSCell change can be performed together with the inter-CU MCG LTM.

For option 2, the SCG itself remains unchanged, meaning that both the serving PSCell and the serving SCell(s) are unchanged during the inter-CU MCG LTM.

From our perspective, whether PSCell should be changed together with the inter-CU MCG LTM can be determined by the S-SN (also the T-SN). It is understandable that SCG change does not introduce additional specification impact as compared to the scenario where SCG remains unchanged. In addition, as the UE moves out of the coverage area of the MN, the current PSCell may also go out of coverage and PSCell change may be needed. Supporting inter-CU MCG LTM with intra-SN PSCell change provides more flexibility for the network.

Proposal 14: For the case of inter-CU MCG LTM with SN unchanged, intra-CU PSCell change is supported when performing inter-CU MCG LTM.

As we agreed, how to handle the SCG part (e.g. release or reconfiguration) during the inter-CU MCG LTM is up to network implementation. However, different network implementations will result in different UE behaviors and specification impacts. The following two kinds of network implementation are possible from our view:

- Option 1: NW implementation ensures that SN is always released or always unchanged when performing inter-CU MCG LTM with DC.

- Option 2: Each candidate MN decides whether to release the SN or keep the SN unchanged independently, and the UE may perform inter-CU MCG LTM with SN released followed by an inter-CU MCG LTM with SCG unchanged.

Option 1 ensures that inter-CU MCG LTM with SN addition will not happen, but it brings additional restrictions for network implementation. For instance, the MN should be responsible for ensuring that all the prepared LTM candidate cells maintain consistent configurations regarding SN—either all having the SN released or all leaving it unchanged.

For option 2, LTM candidate cells might be configured differently; some may trigger an SN release while others may not. In such scenarios, inter-CU MCG LTM with SCG released followed by an inter-CU MCG LTM involving SN reconfiguration can take place. This implies that inter-CU MCG LTM with SN addition is performed. From our standpoint, including SN addition in inter-CU MCG LTM does not introduce additional specification impact and thus should be supported in Rel-19 inter-CU LTM.

Proposal 15: In cases of inter-CU MCG LTM with SN released or unchanged, each candidate MN should independently decide whether to release the SN or keep it unchanged.

Proposal 16: In cases of inter-CU MCG LTM with DC, the inter-CU MCG LTM with SN addition is supported.

Security key handling

During inter-CU MCG LTM with DC, the SN key is updated concurrently with any MN key change. As per TS 33.501[5], the MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. Therefore, when the KNG-RAN is updated due to inter-CU MCG LTM, the sk-Counter utilized by UE for generating the SN key should be set to zero.

TS 33.501

6.10.3.1	SN Counter maintenance

The MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN.

In our view, there are two options for ensuring that sk-Counter = 0 is used when inter-CU MCG LTM with SN unchanged or SN addition is performed:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

Both the options work well and RAN2 can select one for handling the security key in this scenario.

Proposal 17: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

In addition, similar to Rel-17 CHO with SCG, inter-CU MCG LTM with SN unchanged also requires the signaling enhancement between MN and SN to make the SN recognize that the Addition/Modification is related to an LTM. For instance, an indicator should be added in the SN Addition Request and SN Modification Request message. Furthermore, the MN and UE IDs should also be included in the SN Addition Request to help the SN to optimize resource allocation.

Proposal 18: Network signaling enhancements between MN and SN (similar to Rel-17 CHO with SCG) are needed for inter-CU MCG LTM with SN unchanged. Send an LS to inform RAN3.

2.2.3 Coexistence of inter-CU MCG LTM and intra-CU SCG LTM

In the last meeting, the coexistence of inter-CU MCG LTM/intra-CU SCG LTM and inter-CU SCG LTM/intra-CU MCG LTM is supported.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

In scenarios where the SCG is configured with Rel-18 intra-CU LTM and the MCG is configured with Rel-19 inter-CU LTM, different procedures for processing the LTM candidate configuration can be applied, depending on whether the MCG LTM is executed first or the SCG LTM is executed first.

If intra-CU SCG LTM is executed first, the inter-CU MCG LTM candidate configuration can be kept, as the MCG is not expected to be affected by the Rel-18 intra-CU SCG LTM.

Proposal 19: For the coexistence of inter-CU MCG LTM and intra-CU SCG LTM scenario, UE shall keep the inter-CU MCG LTM configuration when intra-CU SCG LTM is performed.

If inter-CU MCG LTM with SN released is performed, the SCG LTM configuration will be released as part of the SN release procedure in Rel-18 specification. When inter-CU MCG LTM with SN unchanged is performed, the decision to keep or release the SCG LTM configurations can be made by the network and indicated in the SCG part of the MCG LTM candidate configuration.

Observation 3: For the coexistence of inter-CU MCG LTM with SN released and intra-CU SCG LTM scenario, the intra-CU SCG LTM configuration will be released as part of the SN release procedure when inter-CU MCG LTM with SN released is performed.

Proposal 20: For the coexistence of inter-CU MCG LTM and intra-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU SCG LTM candidate configurations by explicit RRC signaling when performing the inter-CU MCG LTM with SN unchanged.

2.2.4 Coexistence of intra-CU MCG LTM and inter-CU SCG LTM

In this scenario, MCG is configured with Rel-18 intra-CU LTM while the SCG is configured with Rel-19 inter-CU LTM.

Rel-18 supports intra-CU MCG LTM with SCG released or SCG unchanged. If intra-CU MCG LTM with SCG released is executed first, the inter-CU SCG LTM candidate configurations should be released together with the SCG release.

Proposal 21: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, the inter-CU SCG LTM candidate configurations should be released when the intra-CU MCG LTM with SN release is performed.

If the intra-CU MCG LTM with SCG unchanged is executed first, the current MCG configuration might be changed. Consequently, the inter-CU SCG LTM configuration may become invalid in case no MCG part configuration is included in either the reference configuration or the inter-CU SCG LTM candidate configurations, as discussed in Proposal 9. In such cases, the network is responsible for indicating whether to keep or release the inter-CU SCG LTM candidate configurations by explicit RRC signaling.

Proposal 22: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, network indicates whether to keep or release the inter-CU SCG LTM candidate configurations by explicit RRC signaling when performing the intra-CU MCG LTM with SCG unchanged.

As discussed in section 2.2.1, the inter-CU SCG LTM candidate configuration may also include the MCG part configuration. Therefore, if the inter-CU SCG LTM is performed first, the network can indicate whether the intra-CU MCG LTM candidate configuration should be kept or released within the inter-CU SCG LTM candidate configurations.

Proposal 23: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU MCG LTM candidate configurations by explicit RRC signaling when performing the inter-CU SCG LTM.

- Conclusion

In this paper, we discuss the inter-CU LTM remaining open issues and also the specification impact of inter-CU LTM in DC scenarios. We have the following observations and proposals:

Remaining issues for Inter-CU LTM in SA

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 1: RAN2 confirms that avoiding RRCReconfiguration is necessary for subsequent LTM during LTM cell switch.

Proposal 2: RAN2 discusses the following two options for the security key handling of subsequent inter-CU LTM:

- Option 1:  Reply LS to inform SA3 that avoiding RRCReconfiguration is necessary for subsequent LTM and request SA3 to further explore solutions that can support subsequent LTM without the need for RRCReconfiguration.

- Option 2: RAN2 confirms that subsequent inter-CU LTM is not supported in Rel-19.

Proposal 3: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

Proposal 4: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 5: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

Proposal 6:  CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

Inter-CU SCG LTM

Observation 2: The LTM reference configurations of inter-CU SCG LTM can include both SCG configuration and possible corresponding MCG configuration

Proposal 7: Source SN sends measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN.

Proposal 8: The LTM candidate configurations of inter-CU SCG LTM can include both SCG configuration and possible corresponding MCG configuration, similar to S-CPAC.

Proposal 9: The network implementation should ensure the validity of the combined configuration applied by UE during the inter-CU SCG LTM cell switch when the MCG part configuration is absent from both the reference and candidate configuration for inter-CU SCG LTM.

Proposal 10: Source SN notifies the MN after LTM cell switch is triggered, and the MN forwards the cell switch notification to the target SN.

Proposal 11: For DRBs with termination point change or security key update, UE follows the same L2 reset behavior for inter-CU SCG LTM as in Rel-18 S-CPAC, i.e., PDCP re-establishment, RLC re-establishment, and SCG MAC reset.

Proposal 12: For DRBs without termination point change or security key update, UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and determines whether SCG RLC re-establishment and PDCP data recovery(only for AM DRB) should be performed based on the L2 reset mechanism as in Rel-18 LTM.

Proposal 13: For SRBs, UE follows the same L2 reset operation as defined in Rel-18 CPAC during inter-CU SCG LTM, i.e., Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3, and UE relies on NW indication to perform L2 reset for SRB1/SRB2.

Proposal 14: RAN2 takes the procedure illustrated in Figure 1 as baseline for inter-CU SCG LTM signaling procedure.

Inter-CU MCG LTM with SN released or unchanged

Observation 3: For the coexistence of inter-CU MCG LTM with SN released and intra-CU SCG LTM scenario, the intra-CU SCG LTM configuration will be released as part of the SN release procedure when inter-CU MCG LTM with SN released is performed.

Proposal 15: In cases of inter-CU MCG LTM with SN released or unchanged, each candidate MN should independently decide whether to release the SN or keep it unchanged.

Proposal 16: In cases of inter-CU MCG LTM with DC, the inter-CU MCG LTM with SN addition is supported.

Proposal 17: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

Proposal 18: Network signaling enhancements between MN and SN (similar to Rel-17 CHO with SCG) are needed for inter-CU MCG LTM with SN unchanged. Send an LS to inform RAN3.

Proposal 19: For the coexistence of inter-CU MCG LTM and intra-CU SCG LTM scenario, UE shall keep the inter-CU MCG LTM configuration when intra-CU SCG LTM is performed.

Proposal 20: For the coexistence of inter-CU MCG LTM and intra-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU SCG LTM candidate configurations by explicit RRC signaling when performing the inter-CU MCG LTM with SN unchanged.

Coexistence of intra-CU MCG LTM and inter-CU SCG LTM

Proposal 21: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, the inter-CU SCG LTM candidate configurations should be released when the intra-CU MCG LTM with SN release is performed.

Proposal 22: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, network indicates whether to keep or release the inter-CU SCG LTM candidate configurations by explicit RRC signaling when performing the intra-CU MCG LTM with SCG unchanged.

Proposal 23: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU MCG LTM candidate configurations by explicit RRC signaling when performing the inter-CU SCG LTM.

- References

- RAN2#127bis_ChairNotes.

- S3-244316, Reply LS on security handling for inter-CU LTM in non-DC cases.

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4.

- R4-2314455, Reply LS beam application time and UE based TA measurement for LTM. Ericsson.

- TS 33.501 v19.0.0, Security architecture and procedures for 5G system (Release 19).