3GPP TSG-RAN WG2 Meeting #128                                 R2-2409823

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:		8.3.2.1

Source:				Samsung

Title:					Discussion on the simulation results for RRM measurement prediction

Document for:		Discussion & Decision

1	Introduction

In RAN2#126, RAN2 agreed the following table on the prioritization between different evaluation scenarios/cases for RRM measurement prediction.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we show our initial evaluation results for the cases with ‘High’ priority and discuss some observations based on the results.

2	Discussion

2.1	Evaluation results for RRM measurement prediction

In this section, we would like to show our initial evaluation results for the high priority cases and share some observation based on the results. At this time, we focus on the evaluation for L3 cell-level RSRP prediction and beam-level spatial domain prediction is deprioritized since it is already studied in R18 AI/ML for Air SI. Also, our system level simulator adopts all the agreed mandatory simulation assumption that should be used by all the companies as summarized in Annex part.

2.1.1 General evaluation parameters/assumptions

As a result of the offline e-mail discussion [POST126][031], some simulation parameters/assumptions are agreed to be reported by the companies with their simulation results [1]. The following Table 1 shows the set of parameters commonly used for all the cases in our evaluation. The other parameters specific to each case are reported in the following sections for each case, separately.

Table 1

| Report parameters |  | Samsung |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option | boundary-terminated model |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 Km/h (for temporal domain prediction) 120Km/h (for frequency domain prediction) |
|  | Whether LOSsoft is modeled or not | Yes |
|  | Spatial consistency option (A or B in TR 38.901) | B |
|  | Number of TX beams | 1 (for FR1) / 32 (for FR2) |
|  | Number of RX beams | 1 (for FR1) / 4 (for FR2) |
| Any other parameters | L1/L3-filtering option (sliding or non-sliding) | sliding |
|  | L1-filtering window length | 10 samples |
|  | Target evaluation cells | 7 nearest cells (serving + 1st-tier neighbour cells) |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM autoencoder 2 LSTM layers for encoder 2 LSTM layers + 3 dense layers for decoder |
|  | Model complexity in a number of parameters(M) | 4.5K |
|  | Model complexity in model size (e.g. Mbyte) | ~200 Kbyte |
|  | Computational complexity [FLOPs] | ~20K |

For L1/L3-filtering option, we assume the sliding case where the L1/L3 measurement results are produced every sampling periods (i.e., 40msec for FR1 and 80msec for FR2) as described in Figure 1 in [2]. From our understanding, the measurement period in RAN4 specification (e.g., 200msec) just provides minimum time requirements for RRM measurement and thus it does not mean that L1/L3 measurement results can not be generated every sampling period. In practice, for measurement event triggering, the UE should continuously checks if an entry condition is fulfilled for the measurement results during TTT (e.g., 0ms, 40 ms, 64 ms, …). Thus, we think it is reasonable that the UE can have L1/L3 measurement results with the agreed sampling period rather than the measurement period.

For evaluation of the prediction accuracy, we use the data sets from UE’s current serving cell and the 1st-tier neighbour cells (i.e., not consider the 2nd/3rd-tier neighbour cells for prediction target cells). In our view, there is no need for UE to perform RRM measurement prediction for the 2nd/3rd-tier neighbour cells since they are unlikely to be the serving cell/HO target cells for the UE. Evaluating the prediction accuracy including the cells that can not be the prediction target cells in practice may distort the evaluation results. Therefore, we limit the target evaluation cells to the UE’s current serving and 1st-tier neighbour cells.

Observation 1: In practice, the prediction target cells would be serving/1st-tier neighbor cells for RRM measurement prediction.

Proposal 1: RRM measurement prediction accuracy (i.e., RSRP difference) is evaluated for the UE’s current serving/1st-tier neighbor cells.

2.1.2 FR2 to FR2 intra-frequency temporal domain case A (Case 4)

The following Table 2 shows our simulation assumption and results for the Case 4 (i.e., temporal domain prediction Case A in FR2).

Table 2

| Report parameters |  | Samsung |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | OW | 80/400/800/1600 msec (1/5/10/20 sampling periods) |  |  |  |  |
|  | PW | 400 msec (5 sampling periods) |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 31000/3500 samples* |  |  |  |  |
|  | Testing | 9000 samples* |  |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | ~200 Kbyte |  |  |  |  |
|  | Computational complexity [FLOPs] | 20K |  |  |  |  |
| AI/ML model input/output | Model input | Historical L3 cell-level RSRP within OW |  |  |  |  |
|  | Model output | Future L3 cell-level RSRP within PW |  |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | OW | Average RSRP diff. (dB) |  |  |  |
|  |  |  | 30km/h | 60km/h | 90km/h | 120km/h |
|  |  | 80 msec | 1.432 | - | - | - |
|  |  | 400 msec | 0.923 | - | - | - |
|  |  | 800 msec | 0.912 | - | - | - |
|  |  | 1600 msec | 0.874 | 1.292 | 1.349 | 1.551 |

*Note: Each sample consists of input RSRP values and output RSRP values.

For Case A prediction, the model is trained to predict the future L3 cell-level RSRP within the prediction window based on the historical L3 cell-lever RSRP within the observation window. Here, we fix the prediction window as the agreed initial value (i.e., 400 msec for FR2) and evaluate the prediction accuracy with different observation window size. The results for the UE speed 30 km/h show that the average L3 RSRP difference can be 1.43/0.923/0.912/0.874 dB for distinct observation window lengths of 1/5/10/20 samples, respectively.

Observation 2: For Case A temporal domain prediction in FR2, the evaluation results show that the average L3 RSRP difference can be 1.43/0.923/0.912/0.874 dB for distinct observation window lengths of 1/5/10/20 samples, respectively.

From the evaluation results, it is observed that the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved) as we use the longer observation window size. This makes sense because a longer observation window provides more information for the AI/ML model, which can be used to improve the prediction accuracy. However, Figure 1 below shows that the accuracy improvement becomes marginal when the observation window length is over 5 samples.

Figure 1. L3 RSRP difference with different observation window sizes

Observation 3: As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved). However, the accuracy improvement becomes marginal when the observation window length is over 5 samples.

On the other hand, the longer observation window can increase the signalling/memory overhead in the model training/inference procedure. For example, if we assume the use of gNB-side model for RRM measurement prediction, the UE is required to report the input data (i.e., historical L3 RSRP measurement within the observation window) to the gNB for inference. Therefore, in this case, the use of longer observation window will increase the signalling overhead between UE and gNB and also require larger memory size at the UE side. The same goes for the UE-side model as well since the UE needs to provide the label data (i.e., set of input and output data) to the training entity (e.g., UE-side OTT server).

Observation 4: For RRM measurement prediction, the use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

Based on the observations above, there exists a trade-off between prediction accuracy and signalling/memory overhead depending on the length of observation window. Therefore, we cannot continuously extend the observation window to improve the prediction accuracy but instead should determine a suitable value for it. In our evaluation,’20 samples’ is chosen as a value for the observation window length, and this value is commonly used in the other cases (i.e., Case 2/3) as well.

Observation 5: There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window. A reasonable value for the length of observation window should be determined considering the trade-off relation.

Proposal 2: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of observation window length on prediction accuracy and signaling/memory overhead.

- As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved).

- The use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

- There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window.

- A reasonable value for the length of observation window should be determined considering the trade-off relation.

For cell level RRM measurement prediction, RAN2 agreed to study the following sub-cases at RAN2#125bis.

- Sub-case 1: To predict beam level results, then generate cell level results based on the predicted beam results.
(Input: L1 beam-level RSRP, Output: L1 beam-level RSRP  L3 cell-level RSRP after consolidation)

- Sub-case 2: To directly predict cell level results based on cell level results.
(Input: L3 cell-level RSRP, Output: L3 cell-level RSRP)

- Sub-case 3: To directly predict cell level results based on beam level results
(Input: L1 beam-level RSRP, Output: L3 cell-level RSRP)

To check the performance of the sub-cases 1/2/3, we evaluate the accuracy of RRM measurement prediction (i.e., L3 RSRP difference) for each of the sub-cases with the same parameters in Table 1&2, except the model input/output. The following Table 3 shows the evaluation results.

Table 3

|  | Sub1 (L1 beam L1 beam) | Sub2 (L3 cell L3 cell) | Sub3 (L1 beam L3 cell) |
| --- | --- | --- | --- |
| L3 RSRP difference | 2.019 dB | 0.874 dB | 1.915 dB |

Observation 6: For Case A temporal domain prediction in FR2, the evaluation results show that the average L3 cell level RSRP difference can be 2.091/0.874/1.915 dB for each of the sub-case 1/2/3, respectively.

In our view, the important point here is whether the use of L1 beam-level RSRP input really helps improve the prediction accuracy and the answer from the result is ‘No’. We can see that the subcase 2 with L3 cell-level input shows much better performance compared to the subcase 1&3 with L1 beam-level input in prediction accuracy. On the other hand, it is obvious that the use of L1 beam-level RSRP will result in an increase in signalling/memory overhead for model training/inference. The input data size will increase by up to 64 times when we assume the use of L1 beam-level RSRP. From our understanding, if we provide L1 beam-level RSRP as an input data for AI/ML model for the subcase 1&3, the model should become much more complex to learn the meaning of input data in the extended dimension. Moreover, for the sub case 3, the model should additionally learn the logic of the beam consolidation mechanism, which does not need to be part of AI/ML model inference. It can be performed by the legacy rule-based algorithm without the unnecessary overhead. In summary, the use of L1 beam-level input can increase the signalling/memory overhead without much gain in RRM measurement prediction.

Observation 7: Sub-case 2 with L3 cell-level input shows much better prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

Observation 8: The use of L1 beam-level input can increase signalling/memory overhead without gain in RRM measurement prediction.

Based on the observation above, we would like to propose the following.

Proposal 3: For RRM measurement prediction, RAN2 to capture the following observation on the use of L1 beam-level input in TR 38.744.

- Sub-case 2 with L3 cell-level input shows much better prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

- The use of L1 beam-level input can increase signalling/memory overhead without gain in RRM measurement prediction.

2.1.3 FR1 to FR1 intra-frequency temporal domain case B (Case 2)

The following Table 4 shows our simulation assumption for the Case 2 (i.e., temporal domain prediction Case B in FR1).

Table 4

| Report parameters |  | Samsung |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | Measurement reduction rate (R) | 50% | 66% |  | 80% |  |
|  | OW | 800 ms | 800 ms |  | 800 ms |  |
|  | PW | 40 ms | 80 ms |  | 160 ms |  |
|  | Filtering options (1/2/3) in [4] | Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement) |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 59000/6500 samples* |  |  |  |  |
|  | Testing | 9500 samples* |  |  |  |  |
| AI/ML model input/output | Model input | Historical L3 cell level RSRP in OW with the interval (= sampling period * 1/(1-R) ) |  |  |  |  |
|  | Model output | Future L3 cell level RSRP in PW with the interval (= sampling period) |  |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | Reduction rate | UE speed |  |  |  |
|  |  |  | 30km/h | 60km/h | 90km/h | 120km/h |
|  |  | 50% | 0.546 | 0.735 | 1.006 | 1.215 |
|  |  | 66% | 0.905 | 1.239 | 1.657 | 1.842 |
|  |  | 80% | 1.538 | 2.094 | 2.626 | 2.934 |

*Note: Each sample consists of input RSRP values and output RSRP values.

For Case B prediction, we assume the example 2 described in Figure 2.1-2 in the summary of [POST127][030] [3]. The model is trained to predict the future L3 cell-level RSRP with the interval of ‘sampling period (i.e., 40msec)’ in PW based on the historical L3 cell-level RSRP with the interval of ‘sampling period *1/(1-R)’ in OW. For example, let’s assume the case with measurement reduction rate (= 2/3) as in Figure 2 below. In this scenario, at the specific time instance ‘Tn’, the model inference can be performed to predict the future RSRP values for 2 distinct time instances between the current (Tn) and the subsequent (Tn+1) measurement time instance. With the assumption, the number of predicted time instances can be 1, 2, 4 for the distinct measurement reduction rates 1/2(=50%), 2/3(=66%), 4/5(=80%), respectively.

Figure 2. Example of Case B prediction with the measurement reduction rate 2/3(=66%)

During simulation the input L3 filtered RSRP result is achieved based on L1 beam results of dataset. In Case B where some measurements are skipped, it is unclear how to perform L3 filtering (at T3 in Figure 3) in the case that previous L3 filtered result is from prediction (at T2 in Figure 3). The following three options are provided in [4].

Figure 3. Temporal domain Case B example 2

- Filtering option 1: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T2;

- Filtering option 2: L3 cell result at T3 is filtered based on the L1 filtered result at T3; in other words, not consider old L3 filtered result.

- Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement);

For option 1, the output from the previous prediction would be used as part of input for the following prediction. I.e., the predicted value at T2 is used to calculate the L3 filtered value at T3 which is provided as input data for the prediction at T4. In this case, the prediction error would be propagated/accumulated over time. Also, this can increase our simulation time too much because all the prediction should be performed in order, which doesn’t allows parallel computing. The option 2 means no L3 filtering in practice. The option 3 increases the interval between two samples (L3 filtered value at T1 and T3) for L3 filtering, which can make the actual channel variation applied to L3 filtered RSRP more slowly compared to the option 1.

In our view, the option 1 is not aligned with the legacy L3 filtering process since it is not only based on actual measurement but also based on predicted results, which also makes the prediction error accumulated over time. The option 2 does not make sense since it actually means no L3 filtering. Thus, in our simulation, we adopt the option 3 as our L3 filtering option.

Table 4 shows the evaluation results that the average L3 cell level RSRP difference for the UE speed 30km/h can be 0.546, 0.905, 1.538 dB for each of the different measurement reduction rates 1/2(=50%), 2/3(=66%), 4/5(=80%), respectively. From the results, we can also find the tradeoff between measurement reduction rate and prediction accuracy. As the higher measurement reduction rate is used for Case B prediction, the measurement overhead at the UE side decreases, but the RSRP difference increases, which can result in HO KPI degradation. From UE’s perspective, it can prefer a higher measurement reduction rate to reduce its measurement overhead, but NW may be conservative about reducing the measurement overhead too much to maintain overall HO performance. Thus, we need to study how UE/gNB can determine or negotiate a suitable value of measurement reduction rate for Case B prediction.

Observation 9: For Case B temporal domain prediction in FR1, the evaluation results show that the average L3 cell level RSRP difference can be 0.546, 0.905, 1.538 dB for each of the measurement reduction rate 1/2(=50%), 2/3(=66%), 4/5(=80%), respectively.

Observation 10: For Case B prediction, there is a tradeoff between measurement reduction rate and prediction accuracy (i.e., L3 RSRP difference).

Based on the observations above, we would like to propose the following.

Proposal 4: For Case B temporal domain prediction, RAN2 to capture the tradeoff between measurement reduction rate and prediction accuracy in TR 38.744.

Proposal 5: RAN2 to study how UE/gNB can determine a suitable value of measurement reduction rate for Case B prediction.

2.1.4 FR1 to FR1 inter-frequency (frequency domain, Case 3)

The following Table 5 shows our simulation assumption/results for the Case 3 (i.e., inter-frequency prediction in FR1).

Table 5

| Report parameters |  | Samsung |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | Inter-frequency correlation assumption in general (yes or no) | Yes |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) | Full |  |  |
| Data Size (Number of Samples) | Training/validity | 59000/6500 samples* |  |  |
|  | Testing | 9500 samples* |  |  |
| AI/ML model input/output | Model input | L3 cell level RSRP at the baseline freq. (4GHz) |  |  |
|  | Model output | L3 cell level RSRP at another freq. (2GHz) |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | UE speed |  |  |
|  |  | 30km/h | 60km/h | 120km/h |
|  |  | 4.144 | 3.907 | 3.61 |
|  | Channel correlation coefficient between two frequency layers | 0.899 | 0.902 | 0.918 |

*Note: Each sample consists of input RSRP values and output RSRP values.

For inter-freq. prediction, the model is trained to predict L3 cell-level RSRP of the cell in the inter-frequency based on L3 cell-level RSRP of the cell in the serving frequency. Specifically, we use 20 samples of the measured L3 RSRP in 4GHz to predict the 20 samples of the L3 RSRP at the same time instances in 2GHz. For the inter-freq. correlation, we assume the case where the cell to be measured and the cell to be predicted are co-located (i.e., located in the same sector/site with the same antenna configuration) and adopt the inter-frequency shadow fading model in TR 38.901.

Figure 4. L3 RSRP of the two cells in different frequency

Figure 4 depicts the true L3 RSRP in 4GHz, the true L3 RSRP in 2GHz, and the predicted L3 RSRP in 2GHz. In general, the results show quite a high correlation between the L3 RSRP of the two cells in different frequency. The correlation coefficient between the L3 RSRP of the two cells is observed as 0.899, 0.902, 0.918 for each of UE speed 30, 60, 120 km/h, respectively.

Observation 11: For the inter-frequency prediction, the evaluation results show a high inter-frequency correlation between the L3 cell-level RSRP of the two co-located cells. The correlation coefficient is observed as 0.899, 0.902, 0.918 for each of UE speed 30, 60, 120km/h, respectively.

In our simulation, sequence of RSRP values for the same cell are correlated in time domain based on the spatial consistency modelling. Meanwhile, RSRP values for the two co-located cells are correlated in frequency domain based on the inter-frequency correlation modelling. Therefore, RSRP value at a certain time instance for a cell are affected by both spatial and frequency domain correlation modelling. From our understanding, the higher UE speed results in lower correlation between RSRP values in time domain, which relatively increase the effect of frequency domain correlation. Therefore, we observe higher correlation coefficient between L3 RSRP of the two co-located cells with higher UE speed in our simulation. However, we understand companies can have different observation on the relation between UE speed and the correlation coefficient depending on their assumption on spatial consistency/inter-frequency channel modelling.

With higher correlation coefficient, we can expect higher prediction accuracy (i.e., low RSRP difference) for the inter-frequency prediction. In our simulation, the average L3 RSRP difference is observed as 4.144, 3.907, 3.61 dB with the correlation coefficient 0.899, 0.902, 0.918, respectively. The results show that the prediction accuracy increases as the correlation coefficient increases.

Observation 12: For the inter-frequency prediction, the average L3 RSRP difference is observed as 4.144, 3.907, 3.61 dB with the correlation coefficient of 0.899, 0.902, 0.918, respectively.

Observation 13: For the inter-frequency prediction, the evaluation results show that higher correlation coefficient between two frequency layers results in higher prediction accuracy.

Proposal 6: For the inter-frequency prediction, RAN2 to capture that higher correlation coefficient between two frequency layers results in higher prediction accuracy in TR 38.744.

With the high correlation between L3 RSRP of the two cells, we can expect a high prediction accuracy (i.e., low RSRP difference) for the inter-frequency prediction. However, the actual RSRP difference for inter-frequency prediction is evaluated as 3.61, 3.907, 4.144 dB that is a bit larger compared to the values in other cases (i.e., the temporal domain prediction Case A&B).

In the inter-frequency prediction, unlike other cases, the L3 RSRP of the cell to be predicted is not provided to the model as input data. Additionally, according to the inter-frequency channel model in TR 38.901, there is no assumed correlation in terms of fast fading between the two different frequencies. Therefore, the observed RSRP difference seems reasonable to us. In our view, for the inter-frequency prediction, the model can be trained to learn the average offset between the L3 RSRP of the two distinct cells, which represents the best outcome we can expect.

Observation 14: Despite the high correlation between the L3 RSRP of the two cells in different freq., the L3 RSRP difference in inter-frequency prediction is a bit larger compared to the values in other cases (i.e., temporal domain prediction Case A&B).

2.2	Impact of L1 filtering window length on prediction accuracy

In this section, we would like to discuss the impact of L1 filtering window length on RRM measurement prediction. For evaluation, RAN2 agreed to use fast fading channel model for RRM measurement prediction. With the fast fading channel model, the L1 raw RSRP of a certain beam/cell can randomly fluctuate even though it has time domain correlation according to the spatial consistency option A/B in TR 38.901. In our simulation, we adopt the option B and observe that L1 raw RSRP randomly fluctuates with an amplitude of about 6 dB.

Observation 15: With the agreed channel model based on TR 38.901, L1 raw RSRP of a certain beam/cell randomly fluctuates with an amplitude of about 6 dB.

In our view, the random fluctuation in L1 raw RSRP is reasonable considering the effect of fast fading and noise. However, in practice, it is impossible for the AI/ML model to predict that kind of randomness in L1 raw RSRP and thus we need to use the L1/L3-filtered value for RRM measurement prediction as in the legacy RRM operation.

For L3 filtering, RAN2 already agreed the value of L3 filtering parameter (i.e., FilterCoefficient = 4) for evaluation. However, for L1 filtering, it is up to companies what L1 filtering method to use for simulation. At the last meeting, RAN2 discussed two different L1 filtering options (i.e., sliding vs. non-sliding in [xx]) and agreed to keep both on the table. In our evaluation, we adopt the option with sliding and assume the simplest filtering method of averaging the latest L1 raw RSRP values within the L1 filtering window.

To figure out the proper length of L1 filtering window, we evaluate the prediction accuracy of Case A prediction in FR2 with different length of L1 filtering window. Figure 4 below depicts the average L3 cell level RSRP difference according to prediction step with the different L1 filtering window length (i.e., 10, 20, 40, 80). Here, prediction step 1/2/3/4/5 means the prediction on the upcoming 1st/2nd/3rd/4th/5th simple periods from the inference time.

Figure 5. L3 RSRP difference with different L1 filtering window length

Figure 5 shows a trend the RSRP difference increases as the prediction step count increases. I.e., the accuracy of predication decreases when the RSRP prediction is for further future. The results indicate that the RSRP difference can be 0.846/0.551/0.383/0.283 dB for varying L1 filtering window lengths of 10/20/40/80 samples, respectively. Based on the results, it is observed that using a longer L1 filtering window can bring significant improvements in prediction accuracy.

Observation 16: The prediction accuracy can be improved as we use a longer L1 filtering window length. The L1 filtering window length has impact on the RSRP difference.

Figure 6 below explains how the L1 filtering window length affects the RSRP difference of RRM measurement prediction. In the sub-figure (a), we can see that L3 RSRP randomly fluctuates a lot when there is no L1 filtering (i.e., L1 filtering window length = 1), which demonstrates the ineffectiveness of L3 filtering and highlights the necessity of L1 filtering. From the sub-figure (b)~(d), it is observed that the volatility caused by fast fading/noise in L3 RSRP decreases as the L1 filtering window length increases from 10 to 40. From our understanding, since it is impossible for AI/ML models to predict the random variation in L3 RSRP, the accuracy of prediction can be improved as the randomness decreases by using longer L1 filtering length.

Observation 17: It is impossible for AI/ML models to predict the random variation in L3 RSRP caused by fast fading/noise.

Observation 18: The prediction accuracy can be improved as the randomness in L3 RSRP decreases by using longer L1 filtering window length.

Figure 6. L3 cell-level RSRP with varying L1 filtering window length

At first glance, one might think that using a longer L1 filtering length is always better to reduce the RSRP difference. However, if the L1 filtering length becomes too long, it will cause a delay in reflecting the real time channel variation to the L3 RSRP, which can lead to degradation of HO KPI. Therefore, we need to set a reasonable value for the L1 filtering window length to ensure objective evaluation of RRM measurement prediction.

Observation 19: There can be a trade-off between prediction accuracy and HO performance depending on L1 filtering window length.

Based on the observations above, we would like to propose the following.

Proposal 7: For RRM measurement prediction, RAN2 to discuss the impact of L1 filtering window length on the prediction accuracy (i.e., RSRP difference) and agree to let company report L1 filtering window length used for their evaluation.

2.3	Cluster-based approach

For the RRM measurement result prediction, we can consider two different approaches (i.e., cell-specific approach and cluster approach). The cell-specific approach refers to “use measurement results of one cell to predict the measurement of one cell” while the cluster approach refers to “use measurement results of multiple cells to predict the measurement of one or more cells”. In our view, for RRM measurement prediction, the cluster approach can have some benefits compared to the cell-specific approach in the following aspects:

- Location (spatial) information

  - In the cluster approach, the measurement results for multiple cells are used as input of the model, which can implicitly provide the information on UE’s location (like footprint) to the prediction model. The location information can be useful for RRM measurement prediction due to the correlation between the channel condition and UE’s location.

- Diversity gain from multiple cell measurement

  - In the cell-specific approach, the measurement results for one cell may not be enough for stable prediction. For example, when the measurement results for an input cell suddenly changes due to a random blocking obstacle, it can lead to sudden change in the prediction results for an output cell even though the event does not actually affect the channel condition for the output cell. With the cluster approach, we can avoid that kind of unstable prediction by having diversity gain from multiple cell measurement.

- # of models needed for prediction

  - For the cell-specific approach, AI/ML model can be cell-specific and a separate model is required for all potential input/output cell combinations. In this case, UE should store/manage multiple models for RRM measurement prediction and may need to download a new set of models upon HO. On the other hand, for the cluster approach, one area-specific model can be used for all potential input/output cell combinations and the UE can continuously use that model within a certain area.

To check the benefit of the cluster approach over the cell-specific, we evaluate the prediction accuracy of the two approaches for both temporal domain prediction Case A in FR2 and frequency domain prediction in FR1. For the cell-specific approach, we train 21 separate models, where each model is trained by the measurement results for the corresponding cell. On the other hand, for the cluster approach, only one area-specific model is trained by the measurement results of all 21 cells in the simulation area. Other simulation assumptions are the same as in Table 2 and Table 5 for temporal domain prediction Case A and frequency domain prediction, separately. The following table shows the evaluation results.

Table 6

| Use case | Approach | AI input/output | Avg. L3 cell-level RSRP diff. (dB) | Model size |
| --- | --- | --- | --- | --- |
| Temporal domain prediction (Case A, FR2, 30km/h) | Cell-specific | Historical/future L3 cell-level RSRP for a cell | 0.863 | 200Kbyte*21 = ¦4.2 Mbyte |
|  | Cluster | Historical/future L3 cell-level RSRP for 21 cells | 2.145 | 3.5 Mbyte |
| Frequency domain prediction (FR1, 30km/h) | Cell-specific | L3 cell-level RSRP for a cell at the frequency 4GHz/2GHz | 3.611 | 200Kbyte*21 = ¦4.2 Mbyte |
|  | Cluster | L3 cell-level RSRP for 21 cells at the frequency 4GHz/2GHz | 2.657 | 3.5 Mbyte |

The evaluation results show that the cluster approach outperforms the cell-specific approach not in temporal domain prediction but in frequency domain prediction. From our understanding, in temporal domain prediction, the measurement results of the target cell has the most directly correlated information. Therefore, the measurement results from other cells may not be that useful and can be regarded as a noise if the model is too simple to handle the measurement results from multiple cells or the amount of training data set is not enough to train the complex model. On the other hand, in frequency domain prediction, it is inter-cell prediction and thus measurement results for the target cell are not given as input data. In this case, the measurement results from the multiple cells can be relatively more useful than in the time domain prediction.

Observation 20: The cluster approach outperforms the cell-specific approach in frequency domain prediction.

Observation 21: In temporal domain prediction (intra-cell prediction), the measurement results of the target cell has the most directly correlated information and thus the cluster approach using measurement results from other cells may not be that useful.

Observation 22: In frequency domain prediction (inter-cell prediction), there is no measurement results for the target cell in input data and thus the cluster approach using measurement results from other cells can be relatively more useful.

Based on the observation above, we would like to propose the following.

Proposal 8: RAN2 to support the study on benefit of using the cluster approach at least for inter-cell measurement prediction scenarios (e.g., frequency domain prediction).

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

General evaluation parameters/assumptions:

Observation 1: In practice, the prediction target cells would be serving/1st-tier neighbour cells for RRM measurement prediction.

Proposal 1: RRM measurement prediction accuracy (i.e., RSRP difference) is evaluated for the UE’s current serving/1st-tier neighbor cells.

FR2 to FR2 intra-frequency temporal domain case A (Case 4):

Observation 2: For Case A temporal domain prediction in FR2, the evaluation results show that the average L3 RSRP difference can be 1.43/0.923/0.912/0.874 dB for distinct observation window lengths of 1/5/10/20 samples, respectively.

Observation 3: As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved). However, the accuracy improvement becomes marginal when the observation window length is over 5 samples.

Observation 4: For RRM measurement prediction, the use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

Observation 5: There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window. A reasonable value for the length of observation window should be determined considering the trade-off relation.

Proposal 2: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of observation window length on prediction accuracy and signaling/memory overhead.

- As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved).

- The use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

- There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window.

- A reasonable value for the length of observation window should be determined considering the trade-off relation.

FR2 to FR2 intra-frequency temporal domain case B (Case 2):

Observation 9: For Case B temporal domain prediction in FR1, the evaluation results show that the average L3 cell level RSRP difference can be 0.546, 0.905, 1.538 dB for each of the measurement reduction rate 1/2(=50%), 2/3(=66%), 4/5(=80%), respectively.

Observation 10: For Case B prediction, there is a tradeoff between measurement reduction rate and prediction accuracy (i.e., L3 RSRP difference).

Proposal 4: For Case B temporal domain prediction, RAN2 to capture the tradeoff between measurement reduction rate and prediction accuracy in TR 38.744.

Proposal 5: RAN2 to study how UE/gNB can determine a suitable value of measurement reduction rate for Case B prediction.

FR1 to FR1 inter-frequency (frequency domain, Case 3):

Observation 11: For the inter-frequency prediction, the evaluation results show a high inter-frequency correlation between the L3 cell-level RSRP of the two co-located cells. The correlation coefficient is observed as 0.899, 0.902, 0.918 for each of UE speed 30, 60, 120km/h, respectively.

Observation 12: For the inter-frequency prediction, the average L3 RSRP difference is observed as 4.144, 3.907, 3.61 dB with the correlation coefficient of 0.899, 0.902, 0.918, respectively.

Observation 13: For the inter-frequency prediction, the evaluation results show that higher correlation coefficient between two frequency layers results in higher prediction accuracy.

Proposal 6: For the inter-frequency prediction, RAN2 to capture that higher correlation coefficient between two frequency layers results in higher prediction accuracy in TR 38.744.

Observation 14: Despite the high correlation between the L3 RSRP of the two cells in different freq., the L3 RSRP difference in inter-frequency prediction is a bit larger compared to the values in other cases (i.e., temporal domain prediction Case A&B).

Impact of L1 filtering window length on prediction accuracy:

Observation 15: With the agreed channel model based on TR 38.901, L1 raw RSRP of a certain beam/cell randomly fluctuates with an amplitude of about 6 dB.

Observation 16: The prediction accuracy can be improved as we use a longer L1 filtering window length. The L1 filtering window length has impact on the RSRP difference.

Observation 17: It is impossible for AI/ML models to predict the random variation in L3 RSRP caused by fast fading/noise.

Observation 18: The prediction accuracy can be improved as the randomness in L3 RSRP decreases by using longer L1 filtering window length.

Observation 19: There can be a trade-off between prediction accuracy and HO performance depending on L1 filtering window length.

Proposal 7: For RRM measurement prediction, RAN2 to discuss the impact of L1 filtering window length on the prediction accuracy (i.e., RSRP difference) and agree to let company report L1 filtering window length used for their evaluation.

Cluster-based approach :

Observation 20: The cluster approach outperforms the cell-specific approach in frequency domain prediction.

Observation 21: In temporal domain prediction (intra-cell prediction), the measurement results of the target cell has the most directly correlated information and thus the cluster approach using measurement results from other cells may not be that useful.

Observation 22: In frequency domain prediction (inter-cell prediction), there is no measurement results for the target cell in input data and thus the cluster approach using measurement results from other cells can be relatively more useful.

Proposal 8: RAN2 to support the study on benefit of using the cluster approach at least for inter-cell measurement prediction scenarios (e.g., frequency domain prediction).

4	Reference

- R2-2406095, “Summary of [POST126][031][AIMob] Simulations (OPPO)”, OPPO

- R2-2407849, “Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO)”, OPPO

- R2-2407885, “Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions-phase 2(OPPO)”, OPPO

- R2-2409207	Evaluation on RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Annex: Simulation assumptions

Table 5.1-1 Simulation assumptions of FR1 and FR2

| Parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; Inter-frequency correlation model is optional. without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Other assumptions are not precluded. |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Other assumptions are not precluded |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | 3 options in 38.843 section 6.3.1 | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model |
| Sampling period | 40ms | 80ms |