3GPP TSG-RAN WG2 Meeting #128                                 R2-2409824

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda item:		8.1.2.3

Source:				Samsung

Title:					Discussion on LCM for POS use case

Document for:		Discussion & Decision

1	Introduction

This contribution provides our view on LCM for POS use case according to the following objective in WID [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

2	Discussion

According to WID [1], there are five positioning sub use cases, which can be summarized as below.

| Case | Positioning type | Model entity | AI/ML for Pos type |
| --- | --- | --- | --- |
| 1 (1st priority) | UE-based positioning | UE-side model | Direct |
| 2a (2nd priority) | UE-assisted/LMF-based positioning | UE-side model | assisted |
| 2b (2nd priority) | UE-assisted/LMF-based positioning | LMF-side model | Direct |
| 3a (1st priority) | NG-RAN node assisted positioning | gNB-side model | assisted |
| 3b (1st priority) | NG-RAN node assisted positioning | LMF-side model | Direct |

Until now, the following agreements were made for the use case 3a/b with gNB/LMF-side model.

For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3. RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

According to the agreement above, we do not see any potential RAN2 spec. impact for the case 3a/b (with gNB/LMF-side model). Thus, from RAN2 perspective, the case 3a/b can be deprioritized compared to the case 1 (with UE-side model). Also, since the case 2a/b already have 2nd priority according to WID [1], RAN2 can focus on the use case 1 for now and further discuss other cases later.

Proposal. 1: For POS use cases, RAN2 can prioritize the discussion on the case 1 over other cases.

Based on the observation above, we discuss the LCM aspect mainly considering the case 1 with UE-side model in the following part.

2.1 Baseline LCM procedures for Case 1

In RAN2#127bis, the following baseline procedure is agreed for LCM for Case 1.

1: 	Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

2: 	Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

3: 	Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

4: 	FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

5:   RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

However, there are some remaining issues marked as FFS and we would like to further discuss them one by one.

Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress):

First, we would like to clarify what inference configuration is. From our understanding, the configuration for location inference in POS Case1 can consist of two parts. The first part is set of parameters related to inference that can be used for applicability check (e.g., DL-PRS configuration). The second part is configuration to configure UE to perform inference (e.g., request for AI-based location inference). Considering the discussion on BM use case in RAN1, we would like to clarify that only the second part above can be regarded as inference configuration since UE can perform the AI-based location inference after receiving it.

Proposal. 2: For POS Case 1, configuration for AI-based location inference consists of the following two parts:
- Set of inference related parameters that can be used for applicability check (e.g., DL-PRS configuration). 
- Inference configuration to configure UE to perform inference (e.g., request for AI-based location inference).

In legacy positioning, for the POS methods based on downlink measurement (e.g., DL-TDOA, DL-AoD), LMF provides UE with which DL-PRS to measure via LPP ProvideAssistanceData message and it would be the same for AI-based POS Case 1. For POS Case 1, the assistance data related to DL PRS reception is necessary for UE to generate the channel measurements for model input, and thus the LMF should provide the required assistance data (i.e., DL-PRS configuration) for UE in step 3 as in legacy. Therefore, the DL-PRS configuration can be regarded as set of inference related parameters, which is used for applicability check before step 4. Meanwhile, in the current LPP, LPP RequestLocationInformation message is used for LMF to request UE to perform the required measurement and to report the results (e.g., estimated location). Considering the intended usage of each LPP message, it is also obvious that LMF requests UE to perform AI-based location inference via LPP RequestLocationInformation message in step 5, which can be regarded as inference configuration in POS Case 1.

Although, we note from the agreement from RAN2#127bis:

"Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress)"

We would like to propose based on the discussion above:

Proposal. 3: For POS Case 1, RAN2 confirm that set of inference related parameters (e.g., DL-PRS configuration) is provided in step 3 and inference configuration (e.g., request for AI-based location inference) is provided in step 5.

Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress):

According to the latest FL summary on POS case in RAN1 [3], RAN1 is still discussing what/whether NW-side additional condition is needed. RAN2 can just wait for RAN1’s conclusion on this.

Whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

First, we would like to remind that UE is always allowed to provide an unsolicited capability without any configuration from LMF in legacy LPP (refer to 5.1.2 in TS 37.355). Therefore, we do not see any need/possibility of LMF’s control to allow UE’s unsolicited capability report. I.e., UE can send ProvideCapabilities message to LMF whenever it is needed.

For POS Case 1, the applicability of AI-based location estimate at UE can change dynamically due to either UE’s internal condition (e.g., model availability) or NW configuration from LMF (e.g., DL PRS configuration). When the applicability change due to UE’s internal condition, UE can update the applicability by sending an unsolicited LPP ProvideCapabilities message to LMF, which can be understood as “proactive” report. Meanwhile, if the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. In this case, the applicability report from UE can be regarded as “reactive” report.

Based on the discussion above, we would like to propose:

Proposal. 4: For POS Case 1, RAN2 confirm that there is no LMF’s control to allow UE to send unsolicited LPP ProvideCapabilities message for applicability report. Once an LPP session has been established, UE can send LPP ProvideCapabilities message to LMF whenever it is needed as in legacy.

Proposal. 5: For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” manner without further enhancement.

- Proactive case: When the applicability change due to UE’s internal condition, UE can send an unsolicited LPP ProvideCapabilities message to LMF.
- Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF upon reception of ProvidAssistanceData message.

2.2 Model Determination/Selection

For UE-side model case, the UE can have multiple AI/ML models that are applicable for the same functionality. In that case, one of the models should be selected to use when the AI/ML functionality is configured/applied. For the model selection, we can consider the following options:

- Option 1: The UE decides which model to use among the applicable models.

- Option 2: The UE decides which model to use among the applicable models and indicates it to the LMF.

- Option 3: The LMF decides which model to use among the applicable models and indicates it to the UE.

For the option 1, the UE can decide one model to use among the applicable models. For applicability check, the LMF can provide some assistance information (e.g., additional condition like DL-PRS configuration) or some criteria (e.g., POS QoS requirement on accuracy/latency) to UE and the UE checks whether each model is applicable or not for the functionality.

The option 2 is almost same with the option 1 except the point that the UE can indicate the selected model to the LMF. However, we don’t see any reason why the LMF should be aware of the model used at the UE side. In our view, the model used at UE side can be transparent to the LMF as in the case of option 1 unless the LMF decides which model to use as in the option 3.

For the option 3, the LMF can decide which model to use among the applicable models and indicates it to the UE. In this case, the LMF should be able to check whether each model at the UE side is applicable or not for the functionality first. For the applicability check, the LMF needs to know not only the NW-side additional conditions but also the UE-side conditions. However, it seems ineffective/impractical for UE to provide the UE-side additional conditions (e.g., requirement on computation/storage/power/processing) for the LMF.

Based on the discussion above, the option 1 seems most reasonable.

Proposal. 6: For POS Case 1, RAN2 assume UE decides/selects which model to use among applicable models for the configured functionality.

2.3 Performance monitoring

For UE-side model case, the UE who deploy the model can do performance monitoring as baseline, but it may not be the only entity for performance monitoring. From our understanding, RAN1 is still considering at least the LMF as another monitoring entity for UE-side model case.

The discussion on which entity to perform performance monitoring for UE-side model case is actually associated with the discussion on what monitoring metric to use for the performance monitoring.

- Model output based: This type is most straightforward one which is based on the comparison between model output and label. For this, the monitoring entity should be able to have both model output (e.g., estimated location from AI/ML) and label (e.g., known location or estimated location from other methods).

- Model input based: This type is using the feature of input data matching or not to decide whether the AI model is still applicable to the current situation. This is more like stochastic way to monitor the model, which may require the AI model shows some certain connection with the input data characteristic, e.g., certain distribution etc.

- Other (measurement) based: This type is usually consisting of side information which associates the AI model. Such side information could be reflected by other measurement instead of the measurement for data input. For example, if certain SNR level or RSRP level shows for an AI model characters, based on the measurement of the non-AI used reference signals can also represents the current SNR or RSRP level, which could be used for the monitoring.

In RAN1#116bis, it is agreed to study the following options for label-based (model output based) monitoring.

- Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Since those options are still under discussion in RAN1, we can just wait for RAN1’s conclusion on this to avoid any potential duplicated work.

Proposal. 7: For POS Case1, RAN2 wait for RAN1’s progress on model performance monitoring.

2.4 Functionality management

In RAN2#125bis, RAN2 agreed the following for common LCM framework/signaling.

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

Open issue is whether/how we support “UE autonomous, decision reported to the network” and “Network decision, UE-initiated”. In our understanding, one difference between “UE autonomous, decision reported to the network” and “Network decision, UE-initiated” would be who makes final decision. In case of “UE autonomous, decision reported to the network”, UE applies decision and report to gNB i.e. UE makes final decision, while in case of “Network decision, UE-initiated”, UE requires to get gNB decision to update functionality.

For “UE autonomous, decision reported to the network”, we can consider the following scenario in the POS case 1.

- The LMF can request UE to estimate its location using AI/ML model via LPP RequestLocationInformation message.

- The UE performs the location estimation using AI/ML model, but the performance monitoring result can be ‘fail’.

- The UE can determine to fall back to the legacy positioning method (i.e., “UE autonomous decision”) and reports the estimated location with the fall back indication (i.e., “report to the network”) via LPP ProvideLocationInformation.

For “Network decision, UE-initiated”, we can consider the following scenario for the POS case 1.

- The LMF can request UE to estimate its location using AI/ML model via LPP RequestLocationInformation message.

- The UE performs the location estimation using AI/ML model, but the performance monitoring result can be ‘fail’.

- The UE reports some error cause (e.g., performanceMonitoringFail) via LPP ProvideLocationInformation (i.e., “UE-initiated”).

- The LMF can determine to fall back to some legacy positioning method (i.e., “NW decision”). The LMF requests UE to estimate its location using the legacy method via LPP RequestLocationInformation message.

- The UE reports the estimated location via LPP ProvideLocationInformation.

In the expected signalling flow above, it is assumed that the UE performs the monitoring since both UE autonomous decision and initiation from UE make sense only if the performance monitoring is done at UE side. On the other hand, for “network decision, network-initiated”, it is more practical with LMF-side performance monitoring.

For the UE-side monitoring case, both “UE autonomous, decision reported to the network” and “Network decision, UE-initiated” can work, but the latter can bring additional delay from step4/5 in POS procedure. Thus, in our view, “UE autonomous, decision reported to the network” can be prioritized over “Network decision, UE-initiated”.

Meanwhile, according to the FL summary on POS in RAN1#118 [2], RAN1 is still discussing the following options for functionality/model management for label-based model performance monitoring.

For label-based model performance monitoring of AI/ML positioning Case 1, study the following options: - Option 1: the target UE makes the functionality/model management decision/recommendation (e.g., activate or deactivate a model, update a model, fallback to legacy positioning methods) as a result of model performance monitoring. - The target UE notifies the LMF of its functionality/model management decision/recommendation. - Option 1 is at least applicable if Option A (The target UE side performs monitoring metric calculation) is supported. - Option 2: LMF makes the functionality/model management decision (e.g., model deactivation, fallback) as a result of model performance monitoring, and notifies the target UE. - Option 2 is at least applicable if Option B (The LMF performs monitoring metric calculation) is supported.

Based on the options on the table in RAN1, it is likely that both Option 1 (“UE autonomous, decision reported to the network”) and Option 2 (“network decision, network-initiated”) can be studied further. Meanwhile, “Network decision, UE-initiated” seems to be excluded. Since RAN1 is still studying the feasibility of those options and do not make any conclusion/agreement yet, it is too early for RAN2 to discuss the detailed signaling procedure for functionality management for Case 1.

Proposal. 8: For POS Case1, RAN2 wait for RAN1’s progress on functionality management.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

General aspect:

Proposal. 1: For POS use cases, RAN2 can prioritize the discussion on the case 1 over other cases.

Baseline LCM procedure for Case 1:

Proposal. 2: For POS Case 1, configuration for AI-based location inference consists of the following two parts:
- Set of inference related parameters that can be used for applicability check (e.g., DL-PRS configuration). 
- Inference configuration to configure UE to perform inference (e.g., request for AI-based location inference).

Proposal. 3: For POS Case 1, RAN2 confirm that set of inference related parameters (e.g., DL-PRS configuration) is provided in step 3 and inference configuration (e.g., request for AI-based location inference) is provided in step 5.

Proposal. 4: For POS Case 1, RAN2 confirm that there is no LMF’s control to allow UE to send unsolicited LPP ProvideCapabilities message for applicability report. Once an LPP session has been established, UE can send LPP ProvideCapabilities message to LMF whenever it is needed as in legacy.

Proposal. 5: For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” manner without further enhancement.

- Proactive case: When the applicability change due to UE’s internal condition, UE can send an unsolicited LPP ProvideCapabilities message to LMF .
- Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF upon reception of ProvidAssistanceData message.

Model selection:

Proposal. 6: For POS Case 1, RAN2 assume UE decides/selects which model to use among applicable models for the configured functionality.

Performance monitoring:

Proposal. 7: For POS Case1, RAN2 wait for RAN1’s progress on model performance monitoring.

Functionality management:

Proposal. 8: For POS Case1, RAN2 wait for RAN1’s progress on functionality management.

4	Reference

- RP-240774, ‘Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface’, Qualcomm

- R1-2407269, ‘Summary #3 of specification support for positioning accuracy enhancement’, Ericsson

- R1-2409032, ‘Summary #5 of specification support for positioning accuracy enhancement’, Ericsson