3GPP TSG-RAN WG2 Meeting #128                                 R2-2409829

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda item:		8.3.5

Source:				Samsung

Title:					Discussion on Generalization Issues for AI/ML Mobility

Document for:		Discussion & Decision

1	Introduction

In RAN2#127bis meeting, the following statement is agreed.

“Discussion on what (type of information)/how generalization study can take place in Nov. meeting”

This contribution provides our view on generalization issues for AI/ML mobility

2	Discussion

In recent RAN2 meetings, the performance for RRM measurement prediction has been verified for temporal and frequency domain predictions in many aspects. However, the performance for event prediction has hardly been verified until now. Considering that, at this point, we propose to study the generalization issues on RRM measurement prediction with high priority. Moreover, since both RRM measurement prediction and event prediction are expected be affected by similar aspects in generalization issue, it seems sufficient to check generalization issue on RRM measurement prediction.

Proposal 1: RAN2 to prioritize generalization issues on RRM measurement prediction while deprioritizing generalization issues on measurement/RLF/HOF event prediction.

There could be various aspects to consider for the generalization of AI/ML model performance.

Mobility:

The performance has been verified for UEs with one of fixed speeds among 30 km/h, 60 km/h, 90 km/h, and 120 km/h by many companies during last RAN2 meetings. Evaluation results show that UE speed affects the prediction performance in temporal domain because higher UE speed result in faster channel changes thus making prediction more difficult. In reality, there are a lot of UEs with different speeds in a cell so it is not desirable to use speed-specific model for RRM measurement prediction in temporal domain. Therefore, we propose to study generalization issue on RRM measurement prediction in temporal domain with different UE speed.

Observation 1: The performance of RRM measurement prediction is affected by UE speed.

Proposal 2: RAN2 to study generalization issue on RRM measurement prediction in temporal domain with different UE speeds.

To measure the impact of generalization issue on speed, we propose to use the following metric.

Lgeneral, speed (Loss of generalization on speed):

Lgeneral, speed=RDgeneral, speed-RDref, speedRDref,speed

RDgeneral, speed (RSRP difference of generalization on speed): RSRP difference for generalized model. AI/ML model is trained with combined training dataset for 30/60/90/120 km/h cases and the performance is compared with the predicted RSRP value and the real RSRP value. For predicted RSRP values, it is inferred with combined test dataset for 30/60/90/120 km/h cases.

RDref, speed (RSRP difference as reference on speed): Averaged RSRP difference for each reference model, where model is trained with specific speed dataset and RSRP is predicted with the specific speed test dataset. i.e.,

RDref, speed=k=30, 60,90,120RDref, speed_k4

RDref, speed_k (RRM difference with k km/h dataset): AI/ML model is trained with k km/h training dataset and the performance is compared with the predicted RSRP value and the real RSRP value. For predicted RSRP values, it is inferred with k km/h test dataset.

Proposal 3: RAN2 to study the generalization issue with different UE speed in temporal domain RRM prediction based on the following metric.

Lgeneral, speed (Loss of generalization on speed):

Lgeneral, speed=RDgeneral, speed-RDref, speedRDref,speed

Cell deployment:

In reality, cell deployment is irregular thus every cell has different properties from other cells. Therefore, generalization on different cells must be one of important issues to be checked. However, as for agreed SLS simulation assumptions for this study item, all base stations are deployed regularly and all antenna configurations are same. In our opinion, it seems difficult to capture the impact of generalization on different cells unless we change our SLS assumptions. Thus, we propose to deprioritize the evaluation of generalization issue with different cells at least for the current simulation assumptions.

Proposal 4: RAN2 to deprioritize the study on generalization issue on RRM measurement prediction with different cells at least for the current simulation assumptions.

Frequency combinations:

During last RAN2 meetings, performance evaluation for RRM measurement prediction in frequency domain has been conducted using only a single option for source/target frequency combination (i.e., 2 GHz and 4 GHz) as agreed for SLS evaluation. In reality, there can be hundreds of frequency combinations. If different AI/ML model is used for each frequency combination, it can be too inefficient. Therefore, we propose to study generalization issue on RRM measurement prediction in frequency domain with different frequency combinations.

Proposal 5: RAN2 to study generalization issue on RRM measurement prediction in frequency domain with different frequency combinations.

Considering that path loss in wireless channel is mainly determined by a frequency of carrier, if source/target frequency combination information is also given to AI/ML model as input, we can have one general model for various frequency combinations. Thus, we propose to study the impact of including source and target frequency information as AI/ML input to evaluate RRM measurement prediction in frequency domain with various frequency pairs.

Proposal 6: RAN2 to verify the impact of including source/target frequency information as input of AI/ML to study generalization issue on RRM measurement prediction in frequency domain with different frequency combinations

To measure the impact of generalization issue on frequency pairs, we propose to use the following metric.

Lgeneral,freq (Loss of generalization on frequency):

Lgeneral, freq=RDgeneral, freq-RDref, freqRDref, freq

RDgeneral, freq (RSRP difference of generalization on frequency): RSRP difference for generalized model. AI/ML model is trained with combined training dataset for various frequency combinations (e.g., fs, 1→ft,1, fs, 2→ft,2, fs,3→ft,3, …) and the performance is compared with the predicted RSRP value and the real RSRP value. For predicted RSRP values, it is inferred with combined test dataset for various frequency combinations (e.g., fs, 1→ft,1, fs, 2→ft,2, fs,3→ft,3, …).

RDref,freq (RSRP difference as reference on frequency): Averaged RSRP difference for each reference, where model is trained with specific frequency combination dataset and RSRP is predicted with the specific frequency combination test dataset. i.e.,

RDref,freq=k=1NRDref,freq_kN

RDref,freq_k (RRM difference with frequency combination k dataset, i.e., fs, k→ft,k): AI/ML model is trained with specific frequency combination k training dataset and the performance is compared with the predicted RSRP value and the real RSRP value. For predicted RSRP values, it is inferred with specific frequency combination k test dataset. In here, frequency combination information does not need to be used as input for AL/ML model.

Proposal 7: RAN2 to study the generalization issue with different source/target frequency combinations in frequency domain RRM prediction based on the following metric

Lgeneral,freq (Loss of generalization on frequency):

Lgeneral, freq=RDgeneral, freq-RDref, freqRDref, freq

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: RAN2 to prioritize generalization issues on RRM measurement prediction while deprioritizing generalization issues on measurement/RLF/HOF event prediction.

Observation 1: The performance of RRM measurement prediction is affected by UE speed.

Proposal 2: RAN2 to study generalization issue on RRM measurement prediction in temporal domain with different UE speeds.

Proposal 3: RAN2 to study the generalization issue with different UE speed in temporal domain RRM prediction based on the following metric

Lgeneral, speed (Loss of generalization on speed):

Lgeneral, speed=RDgeneral, speed-RDref, speedRDref,speed

Proposal 4: RAN2 to deprioritize the study on generalization issue on RRM measurement prediction with different cells at least for the current simulation assumptions.

Proposal 5: RAN2 to study generalization issue on RRM measurement prediction in frequency domain with different frequency combinations.

Proposal 6: RAN2 to verify the impact of including source/target frequency information as input of AI/ML to study generalization issue on RRM measurement prediction in frequency domain with different frequency combinations

Proposal 7: RAN2 to study the generalization issue with different source/target frequency combinations in frequency domain RRM prediction based on the following metric

Lgeneral,freq (Loss of generalization on frequency):

Lgeneral, freq=RDgeneral, freq-RDref, freqRDref, freq

4	Reference