3GPP TSG-RAN WG2 Meeting #128	R2-2409847

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.7.5

Source: 	Fujitsu

Title: 	Discussions on RLC enhancements

Document for:	Discussion and decision

Introduction

On RLC retransmission enhancement in XR, there are following agreements in RAN2#127bis meeting [1] for ensuring timely RLC retransmissions and for avoiding unnecessary retransmissions:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this contribution, we would like to discuss the potential enhancements to achieve timely RLC retransmissions and to avoid unnecessary RLC retransmissions for XR.

Discussion

Ensuring timely RLC retransmission(s) for XR

As agreed by RAN2, for ensuring timely RLC retransmissions for XR, the option of enhanced status reporting is excluded. For the remaining two options, i.e., autonomous retransmission and polling enhancements, we would like to discuss how each option affects the capacity and packet delay in the following.

Autonomous retransmission can achieve high reliability for XR traffics in case of small PDB. Moreover, it can reduce the packet delay to a great extent, since the RLC Tx side can perform RLC retransmissions in the consecutive transmission occasions without waiting for the status reports from the RLC Rx side. In the previous meetings, some companies have concerned that autonomous retransmissions have impact on the system capacity. From our perspective, however, autonomous retransmission can be applied in certain cases to save the radio resource occupation. For example, the trigger of autonomous retransmission can be e.g., when the remaining time of the data becomes lower than a threshold, or when the network configures the UE to perform the autonomous retransmissions. When the PDB of the XR traffic is reached, the autonomous retransmission in the corresponding RLC entity should be stopped. Therefore, the impact of autonomous retransmission to the system capacity would be acceptable.

Observation 1: Autonomous retransmission achieves high reliability and low packet delay. It has acceptable impact on system capacity with proper triggers.

On the other hand, the options enhanced polling may be simpler compared with autonomous retransmissions, and the specification impact of enhanced polling is less with introducing small values of pollPDU or pollByte. However, enhanced polling has overhead due to transmissions of more RLC control PDUs, since the RLC Tx side would transmit polling and the RLC Rx side would transmit the status reports more frequently. Besides, the transmission of polling and status reports would delay the transmission of data, which also has impact on the system capacity. Moreover, the option of enhanced polling may lead to longer packet delay in the RLC Tx side due to transmissions of polling and waiting for status report before the RLC retransmissions.

Observation 2: Retransmission based on enhanced polling is simpler and it has smaller specification impact. But it may introduce more overhead, and it also has impact on capacity and packet delay.

Based on the above observations, we propose to adopt autonomous retransmission to achieve timely RLC retransmission, i.e., the Tx side of an AM RLC entity can perform retransmission even if it does not receive the negative acknowledgement. By this way, the number of RLC retransmission(s) is increased in the PDB compared with the current scheme, and thus the reliability of the XR traffic can be ensured.

Proposal 1: RAN2 adopts autonomous retransmissions to achieve timely RLC retransmission in RLC AM.

Proposal 1a: Autonomous retransmission can be activated/deactivated by the network or can be triggered when the remaining time of the data becomes lower than a threshold.

The autonomous RLC retransmission(s) can occur in the consecutive slots or can have an interval. If positive acknowledgement is received by the transmitting side of the AM RLC entity, these autonomous RLC retransmission(s) can be stopped. If negative acknowledgement is received by the transmitting side of the AM RLC entity, the transmitting side of the AM RLC entity can continue the autonomous RLC retransmissions.

The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC. In our opinion, these autonomous RLC retransmissions should not be counted into RETX_COUNT, so that RETX_COUNT would not easily reach the maximum retransmission number (maxRetxThreshold) and RLF would not be initiated.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

Avoiding unnecessary retransmissions

For avoiding unnecessary RLC retransmissions, a “combined” approach has been agreed by RAN2 in the last meeting, where Tx side stops transmissions of an outdated SDU and Rx side abandons the SDU based on a local timer. In addition, Rx side informs Tx side about the abandoned SDUs and RAN2 assume existing status report can be reused as a baseline.

The first issue that can be discussed is when the Tx side stops transmissions of an outdated SDU. In our assumption, RLC Tx side can stop retransmissions of an RLC SDU e.g., upon receiving PDCP discard indication about the discarded RLC SDU, or when the number of transmissions reaches a number configured by the network based on the PDB, or when receiving the status report from RLC Rx side indicating that the RLC SDU is ACKed. From our perspective, each option can work, so we have no strong preference.

Proposal 4: In the combined approach, Tx side stops retransmissions of an RLC SDU upon receiving PDCP discard indication or when the number of transmissions reaches a number configured by the network based on the PDB or when receiving the status report.

The next issue is how to determine the local timer at the Rx side. For the uplink XR service, the Rx side is at the network side and the local timer need not to be specified in the standard. On the other hand, for the downlink XR service, the Rx side is at the UE side, so the definition of the local timer and the related UE behaviours should be specified. In our understanding, the local timer should be configured by the network based on the PDB of the XR service, like the PDCP discard timer. How to configure the value of the local timer may depend on the network implementation. For an RLC SDU the associated local timer can be started when another RLC SDU with a higher SN is received in the Rx side. If the RLC SDU is received, the local timer associated with it can be stopped if running. When the local timer expires, Rx side can consider the associated RLC SDU as abandoned.

Proposal 5: The local timer at the Rx side is configured by the network.

Another issue to be discussed is whether there are issues if the existing status report from Rx side to Tx side is reused. In our opinion, in case that Rx side abandons an RLC SDU, by the existing status report from Rx side to Tx side, Tx side does not know that the abandoned RLC SDU is not received by the Rx side. However, for XR, it is not necessary in Tx side to trigger RLF when the transmission failure of an RLC SDU happens. Moreover, PDCP Tx side does not need to (re)transmit the outdated PDCP SDU/PDU as well. Therefore, we think that the existing status report can be reused.

Proposal 6: Rx informs Tx side about the abandoned SDUs by the existing status report where the SDU is indicated as ACK.

In the last meeting, there is an FFS issue if some indication is sent from Tx side to Rx side. In our understanding, the indication from Tx side to Rx side is not necessary, since the Rx side can already abandon the outdated RLC SDU based on the local timer. The indication from Tx side to Rx side would introduce more specification impact, e.g., on the content of the indication and the related behaviours at Tx side and Rx side, which is not preferred from our perspective.

Proposal 7: No indication is sent from Tx side to Rx side.

Conclusion

In this contribution, we have discussed the RLC enhancements to achieve timely RLC retransmission and to avoid unnecessary retransmissions. We have the following proposals:

Proposal 1: RAN2 adopts autonomous retransmissions to achieve timely RLC retransmission in RLC AM.

Proposal 1a: Autonomous retransmission can be activated/deactivated by the network or can be triggered when the remaining time of the data becomes lower than a threshold.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

Proposal 4: In the combined approach, Tx side stops retransmissions of an RLC SDU upon receiving PDCP discard indication or when the number of transmissions reaches a number configured by the network based on the PDB or when receiving the status report.

Proposal 5: The local timer at the Rx side is configured by the network.

Proposal 6: Rx informs Tx side about the abandoned SDUs by the existing status report where the SDU is indicated as ACK.

Proposal 7: No indication is sent from Tx side to Rx side.

References

- R2-2409215, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#127bis meeting