3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409853

Orlando, USA, 18th – 22nd Nov, 2024

Source:	CATT

Title:	Clarification on the Terminology of Peer UE

Agenda Item:	7.5.3

Document for:	Discussion and Decision

Introduction

In TS38.331, terminology of peer UE is used in many subclauses, but it has different meaning even in the same subclauses. Hence, in this contribution, we try to find a way to improve the readability of the specification.

Discussion

In subclause 5.8.9.1.1of TS38.331 [1], the following was captured:

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases: - the release of sidelink DRBs associated with the peer UE, or L2 U2U Relay UE and peer L2 U2U Remote UE in case of L2 U2U Relay operation, as specified in clause 5.8.9.1a.1; - the establishment of sidelink DRBs associated with the peer UE, or L2 U2U Relay UE and peer L2 U2U Remote UE in case of L2 U2U Relay operation, as specified in clause 5.8.9.1a.2; - the modification for the parameters included in SLRB-Config of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.2; - the release of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.5; - the establishment of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.6; - the modification for the parameters included in SL-RLC-BearerConfig of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.6; …… - the (re-)configuration of the peer UE to perform sidelink DRX;

In the above table, the meaning of peer UE in different bullet is different. For example:

For the first two bullets of the above table, the “peer UE” includes two types of UEs, listed below:

- Type 1: The target UE who is performing sidelink communication direcely with the UE (non-Relay case);

- Type 2: The U2N Relay UE (U2N Relay case).

But in the third bullet of the above table, peer UE includes four types of UEs, listed below:

- Type 1: The target UE who is performing sidelink communication directly with the UE (non-relay case);

- Type 2: The U2N Relay UE (U2N Relay case);

- Type 3: The U2U Relay UE (U2U Relay case);

- Type 4: The target U2U Remote UE (U2U Relay case);

Proposal 1: Suggest RAN2 to confirm that although the same terminology of peer UE is used, but it refers to different types of UEs in the same or different sections of TS38.331. E.g., in somewhere, it does not include the U2U relay/remote UE; while in other places, it includes the U2U relay/remote UE.

In order to improve the readability and avoid the misunderstanding, three options can be considered:

- Option 1: The peer UE only refers to Rel-16 V2X target UE and Rel-17 U2N Relay UE (same principle as in the Rel-17 specification);

- Option 2:  The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. No need to identify it in the general description.

- Option 3:  The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. A definition of peer UE or a NOTE can be added in the specification.

For Option 1, in the subclauses which mentioned the peer UE, RAN2 should check whether it is only applied to Rel-16 V2X target UE and Rel-17 U2N Relay UE one by one. And if the answer is No, clarification should be added in the corresponding subclause(s). In order to implement Option1, it will consume many times to perform the check.

For Option 2, considering the procedure has clearly described which the peer UE refers to, hence, in the general description such as in subclause 5.8.9.1.1, there is no need to identify the legacy scenario and the U2U relay operation. The corresponding modification can be performed in subclause 5.8.9.1.1:

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases: - the release of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.1; - the establishment of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.2;

For Option 3, the same modification as Option 2 should be performed. Furthermore, a definition or a NOTE should be added to clarify the peer UE includes the V2X sidelink UE, the U2N Relay, the U2U Relay and peer U2U Remote UE.

From our perspective, compared with Option 3, Option 2 is slightly preferred. Take a step back, Option 3 is also acceptable for us.

Proposal 2: Suggest RAN2 to discuss how to handle the issue that the same terminology of peer UE refers to different types of UEs in same/different sections:

- Option 1: The peer UE only refers to Rel-16 V2X target UE and Rel-17 U2N Relay UE (same principle as in the Rel-17 specification);

- Option 2: The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. No need to identify it in the general description;

- Option 3: The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. A definition of peer UE or a NOTE can be added in the specification.

Proposal 3: If Option 2 is accepted, RAN2 to agree the modification in subclause 5.8.9.1.1 of TS38.331.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Suggest RAN2 to confirm that although the same terminology of peer UE is used, but it refers to different types of UEs in the same or different sections of TS38.331. E.g., in somewhere, it does not include the U2U relay/remote UE; while in other places, it includes the U2U relay/remote UE.

Proposal 2: Suggest RAN2 to discuss how to handle the issue that the same terminology of peer UE refers to different types of UEs in same/different sections:

- Option 1: The peer UE only refers to Rel-16 V2X target UE and Rel-17 U2N Relay UE (same principle as in the Rel-17 specification);

- Option 2: The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. No need to identify it in the general description;

- Option 3: The peer UE refers to the target UE which has direct PC5 or indirect PC5 connection with the UE which depends on the detailed procedure description. A definition of peer UE or a NOTE can be added in the specification.

Proposal 3: If Option 2 is accepted, RAN2 to agree the modification in subclause 5.8.9.1.1 of TS38.331.

The corresponding TP corresponding to Option 2 is in the Annex.

Reference

- TS38.331  NR; Radio Resource Control (RRC) protocol specification (Release-18)

Annex

-----------------------------------------------------Text Proposal---------------------------------------------------

START OF CHANGE

#### 5.8.9.1	Sidelink RRC reconfiguration

##### 5.8.9.1.1	General

……

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases:

-	the release of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.1;

-	the establishment of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.2;

-	the modification for the parameters included in SLRB-Config of sidelink DRBs associated with the peer UE, as specified in clause 5.8.9.1a.2;

-	the release of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.5;

-	the establishment of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.6;

-	the modification for the parameters included in SL-RLC-BearerConfig of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.6;

-	the release of PC5 Relay RLC channels for L2 U2N/U2U Relay UE and Remote UE, as specified in clause 5.8.9.7.1;

-	the establishment of PC5 Relay RLC channels for L2 U2N/U2U Relay UE and Remote UE, as specified in clause 5.8.9.7.2;

-	the modification for the parameters included in SL-RLC-ChannelConfigPC5 of PC5 Relay RLC channels for L2 U2N/U2U Relay UE and Remote UE, as specified in clause 5.8.9.7.2;

-	the release of sidelink carrier associated with the peer UE, as specified in clause 5.8.9.1b.1;

-	the addition of sidelink carrier associated with the peer UE, as specified in clause 5.8.9.1b.2;

-	the (re-)configuration of the peer UE to perform NR sidelink measurement and report.

-	the (re-)configuration of the sidelink CSI reference signal resources and CSI reporting latency bound;

-	the (re-)configuration of the peer UE to perform sidelink DRX;

-	the (re-)configuration of the latency bound of SL Inter-UE coordination report;

-	the (re-)configuration of the local UE ID pair for L2 U2U Remote UE and its peer L2 U2U Remote UE by L2 U2U Relay UE.

-	the response to the request in a RemoteUEInformationSidelink message for the SFN-DFN offset from the L2 U2N Remote UE;

-	the change in the value of the SFN-DFN offset at the L2 U2N Relay UE.

NOTE:	It is up to L2 U2N Relay UE implementation to determine when the SFN-DFN offset has changed in value to a degree requiring an update to be sent to the L2 U2N Remote UE.

……

END OF CHANGE