3GPP TSG-RAN WG2 Meeting #128	                                    R2-2409875

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.6.4

Source: 	OPPO

Title:  	Discussion on conditional LTM

Document for:	Discussion and Decision

Introduction

In latest meeting, RAN2 initially discussed conditional LTM and the following agreements were reached:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this contribution, we discuss the open issues for supporting conditional LTM.

Discussion

Conditional LTM is introduced in Rel-19 which aims to achieve high robustness by enabling the procedure to be executed without explicit signalling exchange with source cell beforehand. As indicated in WID, conditional LTM is limited to intra-CU scenario, which is same as Rel-18 LTM. In the following section, we will analyse the additional part for supporting CLTM.

Early sync

In last meeting, it has agreed that PDCCH ordered early TA acquisition is supported for conditional LTM. For Rel-18 NW triggered intra-CU LTM, the early UL synchronization is triggered by PDCCH order, UE performs PRACH transmission without receiving RAR. Then, the candidate cell indicates the computed TA to source cell and the source cell forwards the TA to UE through LTM cell switch MAC CE. For CLTM, explicit NW signalling which triggers cell switch execution is omitted to guarantee a robust HO procedure, that is, there is no LTM cell switch MAC CE delivered to UE.

To support early sync procedure for CLTM, RA response can be reused for TA provision. And there are two way for the RA Response reception:

- Option 1: RA Response is transmitted by candidate cell

- Option 2: RA Response is transmitted by serving cell

In our understanding, both options work. While option 2 have more benefit as the serving cell service will not be impacted due to RAR monitoring. As the RACH is performed towards a candidate cell, and UE is maintaining the connection with serving cell, monitoring RAR on candidate cell may cause additional data interruption for serving cell. The RA response for early sync could be delivered to UE by serving cell in CLTM.

In addition to TA, legacy RAR also includes the fields to indicate TC-RNTI and UL grant. While the candidate cell configuration provided to UE already includes C-RNTI for the candidate cell, there is no necessary to provide redundant RNTI to UE. Regarding to UL grant of candidate cell, it is waste of resources to provide candidate cell resource before cell switch since UE will only perform data transmission with current serving cell instead of candidate cell. Thus, there is no need to include the field of UL grant (candidate cell) and TC-RNTI in RAR for the RACH triggered for early synchronization. RAN2 can further study whether to introduce new RAR format or reuse legacy Absolute Timing Advance Command MAC CE/Timing Advance Command MAC CE. The pros and cons are analysed in the following table:

|  | pros | cons |
| --- | --- | --- |
| Option1: legacy RAR format without UL grant (candidate cell) and TC-RNTI | Less spec impact, legacy RAR procedure can be reused. | Additional signaling overhead due to the field of TC-RNTI and UL grant. |
| Option 2: legacy Absolute Timing Advance Command MAC CE | The format is suitable | spec impact is more than option1 as legacy Absolute Timing Advance Command MAC CE is only used for 2-step RACH |
| Option 3: introduce new TA indication MAC CE | Clean design | Effort to design new MAC CE |
| Option 4: legacy TA MAC CE | Signaling overhead is small | Only TA adjustment can be indicated. |

For CLTM, RA response is delivered to UE by serving cell for early sync procedure.

RAN2 determine the RA Response format from the following options:

- Option1: legacy RAR format with ignoring the field of UL grant (candidate cell) and TC-RNTI

- Option 2: legacy Absolute Timing Advance Command MAC CE

- Option 3: introduce new TA indication MAC CE

Besides PDCCH ordered early sync, there is another alternative for triggering the early sync procedure, i.e. event triggered early sync. UE can evaluate the candidate cell/RSs and trigger early sync to a triggered cell/beam to acquire the TA value. In R19 inter-CU LTM, companies also reach consensus that the event configuration can be used for early sync scenario since UE have the real-time L1 measurement of the candidate cells/RSs.

Event triggered early sync to candidate cell(s) is supported for CLTM.

Furthermore, if RAR reception is supported for early sync to candidate cell, it is questionable whether and how the TA maintenance is supported.

For serving cells, maintenance of uplink time alignment relies on a TA timer that is associated with a TAG. UE starts/restarts the corresponding TA timer if TA is indicated by NW. For early sync procedure, if RAR including TA information is provided to UE through RAR, the TA maintenance associated to candidate cell should be studied.

One straightforward way is to reuse TAG for TA indication and maintenance. Similar as serving cell TA maintenance, UE starts or restarts the timeAlignmentTimer associated with this TAG when a timing advance command is received in a random access response message for a candidate cell belonging to a TAG. In that way, TAG ID should be unique at least among serving cell(s) and candidate cells. Another way is to maintain the TA of candidate cell in the granularity of cell, i.e. the TA timer is maintained per candidate cell.

Candidate cell TA validity is guaranteed by TA timer.

RAN2 study how to perform TA maintenance for candidate cell for CLTM with following options:

- Option1: per TAG

- Option 2:per candidate cell

L1 measurement reporting and early TCI activation

In Rel-18, NW configured L1 measurement reporting for candidate cells are introduced, and we further introduce event triggered L1 measurement reporting in Rel-19 for signalling overhead reduction. From CLTM perspective, we do see the need on performing L1 measurement reporting to assist NW to make decision for NW triggered procedures. For example, NW can trigger the early sync procedure by sending PDCCH order which indicates the proper contention free PRACH resources as well as beam info according to the received L1 measurement results. Similar for early TCI state activation/deactivation, NW need to know well about the radio conditions of each candidate TCI state . And both NW configured and event triggered L1 reporting can be supported by CLTM.

Both NW configured and event triggered L1 measurement reporting are supported for CLTM.

RACH-less CLTM execution

As in NW triggered intra-CU LTM, both RACH-based and RACH-less cell switch can be supported for CLTM execution. For RACH-less CLTM, as the cell switch execution is triggered by UE itself, NW may not be able to schedule a dynamic grant to UE immediately upon cell switch execution. In last meeting, CG based RACH-less CLTM is supported, and there is one FFS on whether the DG based RACH-less CLTM should also be supported. To support DG based RACH-less CLTM, one essential issue is to align the beam info between UE and NW, that is, NW have to know whether the UE is accessing the cell and provide the dynamic grant on the correct beam that UE is monitoring on. The beam info alignment can be resolved by introducing additional indication info between UE and NW, while one may argue that the additional beam indication is against with the motivation of supporting CLTM.

To be simple, we prefer to only support CG based RACH-less for CLTM.

Only CG based RACH-less CLTM is supported if TA is available. DG based RACH-less is not supported for CLTM.

And for CG resource selection for RACH-less CLTM, UE can follow the way in RACH-less HO in NTN and mIAB. UE selects a configured uplink grant for initial uplink transmission according to the configured threshold.

Threshold based TCI state selection is used for RACH-less CLTM.

The CG resources associated with the selected TCI state is applied for first UL transmission.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	For CLTM, RA response is delivered to UE by serving cell for early sync procedure.

Proposal 2	RAN2 determine the RA Response format from the following options:

	Option1: legacy RAR format with ignoring the field of UL grant (candidate cell) and TC-RNTI

	Option 2: legacy Absolute Timing Advance Command MAC CE

	Option 3: introduce new TA indication MAC CE

Proposal 3	Event triggered early sync to candidate cell(s) is supported for CLTM.

Proposal 4	Candidate cell TA validity is guaranteed by TA timer.

Proposal 5	RAN2 study how to perform TA maintenance for candidate cell for CLTM with following options:

Option1: per TAG

Option 2:per candidate cell

Proposal 6	Both NW configured and event triggered L1 measurement reporting are supported for CLTM.

Proposal 7	Only CG based RACH-less CLTM is supported if TA is available. DG based RACH-less is not supported for CLTM.

Proposal 8	Threshold based TCI state selection is used for RACH-less CLTM.

Proposal 9	The CG resources associated with the selected TCI state is applied for first UL transmission.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx