3GPP TSG-RAN2 Meeting #128	R2- 2409910

# Orlando, United States, Nov 18th – 22nd, 2024

Agenda item:		8.7.4.1 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on LCP enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to define additional Logical channel priority handling based on the delay/deadline information.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This document continues discussion on further details of LCP enhancement for XR.

2.	Discussion

In RAN2#127bis meeting [2], it is agreed as a baseline that the additional LCH priority is applied for the whole LCH with LCH priority adjusted data, including the 1st Round and 2nd Round of the LCP procedure. However, it was still remained as an FFS on whether the UE can further check whether the LCH priority can be adjusted on the 2nd Round of LCP procedure, without causing additional UE complexity.

As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

In our understanding, in order to apply the additional LCH priority, the UE needs to check whether there is LCH priority-adjusted data for each LCH before the UE starts LCP procedure based on received uplink grant. Here, given that LCH priority adjusted data is determined based on remaining time of PDCP discard timer handled in the PDCP layer, PDCP layer indicates to the MAC layer if there is any LCH-priority adjusted data for each LCH.

Then, if the UE is allowed to change the priority during the current LCP procedure (i.e., before the 2nd round), the UE needs to check again with PDCP layer on whether LCH priority-adjusted data still remains for each LCH, causing additional UE complexity on LCP procedure. On the other hand, if the UE determines whether to apply additional LCH priority based on the size of LCH priority-adjusted data as suggested in [3], the UE needs to calculate the amount of LCH priority-adjusted data in PDCP and RLC layer for each LCH right before the LCP procedure, which also causes the additional UE complexity.

In our view, UE complexity on LCP enhancement should be carefully considered, since the current LCP procedure is time-critical mechanisms. In this sense, RAN2#126 meeting [4], it was confirmed that the delay-aware LCP should consider the UE complexity. Therefore, it would be safer to simplify the delay-aware LCP by applying the same priority both in 1st and 2nd Rounds of LCP procedure, in order to ensure enough time to prepare the TB after receiving the uplink grant.

Proposal 1. Once the additional LCH priority is applied, the additional LCH priority is applied to both 1st and 2nd round of the current LCP procedure.

In RAN2#127bis meeting [2], it is agreed to define additional delay threshold to determine LCH-priority adjusted data

Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

On the other hand, during the RAN2#125bis meeting [3], some companies suggested to consider the PDU set importance for delay-aware LCP for XR. In our understanding, low importance data should not be prioritized in congestion scenario, since the transmission of low importance data is not essential although it has less remaining time due to reduced PDCP discard timer value. Therefore, when the PSI-based discard is activated, low importance data should not be considered as LCH priority-adjusted data.

Proposal 2. When the PSI-based discard is activated, low importance data should not be considered as LCH priority-adjusted data.

Meanwhile, according to the current LCP procedure, only PBR of each LCH is included in the first round. That is, if the LCH1 has LCH priority-adjusted data and LCH2 has non-LCH priority-adjusted data, even though LCH1 is adjusted with higher priority over LCH2, the MAC entity would assemble the data from LCH1 only up to its PBR, then the data in LCH2 will be included before including the remaining data in LCH1. In this case, if the amount of buffered LCH priority-adjusted data in LCH1 is larger than the PBR, non LCH priority-adjusted data in LCH2 will be included prior to the LCH priority-adjusted data in LCH1.

Therefore, in order to ensure that all the LCH priority-adjusted data in a logical channel can be included prior to other data in another logical channel, it should be possible to set the PBR of LCH with LCH priority-adjusted data to infinity.

Proposal 3. For adjustment of LCP of LCH with LCH priority-adjusted data, the PBR should be set to infinity.

In RAN2#127 meeting [5], it was remained as FFS on whether the additional priority for LCH with LCH priority-adjusted can be applied to intra-UE prioritization as well.

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In NR Rel-16, the intra-UE prioritization mechanism is introduced to support tighter latency and reliability. In the Rel-16 intra-UE prioritization, when the two UL grants are overlapped, the priority of each UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU. In other words, logical channel priority is also considered when the UE determines which UL grant should be prioritized.

However, the Rel-16 intra-UE prioritization does not consider the remaining time of data, and it may have trouble to satisfy strict delay requirement for XR service because one UL grant which is used for data from the highest priority of logical channel may be always prioritized over any other UL grant. For example, when two UL grants are given for XR service and one of UL grants has the highest priority logical channel with the data having long remaining time till the PDCP discard timer expiry but another UL grant has the relatively lower priority with the data having short remaining time, the Rel-16 intra-UE prioritization prioritizes the UL grant having highest priority logical channel with the data having long remaining time. However, from XR service perspective, another UL grant for data with short remaining time should be prioritized in this case because it is not urgent to transmit the data with long remaining time in the highest priority logical channel and it can be transmitted later due to sufficient remaining time. In our understanding, it is a similar issue as in delay-aware LCP, since it is defined in order to ensure the transmission of data with short remaining time.

Therefore, given that delay-aware LCP is defined in order to ensure the transmission of LCH priority-adjusted data, the additional LCH priority should also be applied to the intra-UE prioritization as well, in order to ensure the transmission of LCH priority-adjusted data when the two UL grants are overlapped.

Proposal 4. The additional priority for LCH with LCH priority-adjusted data should also be applied on intra-UE prioritization.

3.	Conclusion

In this contribution, it is discussed for the LCP enhancement for XR. This document includes following proposals.

Proposal 1. Once the additional LCH priority is applied, the additional LCH priority is applied to both 1st and 2nd round of the current LCP procedure.

Proposal 2. When the PSI-based discard is activated, low importance data should not be considered as LCH priority-adjusted data.

Proposal 3. For adjustment of LCP of LCH with LCH priority-adjusted data, the PBR should be set to infinity.

Proposal 4. The additional priority for LCH with LCH priority-adjusted data should also be applied on intra-UE prioritization.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024

[3] R2-2408916	LCP enhancements	Ericsson

[4] Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024

[5] Draft Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands, 19 - 23 August, 2024