3GPP TSG-RAN WG2 Meeting #128	R2-2409933

Orlando, US, 18th – 22nd Nov, 2024

Agenda Item:	8.10.2

Source: 	China Unicom

Title:  	Discussion on MRO enhancement for LTM

Document for:	Agreement

Introduction

In the prior meeting, RAN2#127bis and RAN3#125 reached consensus on several enhancements for MRO related to LTM as referenced [1] [2].

- Agreements - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation. - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

For TP to BL CR to TS 38.300: - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions. - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM. - Change “successful handover" definition, as proposed in R3-245230. For TP to BL CR to TS 38.401: - Capture wrong LTM prepared cell selection. Add failure type to F1AP ACCESS AND MOBILITY INDICATION message. CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

This document offers further analysis of stage-3 impacts of MRO enhancement for LTM.

Discussion

- 2.1	MRO enhancement for LTM

RA-based LTM failure

RAN2 has agreed to log and report whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined), which can be explained as the logged information can be applicable for both RA-based and RACH-less based LTM failure cases. Thus, it’s proposed to remove the “Additional information is TBD” sentence and make the clarification.

Proposal 1: RAN2 to clarify that logged information of whether and how the UE got the TA value (gNB indicated or UE determined) in RLF report is also applicable for RA-based LTM failure case.

Sub-optimal LTM cell switch

The network's objective in configuring UE measurement-based TA acquisition or transmitting PDCCH order-based TA via MAC CE is to reduce LTM cell switch time and service interruption. If the network, after sending the configuration or TA via MAC CE, discovers that RACH-based LTM, not RACH-less LTM, is triggered due to the UE didn’t get the valid TA, this indicates a sub-optimal LTM cell switch procedure, leading to the wastage of radio and signaling resources.

For sub-optimal LTM cell switching, RAN2 has agreed to include RACH-less and RACH-based indicators in SHR, but there are still a number of cases that need to be figured out for optimization, e.g., UE may not get valid TA even it was configured to acquire the TA, so it is necessary to log detailed reasons of whether and how UE acquires valid TA in SHR. The detailed analysis includes the following scenarios:

Scenario 1: RACH-less indicator is included in SHR, and the UE acquires the valid TA successfully with either gNB-indicated or UE-determined or both ways.

Scenario 2: RACH based indicator is included in SHR, and the UE acquires the valid TA unsuccessfully with configurated either gNB-indicated or UE-determined or both configurations.

Scenario 3: RACH based indicator is included in SHR, and the UE was not configured with any early TA acquisition configurations.

Taking all the above scenarios into consideration, similar with what RAN2 has agreed for RLF, it’s needed to determine whether and how the UE got the TA value used for a successful LTM switch, which information should be logged into SHR.

Proposal 2: UE logs whether and how the UE got the TA value used for a successful LTM switch (gNB indicated or UE determined) in SHR.

Conclusion

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: RAN2 to clarify that logged information of whether and how the UE got the TA value (gNB indicated or UE determined) in RLF report is also applicable for RA-based LTM failure case.

Proposal 2: UE logs whether and how the UE got the TA value used for a successful LTM switch (gNB indicated or UE determined) in SHR.

References

- (Mattias)_20240823_1000R2_127b_Maint_SONMDT_Mattias_20241017_1300

- RAN3_125bis_agenda_20241018_1430_EOM